- Workpaper

- Consolidation of recommendations R05002 and R04035

- to see resulting

- Standards 9 and 10.

- _____________________________________________________________________________

- Standard 9. Requirements for dealing with Redirects on a Firm basis.

9.1.  The Transmission Customer (TC) shall have the right to request modifications to Points of Receipt and/or Points of Delivery (including source or sink, where required) on a firm basis for a Confirmed Point-to-Point Firm Transmission Service reservation (i.e., Parent Reservation).  This will be referred to as a Redirect on a Firm basis.

9.1.1.  The TC may Redirect on a Firm basis any confirmed Firm Point-to-Point Parent Reservation regardless of the request type.

9.1.2.  A request to Redirect on a Firm basis shall be submitted to the primary Transmission Provider with a request type of REDIRECT.

9.1.3.  A request to Redirect on a Firm basis shall be queued and treated evaluated in the same manner (i.e., same service priority) as any other fFirm pPoint to pPoint request, subject to the other requirements of this standard.

9.1.4.  No additional deposit shall be required for a request to Redirect on a Firm basis.

9.2.  The TC shall be allowed to request a Redirect on a Firm basis for a portion or all of the Capacity Available to Redirect, even if the transmission scheduling rights on the Parent Reservation have been limited due to outages or other reliability-related events. An example is shown in Appendix B.

9.3.  The TC shall be allowed to request a Redirect on a Firm basis for a portion or all of the time period of the Parent Reservation (i.e., bound by the start/stop times of the Parent Reservation).  An example is shown in Appendix B.

9.3.1.  A request for Redirect on a Firm basis must be submitted, and is subject to all request timing requirements consistent with a reservation for Firm service of similar duration.

9.3.2.  A request for Redirect on a Firm basis must represent an established Firm Point-to-Point Service Increment (e.g., Daily, Monthly, etc.) offered by the Transmission Provider.

9.4. The TC’s rights on the Parent Reservation shall remain unaffected during the Transmission Provider evaluation of the request to Redirect on a Firm basis.

9.4.1.  If the request to Redirect on a Firm basis is denied for any reason, all rights and obligations shall remain per the Parent Reservation.  An example is shown in Appendix B.

9.4.2.  The TC shall be allowed to submit and have pending multiple requests for Redirects on a Firm basis against the same Capacity Available to Redirect.  The TP shall evaluate each such request with the knowledge that only those requests up to the Capacity Available to Redirect may ultimately be confirmed.  An example is shown in Appendix B.

9.4.x.  If the TP determines that only a portion of the requested capacity can be accommodated, the TP shall extend to the TC that portion of the capacity (i.e., partial service) that ca be accommodated through a COUNTEROFFER.

9.5.  Upon confirmation of the request to Redirect on a Firm basis, the Capacity Available to Redirect shall be reduced by the amount of the redirected capacity granted for the time period of that Redirect.   An example is shown in Appendix B.

9.5.1.  The TC shall not confirm any request to Redirect on a Firm basis that would exceed the Capacity Available to Redirect at that point in time (i.e., at the time of attempted confirmation and over the time interval of the Redirect).  The TP shall have the right to block any such confirmation. An example is shown in Appendix B.

9.5.2.   The TC shall should withdraw any request to Redirect on a Firm basis that would exceed the Capacity Available to Redirect at that point in time (i.e., at the time of attempted confirmation and over the time interval of the Redirect). The TP shall have the right to withdraw their acceptance of any request to Redirect on a Firm basis that cannot be confirmed due to limitations in the Capacity Available to Redirect by setting the OASIS standard STATUS data element to the value of SUPERSEDED.

9.5.3.   Redirects on a Firm basis shall have all the rights and obligations of an original reservation for Firm service (with the exception of renewal/roll-over rights), including the rights to be Redirected on a Firm and/or Non-Firm basis.

9.6.  For the purposes of curtailment and other capacity reductions, confirmed Redirects on a Firm basis shall be treated comparably to all other types of Firm Point-to-Point Service.

9.6.1.  Curtailments or other capacity reductions to the remaining portion of the reserved capacity on the Parent Reservation shall not affect the Redirect reservation.

9.6.2.  Curtailments or other capacity reductions affecting the reserved capacity on the Redirect reservation shall not affect the Parent Reservation nor result in a reinstatement of capacity on the Parent Reservation.

9.7.  Unless otherwise mutually agreed to by the primary provider and original customer,  a request for Redirect on a Firm basis does not impact the TC’s long term firm renewal rights (e.g., rollover or evergreen rights) on the original path, nor does it confer any renewal rights on the redirected path.

9.8.  Any differences in charges associated with the Redirect on a Firm basis will be settled in accordance with the Transmission Provider’s tariff.

9.8.1.  If not addressed in the Transmission Provider’s tariff or in a Service Agreement, a  credit on the Parent Reservation shall be computed as the total reservation charge divided by the total megawatt hours reserved times the megawatt hours redirected.  The redirected reservation shall be charged as if it were a reservation with a request type of ORIGINAL.

- Standard 10. Requirements for dealing with Redirects on a Non-Firm basis.

10.1.  The Transmission Customer (TC) shall have the right to request an alternate, or secondary, Point of Receipt and/or Point of Delivery (including source and sink, if required) on a non-firm basis for a Confirmed Point-to-Point Firm Transmission Service reservation (i.e., Parent Reservation).  This will be referred to as a Redirect on a Non-Firm basis.

10.1.1.  The TC may Redirect on a Non-Firm basis any confirmed Firm Point-to-Point Parent Reservation regardless of the request type.

10.1.2.  A request to Redirect on a Non-Firm basis shall be submitted to the primary Transmission Provider with a request type of REDIRECT.

10.1.3.  A request to Redirect on a Non-Firm basis shall be queued and treated in the same manner as any other non-firm point to point request, subject to the other requirements of this standard.

10.1.43.  Redirects on a Non-Firm basis shall have a service priority that is lower than non-firm hourly point-to-point service.

10.1.54.  Requests for Redirects on a Non-Firm basis shall specify: the following transmission service attributes in their request:

- SERVICE_INCREMENT=HOURLY

- TS_CLASS=SECONDARY

- TS_TYPE=POINT_TO_POINT

- TS_PERIOD=FULL_PERIOD

- TS_WINDOW=FIXED

TS_PERIOD, TS_WINDOW,and SERVICE_INCREMENT shall specify any valid value offered by the TP for Non-Firm Point-to-Point service.

10.1.65.  Requests for Redirects on a Non-Firm basis shall be submitted by the TC as pre-confirmed.

10.2.  The TC shall be allowed to request a Redirect on a Non-Firm basis for a portion or all of the Capacity Available to Redirect, even if the transmission scheduling rights on the Parent Reservation have been limited due to outages or other reliability-related events.  An example is shown in Appendix B.

10.3.  The TC shall be allowed to request a Redirect on a Non-Firm basis for a portion or all of the time period of the Parent Reservation (i.e., bound by the start/stop times of the Parent Reservation). An example is shown in Appendix B.

10.3.1.  A request for Redirect on a Non-firm basis must be submitted, and is subject to all request timing requirements consistent with reservations for Non-Firm Point-to-Point service of similar duration.

10.4. The TC’s rights on the Parent Reservation shall remain unaffected during the Transmission Provider evaluation of the request to Redirect on a Non-Firm basis.

10.4.1.  If the request to Redirect on a Non-Firm basis is denied for any reason, all rights and obligations shall remain per the Parent Reservation. An example is shown in Appendix B.

10.4.2.  The TC shall be allowed to submit and have pending multiple requests for Redirects on a Non-Firm basis against the same Capacity Available to Redirect.  The TP shall evaluate each such request with the knowledge that only those requests up to the Capacity Available to Redirect may ultimately be confirmed. An example is shown in Appendix B.

10.4.x.  If the TP determines that only a portion of the requested capacity can be accommodated, the TP is not obligated to extend to the TC that portion of the capacity (i.e., partial service) that can be accommodated.

10.5.  Upon confirmation of the request to Redirect on a Non-Firm basis, the Capacity Available to Redirect shall be reduced by the amount of the redirected capacity granted for the time period of that Redirect. An example is shown in Appendix B.

10.5.1.  The TC shall not confirm any request to Redirect on a Non-Firm basis that would exceed the Capacity Available to Redirect at that point in time (i.e., at the time of attempted confirmation and over the time interval of the Redirect).  The TP shall have the right to block any such confirmation.

10.5.2.  The TC shall should withdraw any request to Redirect on a Non-Firm basis that would exceed the Capacity Available to Redirect at that point in time (i.e., at the time of attempted confirmation and over the time interval of the Redirect). The TP shall have the right to withdraw their acceptance of any request to Redirect on a Non-Firm basis that cannot be confirmed due to limitations in the Capacity Available to Redirect by setting the OASIS standard STATUS data element to the value of SUPERSEDED.

10.5.3.   The TC shall have the right to request the TP to release unscheduled capacity associated with a confirmed request to Redirect on a Non-Firm basis and reinstate that capacity to the Parent (Firm) Reservation.  The TP shall honor all such valid requests, and reinstate the capacity on the Parent Reservation. such that it may subsquently be scheduled, Redirected on a Firm or Non-Firm basis to a different path, resold, etc.

10.6.  For the purposes of curtailment and other capacity reductions, confirmed Redirects on a Non-Firm (Secondary) basis shall have a lower priority than any be treated comparably to all other types of Non-Firm Secondary Point-to-Point Transmission Service.

10.6.1.  Curtailments or other capacity reductions to the remaining portion of the reserved capacity on the Parent Reservation shall not affect the Redirect reservation.

10.7.  Any differences in charges associated with a Redirect on a Non-Firm basis will be settled in accordance with the Transmission Provider’s tariff.

10.7.1.  Unless otherwise provided for in the TP’s tariff, there shall be no charge to Redirect on a Non-Firm basis.

10.8.  TPs shall have the right, but are in no means obligated, to accept requests for Redirect on a Non-Firm basis based on the submission of an Electronic Tag (ETAG e-Tag) using protocols compliant with Version 1.7.095 NERC Transaction Information System Working Group (TISWG) Electronic Tagging Functional Specification.

10.8.1.  The TC submitting a Redirect on a Non-Firm basis via a tag shall be subject to the same transaction timing requirements specified for submission of such requests directly on OASIS.

10.8.2.  A TP accepting Redirects on a Non-Firm basis via ETAG e-Tag shall allow a  TC to request redirected service for one or more path segments of the tag by designating:

(a) 1-NS as the transmission product code under the OASIS block,

(b) the OASIS reservation identifier of the Firm Parent Reservation to be redirected, and

(c) the secondary points of receipt and delivery being requested.

10.8.3.  A TP accepting Redirects on a Non-Firm basis via ETAG e-Tag shall determine the amount of the redirect request from:

(a) The amount of the TP Product.

(b) If the TP Product is not specified, the MW amount at the POR or POD for that TP in the Loss Table in accordance with the TP’s tariff

(c) , if neither TP Product amount nor Provider Loss Table amounts are specified, the MW amount in the Energy Profile.

10.8.4.  A TP accepting Redirects on a Non-Firm basis via ETAG e-Tag shall consider the ETAG e-Tag as a pre-confirmed Redirect request on a Non-Firm basis that is to be processed on a comparable basis with all such requests made directly on OASIS, with all obligations associated with such a request to be borne by the TC holding the Parent Reservation (e.g., any ancillary services, charges or credits for redirect, etc.), and subject to all other requirements of this Standard.

10.8.5.  The OASIS queue time of a Redirect requested via ETAG e-Tag shall be the TP’s ETAG e-Tag Approval Service receipt time, unless a system failure requires the use of backup procedures, in which case the OASIS queue time shall be the time the ETAG e-Tag is received by the TP.

10.8.6.  Once an ETAG e-Tag designating 1-NS service becomes iImplemented, the TP shall consider the associated Redirect request(s) to be confirmed.  Prior to or coincident with the tag becoming Implemented, the TP shall post the Redirect on OASIS.

10.8.x.  If the e-Tag is terminated prior to its original stop time, the TP shall consider this equivalent to a release request by the TC and resinstate capacity on the Parent Reservation.

This document does not contain changes recommended in R04006D.