# Coordinate Interchange

Purpose: Incorporate necessary revisions to the approved NAESB Coordinate Interchange Business Practice (R03013) to include added definitions, Industry transformation from Reliability Authority (RA) to the Reliability Coordinator (RC), and improvements to certain requirements of the Standard to ensure it is “lock-step” with the NERC Coordinate Interchange Standard.

Applicability: Balancing Authority (BA), Reliability Coordinator (RC), Interchange Authority (IA), Transmission Service Provider (TSP), Purchasing-Selling Entity (PSE), Generator-Provider Entity (GPE), Load-Serving Entity (LSE), and any Purchasing-Selling Entity (PSE) whose transmission approval rights are cited.

## Definitions

### Approval Entity – An entity that has approval rights for an Arranged Interchange; this includes the Transmission Service Providers (TSP), scheduling Balancing Authorities (BA), Generator-Providing Entity (GPE), and the Load-Serving Entity (LSE) that are included in the Arranged Interchange, as well as any Purchasing Selling Entity (PSE) whose transmission rights are cited in the Arranged Interchange.

### Arranged Interchange – The state where the Interchange Authority has received the Interchange information (initial or revised).

Balancing Authority  –The responsible entity that integrates resource plans ahead of time, maintains load-interchange-generation balance within a Balancing Authority Area, and supports Interconnection frequency in real time.

Balancing Authority Area – The collection of generation, transmission, and loads within the metered boundaries of the Balancing Authority. The Balancing Authority maintains load-resource balance within this area.

Confirmed Interchange – The state where the Interchange Authority has verified the Arranged Interchange

Curtailment - A reduction in the scheduled capacity or energy delivery of an Interchange transaction.

Generator-Providing Entity (GPE) – The Purchasing Selling Entity who is responsible for providing the source generation from owned, affiliated, or contractually bound generation.

Implemented Interchange – The state where the Balancing Authority enters the Confirmed Interchange into its Area Control Error equation.

### Interchange – Energy transfers that cross Balancing Authority boundaries.

### Interchange Authority, (IA) - The responsible entity that authorizes implementation of valid and balanced Interchange schedules between Balancing Authority Areas, and ensures communication of Interchange information for reliability assessment purposes.

### Interchange Block Accounting – Energy accounting that assumes a beginning and ending ramp time of zero minutes. For accounting purposes, this moves the energy associated with the starting and ending ramps into the adjacent starting and ending clock time of the Interchange.

### Interconnected Operations Service – A service (exclusive of basic energy and transmission services) that is required to support the reliable operation of the interconnected bulk electric systems.

### Load-Serving Entity, (LSE) –The responsible entity that secures energy and transmission service (and related Interconnected Operations Services) to serve the electrical demand and energy requirements of its end-use customers.

### Market Assembly – The function responsible for coordinating the submittal of a completed and accurate Arranged Interchange from the Requesting Purchasing-Selling Entity to the Interchange Authority within an organized Market.

## Market Period – The period of time beginning with the Requesting Purchasing-Selling Entity (PSE), or its designee, making required purchase, sale, and transmission service arrangements to support the Arranged Interchange through the period of time when the Interchange Authority receives the Arranged Interchange.

Purchasing-Selling Entity, (PSE) – The entity that purchases or sells, and takes title to, energy, capacity, and Interconnected Operations Services. Purchasing-Selling Entities may be affiliated or unaffiliated merchants and may or may not own generating facilities.

Reliability Coordinator - The entity with the highest level of authority that has responsibility for the reliable operation of the bulk electric system, has the wide area view of the bulk electric system, and has the operating tools, processes and procedures, including the authority to prevent or mitigate emergency operating situations in both next-day analysis and real-time operations. The Reliability Coordinator has the purview that is broad enough to enable the calculation of interconnection reliability operating limits, which may be based on the operating parameters of transmission systems beyond any transmission operator’s vision.

## Reliability Period – The period of time beginning with the Interchange Authority (IA) requesting approvals from the reliability Approval Entities through the completion of the physical flow of the energy associated with the originally submitted Arranged Interchange.

### Request For Interchange, (RFI)- A collection of required data, as defined in Appendix C of this standard, necessary for the purpose of submitting to the Interchange Authority as an Arranged Interchange.

### Requesting Purchasing-Selling Entity, (PSE) – The entity that prepares and submits the Request For Interchange (RFI) to the Interchange Authority (IA) and holds the transmission reservation being presented for use.

Sink Balancing Authority  – The Balancing Authority in which the load (sink) is located for an Interchange Transaction.

Source Balancing Authority – The Balancing Authority in which the generation (source) is located for an Interchange Transaction.

## Business Practice Requirements

### All requests to implement bilateral Interchange (excluding Interchange for emergency energy) between a Source Balancing Authority and a Sink Balancing Authority, where a Balancing Authority is located in either the Eastern or Western Interconnection, shall be accomplished by the submission of a completed and accurate Request For Interchange (RFI) to the Interchange Authority (IA).

1.1 To the extent that Intra BA transactions are submitted as a Request for Interchange (RFI), those transactions will be subject to all provisions of this standard.

#### Until other means are adopted by NAESB, the primary method of submitting the Request For Interchange (RFI) to the Interchange Authority shall be electronic (E-Tag) and in accordance with the Version 1.7.095 NERC Transaction Information Systems Working Group (TISWG) Electronic Tagging Functional Specification .

#### A backup or redundant electronic system shall be available for immediate use should the primary electronic means become disabled.

#### Submitting Request for Interchange (RFI) to the Interchange Authority via facsimile is acceptable only as a last resort when the electronic means and its required backup or redundant system are not available.

### Arranged Interchange that crosses Interconnection Boundaries (Eastern, Western, or ERCOT) shall be subject to the submittal and approval timing requirements associated with the most restrictive interconnection involved in the Interchange.

### For Interchange where the sink is in the Western Interconnection for same day transactions, the last Purchasing-Selling Entity before the DC Tie in the Eastern Interconnection shall be responsible for submitting the E-Tag.

### In the event of E-Tag system component failure, the requirements and procedures contained within Appendix A “Electronic Tagging Service Performance Requirements and Failure Procedures” shall be followed.

### It shall be the responsibility of the load serving Purchasing-Selling Entity (PSE), or its designee, to ensure the completed and accurate Request For Interchange (RFI) contains, at a minimum, the information specified in Appendix C “Required and Correctable Tag Data”.

### Approval Entities shall only be allowed to take actions against an Arranged Interchange as specified in Appendix B “Interchange Transaction Tag Actions”.

6.1. Prior to the expiration of the market assessment period defined in the Appendix D, “Timing Table”, Column B, the PSE (whose transmission rights are cited), LSE, and GPE shall respond to a request from the Interchange Authority to transition an Arranged Interchange to a Confirmed Interchange.

    - Each PSE (whose transmission rights are cited), LSE, and GPE shall assess the Arranged Interchange for completeness and accuracy of the information contained in the Arranged Interchange.

    - If the PSE, LSE, and GPE do not respond to a request from the Interchange Authority, the Interchange is considered passively approved.

### All information on energy purchase, energy sale, and transmission service arrangements required for the RFI shall be performed prior to being submitted to the IA.

#### At its discretion, the Requesting Purchasing-Selling Entity may defer this responsibility to the Market Assembly function.

#### Eastern and Western Interconnection Timing Requirements:

The completed and accurate Request for Interchange (RFI), or modification to an Arranged Interchange submitted to the Interchange Authority shall be subject to the timing requirements contained in this standard under Appendix D “Timing Table.”

#### 8.2 Transactions that cross Interconnection boundaries involving ERCOT shall follow the timing requirements contained within this standard.

#### All denials of Arranged Interchange by an Approval Entity during the assessment period (reliability and market) shall be accompanied by the reason for such denial and communicated to the Interchange Authority and by the Interchange Authority to the Requesting Purchasing-Selling Entity.

#### Any changes to the status of the Arranged, Confirmed, or Implemented Interchange shall be communicated by the Interchange Authority to all involved parties of the Interchange, such as Balancing Authorities, Reliability Coordinators, Generator-Providing Entity, Load-Serving Entity, and Transmission Service Providers.

#### The Requesting PSE shall have the right to request modifications to the Arranged, Confirmed or Implemented Interchange for non-reliability related issues according to the timing requirements in Appendix D set forth in this Standard. For Implemented Interchange, only “future” hours may be modified.

#### The Requesting PSE can request to increase or decrease the energy level or the committed transmission(s) profile of an Arranged, Confirmed or Implemented Interchange. For Implemented Interchange, only “future” hours may be modified.

#### a) In the case of an increase in the energy level, the Requesting PSE must provide and modify the necessary transmission capacity to cover the increased energy flow.

#### The Requesting PSE shall have the right to request an extension to the Arranged, Confirmed or Implemented Interchange energy profile to reflect a desire to flow energy during hours not previously specified.

#### The Requesting PSE must provide the necessary transmission capacity with the extension.

#### If the modification is denied by any Approval Entity, the previous confirmed or Implemented Interchange remains valid, including the duration period.

#### All parties involved in the Arranged Interchange shall have, or arrange to have, personnel and facilities on site and immediately available for notification of changes to the Arranged Interchange from the beginning of the Market Period through the time when the energy flow of the Implemented Interchange has been completed.

#### Unless provided for under a FERC approved market mechanism, energy accounting for all Interchange shall be accomplished via Interchange Block Accounting.

#### Settlement of losses shall be either handled as financial or as payment in-kind in accordance with the Transmission Service Provider tariff.

#### For losses handled as payment in-kind, the PSE, or its designee, shall communicate to the IA, via Arranged Interchange the amount of MW losses and the entity that should receive them along the Interchange path.

#### All Reliability Coordinators, Balancing Authorities, Transmission Service Providers, Generator-Providing Entity, Load-Serving Entity, and other entities involved in an Interchange request shall not disclose the Interchange Transaction information to any PSE not involved in the Interchange transaction.

#### After a curtailment of Interchange has ended, the Sink Balancing Authority shall return the Interchange profile to the previous level, unless otherwise specified by the entity submitting the Request For Interchange.

#### Default ramp duration for the North American Interconnection shall be as follows:

#### Default ramp duration for the Eastern Interconnection shall be 10 minutes equally across the start and end times of the Implemented Interchange (i.e., 5 minutes before start and 5 minutes after the end time of the implemented Interchange) unless otherwise agreed to by all parties involved in the Implemented Interchange.

#### Default ramp duration for the Western Interconnection shall be 20 minutes equally across the top of the hour (i.e., 10 minutes before start and 10 minutes after the end time of the Implemented Interchange) of the Implemented Interchange unless otherwise agreed to by all parties involved in the Implemented Interchange.

Appendix A

Electronic Tagging Service Performance Requirements and Failure Procedures

This document describes the performance requirements of the E-Tag System and the procedures to be followed in the event of an E-Tag System component’s failure.  Due to the importance of accurate information flow, these procedures and requirements have been developed to ensure that reliable data communications remain available at all times.

A.	Performance Requirements

## Tag Agent Service Requirements

Entities that are required to use E-Tag agent services are responsible for obtaining an E-Tag agent service in order to conduct business; there are no exemptions to this requirement.  There is no specific requirement against which performance should be measured. However, in cases of E-Tag agent service failure, non-receipt of critical information (such as curtailment notifications, transaction denials, and schedule modifications) due to performance problems shall be the responsibility of the E-Tag agent user.

While it is acceptable for an entity to contract with a third-party to provide for this requirement, it should be understood that the E-Tag agent user is ultimately responsible for the provision of the service. The non-performance of a third party does not excuse the entity from the obligation to provide the service.

## E-Tag Approval Services

Entities that are required to employ E-Tag approval services are responsible for providing an E-Tag approval service as well as providing a level of redundancy; there are no exemptions from this requirement.  At a minimum, E-Tag approval services may not have greater than 1.0% of the E-Tags sent to their system within a calendar month be determined by the Interchange Authority Service as having a state of “COMM_FAIL.”  While there is no specific level of redundancy that is required by this Appendix, sufficient redundancy must be in place that the entity is confident of achieving this standard.

While it is acceptable for an entity to contract with a third-party to provide for this requirement, it should be understood that the entity required to employ the E-Tag approval service is ultimately responsible for the provision of the service.  The non-performance of a third party does not excuse the entity from the obligation to provide the service.

In order to monitor compliance with this requirement, the Balancing Authorities will arrange with their Authority Services to generate compliance reports at the beginning of each month determining this metric for the previous month on a Provider-by-Provider basis.  These results should be available for investigation of any violations and the results of this investigation may be posted once finalized.

## Interchange Authority (IA) Services

As the Interchange Authority service is the most critical element of the E-Tag system, it must meet much higher standards.  These standards can be divided into two areas: Implementation, and Policies and Performance.

## Implementation

Interchange Authority services must be implemented in a manner that provides for redundancy and fault-tolerance through hardware and software; there are no exemptions to this requirement. Specifically, an Interchange Authority service must provide, at a minimum, the following:

- Two or more connections to the Internet, which may either be available concurrently or be switch able on demand (within five minutes);

- Redundant/Fault-Tolerant Networking Equipment between the Internet providers’ demarcation points and the computer systems, as well as between each of the components of the system required to be inter-networked to provide functionality (i.e., FDDI Rings, dual homing, etc…);

- Redundant/Fault-Tolerant computer systems that can immediately recover from a loss of any single component (i.e., mirrored databases, web clusters, etc…).

- Providers of Interchange Authority services may be required to provide documented explanations of how they meet or exceed the above requirements.  These documents may be evaluated for fitness and will be held in confidence.

## Policies and Performance

- The following shall be required of all Interchange Authority services:

- All scheduled outages must be performed between the hours of 01:00 CST and 04:00 CST.  Any maintenance that must be performed outside this three hour window must be accomplished though the use of redundant systems in such a manner that no outage is visible;

- Notice of Scheduled outages must be given to the public at least 24 hours before the outage is to occur.  Notice shall be deemed valid if the following actions have been taken:

- Users of the system are sent notifications, via email or a proprietary system, time stamped at least 24 hours prior to the outage;

- The TISFORUM mailing list is sent Email notification time stamped at least 24 hours prior to the outage;

- The OASIS TSIN mailing list is sent email notification time stamped at least 24 hours prior to the outage.

Any system problem that creates behavior contrary to that described in the E-Tag specification shall constitute an “unscheduled outage.”  For example, a system that begins rejecting every third message it receives due to a component failure in a cluster would constitute an unscheduled outage (although the system was only failing one third of the time, it was not performing as described in the E-Tag specification).

Interchange Authority services may not be in a state of scheduled or unscheduled outage for more than 0.5% of the time for the month, based on outage time (in minutes) for the month divided by total time in the month (in minutes).  Specific allowed outages may be granted to address special circumstances (i.e., scheduled specification changes, major internet outages, etc…).  These specific allowed outages, if granted, may require public posting for all customers to view.

While it is acceptable for an entity to contract with a third-party to provide for these requirements, it should be understood that the entity required to employ the Interchange Authority service is ultimately responsible for the provision of the service.  The non-performance of a third party does not excuse the entity from the obligation to provide the service.

To monitor compliance with these requirements, the Operator of an Interchange Authority system may be required to submit, at the beginning of each month, a report describing outage activity for the previous month.    This report shall consist of the following items:

- The beginning of the outage;

- The ending of the outage;

- The type of outage (Scheduled or Unscheduled);

- The nature of the outage (Maintenance, System Crash, etc…);

- In the event of an Unscheduled Outage, the cause of the outage and the steps taken to ensure the problem has been addressed and will not reoccur.

The report format may be in a standardized electronic form.  These documents may be evaluated and held in confidence.  Statistics may be developed from these reports identifying system outage durations for each month.  These preliminary findings will be held in confidence until they are confirmed. These performance percentages shall be posted and electronically accessible once confirmed, at the end of the month following the month evaluated.

Entities experiencing difficulty due to an unnoticed scheduled or unscheduled outage may send a Request for Investigation.  This request should specify the estimated time the outage occurred, the estimated time the outage ended, and document evidence of the outage (such as TMP logs, email messages, etc…).  Claims may be investigated with the appropriate Tag Authority Service Operator.  Should a Tag Authority Service Operator be unable to refute the claim, and the Investigation Requestor appears to have provided an accurate representation of an undocumented outage, calculated outage percentages may be modified to include the undocumented incident.

B.	Failure Procedures

Backup procedures are needed because, in a communication system that operates on the public Internet, failures are certain to occur.  The failures may be caused as a result of overload of the network, loss of connection to an Internet service provider, corruption of one or more servers by computer hackers, failure of one or more entity’s Internet servers, internal firewall failure, and many other reasons.

Failures also have a wide variety of scopes.  A failure may affect a single entity with a small number of schedules while all of its neighbors continue to operate normally, a small number of utilities in a local area, or a regional RTO with thousands of active schedules.  However failures occur, the operation of the electric utility grid must continue.  This document describes the manner in which operations are to be coordinated should such a failure become a reality.

## Assumptions

A general assumption is that each operational entity in the electric utility industry has an internal energy management system, marketing system, or contract system that will not be affected by the Internet communication failure.

## Participating Entities

Requesting PSE- The entity that prepares and submits an RFI, normally a Purchasing Selling Entity

Path Participant – Any of the entities that are part of an Interchange transaction.

Interchange Authority Service Entity – The entity that provides the Interchange Authority service for an E-Tag.  The Interchange Authority service itself is typically a computer system that maintains the master database for the tag and communicates status with other computer systems.  The Interchange Authority Service Entity is the utility industry entity that is responsible for providing the service.  In E-Tag 1.7.095, this entity is the Sink Balancing Authority.

Approval Entity – An entity that has approval rights for Arranged Interchange; this includes the Transmission Service Providers (TSP), scheduling Balancing Authorities (BA), Generator-Providing Entity (GPE) and the Load-Serving Entity (LSE) involved in the Arranged Interchange

Checkout Partners – The entities that perform the checkout process.  Most commonly two adjacent Balancing Authorities checking net interchange.  It might also be two marketers checking sales and purchases, or a transmission customer checking schedules with a transmission provider.

## Failure Actions

When a failure occurs an entity will soon realize that it has lost communications with the other servers in the electronic tagging arena.  Yet it must still communicate current energy flows across the transmission network and expected flows for the next few hours.  Transmission curtailments must be accounted for in the sense that a required reduction in energy flows or increase in generation needs to be communicated.  However, accounting issues will take a secondary priority to reliability issues in this exchange, and detail relating back to tags, schedules, and transmission reservations can be reconstructed later.

The table below lists typical failures that might occur and the emergency actions that the entity will take to compensate for that failure.

| Entity | Connectivity Problem | Backup actions |
| --- | --- | --- |
| Requesting PSE | Unable to submit tag to Interchange Authority Service. | Ask another entity in the transaction chain to submit the schedule for you. He then becomes the author. Create a backup paper copy of the schedule and fax to authority service entity and all approval entities in the transaction. |
| Path Participant | Not receiving update messages. | Use Recovery Process to resynchronize from authority service. Use telephone with Authority Service Entity to update status. |
| Interchange Authority Service Entity | Unable to send messages to generation or load Balancing Authorities. | Telephone schedule author to notify of the message failure. The author will fax the schedule to the Approval Entity for these control areas. Telephone Approval Entity to notify of the message failure. Approve or deny the schedule at the request of the Approval Entity (override). |
| Interchange Authority Service Entity | Unable to send messages to an Approval Entity for an intermediate Transmission Provider or Balancing Area. | Telephone schedule author to notify of the message failure. The author will fax the schedule to the Approval Entity. Telephone Approval Entity to notify of the message failure. Approve the schedule automatically. Deny the schedule at the request of the Approval Entity (override). |
| Interchange Authority Service Entity | Unable to send messages to an information only entity. | No Action required. |
| Interchange Authority Service Entity | Unable to receive messages. | Broadcast a message by email or fax to all entities that use your authority service. The message should forecast a recovery time for your service. In the meantime, your Interchange Authority Service is down. |
| Approval Entity | Unable to receive messages from an authority service. (The Authority has an obligation to notify you and the authoring PSE. The Authoring PSE has an obligation to fax the tag to the approver.) | Use the Recovery Process to resynchronize from Authority Services or Central Repository. Telephone the Authority Service entity with the approval or denial of the schedule. |
| Approval Entity | Unable to send messages to an authority service. | Telephone the Interchange Authority Service Entity with approval or denial of the schedule. |
| Checkout Partner | Unable to exchange messages. | Telephone net exchange to the checkout partner. Create a backup paper copy of the checkout data and fax to the checkout partner. |

Notes:

- The first action in every case is to attempt to establish connection by using an alternate communication method, a second Internet service provider, dial up connection, or a private network if one is available.

- Next, the backup actions are attempted in the order specified.

- The backup actions include printing paper reports from the internal energy management system.  The reports include a schedule detail report for a short time period, net exchange between two operational entities, and transmission reservation usage between a transmission provider and a customer.

- Every backup action list ends with a fax or telephone call that is completely independent of the public Internet.

## Reports

Three reports have been designed to communicate energy flows and transmission reservation usage between partner entities with a tie where possible back to the schedules as known before the communication failure.

## Net Exchange

A Net Exchange report is a paper summary of Interchange:

- The time span of the report will cover a period of the current hour to a few hours in the future, up to 24 hours.

- The entity and the partner entity are any two entities that share common schedules.

- The date and time are the date and time of the report.

- Net schedules are the net of schedules from and to the other entity.

- TO is a sum of the schedules from the entity to the partner entity.

- FROM is a sum of the schedules from the partner entity to the entity.

- Tag or fragment lines represent the data from each tag or fragment that was known at the time of the failure or has been entered later.

- Recent adjustment lines represent a summary of changes to the schedules that occurred since the failure.

## Schedule Detail

- A Schedule Detail report is a paper copy of an individual schedule.  It includes:

- The schedule identification number and most current active revision number.

- The fully expanded energy schedule for a period of the current hour to a few hours in the future, up to 24 hours.

- The complete path with all OASIS and contract references.

## Reservation Usage

- A transmission Reservation Usage report is a summary of Reservation Usage:

- The time span of the report will cover a period of the current hour to a few hours in the future, up to 24 hours.

- The entities on the report are a transmission provider and a transmission contract holder.

- Gross reservations is the sum of reservations, Usage is the sum of usage.

- The detail lines are tag or fragment usage of reservation, organized by product and OASIS reservation number.

## Recovery Process

The last backup issue is the recovery of current status when the communication link is reestablished.  The recovery is accomplished by a query request to the Interchange Authority service for each entity that the entity does business with.  The query returns a list of all the schedules that reference that entity with the schedule ID, the current version number and the last modified date and time.

The recovering entity then compares with its own database and updates his database to be current with the Interchange Authority’s database.  When all Interchange Authority services have been queried, the recovery is complete.

If the entity desires, it can request a complete audit history of each schedule.

Appendix B

Transaction E-Tag Actions

For Eastern and Western Interconnections

The table below explains the various E-tag actions that are possible, and the entities that are entitled to initiate these actions:

| Desired Policy Action | Reason | Tagging Action | Initiated by | Result |
| --- | --- | --- | --- | --- |
| Approve a Tag Request | Economic, Reliability, or Contractual | Set Status (to Approved) | Approval Entity* | Approver indicates approval |
| Deny a Tag Request | Economic, Reliability, or Contractual | Set Status (to Denied) | Approval Entity* | Approval indicates denial |
| Study a Tag Request | Economic, Reliability, or Contractual | Set Status (to Studied) | Approval Entity* | Approval indicates the tag has been viewed, but have not committed to a decision |
| Withdraw a Tag Request | Economic | Withdraw Request prior to request implementation | Requesting PSE** | Request is dead |
| Cancel a New Tag | Economic | Request Profile Change – Set Energy and Capacity for the transaction to zero prior to transaction start | Requesting PSE** | Tag is dead |
| Terminate a Tag | Economic | Request Profile Change – Set Energy and capacity of the transaction to zero from a point of time forward | Requesting PSE** | Portion of tag is dead |
| Extend an E-Tag | Economic | Request Profile Change – Append additional hours onto an existing transaction | Requesting PSE** | Tag is extended |
| Reduce an E-Tag | Economic | Request Profile Change – Decrease Energy flow or Committed Transmission Reservation(s) for a transaction for a specific set of hours | Requesting PSE**, Market Operator*** | Profile is Decreased |
| Increase an E- Tag | Economic | Request Profile Change – Increase Energy flow or Committed Transmission Reservation(s) for a transaction for a specific set of hours | Requesting PSE**, Market Operator*** | Profile is Increased |
| Curtail an E-Tag | Reliability (SOL Violation, Loss of Gen, loss of Load) | Request Profile Change – Limit Energy flow for a transaction for a specific set of hours | Source BA, Sink BA, Transmission Service Provider, Reliability Coordinator | Profile is Decreased |
| Reload an E-Tag | SOL Violation eliminated, Generator Returned, Load Returned | Request Profile Change – Release Limit of Energy flow for a transaction for a specific set of hours | Source BA, Sink BA, Transmission Service Provider, Reliability Coordinator | Profile is Increased |

Notes:

*Purchasing-Selling Entities and Load-Serving Entities may elect to defer their approval rights to the Host Balancing Authority of their facilities.  For more information, see PSE and LSE approval rights below

**In some situations, Balancing Authorities implement certain Interchange Transactions or Interchange Schedules, such as bilateral inadvertent payback, Dynamic Schedules, and emergency schedules from Reserve Sharing Groups. In these situations, the Balancing Authority serves as the Purchasing-Selling Entity and can perform these actions.

***Entities registered as market operators and serving as either source or sink for a Transaction may exercise such functions in order to indicate correct flow based on market clearing.

PSE and LSE Approval Rights

Purchasing-Selling Entities providing generation (GPE) and Load-Serving Entities (LSE) have been granted the right, but not the obligation, to approve Transaction requests using their resources. Only GPE’s who are capable of controlling the output of the source generator should be allowed approval rights. If PSEs and LSEs specify an approval service in the Master Registry, then they are expected to approve/deny Transactions when so requested. Otherwise, their Host Balancing Authority is expected to act on their behalf. .The following table illustrates the proper way to interpret this requirement:

| If the PSE… | Specified an Approval URL | The PSE should be granted rights to approve or deny |
| --- | --- | --- |
|  | Did not specify an Approval URL | The BA should have proxy approval rights for the PSE |

Appendix C

Required and Correctable E-Tag Data

# A.	New Interchange Transactions

A new Interchange transaction is an Arranged Interchange (E-Tag) that has not yet been approved or confirmed for implementation.  Such Arranged Interchange must be presented to those Approval Entities (as E-Tag) that are responsible for the implementation of the Interchange transaction in order that they may evaluate the E-Tag and determine whether or not the Interchange can be implemented.  The following information is to be used to describe such an Interchange transaction.

- Market Information

  - Market Redispatch Information (only required if E-Tag is MRD Transaction).  (See “E-Tag Functional Specification Version 1.7095”)

  - Financial Path (Required)  the description of financially responsible parties for the Interchange in order.  This will typically start with a Purchasing-Selling Entity providing generation (GPE) and finish with a Load Serving Entity (LSE), and where applicable, intermediate Purchasing-Selling Entities between the two.

      - Energy Title Holder(s) (Required) – the identity of the entities financially responsible to take and/or deliver the energy as described in the physical path.  This will typically be a Purchasing-Selling Entity providing generation, a Load Serving Entity, and where applicable, Intermediate Purchasing-Selling Entities.

          - Energy Product Type (Correctable) – the type of energy delivered by the Energy Title Holder.

          - Contract Number(s) (Correctable) – reference to a Transaction entered into by the Energy Title Holder with one or more other participants in the Transaction.

          - Miscellaneous Information (Correctable) – information provided at the Requesting PSE’s option regarding the Transaction.

- Physical Information

  - Physical Path (Required) – the description of physically scheduling parties for the Interchange in order and related to the financially responsible parties described above.  This will always contain a Generation segment, at least one Transmission segment, and a Load segment.

      - Generation (Required) – set of data describing the physical and contractual characteristics of the energy source.

          - Source (Required) – the physical point at which the energy is being generated.  This may vary in granularity, dependent on local business practices.

          - Contract Number(s) (Correctable)  reference to a schedule or agreement entered into by the Purchasing-Selling Entity providing generation and the Generator Operator.

          - Miscellaneous Information (Correctable) – information provided at the Requesting PSE’s option regarding the Interchange Transaction.

          - Energy Profile (Required) – energy to be produced by the Generator Operator for this Transaction.

      - Transmission (Required) – set of data describing the physical and contractual characteristics of a wheel (import, export, or through).

          - Transmission Service Provider (Required) – the identity of the transmission provider that is wheeling the energy.

          - Point of Receipt (Correctable) – valid Point of Receipt for scheduled Transmission Reservation.

            - Point of Delivery (Correctable) – valid Point of Delivery for scheduled Transmission Reservation.

            - Scheduling Entities (Correctable) – entity that is physically scheduling interchange on behalf of the Transmission Service Provider in order to provide wheeling services.  Typically this is the Balancing Authority for the Transmission Service Provider, but may be several Balancing Authorities supporting a regional transmission service.

            - Loss Provision Information (Required) (Correctable)– Information describing the manner in which losses are accounted when they are not scheduled as in-kind megawatt distributions through the original transaction.  Types may be financial (paid in dollars based on tariff provisions), internal (scheduled in megawatts to the Transmission Service Provider from a resource inside the Transmission Service Provider’s area), or external (scheduled in megawatts to the Transmission Service Provider from a resource outside the Transmission Provider’s area). If internal or external, must specify contract numbers or Transaction IDs.

            - Miscellaneous Information (Correctable) – information provided at the Requesting PSE’s option regarding the transaction.

            - POR and POD Profiles (Required) – schedule of Energy Flow imported at the Point of Receipt and exported at the Point of Delivery.

            - Transmission Reservation Number(s) (Required) (Correctable) – reference to a particular transmission reservation being used to provide transmission capacity to support the transaction being described.

              - Transmission Product (Required) (Correctable) – Specifies the firmness of service associated with the transmission reservation being used.

              - Requesting PSE (Required) (Correctable) – identifies the entity that purchased and holds the transmission reservation being presented for use.

              - Transmission Reservation Profile (Required) - information describing the transmission reservation commitment associated with the Transmission Service Provider.

                - Committed Transmission Reservation Level (Required) – schedule of transmission reservation committed by the Requesting Purchasing-Selling Entity for use for this Transaction.

      - Load (Required) – set of data describing the physical and contractual characteristics of the energy sink.

          - Sink (Required) – the physical point at which the energy is being consumed.  This may vary in granularity, dependent on local business practices.

          - Contract Number(s) (Correctable)  reference to a schedule or agreement entered into by the Load Serving Entity and the distribution provider.

          - Miscellaneous Information (Correctable) – information provided at the requesting PSE’s option regarding the Transaction.

          - Energy Profile (Required) – energy to be consumed by the load for this Transaction.

Using Multiple Transmission Reservations to Support a Single Leg of an Interchange Transaction

The use of multiple transmission reservations to support a single leg of an Interchange Transaction is known as transmission stacking.  There are two types of transmission stacking:

- Vertical stacking, in which a Requesting Purchasing-Selling Entity combines multiple reservations to achieve a certain net level of transmission capacity, and

- Horizontal stacking, in which a Requesting Purchasing-Selling Entity combines multiple reservations to achieve a certain transmission capacity coverage over time.

The following diagrams illustrate these concepts more fully.  In both cases, the assumed need is 100 MW of transmission capacity for hours 06:00 through 22:00.

Should a Requesting PSE elect to utilize stacking, including any combination of the two stacking types, to support their Interchange Transaction, they must understand the following requirements:

- Stacks MUST be described through fully qualified profiles for each reservation being used

- At no point may the coverage described by the stack be less than the transmission capacity needed for the Transaction’s energy flow

# B.	Curtailments and Reloads (Reliability Related Profile Modifications)

Curtailments and Reloads are special kinds of modifications to an Interchange transaction’s energy profile based on reliability concerns.  Such modifications must be presented to those entities that are responsible for the implementation of the modification in order that they may evaluate the transaction request and determine whether or not the modification can be implemented.  The following information must be used to describe such a modification.

- The Transaction being curtailed or reloaded

- All necessary profile changes to set the maximum flow allowed for the transaction during the appropriate hours

- A contact person that initiated the curtailment or reload, and

- A description of the necessity for the schedule change.

# C.	Market-Related Profile Modifications

Profile Modifications are changes to an Interchange Transaction’s energy profile based on market desires.  Such modifications must be presented to those entities that are responsible for the implementation of the modification in order that they may evaluate the Interchange Transaction request and determine whether or not the modification can be implemented.  The following information must be used to describe such a modification.

  - The Interchange Transaction being modified

  - All necessary profile changes to set the transmission capacity or energy flow to the desired levels during the appropriate hours, and

  - A contact person that initiated the modification

Appendix D

Commercial Timing Table

Timing Table

Interchange Timeline with Minimum Reliability-Related Response Times

Ramp Start

Request for Interchange Submitted

|  | A | B | C | D |  |
| --- | --- | --- | --- | --- | --- |
| If Actual Arranged Interchange (RFI) is Submitted | IA Makes Initial Distribution of Arranged Interchange | GPE, LSE, and PSE Conduct Market Assessments IA Verifies Reliability Data Complete | IA Compiles and Distributes Status | BA Prepares Confirmed Interchange for Implementation | Minimum Total Reliability Period (Columns A through D) |
| <1 hour prior to ramp start | < 1 minute from RFI submission | < 10 minutes from Arranged Interchange receipt from IA for all Interconnections except WECC | < 1 minute from receipt of all Reliability Assessments | > 3 minutes prior to ramp start | 15 minutes |
| <1 hour prior to ramp start | < 1 minute from RFI submission | < 5 minutes from Arranged Interchange receipt from IA for WECC | < 1 minute from receipt of all Reliability Assessments | > 3 minutes prior to ramp start | 10 minutes |
| >1 hour to < 4 hours prior to ramp start | < 1 minute from RFI submission | < 20 minutes from Arranged Interchange receipt from IA | < 1 minute from receipt of all Reliability Assessments | > 39 minutes prior to ramp start | 1 hour plus 1 minute |
| > 4 hours prior to ramp start | < 1 minute from RFI submission | < 2 hours from Arranged Interchange receipt from IA | < 1 minute from receipt of all Reliability Assessments | > 1 hour 58 minutes prior to ramp start | 4 hours |

This PSE reference applies to PSE’s whose transmission rights are cited on Arranged Interchange.

These Market Assessments take place in concurrence with NERC Reliability Assessments (as found in NERC INT-005-1).