1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

X  Accept as requested			   x Change to Existing Practice

Accept as modified below			      Status Quo

Decline

2.  TYPE OF DEVELOPMENT/MAINTENANCE

Per Request:					Per Recommendation:

Initiation					      Initiation

x   Modification					 x   Modification

Interpretation				      Interpretation

Withdrawal					      Withdrawal

Principle 					      Principle

Definition 					      Definition

x Business Practice Standard 			  x  Business Practice Standard

Document 					      Document

Data Element 				      Data Element

Code Value 					      Code Value

X12 Implementation Guide			     X12 Implementation Guide

Business Process Documentation		      Business Process Documentation

3.  RECOMMENDATION

SUMMARY:	This recommendation proposes modifications to the NAESB Version 0 TLR business practices to remain consistent with the NERC Version 0 TLR reliability standards.  NERC undertook an urgent action to modify its version 0 TLR reliability standards, and as such, NAESB must consider the same modifications to keep the TLR standards identicial until the business practices have been seperated from the reliaiblity standards in the Amended Version 0 TLR standrads.

This activity is not considered an Amended Version 0 effort.  Those efforts are progressing seperately.

The purpose of the revisions is to correct a conflict between INT-004 Requirement 5 and IRO-006 Attachment 1 concerning Dynamic Schedules.  Currently the IDC does not allow entities to comply with INT-004 Requirement 5 during TLR conditions due to a software problem. At the December 2004 joint NERC Operating Committee subcommittee meeting, the subcommittee approved the following motion to correct the conflict: "If the E-Tag is identified as the type “Dynamic,” and the transmission service is considered firm according to the constrained path method, then it will not be held by the IDC during TLR level 4 or lower." The joint ORS/IS met in February 2005 and approved a recommendation to revise IRO-006 and requested an urgent action SAR be drafted. This change needs to be completed as an Urgent Action SAR because currently when a Dynamic Schedule is outside the band mentioned in INT-004 R.5.1 or 5.2 and the PSE attempts to comply with requirement 5 of INT-004 the IDC holds the transaction. The current software treats adjusted firm dynamic transactions that are already flowing as new transactions and holds them. This means PSE can not update the IDC during TLR level 3 greater conditions for a transaction that is already flowing. Because the IDC places a hold on the flowing PSE adjusted schedule the IDC is not being updated with the actual value of flowing Dynamic Schedules. This is not the intent of the reallocation process or the expected operation of the IDC so an Urgent Action on this SAR is requested to ensure the quickest resolution to this reliability issue with both the tool and the standards conflict. IRO-006 is requested to be changed to clarify the intent of the TLR procedure regarding this matter. The ORS, IDWG and IS agree with the requested modification to IRO-006 and the IS request this be handled as an Urgent Action SAR. The Interchange Subcommittee is drafting this Urgent Action SAR in order to make the changes to the Standard in coordination with the software changes which can be implemented by June 2005. This software change will allow for compliance with INT-004 and we are requesting additional clarifying language to be added to the TLR procedure so it is understood that the IDC will not hold Dynamic Schedule transactions that are being modified in accordance with INT-004.

Recommended Standards:

The proposed revision to WEQBPS-006-000 “Transmission Loading Relief – Eastern Interconnection” to confrom with the proposed revisions to standard IRO-006-0 “Transmission Loading Relief Procedure – Eastern Interconnection” is indicated below:

1.6.6. Reallocation. The Reliability Coordinator shall consider for Reallocation

any Transactions of higher priority that meet the approved tag submission

deadline during a TLR Level 3A. The Reliability Coordinator shall

consider for Reallocation any Transaction using Firm Transmission

Service that has met the approved tag submission deadline during a TLR

Level 5A. Note Reallocations for Dynamic Schedules are as follows: If

an Interchange Transaction is identified as a Dynamic Schedule and the

transmission service is considered firm according to the constrained path

method, then it will not be held by the IDC during TLR level 4 or lower.

Adjustments to Dynamic Schedules in accordance with INT-004 R5 will

not be held under TLR level 4 or lower.

NERC Implementation Plan:

The proposed effective date is immediately upon adoption by the Board of Trustees. The

proposed revision will be presented for ballot of stakeholders beginning May 23. If approved by

stakeholders, the revision will be forwarded to the Board of Trustees for adoption on

approximately June 22.

4.  SUPPORTING DOCUMENTATION

a.  Description of Request:

R05009: Modify the NAESB Version 0 TLR business practices to remain consistent with NERC Version 0 TLR reliability standards.

b.  Description of Recommendation:

Please see the following set of WEQ TLR draft minutes for the supporting documentation and discussion.

April 28-29, 2005 		http://www.naesb.org/pdf2/weq_tlr042805dm.doc

c.  Business Purpose:

d.  Commentary/Rationale of Subcommittee(s)/Task Force(s):