| 003-0 OASIS DATA DICTIONARY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Data Dictionary Element Name | Alias | Field Format : minimum characters {type of ASCII} maximum characters | Restricted Values | Definition of Data Element |
| REQUEST_TYPE | REQTYPE | 1{ALPHA}30 | Valid Types: ORIGINAL RESALE RENEWAL MATCHING DEFERRAL REDIRECT RELINQUISH {Registered} | ORIGINAL = typical reservation requests submitted to the Primary Provider (as the Seller of the transmission or ancillary service). RESALE = secondary market requests submitted to a Transmission Customer as Secondary Provider. RENEWAL = request to renew an expiring transmission reservation. MATCHING = request to meet or exceed a competing request to retain transmission service (right of first refusal). DEFERRAL = request to defer or apply for extension on start of transmission service. REDIRECT = request to redirect all or portion of a transmission reservation to an alternate POR/POD and/or make other changes to the terms of service as permitted. RELINQUISH = request to release all or a portion of the capacity of a Redirect on a Non-Firm basis to the Firm Parent Reservation. {registered} = Primary Provider's may register values for REQUEST_TYPE to implement specific provisions of their Tariffs. |
| STATUS | STATUS | 5{ALPHANUMERIC}25 | Valid field (QUEUED, INVALID, RECEIVED, STUDY, REBID, COUNTEROFFER, DECLINED, SUPERSEDED, ACCEPTED, REFUSED, CONFIRMED, WITHDRAWN, DISPLACED, ANNULLED, RETRACTED) | QUEUED = initial status assigned by TSIP on receipt of "customer services purchase request". INVALID = assigned by TSIP or Primary Provider indicating an invalid field in the request, such as improper POR, POD, source, sink, etc. (Final state). RECEIVED= assigned by Primary Provider or Seller to acknowledge QUEUED requests and indicate the service request is being evaluated, including for completing the required ancillary services. STUDY= assigned by Primary Provider or Seller to indicate some level of study is required or being performed to evaluate service request. REFUSED = assigned by Primary Provider or Seller to indicate service request has been denied due to lack of availability of transfer capability. (Final state). |
| STATUS (cont.) |  |  |  | COUNTEROFFER= assigned by Primary Provider or Seller to indicate that a new OFFER_PRICE is being proposed or that CAPACITY_GRANTED is less than CAPACITY_REQUESTED. REBID = assigned by Customer to indicate that a new BID_PRICE is being proposed. SUPERSEDED = assigned by Primary Provider or Seller when a request which has not yet been confirmed is preempted by another reservation request. (Final state). ACCEPTED = assigned by Primary Provider or Seller to indicate the service request at the designated OFFER_PRICE and CAPACITY_GRANTED has been approved/accepted. If the reservation request was submitted PRECONFIRMED and CAPACITY_GRANTED is equal to CAPACITY_REQUESTED, the OASIS Node shall immediately set the reservation status to CONFIRMED. Depending upon the type of ancillary services required, the Seller may or may not require all ancillary service reservations to be completed before accepting a request. |
| STATUS (cont.) |  |  |  | DECLINED = assigned by the Primary Provider or Seller to indicate that the terms and conditions, such as the BID_PRICE, are unacceptable and that negotiations are terminated or that contractual terms have not been met. (Final state). RETRACTED = assigned by Primary Provider or Seller when the Customer fails to confirm or withdraw the request within the required time period. (Final state). WITHDRAWN = assigned by the Customer at any point in request evaluation to withdraw the request from any further action. (Final state). CONFIRMED = assigned by the Customer in response to the Primary Provider or Seller posting "ACCEPTED" status, to confirm service. Once a request has been "CONFIRMED", a transmission service reservation exits. (Final state, unless overridden by DISPLACED or ANNULLED state). DISPLACED = assigned by Primary Provider or Seller when a "CONFIRMED" reservation from a Customer is displaced by a higher priority reservation, and the Customer is not offered or has not exercised right of first refusal (i.e. refused to match terms of new request). (Final State.) ANNULLED = assigned by the Seller when, by mutual agreement with the Customer, a confirmed reservation is to be voided, or assigned unilaterally by the Primary Provider when a confirmed reservation is to be voided. (Final State). |