Business Practices for Open Access Same-Time Information Systems (OASIS) Implementation Guide

Document Change Log:

| Date | Section | Description of Change |
| --- | --- | --- |
| 06/06/2007 | All | Pertinent business process related requirements to be implemented as part of the OASIS Standards have been incorporated into this first version of the OASIS Implementation Guide. |
| 06/06/2007 | All | Global change of Primary Transmission Provider to Primary Provider and global change of Transmission Provider to Primary Provider. |
| 06/06/2007 | 013-2 | OASIS S&CP Standards 002-4.10.2 and 002-4.10.2.1 have been consolidated into 013-2. |
| 06/06/2007 | 013-2.1 | Enumerated values for OASIS REQUEST_TYPE data element from OASIS S&CP Standard 002-4.2.13 have been consolidated into 013-2.1. |
| 06/06/2007 | 013-2.2 | OASIS S&CP Standard 002-4.2.10.2 Status Values has been incorporated into Standard 013-2.2. |
| 06/06/2007 | 013-2.3 through 2.6 | All OASIS S&CP Standards contained under 002-4.2.13 and subordinate Standards have been incorporated and extended under new Standards 013-2.3 through 013-2.6 and subordinate Standards. |
| 06/06/2007 | 013-3 | Standards added to describe specific implementation requirements associated with certain OASIS Standard templates. |
| 06/06/2007 | 013-4.1 | OASIS S&CP Standard 002-4.4, File Examples, and all subordinate standards have been incorporated into this document in their entirety into Standard 013-4.1. |
| 06/06/2007 | 013-4.2 | Examples for Ancillary Services linkage to Transmission Services from OASIS S&CP Standard 002-4.2.12 have been incorporated into this document in their entirety into Standard 013-4.2 |

Business Practices Requirements:

013-1	INTRODUCTION

This OASIS Implementation Guide establishes general and specific transaction processing requirements and related business processes required for OASIS.  The technical standards for OASIS are defined in Standards WEQ 002 and WEQ 003, and the companion business practice standards are defined in Standards WEQ 001.

In the event of a conflict between a Primary Provider’s Tariff, applicable business practices, and this Implementation Guide, the Tariff shall take precedence over all, and business practices shall take precedence over this document.

013-1.1	Usage of Terms

The following terms as used throughout this Implementation Guide are to be interpreted as follows:

OASIS – Refers to the Primary Provider’s implementation of the OASIS Customer interface as defined in Standards WEQ 002, and any back-end supporting systems or user procedures that collectively perform the transaction processing functions associated with handling of requests on OASIS.

Business Practice – Refers collectively to any business practices adopted by the Primary Provider as defined in their Open Access Transmission Tariff (OATT), NAESB ratified Business Practice Standards, or provider specific practices or requirements.

Template – Refers generically, or by reference to a specifically named template, to the templates defined for the Customer interface to OASIS in Standards WEQ 002, including the displays and forms associated with the web browser based user interface implementing the functions of an OASIS defined template.

Must, shall, or required – Define specific requirements for OASIS processing that are not optional and must be implemented as described.

May, should, or optional – Define optional requirements that are recommended for implementation in OASIS but are not specifically required under these Standards.

Additional terms defined in Standards WEQ 001 and WEQ 002 are also hereby incorporated by reference.

013-2	OASIS TRANSACTION PROCESSING

The basic OASIS transaction process is described below.  This Implementation Guide also provides additional requirements and guidance for processing specific types of business transactions in the implementation of OASIS.  Note that the (Primary) Primary Provider may, but is not limited to, interacting with OASIS using the Transmission Customer template or user interface.  Primary Providers may also implement OASIS functions on back-end systems and are not required to perform all transaction processing on an OASIS node proper, provided that the results of all transaction processing are correctly posted on OASIS as required by the Tariff, regulation, or other established business practices.

The following is a summary of the templates used and actions that may be taken by the Customer and Seller to execute a transaction on OASIS.  Note that the OASIS Standards require all template functionality to be provided through a User Interface.  While this discussion focuses on template execution, all actions must be supported through a browser-based User Interface. Detailed examples of the transaction process and description of the business logic envisioned to be implemented as part of the Primary Provider’s OASIS or other back-end transaction support services are provided in subsequent sections of this Implementation Guide.

- a. The transrequest and ancrequest Templates shall be used by the Customer to enter a transaction request for specific transmission or ancillary services from a specified Seller.  All pertinent transaction-specific information must be provided in the template data elements.

- b. The transstatus and ancstatus Templates shall be used by both Customer and Seller to query for the current transaction information (e.g., STATUS).  Alternatively, the Customer may request dynamic notification per WEQ 002-4.2.10.3 whenever the transaction data is changed.

c. The transsell and ancsell Templates shall be used by the Seller to indicate to the Customer whether the request is acceptable or not by setting the transaction STATUS to one of RECEIVED, INVALID, STUDY, COUNTEROFFER, ACCEPTED, REFUSED, SUPERSEDED, DECLINED, DISPLACED, ANNULLED, or RETRACTED. A Primary Provider as the Seller may use the transsell and ancsell Templates to act on requests or may use proprietary software solutions to perform this function in a similar manner.

d. The transcust and anccust Templates shall be used by the Customer to indicate to the Seller whether they wish to negotiate, confirm or withdraw the transaction by setting the transaction STATUS to one of REBID, CONFIRMED, or WITHDRAWN.

- e. The transassign and ancassign Templates shall be used by the Seller to notify the Primary Provider of the transfer of rights from the Seller to the Customer consummated off the OASIS Node.

- f. The source of all Customer and Seller contact information shall be provided under WEQ 002-3.1 REGISTRATION AND LOGIN REQUIREMENTS. Therefore, it shall not be input as part of uploads, but shall be provided as part of all transaction downloads.

- g. OASIS Nodes shall accept a Seller-initiated change in STATUS to ACCEPTED only when OFFER_PRICE matches BID_PRICE.

- h. OASIS Nodes shall accept a Customer-initiated change in STATUS to CONFIRMED only when BID_PRICE matches OFFER_PRICE.

- i. If CAPACITY_GRANTED is null when STATUS is being changed to ACCEPTED or CONFIRMED, the OASIS Node shall set it equal to CAPACITY_REQUESTED.

j. OASIS Nodes shall set the initial transaction STATUS of the request to QUEUED and assign a unique ASSIGNMENT_REF identifier for the transaction.

k. If the Customer has identified the transaction as PRECONFIRMED and the Seller has set the transaction STATUS to ACCEPTED, OASIS Nodes shall automatically set the transaction’s STATUS to CONFIRMED without any Customer interaction required.

l. If the Customer has identified the transaction as PRECONFIRMED and the Seller has set the transaction STATUS to COUNTEROFFER, OASIS Nodes shall take no automatic confirmation action on the transaction and require explicit confirmation by the Customer.

This transaction process flow is depicted in the diagram below.

Exhibit 1 Transaction Template Usage Diagram

013-2.1	Transaction Request Types

The following are the valid OASIS transaction request types (template data element REQUEST_TYPE) that may be submitted by the Customer unless otherwise noted, along with a brief description of their intended use:

ORIGINAL =  typical reservation requests submitted to the Primary Provider (as the Seller of the transmission or ancillary service)

RESALE = secondary market requests submitted to a Transmission Customer as Secondary Provider

RENEWAL = request to renew an expiring transmission reservation

MATCHING = request to meet or exceed a competing request to retain transmission service (right of first refusal)

DEFERRAL = request to defer or apply for an extension on start of transmission service

REDIRECT = request to redirect all or portion of a transmission reservation to an alternate POR/POD and/or make other changes to the terms of service as permitted

RELINQUISH = request to release all or a portion of the capacity of a Redirect on a Non-Firm basis to the Firm Parent Reservation

{registered} = A Primary Provider may register values for REQUEST_TYPE to implement specific provisions of their Tariff.

This Implementation Guide contains detailed descriptions on the use of each transaction REQUEST_TYPE and explains the business processes to be implemented in association with each of these requests as specified by the OASIS Business Practice Standards, WEQ 001.

013-2.2	Transaction Status

The following are the defined values that may appear in the STATUS data element associated with a given OASIS transaction:

QUEUED = initial status assigned by TSIP on receipt of "customer services purchase request."

INVALID = assigned by TSIP, or Primary Provider indicating an invalid field in the request, such as improper POR, POD, source, sink, etc. (Final state).

RECEIVED = assigned by Primary Provider or Seller to acknowledge QUEUED requests and indicate the service request is being evaluated, including for completing the required ancillary services.

STUDY= assigned by Primary Provider or Seller to indicate some level of study is required or being performed to evaluate service request.

REFUSED = assigned by Primary Provider or Seller to indicate service request has been denied due to lack of available transfer capability. (Final state).

COUNTEROFFER = assigned by Primary Provider or Seller to indicate that a new OFFER_PRICE is being proposed or that CAPACITY_GRANTED is less than CAPACITY_REQUESTED.

REBID = assigned by Customer to indicate that a new BID_PRICE is being proposed.

SUPERSEDED = assigned by Primary Provider or Seller when a request which has not yet been confirmed is preempted by another reservation request. (Final state).

ACCEPTED = assigned by Primary Provider or Seller to indicate the service request at the designated OFFER_PRICE and CAPACITY_GRANTED has been approved/accepted. If the reservation request was submitted PRECONFIRMED and CAPACITY_GRANTED is equal to CAPACITY_REQUESTED, the OASIS Node shall immediately set the reservation status to CONFIRMED. Depending upon the type of ancillary services required, the Seller may or may not require all ancillary service reservations to be completed before accepting a request.

DECLINED = assigned by Primary Provider or Seller to indicate that the terms and conditions of the request, such as the BID_PRICE, are unacceptable and that negotiations are terminated or that contractual terms and conditions have not been met. (Final state).

CONFIRMED = assigned by Customer in response to Primary Provider or Seller posting "ACCEPTED" status, to confirm service. Once a request has been "CONFIRMED," a transmission service reservation exists. (Final state, unless overridden by DISPLACED or ANNULLED state).

WITHDRAWN = assigned by Customer at any point in request evaluation to withdraw the request from any further action. (Final state).

DISPLACED = assigned by Primary Provider or Seller when a "CONFIRMED" reservation from a Customer is displaced by a higher priority reservation and the Customer is not offered or has not exercised right of first refusal (i.e. refused to match terms of new request). (Final state).

ANNULLED = assigned by the Seller when, by mutual agreement with the Customer, a confirmed reservation is to be voided, or assigned unilaterally by the Primary Provider when a confirmed reservation is to be voided. (Final state).

RETRACTED = assigned by Primary Provider or Seller when the Customer fails to confirm or withdraw the request within the required time period. (Final state).

The following state transition diagram can be used as a business process guideline and illustrates the valid changes that may be made to the STATUS value by the Seller and Customer during the transaction process; however, individual tariffs may dictate specific allowed actions between states that are not reflected in this diagram.

Exhibit 2 - State Diagram of Purchase Transactions

013-2.3	Basic OASIS Transaction Handling

Requests to reserve or purchase transmission or ancillary service shall be submitted to OASIS by the Transmission Customer via the transrequest or ancrequest templates.

The Seller specified in the request must be the Primary Provider for REQUEST_TYPEs of ORIGINAL, REDIRECT, RELINQUISH, RENEWAL, or DEFERRAL.  The Seller specified in the request must be a registered entity other than the Primary Provider for REQUEST_TYPE of RESALE or TRANSFER.  The Seller may be either the Primary Provider or another registered entity for REQUEST_TYPE of MATCHING.

OASIS should screen submitted requests to validate proper use of REQUEST_TYPE.  Additional restrictions based on specific  REQUEST_TYPEs are detailed in subsequent Standards.  Validations on the service requested, service start time and duration, submission time, etc., are established by business practice.

Once successfully submitted on OASIS, the Seller may take any of the following actions via the transsell/ancsell template:

- Acknowledge receipt by setting STATUS to RECEIVED or STUDY

- Deny the request by setting STATUS to INVALID, DECLINED, or REFUSED

- Approve the request by setting STATUS to ACCEPTED or COUNTEROFFER

At any time during the processing of the request, the Customer may set STATUS to WITHDRAWN to remove the request from further consideration by the Seller.

Once the Seller approves the request, the Customer may take any of the following actions via the transcust/anccust template:

- WITHDRAW the request

- Continue negotiation of the request by setting STATUS to REBID

- Complete the request by setting STATUS to CONFIRMED

Prior to final confirmation by the Customer, the Seller may override their approval of the request with the following actions:

- Retract approval based on exceeding of customer confirmation time limits and/or scheduling deadlines or other criteria established by business practice by setting the STATUS to RETRACTED

- Retract approval based on receipt of a higher priority competing request by setting the STATUS to SUPERSEDED

Once CONFIRMED on OASIS by the Customer a service reservation shall be deemed to exist.  The Seller or the Primary Provider may take the following actions on a CONFIRMED service reservation:

- Nullify the reservation for cause by setting the STATUS to ANNULLED

- Displace the reservation in-whole to accommodate a higher priority competing request by setting the STATUS to DISPLACED

Seller’s shall provide a reason in the SELLER_COMMENTS whenever a service request is set to the STATUS of INVALID, REFUSED, DECLINED, RETRACTED, SUPERSEDED, ANULLED or DISPLACED.  The Primary Provider, when not acting as the Seller, shall provide a reason in the PROVIDER_COMMENTS whenever a service request is set to the STATUS of ANNULLED or DISPLACED.

013-2.4	Displacement

Displacement of an OASIS reservation may occur when a higher priority transmission service request is received by the Primary Provider and there is insufficient Available Transfer Capacity to honor that request, but capacity could be made available through the displacement of lower priority requests/reservations.  The following Standards describe the business process to be implemented on OASIS when this occurs.

013-2.4.1	Displacement – No Right of First Refusal

Confirmed transmission reservations may be subject to displacement in the event competing, higher priority requests are received by the Primary Provider.

If the existing Customer does not have the right of first refusal and all capacity from that Customer’s confirmed reservation is required to accommodate the higher priority request, the Primary Provider shall set the existing reservation's STATUS to DISPLACED. The STATUS of DISPLACED indicates that the reservation has been displaced in its entirety.

If only a portion of the confirmed reservation's capacity is required to accommodate the higher priority request, the Primary Provider shall document the "recall" of reserved capacity from the lower priority confirmed reservation by incrementing the IMPACTED counter on that reservation and posting on OASIS the amount and time frames over which that reservation's capacity was reduced, i.e., a partial displacement. The Transmission Customer may view all impacts to existing transmission service reservations (e.g., partial displacements, secondary sales, etc.) using the reduction Template.

A reference to the competing transmission service request that forced the displacement should be entered in the SELLER_COMMENTS field of the displaced reservation.

013-2.4.2	Displacement – With Right of First Refusal

Confirmed transmission reservations may be subject to displacement in the event competing, higher priority requests are received by the Primary Provider, but have the right of first refusal to retain their transmission service.

If the Primary Provider's Tariff obligates, or the Primary Provider elects to grant the original Customer the right of first refusal, the existing Customer shall be notified of the competing request. The Primary Provider shall set the existing lower priority reservation's COMPETING_REQUEST_FLAG to Y and update the SELLER_COMMENTS to include the ASSIGNMENT_REF associated with the higher priority competing request. These actions will initiate electronic notification, provided the Customer has elected to receive such notification.

If the existing Customer elects to meet the terms and conditions of the competing request,  that Customer shall submit a new MATCHING reservation request using the transrequest template.  The specific requirements associated with submission of MATCHING requests are detailed in Standard 013-2.6.3.

If the Primary Provider accepts and the Customer confirms the MATCHING request, the Primary Provider shall set the STATUS of the competing request to "REFUSED" and set the STATUS of the existing lower priority confirmed reservation to "DISPLACED". The STATUS of DISPLACED indicates that the reservation has been displaced in its entirety and has been replaced by the confirmed MATCHING reservation.

If the existing Customer does not elect to meet the terms of the competing request, the Primary Provider shall displace the existing lower priority reservation, in whole or in part, in the same manner described for displacement of reservations that are not extended a right of first refusal.

Once the result of the competition is resolved, whether through MATCHING or displacement of the existing reservation, the Primary Provider shall reset the COMPETING_REQUEST_FLAG to "N" in the reservation subject to displacement.

013-2.5	Primary Provider Recalls

There are cases in implementing provisions of the Primary Provider's Tariff that the capacity reserved by a Transmission Customer may be reduced in whole or in part, e.g. Displacement or Interruption. The particular reasons for these reductions are Tariff specific.

The Primary Provider shall provide a mechanism to post on OASIS any such reductions or "recalls" in reserved capacity. The Customer shall be notified of any and all such reductions in reserved capacity by the incrementing of the IMPACTED counter in association with those reservations that are reduced; the IMPACTED flag is viewable with the transstatus template.  Specific information regarding the exact nature of each reduction in the reserved capacity under a given transmission reservation shall be posted and viewable with the reduction template.

A specific example of a Primary Provider initiated recall of reserved capacity is the implementation of a partial displacement of a transmission reservation. In this instance, the Customer has not elected (or was not required to be offered) to match the terms of a higher priority, competing request. The Primary Provider "recalls" that capacity necessary to accommodate the higher priority request from the existing lower priority reservation. The IMPACTED counter of that reservation is incremented, and a query using the reduction template for that reservation would show the Customer the amount and time-frame over which the Customer's reserved capacity was recalled by the Primary Provider.

Interruption of transmission service, where that interruption directly impacts the rights of the Customer to schedule any service under that reservation, is another example of an impact to reserved capacity that would be posted as a Primary Provider initiated recall of reserved capacity.   Secondary market sales of transmission rights are not examples of a Primary Provider initiated recall of reserved capacity, but the impact of any such sales shall also be returned in response to execution of the reduction Template.

013-2.6	Transaction Specific Handling

The following Standards identify specific OASIS data elements and processing requirements that must be implemented by OASIS and/or associated back-end support systems.  The results of all transaction processing shall be viewable by all appropriate entities via the transstatus/ancstatus templates and corresponding OASIS User Interface.

013-2.6.1	ORIGINAL Requests

ORIGINAL requests shall be submitted by the Customer to arrange for new transmission or ancillary service with the Primary Provider.

The following are specific restrictions or requirements for OASIS service requests with REQUEST_TYPE of ORIGINAL.  If REQUEST_TYPE is not specified by the Customer and SELLER_CODE and SELLER_DUNS are the same as PRIMARY_PROVIDER_CODE and PRIMARY_PROVIDER_DUNS, OASIS shall default REQUEST_TYPE to ORIGINAL.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be ORIGINAL |
| RELATED_REF | Must be null |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |

Additional requirements related to the specification of service points, attributes, pricing, and timing are subject to the Primary Provider’s business practices.

The Transmission Customer may submit a time varying profile of capacity as allowed by the Primary Provider’s business practice by repeating the template data elements of CAPACITY_REQUESTED, START_TIME and STOP_TIME in template continuation records.

013-2.6.1.1	Offering of Partial Service

If in the evaluation of a transmission request, the Primary Provider determines that only a portion of the Customer's requested capacity (CAPACITY_REQUESTED Data Element) can be accommodated and that the Primary Provider is obligated or elects to offer the Customer only a portion of the requested capacity, the Primary Provider shall set the CAPACITY_GRANTED Data Element associated with that transmission service request to the amount available, and set the request’s STATUS to COUNTEROFFER.

If the CAPACITY_REQUESTED and/or CAPACITY_GRANTED are not constant over time, continuation records shall be used to convey the time varying profile of MW capacity associated with the transmission request via the Data Elements CAPACITY_REQUESTED, CAPACITY_GRANTED, START_TIME and STOP_TIME.  The profile of CAPACITY_GRANTED must span the entire START_TIME to STOP_TIME interval initially requested by the Customer even if CAPACITY_GRANTED is zero.

The Customer shall recognize the offer of partial service by CAPACITY_GRANTED not being equal to CAPACITY_REQUESTED and the request STATUS of COUNTEROFFER. The Customer may elect to CONFIRM, WITHDRAW, or REBID the reservation using the transcust Template.

If the transmission reservation request was marked PRECONFIRMED by the Customer and an offer of partial service is extended, the reservation request must be explicitly CONFIRMED by the Customer.  The OASIS node shall not automatically confirm a request where CAPACITY_REQUESTED does not equal CAPACITY_GRANTED when/if an attempt is made to set STATUS to ACCEPTED.

The Primary Provider shall use this same process in handling the deferral to the start of transmission service due to delays in completing the necessary transmission system studies associated with the request.  In these cases, the Primary Provider shall document the deferral by setting an initial transmission service profile record with CAPACITY_GRANTED set to 0 MWs and beginning with the originally requested START_TIME and STOP_TIME coincident with the delayed start of service.  The Primary Provider shall then specify the capacity to be made available to the Customer in one or more subsequent transmission service profile continuation records by defining CAPACITY_GRANTED, START_TIME and STOP_TIME as appropriate.

013-2.6.1.2	Negotiation of Price

Negotiation of price is initiated by the Customer submitting a service request (via transrequest/ancrequest) with a BID_PRICE that is different (higher or lower) from the currently posted offer price, or the tariff rate, for that service.  The following negotiation process is required where the Seller is the Primary Provider.  Resales or transfers between Transmission Customers may use this process, but there is no obligation on the (Re)Seller to offer a negotiated rate to other Customers.

If the Seller determines that the BID_PRICE is acceptable, the following actions must be taken (via transsell/ancsell):

- Update the currently posted offer price for the service requested and all other applicable services offered as dictated by current discounting policy (e.g., all unconstrained paths to the same point of delivery) to match BID_PRICE;

- Update the request’s NEGOTIATED_PRICE_FLAG to ‘L’ or ‘H’ if the BID_PRICE was lower than or higher than, respectively, the posted price when the request was submitted;

- Set the OFFER_PRICE equal to the BID_PRICE;

- Set the CAPACITY_GRANTED appropriately (if left null or undefined, OASIS shall set CAPACITY_GRANTED equal to CAPACITY_REQUESTED when STATUS is set to ACCEPTED);

- Set the request STATUS to ACCEPTED (or COUNTEROFFER if offering partial service)

The Customer may then confirm the purchase or withdraw the request by updating the request STATUS (via transcust/anccust).

If the Seller determines that the BID_PRICE is unacceptable, and negotiation of price is not going to be entertained, the Seller shall set the request STATUS to DECLINED (via transsell/ancsell):

If the Seller elects to enter into price negotiation, the following actions must be taken (via transsell/ancsell):

- If the price to be counter offered to the Customer is different than the currently posted offer price:

  - Update the currently posted offer price for the service requested and all other applicable services offered as dictated by current discounting policy (e.g., all unconstrained paths to the same point of delivery) to match the price to be counteroffered;

  - Update the request’s NEGOTIATED_PRICE_FLAG to ‘L’ or ‘H’ if the price to be counter offered is lower than or higher than, respectively, the posted price when the request was submitted;

- Set the OFFER_PRICE and CAPACITY_GRANTED appropriately;

- Set the request STATUS to COUNTEROFFER.

The Customer may then confirm the purchase, withdraw the request, or propose a new BID_PRICE by performing the following (via transcust/anccust):

- Update the request BID_PRICE;

- Set the request STATUS to REBID.

The Seller may then act on the new BID_PRICE by declining the request, accepting the BID_PRICE, or counter offering a new OFFER_PRICE using the same sequence of actions as stated above.

Negotiation of price may also be initiated on receipt of a request for similar service submitted with a higher BID_PRICE.  If required by business practice, the Seller (Primary Provider) may update any ACCEPTED but unconfirmed requests to COUNTEROFFER with the associated OFFER_PRICE set to meet the higher received BID_PRICE, and the negotiation of price can proceed as described above.

013-2.6.2	RENEWAL Requests

Transmission Customers shall use the REQUEST_TYPE of RENEWAL to exercise rollover rights associated with an existing transmission service reservation held by the Customer.  RENEWAL requests must always specify the Primary Provider as SELLER.

The following are specific restrictions or requirements for OASIS service requests with REQUEST_TYPE of RENEWAL.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be RENEWAL |
| RELATED_REF | Must specify the ASSIGNMENT_REF associated with an existing confirmed transmission service reservation held by the Customer that 1) has rollover rights, and 2) whose rollover rights have not expired. |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| PATH | Must represent the same corresponding service points in the reservation specified in RELATED_REF |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must specify a set of valid transmission service attributes recognized by the Primary Provider as a valid service designation eligible for exercising of rollover rights held by the reservation specified in RELATED_REF |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must match the STOP_TIME of the reservation specified in RELATED_REF |
| STOP_TIME | With START_TIME, must specify a valid interval of service eligible for exercising of rollover rights held by the reservation specified in RELATED_REF |
| CAPACITY_REQUESTED | Must be less than or equal to the amount of capacity eligible for rollover over the interval of service |

RENEWAL requests must be submitted on OASIS prior to expiration of the Customer’s rollover rights as established by business practice.

The transmission service attributes, e.g., TS_CLASS, etc., should match the corresponding attributes in the reservation specified in RELATED_REF.  However, changes may be made to these attributes over time such that some differences are necessary to accommodate changes in the Primary Provider’s business practices.  This also applies to changes in service points, e.g., PATH, etc., over time.

RENEWAL requests may be subject to offering of partial service and negotiation just as an ORIGINAL request.

013-2.6.3	MATCHING Requests

Transmission Customers shall use the REQUEST_TYPE of MATCHING to exercise right of first refusal to avoid being displaced by a higher priority competing request.

The following are specific restrictions or requirements for OASIS service requests with REQUEST_TYPE of MATCHING.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be MATCHING |
| RELATED_REF | Must specify the ASSIGNMENT_REF associated with an existing confirmed transmission service reservation held by the Customer that is subject to displacement. |
| PATH | Must represent the same corresponding service points in the reservation specified in RELATED_REF |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must specify a set of valid transmission service attributes that meet the requirements to exercise right of first refusal |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must specify the start time that meets the requirements to exercise right of first refusal |
| STOP_TIME | With START_TIME, must specify a valid interval of service eligible for exercising right of first refusal |
| CAPACITY_REQUESTED | Must specify the capacity that meets the requirements to exercise right of first refusal |

013-2.6.4	DEFERRAL Requests

The DEFERRAL request shall be used by Transmission Customers to request a delay in the start of an existing transmission service reservation held by the Customer.  DEFERRAL requests must always specify the Primary Provider as SELLER.

The following are specific restrictions or requirements for OASIS service requests with REQUEST_TYPE of DEFERRAL.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be DEFERRAL |
| RELATED_REF | If submitted by the Customer, must specify the ASSIGNMENT_REF associated with an existing confirmed transmission service reservation held by the Customer whose start time is to be delayed/deferred. If posted by the Primary Provider, must specify the ASSIGNMENT_REF associated with a pending transmission service request submitted by the Customer whose start time is to be deferred. |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| PATH | Must represent the same corresponding service points in the reservation/request specified in RELATED_REF |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must represent the same set of valid transmission service attributes recognized by the Primary Provider as a valid service designation specified in the reservation/request specified in RELATED_REF |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must specify the new requested time for the start of transmission service and must be greater than the START_TIME of the reservation/request specified in RELATED_REF |
| STOP_TIME | With START_TIME, must specify a valid interval of service based on the transmission service attributes |
| CAPACITY_REQUESTED | Must be equivalent to the amount of capacity originally reserved/requested |

When submitted by the Transmission Customer and ultimately approved and confirmed, OASIS or the Primary Provider procedurally must set the status of the reservation specified in RELATED_REF to ANNULLED.  The DEFERRAL request becomes the new transmission service reservation reflecting the delay in start of service requested.

The Primary Provider shall evaluate pending DEFERRAL requests relative to other transmission requests based on the TIME_QUEUED of the Customer’s ORIGINAL request for service.

013-2.6.5	REDIRECT Requests

The REDIRECT request is submitted by an existing firm point-to-point transmission customer to request the use of alternate points of receipt and/or delivery from the Primary Provider.  By definition, the Seller in a REDIRECT request must be the Primary Provider even if those rights being redirected were acquired from another Transmission Customer via Resale or Transfer.

The following Standards set forth the requirements for submission of REDIRECT requests on either a Firm or Non-Firm basis.

013-2.6.5.1	REDIRECT on a Firm Basis

A Customer holding confirmed firm point-to-point transmission rights may request the use of those rights on alternate points of receipt and/or delivery on a firm basis by submission of a REDIRECT request to the Primary Provider as Seller.  The following information must be submitted by the customer in the REDIRECT request via the transrequest template.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be REDIRECT |
| RELATED_REF | Must identify by ASSIGNMENT_REF a confirmed transmission service reservation for firm point-to-point service held by the submitting Customer |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| PATH | Must represent the new transmission service points being requested |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must represent a set of valid transmission service attributes for firm point-to-point service offered by the Primary Provider and being requested on the new service points by the Customer |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must specify the requested start of transmission service and must be within the bounds of START_TIME specified in the reservation identified in RELATED_REF |
| STOP_TIME | Must specify the requested stop/end of transmission service and must be within the bounds of STOP_TIME specified in the reservation identified in RELATED_REF, and with START_TIME must represent a valid interval of service for the firm point-to-point service being requested |
| CAPACITY_REQUESTED | Must specify the amount of transmission capacity being requested |
| BID_PRICE | Should specify the price for the service being requested; may be null |

The Primary Provider shall evaluate each REDIRECT on a Firm basis as any other new request for firm point-to-point transmission service.  Primary Provider business practices establish the requirements for service duration, submission time, evaluation time, confirmation time, etc.

OASIS or Primary Provider procedures should verify that the transmission reservation identified in RELATED_REF meets all the requirements to support the redirect of transmission rights to the new service points.  This should include the validation that the current rights, the Capacity Available for Redirect, held on that reservation in the amount of the REDIRECT over time have not been encumbered by any other confirmed redirects, resales, schedules, etc.  This capacity validation may occur at any point in the request process, but shall always be performed prior to setting the REDIRECT STATUS to CONFIRMED.

Once CONFIRMED, the transmission rights held on the RELATED_REF reservation in the amount of the REDIRECT shall be permanently released by the Primary Provider and conveyed to the REDIRECT reservation. The only mechanism for the Customer to return to the original points of receipt and/or delivery is to submit another REDIRECT request.

The impact on Available Transfer Capability (ATC) for the reservation identified by RELATED_REF shall be released and the impact of the REDIRECT transaction on ATC shall be accounted for in the amount and over time of the redirect simultaneously.

The impact of the REDIRECT transaction on the reservation(s) identified by RELATED_REF shall be posted and viewable using the reduction  template.

013-2.6.5.2	REDIRECT on a Non-Firm Basis

A Customer holding confirmed firm point-to-point transmission rights may request the use of those rights on alternate points of receipt and/or delivery on a non-firm basis by submission of a REDIRECT request to the Primary Provider as Seller.  The following information must be submitted by the customer in the REDIRECT request via the transrequest template.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be REDIRECT |
| RELATED_REF | Must identify by ASSIGNMENT_REF a confirmed transmission service reservation for firm point-to-point service held by the submitting Customer |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| PATH | Must represent the new transmission service points being requested |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must be HOURLY |
| TS_CLASS | Must be SECONDARY |
| TS_TYPE | Must be POINT_TO_POINT |
| TS_PERIOD | Must be FULL_PERIOD |
| TS_WINDOW | Must be FIXED |
| TS_SUBCLASS | Must be null |
| START_TIME | Must specify the requested start of transmission service and must be within the bounds of START_TIME specified in the reservation identified in RELATED_REF |
| STOP_TIME | Must specify the requested stop/end of transmission service and must be within the bounds of STOP_TIME specified in the reservation identified in RELATED_REF, and with START_TIME must represent a valid interval of service for the firm point-to-point service being requested |
| CAPACITY_REQUESTED | Must specify the amount of transmission capacity being requested |

Primary Provider business practices establish the requirements for service duration, submission time, evaluation time, confirmation time, etc.

OASIS or Primary Provider procedures should verify that the transmission reservation identified in RELATED_REF meets all the requirements to support the redirect of transmission rights to the new service points.  This should include the validation that the current rights, the Capacity Available for Redirect, held on that reservation in the amount of the REDIRECT over time have not be encumbered by any other confirmed redirects, resales, schedules, etc.  This capacity validation may occur at any point in the request process, but shall always be performed prior to setting the REDIRECT STATUS to CONFIRMED.

Once CONFIRMED, the transmission rights held on the RELATED_REF reservation in the amount of the REDIRECT shall be removed from the Capacity Available for Redirect on that reservation. Should the Customer wish to restore the rights to schedule (or redirect, resell, etc.) under the RELATED_REF reservation, they must submit a RELINQUISH request to the Primary Provider to release the capacity held under the REDIRECT request and reinstate that capacity on the original service  points. The Primary Provider must accommodate this release of capacity back to the original service points, and shall through OASIS or Primary Provider procedures insure that honoring this release of capacity and subsequent scheduling under the Customer’s original firm reservation will not unduly cause a reliability problem.

The impact on ATC for the reservation identified by RELATED_REF and the impact of the REDIRECT transaction on ATC shall be accounted for to both preserve the ability of the Customer to return to the original firm path and reflect the fact that the redirected capacity on the RELATED_REF and REDIRECT reservations cannot be scheduled simultaneously.

The impact of the REDIRECT transaction on the reservation(s) identified by RELATED_REF shall be posted and viewable using the reduction  template.

013-2.6.6	RELINQUISH Requests

The RELINQUISH request is submitted in association with a REDIRECT request on a non-firm basis to indicate the Customer’s desire to return the capacity rights held on the REDIRECT to the parent reservation specified in the RELATED_REF of that REDIRECT request.  The following are the specific requirements for the RELINQUISH request submitted via the transrequest template.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be RELINQUISH |
| RELATED_REF | Must specify the ASSIGNMENT_REF of a confirmed REDIRECT on a non-firm basis held by the Customer |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| PATH | Must represent the same corresponding service points in the reservation/request specified in RELATED_REF |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must represent the same set of valid transmission service attributes specified in RELATED_REF |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must specify start of the interval for the release of capacity from RELATED_REF |
| STOP_TIME | Must specify stop/end of the interval for the release of capacity from RELATED_REF |
| CAPACITY_REQUESTED | Must specify the capacity to be released from RELATED_REF over START_TIME to STOP_TIME interval and restored to the parent reservation (i.e., RELATED_REF’s RELATED_REF) |
| PRECONFIRMED | Must be YES |

When accepted by the Primary Provider, OASIS or Primary Provider procedures must remove the capacity held on the REDIRECT reservation over the time interval specified, and reinstate that capacity to the Capacity Available for Redirect of the parent firm reservation.

The combined impact of the REDIRECT and RELATED_REF reservations on ATC shall be reinstated to reflect the impact of the capacity relinquished back to the RELATED_REF reservation.

Note that the RELINQUISH request does not in itself represent a reservation for transmission service, but merely documents an action taken against an existing transmission service reservation.

013-2.6.7	RESALE Requests

RESALE requests may be submitted by the Customer (Assignee) to arrange for the sale or assignment of scheduling rights from another Transmission Customer (Reseller). Resellers and Assignees are not required to use OASIS to arrange for such sale or assignment of scheduling rights.  However, the Reseller is required to notify the Primary Provider and post information documenting the (re)sale on the Primary Provider’s OASIS where the rights were originally acquired.

The following are specific requirements for OASIS service requests with REQUEST_TYPE of RESALE.  If REQUEST_TYPE is not specified by the Customer and SELLER_CODE and SELLER_DUNS are not the same as PRIMARY_PROVIDER_CODE and PRIMARY_PROVIDER_DUNS, OASIS shall default REQUEST_TYPE to RESALE.

013-2.6.7.1	RESALE on OASIS

Resale transactions conducted on OASIS shall adhere to the Basic OASIS Request Processing requirements where the Reseller is identified as the Seller and the Assignee identified as the Customer.

The Assignee (Customer) initiates the resale of scheduling rights by submitting the following required information on OASIS via the transrequest template.  Data elements not listed are optional. There shall be no requirement imposed by OASIS that the Reseller post any corresponding offer of service for sale on that OASIS.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be RESALE |
| SELLER_CODE | Must identify the Reseller (registered entity other than the Primary Provider) |
| SELLER_DUNS | Must identify the Reseller |
| PATH | Must represent the transmission service points being requested |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must represent a set of valid transmission service attributes offered by the Primary Provider and being requested from the Reseller |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must specify the requested start of transmission service |
| STOP_TIME | Must specify the requested stop/end of transmission service |
| CAPACITY_REQUESTED | Must specify the amount of transmission capacity being requested |
| BID_PRICE | Should specify the price being requested; may be null |

If the RESALE meets all OASIS validation requirements, the RESALE request shall be posted on OASIS as a QUEUED transmission service request.  It is the Reseller’s obligation to respond to RESALE requests submitted to them on OASIS.  Resellers may register for dynamic notification of these events if the Primary Provider’s OASIS implementation provides for such functionality.

The Reseller may act on the RESALE request using any of the processes defined for Basic OASIS Request Processing, Offering of Partial Service, and/or Negotiation of Price.  At some point in this process, the Reseller must identify those transmission service reservations whose scheduling rights are to be conferred to the Assignee as follows.

| Data Element | Restriction/Requirement |
| --- | --- |
| REASSIGNED_REF | Collectively represent those confirmed transmission reservations held by the Reseller whose rights to schedule transmission capacity over time are to be conferred to the Assignee. Each reservation’s service points and transmission service attributes must correspond with those specified in the RESALE request |
| REASSIGNED_CAPACITY |  |
| REASSIGNED_START_TIME |  |
| REASSIGNED_STOP_TIME |  |

Business practices may dictate that this information be supplied and validated at the time the RESALE request STATUS is set to either ACCEPTED or COUNTEROFFER.  Business practices may also dictate the types of services that may be resold and/or aggregated in support of the resale as referenced by the REASSIGNED_REF data element.

Once the RESALE request is finally CONFIRMED by the Assignee, OASIS or Primary Provider procedures should (again) insure that every reservation represented in the REASSIGNED_REF data element meet the following requirements:

- Represent valid confirmed transmission service reservations held by the Reseller

- There is sufficient unscheduled or otherwise encumbered (e.g., other RESALE, REDIRECT, etc., transactions)  transmission capacity to meet REASSIGNED_CAPACITY over REASSIGNED_START_TIME to REASSIGNED_STOP_TIME

- Service points, e.g., PATH, etc., correspond correctly with those specified in the RESALE

- Transmission service attributes, e.g., TS_CLASS, etc., correspond correctly with those specified in the RESALE

- The sum of REASSIGNED_CAPACITY over time for all REASSIGNED_REF reservations satisfies the RESALE requirements as specified in CAPACITY_GRANTED, START_TIME and STOP_TIME.

If all of these requirements are met, OASIS or Primary Provider procedures shall convey the specified transmission rights from the Reseller’s reservation to the Assignee’s reservation.  The transmission capacity resold shall not be available to the Reseller to schedule, resell, redirect, or otherwise use.

The impact of the RESALE transaction on the reservation(s) identified by REASSIGNED_REF shall be posted and viewable using the reduction  template.

If the Primary Provider determines the posted RESALE does not comply with their business practices for such transactions, they shall have the right to reset the RESALE STATUS to ANNULLED.  OASIS or Primary Provider procedures shall then reinstate all reassignments of capacity over time that were designated by the Assignee to the parent (REASSIGNED_REF) reservation(s) as appropriate.  The Reseller setting the RESALE STATUS to DISPLACED or ANNULLED shall also reinstate all reassignments of capacity over time that were designated by the Assignee to the parent (REASSIGNED_REF) reservation(s) as appropriate.

013-2.6.7.2	RESALE off OASIS

Resale transactions arranged between Reseller and Assignee off OASIS must be documented on OASIS by the Reseller using the transassign  template.  Transactions arranged off OASIS do not follow the basic request processing steps and shall be posted directly as CONFIRMED transactions.

The following information is required to be supplied by the Reseller.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be RESALE |
| CUSTOMER_CODE | Must identify the Assignee |
| CUSTOMER_DUNS | Must identify the Assignee |
| PATH | Must represent the same corresponding service points as contained in every one of the reservation/request(s) specified by REASSIGNED_REF |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must represent the same corresponding transmission service attributes as contained in every one of the reservation/request(s) specified by REASSIGNED_REF |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must specify the start of transmission service arranged between Reseller and Assignee |
| STOP_TIME | Must specify the stop/end of transmission service arranged between Reseller and Assignee |
| CAPACITY_REQUESTED | Must be equal to the amount of capacity originally reserved/requested by the Assignee |
| CAPACITY_GRANTED | Must be equal to the amount of capacity ultimately resold to the Assignee |
| OFFER_PRICE | Must be the ultimate price arranged for service between Reseller and Assignee |
| REASSIGNED_REF | Collectively represent those confirmed transmission reservations held by the Reseller whose rights to schedule transmission capacity over time are to be conferred to the Assignee |
| REASSIGNED_CAPACITY |  |
| REASSIGNED_START_TIME |  |
| REASSIGNED_STOP_TIME |  |

OASIS or Primary Provider procedures should insure that every reservation represented in the REASSIGNED_REF data element meet the following requirements:

- Represent valid confirmed transmission service reservations held by the Reseller

- There is sufficient unscheduled or otherwise encumbered (e.g., other RESALE, REDIRECT, etc., transactions)  transmission capacity to meet REASSIGNED_CAPACITY over REASSIGNED_START_TIME to REASSIGNED_STOP_TIME

- Service points, e.g., PATH, etc., correspond correctly with those specified in the RESALE

- Transmission service attributes, e.g., TS_CLASS, etc., correspond correctly with those specified in the RESALE

- The sum of REASSIGNED_CAPACITY over time for all REASSIGNED_REF reservations satisfies the RESALE requirements as specified in CAPACITY_GRANTED, START_TIME and STOP_TIME.

Business practices may dictate the types of services that may be resold and/or aggregated in support of the resale as referenced by the REASSIGNED_REF data element.

If all of these requirements are met, OASIS or Primary Provider procedures shall convey the specified transmission rights from the Reseller’s reservation to the Assignee’s reservation.  The transmission capacity resold shall not be available to the Reseller to schedule, resell, redirect, or otherwise use.

The impact of the RESALE transaction on the reservation(s) identified by REASSIGNED_REF shall be posted and viewable using the reduction  template.

If the Primary Provider determines the posted RESALE does not comply with their business practices for such transactions, they shall have the right to reset the RESALE STATUS to ANNULLED.  OASIS or Primary Provider procedures shall then reinstate all reassignments of capacity over time that were designated by the Assignee to the parent (REASSIGNED_REF) reservation(s) if appropriate.  The Reseller setting the RESALE STATUS to DISPLACED or ANNULLED shall also reinstate all reassignments of capacity over time that were designated by the Assignee to the parent (REASSIGNED_REF) reservation(s) as appropriate.

013-3	SPECIFIC TEMPLATE IMPLEMENTATION

013-3.1	Registered Template Data Elements

Certain OASIS defined templates contain data elements that are identified as taking on values registered by the Primary Provider, e.g., reduction, systemdata, security, etc.  These registered values shall be included in the Energy Industry Registry when this registry is made publicly available.

In addition to public registration (when supported), each Primary Provider using a registered value for any OASIS data element shall provide information in their posted business practices as to each of these data element values along with a detailed description and example of how the data element will be used.  This information shall be made available on OASIS.

013-3.2	scheduledetail

The scheduledetail  template shall be used to post specific information related to the scheduled usage of reserved transmission service.

For (transmission) schedules derived from implemented electronic tags (e-Tag) submitted in accordance with the NERC electronic tagging specification, the following information must be posted on OASIS.

From the physical path segment of the e-Tag associated with the Primary Provider:

| Data Element | Restriction/Requirement |
| --- | --- |
| TRANSACTION_ID | The full e-Tag transaction identifier, including the GCA, creating PSE and LCA codes |
| PATH_NAME | Optional; defined by the Primary Provider based on POR/POD and transmission services used to support the schedule. |
| POINT_OF_RECEIPT | POR as identified in the e-Tag physical path |
| POINT_OF_DELIVERY | POD as identified in the e-Tag physical path |
| GCA_CODE | Registered acronym for the generating control area (balancing authority) |
| LCA_CODE | Registered acronym for the load control area (balancing authority) |
| SOURCE | Registered value of the source from the e-Tag located within the GCA |
| SINK | Registered value of the sink from the e-Tag located within the LCA |
| SCHEDULE_PRIORITY | The effective curtailment priority that will be used by the Primary Provider in assessing any curtailment action that may be taken against the schedule. |
| START_TIME/STOP_TIME | The time interval associated with the information associated with this segment of the schedule. |
| SCHEDULE_REQUESTED | The value from the PSE Market Level profile information contained in the e-Tag |
| SCHEDULE_GRANTED | The actual MW level to which the e-Tag was scheduled; for block schedules, SCHEDULE_GRANTED should always be the lower of SCHEDULE_REQUESTED and SCHEDULE_LIMIT; for dynamic schedules SCHEDULE_GRANTED may be higher than SCHEDULE_REQUESTED. |

For each start/stop segment of the posted schedule, the following information shall be provided for each transmission service reservation that is used to support that segment of the schedule.  There may be one or more transmission service reservations used to support a given schedule segment.  These “stacked” reservations shall be communicated through continuation records as defined in the OASIS S&CP (WEQ 002).

| Data Element | Restriction/Requirement |
| --- | --- |
| ASSIGNMENT_REF | The unique OASIS identifier assigned to the reservation supporting the schedule |
| SELLER_CODE SELLER_DUNS | Identification of the seller as listed in the transmission service reservation |
| CUSTOMER_CODE CUSTOMER_DUNS | Identification of the seller as listed in the transmission service reservation |
| AFFILIATE_FLAG | Identification of the reservation as being made by an affiliate of the Primary Provider |
| SERVICE_INCREMENT TS_CLASS TS_TYPE TS_PERIOD TS_WINDOW TS_SUBCLASS NERC_CURTAILMENT_PRIORITY OTHER_CURTAILMENT_PRIORITY | The transmission service attributes and curtailment priority information as specified in the transmission service reservation |
| CAPACITY_USED | The actual MWs of reserved capacity used in support of the schedule derived from the transmission allocation information as specified in the e-Tag |

If the tagged transaction has been subject to a Reliability Adjustment the following information shall be supplied.  This is typically, but not necessarily, indicated by SCHEDULE_GRANTED being less than SCHEDULE_REQUESTED.

| Data Element | Restriction/Requirement |
| --- | --- |
| PROVIDER_ACTION | As specified in the OASIS Data Dictionary, text descriptive of the action being taken by the Primary Provider, e.g., CURTAILMENT. |
| SCHEDULE_LIMIT | The value of the Reliability Limit set against the tagged transaction by any Reliability Entity over this segment of the schedule |
| CURTAILMENT_OPTIONS | Optional; Primary Provider supplied description of options that may be available to the customer, such as redispatch |
| SECURITY_REF | Optional; If the reliability adjustment was the result of a security event that is posted on OASIS via the security template, this shall be set to the OASIS unique identifier assigned to that posting |
| INITIATING_PARTY RESPONSIBLE_PARTY PROCEDURE_NAME PROCEDURE_LEVEL FACILITY_LOCATION FACILITY_NAME FACILITY_CLASS FACILITY_LIMIT_TYPE | Optional; If the reliability adjustment was the result of a security event that is posted on OASIS via the security template, these data elements will be reported as they appear in that associated security event posting |

013-4	EXAMPLE IMPLEMENTATION

013-4.1	File Request and File Download Examples

The following standards provide examples for the implementation of file request and download interactions that must be supported by OASIS.  In these examples, the end-of-line (eol) character is represented by the character,"  ". This symbol may appear different on displays or printouts.

013-4.1.1	File Example for Hourly Offering

Example of the request to Primary Provider, aaa, and response for Seller, wxyz, for PATH_NAME "W/AAAA/PATH-ABC//" for April 10, 1996 from 8 a.m. to 3 p.m. (Note that the PATH_NAME consists of a REGION_CODE, PRIMARY_PROVIDER_CODE, PATH_CODE, and an OPTIONAL_CODE, separated with a slash, "/".)

The VERSION for this release is 1.4. The request is in the form of a URL query string and the response is an ASCII delimited file.

- Query

http://(OASIS Node name)/OASIS/aaa/data/transoffering? ver=1.2&templ=transoffering& fmt=data&pprov=AAAA &pprovduns=123456789& path=W/AAA/ABC// &seller=WXYZAA &sellerduns=987654321& POR=aaa& POD=bbb& servincre=hourly& TSCLASS1=firm &TSCLASS2=non-firm&tz=PD&stime=19960410080000PD&sptime= 19960410150000PD

- Response Data

REQUEST-STATUS=200 (Successful)

TIME_STAMP=19960409113526PD 

VERSION=1.4

TEMPLATE=transoffering

OUTPUT_FORMAT=DATA 

PRIMARY_PROVIDER_CODE=AAAA

PRIMARY_PROVIDER_DUNS=123456789

DATA_ROWS=14

COLUMN_HEADERS=TIME_OF_LAST_UPDATE,SELLER_CODE,SELLER_DUNS,PATH_NAME,

POINT_OF_RECEIPT,POINT_OF_DELIVERY,INTERFACE_TYPE,OFFER_START_TIME, OFFER_STOP_TIME,

START_TIME,STOP_TIME, CAPACITY, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD,

TS_SUBCLASS, SALE_REF, POSTING_REF, CEILING_PRICE, OFFER_PRICE, PRICE_UNITS,

SERVICE_DESCRIPTION,SELLER_NAME,SELLER_PHONE,SELLER_FAX, SELLEREMAIL,

SELLER_COMMENTS 

19960409030000PD,WXYZ,987654321,W/AAA/ABC//,N/A,N/A,E,19960402080000PD,19960410080000PD,19960410

080000PD,19960410090000PD,300, HOURLY, FIRM, POINT_TO_POINT, OFF_PEAK, N/A, N/A, A001,

1.50,1.35,MW,N/A,N/A,N/A,N/A,N/A,10% DISCOUNT 

19960409030000PD,WXYZ,987654321,W/AAA/ABC//,N/A,N/A,E,19960402080000PD,19960410080000PD,19960410

080000PD,19960410090000PD,300, HOURLY, NON-FIRM, POINT_TO_POINT, OFF_PEAK, N/A, N/A,A001,1.50,

1.35,MW,N/A,N/A,N/A,N/A,N/A, 10% DISCOUNT 

19960409030000PD,WXYZ,987654321,W/AAA/ABC//,N/A,N/A,E,19960402080000PD,19960410080000PD,19960410

090000PD,1996041010000PD,300, HOURLY, FIRM, POINT_TO_POINT, OFF_PEAK, N/A,

N/A,A001,1.50,1.35,MW,N/A,N/A,N/A,N/A,N/A,10% DISCOUNT 

19960409030000PD,WXYZ,987654321,W/AAA/ABC//,N/A,N/A,E,19960402080000PD,19960410080000PD,19960410

090000PD,19960410100000PD,300, HOURLY, NON-FIRM, POINT_TO_POINT, OFF_PEAK, N/A, N/A,A001,1.50,

1.35,MW,N/A,N/A,N/A,N/A,N/A, 10% DISCOUNT 

19960409030000PD,WXYZ,987654321,W/AAA/ABC//,N/A,N/A,E,19960402080000PD,19960410080000PD,19960410

100000PD,19960410110000PD,300, HOURLY, FIRM, POINT_TO_POINT, OFF_PEAK, N/A,

N/A,A001,1.50,1.35,MW,N/A,N/A,N/A,N/A,N/A,10% DISCOUNT 

19960409030000PD,WXYZ,987654321,W/AAA/ABC//,N/A,N/A,E,19960402080000PD,19960410080000PD,19960410

100000PD,19960410110000PD,300, HOURLY, NON-FIRM, POINT_TO_POINT, OFF_PEAK, N/A, N/A,A001,1.50,

1.35,MW,N/A,N/A,N/A,N/A,N/A, 10% DISCOUNT 

19960409030000PD,WXYZ,987654321,W/AAA/ABC//,N/A,N/A,E,19960402080000PD,19960410080000PD,19960410

110000PD,19960410120000PD,300, HOURLY, FIRM, POINT_TO_POINT, OFF_PEAK, N/A,

N/A,A001,1.50,1.35,MW,N/A,N/A,N/A,N/A,N/A,10% DISCOUNT 

19960409030000PD,WXYZ,987654321,W/AAA/ABC//,N/A,N/A,E,19960402080000PD,19960410080000PD,19960410

110000PD,19960410120000PD,300, HOURLY, NON-FIRM, POINT_TO_POINT, OFF_PEAK, N/A, N/A,A001,1.50,

1.35,MW,N/A,N/A,N/A,N/A,N/A, 10% DISCOUNT 

. . .

. . .

. . .

19960409030000PD,WXYZ,987654321,W/AAA/ABC//,N/A,N/A,E,19960402080000PD,19960410080000PD,19960410

140000PD,19960410150000PD,300, HOURLY, FIRM, POINT_TO_POINT, OFF_PEAK, N/A,

N/A,A001,1.50,1.35,MW,N/A,N/A,N/A,N/A,N/A,10% DISCOUNT 

19960409030000PD,WXYZ,987654321,W/AAA/ABC//,N/A,N/A,E,19960402080000PD,19960410080000PD,19960410

140000PD,19960410150000PD,300, HOURLY, NON-FIRM, POINT_TO_POINT, OFF_PEAK, N/A, N/A,A001,1.50,

1.35,MW,N/A,N/A,N/A,N/A,N/A, 10% DISCOUNT 

013-4.1.2	File Example for Hourly Schedule Data

This example shows a request for the hourly schedule data from Primary Provider, AAAA, related to the seller, WXYZ, for the period 10 a.m. to 3 p.m. on April 10, 2000.

There are two identical requests examples using two slightly different methods. The first request is using a HTTP URL request string through an HTML GET method. The second request is a similar example using fetch_http from a file using a POST method.

- Query

URL Request (HTTP method=GET)

http://(OASIS Node name)/OASIS/aaaa/data/scheduledetail? ver=1.4& pprov=AAAA& templ=scheduledetail& fmt=data &pprovduns=123456789 &path=W/AAA/ABC//& seller=WXYZ &por=BBB &pod=CCC& tz=PD& stime=20000410100000PD& sptime=20000410150000PD

URL Request (HTTP method=POST)

$ fetch_http http://(OASIS Node name)/OASIS/aaaa/data/OASISdata -f c:/OASIS/wxyz/upload/infile. txt Where in-file.txt contains the following: ver=1.4& pprov=AAAA& templ=scheduledetail& fmt=data &pprovduns=123456789 &path=W/AAA/ABC//& seller=WXYZ &por=BBB &pod=CCC& tz=PD& stime=20000410100000PD& sptime=20000410150000PD

- Response Data

REQUEST_STATUS=200

ERROR_MESSAGE=No error. 

TIME_STAMP=20000410160523ES

VERSION=1.4

TEMPLATE=scheduledetail

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AAAA

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=PD

DATA_ROWS=3

COLUMN_HEADERS=CONTINUATION_FLAG, TIME_OF_LAST_UPDATE, SCHEDULE_REF, TRANSACTION_ID, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, GCA_CODE, LCA_CODE, SOURCE, SINK, SCHEDULE_PRIORITY, START_TIME, STOP_TIME, SCHEDULE_REQUESTED, SCHEDULE_GRANTED, ASSIGNMENT_REF, SELLER_CODE, SELLER_DUNS, CUSTOMER_CODE, CUSTOMER_DUNS, AFFILIATE_FLAG, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, NERC_CURTAILMENT_PRIORITY, OTHER_CURTAILMENT_PRIORITY, CAPACITY_USED, PROVIDER_ACTION, SCHEDULE_LIMIT, CURTAILMENT_OPTIONS, SECURITY_REF, INITIATING_PARTY, RESPONSIBLE_PARTY, PROCEDURE_NAME, PROCEDURE_LEVEL, FACILITY_LOCATION, FACILITY_NAME, FACILITY_CLASS, FACILITY_LIMIT_TYPE

N, 20000409030000PD,12345,2233, W/AAA/ABC//, BBB, CCC,,,,,1, 20000410100000PD, 20000410110000PD,300,300,856743, wxyz,987654321, WXYZAA,987654322, Y, HOURLY, NON_FIRM, POINT_TO_POINT, OFF_PEAK, FIXED,,1,1,300,,,,,,,,,,,, 

N, 20000409030000PD,12346,2233, W/AAA/ABC//, BBB, CCC,,,,,1, 20000410130000PD, 20000410140000PD,300,300,856743,wxyz,987654321, WXYZAA,987654322, Y, HOURLY, NON_FIRM, POINT_TO_POINT, OFF_PEAK, FIXED,,1,1,300,,,,,,,,,,,, 

N, 20000409030000PD,12347,2233, W/AAA/ABC//, BBB, CCC,,,,,1, 20000410140000PD, 20000410150000PD,300,300,856743,wxyz,987654321, WXYZAA,987654322, Y, HOURLY, NON_FIRM, POINT_TO_POINT, OFF_PEAK, FIXED,,1,1,300,,,,,,,,,,,, 

013-4.1.3	Customer Posting a Transmission Service Offering

This example shows how a Customer would post for sale the transmission service that was purchased previously. The Seller would create a file and upload the file using the FETCH_HTTP program to send a file to the OASIS Node of the Primary Provider.

- Input:

VERSION=1.4

TEMPLATE=transpost

OUTPUT_FORMAT=DATA 

PRIMARY_PROVIDER_CODE=AAAA

PRIMARY_PROVIDER_DUNS=123456789

DATA_ROWS=1

COLUMN_HEADERS=PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, INTERFACE_TYPE, CAPACITY, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_SUBCLASS, START_TIME, STOP_TIME, OFFER_START_TIME, OFFER_STOP_TIME, SALE_REF, OFFER_PRICE, SERVICE_DESCRIPTION, SELLER_COMMENTPF

WXYZ,987654321,W/AAA/ABC//,N/A,N/A,E,150, HOURLY, FIRM, POINT_TO_POINT, OFF_PEAK, N/A,, 19960402080000PD,19960410080000PD, 19960410080000PD,19960410150000PD, A123,.90,N/A,"As Joe said, ""It is a good buy"""

FETCH_HTTP Command to send posting $ fetch_http http://(OASIS Node name)/OASIS/abcd/data/transrequest -f c:/OASIS/abcd/upload/post.txt

- Response Data

REQUEST-STATUS=200  (Successful)

TIME_STAMP=19960409113526PD 

VERSION=1.4

TEMPLATE=transpost

OUTPUT_FORMAT=DATA 

PRIMARY_PROVIDER_CODE=AAAA

PRIMARY_PROVIDER_DUNS=123456789

DATA_ROWS=1

COLUMN_HEADERS=RECORD_STATUS, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, INTERFACE_TYPE, CAPACITY, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_SUBCLASS, START_TIME, STOP_TIME, OFFER_START_TIME, OFFER_STOP_TIME, SALE_REF, OFFER_PRICE, SERVICE_DESCRIPTION, SELLER_COMMENTS, ERROR_MESSAGE

200,WXYZ,987654321,W/AAA/ABC//,N/A,N/A,E,150, HOURLY, FIRM, POINT_TO_POINT, OFF_PEAK, N/A, 19960402080000PD,19960410080000PD, 19960410080000PD,19960410150000PD, A123,.90,N/A, "As Joe said, ""It is a good buy""", No Error

013-4.1.4	Example of Re-aggregating Purchasing Services using Reassignment

The following examples do not show the complete Template information, but only show those elements of the Template of interest to the example.

a. Customer #1, "BestE" requests the purchase of 150 MW Firm ATC for 8 a.m. to 5 p.m. for $1.00 from a Primary Provider (transrequest).

TEMPLATE=transrequest

CUSTOMER_CODE=BestE

CAPACITY=150

TS_CLASS="FIRM"

START_TIME="1996050708000000PD"

STOP_TIME="1996050717000000PD"

BID_PRICE="$1.00"

The Information Provider assigns ASSIGNMENT_REF = 5000 on acknowledgment.

b. Customer #1 purchases 120 MW ATC Non-firm for 3 p.m. to 9 p.m. for $.90 (transrequest). The Information Provider assigns the ASSIGNMENT_REF=5001 when the request for purchase is made and is shown in the acknowledgment.

TEMPLATE="transrequest"

CUSTOMER_CODE="BestE"

CAPACITY=120

TS_CLASS="NON-FIRM"

START_TIME="1996050715000000PD"

STOP_TIME="1996050721000000PD"

BID_PRICE="$1.05"

c. Customer #1 becomes Seller #1 and post the Transmission service of 100 MW ATC Non-firm capacity from 8 a.m. to 9 p.m. for resale at $.90/MW-hour.

TEMPLATE="transpost"

SELLER_CODE="BestE"

CAPACITY=100

TS_CLASS="NON-FIRM"

START_TIME="1996050708000000PD"

STOP_TIME="1996050721000000PD"

SALE_REF="BEST100"

OFFER_PRICE=.90

SELLER_COMMENTS="aggregating two previous purchases"

d. Customer #2 then requests purchase of 100 MW Non-firm from Reseller #1 from 8 a.m. to 6 p.m. for $0.90/MW-hour (transrequest).

TEMPLATE="transrequest"

CUSTOMER_CODE="Whlsle"

SELLER_CODE="BestE"

CAPACITY=100

TS_CLASS="NON-FIRM"

START_TIME="1996050708000000PD"

STOP_TIME="1996050721000000PD"

SALE_REF="BEST100"

DEAL_REF="WPC100"

BID_PRICE=.90

CUSTOMER_COMMENTS="Only need service until 6 p.m." 

The Information Provider provides the ASSIGNMENT_REF=5002 for this transaction.

e. Seller informs the Information Provider of the reassignment of the previous transmission rights when the seller accepts the customer purchase request (transsell).

TEMPLATE="transsell"

CUSTOMER_CODE="Whlsle"

SELLER_CODE="BestE"

ASSIGNMENT_REF=5002

STATUS="Accepted"

REASSIGNED_REF1=5000

REASSIGNED_CAPACITY1=100

REASSIGNED_START_TIME1="199605070800PD"

REASSIGNED_STOP_TIME1="199605071700PD"

REASSIGNED_REF2=5001

REASSIGNED_CAPACITY2=100

REASSIGNED_START_TIME2="199605071700PD"

REASSIGNED_STOP_TIME2="199605071800PD"

013-4.1.5	File Examples of the Use of Continuation Records

a. Basic Continuation Records

The first example of the use of Continuation Records is for the transrequest Template submitted by a Seller for purchase of a transmission reservation spanning 16 hours from 06:00 to 22:00 with "ramped" demand at beginning and end of time period. Two additional reservations appear prior to and following the profile to demonstrate the handling of ASSIGNMENT_REF by the OASIS Node.

Only the following fields may be redefined in a continuation record for the transrequest   Template: CAPACITY_GRANTED, START_TIME, STOP_TIME. Specification of any values corresponding to COLUMN_HEADERs other than CAPACITY-GRANTED, START_TIME, and STOP_TIME will be ignored, however commas must be included to properly align the CAPACITY_GRANTED, START_TIME and STOP_TIME fields.

Input:

VERSION=1.4

TEMPLATE=transrequest

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AEP

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=7

COLUMN_HEADERS=CONTINUTATION_FLAG, SELLER_CODE,SELLER_DUNS, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK,CAPACITY_REQUESTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, STATUS_NOTIFICATION,START_TIME,STOP_TIME,BID_PRICE, PRECONFIRMED, ANC_SVC_LNK, POSTING_REF, SALE_REF, REQUEST_REF, DEAL_REF, CUSTOMER_COMMENTS, REQUEST_TYPE,REASSIGNED_REF

N, AEP,123456789, ABC/XY, CE, MECS,,,35, DAILY, FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, pub/AEP/incoming,20000423000000ES,20000424000000ES,24.5, Y, SC:(AEP:RQ);RV:(AEP:RQ);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, P0123 , S123, R765, D123, Standard daily reservation, ORIGINAL, 

N, AEP,123456789, ABC/XY, CE, AMPO,,,5, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, pub/AEP/incoming,20000423060000ES,20000423070000ES,2.5, Y, SC:(AEP:RQ);RV:(AEP:RQ);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, P0123 , S123, R765, D123, First piece of profile spanning 5 records, ORIGINAL, 

Y,,,,,,,,10,,,,,,, ,20000423070000ES,20000423080000ES,,,,,,,, Second piece,, 

Y,,,,,,,,15,,,,,,, ,20000423080000ES,20000423200000ES,,,,,,,, Third piece,, 

Y,,,,,,,,10,,,,,,, ,20000423200000ES,20000423210000ES,,,,,,,, Fourth piece,, 

Y,,,,,,,,5,,,,,,, ,20000423210000ES,20000423220000ES,,,,,,,, Fifth piece,, 

N, AEP,123456789, ABC/XY, CE, MECS, , ,20, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, pub/AEP/incoming,20000423040000ES,20000423160000ES,2, Y, SC:(AEP:RQ);RV:(AEP:RQ);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, P0123 , S123, R765, D123, Standard hourly reservation after profiled reservation, ORIGINAL, 

Response:

REQUEST_STATUS=200

ERROR_MESSAGE=Successfully updated. 

TIME_STAMP=20000422160523ES

VERSION=1.4

TEMPLATE=transrequest

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AEP

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=7

COLUMN_HEADERS=RECORD_STATUS, CONTINUTATION_FLAG, ASSIGNMENT_REF, SELLER_CODE,SELLER_DUNS, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK,CAPACITY_REQUESTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, STATUS_NOTIFICATION,START_TIME,STOP_TIME,BID_PRICE, PRECONFIRMED, ANC_SVC_LNK, POSTING_REF, SALE_REF, REQUEST_REF, DEAL_REF, CUSTOMER_COMMENTS, REQUEST_TYPE,REASSIGNED_REF, ERROR_MESSAGE

200,N,8234, AEP,123456789, ABC/XY, CE, MECS,,,35, DAILY, FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, pub/AEP/incoming,20000423000000ES,20000424000000ES,24.5, Y, SC:(AEP:RQ);RV:(AEP:RQ);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, P0123 , S123, R765, D123, Standard daily reservation, ORIGINAL,, No error

200,N,8235, AEP,123456789, ABC/XY, CE, AMPO,,,5, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, pub/AEP/incoming,20000423060000ES,20000423070000ES,2.5, Y, SC:(AEP:RQ);RV:(AEP:RQ);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, P0123 , S123, R765, D123, First piece of profile spanning 5 records, ORIGINAL,, No error

200,Y,8235,,,,,,,,10,,,,,,,,20000423070000ES,20000423080000ES,,,,,,,, Second piece,,, No error

200,Y,8235,,,,,,,,15,,,,,,,,20000423080000ES,20000423200000ES,,,,,,,, Third piece,,, No error

200,Y,8235,,,,,,,,10,,,,,,,,20000423200000ES,20000423210000ES,,,,,,,, Fourth piece,,, No error

200,Y,8235,,,,,,,,5,,,,,,,,20000423210000ES,20000423220000ES,,,,,,,, Fifth piece,,, No error

200,N,8236, AEP,123456789, ABC/XY, CE, MECS,,,20, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, pub/AEP/incoming,20000423040000ES,20000423160000ES,2, Y, SC:(AEP:RQ);RV:(AEP:RQ);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, P0123 , S123, R765, D123, Standard hourly reservation after profiled reservation, ORIGINAL,, No error

b. Submission of Reassignment Information - Case 1:

In the prior example, a reservation request was submitted to "Reseller" for 20MW of Hourly Non-firm service from 04:00 to 16:00. Assume that Reseller has previously reserved service for the CE-VP path for Daily Firm in amount of 50 MW on 4/23 under ASSIGNMENT_REF=7019, and Hourly Non-Firm in amount of 10 MW from 08:00 to 20:00 on 4/23 under ASSIGNMENT_REF=7880. Reseller must designate which transmission service rights are to be reassigned to Cust to satisfy the 20MW from 04:00 to 16:00. This reassignment information is conveyed by Reseller using the transsell Template when the reservation request is ACCEPTED. At the SELLER's discretion, rights are assigned from the Non-firm reservation first (ASSIGNMENT_REF=7880) with the balance taken up by the Firm reservation (ASSIGNMENT_REF=7019).

The only fields allowed in "continuation" records for transsell Template are REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME, and REASSIGNED_STOP_TIME. Price may not be negotiated for each "segment" in a capacity profile.

Input:

VERSION=1.4

TEMPLATE=transsell

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AEP

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=3

COLUMN_HEADERS=CONTINUATION_FLAG,ASSIGNMENT_REF, START_TIME, STOP_TIME,OFFER_PRICE,CAPACITY_GRANTED, STATUS, STATUS_COMMENTS, ANC_SVC_LINK, ANC_SVC_REQ, NEGOTIATED_PRICE_FLAG, SELLER_REF, SELLER_COMMENTS, RESPONSE_TIME_LIMIT,REASSIGNED_REF,REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME

N,8236, 20000423040000ES, 20000423160000ES,2.5,20, ACCEPTED, Status comments here, SC:(AEP:RQ);RV:(AEP:RQ);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;,, S123, Seller comments here,,7019,20, 20000423040000ES, 20000423080000ES

Y,8236,,, ,, , , ,,,, ,,7880,10, 20000423080000ES, 20000423160000ES

Y,8236,,, ,, , , ,,,, ,,7019,10, 20000423080000ES, 20000423160000ES

Response:

REQUEST_STATUS=200

ERROR_MESSAGE=Successfully updated. 

TIME_STAMP=20000422160523ES

VERSION=1.4

TEMPLATE=transsell

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AEP

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=3

COLUMN_HEADERS=RECORD_STATUS,CONTINUATION_FLAG,ASSIGNMENT_REF, START_TIME, STOP_TIME,OFFER_PRICE,CAPACITY_GRANTED, STATUS, STATUS_COMMENTS, ANC_SVC_LINK, ANC_SVC_REQ, NEGOTIATED_PRICE_FLAG, SELLER_REF, SELLER_COMMENTS, RESPONSE_TIME_LIMIT,REASSIGNED_REF,REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME, ERROR_MESSAGE

N,,8236, 20000423040000ES, 20000423160000ES,2.5,20, ACCEPTED, Status comments here, SC:(AEP:RQ);RV:(AEP:RQ);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;,, S123, Seller comments here,,7019,20, 20000423040000ES, 20000423080000ES, No error

Y,,8236,,, ,, , , ,,,, ,,7880,10, 20000423080000ES, 20000423160000ES, No error

Y,,8236,,, ,, , , ,,,, ,,7019,10, 20000423080000ES, 20000423160000ES, No error

c. Submission of Reassignment Information - Case 2:

Primary Provider, AEP, is notified of a sale/assignment of transmission service rights from "Resell" to "cust". The parameters of the new reservation are for 10MW on 4/23 for "off-peak" hours (00:00- 06:00 and 22:00-24:00) on POR/POD CE-VP. Reseller is assigning rights to 10MW from a prior reservation for the CE-VP path for Daily Firm in amount of 50 MW on 4/23 under ASSIGNMENT_REF=7019 to Cust. AEP would submit the following information using the transassign Template to post this (re)sale. The only fields allowed in "continuation" records for the transassign Template are CAPACITY, START_TIME , STOP_TIME, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME, and REASSIGNED_STOP_TIME. Even though there is a one-to-one correspondence between the segments of the new reservations and the reassignment of service from a prior reservation, it is entirely possible that a reservation spanning a single contiguous period would require multiple continuation records to convey reassignment information, and vice versa.

Fields for CUSTOMER_NAME and SELLER_NAME were used to convey user names for subsequent resolution of contact information from user registration.

Input:

VERSION=1.4

TEMPLATE=transassign

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AEP

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=2

COLUMN_HEADERS=CONTINUATION_FLAG, CUSTOMER_CODE, CUSTOMER_DUNS, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK, CAPACITY_REQUESTED, CAPACITY_GRANTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD,TS_WINDOW, TS_SUBCLASS, START_TIME, STOP_TIME, OFFER_PRICE, ANC_SVC_LNK, POSTING_NAME, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME , SELLER_COMMENTS, SELLER_REF

N, ACSTMR,987654321, , CE, VP,,,10,10, HOURLY, NON-FIRM, POINT_TO_POINT, OFF_PEAK,FIXED,, 20000423000000ES, 20000423060000ES,2, SC:(AEP:RQ);RV:(AEP:RQ);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, Jane Doe ,7019,10, 20000423000000ES, 20000423060000ES, Seller comments go here, S123

Y,,,,,,,,10,10,,,,,,, 20000423220000ES, 20000424000000ES,,,,7019,10, 20000423220000ES, 20000424000000ES,, 

Response:

REQUEST_STATUS=200

ERROR_MESSAGE=Successfully updated. 

TIME_STAMP=20000422160523ES

VERSION=1.4

TEMPLATE=transassign

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AEP

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=2

COLUMN_HEADERS=RECORD_STATUS,CONTINUATION_FLAG,ASSIGNMENT_REF, CUSTOMER_CODE, CUSTOMER_DUNS, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK, CAPACITY_REQUESTED, CAPACITY_GRANTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD,TS_WINDOW, TS_SUBCLASS, START_TIME, STOP_TIME, OFFER_PRICE, ANC_SVC_LNK, POSTING_NAME, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME , SELLER_COMMENTS, SELLER_REF5, ERROR_MESSAGE

200,N,8207, ACSTMR,987654321, , CE, VP,,,10,10, HOURLY, NON-FIRM, POINT_TO_POINT, OFF_PEAK,FIXED,, 20000423000000ES, 20000423060000ES,2, SC:(AEP:RQ);RV:(AEP:RQ);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, Jane Doe ,7019,10, 20000423000000ES, 20000423060000ES, Seller comments go here, S1235, No error

200,Y,8207,,,,,,,,10,10,,,,,,, 20000423220000ES, 20000424000000ES,,,,7019,10, 20000423220000ES, 20000424000000ES,,5, No error

d. Query of Transmission Reservation Status:

The following typical response to a transstatus query might be delivered for 4/23 based on prior examples. Note that the only fields returned in "continuation" records are, ASSIGNMENT_REF, CAPACITY_REQUESTED, CAPACITY_GRANTED, START_TIME, STOP_TIME, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME, and REASSIGNED_STOP_TIME (price fields are debatable).

Input:

<appropriate query name/value pairs to return reservations for 4/23>

Response:

REQUEST_STATUS=200

ERROR_MESSAGE=No error. 

TIME_STAMP=20000423160523ES

VERSION=1.4

TEMPLATE=transstatus

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AEP

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=11

COLUMN_HEADERS=CONTINUATION_FLAG, ASSIGNMENT_REF, SELLER_CODE, SELLER_DUNS, CUSTOMER_CODE, CUSTOMER_DUNS, AFFILIATE_FLAG, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK, CAPACITY_REQUESTED, CAPACITY_GRANTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD,TS_WINDOW, TS_SUBCLASS, NERC_CURTAILMENT_PRIORITY, OTHER_CURTAILMENT_PRIORITY, START_TIME , STOP_TIME, CEILING_PRICE, OFFER_PRICE, BID_PRICE,PRICE_UNITS, PRECONFIRMED, ANC_SVC_LINK, ANC_SVC_REQ, POSTING_REF, SALE_REF, REQUEST_REF, DEAL_REF,IMPACTED,REQUEST_TYPE,RELATED_REF, NEGOTIATED_PRICE_FLAG, STATUS,STATUS_NOTIFICATION, STATUS_COMMENTS, TIME_QUEUED, TIME_OF_LAST_UPDATE, PRIMARY_PROVIDER_COMMENTS,SELLER_REF, SELLER_COMMENTS, CUSTOMER_COMMENTS, SELLER_NAME, SELLER_PHONE, SELLER_FAX, SELLER_EMAIL, CUSTOMER_NAME, CUSTOMER_PHONE, CUSTOMER_FAX, CUSTOMER_EMAIL, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME

N,8207, RSELLR,234567890, ACSTMR,987654321, N, , CE, VP, ,,10,10, HOURLY, FIRM, POINT_TO_POINT, OFF_PEAK,,,,, 20000423000000ES, 20000423060000ES,2.25,2,2,$/MW, N, SC:(AEP:AR:121);RV:(AEP:AR:122);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, ,S1235, , ,0, RESALE,, L, CONFIRMED,, , 20000422121354ES, 20000422123054ES, TP Comments,, Seller comments go here, Customer comments, Joe Smith, (888)-123-4567, (888)-123-1231, jsmith@xyz.com, Jane Doe, (999)-123-4567, (999)-123-8823,,7019,10, 20000423000000ES, 20000423060000ES

Y,8207,,,,,,,,,,,10,10,,,,,,,,, 20000423220000ES, 20000424000000ES,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,7019,10, 20000423220000ES, 20000424000000ES

N,8234, AEP,123456789, CUSTMR,345678912, N, , CE, MECS, ,,35,35, DAILY, FIRM, POINT_TO_POINT,FULL_PERIOD,FIXED,,,, 20000423000000ES, 20000423060000ES,42,24.5,24.5,$/MW, N, SC:(AEP:AR:123);RV:(AEP:AR:124);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, P0123 , S123, R765, D123,0, ORIGINAL,, L, CONFIRMED, pub/AEP/incoming,, 20000422131354ES, 20000422133354ES, Standard daily reservation,, System Operator, Customer comments, Frank Orth, (999)-123-4567, (999)-123-1231, jsmith@xyz.com, Jane Doe, (999)-123-4567, (999)-123-8823,,7019,10, 20000423000000ES, 20000423060000ES

N,8235, AEP,123456789, CUSTMR,345678912, N, , CE, AMPO, ,,5,5, HOURLY, NON-FIRM, POINT_TO_POINT,FULL_PERIOD,FIXED,,,, 20000423060000ES, 20000423070000ES,2.5,2.5,2.5,$/MW, N, SC:(AEP:AR:125);RV:(AEP:AR:126);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, P0123 , S123, R765, D123,0, ORIGINAL,,, CONFIRMED, pub/AEP/incoming,, 20000422160523ES, 20000422170523ES, Profile verified,, First piece, Customer comments, System Operator, (888)-123-4567, (888)-123-1231, jsmith@xyz.com, Jane Doe, (999)- 123-4567, (999)-123-8823,,7019,10, 20000423000000ES, 20000423060000ES

Y,8235,,,,,,,,,,,10,10,,,,,,,,, 20000423070000ES, 20000423080000ES,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,5 Y,8235,,,,,,,,,,,15,15,,,,,,,,, 20000423080000ES, 20000423200000ES,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

Y,8235,,,,,,,,,,,10,10,,,,,,,,, 20000423200000ES, 20000423210000ES,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,5 Y,8235,,,,,,,,,,,5,5,,,,,,,,, 20000423210000ES, 20000423220000ES,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

N,8236, AEP,123456789, CUSTMR,345678912, N, , CE, VP, ,,20,20, HOURLY,NON_FIRM, POINT_TO_POINT,FULL_PERIOD,FIXED,,,, 20000424040000ES, 20000424160000ES,2.5,2.5,2.5,$/MW, N, SC:(AEP:AR:127);RV:(AEP:AR:128);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, P0123 , S123, R765, D123,0, ORIGINAL,,, CONFIRMED, pub/AEP/incoming,, 20000422160723ES, 20000422171523ES, Bid price refused,, Negotiated OFFER_PRICE accepted,, Joe Smith, (888)-123-4567, (888)-123-1231, jsmith@xyz.com, Jane Doe, (999)-123-4567, (999)-123-8823,,7019,20, 20000423040000ES, 20000423080000ES

Y,8236,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,7880,10, 20000423080000ES, 20000423160000ES

Y,8236,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,7019,10, 20000423080000ES, 20000423160000ES

013-4.1.6	Examples of Negotiation of Price and Partial Service Offer

013-4.1.6.1	Negotiation with Preconfirmation

a. The Customer submits a PRECONFIRMED transmission service request using the transrequest Template. Initially, the STATUS is set to QUEUED by the OASIS Node.

b. The Seller has the option of setting STATUS via the transsell Template to one of the following: INVALID, RECEIVED, STUDY, COUNTEROFFER, ACCEPTED, DECLINED, or REFUSED.

c. The Seller has the option of entering a CAPACITY_GRANTED and setting the STATUS to COUNTEROFFER via the transell Template if the seller can only provide partial service.

d. If the Seller sets STATUS to ACCEPTED (and, as required by WEQ  013-2.2, the OASIS Node forces the Seller to set OFFER_PRICE equal to BID_PRICE as a condition to setting STATUS to ACCEPTED) and CAPACITY_GRANTED is equal to CAPACITY_REQUESTED, the OASIS Node will immediately set STATUS to CONFIRMED. (WEQ  013-2.2 requires the OASIS Node to set a null CAPACITY_GRANTED equal to CAPACITY_REQUESTED when STATUS is set to ACCEPTED.)

e. The Customer may WITHDRAW request via transcust Template at any time up to point where the Seller sets STATUS to ACCEPTED.

f. Once the STATUS is CONFIRMED, the OFFER_PRICE and CAPACITY_GRANTED officially becomes the terms of the reservation.

013-4.1.6.2	Negotiations without Preconfirmation

a. The Customer submits a transmission reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest Template. Initially the STATUS is set to QUEUED by the OASIS Node.

b. The Seller has the option of setting the STATUS via the transsell Template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, DECLINED, COUNTEROFFER, or REFUSED. If INVALID (due to invalid entries in the request), DECLINED (due to the Seller determining that the proposed price is not acceptable and further negotiations are not desired), or REFUSED (due to the unavailability of the requested service) are set, the transmission reservation request is terminated.

c. The Seller has the option of entering a CAPACITY_GRANTED and setting the STATUS to COUNTEROFFER via the transell Template if the seller can only provide partial service.

d. If the Seller set the STATUS to RECEIVED or STUDY, and determines that the BID_PRICE is too low, the Seller sets the OFFER_PRICE to the price desired, and sets the STATUS to COUNTEROFFER via the transsell Template.

e. The Customer agrees to the OFFER_PRICE, sets the BID_PRICE equal to the OFFER_PRICE, and sets the STATUS to CONFIRMED via the transcust Template.

f. The OFFER_PRICE and CAPACITY_GRANTED with the STATUS of CONFIRMED locks in the terms of the reservation.

013-4.1.6.3	Multiple Step Negotiations

a. The Customer submits a transmission reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest Template. Initially the STATUS is set to QUEUED by the OASIS Node.

b. The Seller has the option of setting the STATUS via the transsell Template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, DECLINED, COUNTEROFFER, or REFUSED. If INVALID, DECLINED, or REFUSED are set, the transmission reservation request is terminated.

c. The seller has the option of entering a CAPACITY_GRANTED and setting the STATUS to COUNTEROFFER via the transell Template if the seller can only provide partial service. If ATC changes before the request reaches the STATUS of CONFIRMED, seller may change the CAPACITY_GRANTED.

d. The Seller determines that the BID_PRICE is too low, sets the OFFER_PRICE to the desired value, and sets the STATUS to COUNTEROFFER via the transsell Template.

e. The Customer responds to the new OFFER_PRICE with an updated BID_PRICE and sets the STATUS to REBID for re-evaluation by the Seller.

f. The Seller determines that the BID_PRICE now is acceptable, and sets the STATUS to ACCEPTED via the transsell Template. The transition to ACCEPTED state requires the OFFER_PRICE to be set to the BID_PRICE: accepting a reservation with an OFFER_PRICE different from BID_PRICE would require the STATUS be set to COUNTEROFFER rather than ACCEPTED (see item c).

g. The Customer agrees to the OFFER_PRICE and sets the STATUS to CONFIRM via the transcust Template.

h. The OFFER_PRICE and CAPACITY_GRANTED with the STATUS as CONFIRMED locks in the terms of the reservation.

013-4.1.6.4	Negotiations Declined by Seller

a. The Customer submits a transmission reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest Template. Initially the STATUS is set to QUEUED by the OASIS Node.

b. The Seller has the option of setting the STATUS via the transsell Template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, DECLINED, COUNTEROFFER, or REFUSED. If INVALID, DECLINED, or REFUSED are set, the transmission reservation request is terminated.

c. The Seller determines that the BID_PRICE is too low, sets OFFER_PRICE to his desired value, and sets STATUS to COUNTEROFFER via the transsell Template.

d. The Customer responds to OFFER_PRICE with updated BID_PRICE and sets the STATUS to REBID via the transcust Template for re-evaluation by Seller.

e. The Seller breaks off all further negotiations by setting the STATUS to DECLINED, indicating that the price is unacceptable and that he does not wish to continue negotiations.

013-4.1.6.5	Negotiations Withdrawn by Customer

a. The Customer submits a transmission reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest. Initially the STATUS is set to QUEUED by the OASIS Node.

b. The Seller has the option of setting the STATUS via the transsell Template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, DECLINED, COUNTEROFFER, or REFUSED. If INVALID, DECLINED, or REFUSED are set, the transmission reservation request is terminated.

c. The Seller has the option of entering a CAPACITY_GRANTED and setting the STATUS to COUNTEROFFER via the transell Template if the seller can only provide partial service.

d. The Seller determines that the BID_PRICE is too low, sets the OFFER_PRICE to his desired value, and sets the STATUS to COUNTEROFFER via the transsell Template.

e. The Customer responds to the OFFER_PRICE with an updated BID_PRICE and sets the STATUS to REBID for re-evaluation by Seller.

f. The Seller determines that the BID_PRICE is still too low, sets the OFFER_PRICE to another value, and sets STATUS to COUNTEROFFER via the transsell Template.

g. The Customer breaks off all further negotiations, either because the OFFER_PRICE or CAPACITY_GRANTED are unacceptable, by setting STATUS to WITHDRAWN (or the Customer/Seller could go through additional iterations of REBID/COUNTEROFFER until negotiations are broken off or the reservation is CONFIRMED).

013-4.1.6.6	Negotiations Superseded by Higher Priority Reservation

a. The Customer submits a transmission reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest Template. Initially the STATUS is set to QUEUED by the OASIS Node.

b. The Seller has the option of setting the STATUS via the transsell Template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, DECLINED, COUNTEROFFER, or REFUSED. If INVALID, DECLINED, or REFUSED are set, the transmission reservation request is terminated.

c. If the Seller determines that another reservation has higher priority and must displace this request, he sets the STATUS to SUPERSEDED and the negotiations are terminated.

d. However, if desired and permitted by the tariff, the Seller may set the STATUS of a request in any of these previous states (including COUNTEROFFER and ACCEPTED) to COUNTEROFFER with an OFFER_PRICE which could avoid the request being superseded, thus allowing the Customer the choice of being SUPERSEDED or accepting the proposed OFFER_PRICE.

013-4.1.7	Audit Template examples

The following examples are included to show the general type of audit report responses that could be expected to be returned by implementations of the audit reporting Templates as documented above.

013-4.1.7.1	Offerings

The following is an example of a hypothetical audit query for daily non-firm offerings to the "DDD" point of delivery for Monday August 17, 1998 (line breaks and indentations added to improve readability):

REQUEST_STATUS=200 

ERROR_MESSAGE 

TIME_STAMP=19980821091601ES 

VERSION=1.4 

TEMPLATE=transofferingaudit 

OUTPUT_FORMAT=DATA 

PRIMARY_PROVIDER_CODE=WXYZ 

PRIMARY_PROVIDER_DUNS=78912345 

RETURN_TZ=ES 

DATA_ROWS=14 

COLUMN_HEADERS=RECORD_TYPE,TIME_OF_LAST_UPDATE,MODIFYING_COMPANY_CODE,MODIFYIN G_NAME, TIME_OF_LAST_UPDATE,SELLER_CODE,SELLER_DUNS,PATH_NAME,POINT_OF_RECEIPT,POINT_OF_DEL IVERY, INTERFACE_TYPE,OFFER_START_TIME,OFFER_STOP_TIME,START_TIME,STOP_TIME,CAPACITY,SERVIC E_INCREMENT, TS_CLASS,TS_TYPE,TS_PERIOD,TS_WINDOW,TS_SUBCLASS,ANC_SVC_REQ,SALE_REF,POSTING_REF,CEI LING_PRICE, OFFER_PRICE,PRICE_UNITS,SERVICE_DESCRIPTION,NERC_CURTAILMENT_PRIORITY,OTHER_CURTAIL MENT_PRIORITY, SELLER_NAME,SELLER_PHONE,SELLER_FAX,SELLER_EMAIL,SELLER_COMMENTS 

U,19980815131756ES,WXYZ,Jane Doe,19980815131756ES, WXYZ,78912345,X/WXYZ/AAA-DDD//,AAA,DDD,E,19 980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,800, DAILY,NON-FIRM,POINT_TO_POINT,FULL_PERIOD,FIXED,,SC:M;RF:M,,48732,102.00,85.00,$/MW-Day,,3,, Jane Doe,123-456-7813,123-456-7801, doej@wxyz.com 

U,19980815124022ES,WXYZ, Jane Doe,19980815124022ES, WXYZ,78912345,X/WXYZ/AAA-DDD//,AAA,DDD,E, 1 9980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,850, DAILY,NON-FIRM,POINT_TO_ POINT,FULL_PERIOD,FIXED,,SC:M;RF:M,, 48732,102.00,85.00,$/MW-Day,,3,, Jane Doe,123-456-7813,123-456-780 1,doej@wxyz.com, 

U,19980814120018ES,WXYZ, Joe Smith,19980814120018ES,WXYZ,78912345,X/WXYZ/AAA-DDD//,AAA,DDD,E,1 9980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,850, DAILY,NON-FIRM,POINT_TO_ POINT,FULL_PERIOD,FIXED,,SC:M;RF:M,,48732,102.00,90.00,$/MW-Day,,3,,Joe Smith,123-456-7893,123-456-7801 ,smithj@wxyz.com 

I,19980813171204ES,WXYZ, Supervisor,19980813171204ES, WXYZ,78912345,X/WXYZ/AAA-DDD//,AAA,DDD,E, 19980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,850, DAILY,NON-FIRM,POINT_TO _POINT,FULL_PERIOD,FIXED,,SC:M;RF:M,,48732,102.00,95.00,$/MW-Day,,3,, Supervisor,123-456-7890,123-456-78 01 

U,19980815124022ES,WXYZ, Jane Doe,19980815124022ES, WXYZ,78912345,X/WXYZ/BBB-DDD//,BBB,DDD,E,19 980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,1200, DAILY,NON-FIRM,POINT_TO_ POINT,FULL_PERIOD,FIXED,,SC:M;RF:M,,48783,102.00,85.00,$/MW-Day,,3,,Jane Doe,123-456-7813,123-456-7801, doej@wxyz.com 

U,19980814120022ES,WXYZ,Joe Smith,19980814120022ES,WXYZ,78912345,X/WXYZ/BBB-DDD//,BBB,DDD,E,19 980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,1200,DAILY,NON-FIRM,POINT_TO_ POINT,FULL_PERIOD,FIXED,,SC:M;RF:M,,48783,102.00,90.00,$/MW-Day,,3,,Joe Smith,123-456-7893,123-456-7801 ,smithj@wxyz.com 

I,19980813171210ES,WXYZ,Supervisor,19980813171210ES,WXYZ,78912345,X/WXYZ/BBB-DDD//,BBB,DDD,E,19 980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,1200,DAILY,NON-FIRM,POINT_TO_ POINT,FULL_PERIOD,FIXED,,SC:M;RF:M,,48783,102.00,95.00,$/MW-Day,,3,,Supervisor,123-456-7890,123-456-780 1, 

U,19980816101000ES,WXYZ,Supervisor,19980816101000ES,WXYZ,78912345,X/WXYZ/CCC-DDD//,CCC,DDD,E,19 980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,85,DAILY,NON-FIRM,POINT_TO_PO INT,FULL_PERIOD,FIXED,,SC:M;RF:M,,48820,102.00,102.00,$/MW-Day,,3,,Supervisor,123-456-7810,123-456-7801, 5U,19980814143807ES,WXYZ,Supervisor,19980814143807ES,WXYZ,78912345,X/WXYZ/CCC-DDD//,CCC,DDD,E,19 980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,90,DAILY,NON-FIRM,POINT_TO_PO INT,FULL_PERIOD,FIXED,,SC:M;RF:M,,48820,102.00,102.00,$/MW-Day,,3,,Supervisor,123-456-7890,123-456-7801, 5U,19980814120023ES,WXYZ,Joe Smith,19980814120023ES,WXYZ,78912345,X/WXYZ/CCC-DDD//,CCC,DDD,E,19 980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,110,DAILY,NON-FIRM,POINT_TO_P OINT,FULL_PERIOD,FIXED,,SC:M;RF:M,,48820,102.00,90.00,$/MW-Day,,3,,Joe Smith,123-456-7893,123-456-7801,s mithj@wxyz.com 

I,19980813171214ES,WXYZ,Supervisor,19980813171214ES,WXYZ,78912345,X/WXYZ/CCC-DDD//,CCC,DDD,E,19 980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,110,DAILY,NON-FIRM,POINT_TO_P OINT,FULL_PERIOD,FIXED,,SC:M;RF:M,,48820,102.00,95.00,$/MW-Day,,3,,Supervisor,123-456-7890,123-456-7801, 

U,19980815124023ES,WXYZ,Jane Doe,19980815124023ES,WXYZ,78912345,X/WXYZ/WXYZ-DDD//,WXYZ,DDD, E,19980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,340,DAILY,NON-FIRM,POINT_T O_POINT,FULL_PERIOD,FIXED,,SC:M;RF:M,,48855,102.00,85.00,$/MW-Day,,3,,Jane Doe,123-456-7813,123-456-78 01,doej@wxyz.com 

U,19980814120023ES,WXYZ,Joe Smith,19980814120023ES,WXYZ,78912345,X/WXYZ/WXYZ-DDD//,WXYZ,DDD, E,19980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,340,DAILY,NON-FIRM,POINT_T O_POINT,FULL_PERIOD,FIXED,,SC:M;RF:M,,48855,102.00,90.00,$/MW-Day,,3,,Joe Smith,123-456-7893,123-456-7 801,smithj@wxyz.com 

I,19980813171222ES,WXYZ,Supervisor,19980813171222ES,WXYZ,78912345,X/WXYZ/WXYZ-DDD//,WXYZ,DDD, E,19980814000000ES,19980817000000ES,19980817000000ES,19980818000000ES,340, DAILY,NON-FIRM,POINT_T O_POINT,FULL_PERIOD,FIXED,,SC:M;RF:M,,48855,102.00,95.00,$/MW-Day,,3,,Supervisor,123-456-7890,123-456-7 801, 

From this audit report, the daily non-firm offerings on the four paths to "DDD" (AAA-DDD, BBB-DDD, CCC-DDD, and WXYZ-DDD) were all originally posted by WXYZ's "Supervisor" at approximately 17:12 on 8/13 at a price of $95.00 /MW-Day discounted from a ceiling price of $102.00.

At approximately 12:00 on 8/14, Joe Smith changed the offer price to $90.00 on all four paths.

At 14:14 on 8/14, "Supervisor" adjusted the capacity available on path X/WXYZ/CCC-DDD// to 90 MW (POSTING_REF = 48820) and set the offer price up to match the tariff ceiling rate (presumably due to the path now being constrained and released from the requirement to have discounted service offered at the same rate as all other unconstrained paths to DDD). Capacity on this path was last updated to a value of 85 MW at 10:10 on 8/16, which is the current information posted on OASIS for this path at the time of the query.

Jane Doe adjusted the price on the three presumably unconstrained paths to DDD at 12:40 on 8/15 to $85.00, which may have been in response to a negotiation for service on one of these paths. No further updates have occurred to the offerings on paths BBB-DDD and WXYZ-DDD since that time.

Finally, the capacity available on path X/WXYZ/AAA-DDD// was updated by Jane Doe from 850 to 800 MW at 13:17 on 8/15, which may have corresponded with final confirmation of a reservation at a negotiated discount by the customer that initiated the price change from $90.00 to $85.00.

013-4.1.7.2	Reservations

The following is an example of a hypothetical audit query for a specific transmission service reservation (line breaks and indentations added to improve readability):

REQUEST_STATUS=200

ERROR_MESSAGE=

TIME_STAMP=19980821092048ES

VERSION=1.4

TEMPLATE=transstatusaudit

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=WXYZ

PRIMARY_PROVIDER_DUNS=78912345

RETURN_TZ=ES

DATA_ROWS=9

COLUMN_HEADERS=RECORD_TYPE,TIME_OF_UPDATE,MODIFYING_COMPANY_CODE,MODIFYING_NAME, CONTINUATION_FLAG,ASSIGNMENT_REF,SELLER_CODE,SELLER_DUNS,CUSTOMER_CODE, CUSTOMER_DUNS,AFFILIATE_FLAG,PATH_NAME,POINT_OF_RECEIPT,POINT_OF_DELIVERY, SOURCE,SINK,CAPACITY,SERVICE_INCREMENT,TS_CLASS,TS_TYPE,TS_PERIOD,TS_WINDOW, TS_SUBCLASS,NERC_CURTAILMENT_PRIORITY,OTHER_CURTAILMENT_PRIORITY,START_TIME, STOP_TIME,CEILING_PRICE,OFFER_PRICE,BID_PRICE,PRICE_UNITS,PRECONFIRMED,ANC_SVC_LINK,ANC _SVC_REQ,POSTING_REF,SALE_REF,REQUEST_REF,DEAL_REF,NEGOTIATED_PRICE_FLAG, STATUS,STATUS_NOTIFICATION,STATUS_COMMENTS,TIME_QUEUED,RESPONSE_TIME_LIMIT, TIME_OF_LAST_UPDATE,PRIMARY_PROVIDER_COMMENTS,SELLER_COMMENTS, CUSTOMER_COMMENTS,SELLER_NAME,SELLER_PHONE,SELLER_FAX,SELLER_EMAIL, CUSTOMER_NAME,CUSTOMER_PHONE,CUSTOMER_FAX,CUSTOMER_EMAIL,REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME,REASSIGNED_STOP_TIME

U,19980815131629ES,DEFPM,Alan Trader,N,104392,WXYZ,78912345,DEFPM,912876543,N,X/WXYZ/AAA-DDD//,A AA,DDD,AAA,ZZZ,50,DAILY,NON-FIRM,POINT_TO_POINT,FULL_PERIOD,FIXED,,3,,19980817000000ES,199808 18000000ES,102.00,85.00,85.00,$/MW-Day,N,,SC:M;RF:M,,,,,,CONFIRMED,,,19980815121510ES,19980815144100ES, 19980815131629ES,,,,Jane Doe,123-456-7813,123-456-7801,doej@wxyz.com,Alan Trader,312-678-9104,312-678-9100,a .trader@defmarketing.com,,,, 

U,,,,Y,,,,,,,,,,,,75,,,,,,,,,19980818000000ES,19980819000000ES,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U,,,,Y,,,,,,,,,,,,100,,,,,,,,,19980819000000ES,19980820000000ES,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U,19980815125042ES,WXYZ,Jane Doe,N,104392,WXYZ,78912345,DEFPM,912876543,N,X/WXYZ/AAA-DDD//,AAA ,DDD,AAA,ZZZ,50,DAILY,NON-FIRM,POINT_TO_POINT,FULL_PERIOD,FIXED,,3,,19980817000000ES,19980818 000000ES,102.00,85.00,82.00,$/MW-Day,N,,SC:M;RF:M,,,,,,COUNTEROFFER,,,19980815121510ES,19980815144100 ES,19980815125042ES,,,,Jane Doe,123-456-7813,123-456-7801,doej@wxyz.com,Alan Trader,312-678-9104,312-678-910 0,a.trader@defmarketing.com,,,, 

U,19980815124811ES,DEFPM,Alan Trader,N,104392,WXYZ,78912345,DEFPM,912876543,N,X/WXYZ/AAA-DDD//,A AA,DDD,AAA,ZZZ,50,DAILY,NON-FIRM,POINT_TO_POINT,FULL_PERIOD,FIXED,,3,,19980817000000ES,199808 18000000ES,102.00,85.00,82.00,$/MW-Day,N,,SC:M;RF:M,,,,,,REBID,,,19980815121510ES,19980815144100ES,19980 815124811ES,,,,Jane Doe,123-456-7813,123-456-7801,doej@wxyz.com,Alan Trader,312-678-9104,312-678-9100,a.trader @defmarketing.com

U,19980815124100ES,WXYZ,Jane Doe,N,104392,WXYZ,78912345,DEFPM,912876543,N,X/WXYZ/AAA-DDD//,AAA ,DDD,AAA,ZZZ,50,DAILY,NON-FIRM,POINT_TO_POINT,FULL_PERIOD,FIXED,,3,,19980817000000ES,19980818 000000ES,102.00,85.00,80.00,$/MW-Day,N,,SC:M;RF:M,,,,,,COUNTEROFFER,,,19980815121510ES,19980815144100 ES,19980815124100ES,,,,Jane Doe,123-456-7813,123-456-7801,doej@wxyz.com,Alan Trader,312-678-9104,312-678-910 0,a.trader@defmarketing.com

I,19980815121510ES,DEFPM,Alan Trader,N,104392,WXYZ,78912345,DEFPM,912876543,N,X/WXYZ/AAA-DDD//,A AA,DDD,AAA,ZZZ,50,DAILY,NON-FIRM,POINT_TO_POINT,FULL_PERIOD,FIXED,,3,,19980817000000ES,199808 18000000ES,102.00,90.00,80.00,$/MW-Day,N,,SC:M;RF:M,,,,,,QUEUED,,,19980815121510ES,,19980815121510ES,,,,Company Default,123-456-7800,123-456-7801,,Alan Trader,312-678-9104,312-678-9100,a.trader@defmarketing.com

I,,,,Y,,,,,,,,,,,,75,,,,,,,,,19980818000000ES,19980819000000ES,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

I,,,,Y,,,,,,,,,,,,100,,,,,,,,,19980819000000ES,19980820000000ES,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

First, this example shows the handling of continuation records which conveyed a time varying demand of 50 MW on 8/17, 75 MW on 8/18, and 100 MW on 8/19. This demand profile was initially entered with the original reservation request (transrequest Template) at 12:15 on 8/15 by Alan Trader. Since the Data Elements associated with the profile were never modified, the intervening audit response records do not repeat the data from these continuation records.

As part of the original reservation, Alan Trader attempted to negotiate a price for service of $80.00 /MW-Day. Jane Doe responded to this request with a counter offer at the rate of $85.00 /MW-Day at 12:41 on 8/15. Since the status of COUNTEROFFER constitutes acceptance of all terms of the reservation except price (i.e., transmission capability has been evaluated and is available), the RESPONSE_TIME_LIMIT Data Element has been updated to reflect the time by which the customer must confirm service (assuming the establishment of customer confirmation time limits is approved by FERC).

At 12:48, Alan Trader attempted to negotiate further for a rate of $82.00 /MWDay and the reservation status was set to REBID. Jane Doe responded at 12:50 with a second counter offer restating a rate of $85.00, which Alan Trader finally agreed to at 13:16 on 8/15. The current posted information on OASIS shows this final CONFIRMED reservation.

013-4.2	Ancillary Service Linkage Examples

The following examples document the handling of ancillary service linkage to transmission service requests/reservations using the ANC_SVC_LINK data element.

Example 1:

Assume ancillary services SC and RV are mandatory from the TP, whose code is "TPEL", and ancillary services RF, EI, SP and SU are required, but will be defined at a future time.

"SC: (TPEL:RQ); RV: (TPEL:RQ); RF:(:FT); EI:(:FT); SP:(:FT); SU:(:FT)";

Example 2:

Assume ancillary services SC and RV are mandatory from the TP, whose code is "TPEL", and RF, EI, SP and SU are self-supplied. The customer code is "CPSE"

"SC: (TPEL:RQ); RV: (TPEL:RQ); RF:(CPSE:SP); EI:(CPSE:SP);

SP:(CPSE:SP); SU:(CPSE:SP)"

Example 3:

Assume ancillary services SC and RV are mandatory from the TP, whose code is "TPEL", and ancillary services RF, EI, SP and SU were purchased via a prior OASIS reservation from seller "SANC" whose reservation number was "39843". There is sufficient capacity within the Ancillary reservation to handle this Transmission reservation.

"SC:(TPEL:RQ); RV:(TPEL:RQ); RF:(SANC:AR:39843);

EI:(SANC:AR:39843) SP:(SANC:AR:39843); SU:(SANC:AR:39843)"

Example 4:

Assume ancillary services SC and RV are mandatory from the TP, whose code is "TPEL", and ancillary services RF, EI, SP and SU were purchased via prior OASIS reservations from sellers "SANC" and "TANC", whose reservation numbers where "8763" and "9824" respectively. There is not sufficient capacity within the Ancillary reservation from seller "SANC" to handle this Transmission reservation. In this case the OASIS reservation number 8763 will be depleted for the time frame specified within the transmission reservation and the remaining required amount will come from reservation number "9824".

"SC:(TPEL:RQ); RV:(TPEL:RQ); RF:((SANC:AR:8763)(TANC:AR:9824));

EI:((SANC:AR:8763)(TANC:AR:9824));

SP:((SANC:AR:8763)(TANC:AR:9824));

SU:((SANC:AR:8763)(TANC:AR:9824))"

Example 5:

Assume a transmission reservation in the amount of 100 mw/hour for a period of one day is made. Ancillary services SC and RV are mandatory from the TP, whose code is "TPEL", and ancillary services RF, EI, SP and SU were purchased via prior OASIS reservations from sellers "SANC" and "TANC", whose reservation numbers where "8763" and "9824" respectively. There is sufficient capacity within the Ancillary reservation from seller "SANC" to handle this Transmission reservation, however the purchaser wishes to use only "40 mw's" from this seller. In this case the OASIS reservation number 8763 will be depleted in the amount of "40 mw's" for the time frame specified within the transmission reservation and the remaining required amount will come from reservation number "9824".

"SC:(TPEL:RQ); RV:(TPEL:RQ); RF:((SANC:AR:8763:40)(TANC:AR:9824));

EI:((SANC:AR:8763:40)(TANC:AR:9824));

SP:((SANC:AR:8763:40)(TANC:AR:9824));

SU:((SANC:AR:8763:40)(TANC:AR:9824))"