To: 		NAESB WEQ ESS / ITS

From: 		PJM Interconnection, LLC

Subject:	Formal Comments on  the WEQ Request No.: 2008 AP Item 2.b.v.3 & 2008 AP Item 2.a.i.3

Request Title:	ATC/TTC Narratives

Date: 	March 31, 2008

PJM has reviewed the ESS / ITS Subcommittee’s request for formal comments on the:

WEQ Request No.: 2008 AP Item 2.b.v.3 & 2008 AP Item 2.a.i.3

Request Title:	ATC/TTC Narratives

Attached please find PJM provided comments and a redline version of the original request.

Your consideration of these formal comments is most appreciated.

Respectfully Submitted:

Patrick Brown, Manager, NERC & Regional Coordination

Chris Advena, Manager, Tariff Integration

Kevin Gresh, Analyst, Tariff Integration

Don Williams, Senior Engineer, Tariff Integration

Joel Segal, Consultant, Tariff Integration

NAESB WEQ Request for Formal Comments for WEQ Recommendation –

Due March 31, 2008- PJM Specific Comments

PJM would like to thank the ESS/ITS/BPS Subcommittee for the hard work required to get this standard out for formal comments.  However, PJM has recommendations and comments to the standard as included below and have attached a redline of the standard showing our recommended changes.  Please consider these for the next draft.

1. Changes in the Standard Z.2.3.5:

PJM is recommending changes in the Standard Z.2.3.5: ATC Change Text String Information to line up with the NERC MOD Standards.  According to NERC Standard MOD-030 Requirement R6 and R7 many of the components identified in NAESB Standard Z.2.3.5 as reasons for change in TFC are contained in the ETC (Existing Transmission Commitments) component whereas for NERC Standard MOD-028 and 029 TTC component can include those components listed.  PJM recommends that an additional item be added to the list to recognize the aggregated effect of these components in the ETC identified in the Standard.  Also FERC recognized in Order 890A paragraph 127 (shown below) that PJM does not base changes in ATC on changes in TTC, but on AFC values. Therefore, aggregate changes in ETC or AFC should be includes in language for Standard Z.2.3.5 as valid reasons for changes in ATC.  FERC Order 890 (paragraph 369) affirms that narratives are not required for ATC changes as a result of transactions being reserved.  Since MOD-030 R6 does not include a specific formula for ETC recognizing that the components may be difficult to isolate, this further supports the need to add the ATC narrative explanation related to aggregate changes in ETC.

127. In response to PJM, we reiterate that all transmission providers must comply with this posting requirement. Transmission providers using an AFC calculation method that does not base changes in ATC on changes in TTC may comply with this requirement by posting narrative explanations of the reasons for changes in AFC values as a result of changes in AFC inputs that cause ATC or TTC to change by 10 percent or more. We direct each transmission provider that employs the AFC calculation methodology to provide a statement in the compliance filing required in section II.C describing how the narrative is derived for ATC/TTC postings or, if such information was provided in a prior compliance filing, a reference to that filing.

2. Clearer Explanation of a constrained Posted Path:

PJM believes clearer explanation of a constrained Posted Path is needed to allow consistent application among all TPs.  PJM can provide a definition is needed.

3. Eliminate Non-Firm from Standard:

PJM believes that this Standard should only apply to Firm ATC.  Application of this Standard to Non-Firm ATC should be removed from the Standard.  There appears to be no reference to include Non-Firm ATC in the FERC Orders cited in the standard.

4. Allow Alternate Method to Post ATC Narrative in Flat File:

PJM believes the Standard should allow TP to a flat file located in a standard location addressed in NAESB Standard  “ATC Information Link on OASIS” is an acceptable method.  PJM believes that to utilize the OASIS database and force customers to query the system instead of being able to web-browse our OASIS ATC information seems to make it more difficult for customers to see changes.  A web posting that has a last updated/last changed date would seem like a better advantage for the customer, and not complicate the transaction processing functionality of the OASIS database.

5. Data Retention for STOP_TIME with null value:

PJM believes that data retention of ATC Narratives should be addressed in the standard because the data retention may be different then that for OASIS posting data to remain online.  In addition, if the STOP_TIME is null for an ATC Narrative, how long is reasonable to keep the data online before it is removed? Can this purge be performed manually based on some duration?  PJM believes that the ATC narrative can be removed after 90 days if it is no long needed or at most after one year.

6. Format Change:

Change order of sections in WEQ 013-3.3.1 for systemdata template Data Element descriptions to line up with Standard Y (Zero ATC) first then address Standard Z (ATC 10%). Currently, WEQ 013-3.3.1 addresses these in the reverse order.

7. Appendix A Example use Clearer Terminology:

PJM believes that the terminology used should be more consistent.  (See related PJM comment 2). PJM believes that the term constrained commercial path or commercial path used in some places in the Appendix A example should be changed to constrained Posted Path as used in the standard and.  PJM also believes the Appendix A examples provide assistance in understanding the standard.

If you have any questions or need further clarification on PJM’s comments please contact

Don Williams (610)666-4644 willid@pjm.com

- 1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

X   Accept as requested			   X  Change to Existing Practice

Accept as modified below		      Status Quo

Decline

- 2.  TYPE OF DEVELOPMENT/MAINTENANCE

- Per Request:					Per Recommendation:

X Initiation					   X  Initiation

Modification				      Modification

Interpretation				      Interpretation

Withdrawal				      Withdrawal

Principle 					      Principle

Definition 				      Definition

X  Business Practice Standard 		  X   Business Practice Standard

Document 				      Document

Data Element 				      Data Element

Code Value 				      Code Value

X12 Implementation Guide		     X12 Implementation Guide

Business Process Documentation	      Business Process Documentation

- 3.  RECOMMENDATION

- SUMMARY:

In the FERC Orders 890 (paragraphs 369-371) and 890-A (paragraphs 123-128) under Docket Nos. RM05-17 and RM05-25, the Commission made conclusions about ATC/TTC Narratives as more fully described in section 4 of this recommendation.

The ESS/ITS/BPS has reviewed Orders 890 and 890-A and recommends adoption of the following standards.  The Subcommittee feels this recommendation complies with the guidance provided by the Commission.

The ESS/ITS/BPS is recommending the approval of this recommendation pending the approval of recommendation Number: WEQ Annual Plan item 2(ii)  Title: Group 1: Conditional Firm, Annotations For ATC; Load Forecast And Actual Load; Re-Bid Of Partial Service; And Preconfirmation Priority

- Recommended Standards:

- Business Practice Standards for Posting the ATC and TTC Narratives

- Standard Y: Zero ATC Narrative

The Transmission Provider shall post on OASIS a brief, but specific, narrative associated with yearly or monthly ATC values when either the posted firm or non-firm ATC remains unchanged at a value of zero for six months or longer.

- Standard Y.1:  Frequency and Timing of Zero ATC Narrative

- A zero ATC narrative shall be posted when ATC values change according to the following requirements:

- Standard Y.1.1:  Initial Zero ATC Narrative

The Transmission Provider shall post a zero ATC narrative when ATC for a constrained Posted Path meets any of the following criteria (for examples see WEQ 013.XX):

- The monthly ATC value for a given month has been posted at a value of zero for the past six consecutive months

- The monthly ATC is posted at a value of zero for a duration of six consecutive months; where that six-month duration could span past, present, and future months.

- The yearly ATC is posted at a value of zero for one or more future years

- Standard Y.1.2:  Successive Zero ATC Narrative

Once the initial zero ATC narrative has been posted, a Transmission Provider shall post successive narratives when the posted ATC for a constrained Posted Path remains zero and meets either of the following criteria:

- The conditions leading to requiring the posting of such narrative have changed.

- The conditions leading to requiring the posting of such narrative have not changed for six consecutive months and are still applicable.

- Standard Y.1.3: Zero ATC Narrative Timing

TheTransmission Provider shall post a zero ATC narrative for each constrained Posted Path within five (5) business days of meeting  the criteria specified in Y.1.1 or Y.1.2.

- Standard Y.2: Zero  ATC Narrative Posting On OASIS

A zero ATC narrative shall be posted on OASIS for ATC on each constrained Posted Path which meets one or more of the criteria specified  in Standard Y.1.1 and Y.1.2.

Standard Y.2.1: Use of systemdata

A zero ATC narrative shall be posted such that it may be queried and viewed on OASIS using the systemdata template.

Standard Y.2.2: systemdata Information

There shall be separate narrative postings for zero firm ATC and zero non-firm ATC as follows:

- SYSTEM_ELEMENT_TYPE = “PATH”

- SYSTEM_ELEMENT  = Name of the path associated with the posted ATC

- SYSTEM_ATTRIBUTE = “ZERO_FATC_NARRATIVE” or “ZERO_NFATC_NARRATIVE”, as applicable.

- START_TIME = Date and time that the posted ATC initially became zero, which may be prior to the date of the zero ATC narrative posting

- STOP_TIME = Date and time when the posted ATC is expected to be greater than zero. If such date and time is unknown, then a null value shall be returned for STOP_TIME.

- ATTRIBUTE_VALUE = “0”

- ATTRIBUTE_UNITS = “MW”

- ANNOTATION = Narrative text (see standard Y.2.3)

Standard Y.2.3  Zero ATC Narrative Text

The narrative text shall include the following information, which shall be delimited by semicolons “;”:

Standard Y.2.3.1  Zero ATC Text String Information

One or more of the following text strings, as applicable, to explain the reason for a zero ATC value:

      - All ATC is reserved

      - CBM increased

      - TRM increased

      - TTC reduced by less than ten percent

      - TTC reduced by ten percent or more

      - Honoring external constraint of entity <abc> (where <abc> is the NERC Reliability Functional Model entity code of the external entity that triggered the change in ATC)

      - Other (use of this text string requires additional explanatory text)

Standard Y.2.3.2 EVENT_ID

The security template EVENT_ID associated with the event that resulted in the posted  ATC change,shall also be included in the zero ATC narrative text, where applicable.

- Standard Z:  ATC Change Narrative

The Transmission Provider shall post on OASIS a brief, but specific, narrative explanation of the reason for a change in the monthly or yearly firm or non-firm ATC value on a constrained Posted Path when the monthly or yearly ATC value changes as a result of a 10 percent or greater change in the related posted TTC or Total Flowgate Capability (TFC).

- Standard Z.1: Frequency and Timing of the ATC Change Narrative

- An ATC change narrative shall be posted when ATC values change according to the following requirements:

- Standard Z.1.1: ATC Change Narrative

The  Transmission Provider shall post an ATC change narrative when a Transmission Provider’s ATC changes as a result of a 10% or more change in the Transmission Provider’s TTC/TFC, as follows (for examples see WEQ 013.X):

a.   a monthly or yearly ATC posted value changes on a constrained Posted Path as a result of a 10 percent or greater change in the TTC posted value, or

b.  where the Available Flowgate Capability (AFC) calculation method is used, a converted monthly or yearly ATC posted value changes on a constrained Posted Path as a result of a 10 percent or greater change in the TFC value.

- Standard Z.1.2:  ATC Change Narrative Timing

The Transmission Provider shall post an ATC change narrative within five (5)  business days of meeting the criteria identified in Z.1.1.

Standard Z.1.3: ATC Change  Narrative Updates

The Transmission Provider shall not update posted ATC change narratives.  This does not preclude the Transmission Provider from posting an additional ATC change narrative should updated information become available.

- Standard Z.2: ATC Change Narrative Posting on OASIS

An ATC change narrative shall be posted for each constrained Posted Path which meets the criteria specified in Standard Z.1.1.

Standard Z.2.1: Use of systemdata

An ATC change narrative shall be posted such that it may be queried and viewed on OASIS using the systemdata template.

Standard Z.2.2: systemdata Information

There shall be separate narrative postings for firm ATC changes and non-firm ATC changes as follows:

- SYSTEM_ELEMENT_TYPE = “PATH”

- SYSTEM_ELEMENT  = Name of the path whose posted ATC changed due to meeting the criteria specified in Z.1.1

- SYSTEM_ATTRIBUTE = “FATC_CHANGE_NARRATIVE” or “NFATC_CHANGE_NARRATIVE”,  as applicable.

- START_TIME = Date and time that the Transmission Provider’s most limiting TTC path’s TTC or TFC flowgate’s TFC changed consistent with the criteria specified in Z.1.1.

- STOP_TIME = Date and time of the end of the reason for the change in the Transmission Provider’s most limiting TTC path’s TTC or TFC flowgate’s TFC, consistent with the criteria specified in Z.1.1, if a reasonable estimate is known at the time of the posting. . If the date and time for the end of the reason for the change is (1) unknown or is indeterminate, (2) beyond the OASIS posting horizon, or (3) the change is permanent then a null value shall be returned for STOP_TIME.

- ATTRIBUTE_VALUE =  The maximum change in TTC/TFC over the start /stop time interval rounded to the nearest percent (e.g., an increase of 13.7% in TTC would be posted as +14% and a decrease of 13.7% in TTC would be posted as -14%)

- ATTRIBUTE_UNITS = “%”

- ANNOTATION = Narrative text (see standard Z.2.3)

- Standard Z.2.3: ATC Change Narrative Text

- An ATC change narrative text shall include the following fields of information, which shall be delimited by semicolons “;”:

Standard Z.2.3.1: Entity Code

The  NERC Reliability Functional Model entity code of the entity whose event triggered the change.

Standard Z.2.3.2: Limiting TTC or TFC

The name of the most limiting TTC path or TFC flowgate on the Transmission Provider’s system contributing to the ATC change.

Standard Z.2.3.3: EVENT_ID

The security template EVENT_ID associated with the event that resulted in the posted  ATC change,shall also be included in the ATC change narrative text, where applicable.

Standard Z.2.3.4: STOP_TIME of Null Value

When the Transmission Provider uses a null value in STOP_TIME, the Transmission Provider shall include in the narrative text one of the following text strings:

- STOP_TIME is indeterminate

- STOP_TIME is beyond the OASIS posting horizon

- Change to TTC/TFC is permanent

Standard Z.2.3.5: ATC Change Text String Information

One or more of the following text strings, as applicable, to explain the reason for the change in underlying TTC or TFC that resulted in the change in ATC::

- Load forecast change

- Generation dispatch change

- Generation forced outage

- Generation scheduled outage

- Generation addition

- Generation return to service

- Generation retirement

- Transmission configuration change

- System element forced outage

- System element planned outage

- System element addition

- System element return to service

- System element retirement

- Aggregate effect in AFC or ETC, but TFC remained unchanged. (This Narrative can be used when there are multiple components from the List above that changed ETC with no one discernable primary cause that effected the ATC by more than 10%. This Narrative may be used by those Entities using the AFC Flowgate Methodology for ATC Calculation due to the complexity of determininga primary cause).

- Addition of a flowgate

- Removal of a flowgate

- Facility rating change, including SOL

- Change in transfer capability allocation

- Honoring external constraint of entity <abc> (where <abc> is the NERC Reliability Functional Model entity code of the external entity that triggered the change in ATC)

- Other (use of this text string requires additional explanatory text)

Corresponding changes should be made to WEQ 013-3.3.1 as shown below:

013-3.3.1	ATC Related Query/Response Requirements

Transmission Providers shall provide applicable data associated with the following SYSTEM_ATTRIBUTE values:

- TTC – Total Transfer Capability

- CBM – Capacity Benefit Margin

- FTRM –  Transmission Reliability Margin for use in FATC

- NFTRM –  Transmission Reliability Margin for use in NFATC

- FATC –Firm Available Transfer Capability

- NFATC –Non-Firm Available Transfer Capability

- ZERO_FATC_NARRATIVE – Narrative when a posted firm monthly or yearly ATC value remains unchanged at a value of zero consistent with WEQ-001-Y

- ZERO_NFATC_NARRATIVE – Narrative when a posted non-firm monthly or yearly ATC value remains unchanged at a value of zero consistent with WEQ-001-Y

- FATC_CHANGE_NARRATIVE – Narrative when a posted firm monthly or yearly ATC changes as a result of a 10 percent change in TTC/TFC consistent with WEQ-001-Z

- NFATC_CHANGE_NARRATIVE – Narrative when a posted non-firm monthly or yearly ATC changes as a result of a 10 percent change in TTC/TFC consistent with WEQ-001-Z

- ATC_ANNOTATION – Annotation for a change in monthly or yearly posted ATC (no longer used replaced by ZERO_FATC_NARRATIVE, ZERO_NFATC_NARRATIVE, FATC_CHANGE_NARRATIVE, NFATC_CHANGE_NARRATIVE)

- {registered} – Provider defined attributes necessary for posting required data

The SYSTEM_ELEMENT_TYPE and SYSTEM_ELEMENT shall be used as follows:

- SYSTEM_ELEMENT_TYPE = PATH

- SYSTEM_ELEMENT = Name of the path associated with the posted ATC component value

Posting of ATC component data and narratives via the systemdata template shall comply with all applicable regulations and business practices.

The following are the systemdata template Data Element requirements for providing ATC component data, i.e., TTC, CBM, FTRM, NFTRM, FATC or NFATC, to the user:

| Data Element | Restriction/Requirement |
| --- | --- |
| POSTING_REF | The unique OASIS identifier assigned to the posting |
| SYSTEM_ELEMENT_TYPE | “PATH” |
| SYSTEM_ELEMENT | Name of the path associated with the posted ATC component value |
| SYSTEM_ATTRIBUTE | “TTC”, “CBM”, “FTRM”, “NFTRM”, “FATC”, or “NFATC” |
| START_TIME | Beginning of interval associated with the posted value |
| STOP_TIME | End of interval associated with the posted value |
| ATTRIBUTE_VALUE | Value of the posted ATC component over the interval of START_TIME/STOP_TIME |
| ATTRIBUTE_UNITS | Units of ATTRIBUTE_VALUE; typically MW |
| ANNOTATION | Optional at discretion of the Provider; typically null |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

The following are the systemdata template Data Element requirements for providing a brief, but specific, narrative explanation of the reason for a change in the monthly or yearly firm or non-firm ATC value on a constrained Posted Path when the monthly or yearly ATC value changes as a result of a 10 percent or greater change in the related posted TTC or Total Flowgate Capability (TFC).

| Data Element | Restriction/Requirement |
| --- | --- |
| POSTING_REF | The unique OASIS identifier assigned to the posting |
| SYSTEM_ELEMENT_TYPE | “PATH” |
| SYSTEM_ELEMENT | Name of the path associated with the posted ATC component value |
| SYSTEM_ATTRIBUTE | ”FATC_CHANGE_NARRATIVE” or “NFATC_CHANGE_NARRATIVE” |
| START_TIME | Date and time that the Transmission Provider’s most limiting TTC path’s TTC or TFC flowgate’s TFC changed consistent with the criteria specified in Z.1.1 |
| STOP_TIME | Date and time of the end of the reason for the change in the Transmission Provider’s most limiting TTC path’s TTC or TFC flowgate’s TFC, consistent with the criteria specified in Z.1.1, if a reasonable estimate is known at the time of the posting. . If the date and time for the end of the reason for the change is (1) unknown or is indeterminate, (2) beyond the OASIS posting horizon, or (3) the change is permanent then a null value shall be returned for STOP_TIME. |
| ATTRIBUTE_VALUE | The maximum change in TTC/TFC over the start /stop time interval rounded to the nearest percent (e.g., an increase of 13.7% in TTC would be posted as +14% and a decrease of 13.7% in TTC would be posted as -14%) |
| ATTRIBUTE_UNITS | % |
| ANNOTATION | Narrative text fields shall be delimited by semicolons “;” (see standard Z.2.3) |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

The following are the systemdata template Data Element requirements for providing a brief, but specific, narrative associated with yearly or monthly ATC values when either the posted firm or non-firm ATC remains unchanged at a value of zero (0) for six (6) months or longer.

| Data Element | Restriction/Requirement |
| --- | --- |
| POSTING_REF | The unique OASIS identifier assigned to the posting |
| SYSTEM_ELEMENT_TYPE | “PATH” |
| SYSTEM_ELEMENT | Name of the path associated with the posted ATC component value |
| SYSTEM_ATTRIBUTE | “ZERO_FATC_NARRATIVE” or “ZERO_NFATC_NARRATIVE” |
| START_TIME | Date and time that the posted ATC initially became zero, which may be prior to the date of the zero ATC narrative posting |
| STOP_TIME | Date and time when the posted ATC is expected to be greater than zero. If such date and time is unknown, then a null value shall be returned for STOP_TIME. |
| ATTRIBUTE_VALUE | 0 |
| ATTRIBUTE_UNITS | “MW” |
| ANNOTATION | Narrative text fields shall be delimited by semicolons “;”(see standard Y.2.3) |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

Corresponding changes should be made to WEQ 003 (OASIS Data Dictionary as shown below:

| 003-0 OASIS DATA DICTIONARY Version 1.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Data Dictionary Element Name | Alias | Field Format : minimum characters {type of ASCII} maximum characters | Restricted Values | Definition of Data Element |
| SYSTEM_ATTRIBUTE | SYSATTR | 0{ALPHANUMERIC}15 | Valid Values: CBM FTRM NFTRM TTC FATC NFATC ATC_ANNOTATION ZERO_FATC_NARRATIVE ZERO_NFATC_NARRATIVE FATC_CHANGE_NARRATIVE NFATC_CHANGE_NARRATIVE ZONE_LOAD_FORECAST SYSTEM_LOAD_FORECAST NATIVE_LOAD_FORECAST ACTUAL_LOAD {Registered} | Type of system data viewed by systemdata template: CBM – Capacity Benefit Margin FTRM – Transmission Reliability Margin for use in FATC NFTRM – Transmission Reliability Margin for use in NFATC TTC – Total Transmission Capability FATC –Firm Available Transmission Capability NFATC – Non-firm Available Transmission Capability ATC_ANNOTATION – Annotation for a change in monthly or yearly posted ATC (no longer used replaced by ZERO_FATC_NARRATIVE, ZERO_NFATC_NARRATIVE, FATC_CHANGE_NARRATIVE, NFATC_CHANGE_NARRATIVE) ZERO_FATC_NARRATIVE – Narrative when a posted firm monthly or yearly ATC value remains unchanged at a value of zero ZERO_NFATC_NARRATIVE – Narrative when a posted non-firm monthly or yearly ATC value remains unchanged at a value of zero FATC_CHANGE_NARRATIVE – Narrative when a posted firm monthly or yearly ATC changes as a result of a 10 percent change in TTC/TFC NFATC_CHANGE_NARRATIVE – Narrative when a posted non-firm monthly or yearly ATC changes as a result of a 10 percent change in TTC/TFC ZONE_LOAD_FORECAST – Forecast of anticipated Load in a metered zone SYSTEM_LOAD_FORECAST – Forecast of total system wide load within a Balancing Area NATIVE_LOAD_FORECAST – Forecast of portion of total system load forecast for native load portion of total system load within a Balancing Area ACTUAL_LOAD – Actual daily peak load values for the metered Balancing Area or metered Zone within the BA posted after the fact {registered} – Provider specific registered name for the data posted |

Corresponding changes should be made to WEQ 002-4.3.4.4 as shown below:

002-4.3.4.4	System Data (systemdata)

The System Data (systemdata) Template is used to query specific, time varying data that is posted on a SYSTEM_ELEMENT basis.  The information specified in SYSTEM_ELEMENT is dependent on SYSTEM_ELEMENT_TYPE.  That is, SYSTEM_ELEMENT_TYPE defines how to interpret the value returned for SYSTEM_ELEMENT. The various types of system elements that may be specified with this template are described in the OASIS Data Dictionary, WEQ 003.

The SYSTEM_ATTRIBUTE Data Element defines the type of information returned in ATTRIBUTE_VALUE Data Element.  The list of restricted values for SYSTEM_ATTRIBUTE are defined in the OASIS Data Dictionary, WEQ 003.  The relationships between and usage of SYSTEM_ELEMENT, SYSTEM_ELEMENT_TYPE and SYSTEM_ATTRIBUTE are described in the OASIS Implementation Guide, WEQ 013-3.3.

Transmission Providers obligated to post values for one or more of the defined SYSTEM_ATTRIBUTEs on specific transmission paths over time (e.g., hourly, then daily, etc.) as called forth in FERC regulations shall return these posted values via the systemdata Template. If SYSTEM_ATTRIBUTE is omitted in the query, then all attributes defined by the transmission provider are returned, subject to the other query attributes constraints. A given SYSTEM_ATTRIBUTE may take on only one value at any given point in time. Note that TTC and ATC information may also be viewed using the transoffering Template at the Transmission Provider's discretion. Offers of service posted by Primary Providers as viewed with the transoffering Template should reflect the applicable ATC(s) posted via systemdata in the CAPACITY Data Element.

Narratives related to specific changes in posted TTC/TFC and/or ATC values shall be denoted by the SYSTEM_ATTRIBUTE of ZERO_FATC_NARRATIVE, ZERO_NFATC_NARRATIVE, FATC_CHANGES_NARRATIVE, or NFATC_CHANGE_NARRATIVE, and the specific narrative corresponding to the TTC/TFC/ATC postings shall be placed in the ANNOTATION Data Element.  The specific values for TTC and/or ATC are posted under the systemdata template for the SYSTEM_ATTRIBUTES of TTC, FATC and NFATC, etc.

Template: systemdata

- Query

SYSTEM_ELEMENT_TYPE*

SYSTEM_ATTRIBUTE*

START_TIME

STOP_TIME

TIME_OF_LAST_UPDATE

- Response (acknowledgment)

POSTING_REF

SYSTEM_ELEMENT_TYPE

SYSTEM_ATTRIBUTE

START_TIME

STOP_TIME

ATTRIBUTE_VALUE

ATTRIBUTE_UNITS

ANNOTATION

TIME_OF_LAST_UPDATE

The following Appendix should be added to WEQ 013 (OASIS Implementation Guide):

013- A		Appendix A  – Example for WEQ-001-Z: ATC Change Narrative

FATC_CHANGE_NARRATIVE or NFATC_CHANGE_NARRATIVE Posting Options (See Standard WEQ-001-Z)

Base Example (TTC changed):

TP1 models 5 network elements with 4 discreet commercial service points A, B, C, D arranged as follows:

TP2 models 1 network element with 1 discreet commercial service point C arranged as follows with TP1:

A

B

D

C

X

Y

K

Given:

- TTC changes by max of -13% in both directions on modeled element X-Y from June 1, 2008 to October 1, 2008.

- TTC was changed due to a planned transmission outage on the ABC system, which is posted under security event id 12345

- The capacity will be restored to the starting value at the end of the outage (+13% change).

- Both Firm and Non-Firm ATCs will be changed due to this transmission outage as shown in the following table:

| Posted Path | Path Constrained | Monthly Firm ATC Change | Monthly Non-Firm ATC Change |
| --- | --- | --- | --- |
| A-B | No | No | No |
| B-A | No | No | No |
| A-C | Yes | July | July |
| C-A | Yes | Jun, Sept | Jun, Sept |
| A-D | No | July | July |
| D-A | No | Jun-Oct | Jun-Oct |

Post narrative only for those commercial paths where the element whose TTC/TFC changed by 10% or more AND the element was actually the limiting element at ANY time over the interval that TTC was impacted.   Assume that X-Y is limiting ATC only for path A-C in July and C-A in June and September.  FATC_CHANGE_NARRATIVE and, in this case,  NFATC_CHANGE_NARRATIVE would be posted for each dependent constrained commercial path for the duration of the impact on X-Y regardless of the actual interval of time that an ATC was actually impacted.  Here are the required postings:

Note: TP2 has no obligation to create an annotation unless the TP Honors the external constraint of entity <abc> in which the annotation would look like the following: ABC;X-Y;12345;; Honoring external constraint of entity ABC.

The following Appendix should be added to WEQ 013 (OASIS Implementation Guide):

013- B	Appendix B  – Example for WEQ-001-Y: Zero Change Narrative

| ZERO_FATC_NARRATIVE or ZERO_NFATC_NARRATIVE Posting Options (See Standard WEQ-001-Y) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WEQ-001-Y.1.1: Initial Zero ATC Narrative |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A Transmission Provider shall post a zero ATC narrative when ATC for a constrained Posted Path meets any of the following criteria: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a. The monthly ATC value for a given month has been posted at a value of zero for the past six consecutive months |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b. The monthly ATC is posted at a value of zero for a duration of six consecutive months; where that six-month duration could span past, present, and future months. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| c. The yearly ATC is posted at a value of zero for one or more future years |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WEQ-001-Y.1.2: Successive ZERO ATC Narrative |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Once the initial zero ATC narrative has been posted, a Transmission Provider shall post successive narratives when the posted ATC for a constrained Posted Path remains zero and meets either of the following criteria: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a. The conditions leading to requiring the posting of such narrative have changed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b. The conditions leading to requiring the posting of such narrative have not changed for six consecutive months and are still applicable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Example 1: WEQ-001-Y.1.1.a The monthly ATC value for a given month has been posted at a value of zero for the past six consecutive months |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Posted on |  | Jun-08 |  |  |  |  |  |  |  |  |  |  |  |
|  | Nov-07 | Monthly ATC | 100 |  |  |  |  |  |  |  |  |  |  |  |
|  | Dec-07 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Jan-08 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Feb-08 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Mar-08 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Apr-08 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | May-08 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Jun-08 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Narratives: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in May 2008, when a value of zero has been posted for six months. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Assuming the reason has not changed, no narrative is required in June 2008. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Example 2: WEQ-001-Y.1.1.b The monthly ATC is posted at a value of zero for a duration of six consecutive months; where that six-month duration could span past, present, and future months. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Posted on |  | Jan-08 | Feb-08 | Mar-08 | Apr-08 | May-08 | Jun-08 | Jul-08 | Aug-08 |  |  |  |  |
|  | Nov-07 | Monthly ATC | 400 | 400 | 200 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |
|  | Dec-07 | Monthly ATC | 400 | 0 | 0 | 0 | 0 | 0 | 0 | 100 |  |  |  |  |
|  | Jan-08 | Monthly ATC | 400 | 0 | 0 | 0 | 0 | 0 | 0 | 100 |  |  |  |  |
|  | Feb-08 | Monthly ATC | 400 | 100 | 0 | 0 | 0 | 0 | 0 | 100 |  |  |  |  |
|  | Mar-08 | Monthly ATC | 400 | 100 | 0 | 0 | 0 | 0 | 0 | 100 |  |  |  |  |
|  | Apr-08 | Monthly ATC | 400 | 100 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |
|  | Narratives: (values highlighted in grey are data that occurs in the past and are not 're-posted') |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in December 2007 since there is a 6-month block of zeros in the future. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Assuming the reason has not changed, no narrative is required in January 2008. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | No narrative is required for February, 2008 since there is no 6-month block of zeros. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | No narrative is required for March 2008 since there is no 6-month block of zeros. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A new narrative is required for April 2008 since there is a 6-month block of zeros with 5 months in the future and one month in the past. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Example 3: WEQ-001-Y.1.1.b The monthly ATC is posted at a value of zero for a duration of six consecutive months; where that six-month duration could span past, present, and future months. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Posted on |  | Jan-08 | Feb-08 | Mar-08 | Apr-08 | May-08 | Jun-08 | Jul-08 | Aug-08 | Sep-08 | Oct-08 | Nov-08 | Dec-08 |
|  | Nov-07 | Monthly ATC | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
|  | Dec-07 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Jan-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Feb-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Mar-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Apr-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | May-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Jun-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Jul-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Aug-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Sep-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Oct-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Nov-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Dec-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Narratives: (values highlighted in grey are data that occurs in the past and are not 're-posted') |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in December 2007 since there is a 6-month block of zeros in the future. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Assuming the reason has not changed, no narrative is required in Jan, Feb, Mar, Apr or May 2008. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in June 2008 since there is a requirement to update the ATC Narrative once every 6-months. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Assuming the reason has not changed, no narrative is required in Jul, Aug, Sep, Oct or Nov 2008. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in December 2008 since there is a requirement to update the ATC Narrative once every 6-months. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Example 4: WEQ-001-Y.1.1.c The yearly ATC is posted at a value of zero for one or more future years |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Posted on |  | 2008 | 2009 |  |  |  |  |  |  |  |  |  |  |
|  | May-07 | Yearly ATC | 0 | 0 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-07 | Yearly ATC | 300 | 400 |  |  |  |  |  |  |  |  |  |  |
|  | Jul-07 | Yearly ATC | 0 | 400 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-07 | Yearly ATC | 300 | 400 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-07 | Yearly ATC | 0 | 400 |  |  |  |  |  |  |  |  |  |  |
|  | Oct-07 | Yearly ATC | 300 | 400 |  |  |  |  |  |  |  |  |  |  |
|  | Narratives: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in May 2007. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in July 2007. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in Sept 2007. |  |  |  |  |  |  |  |  |  |  |  |  |  |

- 4.  SUPPORTING DOCUMENTATION

- a.  Description of Request:

- In the FERC Order 890 under Docket Nos. RM05-17-000 and RM05-25-000 dated February 16, 2007, the Commission made the following conclusions about ATC/TTC Narratives:

- The Commission adopts the NOPR proposal, with the modifications discussed below, to require that the transmission provider post a brief, but specific, narrative explanation of the reason for a change in monthly and yearly ATC values on a constrained path.  Rather than requiring a narrative when a monthly or yearly ATC value changes as a result of transactions being reserved, service ending, or the TTC estimate for the path changing by more than 10 percent, we will require a narrative when a monthly or yearly ATC value changes only as a result of a 10 percent change in TTC.  This will reduce the number of ATC changes for which a narrative will be required and address concerns that the new requirement unduly burdens transmission providers.  Any remaining burden is justified by the benefit to transmission customers of receiving timely information regarding changes in TTC that result in changes to ATC.  In addition, we adopt NAESB’s suggestion that posted information include the (1) specific events which gave rise to the change and (2) new values for ATC on that path (as opposed to all points on the network).

- We reject calls for delays prior to posting data.  While commenters allege the possibility of granting others a competitive advantage through the release of “market-sensitive” data, they have proffered no evidence to support the allegation of potential harm.

- We do require, as suggested in the NOPR, a narrative with regard to monthly or yearly ATC values when ATC remains unchanged at a value of zero for a significant period, and will set that period at six months or longer.  This information will be valuable to customers and regulators in assessing the ability of a transmission provider’s facilities to meet existing service requests.  The information also will provide assurance to customers that the transmission provider is diligent in regularly evaluating ATC on all paths, monitoring persistent constraints and addressing them in its planning processes.

- In the FERC Order 890-A under Docket Nos. RM05-17-001, 002 and RM05-25-001, 002 dated December 28, 2007, the Commission made the following conclusions about ATC/TTC Narratives:

123.	The Commission affirms the decision in Order No. 890 to require transmission providers to post a brief, but specific, narrative explanation of the reason for a change in monthly or yearly ATC values on a constrained path as a result of a change in TTC of 10 percent or more.  As the Commission explained, this will limit the number of ATC changes for which a narrative will be required.

124.	We believe that E.ON U.S. overestimates the burden of complying with this requirement.  Since TTC standardization is ongoing, it is impossible to identify with precision the steps that will need to be taken to comply with the posting requirement.  The appropriate forum to raise concerns regarding the burden of particular TTC calculation requirements is in the NAESB standards development process.  In any event, we would expect that the posting of narratives for changes in monthly and yearly ATC values as a result of a 10 percent change in TTC will be triggered mainly by topology changes resulting from transmission lines and generator in-service status, as well as new facilities additions, that are reported on OASIS.

125.	We clarify in response to Southern that transmission providers do not need to list each and every circumstance or occurrence that impacts TTC values from the previous month or year and, instead, may list the primary events that give rise to the update.  Again, we expect that TTC changes will generally result from topology changes and, therefore, the primary reasons for an update would be changes in schedules of transmission or generation additions, prolonged outages, or changes in maintenance schedules causing a TTC change of 10 percent.  We agree with Southern that the transmission provider should post these narrative explanations on OASIS via a template and data element that is to be defined by NAESB.  We direct transmission providers, working through NAESB, to develop the OASIS functionality necessary for such postings.  Pending completion of this work by NAESB, we direct transmission providers to post these narrative explanations as comments on OASIS.

126.	We deny TDU Systems’ request to change the triggering requirement to a 10 percent decrease in ATC.  In Order No. 890, the Commission relaxed the ATC narrative reporting requirements proposed in the NOPR due to concerns that the posting of those narratives would become burdensome.  We believe the Commission struck the right balance by requiring the posting of narratives only when there is a change in TTC of 10 percent or more and disagree that more limited postings defeats the purpose of the posting obligation.

127.	In response to PJM, we reiterate that all transmission providers must comply with this posting requirement.  Transmission providers using an AFC calculation method that does not base changes in ATC on changes in TTC may comply with this requirement by posting narrative explanations of the reasons for changes in AFC values as a result of changes in AFC inputs that cause ATC or TTC to change by 10 percent or more.  We direct each transmission provider that employs the AFC calculation methodology to provide a statement in the compliance filing required in section II.C describing how the narrative is derived for ATC/TTC postings or, if such information was provided in a prior compliance filing, a reference to that filing.

128.	We also deny TDU Systems’ request to require transmission providers to post a narrative explanation any time ATC values remain at zero for a day for daily service, two business days for weekly service, five business days for monthly or yearly service.  The Commission concludes that a six-month trigger for monthly or yearly ATC values more appropriately balances the benefits of increased transparency for the Commission and customers against the burden on transmission providers to make such postings.  If the frequency of these postings proves inadequate, the Commission can revisit this requirement in a future order.

- WEQ 2008 Annual Plan Item 2.b.v.3

Business practice standards that will set forth how transmission providers will post “explanations of the reason for a change in monthly and yearly ATC values on a constrained path.”  The standards will include a requirement that that the transmission provider posts the reason for the change in a narrative form.  The posted information will include “the (1) specific events which gave rise to the change and (2) new values for ATC on that path (as opposed to all points on the network).”

- The business practices developed to address this item will not require to coordination with the NERC Order 890 reliability standards development

- WEQ 2008 Annual Plan Item 2.a.i.3

Annotations For ATC Business Practice Standards associated with S&CP  Requirements completed in 2(a)(i)(1)

- b.  Description of Recommendation:

- c.  Business Purpose:

- d.  Commentary/Rationale of Subcommittee(s)/Task Force(s):

The guiding principal addressed by the ESS/ITS/BPS for Zero ATC Narratives is reflected in the following statement:

The Transmission Provider shall post a Zero ATC Narrative when the ATC for any constrained Posted Path meets the criteria specified in WEQ-001-Y.1.  Once the condition(s) leading to the unavailability of ATC triggering the need for posting a Zero ATC Narrative has been posted, the Transmission Provider is only required to update the Zero ATC Narrative every six months if those conditions have not changed.  The Transmission Provider shall post a new Zero ATC Narrative if the conditions leading to requiring the posting of such narrative have changed at the time that change in conditions is detected.

Please review the ESS/ITS/BPS Subcommittee meeting minutes from the following days:

November 26, 2007

December 3, 2007

December 7, 2007

December 10, 2007

December 17, 2007

January 3-4, 2008

January 11, 2008

January 16-17, 2008

January 18, 2008

January 28, 2008

February 8, 2008

February 13, 2008

Please review the Motions Document: http://naesb.org/pdf3/weq_atc_afc021308a5.doc