FORMAL COMMENTS

Quadrant:	Wholesale Electric Quadrant

Recommendation: 	Number: 2008 AP Item 2.b.v.3 & 2008 AP Item 2.a.i.3

Title:  ATC/TTC Narratives

Submitted By:	Standards Review Subcommittee

Date:	March 29, 2008

Under the Standards Review Subcommittee (SRS) Scope of Work, which was approved by the SRS on March 6, 2008, the SRS agreed to review recommendations and if subcommittee deemed appropriate, they would submit advisory comment to the Executive Committee for consideration.  As stated in the Scope of Work these comments are “not intended to change the scope of the Business Practices or recommendation, but to provide consistency and uniformity across all WEQ Business Practices.”

The SRS is requesting the Executive Committee consider the following redlined advisory comments in their review of this recommendation.  An “Explanation for change” is included with each proposed change.

Z.2.3.5	…

One or more of the following text strings, as applicable, to explain the reason for the change in underlying TTC or TFC that resulted in the change in ATC:

…

Explanation for change: Delete second colon.

013-3.3.1	…

The following are the systemdata template Data Element requirements for providing a brief, but specific, narrative explanation of the reason for a change in the monthly or yearly firm or non-firm ATC value on a constrained Posted Path when the monthly or yearly ATC value changes as a result of a 10 percent or greater change in the related posted TTC or TFC.

…

Explanation for change: The full name of the acronym will be documented earlier in the Business Practices Book under WEQ-001 Standard Z: ATC Change Narrative.

002-4.3.4.4	…

Transmission Providers obligated to post values for one or more of the defined SYSTEM_ATTRIBUTEs on specific transmission paths over time (e.g., hourly, then daily, etc.) as called forth in FERC regulations shall return these posted values via the systemdata Template. If SYSTEM_ATTRIBUTE is omitted in the query, then all attributes defined by the Transmission Provider are returned, subject to the other query attributes constraints. A given SYSTEM_ATTRIBUTE may take on only one value at any given point in time. Note that TTC and ATC information may also be viewed using the transoffering Template at the Transmission Provider's discretion. Offers of service posted by Primary Providers as viewed with the transoffering Template should reflect the applicable ATC(s) posted via systemdata in the CAPACITY Data Element.

…

Explanation for change: While making changes to this section of the Business Practice it is suggested that we capitalize Transmission Provider to be consistent with other occurrence of the term.

013-A 	…

Transmission Provider 1 (TP1) models 5 network elements with 4 discreet commercial service points A, B, C, D arranged as follows:

Transmission Provider 2 (TP2) models 1 network element with 1 discreet commercial service point C arranged as follows with TP1:

…

Explanation for change: Describe an acronym the first time it is used.

013-B 	…

| ZERO_FATC_NARRATIVE or ZERO_NFATC_NARRATIVE Posting Options (See Standard WEQ-001-Y) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Example 1: WEQ-001-Y.1.1.a The monthly ATC value for a given month has been posted at a value of zero for the past six consecutive months |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Posted on |  | Jun-08 |  |  |  |  |  |  |  |  |  |  |  |
|  | Nov-07 | Monthly ATC | 100 |  |  |  |  |  |  |  |  |  |  |  |
|  | Dec-07 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Jan-08 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Feb-08 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Mar-08 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Apr-08 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | May-08 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Jun-08 | Monthly ATC | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Narratives: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in May 2008, when a value of zero has been posted for six months. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Assuming the reason has not changed, no narrative is required in June 2008. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Example 2: WEQ-001-Y.1.1.b The monthly ATC is posted at a value of zero for a duration of six consecutive months; where that six-month duration could span past, present, and future months. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Posted on |  | Jan-08 | Feb-08 | Mar-08 | Apr-08 | May-08 | Jun-08 | Jul-08 | Aug-08 |  |  |  |  |
|  | Nov-07 | Monthly ATC | 400 | 400 | 200 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |
|  | Dec-07 | Monthly ATC | 400 | 0 | 0 | 0 | 0 | 0 | 0 | 100 |  |  |  |  |
|  | Jan-08 | Monthly ATC | 400 | 0 | 0 | 0 | 0 | 0 | 0 | 100 |  |  |  |  |
|  | Feb-08 | Monthly ATC | 400 | 100 | 0 | 0 | 0 | 0 | 0 | 100 |  |  |  |  |
|  | Mar-08 | Monthly ATC | 400 | 100 | 0 | 0 | 0 | 0 | 0 | 100 |  |  |  |  |
|  | Apr-08 | Monthly ATC | 400 | 100 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |
|  | Narratives: (values highlighted in grey are data that occurs in the past and are not 're-posted') |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in December 2007 since there is a 6-month block of zeros in the future. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Assuming the reason has not changed, no narrative is required in January 2008. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | No narrative is required for February 2008 since there is no 6-month block of zeros. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | No narrative is required for March 2008 since there is no 6-month block of zeros. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A new narrative is required for April 2008 since there is a 6-month block of zeros with 5 months in the future and one month in the past. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Example 3: WEQ-001-Y.1.1.b The monthly ATC is posted at a value of zero for a duration of six consecutive months; where that six-month duration could span past, present, and future months. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Posted on |  | Jan-08 | Feb-08 | Mar-08 | Apr-08 | May-08 | Jun-08 | Jul-08 | Aug-08 | Sep-08 | Oct-08 | Nov-08 | Dec-08 |
|  | Nov-07 | Monthly ATC | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
|  | Dec-07 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Jan-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Feb-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Mar-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Apr-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | May-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Jun-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Jul-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Aug-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Sep-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Oct-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Nov-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Dec-08 | Monthly ATC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Narratives: (values highlighted in grey are data that occurs in the past and are not 're-posted') |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in December 2007 since there is a 6-month block of zeros in the future. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Assuming the reason has not changed, no narrative is required in Jan, Feb, Mar, Apr or May 2008. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in June 2008 since there is a requirement to update the ATC Narrative once every 6 months. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Assuming the reason has not changed, no narrative is required in Jul, Aug, Sep, Oct or Nov 2008. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in December 2008 since there is a requirement to update the ATC Narrative once every 6 months. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Example 4: WEQ-001-Y.1.1.c The yearly ATC is posted at a value of zero for one or more future years |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Posted on |  | 2008 | 2009 |  |  |  |  |  |  |  |  |  |  |
|  | May-07 | Yearly ATC | 0 | 0 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-07 | Yearly ATC | 300 | 400 |  |  |  |  |  |  |  |  |  |  |
|  | Jul-07 | Yearly ATC | 0 | 400 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-07 | Yearly ATC | 300 | 400 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-07 | Yearly ATC | 0 | 400 |  |  |  |  |  |  |  |  |  |  |
|  | Oct-07 | Yearly ATC | 300 | 400 |  |  |  |  |  |  |  |  |  |  |
|  | Narratives: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in May 2007. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in July 2007. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A narrative should be posted in Sept 2007. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Explanation for change: The standards documented in WEQ-001-Y.1.1 and WEQ-001-Y.1.2 should not be duplicated in an appendix in another Business Practice (WEQ-013).