| 003-0 OASIS DATA DICTIONARY Version 1.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Data Dictionary Element Name | Alias | Field Format : minimum characters {type of ASCII} maximum characters | Restricted Values | Definition of Data Element |
| AFFILIATE_FLAG | AFFLAG | 2{ALPHANUMERIC}3 | Valid Values YES NO | Set to YES if customer is an affiliate of the provider |
| ANC_SERVICE_POINT | ANCPOINT | 0{ALPHANUMERIC}12 | Free form text, null can be used if there is no ancillary service point other than the control area | Name of ancillary service point within a control area, such as a POR/POD/SOURCE/SINK from which the ancillary service is provided |
| ANNOTATION | ANNOTATE | 0{ALPHANUMERIC}1000 | Free-form text | Additional notes, comments or other descriptive information with the posted event. |
| AS_TYPE | ASTYPE | 1{ALPHANUMERIC}20 | Valid Values SC RV RF EI SP SU GI DT TL BS {Registered} | SC- Scheduling, system Control and Dispatch RV - Reactive supply and Voltage Control RF- Regulation and Frequency response EI - Energy Imbalance SP - Spinning Reserve SU - Supplemental Reserve GI – Generator Imbalance DT - Dynamic Transfer TL - Real power Transmission Loss BS - System Black Start capability {Registered} must be registered with www.tsin.com and listed in the ANCSERV Template |
| ANC_SVC_LINK | ANCSVCLINK | 0{ALPHANUMERIC}300 | Formatted string as follows: SC:(AA[:xxx[:yyy[:nnn] ]]); RV:(AA[:xxx[:yyy[:nnn ]]]); RF:(AA[:xxx[:yyy[:nnn] ]]); EI:(AA[:xxx[:yyy[:nnn]] ]); SP:(AA[:xxx[:yyy[:nnn] ]]); SU:(AA[:xxx[:yyy[:nnn] ]]); GI:(AA[:xxx[:yyy[:nnn] ]]); DT:(AA[:xxx[:yyy[:nnn] ]]); TL:(AA[:xxx[:yyy[:nnn] ]]); BS:(AA[:xxx[:yyy[:nnn] ]]); {Registered}:(AA[:xxx[ :yyy[:nnn]]]) | The method for linking ancillary services to a transmission service request. The provider and capacity of each ancillary service is identified using the formatted string: SC:(AA[:xxx[:yyy[:nnn]]]); RV:(AA[:xxx[:yyy[:nnn]]]); RF:(AA[:xxx[:yyy[:nnn]]]); EI:(AA[:xxx[:yyy[:nnn]]]); SP:(AA[:xxx[:yyy[:nnn]]]); SU:(AA[:xxx[:yyy[:nnn]]]); GI:(AA[:xxx[:yyy[:nnn]]]); DT:(AA[:xxx[:yyy[:nnn]]]); TL:(AA[:xxx[:yyy[:nnn]]]); BS:(AA[:xxx[:yyy[:nnn]]]); {Registered}:(AA[:xxx[:yyy[:nnn]]]) where AA is the appropriate PRIMARY_PROVIDER_CODE, SELLER_CODE, or CUSTOMER_CODE, and represents the company providing the ancillary services. "AA" may be unspecified for "xxx" type identical to "FT", in which case the ":" character must be present and precede the "FT" type. If multiple "AA" terms are necessary, then each "AA" grouping will be enclosed within parenthesis, with the overall group subordinate to the AS_TYPE specified within parenthesis and where xxx represents either: - "FT" to indicate that the Customer will determine ancillary services at a future time, or |
| ANC_SVC_LINK (cont.) |  |  |  | - "SP" to indicate that the Customer will self-provide the ancillary services, or - "RQ" to indicate that the Customer is asking the OASIS Node to initiate the process for making an ancillary services reservation with the indicated Provider or Seller on behalf of the Customer. The Customer must then continue the reservation process with the Provider or Seller. If the transmission services request is for preconfirmed service, then the ancillary services shall also be preconfirmed, or - "AR" to indicate an assignment reference number sequence follows.The terms "yyy" and "nnn" are subordinate to the xxx type of "AR". yyy represents the ancillary services reservation number (ASSIGNMENT_REF) and nnn represents the capacity of the reserved ancillary services. Square brackets are used to indicated optional elements and are not used in the actual linkage itself. Specifically, the :yyy is applicable to only the "AR" term and the :nnn may optionally be left off if the capacity of ancillary services is the same as for the transmission services, and optionally multiple ancillary reservations may be indicated by additional (xxx[:yyy[:nnn]]) enclosed within parenthesis. If no capacity amount is indicated, the required capacity is assumed to come from the ancillary reservations in the order indicated in the codes, on an "as-needed" basis. |
| ANC_SVC_REQ | ANCSVCREQ | 0{ALPHANUMERIC}100 | SC:{M,R,O,U}; RV:{M,R,O,U}; RF:{M,R,O,U}; EI:{M,R,O,U}; SP:{M,R,O,U}; SU:{M,R,O,U}; GI:{M,R,O,U}; DT:{M,R,O,U}; TL:{M,R,O,U}; BS:{M,R,O, U}; {registered}:{M,R,O,U } | Ancillary services required for a transmission services offering. The appropriate letter {M,R,O,U} will be assigned to each of the six Proforma Ferc ancillary services (see AS_TYPE), where the letters mean the following: · (M) Mandatory, which implies that the Primary Provider must provide the ancillary service · (R) Required, which implies that the ancillary service is required, but not necessarily from the Primary Provider · (O) Optional, which implies that the ancillary service is not necessarily required, but could be provided · (U) Unknown, which implies that the requirements for the ancillary service are not known at this time |
| ASSIGNMENT_REF | AREF | 1{ALPHANUMERIC}12 | Unique value | A unique reference number assigned by a Transmission Information Provider to provide a unique record for each transmission or ancillary service request. A single transmission or ancillary service request will be over a contiguous time period, i.e. from a START_TIME to an STOP_TIME. |
| ATTRIBUTE_UNITS | ATTRUNITS | 1{ALPHANUMERIC}20 | Free form text | System data attribute units |
| ATTRIBUTE_VALUE | ATTRVALUE | 1{NUMERIC}12 | Real number | System data attribute value |
| BID_PRICE | BIDPR | 1{NUMERIC}5 + "." + 2{NUMERIC}4 | Positive number with 2 to 4 decimals | The current bid price of a Service in dollars and cents. Used by Customers to designate a price being bid. |
| CAPACITY | CAP | 0{NUMERIC}12 | Number in units of MW | Transfer capability is the measure of the ability of the interconnected electric system to readily move or transfer power from one area to another over all transmission lines (or paths) between those areas under specified system conditions. In this context "area" may be an individual electric system, powerpool, control area, subregion, or NERC region or portion thereof. |
| CAPACITY_AVAILABLE | CAPAVAIL | 0{NUMERIC}12 | Non-negative number in units of MW | Amount of transmission capacity available after all the reductions are applied to CAPACITY_GRANTED over the time interval |
| CAPACITY_CURTAILED | CAPCUR | 1{NUMERIC}12 | Non-negative number in units of MW | The amount of transfer capability curtailed by the Primary provider for emergency reasons. |
| CAPACITY_GRANTED | CAPGRNT | 0{NUMERIC}12 | Non-negative number in units of MW | The amount of capacity offered/granted by the seller equal to or less than CAPACITY_REQUESTED by the TC. |
| CAPACITY_REDUCED | CAPREDU | 0{NUMERIC}12 | Negative number in units of MW | Amount of transmission capacity reduced |
| CAPACITY_REQUES TED | CAPREQ | 0{NUMERIC}12 | Non-negative number in units of MW | Transmission capacity requested by the Transmission Customer (TC) |
| CAPACITY_USED | CAPUSED | 0{NUMERIC}12 | Non-negative number in units of MW | Reflects the peak MW amount of the reservation used to support the scheduled transaction |
| CATEGORY | CAT | 0{ALPHANUMERIC}25 | Valid name from CATEGORY in LIST Template | A name to be used to categorize messages. Valid names would include: , Want-Ad, Curtailment, Outage, OASIS_Maintenance_Outage |
| CEILING_PRICE | CEILPR | 1{NUMERIC}5 + "." + 2{NUMERIC}4 | Positive number with 2 to 4 decimals | Ceiling price of the Service as entered by the Transmission Provider. |
| COLUMN_HEADERS | HEADERS | 1{ALPHANUMERIC} Limited to all the elements names in one Template | Headers separated by commas. Limited to valid Template element names. Must use full element name and not alias. | Example: COLUMN_HEADERS=PATH_NAME,POI NT_OF_RECEIPT,POINT_OF_DELIVERY,SOURCE,SINK |
| COMPETING_REQU EST_FLAG | COMPREQ | 1{ALPHANUMERIC}1 | Valid Values Y N | If "Y", indicates there is one or more competing requests for this reservation. The competing request AREFs are listed in the SELLER_COMMENTS |
| CONTINUATION_FLAG | CONT | 1{ALPHANUMERIC}1 | Valid Values Y N | Indicates whether or not this record is a continuation from the previous record |
| CONTROL_AREA | AREA | 1{ALPHANUMERIC}20 | Valid name of a control area | A part of the power system with metered tie lines and capable of matching generation and load while meeting scheduled interchange. Location of Ancillary Services is my CONTROL_AREA. |
| CURTAILMENT_OPT IONS | CUROPT | 0{ALPHANUMERIC}80 | Free form text | Customer options, if any, to avoid curtailment |
| CUSTOMER_CODE | CUST | 1{ALPHANUMERIC}6 | Unique value, registered on TSIN.COM | Any entity (or its designated agent) that is eligible to view OASIS information, to execute a service agreement, and/or to receive transmission service. |
| CUSTOMER_COMM ENTS | CUSTCOM | 0{ALPHANUMERIC}255 | Free form text | Informative text. For information to be communicated between the customer and seller. |
| CUSTOMER_DUNS | CUSTDUNS | 9{NUMERIC}9 | Unique DUNS number | Unique DUNS number for a Customer |
| CUSTOMER_EMAIL | CUSTEMAIL | 1{ALPHANUMERIC}25 | Valid Internet E-Mail address | Internet E-Mail address of Customer contact person |
| CUSTOMER_FAX | CUSTFAX | 14{ALPHANUMERIC}20 | Area code and telephone number, plus any extensions (aaa)-nnn-nnnn xnnnn | FAX phone number of Customer contact person |
| CUSTOMER_NAME | CUSTNAME | 1{ALPHANUMERIC}25 | Free form text | Name of Customer contact person |
| CUSTOMER_PHONE | CUSTPHON | 14{ALPHANUMERIC}20 | Area code and telephone number, plus any extensions (aaa)-nnn-nnnn xnnnn | Telephone of Customer contact person |
| DATA_ROWS | ROWS | 1{NUMERIC}unlimited | Positive Number | Number of records (rows) of data exclusive of header information that are to be uploaded or downloaded in a file. |
| DATE_TIME_EFFECTIVE | TIMEEFCT | 16{ALPHANUMERIC}16 | Valid date and time in seconds yyyy+mo+dd+hh +mm+ss+tz | Date and time a message or service offer is in effect |
| DEAL_REF | DREF | 0{ALPHANUMERIC}12 | Unique value, Assigned by Customer | The unique reference assigned by a Customer to two or more service purchases to identify each of them as related to others in the same power service deal. These requests may be related to each other in time sequence through a single Provider, or as a series of wheels through multiple Providers, or a combination of both time and wheels. The User uses the DEAL_REF to uniquely identify a combination of requests relating to a particular deal. |
| DISCRETION_DESCRIPTION | DISCDESC | 0{ALPHANUMERIC}1000 | Free form text | A detailed description of the discretion being reported |
| ELEMENT_NAME | ELEMENT | 1{ALPHANUMERIC}40 | Valid Template element name | Template element name as indicated in data dictionary |
| EMPLOYEE_NAME | EMPNAME | 1{ALPHANUMERIC}25 | Free form text | Name of person who is transferring from one position to another |
| ERROR_MESSAGE | ERROR | 1{ALPHANUMERIC}250 | Free form text | Error message related to a RECORD_STATUS or REQUEST_STATUS |
| EVENT_ID | EVENTID | 0{ALPHANUMERIC}25 | Free form text | The EVENT_ID Data Element is any regional or interconnection-wide recognized security event identifier for events that are of greater scope than those administered locally by the Provider (e.g., a NERC Security Coordinator assigned identifier corresponding to a particular implementation of the NERC TLR procedure). |
| FACILITY_CLASS | FACLASS | 0{ALPHANUMERIC}25 | Free form text, for example: TRANSFORMER, LINE, FLOWGATE Or as defined in the LIST Template | Type of limiting device such as ‘transformer’, ‘line’ or ‘flowgate’ |
| FACILITY_LIMIT_TYPE | FACLIMTYP | 0{ALPHANUMERIC}25 | thermal, stability, voltage or defined in LIST Template | For example: thermal, stability, voltage |
| FACILITY_LOCATION | FACLOC | 0{ALPHANUMERIC}8 | Free form text, for example: INTERNAL EXTERNAL Or as defined in the LIST Template | Location of facility that caused the interruption, either internal to the TP or external to the TP grid |
| FACILITY_NAME | FACNAME | 0{ALPHANUMERIC}100 | Free form text | Name of facility, such as name of path or name of flowgate |
| FORMER_COMPANY | FORMCO | 1{ALPHANUMERIC}25 | Free form text | Former company of the person who is transferring |
| FORMER_DEPARTM ENT | FORMDEPT | 1{ALPHANUMERIC}52 | Free form text | Former department of the person who is transferring |
| FORMER_POSITION | FORMPOS | 1{ALPHANUMERIC}25 | Free form text | Former position held by the person who is transferring |
| GCA_CODE | GCA | 1{ALPHANUMERIC}4 | Registered control area company code | Generator Control Area Code. Information from Tag |
| IMPACTED | IMPACTED | 0{NUMERIC}4 | Number | Indicates whether the reservation has been impacted by another reservation. For an original reservation this counter is 0. This counter is incremented by 1 by TSIP on the parent request when its ASSIGNMENT_REF is entered in any other reservation’s REASSIGNED_REF or RELATED_REF or in entered in any reduction. |
| IMPACTING_REF | IMPACTRE F | 0{ALPHANUMERIC}12 | Unique reference | References the ASSIGNMENT_REF of the associated transmission reservation (if applicable) that caused the reduction in capacity |
| INITIATING_PARTY | INITPARTY | 0{ALPHANUMERIC}4 | Registered company code for a Transmission Provider (TP), Security Coordinator (SC) or Control Area (CA) | Company code for company responsible for initiating execution of a transmission security procedure. |
| INTERFACE_TYPE | INTERFACE | 0{ALPHANUMERIC}1 | Valid Values I E | Type of interface define by path: Internal (I) to a control area or External (E) to a control area |
| LCA_CODE | LCACODE | 0{ALPHANUMERIC}4 | Valid registered control area code | Load Control Area registered code. Information comes from tag |
| LIST_ITEM | ITEM | 1{ALPHANUMERIC}50 | Free form text | Item from LIST, such as list of SELLER, list of PATH_NAME, list of POINT_OF_RECEIPT, list of POINT_OF_DELIVERY, list of SERVICE_INCREMENT, list of TS_CLASS, list of TS_TYPE, list of TS_PERIOD, list of TS_WINDOW, list of TS_SUBCLASS, list of AS_TYPE, list of REQUEST_TYPE, list of ANC_SERVICE_POINT, list of FACILITY_CLASS, list of FACILITY_LIMIT_TYPE, list of PROCEDURE_NAME, list of SYSTEM_ATTIBUTE, list of SECURITY_TYPE, list of FACILITY_LOCATION, list of NERC_CURTAILMENT_PRIORITY, list of OTHER_CURTAILMENT_PRIORITY, list of CATEGORY, list of TEMPLATE, list of LIST |
| LIST_ITEM_DESCRIPTION | ITEMDESC | 0{ALPHANUMERIC}100 | Free form text | A detailed description of the LIST_ITEM |
| LIST_NAME | LIST | 1{ALPHANUMERIC}50 | Valid Values LIST SELLER PATH POR POD, SERVICE_INCREMENT TS_CLASS TS_TYPE TS_PERIOD TS_SUBCLASS AS_TYPE NERC_CURTAILMENT_PRIORITY REQUEST_TYPE ANC_SERVICE_POINT FACILITY_CLASS FACILITY_LIMIT_TYPE PROCEDURE_NAME SYSTEM_ATTRIBUTE SECURITY_TYPE FACILITY_LOCATION OTHER_CURTAILMENT_PRIORITY CATEGORY TEMPLATE | List of valid names for each of the types of lists. The minimum set of lists defined must be implemented. |
| MESSAGE | MSG | 1{ALPHANUMERIC}200 | Free form text | An informative text message |
| MODIFYING_COMPANY_CODE | MODCODE | 1{ALPHANUMERIC}6 | Registered company code for a TP, SC or CA | Contains the registered company code that modified the transaction, used in the audit Templates |
| MODIFYING_NAME | MODNAME | 0{ALPHANUMERIC}25 | free form text | Contain the name of the person that modified the transaction, used in the audit Templates |
| MODIFICATION_REF | MODREF | 1{ALPHANUMERIC}12 | Valid ASSIGNMENT_REF | Forward pointer. Pointing to next reservation that replaces the current reservation |
| NEGOTIATED_PRICE_FLAG | NGPRIFLG | 0{ALPHANUMERIC}1 | Valid Values H L null | Set to H if OFFER_PRICE is higher than the currently posted price; Set to L if OFFER_PRICE is lower than the currently posted price |
| NERC_CURTAILMENT_PRIORITY | NERCURT | 1{INTEGER}1 | Integer | One of the NERC curtailment priorities, documented in LIST Template |
| NEW_COMPANY | NEWCO | 1{ALPHANUMERIC}25 | Free form text | New company of the person who is transferring |
| NEW_DEPARTMENT | NEWDEPT | 1{ALPHANUMERIC}52 | Free form text | New department of the person who is transferring |
| NEW_POSITION | NEWPOS | 1{ALPHANUMERIC}25 | Free form text | New position held by the person who is transferring |
| OFFER_PRICE | OFFPR | 1{NUMERIC}5 + "." + 2{NUMERIC}4 | Positive number with 2 to 4 decimals | The current offered price of a Service in dollars and cents. Used by the Seller to indicate the offering price. |
| OFFER_START_TIME | OFFSTIME | 0,16{ALPHANUMERIC}16 | Valid Date and Time to seconds: yyyy+mo+dd+hh +mm+ss+tz | Start time of the window during which a Customer may request the service posted in the service offering, i.e., opening time of an offer. If null, no restrictions on the start of the offering time is implied (other than tariff requirements). |
| OFFER_STOP_TIME | OFFSPTIME | 0,16{ALPHANUMERIC}16 | Valid Date and Time to seconds: yyyy+mo+dd+hh +mm+ss+tz | Stop time of the window during which a Customer may request the service posted in the service offering, i.e., expiration time of an offer. If null, no restrictions on the end of the offering time is implied (other than tariff requirements). |
| OPTIONAL_CODE | N/A | 0{ALPHANUMERIC}25 | Unique path name within region | Unique for Path. If used for directionality, then the first 12 characters shall represent POR, followed by "-", followed by 12 characters which shall represent POD. Used by PATH_NAME. |
| OTHER_CURTAILMENT_PRIORITY | OTHCUR | 0{ALPHANUMERIC}8 | Valid Values: {Registered} | Other than NERC curtailment priorities, such as regional curtailment priorities. Suggested format region+number, for example MAPP4, WECC7. Documented in LIST Template and registered with central registry. |
| OUTPUT_FORMAT | FMT | 4{ALPHANUMERIC}4 | Valid Values HTML DATA | Format of response: HTML = hypertext markup language for presentation using a web browser DATA = text for use in a downloaded file. |
| PATH_CODE | N/A | 0{ALPHANUMERIC}12 | Unique code for each path as defined by primary provider | Unique code within a Region for each path. Used by PATH_NAME |
| PATH_NAME | PATH | 5{ALPHANUMERIC}50 | Unique value | The unique name assigned to a single transmission line or the set of one or more parallel transmission lines whose power transfer capabilities are strongly interrelated and must be determined in aggregate. These lines are typically described as being on a path, corridor or interconnection in some regions, or as crossing an interface or cut-plane in other regions. Multiple lines may be owned by different parties and require prorating of capability shares. The name is constructed from the following codes, with each code separated by a "/". Trailing "/" may be omitted, if there are no values for OPTION_CODE and SPARE_CODE: REGION_CODE - 2 chars, unique to OASIS System PRIMARY_PROVIDER_CODE - 4 chars, unique within Region PATH_CODE - 12 chars, unique for Primary Provider OPTIONAL_CODE - 25 chars, unique for Path. If used for directionality, then the first 12 characters shall represent POR, followed by "-", followed by 12 characters which shall represent POD SPARE_CODE - 3 chars. |
| POINT_OF_DELIVER Y | POD | 1{ALPHANUMERIC}12, Only non-numeric and non-alpha character allowed is ".". | Unique value within Primary Provider. Only special character allowed is ".", for example, ab.cde.123 | One or more point(s) of interconnection on the Transmission Provider's transmission system where capacity and/or energy transmitted by the Transmission Provider will be made available to the Receiving Party. This is used along with Point of Receipt to define a Path and direction of flow on that path. For internal paths, this would be a specific location(s) in the area. For an external path, this may be an area-to-area interface. |
| POINT_OF_RECEIPT | POR | 1{ALPHANUMERIC}12 Only non-numeric and non-alpha character allowed is ".". | Unique value within Primary Provider. Only special character allowed is ".", for example, ab.cde.123 | One or more point(s) of interconnection on the Transmission Provider's transmission system where capacity and/or energy transmitted will be made available to the Transmission Provider by the Delivering Party. This is used along with Point of Delivery to define a Path and direction of flow on that path. For internal paths, this would be a specific location(s) in the area. For an external path, this may be an area-to-area interface. |
| POSTING_NAME | POSTNAME | 1{ALPHANUMERIC}25 | Free form text | Name of person who is posting the information on the OASIS Node |
| POSTING_REF | POSTREF | 1{ALPHANUMERIC}12 | Unique Value | Assigned by TSIP when Service or Message is received by TSIP. Unique reference can be used by the user to modify or delete the posting. |
| PRECONFIRMED | PRECONF | 2{ALPHA}3 | Valid Values YES NO | Used by Customer to preconfirm sale in Template TRANSREQUEST or ANCREQUEST. If customer indicates sale is preconfirmed, then the response is YES and the customer does not need to confirm the sale. |
| PRICE_UNITS | UNITS | 0(ALPHA)20 | Free form text | The units used for CEILING_PRICE, OFFER_PRICE, and BID_PRICE. Examples: $/MWhr, $/MWmonth |
| PRIMARY_PROVIDER_APPROVAL | PPAPP | 0{ALPHA}1 | Y N | To indicate Primary Provider approval of a Transfer. Y = Yes, Primary Provider has approved Transfer N = No, Primary Provider does not approve the Transfer |
| PRIMARY_PROVIDER_CODE | PROVIDER | 1{ALPHANUMERIC}4 | Unique code | Unique code for each Primary Provider. Used by PATH_NAME and in URL. Registered as part of URL at www.tsin.com. |
| PRIMARY_PROVIDER_COMMENTS | PPROVCOM | 0{ALPHANUMERIC}255 | Free form text | Informative text. Usually entered by the Primary Provider through a back end system. For information communicated between primary transmission provider and all other parties. |
| PRIMARY_PROVIDER_CONDITIONS | PPCOND | 0{ALPHANUMERIC}255 | Free form text | Fully qualified URL pointing to a document specifying the pertinent terms and conditions of service for the Conditional Curtailment Option for long term firm service. |
| PRIMARY_PROVIDER_DUNS | PPROVDUNS | 9{NUMERIC}9 | Valid DUNS number | Unique code for each Primary. Provided by Dun and Bradstreet. |
| PROCEDURE_NAME | PROCNAM E | 0{ALPHANUMERIC}25 | Valid Values: NERC TLR WECC USF {Registered} | Name of a transmission security procedure: - NERC TLR as defined in NERC Policy 9 - WECC USF as defined in WECC Policy - Local procedure as registered by Transmission Providers |
| PROCEDURE_LEVEL | PROCLVL | 1{ALPHANUMERIC}25 | Valid Values: (NERC TLR Levels} {WECC USF Levels} {Registered} | Levels or stages associated with actions to be taken in implementation of a transmission security procedure as defined in: - NERC Policy 9 for the NERC TLR procedure - WECC Policy for the WECC USF procedure - Local procedures as registered by Transmission Providers |
| PROVIDER_ACTION | PROVACT | 1{ALPHANUMERIC}25 | Free form text, for example: DENIED CURTAILED INTERRUPTED | Indicates the particular action taken by the Transmission Provider with respect to the scheduled transaction; specific values to be returned are, DENIED if the schedule was not started as requested, CURTAILED if the scheduled MW was limited for reliability reasons, or INTERRUPTED if the scheduled MW was limited for economic reasons. |
| REASSESSMENT_DUE_TIME | RASMTTIME | 0,16{ALPHANUMERIC}16 | Valid date and time to seconds: yyyy+mo+dd+hh+tz | Deadline when the Primary Provider may exercise their right to review conditions associated with granted long-term service with a Conditional Curtailment Option. |
| REASSIGNED_CAPACITY | RASCAP | 1{NUMERIC}12 | Positive number, cannot exceed previous assigned capacity | The amount of transfer capability that was reassigned from one entity to another. |
| REASSIGNED_REF | REREF | 1{ALPHANUMERIC}12 | Unique value | Contains the ASSIGNMENT_REF of any preceding (parent) requests that are affected by this request. Used only for secondary market sales. |
| REASSIGNED_START_TIME | RESSTIME | 16{ALPHANUMERIC}16 | Valid date and time to seconds: yyyy+mo+dd+hh+tz | Beginning date and time of the reassigned transmission service |
| REASSIGNED_STOP_TIME | RESSPTIME | 16{ALPHANUMERIC}16 | Valid date and time to hour: yyyy+mo+dd+hh+tz | Date and time of the end of the transmission service that is reassigned to another User. |
| RECORD_STATUS | RECSTATUS | 1{NUMERIC}3 | Error number | Record status indicating record was successful or error code if unsuccessful. 200 = Successful |
| RECORD_TYPE | RECTYPE | 1{ALPHA}1 | Valid Values: I U D | Indicates the type of information reported in a response record generated by an audit Template. "I" designates information as it was initially inserted (posted) on OASIS; "U" designates information updated (modified) on OASIS; "D" designates deleted information as it appeared on OASIS just prior to being deleted (as appropriate). |
| REDUCTION_REASON | REDREAS | 1{ALPHANUMERIC}50 | Free form text | Reason for the reduction |
| REDUCTION_TYPE | REDTYPE | 1{ALPHANUMERIC}25 | Valid values: CONDITIONAL_EVENT CONDITIONAL_HOURS RECALL REDIRECT RESALE TRANSFER {Registered} | Type of reduction in reserve capacity or service curtailment priority |
| REGION_CODE | N/A | 1{ALPHANUMERIC}2 | Unique within OASIS System | Defined for NERC regions, with the following defined: S - SERC T - ERCOT A - MAPP P - SPP N - NPCC W - WECC F - FRCC R - RFC Second character or digit reserved for sub-region id as defined by each region. |
| RELATED_REF | RELREF | 1{ALPHANUMERIC}12 | Unique reference | Contains the ASSIGNMENT_REF of any preceding (parent) requests that are affected by this request |
| RENEWAL_DUE_TIME | RENEWTIME | 0,16{ALPHANUMERIC}16 | Valid date and time to seconds: yyyy+mo+dd+hh+tz | The deadline for Customer submission of a renewal request to exercise their on-going rollover rights. |
| REQUEST_REF | RREF | 0{ALPHANUMERIC}12 | Unique value | A reference uniquely assigned by a Customer to a request for service from a Provider. |
| REQUEST_STATUS | RSTATUS | 1{NUMERIC}3 | Error number | Message status indicating message was successful (if all RECORD_STATUS show success) or error code if any RECORD_STATUS showed unsuccessful. 200 = Successful |
| REQUEST_TYPE | REQTYPE | 1{ALPHA}30 | Valid Values: ORIGINAL RESALE RENEWAL MATCHING DEFERRAL REDIRECT RELINQUISH FULL_TRANSFER PART_TRANSFER RECALL {Registered} | ORIGINAL = typical reservation requests submitted to the Primary Provider (as the Seller of the transmission or ancillary service). RESALE = secondary market requests submitted to a Transmission Customer as Secondary Provider. RENEWAL = request to renew an expiring transmission reservation that has rollover rights. MATCHING = request to meet or exceed a competing request to retain transmission service (right of first refusal). DEFERRAL = request to defer or apply for extension on start of transmission service. REDIRECT = request to redirect all or portion of a transmission reservation to an alternate POR/POD and/or make other changes to the terms of service as permitted. RELINQUISH = request to release all or a portion of the capacity of a Redirect on a Non-Firm basis to the Firm Parent Reservation. FULL_TRANSFER = request to transfer all capacity, rights, encumbrances and obligations, including financial liability to the Primary Provider, from one Transmission Customer to another. PART_TRANSFER = request to transfer a portion, but not all, capacity, and all rights, and obligations, including financial liability associated with the transferred capacity to the Primary Provider, from one Transmission Customer to another. No encumbrances (resales, etc) may be transferred with a PART_TRANSFER. RECALL = request submitted by the Seller (Reseller or Primary Provider) to take back all or a portion of the capacity of a transmission reservation {registered} = Primary Provider's may register values for REQUEST_TYPE to implement specific provisions of their Tariffs. |
| RESPONSE_TIME_LIMIT | RESPTL | 16{ALPHANUMERIC}16 | Valid date and time to seconds: yyyy+mo+dd+hh +mm+ss+tz | Date and time to seconds by when a response must be received from a Customer |
| RESPONSIBLE_PARTY | PARTY | 1{ALPHANUMERIC}4 | Registered company code for a TP, SC or CA | The company code of the entity responsible for administering a transmission security procedure. |
| RESPONSIBLE_PARTY_ NAME | PARTNAME | 1{ALPHANUMERIC}25 | Free form text | The name of the person responsible for granting the discretion. |
| RETURN_TZ | TZ | 2{ALPHANUMERIC}2 | Valid Values AD AS PD PS ED ES MD MS CD CS UT | A time zone code, indicating the base time zone, and whether daylight saving time is to be used. This field may be set by a Customer in a query. Returned date and time data is converted to this time zone. Time zones: Atlantic time = AD, AS Eastern time = ED, ES Central time = CD, CS Mountain time = MD, MS Pacific time = PD, PS Universal time = UT |
| ROLLOVER_CAPACITY | ROLLCAP | 0{NUMERIC}12 | Non-negative number in units of MW | The amount of reserve capacity eligible for Rollover over the interval of ROLLOVER_START_TIME to ROLLOVER_STOP_TIME. |
| ROLLOVER_START_TIME | ROLLSTTIME | 0,16{ALPHANUMERIC}16 | Valid Date and Time to seconds: yyyy+mo+dd+hh +mm+ss+tz | Start of the interval designating the amount of ROLLOVER_CAPACITY available. |
| ROLLOVER_STOP_TIME | ROLLSPTIME | 0,16{ALPHANUMERIC}16 | Valid Date and Time to seconds: yyyy+mo+dd+hh +mm+ss+tz | End of the interval designating the amount of ROLLOVER_CAPACITY available. |
| SALE_REF | SREF | 0{ALPHANUMERIC}12 | Unique value | Identifier which is set by seller (including Primary Provider) when posting a service for sale |
| SCHEDULE_GRANTED | SCHEDGRNTED | 0{NUMERIC}12 | Non-negative number in units of MW | Reflects the MW value of energy actually scheduled by the Transmission Provider at either the point of receipt or delivery, whichever is larger, over the START_TIME/STOP_TIME time interval |
| SCHEDULE_LIMIT | SCHEDULELIM | 0{NUMERIC}12 | Non-negative number in units of MW | Reflects the maximum MW value over the START_TIME/STOP_TIME interval that the Provider has determined can be scheduled |
| SCHEDULE_PRIORITY | SPRIORITY | 0{NUMERIC}2 | Positive Number | Identifies the relative priority of this particular interchange transaction as compared to all other scheduled transactions with respect to the application of curtailments or interruptions. SCHEDULE_PRIORITY would typically reflect the curtailment priority Data Elements associated with the OASIS transmission reservation used to support the schedule (i.e., NERC_CURTAILMENT_PRIORITY or OTHER_CURTAILMENT_PRIORITY). |
| SCHEDULE_REF | SCHDREF | 0{ALPHANUMERIC}20 | Unique reference | Unique reference assigned by Transmission Provider to a posting of a schedule information |
| SCHEDULE_REQUESTED | SCHEDULEREQ | 0{NUMERIC}12 | Non-negative number in units of MW | Scheduled energy requested by the Transmission Customer (TC) |
| SECURITY_REF | SECREF | 1{ALPHANUMERIC}10 | Unique value | Unique value generated by company initiating the security for each security event in the SECURITY Template. |
| SECURITY_TYPE | SECTYPE | 1{ALPHANUMERIC}6 | Valid Values: OUTAGE LIMIT {Registered} | Identifies the type of information posted for the event; restricted values are: OUTAGE = for postings reflecting the state of critical transmission facilities LIMIT= for postings reflecting the implementation of security procedures to limit or reduce scheduled transactions. |
| SELLER_CODE | SELLER | 1{ALPHANUMERIC}6 | Unique value | Organization name of Primary Provider or Reseller. |
| SELLER_COMMENTS | SELCOM | 0{ALPHANUMERIC}255 | Free form text | Informative text provided by the Seller. For information communicated between the seller (either Primary Provider or reseller) to the customer of the services. |
| SELLER_DUNS | SELDUNS | 9{NUMERIC}9 | Valid DUNS number | Unique Data Universal Numbering System provided by Dun and Bradstreet. Code for a Primary Provider or Seller. |
| SELLER_EMAIL | SELEMAIL | 5{ALPHANUMERIC}60 | Valid network reference | E-Mail address of Seller contact person |
| SELLER_FAX | SELFAX | 14{ALPHANUMERIC}20 | Area code and telephone number, plus any extensions Example: (aaa)-nnn-nnnn xnnnn | The fax telephone number for contact person at Seller. |
| SELLER_NAME | SELNAME | 1{ALPHANUMERIC}25 | Free form text | The name of an individual contact person at the Seller. |
| SELLER_PHONE | SELPHONE | 14{ALPHANUMERIC}20 | Area code and telephone number, plus any extensions (aaa)-nnn-nnnn xnnnn | The telephone number of a contact person as a Seller |
| SELLER_REF | SELREF | 0{ALPHANUMERIC}12 | Free form text | Identifier which is set by seller (including Primary Provider) to uniquely identify reservation requests for seller’s own internal use |
| SERVICE_DESCRIPTION | SVCDESC | 0{ALPHANUMERIC}200 | Free form text | Information regarding a service. |
| SERVICE_INCREMENT | SRVINCR | 1{ALPHANUMERIC}8 | Valid Values HOURLY DAILY WEEKLY MONTHLY YEARLY {Registered} | The transmission service increments provided. Five are pre-defined, while additional increments can be used if they are registered on TSIN.COM and shown in the Provider's LIST Template |
| SERVICE_NAME | SVCNAME | 1{ALPHANUMERIC}25 | Free form text | Name of service affected by the discretionary action |
| SERVICE_TYPE | SVCTYPE | 1{ALPHANUMERIC}25 | Free form text | Type of service affected by the discretionary action. |
| SINK | SINK | 0{ALPHANUMERIC}14 | Valid area name | The area in which the SINK is located. |
| SOURCE | SOURCE | 0{ALPHANUMERIC}14 | Valid area name | The area in which the SOURCE is located. |
| SPARE_CODE | N/A | 0{ALPHANUMERIC}3 | Defined by region | Spare code to be used at a later time. Used by PATH_NAME |
| STANDARDS_OF_CONDUCT_ISSUES | STDISSUE | 0{ALPHANUMERIC}800 | Free form text | Issues that were in violation of the FERC Standards of Conduct. This text may include a reference pointer to a more detailed description. |
| START_TIME | STIME | 16{ALPHANUMERIC}16 | Valid Date and Time to seconds: yyyy+mo+dd+hh +mm+ss+tz | Start date and clock time of a service. When used as a Query Variable, it requires the return of all items whose Stop time is after the Start time. Note that for some Templates when used as a Query Variable the time may be only valid up to the hour, day or month. If more data is given than is valid, the hour, day or month will be used to make the date and time inclusive, i.e. date or time will be truncated to valid hour, day or month. |
| START_TIME_POSTED | STIMEP | 16{ALPHANUMERIC}16 | Valid Date and Time to seconds: yyyy+mo+dd+hh +mm+ss+tz | Query parameter to indicate all the records are to be retrieved that were posted on or after this time. |
| START_TIME_QUEUED | STIMEQ | 16{ALPHANUMERIC}16 | Valid Date and Time to seconds: yyyy+mo+dd+hh +mm+ss+tz | Start date and clock time of a service, used for requesting transactions queued after this time |
| STATUS | STATUS | 5{ALPHANUMERIC}25 | Valid Values QUEUED INVALID RECEIVED STUDY REBID COUNTEROFFER DECLINED SUPERSEDED, ACCEPTED REFUSED CONFIRMED WITHDRAWN DISPLACED ANNULLED RETRACTED | QUEUED = initial status assigned by TSIP on receipt of "customer services purchase request". INVALID = assigned by TSIP or Primary Provider indicating an invalid field in the request, such as improper POR, POD, source, sink, etc. (Final state). RECEIVED= assigned by Primary Provider or Seller to acknowledge QUEUED requests and indicate the service request is being evaluated, including for completing the required ancillary services. STUDY= assigned by Primary Provider or Seller to indicate some level of study is required or being performed to evaluate service request. REFUSED = assigned by Primary Provider or Seller to indicate service request has been denied due to lack of availability of transfer capability. (Final state). |
| STATUS (cont.) |  |  |  | COUNTEROFFER= assigned by Primary Provider or Seller to indicate that a new OFFER_PRICE and/or CAPACITY_GRANTED over time is being proposed in the negotiation of requested service (i.e., offering of partial service or negotiation of price). REBID = assigned by Customer to indicate that a new BID_PRICE and/or CAPACITY_REQUESTED over time is being proposed. SUPERSEDED = assigned by Primary Provider or Seller when a request which has not yet been confirmed is preempted by another reservation request. (Final state). ACCEPTED = assigned by Primary Provider or Seller to indicate the service request at the designated BID_PRICE and CAPACITY_REQUESTED has been approved/accepted. Depending upon the type of ancillary services required, the Seller may or may not require all ancillary service reservations to be completed before accepting a request. |
| STATUS (cont.) |  |  |  | DECLINED = assigned by the Primary Provider or Seller to indicate that the terms and conditions, such as the BID_PRICE, are unacceptable and that negotiations are terminated or that contractual terms have not been met. (Final state). RETRACTED = assigned by Primary Provider or Seller when the Customer fails to confirm or withdraw the request within the required time period. (Final state). WITHDRAWN = assigned by the Customer during request evaluation to withdraw the request from any further action. (Final state). CONFIRMED = assigned by the Customer in response to the Primary Provider or Seller posting "ACCEPTED" status, to confirm service. Once a request has been "CONFIRMED", a transmission service reservation exits. (Final state, unless overridden by DISPLACED or ANNULLED state). DISPLACED = assigned by Primary Provider or Seller when a "CONFIRMED" reservation from a Customer is displaced by a higher priority reservation, and the Customer is not offered or has not exercised right of first refusal (i.e. refused to match terms of new request). (Final State.) ANNULLED = assigned by the Seller when, by mutual agreement with the Customer, a confirmed reservation is to be voided, or assigned unilaterally by the Primary Provider when a confirmed reservation is to be voided. (Final State). |
| STATUS_COMMENTS | STACOM | 0{ALPHANUMERIC}255 | Free form text | Informative: For information to be communicated by any party to all other parties. |
| STATUS_NOTIFICATION | STATNOT | 0{ALPHANUMERIC}200 | http://URL:portnumber/direcotry/cgi script/query parameters or Mailto: <e-mail address> | Shall contain the protocol field "http:", which designates the notification method/protocol to be used, followed by all resource location information required; the target domain name and port designations shall be inserted into the notification URL based on the Customer's Company registration information. The resource location information may include directory information, cgi script identifiers and URL encoded query string name/value pairs as required by the Customer's application. or Mailto and email address for the status information the Customer wants to receive upon a change in STATUS of transstatus, or ancstatus |
| STOP_TIME | SPTIME | 16{ALPHANUMERIC}16 | Valid date and time yyyy+mo+dd+hh +mm+ss+tz | Stop date and clock time. When used as a Query Variable, it requires the return of all items which start before the Stop time. Note that for some Templates when used as a Query Variable the time may be only valid up to the hour, day or month. If more data is given than is valid, the hour, day or month will be used to make the date and time inclusive, i.e. date or time will be increased to include STOP_TIME. |
| STOP_TIME_POSTED | STPTIMEP | 16{ALPHANUMERIC}16 | Valid Date and Time to seconds: yyyy+mo+dd+hh +mm+ss+tz | Query parameter to indicate all the records are to be retrieved that were posted on or before this time. |
| STOP_TIME_QUEUED | SPTIMEQ | 16{ALPHANUMERIC}16 | Valid Date and Time to seconds: yyyy+mo+dd+hh +mm+ss+tz | Stop date and clock time, used for requesting transactions queued before this time |
| SUBJECT | SUBJ | 0{ALPHANUMERIC}80 | Free form text | Informative text used to summarize a topic in a message |
| SYSTEM_ATTRIBUTE | SYSATTR | 0{ALPHANUMERIC}15 | Valid Values: CBM FTRM NFTRM TTC FATC NFATC ATC_ANNOTATION ZONE_LOAD_FORECAST SYSTEM_LOAD_FORECAST NATIVE_LOAD_FORECAST ACTUAL_LOAD {Registered} | Type of system data viewed by systemdata template: CBM – Capacity Benefit Margin FTRM – Transmission Reliability Margin for use in FATC NFTRM – Transmission Reliability Margin for use in NFATC TTC – Total Transmission Capability FATC –Firm Available Transmission Capability NFATC – Non-firm Available Transmission Capability ATC_ANNOTATION – Annotation for a change in monthly or yearly posted ATC. ZONE_LOAD_FORECAST – Forecast of anticipated Load in a metered zone SYSTEM_LOAD_FORECAST – Forecast of total system wide load within a Balancing Area NATIVE_LOAD_FORECAST – Forecast of portion of total system load forecast for native load portion of total system load within a Balancing Area ACTUAL_LOAD – Actual daily peak load values for the metered Balancing Area or metered Zone within the BA posted after the fact {registered} – Provider specific registered name for the data posted |
| SYSTEM_ELEMENT | SYSELM | 1{ALPHANUMERIC}255 | Free form text | The name of a transmission system element associated with the SYSTEM_ATTRIBUTE being posted. For example, the value of PATH_NAME for records with SYSTEM_ELEMENT_TYPE = PATH. See WEQ 013 for further information. |
| SYSTEM_ELEMENT_TYPE | SYSELMTY | 1{ALPHA}30 | Valid Types: PATH LOAD_ZONE {Registered} | PATH - Designates that the value of SYSTEM_ELEMENT is the name of a posted path (PATH_NAME). LOAD_ZONE - Designates that the value of SYSTEM_ELEMENT is the name of the metered zone whose forecast and actual load data is being posted. {registered} - Primary Provider's may register values for SYSTEM_ELEMENT_TYPE to implement specific provisions of their Tariffs. |
| TARIFF_REFERENCE | TARIFF | 0{ALPHANUMERIC}150 | Free form text | Tariffs approved by FERC Name and description of Tariff |
| TEMPLATE | TEMPL | 1{ALPHANUMERIC}20 | Valid Name of Template from Standard 002-4.3.1 or from LIST Template | The name of a logical collection of DATA_ELEMENTS in a User's interaction with an OASIS Node. |
| TIME_OF_LAST_UPDATE | TLUPDATE | 16{ALPHANUMERIC}16 | Valid date and time to seconds: yyyy+mo+dd+hh +mm+ss+tz | Date and time to seconds that data was last updated. May be used to search data updated since a specific point in time. |
| TIME_POSTED | TIMEPST | 16{ALPHANUMERIC}16 | Valid Date and Time to seconds: yyyy+mo+dd+hh +mm+ss+tz | Date and time a message is posted |
| TIME_QUEUED | TIMEQ | 16{ALPHANUMERIC}16 | Valid Date and Time to seconds: yyyy+mo+dd+hh +mm+ss+tz | Date and time that the request was queued |
| TIME_STAMP | TSTAMP | 16{ALPHANUMERIC}16 | Valid date and Time to seconds yyyy+mo+dd+hh+mm+ss+tz | Time data is created |
| TRANSACTION_ID | TRANSID | 1{ALPHANUMERIC}30 | Free form text | Identifier associated with an interchange transaction that may span multiple SCHEDULE_REF records. May be the NERC Tag id as specified in the NERC Electronic Tagging Functional Specification. |
| TS_CLASS | TSCLASS | 1{ALPHANUMERIC}20 | Valid Values: FIRM NON-FIRM TTC SECONDARY {Registered} | The transmission service classes provided. Four are pre-defined, while additional classes can be used if they are registered on TSIN.COM and shown in the Provider's LIST Template page. SECONDARY is defined as alternate points of receipt or delivery for POINT_TO_POINT, or as non-designated resources for NETWORK service. TTC is retained as a Valid Value for historical purposes only. |
| TS_PERIOD | TSPER | 1{ALPHANUMERIC}20 | Valid Values: ON_PEAK OFF_PEAK FULL_PERIOD {Registered} | The transmission service periods provided. Three are pre-defined, while additional periods can be used if they are registered on TSIN.COM and shown in the Provider's LIST Template |
| TS_SUBCLASS | TSSUBC | 0{ALPHANUMERIC}20 | Free form text | The transmission service subclasses provided. These are free form. |
| TS_TYPE | TSTYPE | 1{ALPHANUMERIC}20 | Valid Values POINT_TO_POIN T NETWORK ATC {Registered} | The transmission service types provided. Three are pre- defined, while additional types can be used if they are registered on TSIN.COM and shown in the Provider's LIST Template ATC is retained as a Valid Value for historical purposes only. |
| TS_WINDOW | TSWIND | 1{ALPHANUMERIC}20 | Valid Values FIXED SLIDING EXTENDED NEXT_INCREME NT {Registered } | The transmission service windows provided. Four are pre-defined, while additional windows can be used if they are registered on TSIN.COM and shown in the Provider's LIST Template |
| TZ | TZ | 2{ALPHANUMERIC}2 | Valid Values AD AS PD PS ED ES MD MS CD CS UT | Valid time zone and indication whether daylight savings time is to be used Time zones: Atlantic time = AD, AS Eastern time = ED, ES Central time = CD, CS Mountain time = MD, MS Pacific time = PD, PS Universal time = UT |
| VALID_FROM_TIME | VALFTIME | 16{ALPHANUMERIC}16 | Valid date and time yyyy+mo+dd+hh +mm+ss+tz | Date and time after which the message is valid |
| VALID_TO_TIME | VALTTIME | 16{ALPHANUMERIC}16 | Valid date and time yyyy+mo+dd+hh +mm+ss+tz | Date and time before which the message is valid |
| VERSION | VER | 1{REAL NUMBER}6 | Range of 1.0 to 9999.9 | Specifies which version of the OASIS Standards and Communication Protocol to use when interpreting the request |

For all Data Elements indicated as being free form text, the Field Format designation of ALPHANUMERIC shall represent any valid printable character in the ASCII character set.