Business Practices for Open Access Same-Time Information Systems (OASIS) Implementation Guide

Version 1.5

Document Change Log:

| Date | Section | Description of Change |
| --- | --- | --- |
| 06/06/2007 | All | Pertinent business process related requirements to be implemented as part of the OASIS Standards have been incorporated into this first version of the OASIS Implementation Guide. |
| 06/06/2007 | All | Global change of Primary Transmission Provider to Primary Provider and global change of Transmission Provider to Primary Provider. |
| 06/06/2007 | 013-2 | OASIS S&CP Standards 002-4.10.2 and 002-4.10.2.1 have been consolidated into 013-2. |
| 06/06/2007 | 013-2.1 | Enumerated values for OASIS REQUEST_TYPE data element from OASIS S&CP Standard 002-4.2.13 have been consolidated into 013-2.1. |
| 06/06/2007 | 013-2.2 | OASIS S&CP Standard 002-4.2.10.2 Status Values has been incorporated into Standard 013-2.2. |
| 06/06/2007 | 013-2.3 through 2.6 | All OASIS S&CP Standards contained under 002-4.2.13 and subordinate Standards have been incorporated and extended under new Standards 013-2.3 through 013-2.6 and subordinate Standards. |
| 06/06/2007 | 013-3 | Standards added to describe specific implementation requirements associated with certain OASIS Standard templates. |
| 06/06/2007 | 013-4.1 | OASIS S&CP Standard 002-4.4, File Examples, and all subordinate standards have been incorporated into this document in their entirety into Standard 013-4.1. |
| 06/06/2007 | 013-4.2 | Examples for Ancillary Services linkage to Transmission Services from OASIS S&CP Standard 002-4.2.12 have been incorporated into this document in their entirety into Standard 013-4.2 |
| 09/18/2007 | Various | Minor edits including spacing, grammatical changes resulting from review using Microsoft Word Spelling and Grammar Check function. |
| 09/18/2007 | Various | Formatted OASIS template names so all template names when referenced in paragraphs are designated bold/italicized. |
| 09/18/2007 | Various | Consistent formatting of data element names and data values |
| 09/18/2007 | Various | Consistent use of term Transmission Customer. |
| 09/18/2007 | Header | Added reference to indicate the OASIS Version Number |
| 09/18/2007 | 013-2.6.2 through 13.2 | Added note for each table in the following sections to indicate the data elements listed are not necessarily the same order as they are required within the OASIS templates |
| 09/18/2007 | 013-2.6.6 | Changed Capacity Available to a data element name and Redirect to a data value in the second paragraph. |
| 09/18/2007 | 013-2.6.7.1 | Added section references to the fourth paragraph. |
| 09/18/2007 | 013-3.2 | Updated reference to NERC Electronic Tagging Functional Specification. |
| 09/18/2007 | 013-3.2 | Second table corrected Restriction/Requirement for CUSTOMER_CODE and CUSTOMER_DUNS to reference Transmissions Customer rather than SELLER. |
| 09/18/2007 | 013-4 through 013.4.1.7.2 | Updated all examples in sub-sections to match order of data elements documented in the templates as specified in WEQ 002 OASIS Standards and Communication Protocols Document. |
| 09/18/2007 | 013-4.1 | Added last sentence at the end of section. |
| 09/18/2007 | 013-4.1.4 | Replaced references of Information Provider to OASIS in examples a, b, and d. Replaced reference of Information Provider to Primary Provider in example e. |
| 09/25/2007 | Various | General change to bid/offer data element names incorporated per corresponding change made to WEQ 002. |
| 09/25/2007 | 013-2 | For Transfers, require both Seller and Provider to set request to accepted/approved. |
| 09/25/2007 | 013-2.1 | Add REQUEST_TYPEs of FULL_TRANSFER and PART_TRANSFER. |
| 09/25/2007 | 013-2.2 | Added restriction on withdrawal of certain pre-confirmed requests and actions to be taken by the TP on customer request to withdraw such request. |
| 09/25/2007 | 013-2.2.1 | Document restrictions on setting request state to ACCEPTED. |
| 09/25/2007 | 013-2.2.2 | Document restrictions on setting request state to CONFIRMED. |
| 09/25/2007 | 013-2.3 | Added language to include FULL and PART_TRANSFER for REQUEST_TYPES. Added limitation that Customer cannot set STATUS to WITHDRAWN for pre-confirmed request. |
| 09/25/2007 | 013-2.4.2 | Added note about new set of BPs related to displacement/competition that have yet to be established |
| 09/25/2007 | 013-2.5 | Added REQUEST_TYPE of RECALL |
| 09/25/2007 | 013-2.6.1 | Modified Data Elements for ORIGINAL Requests and added definition of term of service. |
| 09/25/2007 | 013-2.6.1.1 | Modified data elements and added language on Customer rebid. |
| 09/25/2007 | 013-2.6.1.2 | Modified data elements and added language on OASIS verifying bid and offer data elements matching to confirm requests. |
| 09/25/2007 | 013-2.6.1.3 | Modified to reflect new bid/offer data elements and added Rollover Rights to transstatus template |
| 09/25/2007 | 013-2.6.2 | Added caveat that Customer of RENEWAL may not be current rights holder due to agency arrangements between LSE and scheduling PSE(s). |
| 09/25/2007 | 013-2.6.3 | Modified data elements |
| 09/25/2007 | 013-2.6.4 | Modified data elements |
| 09/25/2007 | 013-2.6.5.1 | Added obligation to document conveyance of any rollover rights from parent to redirect request on a firm basis. |
| 09/25/2007 | 013-2.6.5.2 | Modified data elements, added PRECONFIRMED, and removed language requiring evaluating REDIRECT on Non-Firm basis same as other non-firm point to point. |
| 09/25/2007 | 013-2.6.6.1 | Modified data elements for RELINQUISH |
| 09/25/2007 | 013-2.6.7.1 | Modified data elements for RESALE on OASIS, added requirement for bid price, and added requirement for executed service agreement |
| 09/25/2007 | 013-2.6.7.2 | Modified data elements for RESALE off OASIS, added requirement for bid price, and added requirement for executed service agreement |
| 09/25/2007 | 013-2.6.8 | Section and subsections added to document Transfers. |
| 09/25/2007 | 013-3.2 | Include new ANNOTATION data element associated with security events. |
| 09/25/2007 | 013-3.3 | New section on systemdata template with subsections describing posting requirements for path ATC, and load data. |
| 09/25/2007 | 013-3.4 | New section and subsections on security template describing requirements for posting and examples of other uses of this template. |
| 09/25/2007 | 013-3.5 | New section describing transoffering template handling of offers posted for resale. |
| 09/25/2007 | 013-3.6 | New section describing transpost/transupdate template handling of posting and updating offers for resale. |
| 09/25/2007 | 013-3.7 | New section describing transstatus template reporting of reservation profiles, |
| 09/25/2007 | 013-3.8 | New section describing reduction template reporting of reservation impacts on capacity or priority, |
| 09/25/2007 | 013-4.1.8 | New section documenting file response format for the reduction template. |
| 09/25/2007 | 013-4.1 | Revised all file examples to reflect changes made to template structures for Version 1.5. |

Business Practices Requirements:

013-1	INTRODUCTION

This OASIS Implementation Guide establishes general and specific transaction processing requirements and related business processes required for OASIS.  The technical standards for OASIS are defined in Standards WEQ 002 and WEQ 003, and the companion business practice standards are defined in Standards WEQ 001.

In the event of a conflict between a Primary Provider’s Tariff, applicable business practices, and this Implementation Guide, the Tariff shall take precedence over all, and business practices shall take precedence over this document.  A Primary Provider is not obligated to implement a particular requirement in this Implementation Guide if the Primary Provider’s Tariff does not contain the associated provision or the Primary Provider has received a waiver.

This version of the OASIS Implementation Guide corresponds to the OASIS Standards and Communication Protocols (S&CP) Version 1.5.

013-1.1	Usage of Terms

The following terms as used throughout this Implementation Guide are to be interpreted as follows:

OASIS – Refers to the Primary Provider’s implementation of the OASIS Transmission Customer interface as defined in Standards WEQ 002, and any back-end supporting systems or user procedures that collectively perform the transaction processing functions associated with handling of requests on OASIS.

Business Practice – Refers collectively to any business practices adopted by the Primary Provider as defined in their Open Access Transmission Tariff (OATT), NAESB ratified Business Practice Standards, or provider specific practices or requirements.

Template – Refers generically, or by reference to a specifically named template, to the templates defined for the Transmission Customer interface to OASIS in Standards WEQ 002, including the displays and forms associated with the web browser based user interface implementing the functions of an OASIS defined template.

Data Element – Refers generically, by either full name or alias, to a data element defined in the Data Dictionary (WEQ 003).  No distinction between the use of full name or alias is intended.

Must, shall, or required – Define specific requirements for OASIS processing that are not optional and must be implemented as described.

May, should, or optional – Define optional requirements that are recommended for implementation in OASIS but are not specifically required under these Standards.

Additional terms defined in Standards WEQ 001 and WEQ 002 are also hereby incorporated by reference.

013-2	OASIS TRANSACTION PROCESSING

The basic OASIS transaction process is described below.  This Implementation Guide also provides additional requirements and guidance for processing specific types of business transactions in the implementation of OASIS.  Note that the Primary Provider may, but is not limited to, interacting with OASIS using the Transmission Customer template or user interface.  Primary Providers may also implement OASIS functions on back-end systems and are not required to perform all transaction processing on an OASIS node proper, provided that the results of all transaction processing are correctly posted on OASIS as required by the Tariff, regulation, or other established business practices.

The following is a summary of the templates used and actions that may be taken by the Transmission Customer and Seller to execute a transaction on OASIS.  Note that the OASIS Standards require all template functionality to be provided through a User Interface.  While this discussion focuses on template execution, all actions must be supported through a browser-based User Interface. Detailed examples of the transaction process and description of the business logic envisioned to be implemented as part of the Primary Provider’s OASIS or other back-end transaction support services are provided in subsequent sections of this Implementation Guide.

- a. The transrequest and ancrequest templates shall be used by the Transmission Customer to enter a transaction request for specific transmission or ancillary services from a specified Seller.  All pertinent transaction-specific information must be provided in the template data elements.

- b. The transstatus and ancstatus templates shall be used by both Transmission Customer and Seller to query for the current transaction information (e.g., STATUS).  Alternatively, the Transmission Customer may request dynamic notification per WEQ 002-4.2.10.3 whenever the transaction data is changed.

c. The transsell and ancsell templates shall be used by the Seller to indicate to the Transmission Customer whether the request is acceptable or not by setting the transaction STATUS to one of RECEIVED, INVALID, STUDY, COUNTEROFFER, ACCEPTED, REFUSED, SUPERSEDED, DECLINED, DISPLACED, ANNULLED, or RETRACTED. A Primary Provider as the Seller may use the transsell and ancsell templates to act on requests or may use proprietary software solutions to perform this function in a similar manner.

d. The transcust and anccust templates shall be used by the Transmission Customer to indicate to the Seller whether they wish to negotiate, confirm or withdraw the transaction by setting the transaction STATUS to one of REBID, CONFIRMED, or WITHDRAWN.

- e. The transassign and ancassign templates shall be used by the Seller to notify the Primary Provider of the transfer of rights from the Seller to the Transmission Customer consummated off the OASIS Node.

- f. The source of all Transmission Customer and Seller contact information shall be provided under WEQ 002-3.1 REGISTRATION AND LOGIN REQUIREMENTS. Therefore, it shall not be input as part of uploads, but shall be provided as part of all transaction downloads.

- g. OASIS Nodes shall accept a Seller-initiated change in STATUS to ACCEPTED only when OFFER_PRICE matches BID_PRICE.

- h. OASIS Nodes shall accept a Transmission Customer-initiated change in STATUS to CONFIRMED only when BID_PRICE matches OFFER_PRICE.

- i. If CAPACITY_GRANTED is null when STATUS is being changed to ACCEPTED or CONFIRMED, the OASIS Node shall set it equal to CAPACITY_REQUESTED.

j. OASIS Nodes shall set the initial transaction STATUS of the request to QUEUED and assign a unique ASSIGNMENT_REF identifier for the transaction.

k. If the Transmission Customer has identified the transaction as PRECONFIRMED and the Seller has set the transaction STATUS to ACCEPTED, OASIS Nodes shall automatically set the transaction’s STATUS to CONFIRMED without any Transmission Customer interaction required. For transfers (PART_TRANSFER and FULL_TRANSFER), the Seller must have posted “ACCEPTED” in the STATUS and the Primary Provider must have posted “Y” in the   PRIMARY_PROVIDER_APPROVAL data elements respectively before the OASIS Nodes will automatically set the transaction’s STATUS to CONFIRMED.

l. If the Transmission Customer has identified the transaction as PRECONFIRMED and the Seller has set the transaction STATUS to COUNTEROFFER, OASIS Nodes shall take no automatic confirmation action on the transaction and require explicit confirmation by the Transmission Customer.

This transaction process flow is depicted in the diagram below.

Exhibit 1 Transaction Template Usage Diagram

The above sequence of actions is altered slightly in the implementation of REQUEST_TYPE of PART_TRANSFER or FULL_TRANSFER.  For these transactions, approval is required from both the Seller (Reseller) and the Primary Provider.  OASIS shall require that the Seller has indicated their approval of the transfer by setting the STATUS to ACCEPTED and the Primary Provider has indicated their approval by setting PRIMARY_PROVIDER_APPROVAL to Y prior to allowing the Customer to set the transaction status to CONFIRMED.

013-2.1	Transaction Request Types

The following are the valid OASIS transaction request types (template data element REQUEST_TYPE) that may be submitted by the Transmission Customer unless otherwise noted, along with a brief description of their intended use:

ORIGINAL = typical reservation requests submitted to the Primary Provider (as the Seller of the transmission or ancillary service)

RESALE = secondary market requests for assignment of scheduling rights submitted to a Transmission Customer as Secondary Provider

RENEWAL = request to renew an expiring transmission reservation that has rollover rights

MATCHING = request to meet or exceed a competing request to retain transmission service (right of first refusal)

DEFERRAL = request to defer or apply for an extension on start of transmission service

REDIRECT = request to redirect all or portion of a transmission reservation to an alternate POR/POD and/or make other changes to the terms of service as permitted

RELINQUISH = request to release all or a portion of the capacity of a Redirect on a Non-Firm basis to the Firm Parent Reservation

FULL_TRANSFER = request to transfer all capacity, rights, encumbrances and obligations, including financial liability to the Primary Provider, from one Transmission Customer to another.

PART_TRANSFER = request to transfer a portion, but not all, capacity, and all rights, and obligations, including financial liability associated with the transferred capacity to the Primary Provider, from one Transmission Customer to another.  No encumbrances (resales, etc) may be transferred with a PART_TRANSFER.

RECALL = request submitted by the Seller (Reseller or Primary Provider) to take back all or a portion of the capacity of a transmission reservation

{registered} = A Primary Provider may register values for REQUEST_TYPE to implement specific provisions of their Tariff.

This Implementation Guide contains detailed descriptions on the use of each transaction REQUEST_TYPE and explains the business processes to be implemented in association with each of these requests as specified by the OASIS Business Practice Standards, WEQ 001.

013-2.2	Transaction Status

The following are the defined values that may appear in the STATUS data element associated with a given OASIS transaction:

QUEUED = initial status assigned by TSIP on receipt of "customer services purchase request."

INVALID = assigned by TSIP, or Primary Provider indicating an invalid field in the request, such as improper POR, POD, source, sink, etc. (Final state)

RECEIVED = assigned by Primary Provider or Seller to acknowledge QUEUED requests and indicate the service request is being evaluated, including for completing the required ancillary services.

STUDY= assigned by Primary Provider or Seller to indicate some level of study is required or being performed to evaluate service request.

REFUSED = assigned by Primary Provider or Seller to indicate service request has been denied due to lack of available transfer capability. (Final state)

COUNTEROFFER = assigned by Primary Provider or Seller to indicate that a new value for OFFER_PRICE and/or CAPACITY_GRANTED over time is being proposed in the negotiation of requested service, i.e., offering of partial service or negotiation of price..

REBID = assigned by Transmission Customer to indicate that a new value for BID_PRICE and/or CAPACITY_REQUESTED over time is being proposed.

SUPERSEDED = assigned by Primary Provider or Seller when a request which has not yet been confirmed is preempted by another reservation request. (Final state)

ACCEPTED = assigned by Primary Provider or Seller to indicate the service request at the designated BID_PRICE and CAPACITY_REQUESTED has been approved/accepted. Depending upon the type of ancillary services required, the Seller may or may not require all ancillary service reservations to be completed before accepting a request.

DECLINED = assigned by Primary Provider or Seller to indicate that the terms and conditions of the request, such as the BID_PRICE, are unacceptable and that negotiations are terminated or that contractual terms and conditions have not been met. (Final state)

CONFIRMED = assigned by Transmission Customer in response to Primary Provider or Seller posting "ACCEPTED" status, to confirm service. Once a request has been "CONFIRMED," a transmission service reservation exists. (Final state, unless overridden by DISPLACED or ANNULLED state)

WITHDRAWN = assigned by Transmission Customer during in request evaluation to withdraw the request from any further action. (Final state)

DISPLACED = assigned by Primary Provider or Seller when a "CONFIRMED" reservation from a Transmission Customer is displaced by a higher priority reservation and the Transmission Customer is not offered or has not exercised right of first refusal (i.e. refused to match terms of new request). (Final state)

ANNULLED = assigned by the Seller when, by mutual agreement with the Transmission Customer, a confirmed reservation is to be voided, or assigned unilaterally by the Primary Provider when a confirmed reservation is to be voided. (Final state)

RETRACTED = assigned by Primary Provider or Seller when the Transmission Customer fails to confirm or withdraw the request within the required time period. (Final state)

The following state transition diagram should be used as a business process guideline and illustrates the valid changes that may be made to the STATUS value by the Seller and Transmission Customer during the transaction process; however, individual tariffs may dictate specific allowed actions between states that are not reflected in this diagram.

Exhibit 2 - State Diagram of Purchase Transactions

Pre-confirmed requests for Short term Firm and Non-Firm Point-to-Point service may not be withdrawn by the Customer unless the request has been counter offered by the Seller.  At the Customer’s request, however, the Transmission Provider may use its discretion and move the request from any STATUS value to ANNULLED to void the transaction. For clarity, this is not shown in the above exhibit. Pre-confirmed request for Long term Firm Point-to-Point service, Network service, and Point-to-Point redirects on a Non-Firm basis may be withdrawn by the Customer at any time prior to confirmation.

013-2.2.1	ACCEPTED Status Restrictions

OASIS shall block a request from being set to a STATUS of ACCEPTED by the Seller if the values for OFFER_PRICE and CAPACITY_GRANTED, do not exactly match the corresponding values for BID_PRICE and CAPACITY_REQUESTED over every START_TIME/STOP_TIME interval.  This ensures that both parties to the transaction are in agreement to the terms of the request.

If the request is flagged as PRECONFIRMED=YES, OASIS shall immediately set the request STATUS to CONFIRMED when it is ACCEPTED, with the exception of the PART_TRANSFER and FULL_TRANSFER requests.  For these two REQUEST_TYPEs, OASIS shall require that STATUS has been set to ACCEPTED and PRIMARY_PROVIDER_APPROVAL has been set to Y before moving the request to CONFIRMED.

013-2.2.2	CONFIRMED Status Restrictions

OASIS shall block a request from being set to a STATUS of CONFIRMED by the Customer (or automatically by OASIS if pre-confirmed) if the values for BID_PRICE and CAPACITY_REQUESTED do not exactly match the corresponding values for OFFER_PRICE and CAPACITY_GRANTED over every START_TIME/STOP_TIME interval.  This ensures that both parties to the transaction are in agreement to the terms of the request.

For PART_TRANSFER and FULL_TRANSFER requests, OASIS shall require that STATUS has been set to ACCEPTED and PRIMARY_PROVIDER_APPROVAL has been set to Y before allowing the Customer to set STATUS to CONFIRMED.

013-2.3	Basic OASIS Transaction Handling

Requests to reserve or purchase transmission or ancillary service shall be submitted to OASIS by the Transmission Customer via the transrequest or ancrequest templates.

The Seller specified in the request must be the Primary Provider for REQUEST_TYPE of ORIGINAL, REDIRECT, RELINQUISH, RENEWAL, or DEFERRAL.  The Seller specified in the request must be a registered entity other than the Primary Provider for REQUEST_TYPE of RESALE, FULL_TRANSFER or PART_TRANSFER.  The Seller may be either the Primary Provider or another registered entity for REQUEST_TYPE of MATCHING.

OASIS should screen submitted requests to validate proper use of REQUEST_TYPE.  Additional restrictions based on specific REQUEST_TYPEs are detailed in subsequent Standards.  Validations on the service requested, service start time and duration, submission time, etc., are established by business practice.

Once successfully submitted on OASIS, the Seller may take any of the following actions via the transsell/ancsell template:

- Acknowledge receipt by setting STATUS to RECEIVED or STUDY

- Deny the request by setting STATUS to INVALID, DECLINED, or REFUSED

- Approve the request by setting STATUS to ACCEPTED or COUNTEROFFER

At any time during the processing of a request, the Transmission Customer may set STATUS to WITHDRAWN to remove the request from further consideration by the Seller with the exception of certain pre-confirmed requests as specified in WEQ 013-2.2.

Once the Seller approves the request, the Transmission Customer may take any of the following actions via the transcust/anccust template:

- WITHDRAW the request

- Continue negotiation of the request by setting STATUS to REBID

- Complete the request by setting STATUS to CONFIRMED

Prior to final confirmation by the Transmission Customer, the Seller may override their approval of the request with the following actions:

- Retract approval based on exceeding of Transmission Customer confirmation time limits and/or scheduling deadlines or other criteria established by business practice by setting the STATUS to RETRACTED

- Retract approval based on receipt of a higher priority competing request by setting the STATUS to SUPERSEDED

Once CONFIRMED on OASIS by the Transmission Customer a service reservation shall be deemed to exist.  The Seller or the Primary Provider makes the following changes to a CONFIRMED service reservation:

- Nullify the reservation for cause by setting the STATUS to ANNULLED

- Displace the reservation in-whole to accommodate a higher priority competing request by setting the STATUS to DISPLACED

Seller’s shall provide a reason in the SELLER_COMMENTS whenever a service request is set to the STATUS of INVALID, REFUSED, DECLINED, RETRACTED, SUPERSEDED, ANULLED or DISPLACED.  The Primary Provider, when not acting as the Seller, shall provide a reason in the PROVIDER_COMMENTS whenever a service request is set to the STATUS of ANNULLED or DISPLACED.

013-2.4	Displacement

Displacement of an OASIS reservation may occur when a higher priority transmission service request is received by the Primary Provider and there is insufficient Available Transfer Capability (ATC) to honor that request, but capacity could be made available through the displacement of lower priority requests/reservations.  The following Standards describe the business process to be implemented on OASIS when this occurs.

013-2.4.1	Displacement – No Right of First Refusal

Confirmed transmission reservations may be subject to displacement in the event competing, higher priority requests are received by the Primary Provider.

If the existing Transmission Customer does not have the right of first refusal and all capacity from that Transmission Customer’s confirmed reservation is required to accommodate the higher priority request, the Primary Provider shall set the existing reservation's STATUS to DISPLACED. The STATUS of DISPLACED indicates that the reservation has been displaced in its entirety.

If only a portion of the confirmed reservation's capacity is required to accommodate the higher priority request, the Primary Provider shall document the recall of reserved capacity from the lower priority confirmed reservation by incrementing the IMPACTED counter on that reservation and posting on OASIS the amount and time frames over which that reservation's capacity was reduced, i.e., a partial displacement. The Transmission Customer may view all impacts to existing transmission service reservations (e.g., partial displacements, secondary sales, etc.) using the reduction template.

A reference to the competing transmission service request that forced the displacement should be entered in the SELLER_COMMENTS field of the displaced reservation.

013-2.4.2	Displacement – With Right of First Refusal

Confirmed transmission reservations may be subject to displacement in the event competing, higher priority requests are received by the Primary Provider, but have the right of first refusal to retain their transmission service.

If the Primary Provider's Tariff obligates, or the Primary Provider elects to grant the original Transmission Customer the right of first refusal, the existing Transmission Customer shall be notified of the competing request. The Primary Provider shall set the existing lower priority reservation's COMPETING_REQUEST_FLAG to Y and update the SELLER_COMMENTS to include the ASSIGNMENT_REF associated with the higher priority competing request. These actions will initiate electronic notification, provided the Transmission Customer has elected to receive such notification.

If the existing Transmission Customer elects to meet the terms and conditions of the competing request,  that Transmission Customer shall submit a new MATCHING reservation request using the transrequest template.  The specific requirements associated with submission of MATCHING requests are detailed in Standard 013-2.6.3.

If the Primary Provider accepts and the Transmission Customer confirms the MATCHING request, the Primary Provider shall set the STATUS of the competing request to REFUSED and set the STATUS of the existing lower priority confirmed reservation to DISPLACED. The STATUS of DISPLACED indicates that the reservation has been displaced in its entirety and has been replaced by the confirmed MATCHING reservation.

If the existing Transmission Customer does not elect to meet the terms of the competing request, the Primary Provider shall displace the existing lower priority reservation, in whole or in part, in the same manner described for displacement of reservations that are not extended a right of first refusal.

Once the result of the competition is resolved, whether through MATCHING or displacement of the existing reservation, the Primary Provider shall reset the COMPETING_REQUEST_FLAG to N in the reservation subject to displacement.

013-2.5	Primary Provider Recalls

There are cases in implementing provisions of the Primary Provider's Tariff that the capacity reserved by a Transmission Customer may be reduced in whole or in part, e.g. Displacement or Interruption. The particular reasons for these reductions are Tariff specific.

The Primary Provider shall provide a mechanism to post on OASIS any such reductions or recalls in reserved capacity. The Transmission Customer shall be notified of any and all such reductions in reserved capacity by the incrementing of the IMPACTED counter in association with those reservations that are reduced; the IMPACTED flag is viewable with the transstatus template.  Specific information regarding the exact nature of each reduction in the reserved capacity under a given transmission reservation shall be posted and viewable with the reduction template.

A specific example of a Primary Provider initiated recall of reserved capacity is the implementation of a partial displacement of a transmission reservation. In this instance, the Transmission Customer has not elected (or was not required to be offered) to match the terms of a higher priority, competing request. The Primary Provider recalls that capacity necessary to accommodate the higher priority request from the existing lower priority reservation. The IMPACTED counter of that reservation is incremented, and a query using the reduction template for that reservation would show the Transmission Customer the amount and time-frame over which the Transmission Customer's reserved capacity was recalled by the Primary Provider.

Interruption of transmission service, where that interruption directly impacts the rights of the Transmission Customer to schedule any service under that reservation, is another example of an impact to reserved capacity that would be posted as a Primary Provider initiated recall of reserved capacity.   Secondary market sales of transmission rights are not examples of a Primary Provider initiated recall of reserved capacity, but the impact of any such sales shall also be returned in response to execution of the reduction template.

The Primary Provider may elect to post recalls of reserved capacity using the OASIS REQUEST_TYPE of RECALL.  Documenting Primary Provider initiated recalls of capacity with a RECALL request is optional; posting the impacts of those recalls to be made available under the reduction template is mandatory.  If the RECALL request is used by the Primary Provider, its use must be implemented in compliance with the Standards for RECALL requests.

013-2.6	Transaction Specific Handling

The following Standards identify specific OASIS data elements and processing requirements that must be implemented by OASIS and/or associated back-end support systems.  The results of all transaction processing shall be viewable by all appropriate entities via the transstatus/ancstatus templates and corresponding OASIS User Interface.

013-2.6.1	ORIGINAL Requests

ORIGINAL requests shall be submitted by the Transmission Customer to arrange for new transmission or ancillary service with the Primary Provider.

The following are specific restrictions or requirements for OASIS service requests with REQUEST_TYPE of ORIGINAL.  If REQUEST_TYPE is not specified by the Transmission Customer and SELLER_CODE and SELLER_DUNS are the same as PRIMARY_PROVIDER_CODE and PRIMARY_PROVIDER_DUNS, OASIS shall default REQUEST_TYPE to ORIGINAL.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be ORIGINAL |
| RELATED_REF | Must be null |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

Additional requirements related to the specification of service points, attributes, pricing, and timing are subject to the Primary Provider’s business practices.

The Transmission Customer may submit a time varying profile of capacity as allowed by the Primary Provider’s business practice by repeating the template data elements of BID_PRICE, CAPACITY_REQUESTED, START_TIME and STOP_TIME in template continuation records.  The segments of any submitted profile must not overlap in time.

Once submitted by the Customer the earliest START_TIME and latest STOP_TIME defines the overall term of service.  Any further negotiation or offer of partial service may modify the segments/profiles times, but any such modifications must span the entire period from the original earliest START_TIME to latest STOP_TIME.

013-2.6.1.1	Offering of Partial Service

If in the evaluation of a transmission request, the Primary Provider determines that only a portion of the Transmission Customer's requested capacity (CAPACITY_REQUESTED data element) can be accommodated and that the Primary Provider is obligated or elects to offer the Transmission Customer only a portion of the requested capacity, the Primary Provider shall set the values for START_TIME, STOP_TIME, CAPACITY_GRANTED, and OFFER_PRICE as appropriate, and set the request’s STATUS to COUNTEROFFER.

If the CAPACITY_REQUESTED and/or CAPACITY_GRANTED are not constant over time, continuation records shall be used to convey the time varying profile of MW capacity associated with the transmission request.  The profile of CAPACITY_GRANTED and OFFER_PRICE must span the entire START_TIME to STOP_TIME interval initially requested by the Transmission Customer even if CAPACITY_GRANTED is zero.

The Transmission Customer shall recognize the offer of partial service by CAPACITY_GRANTED not being equal to CAPACITY_REQUESTED and the request STATUS of COUNTEROFFER. The Transmission Customer may elect to CONFIRM, WITHDRAW, or REBID for the partial service using the transcust template.  To rebid for partial service the customer shall specify the revised START_TIME, STOP_TIME, CAPACITY_REQUESTED and BID_PRICE values and set the transaction status to REBID using the transcust Template.  OASIS shall restrict CAPACITY_REQUESTED on a rebid to not exceed the Seller’s most recent CAPACITY_GRANTED over time.

If the transmission reservation request was marked PRECONFIRMED by the Transmission Customer and an offer of partial service is extended, the reservation request must be explicitly CONFIRMED by the Transmission Customer.  The OASIS node shall not automatically confirm a request where CAPACITY_REQUESTED and/or BID_PRICE does not equal CAPACITY_GRANTED and/or OFFER_PRICE over time when/if an attempt is made to set STATUS to ACCEPTED.

The Primary Provider shall use this same process in handling the deferral to the start of transmission service due to delays in completing the necessary transmission system studies associated with the request.  In these cases, the Primary Provider shall document the deferral by setting an initial transmission service profile record with CAPACITY set to zero MWs and START_TIME set to the originally requested START_TIME and STOP_TIME coincident with the delayed start of service.  The Primary Provider shall then specify the capacity to be made available to the Transmission Customer in one or more subsequent transmission service profile continuation records by defining START_TIME, STOP_TIME CAPACITY_GRANTED and OFFER_PRICE as appropriate.

Note that OASIS must verify that the values for CAPACITY_REQUESTED and BID_PRICE match the values for CAPACITY_GRANTED and OFFER_PRICE over each START_TIME/STOP_TIME interval, and block any attempt to set request status to ACCEPTED or CONFIRMED if these Data Elements are not equal.  This ensures that both parties to the transaction agree to the final term and price of service.

013-2.6.1.2	Negotiation of Price

Negotiation of price is initiated by the Transmission Customer submitting a service request (via transrequest/ancrequest) with a BID_PRICE that is different (higher or lower) from the currently posted offer price, or the tariff rate, for that service.  The following negotiation process is required where the Seller is the Primary Provider.  Resales or transfers between Transmission Customers may use this process, but there is no obligation on the (Re)Seller to offer a negotiated rate to other Transmission Customers.

If the Seller determines that the BID_PRICE is acceptable, the following actions must be taken (via transsell/ancsell):

- Update the currently posted offer price for the service requested and all other applicable services offered as dictated by current discounting policy (e.g., all unconstrained paths to the same point of delivery) to match BID_PRICE;

- Update the request’s NEGOTIATED_PRICE_FLAG to L or H if the BID_PRICE was lower than or higher than, respectively, the posted price when the request was submitted;

- Set the OFFER_PRICE equal to the BID_PRICE;

- Set the CAPACITY_GRANTED appropriately (if left null or undefined, OASIS shall set CAPACITY_GRANTED equal to CAPACITY_REQUESTED when STATUS is set to ACCEPTED);

- Set the request STATUS to ACCEPTED (or COUNTEROFFER if offering partial service)

The Transmission Customer may then confirm the purchase or withdraw the request by updating the request STATUS (via transcust/anccust).

If the Seller determines that the BID_PRICE is unacceptable, and negotiation of price is not going to be entertained, the Seller shall set the request STATUS to DECLINED (via transsell/ancsell):

If the Seller elects to enter into price negotiation, the following actions must be taken (via transsell/ancsell):

- If the price to be counter offered by the Primary Provider to the Transmission Customer is different than the currently posted offer price:

  - Update the currently posted offer price for the service requested and all other applicable services offered as dictated by current discounting policy (e.g., all unconstrained paths to the same point of delivery) to match the price to be counteroffered;

  - Update the request’s NEGOTIATED_PRICE_FLAG to L or H if the price to be counter offered is lower than or higher than, respectively, the posted price when the request was submitted;

- Set the OFFER_PRICE and CAPACITY_GRANTED appropriately;

- Set the request STATUS to COUNTEROFFER.

The Transmission Customer may then confirm the purchase, withdraw the request, or propose a new BID_PRICE by performing the following (via transcust/anccust):

- Update the request BID_PRICE appropriately;

- Set the request STATUS to REBID or CONFIRMED.

If the Customer has set the status to REBID, the Seller may then act on the new BID_PRICE and/or CAPACITY_REQUESTED over time by declining the request, accepting the BID_PRICE/CAPACITY_REQUESTED, or counter offering a new OFFER_PRICE and/or CAPACITY_GRANTED over time using the same sequence of actions as stated above.

Negotiation of price may also be initiated on receipt of a request for similar service submitted with a higher BID_PRICE.  If required by business practice, the Seller (Primary Provider) may update any ACCEPTED but unconfirmed requests to COUNTEROFFER with the associated OFFER_PRICE set to meet the higher received BID_PRICE, and the negotiation of price can proceed as described above.

Note that OASIS must verify that the values for CAPACITY_REQUESTED and BID_PRICE match the values for CAPACITY_GRANTED and OFFER_PRICE over each START_TIME/STOP_TIME interval, and block any attempt to set request status to ACCEPTED or CONFIRMED if these Data Elements are not equal.  This ensures that both parties to the transaction agree to the final term and price of service.

013-2.6.1.3	Rollover Rights

For transmission services that have ongoing rollover or renewal rights, the Transmission Provider shall document those rights and any limitations over time through the transstatus template data elements RENEWAL_DUE_TIME, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY.  Continuation records shall be used if the ROLLOVER_CAPACITY value changes as a function of time.  The first value for ROLLOVER_START_TIME shall be set to the value of the latest STOP_TIME value associated with the request/reservation.  If there is no limitation on rollover rights, ROLLOVER_STOP_TIME shall be null.

013-2.6.2	RENEWAL Requests

Transmission Customers shall use the REQUEST_TYPE of RENEWAL to exercise rollover rights associated with an existing transmission service reservation held by the Transmission Customer.  RENEWAL requests must always specify the Primary Provider as SELLER.

The following are specific restrictions or requirements for OASIS service requests with REQUEST_TYPE of RENEWAL.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be RENEWAL |
| RELATED_REF | Must specify the ASSIGNMENT_REF associated with an existing confirmed transmission service reservation held by the Transmission Customer that 1) has rollover rights, and 2) whose rollover rights have not expired. |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| PATH | Must represent the same corresponding service points in the reservation specified in RELATED_REF |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must specify a set of valid transmission service attributes recognized by the Primary Provider as a valid service designation eligible for exercising of rollover rights held by the reservation specified in RELATED_REF |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must match the STOP_TIME of the reservation specified in RELATED_REF |
| STOP_TIME | With START_TIME, must specify a valid interval of service eligible for exercising of renewal/rollover rights held by the reservation specified in RELATED_REF |
| CAPACITY_REQUESTED | Must be less than or equal to the amount of capacity eligible for renewal/rollover over the interval of service |
| BID_PRICE | Must specify the price to be paid for the service requested |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

RENEWAL requests must be submitted on OASIS prior to expiration of the Transmission Customer’s rollover rights as established by the Tariff orbusiness practice.

CUSTOMER_CODE and CUSTOMER_DUNS in the RENEWAL request should correspond to the CUSTOMER_CODE and CUSTOMER_DUNS in the RELATED_REF reservation.  If not, the Transmission Provider should verify that the submitting Customer has a valid agency agreement with the original transmission service agreement holder and is authorized to submit such a request on behalf of that entity.

The transmission service attributes, e.g., TS_CLASS, etc., should match the corresponding attributes in the reservation specified in RELATED_REF.  However, changes may be made to these attributes over time such that some differences are necessary to accommodate changes in the Primary Provider’s business practices.  This also applies to changes in service points, e.g., PATH, etc., over time.

RENEWAL requests may be subject to offering of partial service and negotiation and limitation of on-going rollover rights just as an ORIGINAL request.

013-2.6.3	MATCHING Requests

Transmission Customers shall use the REQUEST_TYPE of MATCHING to exercise right of first refusal to avoid being displaced by a higher priority competing request.

The following are specific restrictions or requirements for OASIS service requests with REQUEST_TYPE of MATCHING.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be MATCHING |
| RELATED_REF | Must specify the ASSIGNMENT_REF associated with an existing confirmed transmission service reservation held by the Transmission Customer that is subject to displacement. |
| PATH | Must represent the same corresponding service points in the reservation specified in RELATED_REF |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must specify a set of valid transmission service attributes that meet the requirements to exercise right of first refusal |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must specify the new requested time for the start of transmission service |
| STOP_TIME | With START_TIME, must specify a valid interval of service based on the transmission service attributes |
| CAPACITY_REQUESTED | Must be equivalent to the amount of capacity required to retain the Customer’s rights originally held on RELATED_REF |
| BID_PRICE | Must specify the price to be paid for the service requested |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

013-2.6.4	DEFERRAL Requests

The DEFERRAL request shall be used by Transmission Customers to request a delay in the start of an existing transmission service reservation held by the Transmission Customer.  DEFERRAL requests must always specify the Primary Provider as SELLER.

The following are specific restrictions or requirements for OASIS service requests with REQUEST_TYPE of DEFERRAL.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be DEFERRAL |
| RELATED_REF | If submitted by the Transmission Customer, must specify the ASSIGNMENT_REF associated with an existing confirmed transmission service reservation held by the Transmission Customer whose start time is to be delayed/deferred. If posted by the Primary Provider, must specify the ASSIGNMENT_REF associated with a pending transmission service request submitted by the Transmission Customer whose start time is to be deferred. |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| PATH | Must represent the same corresponding service points in the reservation/request specified in RELATED_REF |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must represent the same set of valid transmission service attributes recognized by the Primary Provider as a valid service designation specified in the reservation/request specified in RELATED_REF |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must specify the new requested time for the start of transmission service and must be greater than the START_TIME of the reservation/request specified in RELATED_REF |
| STOP_TIME | With START_TIME, must specify a valid interval of service based on the transmission service attributes |
| CAPACITY_REQUESTED | Must be equivalent to the amount of capacity originally reserved/requested |
| BID_PRICE | Must specify the price to be paid for the service requested |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

When submitted by the Transmission Customer and ultimately approved and confirmed; OASIS or the Primary Provider procedurally must set the status of the reservation specified in RELATED_REF to ANNULLED.  The DEFERRAL request becomes the new transmission service reservation reflecting the delay in start of service requested.

The Primary Provider shall evaluate pending DEFERRAL requests relative to other transmission requests based on the TIME_QUEUED of the Transmission Customer’s ORIGINAL request for service.

013-2.6.5	REDIRECT Requests

The REDIRECT request is submitted by an existing firm point-to-point Transmission Customer to request the use of alternate points of receipt and/or delivery from the Primary Provider.  By definition, the Seller in a REDIRECT request must be the Primary Provider even if those rights being redirected were acquired from another Transmission Customer via resale or transfer.

The following Standards set forth the requirements for submission of REDIRECT requests on either a Firm or Non-Firm basis.

013-2.6.5.1	REDIRECT on a Firm Basis

A Transmission Customer holding confirmed firm point-to-point transmission rights may request the use of those rights on alternate points of receipt and/or delivery on a firm basis by submission of a REDIRECT request to the Primary Provider as Seller.  The following information must be submitted by the Transmission Customer in the REDIRECT request via the transrequest template.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be REDIRECT |
| RELATED_REF | Must identify by ASSIGNMENT_REF a confirmed transmission service reservation for firm point-to-point service held by the submitting Transmission Customer |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| PATH | Must represent the new transmission service points being requested |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must represent a set of valid transmission service attributes for firm point-to-point service offered by the Primary Provider and being requested on the new service points by the Transmission Customer |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must specify the requested start of transmission service and must be within the bounds of START_TIME specified in the reservation identified in RELATED_REF |
| STOP_TIME | Must specify the requested stop/end of transmission service and must be within the bounds of STOP_TIME specified in the reservation identified in RELATED_REF, and with START_TIME must represent a valid interval of service for the firm point-to-point service being requested |
| CAPACITY_REQUESTED | Must specify the amount of transmission capacity being requested |
| BID_PRICE | Should specify the price for the service being requested; may be null |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

The Primary Provider shall evaluate each REDIRECT on a Firm basis as any other new request for firm point-to-point transmission service.  Primary Provider business practices establish the requirements for service duration, submission time, evaluation time, confirmation time, etc.

OASIS or Primary Provider procedures should verify that the transmission reservation identified in RELATED_REF meets all the requirements to support the redirect of transmission rights to the new service points.  This should include the validation that the current rights, the Capacity Available for Redirect, held on that reservation in the amount of the REDIRECT over time have not been encumbered by any other confirmed redirects, resales, schedules, etc.  This capacity validation may occur at any point in the request process, but shall always be performed prior to setting the REDIRECT STATUS to CONFIRMED.

Once CONFIRMED, the transmission rights held on the RELATED_REF reservation in the amount of the REDIRECT shall be permanently released by the Primary Provider and conveyed to the REDIRECT reservation. The only mechanism for the Transmission Customer to return to the original points of receipt and/or delivery is to submit another REDIRECT request.

The impact on ATC for the reservation identified by RELATED_REF shall be released and the impact of the REDIRECT transaction on ATC shall be accounted for in the amount and over time of the redirect simultaneously.

The impact of the REDIRECT transaction on the reservation(s) identified by RELATED_REF shall be posted and viewable using the reduction  template.

If the REDIRECT is eligible for rollover/renewal rights, these rights shall be communicated through the transstatus template data elements RENEWAL_DUE_TIME, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY.  Conveyance of rollover rights to the REDIRECT request/reservation may have an impact on those rights held on the RELATED_REF reservation.  These impacts on the RELATED_REF reservation shall be documented through an update to the ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY data elements associated with the RELATED_REF.

013-2.6.5.2	REDIRECT on a Non-Firm Basis

A Transmission Customer holding confirmed firm point-to-point transmission rights may request the use of those rights on alternate points of receipt and/or delivery on a non-firm basis by submission of a REDIRECT request to the Primary Provider as Seller.  The following information must be submitted by the Transmission Customer in the REDIRECT request via the transrequest template.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be REDIRECT |
| RELATED_REF | Must identify by ASSIGNMENT_REF a confirmed transmission service reservation for firm point-to-point service held by the submitting Transmission Customer |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| PATH | Must represent the new transmission service points being requested |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must be HOURLY |
| TS_CLASS | Must be SECONDARY |
| TS_TYPE | Must be POINT_TO_POINT |
| TS_PERIOD | Must be FULL_PERIOD |
| TS_WINDOW | Must be FIXED |
| TS_SUBCLASS | Must be null |
| START_TIME | Must specify the requested start of transmission service and must be within the bounds of BID_START_TIME specified in the reservation identified in RELATED_REF |
| STOP_TIME | Must specify the requested stop/end of transmission service and must be within the bounds of BID_STOP_TIME specified in the reservation identified in RELATED_REF, and with BID_START_TIME must represent a valid interval of service for the firm point-to-point service being requested |
| CAPACITY_REQUESTED | Must specify the amount of transmission capacity being requested |
| BID_PRICE | Should specify the price of $0.00 |
| PRECONFIRMED | Must be YES |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

Primary Provider business practices establish the requirements for service duration, submission time, evaluation time, confirmation time, etc.

OASIS or Primary Provider procedures should verify that the transmission reservation identified in RELATED_REF meets all the requirements to support the redirect of transmission rights to the new service points.  This should include the validation that the current rights, the Capacity Available for Redirect, held on that reservation in the amount of the REDIRECT over time have not been encumbered by any other confirmed redirects, resales, schedules, etc.  This capacity validation may occur at any point in the request process, but shall always be performed prior to setting the REDIRECT STATUS to CONFIRMED.

Once CONFIRMED, the transmission rights held on the RELATED_REF reservation in the amount of the REDIRECT shall be removed from the Capacity Available for Redirect on that reservation. Should the Transmission Customer wish to restore the rights to schedule (or redirect, resell, etc.) under the RELATED_REF reservation, they must submit a RELINQUISH request to the Primary Provider to release the capacity held under the REDIRECT request and reinstate that capacity on the original service  points. The Primary Provider must accommodate this release of capacity back to the original service points, and shall through OASIS or Primary Provider procedures insure that honoring this release of capacity and subsequent scheduling under the Transmission Customer’s original firm reservation will not unduly cause a reliability problem.

The impact on ATC for the reservation identified by RELATED_REF and the impact of the REDIRECT transaction on ATC shall be accounted for to both preserve the ability of the Transmission Customer to return to the original firm path and reflect the fact that the redirected capacity on the RELATED_REF and REDIRECT reservations cannot be scheduled simultaneously.

The impact of the REDIRECT transaction on the reservation(s) identified by RELATED_REF shall be posted and viewable using the reduction template.

013-2.6.6	RELINQUISH Requests

The RELINQUISH request is submitted in association with a REDIRECT request on a non-firm basis to indicate the Transmission Customer’s desire to return the capacity rights held on the REDIRECT to the parent reservation specified in the RELATED_REF of that REDIRECT request.  The following are the specific requirements for the RELINQUISH request submitted via the transrequest template.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be RELINQUISH |
| RELATED_REF | Must specify the ASSIGNMENT_REF of a confirmed REDIRECT on a non-firm basis held by the Transmission Customer |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| PATH | Must represent the same corresponding service points in the reservation/request specified in RELATED_REF |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must represent the same set of valid transmission service attributes specified in RELATED_REF |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must specify start of the interval for the release of capacity from RELATED_REF |
| STOP_TIME | Must specify stop/end of the interval for the release of capacity from RELATED_REF |
| CAPACITY_REQUESTED | Must specify the capacity to be released from RELATED_REF over BID_START_TIME to BID_STOP_TIME interval and restored to the parent reservation (i.e., RELATED_REF’s RELATED_REF) |
| BID_PRICE | Should specify the price of $0.00 |
| PRECONFIRMED | Must be YES |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

When accepted by the Primary Provider, OASIS or Primary Provider procedures must remove the capacity held on the REDIRECT reservation over the time interval specified, and reinstate that capacity to the CAPACITY_AVAILABLE for REDIRECT of the parent firm reservation.

The combined impact of the REDIRECT and RELATED_REF reservations on ATC shall be reinstated to reflect the impact of the capacity relinquished back to the RELATED_REF reservation.

Note that the RELINQUISH request does not in itself represent a reservation for transmission service, but merely documents an action taken against an existing transmission service reservation.

013-2.6.7	RESALE Requests

RESALE requests may be submitted by the Transmission Customer (Assignee) to arrange for the sale or assignment of scheduling rights from another Transmission Customer (Reseller). Resellers and Assignees are not required to use OASIS to arrange for such sale or assignment of scheduling rights.  However, the Reseller is required to notify the Primary Provider and post information documenting the (re)sale on the Primary Provider’s OASIS where the rights were originally acquired.

The following are specific requirements for OASIS service requests with REQUEST_TYPE of RESALE.  If REQUEST_TYPE is not specified by the Transmission Customer and SELLER_CODE and SELLER_DUNS are not the same as PRIMARY_PROVIDER_CODE and PRIMARY_PROVIDER_DUNS, OASIS shall default REQUEST_TYPE to RESALE.

013-2.6.7.1	RESALE on OASIS

Resale transactions conducted on OASIS shall adhere to the Basic OASIS Request Processing requirements where the Reseller is identified as the Seller and the Assignee identified as the Transmission Customer.

The Assignee (Transmission Customer) initiates the resale of scheduling rights by submitting the following required information on OASIS via the transrequest template.  Data elements not listed are optional. There shall be no requirement imposed by OASIS that the Reseller post any corresponding offer of service for sale on that OASIS.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be RESALE |
| SELLER_CODE | Must identify the Reseller (registered entity other than the Primary Provider) |
| SELLER_DUNS | Must identify the Reseller |
| PATH | Must represent the transmission service points being requested |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must represent a set of valid transmission service attributes offered by the Primary Provider and being requested from the Reseller |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must specify the requested start of transmission service |
| STOP_TIME | Must specify the requested stop/end of transmission service |
| CAPACITY_REQUESTED | Must specify the amount of transmission capacity being requested |
| BID_PRICE | Must specify the price being requested in $/MW-Hour |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

If the RESALE meets all OASIS validation requirements, the RESALE request shall be posted on OASIS as a QUEUED transmission service request.  It is the Reseller’s obligation to respond to RESALE requests submitted to them on OASIS.  Resellers may register for dynamic notification of these events if the Primary Provider’s OASIS implementation provides for such functionality.

The Reseller may act on the RESALE request using any of the processes defined in Section 2.3 Basic OASIS Transaction Handling, Section 2.6.1.1 Offering of Partial Service, and/or Section 2.6.1.2 Negotiation of Price.  At some point in this process, the Reseller must identify those transmission service reservations whose scheduling rights are to be conferred to the Assignee as follows.

| Data Element | Restriction/Requirement |
| --- | --- |
| REASSIGNED_REF | Collectively represent those confirmed transmission reservations held by the Reseller whose rights to schedule transmission capacity over time are to be conferred to the Assignee. Each reservation’s service points and transmission service attributes must correspond with those specified in the RESALE request |
| REASSIGNED_CAPACITY |  |
| REASSIGNED_START_TIME |  |
| REASSIGNED_STOP_TIME |  |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

Business practices may dictate that this information be supplied and validated at the time the RESALE request STATUS is set to either ACCEPTED or COUNTEROFFER.  Business practices may also dictate the types of services that may be resold and/or aggregated in support of the resale as referenced by the REASSIGNED_REF data element.

Once the RESALE request is finally CONFIRMED by the Assignee, OASIS or Primary Provider procedures should (again) insure that every reservation represented in the REASSIGNED_REF data element meet the following requirements:

- Represent valid confirmed transmission service reservations held by the Reseller

- There is sufficient unscheduled or otherwise encumbered (e.g., other RESALE, REDIRECT, etc., transactions)  transmission capacity to meet REASSIGNED_CAPACITY over REASSIGNED_START_TIME to REASSIGNED_STOP_TIME

- Service points, e.g., PATH, etc., correspond correctly with those specified in the RESALE

- Transmission service attributes, e.g., TS_CLASS, etc., correspond correctly with those specified in the RESALE

- The sum of REASSIGNED_CAPACITY over time for all REASSIGNED_REF reservations satisfies the RESALE requirements as specified in CAPACITY_GRANTED, START_TIME and STOP_TIME.

- The Assignee has executed a service agreement with the Transmission Provider that covers the terms of the RESALE.

If all of these requirements are met, OASIS or Primary Provider procedures shall convey the specified transmission rights from the Reseller’s reservation to the Assignee’s reservation.  The transmission capacity resold shall not be available to the Reseller to schedule, resell, redirect, or otherwise use.

The impact of the RESALE transaction on the reservation(s) identified by REASSIGNED_REF shall be posted and viewable using the reduction template.

If the Primary Provider determines the posted RESALE does not comply with their business practices for such transactions, they shall have the right to reset the RESALE STATUS to ANNULLED.  OASIS or Primary Provider procedures shall then reinstate all reassignments of capacity over time that were designated by the Assignee to the parent (REASSIGNED_REF) reservation(s) as appropriate.  The Reseller setting the RESALE STATUS to DISPLACED or ANNULLED shall also reinstate all reassignments of capacity over time that were designated by the Assignee to the parent (REASSIGNED_REF) reservation(s) as appropriate.

013-2.6.7.2	RESALE off OASIS

Resale transactions arranged between Reseller and Assignee off OASIS must be documented on OASIS by the Reseller using the transassign template.  Transactions arranged off OASIS do not follow the basic request processing steps and shall be posted directly as CONFIRMED transactions.

The following information is required to be supplied by the Reseller.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be RESALE |
| CUSTOMER_CODE | Must identify the Assignee |
| CUSTOMER_DUNS | Must identify the Assignee |
| PATH | Must represent the same corresponding service points as contained in every one of the reservation/request(s) specified by REASSIGNED_REF |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must represent the same corresponding transmission service attributes as contained in every one of the reservation/request(s) specified by REASSIGNED_REF |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must specify the start of transmission service arranged between Reseller and Assignee |
| STOP_TIME | Must specify the stop/end of transmission service arranged between Reseller and Assignee |
| CAPACITY_REQUESTED | Must be equal to the amount of capacity ultimately resold to the Assignee |
| BID_PRICE | Must be the ultimate price arranged for service between Reseller and Assignee, specified in $/MW-Hour |
| CAPACITY_GRANTED | Must be equal to the amount of capacity ultimately resold to the Assignee |
| OFFER_PRICE | Must be the ultimate price arranged for service between Reseller and Assignee, specified in $MW-Hour |
| REASSIGNED_REF | Collectively represent those confirmed transmission reservations held by the Reseller whose rights to schedule transmission capacity over time are to be conferred to the Assignee |
| REASSIGNED_CAPACITY |  |
| REASSIGNED_START_TIME |  |
| REASSIGNED_STOP_TIME |  |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

OASIS or Primary Provider procedures should insure that every reservation represented in the REASSIGNED_REF data element meets the following requirements:

- Represent valid confirmed transmission service reservations held by the Reseller

- There is sufficient unscheduled or otherwise encumbered (e.g., other RESALE, REDIRECT, etc., transactions)  transmission capacity to meet REASSIGNED_CAPACITY over REASSIGNED_START_TIME to REASSIGNED_STOP_TIME

- Service points, e.g., PATH, etc., correspond correctly with those specified in the RESALE

- Transmission service attributes, e.g., TS_CLASS, etc., correspond correctly with those specified in the RESALE

- The sum of REASSIGNED_CAPACITY over time for all REASSIGNED_REF reservations satisfies the RESALE requirements as specified in CAPACITY_GRANTED, START_TIME and STOP_TIME.

- The Assignee has executed a service agreement with the Transmission Provider that covers the terms of the RESALE.

Business practices may dictate the types of services that may be resold and/or aggregated in support of the resale as referenced by the REASSIGNED_REF data element.

If all of these requirements are met, OASIS or Primary Provider procedures shall convey the specified transmission rights from the Reseller’s reservation to the Assignee’s reservation.  The transmission capacity resold shall not be available to the Reseller to schedule, resell, redirect, or otherwise use.

The impact of the RESALE transaction on the reservation(s) identified by REASSIGNED_REF shall be posted and viewable using the reduction template.

If the Primary Provider determines the posted RESALE does not comply with their business practices for such transactions, they shall have the right to reset the RESALE STATUS to ANNULLED.  OASIS or Primary Provider procedures shall then reinstate all reassignments of capacity over time that were designated by the Assignee to the parent (REASSIGNED_REF) reservation(s) if appropriate.  The Reseller setting the RESALE STATUS to DISPLACED or ANNULLED shall also reinstate all reassignments of capacity over time that were designated by the Assignee to the parent (REASSIGNED_REF) reservation(s) as appropriate.

013-2.6.8	Transfer Requests

Transfer transactions conducted on OASIS shall adhere to the Basic OASIS Request Processing requirements where the Reseller is identified as the Seller and the Assignee identified as the Customer.

Transfer transactions shall use the same basic procedures, data elements and templates as specified for RESALE on OASIS (WEQ 013-2.6.6.1) or RESALE off OASIS (WEQ 013-2.6.6.2), with the exception of REQUEST_TYPE being either FULL_TRANSFER or PART_TRANSFER.

013-2.6.8.1	FULL_TRANSER - Transfers of All Capacity

A Transmission Customer (Reseller) may transfer all of their rights and obligations, including financial liability and encumbrances (e.g., resales), under the Primary Provider’s Tariff to another Transmission Customer (Assignee).  These requests must be initiated by the Assignee through submission of a transmission request with REQUEST_TYPE of FULL_TRANSFER and designation of the Reseller as SELLER.  The transmission service attributes in the TRANSFER request must exactly match those in the transmission reservation(s) held by the Reseller.  TRANSFER requests are handled by the Reseller and the Assignee using the standard Transaction Process as specified for RESALE requests.  OASIS may block submission of a TRANSFER request if the Assignee does not have an executed service agreement with the Primary Provider.

When approved by the Reseller, the Reseller must supply information as to the transmission service rights to convey to the Assignee via the REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME and REASSIGNED_STOP_TIME Data Elements. The transmission service rights being transferred must have been purchased from the Primary Provider (REQUEST_TYPE of ORIGINAL, RENEWAL, Firm REDIRECT or MATCHING with Primary Provider as SELLER) or transferred from another Transmission Customer. The aggregation of all REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME and  REASSIGNED_STOP_TIME Data Elements must match the capacity and time frame of the TRANSFER request as specified in the CAPACITY_GRANTED (and CAPACITY_REQUESTED), START_TIME and STOP_TIME  Data Elements of that transaction.

The Primary Provider must acknowledge and approve of the transfer of rights by setting PRIMARY_PROVIDER_APPROVAL to Y.  If for any reason the Primary Provider disapproves of the TRANSFER, the Provider must set the request’s PRIMARY_PROVIDER_APPROVAL to N, and all rights will remain with the Reseller.

The Primary Provider may post a FULL_TRANSFER request directly on OASIS on behalf of the Original TC and Assignee after confirming the transaction with both parties using the transassign template  The information required to be posted shall be identical to that posted for FULL_TRANSFERs conducted on OASIS.

On confirmation of the FULL_TRANSFER, the IMPACTED attributed will be incremented for each of the Reseller’s reservations referenced by the  REASSIGNED_REF Data Elements and the resulting impacts on each REASSIGNED_REF’s reserved capacity will be viewable with the reduction template.   OASIS must also update all subordinate RESALE, REDIRECT, etc., transactions impacting the REASSIGNED_REF transaction(s) to reflect the transfer of those obligations to the Assignee under the new FULL_TRANSFER reservation.

013-2.6.8.2 	PART_TRANSFER - Transfer of Partial Capacity

A Transmission Customer (Reseller) may transfer a portion of their rights and obligations, including financial liability, under the Primary Provider’s Tariff to another Transmission Customer (Assignee).  These requests must be initiated by the Assignee through submission of a transmission request with REQUEST_TYPE of PART_TRANSFER and designation of the Original Transmission Customer (Reseller) as SELLER.  The transmission service attributes in the TRANSFER request must exactly match those in the transmission reservation(s) held by the Original Transmission Customer.  TRANSFER requests are handled by the Original Transmission Customer (SELLER) and the Assignee (CUSTOMER) using the standard Transaction Process as specified for RESALE requests.  OASIS may block submission of a TRANSFER request if the Assignee does not have an executed service agreement with the Primary Provider.

When approved by the Reseller, the Reseller must supply information as to the transmission service rights to be conveyed to the Assignee via the REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME and REASSIGNED_STOP_TIME Data Elements. The transmission service rights being transferred must have been purchased from the Primary Provider (REQUEST_TYPE of ORIGINAL with Primary Provider as SELLER) or transferred from another Transmission Customer (REQUEST_TYPE of PART_TRANSFER or FULL_TRANSFER with that Transmission Customer as SELLER). The aggregation of all REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME and  REASSIGNED_STOP_TIME Data Elements must match the capacity and time frame of the TRANSFER request as specified in the ,START_TIME, STOP_TIME, and CAPACITY_GRANTED Data Elements of that transaction.  The CAPACITY_GRANTED must be less than or equal to the Capacity Available to Redirect on the REASSIGNED_REF.

The Primary Provider must acknowledge and approve of the transfer of rights by setting PRIMARY_PROVIDER_APPROVAL to Y.  If for any reason the Primary Provider disapproves of the TRANSFER, the Provider must set the request’s PRIMARY_PROVIDER_APPROVAL to N and all rights will remain with the Reseller.

The Transmission Provider may post a PART_TRANSFER request directly on OASIS on behalf of the Reseller and Assignee after confirming the transaction with both parties.  The information required to be posted shall be identical to that posted for PART_TRANSFERs conducted on OASIS.

On confirmation of the FULL_TRANSFER, the IMPACTED attribute will be incremented for each of the Original Transmission Customer’s reservations referenced by the REASSIGNED_REF Data Elements and the resulting impacts on each REASSIGNED_REF’s reserved capacity will be viewable with the reduction template.

013-3	SPECIFIC TEMPLATE IMPLEMENTATION

013-3.1	Registered Template Data Elements

Certain OASIS defined templates contain data elements that are identified as taking on values registered by the Primary Provider, e.g., reduction, systemdata, security, etc.  These registered values shall be included in the Energy Industry Registry when this registry is made publicly available.

In addition to public registration (when supported), each Primary Provider using a registered value for any OASIS data element shall provide information in their posted business practices as to each of these data element values along with a detailed description and example of how the data element will be used.  This information shall be made available on OASIS.

013-3.2	scheduledetail

The scheduledetail template shall be used to query specific information posted by the Primary Provider related to the scheduled usage of reserved transmission service.

For (transmission) schedules derived from implemented electronic tags (e-Tags) submitted in accordance with the North American Electric Reliability Corporation (NERC) Electronic Tagging Functional Specification, (ver.1.8.0) the following information must be provided on OASIS.

From the physical path segment of the e-Tag associated with the Primary Provider:

| Data Element | Restriction/Requirement |
| --- | --- |
| TRANSACTION_ID | The full e-Tag transaction identifier, including the GCA, creating PSE and LCA codes |
| PATH_NAME | Optional; defined by the Primary Provider based on POR/POD and transmission services used to support the schedule. |
| POINT_OF_RECEIPT | POR as identified in the e-Tag physical path |
| POINT_OF_DELIVERY | POD as identified in the e-Tag physical path |
| GCA_CODE | Registered acronym for the generating control area (balancing authority) |
| LCA_CODE | Registered acronym for the load control area (balancing authority) |
| SOURCE | Registered value of the source from the e-Tag located within the GCA |
| SINK | Registered value of the sink from the e-Tag located within the LCA |
| SCHEDULE_PRIORITY | The effective curtailment priority that will be used by the Primary Provider in assessing any curtailment action that may be taken against the schedule. |
| START_TIME/STOP_TIME | The time interval associated with the information associated with this segment of the schedule. |
| SCHEDULE_REQUESTED | The value from the PSE Market Level profile information contained in the e-Tag |
| SCHEDULE_GRANTED | The actual MW level to which the e-Tag was scheduled; for block schedules, SCHEDULE_GRANTED should always be the lower of SCHEDULE_REQUESTED and SCHEDULE_LIMIT; for dynamic schedules SCHEDULE_GRANTED may be higher than SCHEDULE_REQUESTED. |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

For each start/stop segment of the posted schedule, the following information shall be provided for each transmission service reservation that is used to support that segment of the schedule.  There may be one or more transmission service reservations used to support a given schedule segment.  These “stacked” reservations shall be communicated through continuation records as defined in the OASIS S&CP (WEQ 002).

| Data Element | Restriction/Requirement |
| --- | --- |
| ASSIGNMENT_REF | The unique OASIS identifier assigned to the reservation supporting the schedule |
| SELLER_CODE SELLER_DUNS | Identification of the Seller as listed in the transmission service reservation |
| CUSTOMER_CODE CUSTOMER_DUNS | Identification of the Transmission Customer as listed in the transmission service reservation |
| AFFILIATE_FLAG | Identification of the reservation as being made by an affiliate of the Primary Provider |
| SERVICE_INCREMENT TS_CLASS TS_TYPE TS_PERIOD TS_WINDOW TS_SUBCLASS NERC_CURTAILMENT_PRIORITY OTHER_CURTAILMENT_PRIORITY | The transmission service attributes and curtailment priority information as specified in the transmission service reservation |
| CAPACITY_USED | The actual MWs of reserved capacity used in support of the schedule derived from the transmission allocation information as specified in the e-Tag |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

If the tagged transaction has been subject to a Reliability Adjustment the following information shall be supplied.  This is typically, but not necessarily, indicated by SCHEDULE_GRANTED being less than SCHEDULE_REQUESTED.

| Data Element | Restriction/Requirement |
| --- | --- |
| PROVIDER_ACTION | As specified in the OASIS Data Dictionary, text descriptive of the action being taken by the Primary Provider, e.g., CURTAILMENT. |
| SCHEDULE_LIMIT | The value of the Reliability Limit set against the tagged transaction by any Reliability Entity over this segment of the schedule |
| CURTAILMENT_OPTIONS | Optional; Primary Provider supplied description of options that may be available to the Transmission Customer, such as redispatch |
| SECURITY_REF | Optional; If the reliability adjustment was the result of a security event that is posted on OASIS via the security template, this shall be set to the OASIS unique identifier assigned to that posting |
| INITIATING_PARTY RESPONSIBLE_PARTY PROCEDURE_NAME PROCEDURE_LEVEL FACILITY_LOCATION FACILITY_NAME FACILITY_CLASS FACILITY_LIMIT_TYPE ANNOTATION | Optional; If the reliability adjustment was the result of a security event that is posted on OASIS via the security template, these data elements will be reported as they appear in that associated security event posting |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

013-3.3	systemdata

The systemdata template is designed to present time series data in an efficient form.  It is an extensible template as the data elements SYSTEM_ELEMENT_TYPE and SYSTEM_ATTRIBUTE may take on Provider specific registered values for posting of data that is not already defined in the OASIS Data Dictionary.

The SYSTEM_ELEMENT and SYSTEM_ELEMENT_TYPE data elements define the unique name and what type of system element that name represents.  The SYSTEM_ATTRIBUTE data element identifies what type of data value is represented in the ATTRIBUTE_VALUE data element.

There must be only one unique ATTRIBUTE_VALUE at any given point/interval in time.  The posting of a new value by the Primary Provider that overlaps all or any portion of existing data present in OASIS shall supersede that existing value for the interval specified.

The following subsections describe specific implementation for data that is required to be provided using this template.

013-3.3.1	ATC Related Query/Response Requirements

Transmission Providers that compute Available Transfer Capability on a contract or rated path basis shall provide data associated with the following SYSTEM_ATTRIBUTE values:

- TTC – Total Transfer Capability

- CBM – Capacity Benefit Margin

- FTRM –  Transmission Reliability Margin for use in FATC

- NFTRM –  Transmission Reliability Margin for use in NFATC

- FATC –Firm Available Transfer Capability

- NFATC –Non-Firm Available Transfer Capability

- ATC_ANNOTATION – Annotation for a change in monthly or yearly posted ATC

- {registered} – Provider defined attributes necessary for posting required data

The SYSTEM_ELEMENT_TYPE and SYSTEM_ELEMENT shall be used as follows:

- SYSTEM_ELEMENT_TYPE = PATH

- SYSTEM_ELEMENT = Name of the path associated with the posted ATC component value

Posting of ATC component data and annotations via the systemdata template shall comply with all applicable regulations and business practices.

The following are the systemdata template Data Element requirements for providing ATC component data, i.e., TTC, CBM, FTRM, NFTRM, FATC or NFATC, to the user:

| Data Element | Restriction/Requirement |
| --- | --- |
| POSTING_REF | The unique OASIS identifier assigned to the posting |
| SYSTEM_ELEMENT_TYPE | “PATH” |
| SYSTEM_ELEMENT | Name of the path associated with the posted ATC component value |
| SYSTEM_ATTRIBUTE | “TTC”, “CBM”, “FTRM”, “NFTRM”, “FATC”, or “NFATC” |
| START_TIME | Beginning of interval associated with the posted value |
| STOP_TIME | End of interval associated with the posted value |
| ATTRIBUTE_VALUE | Value of the posted ATC component over the interval of START_TIME/STOP_TIME |
| ATTRIBUTE_UNITS | Units of ATTRIBUTE_VALUE; typically MW |
| ANNOTATION | Optional at discretion of the Provider; typically null |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

The following are the systemdata template Data Element requirements for providing information on ATC annotations for instances where changes in monthly or yearly ATC are a result of a change in TTC greater than 10 %:

| Data Element | Restriction/Requirement |
| --- | --- |
| POSTING_REF | The unique OASIS identifier assigned to the posting |
| SYSTEM_ELEMENT_TYPE | “PATH” |
| SYSTEM_ELEMENT | Name of the path associated with the posted ATC component value |
| SYSTEM_ATTRIBUTE | “ATC_ANNOTATION |
| START_TIME | Time stamp of when path TTC changed by more than 10 % |
| STOP_TIME | Set equal to START_TIME |
| ATTRIBUTE_VALUE | New value of TTC on the posted path at START_TIME |
| ATTRIBUTE_UNITS | Units of ATTRIBUTE_VALUE; typically MW |
| ANNOTATION | Annotation providing information related to a specific change in ATC that requires the Primary Provider to post a narrative related to that change. |
| TIME_OF_LAST_UPDATE | Time stamp of when this posted data was last updated |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

The following are the systemdata template Data Element requirements for providing information on ATC annotations for instances where monthly or yearly ATC is posted at zero (0) for more than six (6) months:

| Data Element | Restriction/Requirement |
| --- | --- |
| POSTING_REF | The unique OASIS identifier assigned to the posting |
| SYSTEM_ELEMENT_TYPE | “PATH” |
| SYSTEM_ELEMENT | Name of the path associated with the posted ATC component value |
| SYSTEM_ATTRIBUTE | “ATC_ANNOTATION |
| START_TIME | Start date/time of the interval where posted ATC is zero. |
| STOP_TIME | End of the interval where posted ATC is zero |
| ATTRIBUTE_VALUE | 0 |
| ATTRIBUTE_UNITS | Units of ATTRIBUTE_VALUE; typically MW |
| ANNOTATION | Annotation providing information related to a specific change in ATC that requires the Primary Provider to post a narrative related to that change. |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

013-3.3.2	System Load Related Query/Response Requirements

Transmission Providers shall provide data for the following SYSTEM_ATTRIBUTE values related to System load as required by all applicable regulations and business practices:

- ZONE_LOAD_FORECAST – Forecast of anticipated Load in a metered zone

- SYSTEM_LOAD_FORECAST – Forecast of total systemwide load within a Balancing Area

- NATIVE_LOAD_FORECAST – Forecast of portion of total system load forecast for native load Forecast of total system load within a Balancing Area

- ACTUAL_LOAD – Actual daily peak load values for the metered Balancing Area or metered Zone within the Balancing Area posted after the fact

The SYSTEM_ELEMENT_TYPE and SYSTEM_ELEMENT shall be used as follows:

- SYSTEM_ELEMENT_TYPE = LOAD_ZONE

- SYSTEM_ELEMENT = name of the metered zone whose forecast and actual load data is being posted; for SYSTEM_LOAD_FORECAST and NATIVE_LOAD_FORECAST values this shall be the registered NERC acronym for the Balancing Area

The following table shows an example of the systemdata template Data Element usage for providing information on load related data:

| Data Element | Restriction/Requirement |
| --- | --- |
| POSTING_REF | The unique OASIS identifier assigned to the posting |
| SYSTEM_ELEMENT_TYPE | “LOAD_ZONE” |
| SYSTEM_ELEMENT | Name of the specific load or balancing area associated with the posted value |
| SYSTEM_ATTRIBUTE | “ACTUAL_LOAD” |
| START_TIME | Beginning of interval associated with the posted value |
| STOP_TIME | End of interval associated with the posted value |
| ATTRIBUTE_VALUE | Value of the zone’s load or load forecast over the interval of START_TIME/STOP_TIME |
| ATTRIBUTE_UNITS | Units of ATTRIBUTE_VALUE; typically MW |
| ANNOTATION | Optional at discretion of the Provider; typically null |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

013-3.4	security

The security template is used to provide information related to changes in the transmission system that may impact reliability.  The bulk of the data elements that comprise this template are identified as free-form text in the OASIS Data Dictionary to provide Transmission Providers with flexibility in posting different types of information in a structured fashion compatible with their operations and business practices.  .Usage of the data elements within the security template for providing information that may be made available by the Provider are provided as examples only.

013-3.4.1	Outage Query/Response Requirements

The following is an example of the suggested usage of the security template data elements for providing information related to Primary Provider posting of outages.  Note that posting of transmission related outages on OASIS is at the discretion of the Transmission Provider.

| Data Element | Restriction/Requirement |
| --- | --- |
| SECURITY_REF | The unique OASIS identifier assigned to the annotation posting |
| EVENT_ID | Optional at discretion of the Provider |
| SECURITY_TYPE | “OUTAGE” |
| INITIATING_PARTY | Identification of entity requesting the outage |
| RESPONSIBLE_PARTY | Identification of entity responsible for managing the outage |
| PROCEDURE_NAME | Optional at discretion of the Provider; may be used to document specific outage procedures |
| PROCEDURE_LEVEL | Optional at discretion of the Provider; |
| FACILITY_CLASS | “LINE” “TRANSFORMER” Etc. |
| FACILITY_LIMIT_TYPE | “PLANNED” “FORCED” |
| FACILITY_LOCATION | Optional at discretion of the Provider |
| FACILITY_NAME | Name of the transmission facility being outaged |
| START_TIME | Beginning of outage |
| STOP_TIME | End of outage |
| ANNOTATION | Detailed information describing the nature of the outage and all related information of significance. |

013-3.4.2	Curtailment Event Query/Response Requirements

The following is an example of the suggested usage of the security  template data elements for providing information related to Primary Provider posting of system conditions that may result in the curtailment or interruption of transmission service.  The example shown illustrates the data provided for a NERC TLR related event.  Note that the actual curtailment or impact of these events on schedules or transmission reservations will be posted on OASIS and made available via the scheduledetail or reduction  templates.

| Data Element | Restriction/Requirement |
| --- | --- |
| SECURITY_REF | The unique OASIS identifier assigned to the annotation posting |
| EVENT_ID | For NERC TLR, Reliability Coordinator assigned TLR event identifier |
| SECURITY_TYPE | “LIMIT” |
| INITIATING_PARTY | Entity that called for the TLR |
| RESPONSIBLE_PARTY | Reliability Coordinator managing the constrained flowgate |
| PROCEDURE_NAME | “NERC TLR” |
| PROCEDURE_LEVEL | “1”, “2”, etc. |
| FACILITY_CLASS | “FLOWGATE” |
| FACILITY_LIMIT_TYPE | “OSL” |
| FACILITY_LOCATION | “INTERNAL” if flowgate is within the Provider’s system “EXTERNAL” if flowgate is outside the Provider’s system |
| FACILITY_NAME | Name of the flowgate in TLR |
| START_TIME | Beginning of the TLR event |
| STOP_TIME | End of the TLR event |
| ANNOTATION | Detailed information describing the nature of the event and all related information of significance. |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

013-3.5	transoffering

The transoffering  template allows the Customer to selectively query for transmission service offers posted by both the Provider and Resellers.  Resellers control how their offerings are presented to the Customer through the specification of an OFFER_INCREMENT.  Provider postings shall always have OFFER_INCREMENT set equal to SERVICE_INCREMENT.

Transmission rights sold by the Reseller must correspond with the original rights acquired from the Provider.  There is, however, no restriction on the term/increment of service that may be resold.  The OFFER_INCREMENT data element controls when a Resellers capacity posted for resale will be presented to the Customer via the transoffering template.

For example, a Customer (Reseller) holding Monthly Firm transmission rights wishes to resell that capacity on either an hourly or daily basis.  The Reseller may post this Monthly Firm capacity for resale on an hourly or daily basis by posting two offerings with OFFER_INCREMENT of HOURLY and DAILY.  Customers querying for HOURLY or DAILY service would be presented with the Provider’s standard offer postings in addition to the Resellers offer of Monthly Firm capacity available on an HOURLY or DAILY basis.

Note that the basic transmission attributes, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, and TS_SBCLASS, in a posting for resale correspond to the original rights held by the Reseller and would not necessarily be similar to the transmission attributes posted by the Provider.

When queried by the Customer with a specified SERVICE_INCREMENT query parameter, OASIS shall return in the transoffering response all posted offers where SERVICE_INCREMENT or OFFER_INCREMENT matches any of the specified SERVICE_INCREMENT query values.

013-3.6	transpost/transupdate

The transpost/transupdate templates allows a Customer (Reseller) to post or modify an offer to resell their reserved capacity on OASIS in a manner comparable to the offerings posted by the Primary Provider.  The service offered for resale is dictated by the service held by the customer as defined in the SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, and TS_SUBCLASS Data Elements.  However, the Customer may offer that service in any arbitrary increment of whole hours from hourly, to daily, to monthly, etc.

The OFFER_INCREMENT Data Element controls when the Reseller’s transmission service offering will be returned to a user submitting the transoffering template.  For example, to post Monthly Firm service reserved from the Primary Provider as an HOURLY offering, the SERVICE_INCREMENT Data Element will be MONTHLY to correspond to the reserved service being resold and the OFFER_INCREMENT would be submitted as HOURLY.  When queried by a user with SERVICE_INCREMENT=HOURLY, the transoffering template would return all qualifying posted offerings from the Primary Provider where SERVICE_INCREMENT=HOURLY and any resale offerings posted by Resellers where the OFFER_INCREMENT=HOURLY. To post that Monthly capacity on a Daily basis, the reseller would submit another transmission service offering specifying the OFFER_INCREMENT=DAILY.

The following table shows the specific template Data Element usage for the transpost/transupdate template:

| Data Element | Restriction/Requirement |
| --- | --- |
| POSTING_REF | The unique OASIS identifier assigned to the posting |
| PATH_NAME POINT_OF_RECEIPT POINT_OF_DELIVERY INTERFACE_TYPE | Must correspond with the same data elements as represented in the Reseller’s transmission service reservation(s) that is being offered for sale. |
| CAPACITY | Amount of service offered for sale |
| SERVICE_INCREMENT TS_CLASS TS_TYPE TS_PERIOD TS_WINDOW TS_SUBCLASS | Must correspond with the same data elements as represented in the Reseller’s transmission service reservation(s) that is being offered for sale. |
| ANC_SVC_REQ | Associated ancillary service requirements, if any |
| START_TIME STOP_TIME | Interval for the transmission service being sold |
| OFFER_INCREMENT | Increment of service the Reseller is offering for sale |
| OFFER_START_TIME OFFER_STOP_TIME | The time interval during which the Reseller will accept/consider requests to purchase to capacity offered for sale |
| SALE_REF | Optional Reseller internal reference value |
| OFFER_PRICE | Price posted for the service offered for sale |
| SERVICE_DESCRIPTION | Optional description or other terms and conditions of the resale |
| SELLER_COMMENTS | Seller comments |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

013-3.7	transstatus

The transstatus template provides the Customer with a view of transactions submitted on OASIS.  OASIS shall be responsible for merging any transaction profile data submitted by the Customer (START_TIME, STOP_TIME, CAPACITY_REQUESTED, and BID_PRICE) or the Seller (START_TIME, STOP_TIME, CAPACITY_GRANTED, and OFFER_PRICE) into a single set of time profile data records consisting of START_TIME, STOP_TIME, CAPACITY_REQUESTED, BID_PRICE, CAPACITY_GRANTED, OFFER_PRICE, and CEILING_PRICE to be returned in the transstatus template response.

013-3.8	reduction

The reduction template provides the Customer with a view of transactions or Primary Provider actions which impact either the capacity available on a given transmission service reservation or the service curtailment priority in effect for the reservation.  Capacity impacts are due to transactions such as redirects, resales, transfers, recalls, etc.; curtailment priority impacts are due to criteria established in granting long-term firm service where firm curtailment priority cannot be granted for all periods or under certain specific operating conditions as specified in the Customer’s service agreement.

For a given transmission service request, there may never be two reduction template response records that overlap in time.  For any given interval in time, all transactions or Primary Provider actions that impact the transmission service request over that interval shall be returned in one or more continuation records.

The following are the specific values for REDUCTION_TYPE impacting the capacity available on a reservation that shall be supported:

- REDIRECT – impact is the result of a REDIRECT on a Firm basis

- RESALE – impact is the result of a sale of transmission scheduling rights on the secondary market

- TRANSFER – impact is the result of a transfer of all rights and obligations from one Customer to another Customer

- RECALL – impact is the result of the Primary Provider recalling reserved capacity for actions such as the partial displacement of service by higher priority request(s)

- {Registered} – Primary Provider specific value registered by the Provider

For each of these reductions, the magnitude of the reduction in capacity must be reported in the template response CAPACITY_REDUCED Data Element as a negative value.  The sum of all reductions of this type should equal the difference in values reported in CAPACITY_AVAILABLE and CAPACITY_GRANTED.

The following are the specific values for REDUCTION_TYPE used to document a change in reservation service curtailment priority.  Posting of this information by the Primary Provider must be in accordance with the conditional curtailment option agreed to by the Provider and Customer in the governing transmission service agreement.

- CONDITIONAL_HOURS – reduction in service curtailment priority related to a specific number of hours per year as specified in the service agreement

- CONDITIONAL_EVENT– reduction in service curtailment priority due to the occurrence of a specific change in system conditions

- {registered} – Primary Provider specific registered name for the type of reduction posted

In the template response records for reductions in service curtailment priority, CAPACITY_REDUCED shall specify the amount of reserved capacity whose curtailment priority is reduced as a positive valued MW number. NERC_CURTAILMENT_PRIORITY shall reflect the reduction in service curtailment priority.

If the Primary Provider uses a provider specific registered value for REDUCTION_TYPE, the registered value and a full description of how the value is used and the information returned in the reduction template response must be posted on the Provider’s OASIS Home Page.

An example of the reduction template response is provided in WEQ 013-4.1.8.

013-4	EXAMPLE IMPLEMENTATION

013-4.1	File Request and File Download Examples

The following standards provide examples for the implementation of file request and download interactions that must be supported by OASIS.  In these examples, the end-of-line (eol) character is represented by the character,"  ". This symbol may appear different on displays or printouts. Note that any leading or trailing spaces within each data record shown were inserted to facilitate word-wrapping of long records and must not appear in the actual formatted data files returned by OASIS unless they are contained within double quotes.

013-4.1.1	File Example for Hourly Offering

Example of the request to Primary Provider, AAA, for PATH_NAME "W/AAA/ABC//" for April 10, 2007 from 8 a.m. to 3 p.m. (Note that the PATH_NAME consists of a REGION_CODE, PRIMARY_PROVIDER_CODE, PATH_CODE, and an OPTIONAL_CODE, separated with a slash, "/".)

The VERSION for this release is 1.5. The request is in the form of a URL query string and the response is an ASCII delimited file.  There is also one offer posted by the reseller “WXYZ” that has posted their WEEKLY scheduling rights for RESALE on an HOURLY basis.

- Query

http://(OASIS Node name)/OASIS/AAA/data/transoffering? ver=1.5&templ=transoffering& fmt=data&pprov=AAA &pprovduns=123456789& path=W/AAA/ABC//& servincre=hourly

&TSCLASS1=firm &TSCLASS2=non-firm&tz=PD

&stime=20070410080000PD &sptime=20070410150000PD

- Response Data

REQUEST-STATUS=200 (Successful)

TIME_STAMP=20070409113526PD 

VERSION=1.5

TEMPLATE=transoffering

OUTPUT_FORMAT=DATA 

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

DATA_ROWS=13

COLUMN_HEADERS=TIME_OF_LAST_UPDATE, SELLER_CODE, SELLER_DUNS, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, INTERFACE_TYPE, OFFER_START_TIME, OFFER_STOP_TIME, START_TIME, STOP_TIME, CAPACITY, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, ANC_SVC_REQ, SALE_REF, POSTING_REF, CEILING_PRICE, OFFER_PRICE, PRICE_UNITS, SERVICE_DESCRIPTION, NERC_CURTAILMENT_PRIORITY, OTHER_CURTAILMENT_PRIORITY, SELLER_NAME, SELLER_PHONE, SELLER_FAX, SELLER_EMAIL, SELLER_COMMENTS, OFFER_INCREMENT

20070406170000PD, WXYZ,543219876, W/AAA/AAA-ABC//, AAA, ABC, E,,, 20070410000000PD, 20070411000000PD,15, WEEKLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RV:M;LC:O, ,5673,1.5,1.43, $/MW-H,,4,, WXYZ Marketer, 888-789-4321,,, Reselling WEEKLY priority rights, HOURLY

20070409030000PD, AAA,987654321, W/AAA/AAA-ABC//, AAA, ABC, E, 20070402080000PD, 20070410080000PD, 20070410080000PD, 20070410090000PD,50, HOURLY, FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RV:M;LC:O, ,5675,1.5,1.35, $/MW-H,,7,, AAA Provider, 888-321-7654,,, 10% DISCOUNT , HOURLY

20070409030002PD, AAA,987654321, W/AAA/AAA-ABC//, AAA, ABC, E, 20070402080000PD, 20070410080000PD, 20070410080000PD, 20070410090000PD,300, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RV:M;LC:O, ,5676,1.5,1.35, $/MW-H,,2,, AAA Provider, 888-321-7654,,, 10% DISCOUNT , HOURLY

20070409030000PD, AAA,987654321, W/AAA/AAA-ABC//, AAA, ABC, E, 20070402080000PD, 20070410080000PD, 20070410090000PD, 20070410100000PD,50, HOURLY, FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RV:M;LC:O, ,5682,1.5,1.35, $/MW-H,,7,, AAA Provider, 888-321-7654,,, 10% DISCOUNT , HOURLY

20070409030002PD, AAA,987654321, W/AAA/AAA-ABC//, AAA, ABC, E, 20070402080000PD, 20070410080000PD, 20070410090000PD, 20070410100000PD,300, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RV:M;LC:O, ,5688,1.5,1.35, $/MW-H,,2,, AAA Provider, 888-321-7654,,, 10% DISCOUNT , HOURLY

20070409030000PD, AAA,987654321, W/AAA/AAA-ABC//, AAA, ABC, E, 20070402080000PD, 20070410080000PD, 20070410100000PD, 20070410110000PD,0, HOURLY, FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RV:M;LC:O, ,5689,1.5,1.35, $/MW-H,,7,, AAA Provider, 888-321-7654,,, 10% DISCOUNT , HOURLY

20070409030002PD, AAA,987654321, W/AAA/AAA-ABC//, AAA, ABC, E, 20070402080000PD, 20070410080000PD, 20070410100000PD, 20070410110000PD,275, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RV:M;LC:O, ,5693,1.5,1.35, $/MW-H,,2,, AAA Provider, 888-321-7654,,, 10% DISCOUNT , HOURLY

20070409030000PD, AAA,987654321, W/AAA/AAA-ABC//, AAA, ABC, E, 20070402080000PD, 20070410080000PD, 20070410110000PD, 20070410130000PD,0, HOURLY, FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RV:M;LC:O, ,5694,1.5,1.35, $/MW-H,,7,, AAA Provider, 888-321-7654,,, 10% DISCOUNT , HOURLY

20070409030002PD, AAA,987654321, W/AAA/AAA-ABC//, AAA, ABC, E, 20070402080000PD, 20070410080000PD, 20070410110000PD, 20070410130000PD,150, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RV:M;LC:O, ,5699,1.5,1.35, $/MW-H,,2,, AAA Provider, 888-321-7654,,, 10% DISCOUNT , HOURLY

20070409030000PD, AAA,987654321, W/AAA/AAA-ABC//, AAA, ABC, E, 20070402080000PD, 20070410080000PD, 20070410130000PD, 20070410140000PD,25, HOURLY, FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RV:M;LC:O, ,5702,1.5,1.35, $/MW-H,,7,, AAA Provider, 888-321-7654,,, 10% DISCOUNT , HOURLY

20070409030002PD, AAA,987654321, W/AAA/AAA-ABC//, AAA, ABC, E, 20070402080000PD, 20070410080000PD, 20070410130000PD, 20070410140000PD,300, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RV:M;LC:O, ,5703,1.5,1.35, $/MW-H,,2,, AAA Provider, 888-321-7654,,, 10% DISCOUNT , HOURLY

20070409030000PD, AAA,987654321, W/AAA/AAA-ABC//, AAA, ABC, E, 20070402080000PD, 20070410080000PD, 20070410140000PD, 20070410150000PD,25, HOURLY, FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RV:M;LC:O, ,5706,1.5,1.35, $/MW-H,,7,, AAA Provider, 888-321-7654,,, 10% DISCOUNT , HOURLY

20070409030002PD, AAA,987654321, W/AAA/AAA-ABC//, AAA, ABC, E, 20070402080000PD, 20070410080000PD, 20070410140000PD, 20070410150000PD,300, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RV:M;LC:O, ,5707,1.5,1.35, $/MW-H,,2,, AAA Provider, 888-321-7654,,, 10% DISCOUNT , HOURLY

013-4.1.2	File Example for Hourly Schedule Data

This example shows a request for the hourly schedule data from Primary Provider, AAA, related to the Seller, WXYZ, for the period 10 a.m. to 3 p.m. on April 10, 2007.

There are two identical requests examples using two slightly different methods. The first request is using a HTTP URL request string through an HTML GET method. The second request is a similar example using fetch_http from a file using a POST method.

- Query

URL Request (HTTP method=GET):

http://(OASIS Node name)/OASIS/AAA/data/scheduledetail? ver=1.5& pprov=AAA& templ=scheduledetail& fmt=data &pprovduns=123456789 &path=W/AAA/ABC//& seller=WXYZ &por=BBB &pod=CCC& tz=PD&

stime=20070410100000PD & sptime=20070410150000PD

URL Request (HTTP method=POST):

$ fetch_http http://(OASIS Node name)/OASIS/aaaa/data/OASISdata -f c:/OASIS/wxyz/upload/infile. txt Where in-file.txt contains the following: ver=1.5& pprov=AAA& templ=scheduledetail& fmt=data &pprovduns=123456789 &path=W/AAA/ABC//& seller=WXYZ &por=BBB &pod=CCC& tz=PD&

stime=20070410100000PD &

sptime=20070410150000PD

- Response Data

REQUEST_STATUS=200

ERROR_MESSAGE=No error. 

TIME_STAMP=20070410160523ES 

VERSION=1.5

TEMPLATE=scheduledetail

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=PD

DATA_ROWS=3 

COLUMN_HEADERS=CONTINUATION_FLAG, TIME_OF_LAST_UPDATE, SCHEDULE_REF, TRANSACTION_ID, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, GCA_CODE, LCA_CODE, SOURCE, SINK, SCHEDULE_PRIORITY, START_TIME, STOP_TIME, SCHEDULE_REQUESTED, SCHEDULE_GRANTED, ASSIGNMENT_REF, SELLER_CODE, SELLER_DUNS, CUSTOMER_CODE, CUSTOMER_DUNS, AFFILIATE_FLAG, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, NERC_CURTAILMENT_PRIORITY, OTHER_CURTAILMENT_PRIORITY, CAPACITY_USED, PROVIDER_ACTION, SCHEDULE_LIMIT, CURTAILMENT_OPTIONS, SECURITY_REF, INITIATING_PARTY, RESPONSIBLE_PARTY, PROCEDURE_NAME, PROCEDURE_LEVEL, FACILITY_LOCATION, FACILITY_NAME, FACILITY_CLASS, FACILITY_LIMIT_TYPE,ANNOTATION

N, 20070409030000PD,12345, BBB_MKTATAGCODE_CCC, W/AAA/ABC//, BBB, CCC, BBB,CCC,GENX,LOADY,2, 20070410100000PD, 20070410110000PD,280,280,856743, WXYZ,987654321, MKTA,987654322, N, HOURLY, NON_FIRM, POINT_TO_POINT, ON_PEAK, FIXED,,2, ,300,,,,,,,,,,,,, 

N, 20070409030000PD,12346, BBB_MKTATAGCODE_CCC, W/AAA/ABC//, BBB, CCC, BBB,CCC,GENX,LOADY,2, 20070410110000PD, 20070410140000PD,295,295,856743, WXYZ,987654321, MKTA,987654322, N, HOURLY, NON_FIRM, POINT_TO_POINT, ON_PEAK, FIXED,,2, ,300,,,,,,,,,,,,, 

N, 20070409030000PD,12347, BBB_MKTATAGCODE_CCC, W/AAA/ABC//, BBB, CCC, BBB,CCC,GENX,LOADY,2, 20070410140000PD, 20070410150000PD,300,300,856743, WXYZ,987654321, MKTA,987654322, N, HOURLY, NON_FIRM, POINT_TO_POINT, ON_PEAK, FIXED,,2, ,300,,,,,,,,,,,,, 

013-4.1.3	Transmission Customer Posting a Transmission Service Offering

This example shows how a Transmission Customer would post for sale the transmission service that was purchased previously. The Seller would create a file and upload the file using the FETCH_HTTP program to send a file to the OASIS Node of the Primary Provider.  This example shows the posting of MONTHLY service offered for resale on a DAILY increment basis.

- Input:

FETCH_HTTP Command to send posting:

$ fetch_http http://(OASIS Node name)/OASIS/abcd/data/transrequest -f c:/OASIS/abcd/upload/post.txt

Where the file post.txt contains:

VERSION=1.5

TEMPLATE=transpost

OUTPUT_FORMAT=DATA 

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

DATA_ROWS=1

COLUMN_HEADERS=PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, INTERFACE_TYPE, CAPACITY, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, ANC_SVC_REQ, START_TIME, STOP_TIME, OFFER_START_TIME, OFFER_STOP_TIME, SALE_REF, OFFER_PRICE, SERVICE_DESCRIPTION, SELLER_COMMENTS, OFFER_INCREMENT

W/AAA/ABC-DEF//,,, E,150, MONTHLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,,,  200070410000000PD, 20070414000000PD, 20070402080000PD, 20070410070000PD, A123,0.9,,"As Joe said, ""It is a good buy""", DAILY

- Response Data

REQUEST-STATUS=200  (Successful)

TIME_STAMP=20070409113526PD 

VERSION=1.5

TEMPLATE=transpost

OUTPUT_FORMAT=DATA 

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

DATA_ROWS=1

COLUMN_HEADERS=RECORD_STATUS, POSTING_REF, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, INTERFACE_TYPE, CAPACITY, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, ANC_SVC_REQ, START_TIME, STOP_TIME, OFFER_START_TIME, OFFER_STOP_TIME, SALE_REF, OFFER_PRICE, SERVICE_DESCRIPTION, SELLER_COMMENTS, OFFER_INCREMENT, ERROR_MESSAGE

200,7453255, W/AAA/ABC-DEF//,,, E,150, MONTHLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,,,  200070410000000PD, 20070414000000PD, 20070402080000PD, 20070410070000PD, A123,0.9,," As Joe said, ""It is a good buy""", DAILY, No Error

013-4.1.4	Example of Aggregating Purchased Services for Resale using Reassignment

The following examples do not show the complete template information, but only show the values assigned to those Data Elements in the template othat are important the example.

a. Transmission Customer #1, "BestE" requests the purchase of 20 MW Daily Firm service for May 7th and 8th for $1.00 from Primary Provider AAA (transrequest).

TEMPLATE=transrequest

CUSTOMER_CODE=BestE (Implied by the registered entity submitting the template)

SELLER_CODE=AAA

PATH_NAME=W/AAA/AAA-ABC//

SERVICE_INCREMENT=DAILY

TS_CLASS=FIRM

START_TIME=2007050700000000PD

STOP_TIME=20070509000000PD

CAPACITY_REQUESTED=20

BID_PRICE=1.00

REQUEST_TYPE=ORIGINAL

The OASIS assigns ASSIGNMENT_REF = 5000 on acknowledgment.

b. Transmission Customer #1 purchases an additional 70 MW of Daily Firm service on the same path for May 8th for $1.05 (transrequest). The OASIS assigns the ASSIGNMENT_REF=5001 when the request for purchase is made and is shown in the acknowledgment.

TEMPLATE=transrequest

CUSTOMER_CODE=BestE (Implied by the registered entity submitting the template)

SELLER_CODE=AAA

PATH_NAME=W/AAA/AAA-ABC//

SERVICE_INCREMENT=DAILY

TS_CLASS= FIRM

START_TIME=2007050800000000PD

STOP_TIME=2007050900000000PD

CAPACITY_REQUESTED=70

BID_PRICE=1.05

REQUEST_TYPE=ORIGINAL

The OASIS assigns the ASSIGNMENT_REF=5001 when the request for purchase is made and is shown in the acknowledgment.

c. Transmission Customer #1 becomes reeller #1 and posts an offer of Transmission service of 50 MW of their Daily Firm rights purchased above for resale on an Hourly basis on May 8th from 7 a.m. to 11 p.m. at $.90/MW-hour.

TEMPLATE=transpost

SELLER_CODE=BestE(Implied by the registered entity submitting the template)

PATH_NAME=W/AAA/AAA-ABC//

CAPACITY=50

SERVICE_INCREMENT=DAILY

TS_CLASS=FIRM

START_TIME=2007050807000000PD

STOP_TIME=20007050823000000PD

OFFER_INCREMENT=HOURLY

SALE_REF=BEST 50

OFFER_PRICE=.90

SELLER_COMMENTS=aggregating two previous purchases

d. Transmission Customer #2 “Whlsle” then requests purchase of 35 MW firm from Reseller #1 from 8 a.m. to 6 5.m. for $0.90/MW-hour (transrequest).

TEMPLATE=transrequest

CUSTOMER_CODE=Whlsle

SELLER_CODE=BestE (Implied by the registered entity submitting the template)

PATH_NAME=W/AAA/AAA-ABC//

TS_CLASS=FIRM

START_TIME=20070508080000PD

STOP_TIME=20070508170000PD

CAPACITY_REQUESTED=35

BID_PRICE=0.90

CUSTOMER_COMMENTS=Only need service until 5 p.m.

REQUEST_TYPE=RESALE

The OASIS provides the ASSIGNMENT_REF=5002 for this transaction.

e. Seller (Reseller #1) informs the Primary Provider of the reassignment of the previous transmission rights when the Seller accepts the Transmission Customer purchase request (transsell).

TEMPLATE=transsell

SELLER_CODE=BestE (Implied by the registered entity submitting the template)

ASSIGNMENT_REF=5033

START_TIME=20070508080000PD

STOP_TIME=20070508170000PD

CAPACITY_GRANTED=35

OFFER_PRICE=0.90

STATUS=ACCEPTED

REASSIGNED_REF1=5000

REASSIGNED_CAPACITY1=20

REASSIGNED_START_TIME1=20070508080000PD

REASSIGNED_STOP_TIME1=20070508170000PD

REASSIGNED_REF2=5006

REASSIGNED_CAPACITY2=15

REASSIGNED_START_TIME2=20070508080000PD

REASSIGNED_STOP_TIME2=20070508170000PD

Resale Business Practice Standards govern what services may be aggregated and offered for resale.  The sum of all REASSIGNED_CAPACITY over time must be sufficient to satisfy the capacity granted in OFFER_CAPACITY over the interval of OFFER_START_TIME to OFFER_STOP_TIME.

013-4.1.5	File Examples of the Use of Continuation Records

a. Basic Continuation Records

The first example of the use of Continuation Records is for the transrequest template submitted by a Transmission Customer, “MOP” for purchase of a transmission reservation spanning 16 hours from 06:00 to 22:00 with "ramped" demand at beginning and end of time period. Two additional reservation requests are also submitted in this request prior to and following the profile to demonstrate the handling of ASSIGNMENT_REF by the OASIS Node.  The last request is for a purchase from a Reseller, “EFG”.

The OASIS S&CP identifies which Data Elements may appear in continuation records.  For profiled request of capacity, the Data Elements START_TIME, STOP_TIME, CAPACITY_REQUESTED, and BID_PRICE are repeated in continuation records to define each segment of the profiled requests.  Specification of any values corresponding to COLUMN_HEADERs that are not specified as being allowed in continuation records will be ignored, however commas must be included to properly align the Data Elements associated with each continuation record.

Input:

VERSION=1.5

TEMPLATE=transrequest

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=7

COLUMN_HEADERS=CONTINUATION_FLAG, SELLER_CODE,  SELLER_DUNS, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK,CAPACITY_REQUESTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, STATUS_NOTIFICATION, START_TIME, STOP_TIME, BID_PRICE, PRECONFIRMED, ANC_SVC_LINK, POSTING_REF, SALE_REF, REQUEST_REF, DEAL_REF, CUSTOMER_COMMENTS,REQUEST_TYPE,RELATED_REF

N, AAA,123456789, X/AEF/CEF-ECS//, CEF, ECS,,,35, DAILY, FIRM, POINT_TO_POINT, OFF_PEAK, FIXED,, /AAA/incoming?ref=R765, 20070423000000ES, 20070424000000ES,24.5, NO,,,, R765, D123, Standard daily reservation, ORIGINAL, 

N, AAA,123456789,, AEF, MPO,,,5, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, /AAA/incoming?ref=R766, 20070423060000ES, 20070423070000ES,2.5, YES,,,, R766, D123, First piece of profile spanning 5 records, ORIGINAL, 

Y,,,,,,,,10,,,,,,,, 20070423070000ES, 20070423080000ES,2.5,,,,,,,,, 

Y,,,,,,,,15,,,,,,,, 20070423080000ES, 20070423200000ES,2.5,,,,,,,,, 

Y,,,,,,,,10,,,,,,,, 20070423200000ES, 20070423210000ES,2.5,,,,,,,,, 

Y,,,,,,,,5,,,,,,,, 20070423210000ES, 20070423220000ES,2.5,,,,,,,,, 

N, EFG,678912345, X/AEF/CEF-ECS//, CEF, ECS,,,20,DAILY, FIRM, POINT_TO_POINT,FULL_PERIOD, FIXED,, /AAA/incoming?ref=R767, 20070423040000ES, 20070423160000ES,2, YES,,,, R767, D123, Resale hourly reservation after profiled reservation, RESALE, 

Response:

REQUEST_STATUS=200

ERROR_MESSAGE=Success

TIME_STAMP=20070422160523ES 

VERSION=1.5

TEMPLATE=transrequest

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=7

COLUMN_HEADERS=RECORD_STATUS, CONTINUATION_FLAG, ASSIGNMENT_REF, SELLER_CODE,  SELLER_DUNS, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK, CAPACITY_REQUESTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, STATUS_NOTIFICATION, START_TIME, STOP_TIME, BID_PRICE, PRECONFIRMED, ANC_SVC_LINK, POSTING_REF, SALE_REF, REQUEST_REF, DEAL_REF, CUSTOMER_COMMENTS, REQUEST_TYPE, RELATED_REF, ERROR_MESSAGE

200,N,23879032, AAA,123456789, X/AEF/CEF-ECS//, CEF, ECS,,,35, DAILY, FIRM, POINT_TO_POINT, OFF_PEAK, FIXED,, /AAA/incoming?ref=R765, 20070423000000ES, 20070424000000ES,24.5, NO,,,, R765, D123, Standard daily reservation, ORIGINAL, , No Error

200,N,23879037, AAA,123456789,, AEF, MPO,,,5, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, /AAA/incoming?ref=R766, 20070423060000ES, 20070423070000ES,2.5, YES,,,, R766, D123, First piece of profile spanning 5 records, ORIGINAL, , No Error

200,Y,23879037,,,,,,,,10,,,,,,,, 20070423070000ES, 20070423080000ES,2.5,,,,,,,,, , No Error

200,Y,23879037,,,,,,,,15,,,,,,,, 20070423080000ES, 20070423200000ES,2.5,,,,,,,,, , No Error

200,Y,23879037,,,,,,,,10,,,,,,,, 20070423200000ES, 20070423210000ES,2.5,,,,,,,,, , No Error

200,Y,23879037,,,,,,,,5,,,,,,,, 20070423210000ES, 20070423220000ES,2.5,,,,,,,,,, No Error

200,N,23879040, EFG,678912345, X/AEF/CEF-ECS//, CEF, ECS,,,20,DAILY, FIRM, POINT_TO_POINT,FULL_PERIOD, FIXED,, /AAA/incoming?ref=R767, 20070423040000ES, 20070423160000ES,2, YES,,,, R767, D123, Resale hourly reservation after profiled reservation, RESALE, , No Error

b. Submission of Reassignment Information - Case 1:

In the prior example (last data record in transrequest) a RESALE reservation request was submitted to the Reseller “EFG” by the Transmission Customer for 20MW of DAILY firm scheduling rights from 04:00 to 16:00. Assume that the Reseller has previously reserved service for the CEF-ECS path for Daily Firm in amount of 50 MW on 4/23 under ASSIGNMENT_REF=23877019, and another 10 MW on April 23, 2007, under ASSIGNMENT_REF=23877880. Reseller must designate which transmission service rights are to be reassigned to Transmission Customer to satisfy the 20MW from 04:00 to 16:00. This reassignment information is conveyed by Reseller using the transsell template when the reservation request is ACCEPTED. At the SELLER's discretion, rights are assigned for the first four hours of the resale from the first reservation (ASSIGNMENT_REF=23877019).  The balance of the resale is supported by 10 MWs from the second reservation first (ASSIGNMENT_REF=23877880) with the balance taken up by the first reservation (ASSIGNMENT_REF=23877019).

This split of reassignment information is conveyed in "continuation" records via the transsell template Data Elements REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME, and REASSIGNED_STOP_TIME.

Input:

VERSION=1.5

TEMPLATE=transsell

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=3

COLUMN_HEADERS=CONTINUATION_FLAG, ASSIGNMENT_REF, START_TIME, STOP_TIME, OFFER_PRICE, CAPACITY_GRANTED, STATUS, STATUS_COMMENTS, ANC_SVC_LINK, ANC_SVC_REQ,COMPETING_REQUEST_FLAG, NEGOTIATED_PRICE_FLAG, SELLER_REF, SELLER_COMMENTS, RESPONSE_TIME_LIMIT, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME , PRIMARY_PROVIDER_APPROVAL, PRIMARY_PROVIDER_COMMENTS, PRIMARY_PROVIDER_CONDITIONS, REASSESSMENT_DUE_TIME, RENEWAL_DUE_TIME, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, ROLLOVER_CAPACITY

N,23879040, 20070423040000ES, 20070423160000ES,2,20, ACCEPTED, Status comments here, , , N, , S78234, Seller comments here, 20070421153720ES,23877019,20, 20070423040000ES, 20070423080000ES,,,,,,,, 

Y,23879040,,,,,,,,,,,,,,23877880,10, 20070423080000ES, 20070423160000ES,,,,,,,, 

Y,23879040,,,,,,,,,,,,,,23877019,10, 20070423080000ES, 20070423160000ES,,,,,,,, 

Response:

REQUEST_STATUS=200

ERROR_MESSAGE=Successfully updated. 

TIME_STAMP=20070422160523ES 

VERSION=1.5

TEMPLATE=transsell

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=3

COLUMN_HEADERS=RECORD_STATUS, CONTINUATION_FLAG, ASSIGNMENT_REF,  START_TIME, STOP_TIME, OFFER_PRICE,CAPACITY_GRANTED, STATUS, STATUS_COMMENTS, ANC_SVC_LINK, ANC_SVC_REQ, COMPETING_REQUEST_FLAG, NEGOTIATED_PRICE_FLAG, SELLER_REF, SELLER_COMMENTS, RESPONSE_TIME_LIMIT, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME , PRIMARY_PROVIDER_APPROVAL, PRIMARY_PROVIDER_COMMENTS, PRIMARY_PROVIDER_CONDITIONS, REASSESSMENT_DUE_TIME, RENEWAL_DUE_TIME, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, ROLLOVER_CAPACITY, ERROR_MESSAGE

200,N,23879040, 20070423040000ES, 20070423160000ES,2,20, ACCEPTED, Status comments here, , , N, , S78234, Seller comments here, 20070421153720ES,23877019,20, 20070423040000ES, 20070423080000ES,,,,,,,, , No Error

200,Y,23879040,,,,,,,,,,,,,,23877880,10, 20070423080000ES, 20070423160000ES,,,,,,,, , No Error

200,Y,23879040,,,,,,,,,,,,,,23877019,10, 20070423080000ES, 20070423160000ES,,,,,,,, , No Error

c. Submission of Reassignment Information - Case 2:

If the resale transaction from the previous example were conducted off-OASIS, the SELLER, “EFG”, would use the transassign template to post the transaction to OASIS.

Input:

VERSION=1.5

TEMPLATE=transassign

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=3

COLUMN_HEADERS=CONTINUATION_FLAG, CUSTOMER_CODE, CUSTOMER_DUNS, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK, CAPACITY_REQUESTED, CAPACITY_GRANTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, START_TIME, STOP_TIME, OFFER_PRICE, ANC_SVC_LINK, POSTING_NAME, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME , SELLER_COMMENTS, SELLER_REF, REQUEST_TYPE, RELATED_REF

N, MOP,562791881, X/AEF/CEF-ECS//, CEF, ECS,,,20,20, DAILY, FIRM, POINT_TO_POINT,FULL_PERIOD, FIXED,, 20070423040000ES, 20070423160000ES,2,, John Doe,23877019,20, 20070423040000ES, 20070423080000ES, Seller comments here, S78234, RESALE, 

Y,,,,,,,,,,,,,,,,,,,,,23877880,10, 20070423080000ES, 20070423160000ES,, ,, 

Y,,,,,,,,,,,,,,,,,,,,,23877019,10, 20070423080000ES, 20070423160000ES,, ,

Response:

REQUEST_STATUS=200

ERROR_MESSAGE=Successfully updated. 

TIME_STAMP=20070422160523ES 

VERSION=1.5

TEMPLATE=transassign

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=3

COLUMN_HEADERS=RECORD_STATUS, CONTINUATION_FLAG,ASSIGNMENT_REF, CUSTOMER_CODE, CUSTOMER_DUNS, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK, CAPACITY_REQUESTED, CAPACITY_GRANTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, START_TIME, STOP_TIME, OFFER_PRICE, ANC_SVC_LINK, POSTING_NAME, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME , SELLER_COMMENTS, SELLER_REF, REQUEST_TYPE, RELATED_REF,ERROR_MESSAGE

200,N,3377280, MOP,562791881, X/AEF/CEF-ECS//, CEF, ECS,,,20,20, DAILY, FIRM, POINT_TO_POINT,FULL_PERIOD, FIXED,, 20070423040000ES, 20070423160000ES,2,, John Doe,23877019,20, 20070423040000ES, 20070423080000ES, Seller comments here, S78234, RESALE,,No Error

200,Y,3377280,,,,,,,,,,,,,,,,,,,,,23877880,10, 20070423080000ES, 20070423160000ES,, ,,,No Error

200,Y,3377280,,,,,,,,,,,,,,,,,,,,,23877019,10, 20070423080000ES, 20070423160000ES,, ,,,No Error

d. Query of Transmission Reservation Status:

The following is a hypothetical response to a transstatus query that might be delivered for reservations starting on April 23, 2007.

Input:

<appropriate query name/value pairs to return reservations for April 23, 2007>

Response:

REQUEST_STATUS=200

ERROR_MESSAGE=No error. 

TIME_STAMP=20000423160523ES

VERSION=1.5

TEMPLATE=transstatus

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=10

COLUMN_HEADERS=CONTINUATION_FLAG, ASSIGNMENT_REF, SELLER_CODE, SELLER_DUNS, CUSTOMER_CODE, CUSTOMER_DUNS, AFFILIATE_FLAG, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK,CAPACITY_REQUESTED,CAPACITY_GRANTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD,TS_WINDOW, TS_SUBCLASS, NERC_CURTAILMENT_PRIORITY, OTHER_CURTAILMENT_PRIORITY, START_TIME, STOP_TIME, CEILING_PRICE, OFFER_PRICE, BID_PRICE, PRICE_UNITS, PRECONFIRMED, ANC_SVC_LINK, ANC_SVC_REQ, POSTING_REF, SALE_REF, REQUEST_REF, DEAL_REF, IMPACTED, COMPETING_REQUEST_FLAG, REQUEST_TYPE, RELATED_REF, NEGOTIATED_PRICE_FLAG, STATUS, STATUS_NOTIFICATION, STATUS_COMMENTS, TIME_QUEUED, RESPONSE_TIME_LIMIT, TIME_OF_LAST_UPDATE, PRIMARY_PROVIDER_COMMENTS, SELLER_REF, SELLER_COMMENTS, CUSTOMER_COMMENTS, SELLER_NAME, SELLER_PHONE, SELLER_FAX, SELLER_EMAIL, CUSTOMER_NAME, CUSTOMER_PHONE, CUSTOMER_FAX, CUSTOMER_EMAIL, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME, PRIMARY_PROVIDER_APPROVAL, PRIMARY_PROVIDER_CONDITIONS, REASSESSMENT_DUE_TIME, RENEWAL_DUE_TIME, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, ROLLOVER_CAPACITY

N,8207, RSELLR,234567890, ACSTMR,987654321, N, , CE, VP, ,,10,10, HOURLY, FIRM, POINT_TO_POINT, OFF_PEAK, FIXED,,7,, 20070423000000ES, 20070423060000ES,,2,2, $/MW-Hour, NO, SC:(AEP:AR:121);RV:(AEP:AR:122);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, , S1235, , ,0,, RESALE,, L, CONFIRMED,, , 20070422121354ES,, 20070422123054ES, TP Comments,, Seller comments go here, Customer comments, Joe Smith, (777)-312-7456, (777)-312-7450, jsmith@xyz.com, John Dealer, (534)-223-4567,,,7019,10, 20070423000000ES, 20070423060000ES,,,,,,, 

Y,8207,,,,,,,,,,,10,10,,,,,,,,, 20070423220070ES, 20070424000000ES,,2,2,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,7019,10, 20070423220070ES, 20070424000000ES,,,,,,, 

N,8234, AAA,123456789, CUSTMR,345678912, N, , CE, MECS, ,,35,35, DAILY, FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,,7,, 20070423000000ES, 20070424000000ES,42,24.5,24.5, $/MW-Day, NO, SC:(AEP:AR:123);RV:(AEP:AR:124);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, P0123 , S123, R765, D123,0,, ORIGINAL,, L, CONFIRMED, pub/AAA/incoming,, 20070422131354ES, 20070422173354ES, 20070422133354ES, Standard daily reservation,, System Operator, Customer comments, Frank Orth, (999)-123-4567, (888)-123-1231, jsmith@xyz.com, Jane Doe, (999)-123-4567, (999)-123-8823,,,,,,,,,,,, 

N,8235, AAA,123456789, CUSTMR,345678912, N, , CE, AMPO, ,,5,5, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,,2,, 20070423060000ES, 20070423070000ES,2.5,2,2, $/MW-Hour, NO, SC:(AEP:AR:125);RV:(AEP:AR:126);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, P0123 , S123, R765, D123,0,, ORIGINAL,,, CONFIRMED, pub/AAA/incoming,, 20070422160523ES, 20070422171203ES, 20070422170523ES, Profile verified,, First piece, Customer comments, System Operator, (888)-123-4567, (888)-123-1231, jsmith@xyz.com, Jane Doe, (999)-123-4567, (999)-123-8823,,,,,,,,,,,, 

Y,8235,,,,,,,,,,,10,10,,,,,,,,, 20070423070000ES, 20070423080000ES,,2.5,2.5,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

Y,8235,,,,,,,,,,,15,15,,,,,,,,, 20070423080000ES, 20070423200700ES,,2.5,2.5,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

Y,8235,,,,,,,,,,,10,10,,,,,,,,, 20070423200700ES, 20070423210000ES,,2.5,2.5,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

Y,8235,,,,,,,,,,,5,5,,,,,,,,, 20070423210000ES, 20070423220070ES,,2.5,2.5,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

N,8242, AAA,123456789, CUSTMR,345678912, N, , CE, VP, ,,20,20,WEEKLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,,4,, 20070423000000ES, 20070430000000ES,123,123,123,$/MW-Week, NO, SC:(AEP:AR:127);RV:(AEP:AR:128);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, P0123 , S123, R765, D123,0,, ORIGINAL,,, CONFIRMED, pub/AAA/incoming,, 20070422160723ES, 20070422223024ES, 20070422171523ES, Bid price refused,, Negotiated OFFER_PRICE accepted,, Joe Smith, (888)-123-4567, (888)-123-1231, jsmith@xyz.com, Jane Doe, (999)-123-4567, (999)-123-8823,,,,,,,,,,,, 

Y,8242,,,,,,,,,,,18,18,,,,,,,,, 20070430000000ES, 20070507000000ES,,123,123,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

013-4.1.6	Examples of Negotiation of Price and Partial Service Offer

013-4.1.6.1	Negotiation with Preconfirmation

a. The Transmission Customer submits a PRECONFIRMED transmission service request using the transrequest template. Initially, the STATUS is set to QUEUED by the OASIS Node.

b. The Seller has the option of setting STATUS via the transsell Template to one of the following: INVALID, RECEIVED, STUDY, COUNTEROFFER, ACCEPTED, DECLINED, or REFUSED.

c. The Seller has the option of entering a CAPACITY_GRANTED and setting the STATUS to COUNTEROFFER via the transell template if the Seller can only provide partial service.

d. If the Seller sets STATUS to ACCEPTED (and, as required by WEQ  013-2.2, the OASIS Node forces the Seller to set OFFER_PRICE equal to BID_PRICE as a condition to setting STATUS to ACCEPTED) and CAPACITY_GRANTED is equal to CAPACITY_REQUESTED, the OASIS Node will immediately set STATUS to CONFIRMED. (WEQ 013-2.2 requires the OASIS Node to set a null CAPACITY_GRANTED equal to CAPACITY_REQUESTED when STATUS is set to ACCEPTED.)

e. The Transmission Customer may WITHDRAW request via transcust template at any time up to point where the Seller sets STATUS to ACCEPTED.

f. Once the STATUS is CONFIRMED, the OFFER_PRICE and CAPACITY_GRANTED officially becomes the terms of the reservation.

013-4.1.6.2	Negotiations without Preconfirmation

a. The Transmission Customer submits a transmission reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest template. Initially the STATUS is set to QUEUED by the OASIS Node.

b. The Seller has the option of setting the STATUS via the transsell template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, DECLINED, COUNTEROFFER, or REFUSED. If the STATUS is set to INVALID (due to invalid entries in the request), DECLINED (due to the Seller determining that the proposed price is not acceptable and further negotiations are not desired), or REFUSED (due to the unavailability of the requested service), the transmission reservation request is terminated.

c. The Seller has the option of entering a CAPACITY_GRANTED and setting the STATUS to COUNTEROFFER via the transell template if the Seller can only provide partial service.

d. If the Seller set the STATUS to RECEIVED or STUDY, and determines that the BID_PRICE is too low, the Seller sets the OFFER_PRICE to the price desired, and sets the STATUS to COUNTEROFFER via the transsell template.

e. The Transmission Customer agrees to the OFFER_PRICE, sets the BID_PRICE equal to the OFFER_PRICE, and sets the STATUS to CONFIRMED via the transcust template.

f. The OFFER_PRICE and CAPACITY_GRANTED with the STATUS of CONFIRMED locks in the terms of the reservation.

013-4.1.6.3	Multiple Step Negotiations

a. The Transmission Customer submits a transmission reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest template. Initially the STATUS is set to QUEUED by the OASIS Node.

b. The Seller has the option of setting the STATUS via the transsell template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, DECLINED, COUNTEROFFER, or REFUSED. If the STATUS is set to INVALID, DECLINED, or REFUSED, the transmission reservation request is terminated.

c. The Seller has the option of entering a CAPACITY_GRANTED and setting the STATUS to COUNTEROFFER via the transell template if the Seller can only provide partial service. If ATC changes before the request reaches the STATUS of CONFIRMED, Seller may change the CAPACITY_GRANTED.

d. The Seller determines that the BID_PRICE is too low, sets the OFFER_PRICE to the desired value, and sets the STATUS to COUNTEROFFER via the transsell template.

e. The Transmission Customer responds to the new OFFER_PRICE with an updated BID_PRICE and sets the STATUS to REBID for re-evaluation by the Seller.

f. The Seller determines that the BID_PRICE now is acceptable, and sets the STATUS to ACCEPTED via the transsell template. The transition to ACCEPTED state requires the OFFER_PRICE to be set to the BID_PRICE: accepting a reservation with an OFFER_PRICE different from BID_PRICE would require the STATUS be set to COUNTEROFFER rather than ACCEPTED (see item c).

g. The Transmission Customer agrees to the OFFER_PRICE and sets the STATUS to CONFIRM via the transcust template.

h. The OFFER_PRICE and CAPACITY_GRANTED with the STATUS as CONFIRMED locks in the terms of the reservation.

013-4.1.6.4	Negotiations Declined by Seller

a. The Transmission Customer submits a transmission reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest template. Initially the STATUS is set to QUEUED by the OASIS Node.

b. The Seller has the option of setting the STATUS via the transsell template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, DECLINED, COUNTEROFFER, or REFUSED. If the STATUS is set to INVALID, DECLINED, or REFUSED, the transmission reservation request is terminated.

c. The Seller determines that the BID_PRICE is too low, sets OFFER_PRICE to his desired value, and sets STATUS to COUNTEROFFER via the transsell template.

d. The Transmission Customer responds to OFFER_PRICE with updated BID_PRICE and sets the STATUS to REBID via the transcust template for re-evaluation by Seller.

e. The Seller breaks off all further negotiations by setting the STATUS to DECLINED, indicating that the price is unacceptable and that he does not wish to continue negotiations.

013-4.1.6.5	Negotiations Withdrawn by Transmission Customer

a. The Transmission Customer submits a transmission reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest. Initially the STATUS is set to QUEUED by the OASIS Node.

b. The Seller has the option of setting the STATUS via the transsell template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, DECLINED, COUNTEROFFER, or REFUSED. If the STATUS is set to INVALID, DECLINED, or REFUSED, the transmission reservation request is terminated.

c. The Seller has the option of entering a CAPACITY_GRANTED and setting the STATUS to COUNTEROFFER via the transell template if the Seller can only provide partial service.

d. The Seller determines that the BID_PRICE is too low, sets the OFFER_PRICE to his desired value, and sets the STATUS to COUNTEROFFER via the transsell template.

e. The Transmission Customer responds to the OFFER_PRICE with an updated BID_PRICE and sets the STATUS to REBID for re-evaluation by Seller.

f. The Seller determines that the BID_PRICE is still too low, sets the OFFER_PRICE to another value, and sets STATUS to COUNTEROFFER via the transsell template.

g. The Transmission Customer breaks off all further negotiations, either because the OFFER_PRICE or CAPACITY_GRANTED are unacceptable, by setting STATUS to WITHDRAWN (or the Transmission Customer/Seller could go through additional iterations of REBID/COUNTEROFFER until negotiations are broken off or the reservation is CONFIRMED).

013-4.1.6.6	Negotiations Superseded by Higher Priority Reservation

a. The Transmission Customer submits a transmission reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest template. Initially the STATUS is set to QUEUED by the OASIS Node.

b. The Seller has the option of setting the STATUS via the transsell template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, DECLINED, COUNTEROFFER, or REFUSED. If the STATUS is set to INVALID, DECLINED, or REFUSED, the transmission reservation request is terminated.

c. If the Seller determines that another reservation has higher priority and must displace this request, he sets the STATUS of this request to SUPERSEDED and the negotiations are terminated.

d. However, if desired and permitted by the tariff, the Seller may set the STATUS of a request in any of these previous states (including COUNTEROFFER and ACCEPTED) to COUNTEROFFER with an OFFER_PRICE which could avoid the request being superseded, thus allowing the Transmission Customer the choice of being SUPERSEDED or accepting the proposed OFFER_PRICE.

013-4.1.7	Audit Template examples

The following examples are included to show the general type of audit report responses that could be expected to be returned by implementations of the audit reporting Templates as documented above.

013-4.1.7.1	Offerings

The following is an example of a hypothetical audit query for daily non-firm offerings to the "DDD" point of delivery for Monday August 17, 2007 (line breaks and indentations added to improve readability):

Response:

REQUEST_STATUS=200 

ERROR_MESSAGE= 

TIME_STAMP=20070821091601ES 

VERSION=1.5 

TEMPLATE=transofferingaudit 

OUTPUT_FORMAT=DATA 

PRIMARY_PROVIDER_CODE=WXYZ 

PRIMARY_PROVIDER_DUNS=78912345 

RETURN_TZ=ES 

DATA_ROWS=14 

COLUMN_HEADERS=RECORD_TYPE, TIME_OF_LAST_UPDATE, MODIFYING_COMPANY_CODE, MODIFYING_NAME, TIME_OF_LAST_UPDATE, SELLER_CODE, SELLER_DUNS, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DEL IVERY, INTERFACE_TYPE, OFFER_START_TIME, OFFER_STOP_TIME, START_TIME, STOP_TIME, CAPACITY, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, ANC_SVC_REQ, SALE_REF, POSTING_REF, CEILING_PRICE, OFFER_PRICE, PRICE_UNITS, SERVICE_DESCRIPTION, NERC_CURTAILMENT_PRIORITY, OTHER_CURTAILMENT_PRIORITY, SELLER_NAME, SELLER_PHONE, SELLER_FAX, SELLER_EMAIL, SELLER_COMMENTS , OFFER_INCREMENT 

U, 20070815131756ES, WXYZ, Jane Doe, 20070815131756ES, WXYZ,78912345, X/WXYZ/AAA-DDD//, AAA, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,800, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48732,102,85, $/MW-Day,,3,, Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com , , DAILY 

U, 20070815124022ES, WXYZ, Jane Doe, 20070815124022ES, WXYZ,78912345, X/WXYZ/AAA-DDD//, AAA, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,850, DAILY, NON-FIRM, POINT_TO_ POINT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48732,102,85, $/MW-Day,,3,, Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com, , DAILY 

U, 20070814120018ES, WXYZ, Joe Smith, 20070814120018ES, WXYZ,78912345, X/WXYZ/AAA-DDD//, AAA, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,850, DAILY, NON-FIRM, POINT_TO_ POINT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48732,102,90, $/MW-Day,,3,, Joe Smith, 123-456-7893, 123-456-7801, smithj@wxyz.com , , DAILY 

I, 20070813171204ES, WXYZ, Supervisor, 20070813171204ES, WXYZ,78912345, X/WXYZ/AAA-DDD//, AAA, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,850, DAILY, NON-FIRM, POINT_TO _POINT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48732,102,95, $/MW-Day,,3,, Supervisor, 123-456-7890, 123-456-7801,, , DAILY 

U, 20070815124022ES, WXYZ, Jane Doe, 20070815124022ES, WXYZ,78912345, X/WXYZ/BBB-DDD//, BBB, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,1200, DAILY, NON-FIRM, POINT_TO_ POINT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48783,102,85, $/MW-Day,,3,, Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com , , DAILY 

U, 20070814120022ES, WXYZ, Joe Smith, 20070814120022ES, WXYZ,78912345, X/WXYZ/BBB-DDD//, BBB, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,1200, DAILY, NON-FIRM, POINT_TO_ POINT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48783,102,90, $/MW-Day,,3,, Joe Smith, 123-456-7893, 123-456-7801, smithj@wxyz.com , , DAILY 

I, 20070813171210ES, WXYZ, Supervisor, 20070813171210ES, WXYZ,78912345, X/WXYZ/BBB-DDD//, BBB, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,1200, DAILY, NON-FIRM, POINT_TO_ POINT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48783,102,95, $/MW-Day,,3,, Supervisor, 123-456-7890, 123-456-7801,, , DAILY 

U, 20070816101000ES, WXYZ, Supervisor, 20070816101000ES, WXYZ,78912345, X/WXYZ/CCC-DDD//, CCC, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,85, DAILY, NON-FIRM, POINT_TO_PO INT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48820,102,102, $/MW-Day,,3,, Supervisor, 123-456-7810, 123-456-7801,, , DAILY 

U, 20070814143807ES, WXYZ, Supervisor, 20070814143807ES, WXYZ,78912345, X/WXYZ/CCC-DDD//, CCC, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,90, DAILY, NON-FIRM, POINT_TO_PO INT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48820,102,102, $/MW-Day,,3,, Supervisor, 123-456-7890, 123-456-7801,, , DAILY 

U, 20070814120023ES, WXYZ, Joe Smith, 20070814120023ES, WXYZ,78912345, X/WXYZ/CCC-DDD//, CCC, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,110, DAILY, NON-FIRM, POINT_TO_P OINT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48820,102,90, $/MW-Day,,3,, Joe Smith, 123-456-7893, 123-456-7801, smithj@wxyz.com , , DAILY 

I, 20070813171214ES, WXYZ, Supervisor, 20070813171214ES, WXYZ,78912345, X/WXYZ/CCC-DDD//, CCC, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,110, DAILY, NON-FIRM, POINT_TO_P OINT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48820,102,95, $/MW-Day,,3,, Supervisor, 123-456-7890, 123-456-7801,, , DAILY 

U, 20070815124023ES, WXYZ, Jane Doe, 20070815124023ES, WXYZ,78912345, X/WXYZ/WXYZ-DDD//, WXYZ, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,340, DAILY, NON-FIRM, POINT_T O_POINT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48855,102,85, $/MW-Day,,3,, Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com , , DAILY 

U, 20070814120023ES, WXYZ, Joe Smith, 20070814120023ES, WXYZ,78912345, X/WXYZ/WXYZ-DDD//, WXYZ, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,340, DAILY, NON-FIRM, POINT_T O_POINT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48855,102,90, $/MW-Day,,3,, Joe Smith, 123-456-7893, 123-456-7801, smithj@wxyz.com , , DAILY 

I, 20070813171222ES, WXYZ, Supervisor, 20070813171222ES, WXYZ,78912345, X/WXYZ/WXYZ-DDD//, WXYZ, DDD, E, 20070814000000ES, 20070817000000ES, 20070817000000ES, 20070818000000ES,340, DAILY, NON-FIRM, POINT_T O_POINT, FULL_PERIOD, FIXED,, SC:M;RF:M,,48855,102,95, $/MW-Day,,3,, Supervisor, 123-456-7890, 123-456-7801,, , DAILY

From this audit report, the daily non-firm offerings on the four paths to "DDD" (AAA-DDD, BBB-DDD, CCC-DDD, and WXYZ-DDD) were all originally posted by WXYZ's "Supervisor" at approximately 17:12 on August 13, 2007, at a price of $95.00 /MW/day discounted from a ceiling price of $102.00.

At approximately 12:00 on August 14, 2007, Joe Smith changed the offer price to $90.00 on all four paths.

At 14:14 on August 14, 2007, "Supervisor" adjusted the capacity available on path X/WXYZ/CCC-DDD// to 90 MW (POSTING_REF = 48820) and set the offer price up to match the tariff ceiling rate (presumably due to the path now being constrained and released from the requirement to have discounted service offered at the same rate as all other unconstrained paths to DDD). Capacity on this path was last updated to a value of 85 MW at 10:10 on August 16, 2007, which is the current information posted on OASIS for this path at the time of the query.

Jane Doe adjusted the price on the three presumably unconstrained paths to DDD at 12:40 on August 15, 2007, to $85.00, which may have been in response to a negotiation for service on one of these paths. No further updates have occurred to the offerings on paths BBB-DDD and WXYZ-DDD since that time.

Finally, the capacity available on path X/WXYZ/AAA-DDD// was updated by Jane Doe from 850 to 800 MW at 13:17 on August 15, 2007, which may have corresponded with final confirmation of a reservation at a negotiated discount by the Transmission Customer that initiated the price change from $90.00 to $85.00.

013-4.1.7.2	Reservations

The following is an example of a hypothetical audit query for a specific transmission service reservation (line breaks and indentations added to improve readability):

Response:

REQUEST_STATUS=200

ERROR_MESSAGE=

TIME_STAMP=19980821092048ES

VERSION=1.5

TEMPLATE=transstatusaudit

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=WXYZ

PRIMARY_PROVIDER_DUNS=78912345

RETURN_TZ=ES

DATA_ROWS=15

COLUMN_HEADERS=RECORD_TYPE, TIME_OF_UPDATE, MODIFYING_COMPANY_CODE, MODIFYING_NAME, CONTINUATION_FLAG, ASSIGNMENT_REF, SELLER_CODE, SELLER_DUNS, CUSTOMER_CODE, CUSTOMER_DUNS, AFFILIATE_FLAG, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK, CAPACITY_REQUESTED, CAPACITY_GRANTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, NERC_CURTAILMENT_PRIORITY, OTHER_CURTAILMENT_PRIORITY, START_TIME, STOP_TIME, CEILING_PRICE, OFFER_PRICE, BID_PRICE, PRICE_UNITS, PRECONFIRMED, ANC_SVC_LINK, ANC _SVC_REQ, POSTING_REF, SALE_REF, REQUEST_REF, DEAL_REF, IMPACTED, COMPETING_REQUEST_FLAG, REQUEST_TYPE, RELATED_REF, NEGOTIATED_PRICE_FLAG, STATUS, STATUS_NOTIFICATION, STATUS_COMMENTS, TIME_QUEUED, RESPONSE_TIME_LIMIT, TIME_OF_LAST_UPDATE, PRIMARY_PROVIDER_COMMENTS, SELLER_REF, SELLER_COMMENTS, CUSTOMER_COMMENTS, SELLER_NAME, SELLER_PHONE, SELLER_FAX, SELLER_EMAIL, CUSTOMER_NAME, CUSTOMER_PHONE, CUSTOMER_FAX, CUSTOMER_EMAIL, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME, REASSIGNED_STOP_TIME, PRIMARY_PROVIDER_APPROVAL,PRIMARY_PROVIDER_CONDITIONS, REASSESSMENT_DUE_TIME, RENEWAL_DUE_TIME, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, ROLLOVER_CAPACITY

U, 20070815131629ES, DEFPM, Alan Trader, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,50, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,85,85, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, L, CONFIRMED, , , 20070815121510ES, 20070815144100ES, 20070815131629ES, ,, , , Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com, Alan Trader, 312-678-9104, 312-678-9100, a .trader@defmarketing.com, , , , ,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,75,75,,,,,,,,, 20070818000000ES, 20070819000000ES,,85,85,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,100,100,,,,,,,,, 20070819000000ES, 20070820000000ES,,85,85,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U, 20070815125042ES, WXYZ, Jane Doe, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,50, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,85,82, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, L, COUNTEROFFER, , , 20070815121510ES, 20070815144100 ES, 20070815125042ES, ,, , , Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com, Alan Trader, 312-678-9104, 312-678-910 0, a.trader@defmarketing.com, , , , ,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,75,75,,,,,,,,, 20070818000000ES, 20070819000000ES,,85,82,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,100,100,,,,,,,,, 20070819000000ES, 20070820000000ES,,85,82,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U, 20070815124811ES, DEFPM, Alan Trader, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,50, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,90,82, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, , REBID, , , 20070815121510ES, 20070815144100ES, 20070 815124811ES, ,, , , Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com, Alan Trader, 312-678-9104, 312-678-9100, a.trader @defmarketing.com,,,,,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,75,75,,,,,,,,, 20070818000000ES, 20070819000000ES,,90,82,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,100,100,,,,,,,,, 20070819000000ES, 20070820000000ES,,90,82,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U, 20070815124100ES, WXYZ, Jane Doe, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,50, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,90,80, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, , COUNTEROFFER, , , 20070815121510ES, 20070815144100 ES, 20070815124100ES, ,, , , Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com, Alan Trader, 312-678-9104, 312-678-910 0, a.trader@defmarketing.com,,,,,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,75,75,,,,,,,,, 20070818000000ES, 20070819000000ES,,90,80,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,100,100,,,,,,,,, 20070819000000ES, 20070820000000ES,,90,80,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

I, 20070815121510ES, DEFPM, Alan Trader, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,,80, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, , QUEUED, , , 20070815121510ES, , 20070815121510ES, ,, , , Company Default, 123-456-7800, 123-456-7801, , Alan Trader, 312-678-9104, 312-678-9100, a.trader@defmarketing.com,,,,,,,,,,, 

I,,,, Y,104392,,,,,,,,,,,75,,,,,,,,,, 20070818000000ES, 20070819000000ES,,,80,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

I,,,, Y,104392,,,,,,,,,,,100,,,,,,,,,, 20070819000000ES, 20070820000000ES,,,80,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

First, this example shows the handling of continuation records which conveyed a time varying demand of 50 MW on August 1, 2007, 75 MW on August 18, 2007, and 100 MW on August 19, 2007. This demand profile was initially entered with the original reservation request (transrequest template) at 12:15 on August 15, 2007, by Alan Trader.

As part of the original reservation, Alan Trader attempted to negotiate a price for service of $80.00 /MW/day. Jane Doe responded to this request with a counter offer at the non-negotiated rate of $90.00 /MW/day at 12:41 on August 15, 2007. The RESPONSE_TIME_LIMIT data element has been updated to reflect the time by which the Transmission Customer must confirm service.

At 12:48, Alan Trader attempted to negotiate further for a rate of $82.00 /MWDay and the reservation status was set to REBID.  Jane Doe responded at 12:50 with a second counter offer restating a negotiated rate of $85.00, which Alan Trader finally agreed to at 13:16 on August 15, 1998. The current posted information on OASIS would show this final CONFIRMED reservation.

013-4.1.8	Reservation Reductions

The following is an example of the reduction template response format for a reservation, 123456, for one day of service at 50 MWs.  Assume that 1) the Customer has redirected 20 MWs for that day on reservation 987654, 2) resold 10 MWs during the off-peak hours on reservation 345678, and 3) had service starting at 16:00 reduced in priority for two hours.

Response:

REQUEST_STATUS=200

ERROR_MESSAGE=

TIME_STAMP=20070207091601ES

VERSION=1.5

TEMPLATE=reduction

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=WXYZ

PRIMARY_PROVIDER_DUNS=78912345

RETURN_TZ=ES

DATA_ROWS=13

COLUMN_HEADERS=CONTINUATION_FLAG, ASSIGNMENT_REF, CAPACITY_GRANTED, CAPACITY_AVAILABLE, START_TIME, STOP_TIME, REDUCTION_TYPE, REDUCTION_REASON, IMPACTING_REF, CAPACITY_REDUCED, NERC_CURTAILMENT_PRIORITY, OTHER_CURTAILMENT_PRIORITY

N,1234567,50,20, 20070206000000ES, 20070206070000ES,,,,, , 

Y,1234567,,,,, REDIRECT,,987654,-20, , 

Y,1234567,,,,, RESALE,,345678,-10, , 

N,1234567,50,30, 20070206070000ES, 20070206160000ES,,,,, , 

Y,1234567,,,,, REDIRECT,,987654,-20, , 

N,1234567,50,30, 20070206160000ES, 20070206180000ES,,,,, , 

Y,1234567,,,,, REDIRECT,,987654,-20, , 

Y,1234567,,,,, CONDITIONAL_EVENT, OSL violation on line xyz,,30,6, 

N,1234567,50,30, 20070206180000ES, 20070206230000ES,,,,, , 

Y,1234567,,,,, REDIRECT,,987654,-20, , 

N,1234567,50,20, 20070206230000ES, 20070207000000ES,,,,, , 

Y,1234567,,,,, REDIRECT,,987654,-20, , 

Y,1234567,,,,, RESALE,,345678,-10, , 

013-4.2	Ancillary Service Linkage Examples

The following examples document the handling of ancillary service linkage to transmission service requests/reservations using the ANC_SVC_LINK data element.

Example 1:

Assume ancillary services SC and RV are mandatory from the Primary Provider, whose code is TPEL, and ancillary services RF, EI, SP and SU are required, but will be defined at a future time.

"SC: (TPEL:RQ); RV: (TPEL:RQ); RF:(:FT); EI:(:FT); SP:(:FT); SU:(:FT)";

Example 2:

Assume ancillary services SC and RV are mandatory from the Primary Provider, whose code is TPEL, and RF, EI, SP and SU are self-supplied. The CUSTOMER_CODE is "CPSE"

"SC: (TPEL:RQ); RV: (TPEL:RQ); RF:(CPSE:SP); EI:(CPSE:SP);

SP:(CPSE:SP); SU:(CPSE:SP)"

Example 3:

Assume ancillary services SC and RV are mandatory from the Primary Provider , whose code is TPEL, and ancillary services RF, EI, SP and SU were purchased via a prior OASIS reservation from Seller "SANC" whose reservation number was "39843". There is sufficient capacity within the Ancillary reservation to handle this Transmission reservation.

"SC:(TPEL:RQ); RV:(TPEL:RQ); RF:(SANC:AR:39843);

EI:(SANC:AR:39843) SP:(SANC:AR:39843); SU:(SANC:AR:39843)"

Example 4:

Assume ancillary services SC and RV are mandatory from the Primary Provider, whose code is TPEL, and ancillary services RF, EI, SP and SU were purchased via prior OASIS reservations from Sellers SANC and TANC, whose reservation numbers where 8763 and 9824 respectively. There is not sufficient capacity within the Ancillary reservation from Seller SANC to handle this Transmission reservation. In this case the OASIS reservation number 8763 will be depleted for the time frame specified within the transmission reservation and the remaining required amount will come from reservation number 9824.

"SC:(TPEL:RQ); RV:(TPEL:RQ); RF:((SANC:AR:8763)(TANC:AR:9824));

EI:((SANC:AR:8763)(TANC:AR:9824));

SP:((SANC:AR:8763)(TANC:AR:9824));

SU:((SANC:AR:8763)(TANC:AR:9824))"

Example 5:

Assume a transmission reservation in the amount of 100MW/hour for a period of one day is made. Ancillary services SC and RV are mandatory from the Primary Provider, whose code is TPEL, and ancillary services RF, EI, SP and SU were purchased via prior OASIS reservations from Sellers SANC and TANC, whose reservation numbers where 8763 and 9824 respectively. There is sufficient capacity within the Ancillary reservation from Seller SANC to handle this transmission reservation, however the purchaser wishes to use only 40 MWs from this Seller. In this case the OASIS reservation number 8763 will be depleted in the amount of 40 MWs for the time frame specified within the transmission reservation and the remaining required amount will come from reservation number 9824.

"SC:(TPEL:RQ); RV:(TPEL:RQ); RF:((SANC:AR:8763:40)(TANC:AR:9824));

EI:((SANC:AR:8763:40)(TANC:AR:9824));

SP:((SANC:AR:8763:40)(TANC:AR:9824));

SU:((SANC:AR:8763:40)(TANC:AR:9824))"