1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

Accept as requested				      Change to Existing Practice

X  Accept as modified below			      Status Quo

Decline

2.  TYPE OF DEVELOPMENT/MAINTENANCE

Per Request:					Per Recommendation:

X  Initiation					  X  Initiation

Modification					  X  Modification

Interpretation				      Interpretation

Withdrawal					      Withdrawal

Principle 					      Principle

Definition 					      Definition

X  Business Practice Standard 			  X  Business Practice Standard

Document 					      Document

Data Element 				      Data Element

Code Value 					      Code Value

X12 Implementation Guide			      X12 Implementation Guide

Business Process Documentation		      Business Process Documentation

3.  RECOMMENDATION

SUMMARY:

- This recommendation addresses WEQ 2008 Annual Plan item 2(a)(i)(2), Conditional Firm Business Practice Standards associated with S&CP Requirements completed in WEQ 2008 Annual Plan item 2(a)(i)(1) and WEQ 2008 Annual Plan item 2(a)(vii)(1).  The proposed standards will be incorporated into WEQ-001, Business Practice Standards for OASIS.  Complementary revisions are also recommended to Standards WEQ-002 and WEQ-003 to support Long-term Firm Point-to-Point Transmission Service with the Conditional Curtailment Option (CCO).

- Note that in adding support for tracking of the provisions for the Conditional Curtailment Option on OASIS, additional changes are being recommended to the templates and template data elements to support the tracking of rollover rights. This is a change to these rollover related data structures over what was previously recommended and ratified as changes to WEQ-002, WEQ-003 and WEQ-013 in February, 2008.  These revisions are intended to minimize the impact of implementation on existing templates while supporting the additional data structures required by this Business Practice Standards recommendation.

- Recommended Standards:

- Modifications to WEQ-001 (New Requirements)

- Business Practice Standards for the Granting and Management of Long-term Firm Point-to-Point Transmission Service with the Conditional Curtailment Option

- NOTE: NAESB Staff will assign appropriate enumeration to this standard to replace “n” with next sequential standard number within WEQ-001

001-0.nn 	Conditional Curtailment Option (CCO) – An option subject to either a Number-of-Hours Criteria or a System-Conditions Criteria that may be specified in a service agreement for Long-Term Firm Point-to-Point Transmission Service when a Transmission Provider determines that it cannot accommodate a request because of insufficient transfer capability.

001-0.nn 	CCO Reservation  A transmission service reservation defined by a Long-Term Firm Point-to-Point Transmission service agreement wherein the customer has chosen the CCO.

001-0.nn	Conditional Curtailment Priority Level  A curtailment priority level associated with a CCO Reservation equal to the priority level of Secondary Network Transmission Service.

001-0.nn 	Number-of-Hours Criteria  The maximum number of hours in a defined period (e.g., annually) that the Transmission Provider is allowed to curtail transmission service for reliability reasons at the Conditional Curtailment Priority Level, as specified in the Long-Term Firm Point-to-Point Transmission service agreement which specifies a CCO.

001-0.nn 	Biennial Reassessment  A study performed by the Transmission Provider to reassess the provision of service under a CCO Reservation and service agreement once every two years to determine if (i) the curtailment condition(s) in the service agreement needs to be modified for purposes of maintaining reliability, or (ii) the CCO Reservation can no longer be reliably provided.

- 001-0.nn 	System-Conditions Criteria  One or more specific system conditions during which the Transmission Provider is allowed to curtail transmission service for reliability reasons at the Conditional Curtailment Priority Level, as specified in the Long-Term Firm Point-to-Point Transmission service agreement which specifies a CCO.

- 001-n	GRANTING AND MANAGING A CCO RESERVATION

- Except as specified in 001-n.1, Limitations and Conditions of Service, and upon request by the Transmission Customer, a Transmission Provider shall offer a Conditional Curtailment Option (CCO) in accordance with the Transmission Provider’s Tariff and any applicable regional or provider-specific business practices when Long-Term Firm Point-to-Point Transmission Service requested would not otherwise be granted.  When such service is granted the following Standards shall be used to document the service granted and manage and track the use of the CCO Reservation on OASIS.

- 001-n.1		Limitations and Conditions of Service

001-n.1.1	A Transmission Provider is not required to offer the CCO if doing so would interfere with the Transmission Provider’s statutory obligations, impair the provision of reliable service to existing firm customers, or in instances where the Transmission Provider has been granted an exemption or waiver of such requirement.

001-n.1.2	A Transmission Provider is under no obligation to offer CCO if Long-term Firm Point-to-Point Transmission Service is available to satisfy the Transmission Customer’s requested service.

001-n.1.3	The Transmission Provider is only required to study the CCO upon request of the Transmission Customer in response to a pending request for Long-Term Firm Point-to-Point Transmission Service.

001-n.1.3.1	The Transmission Customer shall specify in the System Impact Study Agreement whether the Transmission Provider must study either (i) the System-Conditions Criteria, (ii) the Number-of-Hours Criteria or (iii) both.

001-n.1.4 	The Transmission Customer choosing the CCO must choose to either (i) commit to necessary system upgrades in order to receive Long-Term Firm Point-to-Point Transmission Service without the CCO upon completion of the upgrades (i.e., the service is an interim or bridge service only) or (ii) receive the service as an alternative to the construction of transmission upgrades. Such choice shall be documented in the service agreement.

001-n.1.5	The Transmission Customer choosing the CCO must choose either (i) the System-Conditions Criteria or (ii) the Number-of-Hours Criteria. Such choice shall be documented in the service agreement.

001-n.1.6 	If short term firm capability becomes available that would alleviate the constraint(s) associated with a CCO Reservation, the Transmission Provider shall ensure the CCO Reservation is not subject to curtailment at the Conditional Curtailment Priority Level prior to offering that capability for Short-Term Firm Point-to-Point Transmission Service to other Transmission Customers.

- 001-n.2	Posting Requirements Once Service has been Granted

- The Transmission Provider shall post the following information on OASIS in association with the CCO Reservation:

- 001-n.2.1	In accordance with WEQ-002:

- 001-n.2.1.1	An indication that the transmission service is subject to the terms of a CCO and whether the CCO Reservation is subject to the System-Conditions or Number-of-Hours Criteria.

- 001-n.2.1.2	A description of the curtailment criteria or information on how such information may be obtained. Such service description may consist of a summary of the curtailment criteria, the FERC Docket No. associated with the filed service agreement or revision thereto (once established), or a uniform resource locator (URL) where such documentation may be found.

- 001-n.2.1.3	The date and time that the current Number-of-Hours Criteria or System-Conditions Criteria are subject to change based on the next Biennial Reassessment.

- 001-n.2.1.4	If the CCO Reservation is subject to the Number-of-Hours Criteria, the following information shall be posted for each of the intervals over which a maximum number of curtailment hours applies:

- The start and end date/time for the interval over which a given maximum number of curtailment hours applies;

- The maximum number of curtailment hours over the applicable interval.

- The accumulation of hours toward the maximum number of curtailment hours over the applicable interval pursuant to Standard 001-n.4.3.

- 001-n.3	Biennial Reassessment

- 001-n.3.1	The Transmission Provider shall conduct all necessary system studies to complete a Biennial Reassessment of the curtailment condition(s) associated with a CCO Reservation.

- 001-n.3.1.1 	Biennial Reassessments are not applicable to any CCO Reservation where the Transmission Customer has committed to system upgrades.

- 001-n.3.1.2	The Transmission Provider may elect to waive the right to perform a Biennial Reassessment.

- 001-n.3.1.2.1	Notice of waiver of the Biennial Reassessment shall be provided to the Transmission Customer no less than ninety (90) calendar days prior to the end of the reassessment period when any change in curtailment condition(s) would otherwise take effect.

- 001-n.3.1.2.2	Any such waiver of the Biennial Reassessment shall be posted on OASIS as a discretionary action under the Tariff in accordance with WEQ-002 and WEQ-013 such that this information may be queried and viewed using the discretion template.

- 001-n.3.1.2.3	The Transmission Provider shall also waive all Biennial Reassessment studies that are due within 6 months (measured from the study due date of the initial Biennial Reassessment which was waived) for any CCO Reservations that are limited because of the same constraints or general system limitations.

- 001-n.3.1.2.4	Any such waiver shall not affect the right of the Transmission Provider to conduct future Biennial Reassessments.

- 001-n.3.2	The Transmission Provider has the right to limit the scope of the Biennial Reassessment to the curtailment criteria specified in the service agreement, either System-Conditions or Number-of-Hours.

- 001-n.3.3	The Transmission Provider will provide the Biennial Reassessment study results to the Transmission Customer no less than ninety (90) calendar days prior to the date the current Number-of-Hours Criteria or System-Conditions Criteria are subject to change.

- 001-n.3.3.1	The Transmission Provider may require the execution of a study agreement as part of its Biennial Reassessment process provided that this requirement is included in the Transmission Provider’s business practices.

- 001-n.3.3.2	The Transmission Customer shall reimburse the Transmission Provider for any Biennial Reassessment study costs.

001-n.3.4	If the Biennial Reassessment results show the System-Conditions Criteria or Number-of-Hours Criteria remains the same, service will continue unchanged until the next Biennial Reassessment or the end of the reservation period, whichever is sooner.

001-n.3.5	If the Biennial Reassessment results show  the System-Conditions Criteria or Number-of-Hours Criteria has changed, the Transmission Customer may choose to discontinue the service. If the Transmission Customer chooses to discontinue service, the following shall apply:

001-n.3.5.1	The Transmission Customer must provide notice to the Transmission Provider at least thirty (30) calendar days prior to the the date the current Number-of-Hours Criteria or System-Conditions Criteria are subject to change.

001-n.3.5.2	The Transmission Provider shall recall the reserved capacity for the remaining term of service from the date the current Number-of-Hours Criteria or System-Conditions Criteria would have changed through the end date of the reservation.

001-n.3.5.3	If required, the Transmission Provider shall amend the service agreement to reflect the new contract termination date.

001-n.3.5.4	Any rollover rights associated with the CCO Reservation shall be forfieted by the Transmission Customer.

- 001-n.3.6	If the Biennial Reassessment results show the System-Conditions Criteria or Number-of-Hours Criteria has changed and the Transmission Customer does not provide notice to discontinue service, the service will continue subject to such change and the service agreement shall be amended to reflect the change.

- 001-n.3.7	The Transmission Provider shall update the information required under section 001-n.2 to reflect any changes to the System-Conditions Criteria or Number-of-Hours Criteria as a result of the Biennial Reassessment.

- 001-n.4		Management and Curtailment of Service

001-n.4.1	The Transmission Provider may apply the Conditional Curtailment Priority Level to a CCO Reservation in accordance with the terms of the service agreement any time curtailment of service would help to relieve actual or anticipated reliability problems on the transmission system.

- 001-n.4.2	During the period(s) when a CCO Reservation is subject to potential curtailment at the Conditional Curtailment Priority Level, the Transmission Provider shall assess the need for and implement curtailments of transmission service taken under the CCO Reservation on par with Network transmission service from non-designated resources (i.e., Secondary Network).

- 001-n.4.2.1	Prior to or coincident with the Transmission Provider’s implementing any curtailments of service under a CCO Reservation at the Conditional Curtailment Priority Level, the Transmission Provider shall post on OASIS the reduction in each impacted CCO Reservation’s curtailment priority in acordance with WEQ-002 and WEQ-013 such that this information may be queried and viewed using the reduction template.

- 001-n.4.2.2	The Transmission Provider may unilaterally (re)set the transmission service curtailment priority in any Request For Interchange (RFI) or Pending, Confirmed or Implemented Interchange that references such a CCO Reservation.

001-n.4.3	If the CCO Reservation is subject to the Number-of-Hours Criteria, the Transmission Provider shall post the current cumulative count of the actual hours service was curtailed at the Conditional Curtailment Priority Level from the start of the accumulation interval through the end of the previous calendar day.

001-n.4.3.1	If a given CCO Reservation subject to the Number-of-Hours Criteria is actually curtailed at the Conditional Curtailment Priority Level one or more times within a given clock hour, one hour of curtailment shall be accumulated against that CCO Reservation for that clock hour.

001-n.4.3.2	If a given CCO Reservation subject to the Number-of-Hours Criteria is referenced in multiple Requests for Interchange that are actually curtailed at the Conditional Curtailment Priority Level for a given clock hour, one hour of curtailment shall be accumulated against that CCO Reservation for that clock hour.

001-n.4.3.3	If, for a given Transmission Provider, multiple CCO Reservations subject to the Number-of-Hours Criteria are referenced in a given Request for Interchange, i.e. stacked RFI, which is subject to actual curtailment at the Conditional Curtailment Priority Level for a given clock hour, one hour of curtailment shall be accumulated against each CCO Reservation that is in at the Conditional Curtailment Priority Level for that clock hour.

001-n.4.3.3.1	A Transmission Provider may account for the number of hours of curtailment attributed to each CCO Reservation referenced in a given Request for Interchange in a different manner than specified in n.4.3.3 provided the Transmission Provider documents and posts on OASIS the different methodology in the Transmission Provider’s business practices.

- 001-n.4.4	When the Transmission Provider determines that the System-Conditions Criteria limiting a CCO Reservation to the Conditional Curtailment Priority Level is no longer in effect, or that the Number-of-Hours Criteria for a CCO Reservation has been met within the specified term, or for any other reason that firm service curtailment priority is to be applied to the CCO Reservation, the Transmission Provider shall perform the following actions:

- 001-n.4.4.1	The Transmission Provider shall unilaterally update the transmission service curtailment priority associated with any Request For Interchange (RFI) or Pending, Confirmed or Implemented Interchange that references such a CCO Reservation to be on par with Firm Point-to-Point Transmission Service.

- 001-n.4.4.2	The Transmission Provider shall update the information posted on OASIS via the reduction template, if needed, to reflect the removal of the reduction in curtailment priority over time and the reinstatement of firm curtailment priority.

- 001-n.5	Miscellaneous Provisions

001-n.5.1	A CCO Reservation qualifies as firm service that supports the designation of network resources imported from other regions or Transmission Providers or Balancing Areas.

001-n.5.2	A CCO Reservation has the same Redirect rights and restrictions as any other Long-Term Firm Point-to-Point Transmission Service.

001-n.5.2.1	Redirects of a CCO Reservation do not affect the System-Conditions Criteria or Number-of-Hours Criteria for the Parent Reservation.

001-n.5.2.2	In instances where a short-term firm Redirect would not be granted unless the Criteria contained in the Parent Reservation are applied to the Redirect, the Transmission Provider may apply the same System-Conditions Criteria or Number-of-Hours Criteria when granting such Redirect service.

001-n.5.2.2.1	If a given short-term firm Redirect subject to the Number-of-Hours Criteria is curtailed at the Conditional Curtailment Priority Level for a given clock hour, one hour of curtailment shall be accumulated against the Parent CCO Reservation for that clock hour.

001-n.5.2.2.2	If one hour of curtailment has already been accumulated against the Parent Reservation for any given clock hour as prescribed in WEQ 001-n.4.3, then no additional adjustments shall be incurred for that clock hour.

001-n.5.3	A CCO Reservation has the same Transfer rights and restrictions as any other Long-Term Firm Point-to-Point Transmission Service.

001-n.5.3.1	Any Transfer of a CCO Reservation will establish a new CCO Reservation which contains the same Conditional Curtailment Option as the Parent Reservation.

001-n.5.3.2	At the time the request is granted, accummulated hours of curtailment for the current period will be retained on the Parent Reservation and also  applied to the Transfer.

001-n.5.4	A CCO Reservation may not be resold during any period when a Conditional Curtailment Priority Level may be applied.

001-n.5.4.1	A CCO Reservation not subject to a Conditional Curtailment Priority Level for a given period will have the same Resale rights as any other Long-Term Firm Point-to-Point Transmission Service for that period.

- 001-n.5.5	The CCO Reservation shall be eligible for rollover rights in the same manner as any other Long-Term Firm Point-to-Point reservation.

Modifications to WEQ-001 (Revisions noted with redlines)

001-11.1.2	If the Transmission Provider (TP) determines the Reseller is not the legitimate owner of the reserved capacity in the Parent Reservation(s) or that the Resale is contradictory to 001-n.5.4, the TP has the right to nullify the Resale.

Modifications to WEQ-001 Appendix C (Revisions noted with redlines)

001-C 		APPENDIX C – OASIS Standards Exemptions

001-C.2	Requirements associated with conditional firm point-to-point service documented in WEQ-001 Business Practices for Open Access Same Time Information Systems (OASIS), WEQ-002 Business Practices for Open Access Same-Time Information Systems (OASIS) Standards and Communications Protocols and WEQ-013 Business Practices for Open Access Same-Time Information Systems (OASIS) Implementation Guide do not apply to RTOs and ISOs with real-time energy markets.

Modifications to WEQ-002 (Revisions noted with redlines).  Text highlighted in yellow are directions for NAESB Staff and are not part of the recommendation per se.

Business Practices for Open Access Same-Time Information Systems (OASIS) Standards & Communication Protocols

Version 1.5

Additions to the Document Change Log; Date field to be established by NAESB Staff when these changes are incorporated into WEQ-001:

| Date | Section | Description of Change |
| --- | --- | --- |
| TBD | 002-4.3.6.2 | Removed data elements related to tracking rollover rights and renewal and reassessment deadlines from transstatus template. Added data element to flag that additional reservation “provisions” are associated with the reservation and may (should) be queried via the new rollover and cco templates. |
| TBD | 002-4.3.6.2.1 | New template, rollover, defined to provide information related to a reservation’s rollover rights as an extension of the transstatus template. |
| TBD | 002-4.3.6.2.1 | New template, cco, defined to provide information related to a reservation’s conditional curtailment option as an extension of the transstatus template. |
| TBD | 002-4.3.6.3 | Removed primary provider specific data elements from the transsell template that were added in previous revision. Primary Providers are not limited to using the template interface to post information/changes to OASIS. |

Revisions to section 002-4.3.6.2:

002-4.3.6.2	Status of Customer Purchase Request/Reservation (transstatus)

The Status of Customer Purchase Request/Reservation (transstatus) is provided upon the request of any Customer or Provider to indicate the current status of one or more reservation records. Users may also view any transaction's status. However, the SOURCE and SINK may be masked for User requests until Transmission Providers must make source and sink information available at the time the request status posting is updated to show that a transmission request is confirmed.

Continuation records may be returned in association with a transmission reservation to convey information regarding: 1) sale or assignment of transmission rights on the secondary market (reassignments), 2) profiled requests, or 3) service over multiple paths. Each continuation record associated with a transmission reservation shall be identified by the CONTINUATION_FLAG Data Element set to 'Y' and include the ASSIGNMENT_REF Data Element.

When a transmission reservation request acquires its rights to transmission service as the result of a sale or assignment of transmission rights on the secondary market, the identity of the original reservation, capacity, and time interval over which rights are assigned to the new reservation are defined by the Data Elements REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME, and REASSIGNED_STOP_TIME. These Data Elements will be returned in continuation records when more than one set of reassignment information is associated with a reservation.

If the transmission reservation has an associated profile, either as a result of the Transmission Customer’s submission of CAPACITY_REQUESTED and/or BID_PRICE varying over time (support for Customer reservation profiles is at the discretion of the Provider or the Seller’s offering of partial service or negotiation of price, the set of data elements for START_TIME, STOP_TIME, CAPACITY_REQUESTED, BID_PRICE, CAPACITY_GRANTED, OFFER_PRICE and CEILING_PRICE will be returned in continuation records for each segment of the request/reservation profile.

If the Provider supports reservations submitted on multiple paths, continuation records specifying PATH_NAME, POINT_OF_RECEIPT, and POINT_OF_DELIVERY associated with the reservation would be returned in continuation records.

The AFFILIATE_FLAG will be set by the TSIP to indicate whether or not the Customer is an affiliate of the Primary Provider. The NEGOTIATED_PRICE_FLAG will be set by the TSIP to indicate whether the OFFER_PRICE is higher, lower, or the same as the BID_PRICE. Any time that a confirmed transmission reservation's rights to schedule up to the amount of CAPACITY_GRANTED is reduced, either due to secondary market sales, partial displacements, Provider initiated "recalls" of capacity, etc., the IMPACTED Data Element shall be incremented. Specific information regarding the MW level and reason for reduction in reserved capacity is viewable using the reduction Template.

The PRIMARY_PROVIDER_APPROVAL data element is set by the Primary Provider to indicate their approval of PARTIAL_TRANSFER and FULL_TRANSFER requests.  In all other types of transmission requests, this element will be null.

The PRIMARY_PROVIDER_PROVISIONS data element is set to ‘Y’ by the Primary Provider when certain provisions are imposed on the reservation that may be viewed by issuing a query for the rollover and cco templates, otherwise null shall be used.

If applicable, any limitations to renewal/rollover rights associated with a confirmed reservation are documented in the ROLLOVER_xxx Data Elements.  If limitations on rollover rights vary over time, the set of data elements ROLLOVER_START, ROLLOVER_STOP, and ROLLOVER_CAPACITY will be returned in continuation records.

The REASSESSMENT_DUE_TIME and RENEWAL_DUE_TIME Data Elements indicate the deadline when the Primary Provider may exercise their right to review conditions associated with granted long-term service with a Conditional Curtailment Option and the deadline for Customer submission of a renewal request to exercise their on-going rollover rights respectively.

Template: transstatus

- Query

SELLER_CODE*

SELLER_DUNS*

CUSTOMER_CODE*

CUSTOMER_DUNS*

PATH_NAME*

POINT_OF_RECEIPT*

POINT_OF_DELIVERY*

SERVICE_INCREMENT*

TS_CLASS*

TS_TYPE*

TS_PERIOD*

TS_WINDOW*

TS_SUBCLASS*

STATUS*

START_TIME (Beginning time of service)

STOP_TIME

START_TIME_QUEUED (Beginning time queue)

STOP_TIME_QUEUED

NEGOTIATED_PRICE_FLAG

ASSIGNMENT_REF

REASSIGNED_REF

RELATED_REF

SALE_REF

REQUEST_REF

DEAL_REF

COMPETING_REQUEST_FLAG

TIME_OF_LAST_UPDATE

- Response

CONTINUATION_FLAG

ASSIGNMENT_REF

SELLER_CODE

SELLER_DUNS

CUSTOMER_CODE

CUSTOMER_DUNS

AFFILIATE_FLAG (Set by TSIP)

PATH_NAME

POINT_OF_RECEIPT

POINT_OF_DELIVERY

SOURCE

SINK

CAPACITY_REQUESTED

CAPACITY_GRANTED

SERVICE_INCREMENT

TS_CLASS

TS_TYPE

TS_PERIOD

TS_WINDOW

TS_SUBCLASS

NERC_CURTAILMENT_PRIORITY

OTHER_CURTAILMENT_PRIORITY

START_TIME

STOP_TIME

CEILING_PRICE

OFFER_PRICE

BID_PRICE

PRICE_UNITS

PRECONFIRMED

ANC_SVC_LINK

ANC_SVC_REQ

POSTING_REF

SALE_REF

REQUEST_REF

DEAL_REF

IMPACTED (Greater than 0, if another reservation impacts this reservation)

COMPETING_REQUEST_FLAG

REQUEST_TYPE

RELATED_REF

NEGOTIATED_PRICE_FLAG ("L" if Seller accepted Price is lower than OFFER_PRICE in transoffering Template; "H" if higher; otherwise blank)

STATUS

STATUS_NOTIFICATION

STATUS_COMMENTS

TIME_QUEUED

RESPONSE_TIME_LIMIT

TIME_OF_LAST_UPDATE

PRIMARY_PROVIDER_COMMENTS

SELLER_REF

SELLER_COMMENTS

CUSTOMER_COMMENTS

SELLER_NAME

SELLER_PHONE

SELLER_FAX

SELLER_EMAIL

CUSTOMER_NAME

CUSTOMER_PHONE

CUSTOMER_FAX

CUSTOMER_EMAIL

REASSIGNED_REF

REASSIGNED_CAPACITY (Capacity from each previous transaction)

REASSIGNED_START_TIME

REASSIGNED_STOP_TIME

PRIMARY_PROVIDER_APPROVAL

PRIMARY_PROVIDER_PROVISIONS

REASSESSMENT_DUE_TIME

RENEWAL_DUE_TIME

ROLLOVER_START_TIME

ROLLOVER_STOP_TIME

ROLLOVER_CAPACITY

Addition of new section 002-4.3.6.2.1:

002-4.3.6.2.1	Renewal Provisions  (rollover)

The Renewal Provisions (rollover) template provides users with additional information related to the specific provisions for exercising the right to renewal/rollover the associated transmission service reservation.

The query parameters associated with this template may be specified by the user to limit the set of service reservations whose Renewal Provisions are to be returned in the template response.

The deadline for submitting a RENEWAL request to exercise rollover rights (if any) shall be captured in the RENEWAL_DUE_TIME data element.  If known and applicable, duration and capacity limitations to renewal/rollover rights associated with a confirmed reservation are documented in one or more sets of ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY data elements returned as continuation records. Each continuation record associated with a given transmission reservation shall be identified by the CONTINUATION_FLAG Data Element set to 'Y' and include the ASSIGNMENT_REF Data Element.

If there is no additional renewal/rollover information associated with a reservation selected via the query parameters, there will be no record returned in the template response for that reservation.

The query parameters below are to be applied in the same way as to the associated reservation data elements as defined in the transstatus template to select the specific reservations whose renewal/rollover information is to be returned.

Template: rollover

1.	Query

SELLER_CODE*

SELLER_DUNS*

CUSTOMER_CODE*

CUSTOMER_DUNS*

PATH_NAME*

POINT_OF_RECEIPT*

POINT_OF_DELIVERY*

SERVICE_INCREMENT*

TS_CLASS*

TS_TYPE*

TS_PERIOD*

TS_WINDOW*

TS_SUBCLASS*

START_TIME

STOP_TIME

START_TIME_QUEUED

STOP_TIME_QUEUED

ROLLOVER_START_TIME

ROLLOVER_STOP_TIME

ASSIGNMENT_REF

TIME_OF_LAST_UPDATE

2.	Response

CONTINUATION_FLAG

ASSIGNMENT_REF

RENEWAL_DUE_TIME

ROLLOVER_START_TIME

ROLLOVER_STOP_TIME

ROLLOVER_CAPACITY

TIME_OF_LAST_UPDATE

Addition of new section 002-4.3.6.2.1:

002-4.3.6.2.1	Conditional Curtailment Option Provisions (cco)

The Conditional Curtailment Option Provisions (cco) template provides users with additional information related to the specific provisions of a CCO Reservation.

The query parameters associated with this template may be specified by the user to limit the set of service reservations whose Conditional Curtailment Option Provisions are to be returned in the template response.

The CONDITIONAL_CURTAILMENT_OPTION shall indicate whether the CCO Reservation is subject to either the Number-of-Hours (HOURS) or System-Conditions (CONDITIONS) Criteria.  SERVICE_DESCRIPTION shall provide a brief description of the nature of the curtailment conditions or information to direct the user where to find such information.  REASSESSMENT_DUE_TIME specifies the end of the current Biennial Reassessment period when curtailment conditions may be subject to change (if applicable).  If service is not subject to Biennial Reassessment, REASSESSMENT_DUE_TIME will be returned as a null value.

If the CCO service is subject to a Number-of-Hours Criteria over a defined interval (e.g., annually), the tracking of the number of hours actually curtailed when in a Conditional Curtailment Priority Level within each of those defined intervals spanning the reservation will be returned in the response parameters as one or more sets of CONDITIONAL_START_TIME, CONDITIONAL_STOP_TIME, MAXIMUM_HOURS, and ACCUMULATED_HOURS data elements returned as continuation records. Each continuation record associated with a given transmission reservation shall be identified by the CONTINUATION_FLAG Data Element set to 'Y' and include the ASSIGNMENT_REF Data Element.

MAXIMUM_HOURS shall specify the maximum number of hours of actual curtailment of service when in a Conditional Curtailment Priority Level that the reservation may be subject to over the CONDITIONAL_START_TIME/CONDITIONAL_STOP_TIME interval.   ACCUMULATED_HOURS is the current count of hours service was actually curtailed when in a Conditional Curtailment Priority Level.

If there is no CCO information associated with a reservation selected via the query parameters, there will be no record returned in the template response for that reservation.

The query parameters below are to be applied in the same way as to the associated reservation data elements as defined in the transstatus template to select the specific reservations whose CCO Reservation information is to be returned.

Template: cco

1.	Query

SELLER_CODE*

SELLER_DUNS*

CUSTOMER_CODE*

CUSTOMER_DUNS*

PATH_NAME*

POINT_OF_RECEIPT*

POINT_OF_DELIVERY*

SERVICE_INCREMENT*

TS_CLASS*

TS_TYPE*

TS_PERIOD*

TS_WINDOW*

TS_SUBCLASS*

START_TIME

STOP_TIME

START_TIME_QUEUED

STOP_TIME_QUEUED

CONDITIONAL_START_TIME

CONDITIONAL_STOP_TIME

ASSIGNMENT_REF

TIME_OF_LAST_UPDATE

2.	Response

CONTINUATION_FLAG

ASSIGNMENT_REF

CONDITIONAL_CURTAILMENT_OPTION

SERVICE_DESCRIPTION

REASSESSMENT_DUE_TIME

CONDITIONAL_START_TIME

CONDITIONAL_STOP_TIME

MAXIMUM_HOURS

ACCUMULATED_HOURS

TIME_OF_LAST_UPDATE

Revisions to section 002-4.3.6.2.1:

002-4.3.6.3	Seller Approval of Purchase (transsell)

Seller Approval of Purchase (Input) (transsell) is input by a Seller to modify the status and queue of a request by a Customer.

The following fields may be submitted in continuation records for the transsell Template to convey transmission rights from multiple original transmission reservations to this new reservation: REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME, and REASSIGNED_STOP_TIME. Use of continuation records is only supported when using the CSV format upload of Template data.

If the Provider/Seller cannot accommodate the Customer's CAPACITY_REQUESTED and is obligated or elects to offer the Customer partial service that varies over the total period of the reservation or the Provider/Seller supports the negotiation of price on individual segments of a profiled reservation request (support for reservation profiles is at the discretion of the Provider), the set of data elements START_TIME, STOP_TIME, CAPACITY_GRANTED, and OFFER_PRICE must be specified on the first record and START_TIME and STOP_TIME must be repeated in continuation records as necessary for varying CAPACITY_GRANTED and OFFER_PRICE.  In all cases the full profile should be represented.

SELLER_CODE and SELLER_DUNS shall be determined from the registered connection used to input the request. The SELLER_REF Data Element may be set by the SELLER to a seller specific internal tracking number.

If the reservation is subject to the right of first refusal pending a status change to Displaced, the COMPETING_REQUEST_FLAG shall be set to Y, and SELLER_COMMENTS shall be updated with a reference to the competing request’s ASSIGNMENT_REF. If the reservation is subject to the right of first refusal pending a status change to Superseded, the COMPETING_REQUEST_FLAG shall be set to Y, the OFFER_PRICE shall be updated, the SELLER_COMMENTS shall be updated with a reference to the competing requests ASSIGNMENT_REF, and the STATUS shall be set to COUNTEROFFER. Once the disposition of the request is finalized, the COMPETING_REQUEST_FLAG shall be reset to N and any appropriate status change shall be made.

OASIS shall allow the Seller to set the transmission service request’s STATUS to ACCEPTED only when the CAPACITY_REQUESTED and BID_PRICE match the corresponding CAPACITY_GRANTED and OFFER_PRICE data elements over the START_TIME and STOP_TIME of the request.

With the exception for REQUEST_TYPEs of PART_TRANSFER or FULL_TRANSFER, OASIS shall set a pre-confirmed transmission service request to CONFIRMED when the Provider/Seller sets the request’s STATUS to ACCEPTED.  PART_TRANSFER and FULL_TRANSFER requests require that both STATUS be set to ACCEPTED and PRIMARY_PROVIDER_APPROVAL be set to the value of Y prior to OASIS moving the STATUS to CONFIRMED.

If the service requested is subject to specific non-conforming conditions for service, e.g., Conditional Curtailment Option for long-term Firm service, and those conditions are subject to review and renegotiation by the Primary Provider, the time when such a reassessment may be performed is identified in the REASSESSMENT_DUE_TIME.

The deadline for submitting a RENEWAL request to exercise rollover rights shall be captured in the RENEWAL_DUE_TIME data element.  If applicable, any limitations to renewal/rollover rights associated with a confirmed reservation are documented in one or more sets of ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY data elements.

If this template is submitted by the Provider, and the Provider is not listed as the SELLER in the request, the Provider may only submit non-null values for the CONTINATION_RECORD, ASSIGNMENT_REF, PRIMARY_PROVIDER_ APPROVAL, PRIMARY_PROVIDER_COMMENTS, RENEWAL_DUE_TIME, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY data elements; all other template data elements must be null.

If this template is submitted by the Seller that is not the Primary Provider, the PRIMARY_PROVIDER_APPROVAL, PRIMARY_PROVIDER_COMMENTS, PRIMARY_PROVIDER_CONDITIONS, REASSESSMENT_DUE_TIME, RENEWAL_DUE_TIME, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY data elements must be null.

Template: transsell

- Input

CONTINUATION_FLAG

ASSIGNMENT_REF (Required)

START_TIME

STOP_TIME

OFFER_PRICE

CAPACITY_GRANTED

STATUS

STATUS_COMMENTS

ANC_SVC_LINK

ANC_SVC_REQ

COMPETING_REQUEST_FLAG

NEGOTIATED_PRICE_FLAG

SELLER_REF

SELLER_COMMENTS

RESPONSE_TIME_LIMIT

REASSIGNED_REF

REASSIGNED_CAPACITY (Previous capacity to be reassigned)

REASSIGNED_START_TIME

REASSIGNED_STOP_TIME

PRIMARY_PROVIDER_APPROVAL

PRIMARY_PROVIDER_COMMENTS

REASSESSMENT_DUE_TIME

RENEWAL_DUE_TIME

ROLLOVER_START_TIME

ROLLOVER_STOP_TIME

ROLLOVER_CAPACITY

- Response

RECORD_STATUS

CONTINUATION_FLAG

ASSIGNMENT_REF

START_TIME

STOP_TIME

OFFER_PRICE

CAPACITY_GRANTED

STATUS

STATUS_COMMENTS

ANC_SVC_LINK

ANC_SVC_REQ

COMPETING_REQUEST_FLAG

NEGOTIATED_PRICE_FLAG

SELLER_REF

SELLER_COMMENTS

RESPONSE_TIME_LIMIT

REASSIGNED_REF

REASSIGNED_CAPACITY (Previous capacity to be reassigned)

REASSIGNED_START_TIME

REASSIGNED_STOP_TIME

PRIMARY_PROVIDER_APPROVAL

PRIMARY_PROVIDER_COMMENTS

REASSESSMENT_DUE_TIME

RENEWAL_DUE_TIME

ROLLOVER_START_TIME

ROLLOVER_STOP_TIME

ROLLOVER_CAPACITY

ERROR_MESSAGE

Revisions to section 002-5.10.1:

| Template | Deleted Data Element | New Data Element | Conversion Requirements |
| --- | --- | --- | --- |
| transstatus (response) |  | PRIMARY_PROVIDER_CONDITIONS | No data to convert. |
| Transstatus (response) |  | REASSESSMENT_DUE_TIME | No data to convert. |
| Transstatus (response) |  | RENEWAL_DUE_TIME | No data to convert. |
| Transstatus (response ) |  | ROLLOVER_START_TIME | No data to convert. |
| Transstatus (response) |  | ROLLOVER_STOP_TIME | No data to convert. |
| Transstatus (response) |  | ROLLOVER_CAPACITY | No data to convert. |
| transtatus (response) |  | PRIMARY_PROVIDER_PROVISIONS | No data to convert |
| Transsell (input/response) |  | PRIMARY_PROVIDER_APPROVAL | No data to convert |
| rollover (query) |  | ROLLOVER_START_TIME | No data to convert |
| rollover (query) |  | ROLLOVER_STOP_TIME | No data to convert |
| rollover (response) |  | CONTINUATION_FLAG | The Transmission Provider must provide this data for each confirmed Long Term Firm Point-to-Point reservation with rollover rights which has not passed the deadline for submittal of a renewal request. |
| rollover (response) |  | ASSIGNMENT_REF | The Transmission Provider must provide this data for each confirmed Long Term Firm Point-to-Point reservation with rollover rights which has not passed the deadline for submittal of a renewal request. |
| rollover (response) |  | RENEWAL_DUE_TIME | The Transmission Provider must provide this data for each confirmed Long Term Firm Point-to-Point reservation with rollover rights which has not passed the deadline for submittal of a renewal request. |
| rollover (response) |  | ROLLOVER_START_TIME | The Transmission Provider must provide this data for each confirmed Long Term Firm Point-to-Point reservation with rollover rights which has not passed the deadline for submittal of a renewal request. |
| rollover (response) |  | ROLLOVER_STOP_TIME | The Transmission Provider must provide this data for each confirmed Long Term Firm Point-to-Point reservation with rollover rights which has not passed the deadline for submittal of a renewal request. |
| rollover (response) |  | ROLLOVER_CAPACITY | The Transmission Provider must provide this data for each confirmed Long Term Firm Point-to-Point reservation with rollover rights which has not passed the deadline for submittal of a renewal request. |
| rollover (response) |  | TIME_OF_LAST_UPDATE | The Transmission Provider must provide this data for each confirmed Long Term Firm Point-to-Point reservation with rollover rights which has not passed the deadline for submittal of a renewal request. |
| cco (query) |  | CONDTIONAL_START_TIME | No data to convert. |
| cco (query) |  | CONDITIONAL_STOP_TIME | No data to convert. |
| cco (response) |  | CONTINUATION_FLAG | The Transmission Provider must provide this data for each confirmed CCO Reservation. |
| cco (response) |  | ASSIGNMENT_REF | The Transmission Provider must provide this data for each confirmed CCO Reservation. |
| cco (response) |  | CONDITIONAL_CURTAILMENT_OPTION | The Transmission Provider must provide this data for each confirmed CCO Reservation. |
| cco (response) |  | SERVICE_DESCRIPTION | The Transmission Provider must provide this data for each confirmed CCO Reservation. |
| cco (response) |  | REASSESSMENT_DUE_TIME | The Transmission Provider must provide this data for each confirmed CCO Reservation. |
| cco (response) |  | CONDITIONAL_START_TIME | The Transmission Provider must provide this data for each confirmed CCO Reservation. |
| cco (response) |  | CONDITIONAL_STOP_TIME | The Transmission Provider must provide this data for each confirmed CCO Reservation. |
| cco (response) |  | MAXIMUM_HOURS | The Transmission Provider must provide this data for each confirmed CCO Reservation. |
| cco (response) |  | ACCUMULATED_HOURS | The Transmission Provider must provide this data for each confirmed CCO Reservation. |
| cco (response) |  | TIME_OF_LAST_UPDATE | The Transmission Provider must provide this data for each confirmed CCO Reservation. |
| transsell (input/response) |  | PRIMARY_PROVIDER_APPROVAL | No data to convert |
| transsell (input/response) |  | PRIMARY_PROVIDER_CONDITIONS | No data to convert |
| transsell (input/response) |  | PRIMARY_PROVIDER_COMMENTS | No data to convert |
| transsell (input/response) |  | REASSESSMENT_DUE_TIME | No data to convert |
| transsell (input/response) |  | RENEWAL_DUE_TIME | No data to convert |
| transsell (input/response) |  | ROLLOVER_START_TIME | No data to convert |
| transsell (input/response) |  | ROLLOVER_STOP_TIME | No data to convert |
| transsell (input/response) |  | ROLLOVER_CAPACITY | No data to convert |

Modifications to WEQ-003 (Revisions noted with redlines).  Text highlighted in yellow are directions for NAESB Staff and are not part of the recommendation per se.

Additions and changes to section 003-0:

| 003-0 OASIS DATA DICTIONARY Version 1.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Data Dictionary Element Name | Alias | Field Format : minimum characters {type of ASCII} maximum characters | Restricted Values | Definition of Data Element |
| ACCUMULATED_HOURS | ACCUMHRS | 0{NUMERIC}12 | Integer number | The number of hours a CCO Reservation has actually been curtailed when in a Conditional Curtailment Priority Level under the Number-of-Hours criteria within a specified time interval. |
| CONDITIONAL_CURTAILMENT_OPTION | CCOPT | 1{ALPHANUMERIC}10 | Valid Values HOURS CONDITIONS {registered} | In association with a Conditional Curtailment Option service reservation, indicates whether the Number-of-Hours or System-Conditions criteria is applicable. |
| MAXIMUM_HOURS | MAXHRS | 0{NUMERIC}12 | Integer number | The maximum number of hours a Conditional Curtailment Option service reservation may be curtailed under the Number-of-Hours criteria within a specified time interval. |
| PRIMARY_PROVIDER_APPROVAL | PPAPP | 0{ALPHA}1 | Valid Values Y N | To indicate Primary Provider approval of a Transfer. Y = Yes, Primary Provider has approved Transfer N = No, Primary Provider does not approve the Transfer |
| PRIMARY_PROVIDER_PROVISIONS | PPPROV | 0{ALPHA}1 | Valid Values Y Null | To indicate certain Primary Provider provisions apply to the transmission service reservation, e.g., rollover rights, conditional curtailment option, etc. Y = Yes, additional Primary Provider provisions are associated with the transmission service reservation Null = No additional Primary Provider provisions are associated with the transmission service reservation |
| REASSESSMENT_DUE_TIME | RASMTTIME | 0,16{ALPHANUMERIC}16 | Valid date and time to seconds: yyyy+mo+dd+hh+tz | Deadline when the Primary Provider may exercise their right to review conditions associated with granted long-term service with a Conditional Curtailment Option. Time marking the end of the current biennial reassessment interval, if applicable, when the conditions associated with a reservation subject to a conditional curtailment option may be subject to change. |
| CONDITIONAL_START_TIME | CONDSTTIME | 0,16{ALPHANUMERIC}16 | Valid Date and Time to seconds: yyyy+mo+dd+hh +mm+ss+tz | Start of the interval designating the number of ACCUMULATED_HOURS and MAXIMUM_HOURS available. |
| CONDITIONAL_STOP_TIME | CONDSPTIME | 0,16{ALPHANUMERIC}16 | Valid Date and Time to seconds: yyyy+mo+dd+hh +mm+ss+tz | End of the interval designating the number of ACCUMULATED_HOURS and MAXIMUM_HOURS available. |

Modifications to WEQ-013 (Revisions noted with redlines).  Text highlighted in yellow are directions for NAESB Staff and are not part of the recommendation per se.

Business Practices for Open Access Same-Time Information Systems (OASIS) Implementation Guide

Version 1.5

Additions to the Document Change Log; Date field to be established by NAESB Staff when these changes are incorporated into WEQ-001:

| Date | Section | Description of Change |
| --- | --- | --- |
| TBD | 013-2.6.1.3 | Reflected change in the documenting of ongoing rollover rights through the new rollover template instead of the transstatus template. Clarified documentation when there are no limitations to rollover rights. |
| TBD | 013-2.6.1.4 | New section added to describe documenting for conditional curtailment option provisions using the new cco template. |
| TBD | 013-2.6.2 | Reflect the RENEWAL requests may be subject to conditional curtailment option provisions. |

Revisions to section 013-2.6.1.3:

013-2.6.1.3	Rollover Rights

For transmission services that have ongoing rollover or renewal rights, the Transmission Provider shall document those rights and any limitations over time through thetransstatus rollover template data elements RENEWAL_DUE_TIME, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY.  The Transmission Provider must also flag that the service request/reservation has associated rollover rights by setting the transstatus template PRIMARY_PROVIDER_PROVISIONS data element to “Y”. Continuation records shall be used if the ROLLOVER_CAPACITY value changes as a function of time by listing each applicable time interval and capacity eligible for rollover/renewal in the ROLLOVER_START_TIME, ROLLOVER_STOP_TIME and ROLLOVER_CAPACITY data elements.  The first value for ROLLOVER_START_TIME returned in the rollover template response shall be set to the value of the latest STOP_TIME value associated with the request/reservation as shown in the transstatus template response, i.e., rollover template response records begin at the end of the transmission service requested/reserved.  If there is no limitation on rollover rights, ROLLOVER_STOP_TIME shall be null and the ROLLOVER_CAPACITY shall indicate the capacity eligible for rollover for all future time.

Addition of new section 013-2.6.1.4:

013-2.6.1.4	Conditional Curtailment Option

For transmission services that are subject to a Conditional Curtailment Option (CCO Reservations), the Transmission Provider shall set the PRIMARY_PROVIDER_PROVISIONS data element to “Y” and provide documentation on the nature of the Conditional Curtailment Option such that this information may be queried and viewed using the cco template.  Prior to or upon confirming a transmission service reservation with a Conditional Curtailment Option, the Transmission Provider shall make available the information required in cco template CONDITIONAL_CURTAILMENT_OPTION, SERVICE_DESCRIPTION and REASSESSMENT_DUE_TIME_data elements.  REASSESSMENT_DUE_TIME must reflect the date and time that the current Conditional Curtailment Option conditions may be subject to change as a result of a Biennial Reassessment performed by the Transmission Provider.  If the CCO Reservation is not subject to a Biennial Reassessment or is shorter than two years in duration, REASSESSMENT_DUE_TIME shall be null.

If the CCO Reservation is subject to the Number-of-Hours criteria (CONDITIONAL_CURTAILMENT_OPTION = “HOURS”), the Transmission Provider must indicate the maximum number of actual hours that service may be curtailed at the Conditional Curtailment Priority Level over a specified measurement interval set by the CONDITIONAL_START_TIME/
CONDITIONAL_STOP_TIME data elements.  If there is more than one such interval during the current service period up to the next REASSESSMENT_DUE_TIME, each such interval shall be returned in cco template continuation records containing the CONTINUATION_FLAG, ASSIGNMENT_REF, CONDITIONAL_START_TIME, CONDITIONAL_STOP_TIME, MAXIMUM_HOURS and ACCUMULATED_HOURS Data Elements.

The Transmission Provider shall update the ACCUMULATED_HOURS Data Element with the current cummulative number of hours service under the CCO Reservation was actually curtailed at the Conditional Curtailment Priority Level for that interval. Once the accumulation CONDITIONAL_START_TIME/CONDITIONAL_STOP_TIME interval has expired, the final ACCUMULATED_HOURS over that interval shall be retained and returned in the cco template response as the historical record of curtailments made against the CCO Reservation over time.

For CCO Reservations subject to Biennial Reassessment, the Transmission Provider shall update any new accumulation interval and/or change in maximum curtailment hours through the end of the next reassessment interval by updating the REASSESSMENT_DUE_TIME, CONDITIONAL_START_TIME, CONDITIONAL_STOP_TIME, MAXIMUM_HOURS and ACCUMULATED_HOURS Data Elements returned in the response to a query issued for the cco template.  This information shall be provided at the same time that the reassessment study report is issued to the Transmission Customer,

If the CCO Reservation is subject to the System-Conditions criteria (CONDITIONAL_CURTAILMENT_OPTION = “CONDITIONS”), the Transmission Provider may, at their option, return information on the accummulated number of hours service was actually curtailed using the corresponding cco template Data Elements.  For these CCO Reservations, the MAXIMUM_HOURS Data Element will be returned as a null value.

Each confirmed PART_TRANSFER or FULL_TRANSFER reservation made against a CCO Reservation shall also be a CCO Reservation. Upon acceptance of  the Transfer of a CCO Reservation, the Transmission Provider shall set the PRIMARY_PROVIDER_PROVISIONS data element to “Y” for the Transfer,  populate the cco template and assign the ACCUMULATED_HOURS of the Parent Reservation to the Transfer. Each subsequent curtailment shall be accumulated against the Parent Reservation or Transfer, as applicable.

Revisions to section 013-2.6.2:

013-2.6.2	RENEWAL Requests

Transmission Customers shall use the REQUEST_TYPE of RENEWAL to exercise rollover rights associated with an existing transmission service reservation held by the Transmission Customer.  RENEWAL requests must always specify the Primary Provider as SELLER.

The following are specific restrictions or requirements for OASIS service requests with REQUEST_TYPE of RENEWAL.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be RENEWAL |
| RELATED_REF | Must specify the ASSIGNMENT_REF associated with an existing confirmed transmission service reservation held by the Transmission Customer that 1) has rollover rights, and 2) whose rollover rights have not expired. |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| PATH | Must represent the same corresponding service points in the reservation specified in RELATED_REF |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must specify a set of valid transmission service attributes recognized by the Primary Provider as a valid service designation eligible for exercising of rollover rights held by the reservation specified in RELATED_REF |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must match the STOP_TIME of the reservation specified in RELATED_REF |
| STOP_TIME | With START_TIME, must specify a valid interval of service eligible for exercising of renewal/rollover rights held by the reservation specified in RELATED_REF |
| CAPACITY_REQUESTED | Must be less than or equal to the amount of capacity eligible for renewal/rollover over the interval of service |
| BID_PRICE | Must specify the price to be paid for the service requested |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

RENEWAL requests must be submitted on OASIS prior to expiration of the Transmission Customer’s rollover rights as established by the Tariff orbusiness practice.

CUSTOMER_CODE and CUSTOMER_DUNS in the RENEWAL request should correspond to the CUSTOMER_CODE and CUSTOMER_DUNS in the RELATED_REF reservation.  If not, the Transmission Provider should verify that the submitting Customer has a valid agency agreement with the original transmission service agreement holder and is authorized to submit such a request on behalf of that entity.

The transmission service attributes, e.g., TS_CLASS, etc., should match the corresponding attributes in the reservation specified in RELATED_REF.  However, changes may be made to these attributes over time such that some differences are necessary to accommodate changes in the Primary Provider’s business practices.  This also applies to changes in service points, e.g., PATH, etc., over time.

RENEWAL requests may be subject to offering of partial service and negotiation, and limitation of on-going rollover rights, and conditional curtailment option just as an ORIGINAL request.

Revisions to 013-4.1.5:

b. Submission of Reassignment Information - Case 1:

In the prior example (last data record in transrequest) a RESALE reservation request was submitted to the Reseller “EFG” by the Transmission Customer for 20MW of DAILY firm scheduling rights from 04:00 to 16:00. Assume that the Reseller has previously reserved service for the CEF-ECS path for Daily Firm in amount of 50 MW on 4/23 under ASSIGNMENT_REF=23877019, and another 10 MW on April 23, 2007, under ASSIGNMENT_REF=23877880. Reseller must designate which transmission service rights are to be reassigned to Transmission Customer to satisfy the 20MW from 04:00 to 16:00. This reassignment information is conveyed by Reseller using the transsell template when the reservation request is ACCEPTED. At the SELLER's discretion, rights are assigned for the first four hours of the resale from the first reservation (ASSIGNMENT_REF=23877019).  The balance of the resale is supported by 10 MWs from the second reservation first (ASSIGNMENT_REF=23877880) with the balance taken up by the first reservation (ASSIGNMENT_REF=23877019).

This split of reassignment information is conveyed in "continuation" records via the transsell template Data Elements REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME, and REASSIGNED_STOP_TIME.

Input:

VERSION=1.5

TEMPLATE=transsell

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=3

COLUMN_HEADERS=CONTINUATION_FLAG, ASSIGNMENT_REF, START_TIME, STOP_TIME, OFFER_PRICE, CAPACITY_GRANTED, STATUS, STATUS_COMMENTS, ANC_SVC_LINK, ANC_SVC_REQ, COMPETING_REQUEST_FLAG, NEGOTIATED_PRICE_FLAG, SELLER_REF, SELLER_COMMENTS, RESPONSE_TIME_LIMIT, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME , PRIMARY_PROVIDER_APPROVAL, PRIMARY_PROVIDER_COMMENTS, REASSESSMENT_DUE_TIME, RENEWAL_DUE_TIME, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, ROLLOVER_CAPACITY

N,23879040, 20070423040000ES, 20070423160000ES,2,20, ACCEPTED, Status comments here, , , N, , S78234, Seller comments here, 20070421153720ES,23877019,20, 20070423040000ES, 20070423080000ES,,,,,,, 

Y,23879040,,,,,,,,,,,,,,23877880,10, 20070423080000ES, 20070423160000ES,,,,,,, 

Y,23879040,,,,,,,,,,,,,,23877019,10, 20070423080000ES, 20070423160000ES,,,,,,, 

Response:

REQUEST_STATUS=200

ERROR_MESSAGE=Successfully updated. 

TIME_STAMP=20070422160523ES 

VERSION=1.5

TEMPLATE=transsell

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=3

COLUMN_HEADERS=RECORD_STATUS, CONTINUATION_FLAG, ASSIGNMENT_REF,  START_TIME, STOP_TIME, OFFER_PRICE,CAPACITY_GRANTED, STATUS, STATUS_COMMENTS, ANC_SVC_LINK, ANC_SVC_REQ, COMPETING_REQUEST_FLAG, NEGOTIATED_PRICE_FLAG, SELLER_REF, SELLER_COMMENTS, RESPONSE_TIME_LIMIT, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME , PRIMARY_PROVIDER_APPROVAL, PRIMARY_PROVIDER_COMMENTS, REASSESSMENT_DUE_TIME, RENEWAL_DUE_TIME, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, ROLLOVER_CAPACITY, ERROR_MESSAGE

200,N,23879040, 20070423040000ES, 20070423160000ES,2,20, ACCEPTED, Status comments here, , , N, , S78234, Seller comments here, 20070421153720ES,23877019,20, 20070423040000ES, 20070423080000ES,,,,,,, , No Error

200,Y,23879040,,,,,,,,,,,,,,23877880,10, 20070423080000ES, 20070423160000ES,,,,,,, , No Error

200,Y,23879040,,,,,,,,,,,,,,23877019,10, 20070423080000ES, 20070423160000ES,,,,,,, , No Error

d. Query of Transmission Reservation Status:

The following is a hypothetical response to a transstatus query that might be delivered for reservations starting on April 23, 2007.

Input:

<appropriate query name/value pairs to return reservations for April 23, 2007>

Response:

REQUEST_STATUS=200

ERROR_MESSAGE=No error. 

TIME_STAMP=20000423160523ES

VERSION=1.5

TEMPLATE=transstatus

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=AAA

PRIMARY_PROVIDER_DUNS=123456789

RETURN_TZ=ES

DATA_ROWS=10

COLUMN_HEADERS=CONTINUATION_FLAG, ASSIGNMENT_REF, SELLER_CODE, SELLER_DUNS, CUSTOMER_CODE, CUSTOMER_DUNS, AFFILIATE_FLAG, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK,CAPACITY_REQUESTED,CAPACITY_GRANTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD,TS_WINDOW, TS_SUBCLASS, NERC_CURTAILMENT_PRIORITY, OTHER_CURTAILMENT_PRIORITY, START_TIME, STOP_TIME, CEILING_PRICE, OFFER_PRICE, BID_PRICE, PRICE_UNITS, PRECONFIRMED, ANC_SVC_LINK, ANC_SVC_REQ, POSTING_REF, SALE_REF, REQUEST_REF, DEAL_REF, IMPACTED, COMPETING_REQUEST_FLAG, REQUEST_TYPE, RELATED_REF, NEGOTIATED_PRICE_FLAG, STATUS, STATUS_NOTIFICATION, STATUS_COMMENTS, TIME_QUEUED, RESPONSE_TIME_LIMIT, TIME_OF_LAST_UPDATE, PRIMARY_PROVIDER_COMMENTS, SELLER_REF, SELLER_COMMENTS, CUSTOMER_COMMENTS, SELLER_NAME, SELLER_PHONE, SELLER_FAX, SELLER_EMAIL, CUSTOMER_NAME, CUSTOMER_PHONE, CUSTOMER_FAX, CUSTOMER_EMAIL, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME, PRIMARY_PROVIDER_APPROVAL, PRIMARY_PROVIDER_PROVISIONS 

REASSESSMENT_DUE_TIME, RENEWAL_DUE_TIME, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, ROLLOVER_CAPACITY

N,8207, RSELLR,234567890, ACSTMR,987654321, N, , CE, VP, ,,10,10, HOURLY, FIRM, POINT_TO_POINT, OFF_PEAK, FIXED,,7,, 20070423000000ES, 20070423060000ES,,2,2, $/MW-Hour, NO, SC:(AEP:AR:121);RV:(AEP:AR:122);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, , S1235, , ,0,, RESALE,, L, CONFIRMED,, , 20070422121354ES,, 20070422123054ES, TP Comments,, Seller comments go here, Customer comments, Joe Smith, (777)-312-7456, (777)-312-7450, jsmith@xyz.com, John Dealer, (534)-223-4567,,,7019,10, 20070423000000ES, 20070423060000ES,,,,,, 

Y,8207,,,,,,,,,,,10,10,,,,,,,,, 20070423220070ES, 20070424000000ES,,2,2,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,7019,10, 20070423220070ES, 20070424000000ES,, ,,,, 

N,8234, AAA,123456789, CUSTMR,345678912, N, , CE, MECS, ,,35,35, DAILY, FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,,7,, 20070423000000ES, 20070424000000ES,42,24.5,24.5, $/MW-Day, NO, SC:(AEP:AR:123);RV:(AEP:AR:124);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, P0123 , S123, R765, D123,0,, ORIGINAL,, L, CONFIRMED, pub/AAA/incoming,, 20070422131354ES, 20070422173354ES, 20070422133354ES, Standard daily reservation,, System Operator, Customer comments, Frank Orth, (999)-123-4567, (888)-123-1231, jsmith@xyz.com, Jane Doe, (999)-123-4567, (999)-123-8823,,,,,,, ,,,, 

N,8235, AAA,123456789, CUSTMR,345678912, N, , CE, AMPO, ,,5,5, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,,2,, 20070423060000ES, 20070423070000ES,2,2,2, $/MW-Hour, NO, SC:(AEP:AR:125);RV:(AEP:AR:126);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, P0123 , S123, R765, D123,0,, ORIGINAL,,, CONFIRMED, pub/AAA/incoming,, 20070422160523ES, 20070422171203ES, 20070422170523ES, Profile verified,, First piece, Customer comments, System Operator, (888)-123-4567, (888)-123-1231, jsmith@xyz.com, Jane Doe, (999)-123-4567, (999)-123-8823,,,,,,, ,,,, 

Y,8235,,,,,,,,,,,10,10,,,,,,,,, 20070423070000ES, 20070423080000ES,2.5,2.5,2.5,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, ,,,, 

Y,8235,,,,,,,,,,,15,15,,,,,,,,, 20070423080000ES, 20070423200700ES,2.5,2.5,2.5,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, ,,,, 

Y,8235,,,,,,,,,,,10,10,,,,,,,,, 20070423200700ES, 20070423210000ES,2.5,2.5,2.5,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, ,,,, 

Y,8235,,,,,,,,,,,5,5,,,,,,,,, 20070423210000ES, 20070423220070ES,2.5,2.5,2.5,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, ,,,, 

N,8242, AAA,123456789, CUSTMR,345678912, N, , CE, VP, ,,20,20,WEEKLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,,4,, 20070423000000ES, 20070430000000ES,123,123,123,$/MW-Week, NO, SC:(AEP:AR:127);RV:(AEP:AR:128);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, P0123 , S123, R765, D123,0,, ORIGINAL,,, CONFIRMED, pub/AAA/incoming,, 20070422160723ES, 20070422223024ES, 20070422171523ES, Bid price refused,, Negotiated OFFER_PRICE accepted,, Joe Smith, (888)-123-4567, (888)-123-1231, jsmith@xyz.com, Jane Doe, (999)-123-4567, (999)-123-8823,,,,,,, ,,,, 

Y,8242,,,,,,,,,,,18,18,,,,,,,,, 20070430000000ES, 20070507000000ES,123,123,123,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, ,,,, 

Revisions to 013-4.1.7.2:

013-4.1.7.2	Reservations

The following is an example of a hypothetical audit query for a specific transmission service reservation (line breaks and indentations added to improve readability):

Response:

REQUEST_STATUS=200

ERROR_MESSAGE=

TIME_STAMP=19980821092048ES

VERSION=1.5

TEMPLATE=transstatusaudit

OUTPUT_FORMAT=DATA

PRIMARY_PROVIDER_CODE=WXYZ

PRIMARY_PROVIDER_DUNS=78912345

RETURN_TZ=ES

DATA_ROWS=15

COLUMN_HEADERS=RECORD_TYPE, TIME_OF_UPDATE, MODIFYING_COMPANY_CODE, MODIFYING_NAME, CONTINUATION_FLAG, ASSIGNMENT_REF, SELLER_CODE, SELLER_DUNS, CUSTOMER_CODE, CUSTOMER_DUNS, AFFILIATE_FLAG, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK, CAPACITY_REQUESTED, CAPACITY_GRANTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, NERC_CURTAILMENT_PRIORITY, OTHER_CURTAILMENT_PRIORITY, START_TIME, STOP_TIME, CEILING_PRICE, OFFER_PRICE, BID_PRICE, PRICE_UNITS, PRECONFIRMED, ANC_SVC_LINK, ANC _SVC_REQ, POSTING_REF, SALE_REF, REQUEST_REF, DEAL_REF, IMPACTED, COMPETING_REQUEST_FLAG, REQUEST_TYPE, RELATED_REF, NEGOTIATED_PRICE_FLAG, STATUS, STATUS_NOTIFICATION, STATUS_COMMENTS, TIME_QUEUED, RESPONSE_TIME_LIMIT, TIME_OF_LAST_UPDATE, PRIMARY_PROVIDER_COMMENTS, SELLER_REF, SELLER_COMMENTS, CUSTOMER_COMMENTS, SELLER_NAME, SELLER_PHONE, SELLER_FAX, SELLER_EMAIL, CUSTOMER_NAME, CUSTOMER_PHONE, CUSTOMER_FAX, CUSTOMER_EMAIL, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME, REASSIGNED_STOP_TIME, PRIMARY_PROVIDER_APPROVAL,

PRIMARY_PROVIDER_PROVISIONS

REASSESSMENT_DUE_TIME, RENEWAL_DUE_TIME, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, ROLLOVER_CAPACITY

U, 20070815131629ES, DEFPM, Alan Trader, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,50, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,85,85, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, L, CONFIRMED, , , 20070815121510ES, 20070815144100ES, 20070815131629ES, ,, , , Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com, Alan Trader, 312-678-9104, 312-678-9100, a .trader@defmarketing.com, , , , ,,,,,, 

U,,,, Y,104392,,,,,,,,,,,75,75,,,,,,,,, 20070818000000ES, 20070819000000ES,,85,85,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,100,100,,,,,,,,, 20070819000000ES, 20070820000000ES,,85,85,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U, 20070815125042ES, WXYZ, Jane Doe, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,50, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,85,82, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, L, COUNTEROFFER, , , 20070815121510ES, 20070815144100 ES, 20070815125042ES, ,, , , Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com, Alan Trader, 312-678-9104, 312-678-910 0, a.trader@defmarketing.com, , , , ,,,,,, 

U,,,, Y,104392,,,,,,,,,,,75,75,,,,,,,,, 20070818000000ES, 20070819000000ES,,85,82,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,100,100,,,,,,,,, 20070819000000ES, 20070820000000ES,,85,82,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U, 20070815124811ES, DEFPM, Alan Trader, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,50, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,90,82, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, , REBID, , , 20070815121510ES, 20070815144100ES, 20070 815124811ES, ,, , , Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com, Alan Trader, 312-678-9104, 312-678-9100, a.trader @defmarketing.com,,,,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,75,75,,,,,,,,, 20070818000000ES, 20070819000000ES,,90,82,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,100,100,,,,,,,,, 20070819000000ES, 20070820000000ES,,90,82,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U, 20070815124100ES, WXYZ, Jane Doe, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,50, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,90,80, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, , COUNTEROFFER, , , 20070815121510ES, 20070815144100 ES, 20070815124100ES, ,, , , Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com, Alan Trader, 312-678-9104, 312-678-910 0, a.trader@defmarketing.com,,,,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,75,75,,,,,,,,, 20070818000000ES, 20070819000000ES,,90,80,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

U,,,, Y,104392,,,,,,,,,,,100,100,,,,,,,,, 20070819000000ES, 20070820000000ES,,90,80,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

I, 20070815121510ES, DEFPM, Alan Trader, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,,80, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, , QUEUED, , , 20070815121510ES, , 20070815121510ES, ,, , , Company Default, 123-456-7800, 123-456-7801, , Alan Trader, 312-678-9104, 312-678-9100, a.trader@defmarketing.com,,,,,,,,,, 

I,,,, Y,104392,,,,,,,,,,,75,,,,,,,,,, 20070818000000ES, 20070819000000ES,,,80,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

I,,,, Y,104392,,,,,,,,,,,100,,,,,,,,,, 20070819000000ES, 20070820000000ES,,,80,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, 

First, this example shows the handling of continuation records which conveyed a time varying demand of 50 MW on August 1, 2007, 75 MW on August 18, 2007, and 100 MW on August 19, 2007. This demand profile was initially entered with the original reservation request (transrequest template) at 12:15 on August 15, 2007, by Alan Trader.

As part of the original reservation, Alan Trader attempted to negotiate a price for service of $80.00 /MW/day. Jane Doe responded to this request with a counter offer at the non-negotiated rate of $90.00 /MW/day at 12:41 on August 15, 2007. The RESPONSE_TIME_LIMIT data element has been updated to reflect the time by which the Transmission Customer must confirm service.

At 12:48, Alan Trader attempted to negotiate further for a rate of $82.00 /MWDay and the reservation status was set to REBID.  Jane Doe responded at 12:50 with a second counter offer restating a negotiated rate of $85.00, which Alan Trader finally agreed to at 13:16 on August 15, 1998. The current posted information on OASIS would show this final CONFIRMED reservation.

4.  SUPPORTING DOCUMENTATION

a.  Description of Request:

In the FERC Order 890 under Docket Nos. RM05-17-000 and RM05-25-000 dated February 16, 2007, the Commission made the following determinations about the requirement for the offering of a conditional curtailment option for long-term firm point-to-point transmission service:

911. The Commission has determined that modifications to the current planning redispatch requirement and creation of a conditional firm option are both necessary for provision of reliable and non-discriminatory point-to-point transmission service. The planning redispatch and conditional firm options represent different ways of addressing similar problems. They can be used to remedy a system condition that occurs infrequently and prevents the granting of a long-term firm point-to-point service. These options also can be used to provide service until transmission upgrades are completed to provide fully firm service. Planning redispatch involves an ex ante determination of whether out-of-merit order generation resources can be used to maintain firm service. Conditional firm involves an ex ante determination of whether there are limited conditions or hours under which firm service can be curtailed to allow firm service to be provided in all other conditions or hours. As we explain below, both techniques are currently used under certain conditions by transmission providers to serve native load and, hence, it is necessary to make comparable services available to transmission customers in order to avoid undue discrimination.

926. We are however modifying the planning redispatch obligation, and similarly limiting the conditional firm option, to better reflect the manner in which redispatch or special protections schemes are used by transmission providers, in recognition of certain legitimate reliability concerns and the inherent difficulty of long-term projections in this area. This Final Rule limits transmission providers’ planning redispatch obligations by removing the current obligation to provide planning redispatch for an indefinite period as long as the redispatch is cheaper than the relevant transmission upgrades. We also limit the conditional firm option by linking it to the transmission upgrades or a biennial assessment of the conditions.

958. If the transmission provider determines that planning redispatch is available, it shall provide the customer with non-binding estimates of the incremental costs of redispatch and identify the relevant constrained flowgates for which redispatch will be provided. For the conditional firm option, the transmission provider shall identify the conditions and hours pursuant to which the service may be curtailed, using a secondary network curtailment priority, to maintain reliability. Specifically, the transmission provider shall identify (1) the specific system condition(s) when conditional curtailment may apply and (2) the annual number of hours when conditional curtailment may apply. Customers agreeing to take conditional firm service must choose one of these options, conditions or hours.

959. Where the customer requests firm service for more than two years, but is unwilling to commit to a facilities study or the payment of network upgrade costs, the transmission provider shall identify and provide the planning redispatch or conditional firm options subject to the following limitation. The transmission provider shall have a periodic right to reassess (1) the planning redispatch required to keep the service firm or (2) the conditions or hours under which the transmission provider may conditionally curtail the service. This reassessment may occur every two years during the term of the service, i.e., at the end of year two, year four, year six, and year eight of a ten-year service. The transmission provider may not implement reassessments during intervening periods nor may it reassess the conditions in order to amend the service agreement in an intervening year should it forego any biennial reassessment.

960. The service agreement shall specify the relevant congested transmission facilities and whether the transmission provider will provide planning redispatch, a mix of planning redispatch and conditional firm, or conditional firm in order to provide the point-to-point transmission service. For the conditional firm option, customers must choose among and the service agreement must specify either (1) specific system condition(s) during which conditional curtailment may occur or (2) annual number of conditional curtailment hours during which conditional curtailment may occur. We deem that any service agreement that incorporates planning redispatch or conditional firm options is a non-conforming agreement and must be filed by the transmission provider pursuant to section 205 of the FPA. Additionally, transmission providers must file with the Commission any amendments to these service agreements that result from reassessments. If a transmission provider proposes to change the redispatch or conditional curtailment conditions due to a reassessment, the transmission provider must provide the reassessment study to the customer along with a narrative statement describing the study and reasons for changes to the curtailment conditions or redispatch requirements no later than 90 days prior to the date for imposition of these new conditions or requirements. The transmission provider shall assess the conditions based on two years of service or the continuation of the term of service, whichever is less.

961. In situations in which the customer commits to paying the costs associated with upgrades necessary to provide the service on a fully firm basis, the conditions or hours identified by the transmission provider shall remain in effect until such time as the upgrades have been completed. Also, for such customers, the service agreement shall specify the upgrade costs as determined through the facilities study.

980. For customers supporting the construction of upgrades, the planning redispatch or conditional firm options will serve as a bridge until upgrades are constructed to remedy the congested transmission facilities. For these customers, the transmission provider must offer planning redispatch or conditional firm service until the time when the upgrades are constructed. The conditions or redispatch applicable to this period must be specified in the service agreement and are not subject to change. We impose this requirement because customers who commit to support transmission upgrades are typically those financing and constructing new resources. These customers require certainty both with regard to upgrade costs and, before upgrades can be constructed, the redispatch requirements or curtailment conditions that may apply to their service. We disagree with Williams and BP Energy that requiring transmission providers to offer this bridge product will present more opportunities for undue discrimination. As we note above, available information on transmission providers’ current uses of redispatch and curtailment plans for their retail native load indicates that the mechanisms are used for relatively short periods of time until upgrades are completed to resolve the transmission insufficiencies. Comparable services for long-term point-to-point customers should therefore be similarly limited to shorter time periods or otherwise linked to transmission upgrades.

981. For customers choosing not to support the construction of new facilities, the planning redispatch or conditional firm options also must be made available as a reassessment product, i.e., subject to certain limitations. Although many transmission providers argue that planning redispatch and conditional firm service should be offered only to customers who seek to upgrade the grid, we disagree. We find that there are legitimate circumstances under which customers may not choose to support system upgrades – either because the costs of construction are too high or because the term of service (e.g., less than five years) does not merit the construction of additional facilities. We will therefore make planning redispatch and conditional firm service available to such customers, but subject to certain limitations to reflect the nature of the services. Specifically, we must select a limitation on the term for the conditions that permit interruption or redispatch, given that, for these customers, the term is not circumscribed by the period during which upgrades are constructed. We adopt two years as the appropriate time period to allow the transmission provider to reassess the conditions under which planning redispatch or conditional firm service is provided. The transmission provider will retain the right to reassess the planning redispatch and conditional firm option after the first two years of service, and every two years thereafter. The transmission provider shall reassess (1) the redispatch required to keep the service firm or (2) the conditions or hours under which the transmission provider may conditionally curtail the service. The customer will receive service for the requested term unless the transmission provider determines through its biennial reassessment that the firm point-to-point service can no longer be reliably provided. The customer may also choose to terminate the service at the time of reassessment if the service no longer meets it needs.

1043. The Commission adopts the conditional firm option as a modified form of long term firm point-to-point service that includes less-than-firm service in a defined number of hours of the year or during defined system conditions when firm point-to-point service is not available. The service can be curtailed solely for reliability reasons during the defined system conditions or defined number of hours. We reject EEI’s suggestion to use a monthly non-firm curtailment because it would allow for curtailment of the conditional service for economic reasons.

1046. Further, as discussed in more detail below, we disagree that NERC must modify its processes in order to allow transmission providers to implement this product. However, we will allow for a sufficient period of time for development of business practices and tracking mechanisms to implement the product. We recognize that there may be some regional variation in the way transmission providers approach the provision of conditional firm service beyond the minimum attributes that we establish in this Final Rule. Thus, we do not direct that transmission providers work with NAESB to develop business practices for implementation of the conditional firm service. Rather, we direct transmission providers located in the same region to coordinate such development among themselves. We also encourage participation of non-public utility transmission providers in the region and interested transmission customers in the development of these business practices. Public utility transmission providers should make efforts to include these interested parties in their regional coordination efforts. We direct transmission providers to implement these mechanisms and business practices within 180 days after the publication of this Final Rule in the Federal Register.

1048. Finally, we clarify for Bonneville that customers seeking the conditional firm option must first request long-term firm service. When ATC is unavailable, the transmission provider must study the conditional firm option at the customer’s request. There is no separate queue for the conditional firm option.

1064. The Commission requires that, when conducting the system impact study for the conditional firm option, the transmission provider shall identify: (1) the specific system condition(s) when conditional curtailment may apply; and (2) the annual number of hours when conditional curtailment may apply. A customer must select either conditions or hours for incorporation into its conditional firm service agreement.

1066. We will require specificity of system conditions. Acceptable system conditions include, but are not limited to, designation of limiting transmission elements, such as a transmission line, substation or flowgate. We do not believe, however, that designation of system load levels, standing alone, would qualify as an acceptable system condition. Rather, load levels would have to be linked to a specific constraint or transmission element that is associated with the request for service, e.g., load levels in a constrained load pocket. Otherwise, the system load level would not be specific to the part of the system over which service is requested and, hence, have no necessary relation to the problems, if any, created by the service being requested. Furthermore, because most system loads experience load growth every year, conditional curtailments would necessarily increase over a multi-year conditional firm service term.

1067. We recognize that modeling of the conditional curtailment hours entails difficulties beyond those encountered in modeling ATC. To address these difficulties we are allowing flexibility in determining the number of hours. We clarify that we will not require a standardized method of modeling the conditional curtailment hours. We also note that the Commission’s examination of modeling methods in the NOPR was not meant to propose one method over another; rather, it was meant to examine possible ways to determine a number of conditional curtailment hours to encourage dialog on the issue. Additionally, we will allow transmission providers to add a risk factor to their calculation of annual curtailment hours to account for forecasting risks. Further, we note that our adoption of the conditional bridge and reassessment products, detailed above, address modeling difficulties by limiting the number of years that a transmission provider must model in determining both the number of hours and future system conditions. Moreover, we clarify that if the customer selects the annual hourly cap option, the transmission provider has the flexibility to conditionally curtail the customer for any reliability reason during those hours, including but not limited to, the system condition(s) identified in the system impact study. Without this flexibility the hourly cap option and the specific system condition option would be indistinguishable with a cap on the number of hours that the system conditions interruption could occur.

1068. We will require annual caps on the number of hours because calculating an annual cap entails less risk for the transmission provider and its existing firm customers than monthly or seasonal caps. While we will not require monthly or seasonal caps, we encourage transmission providers to offer them if they can overcome modeling barriers because monthly or seasonal caps give more certainty to customers about the particular aspects of their service. Though we allow for flexibility in modeling and determining the number of conditional curtailment hours for a particular service request, we believe that this will have a minimal impact on conditional firm customers. Transmission providers will be allowed to curtail only for reliability purposes and conditional firm customers during conditional curtailment hours will be curtailed only after all point-to-point nonfirm customers have been curtailed.

The following additional Commission determinations were issued in FERC Order 890-A under Docket Nos. RM05-17-001, 002 and RM05-25-001, 002 as published in the Federal Register on January 17, 2007 (footnotes ommitted):

552. The Commission affirms the decision in Order No. 890 to create a new conditional firm option in the pro forma OATT for customers seeking and denied long-term firm point-to-point transmission service. We reiterate that, like the planning redispatch option, transmission providers are not required to provide conditional firm service if doing so would impair system reliability. Concerns regarding system reliability have thus already been addressed in the design of the conditional firm option.

558. We affirm the decision in Order No. 890 not to create a conditional firm network service. Network customers may designate network resources any time firm transmission is available, and the term of the designation can include periods of less than a year. Network customers can also use secondary network service to access resources during times when firm service is not available. This flexibility to use designated network resources and secondary network service to access undesignated resources already provides a service that is like conditional firm service that can be used to integrate new resources, intermittent or otherwise.

561. We agree with EEI’s requested change to provide consistency between the pro forma OATT and the preamble of Order No. 890. As the Commission stated repeatedly in Order No. 890, transmission providers are obligated to provide conditional firm options only to customers requesting long-term firm point-to-point service. We amend section 15.4(c) of the pro forma OATT accordingly. We also revise sections 19.1 and 19.3 of the pro forma OATT to make clear that the conditional firm option is available to eligible customers, not just existing transmission customers, as provided in Order No. 890.

581. The Commission affirms the decision in Order No. 890 to require transmission providers to provide planning redispatch and conditional firm service subject to a biennial reassessment when transmission customers are unwilling to pay for transmission upgrades. We decline to adopt a longer reassessment period or altogether eliminate the reassessment feature of these services. There are legitimate circumstances under which a customer may choose not to support system upgrades, including high construction costs or a short term of service that does not merit construction. Balanced against these customers’ needs are the needs of transmission providers to reliably provide service and of other customers to continue using their own firm transmission rights. Adopting a two year reassessment period appropriately balances these various interests.

582. The Commission did not, as AWEA suggests, limit the term of the reassessment service. A customer taking planning redispatch or conditional firm service subject to reassessment could receive an unlimited term of service, with the transmission provider reassessing every two years the redispatch required to keep the service firm or the conditions or hours under which the transmission provider may conditionally curtail the service.

585. We also agree with MidAmerican that a transmission provider’s waiver of a reassessment for conditional firm or planning redispatch service does not constitute a waiver of all reassessments for the duration of the service, unless explicitly agreed to by the transmission provider. We reiterate, however, that only one reassessment may be performed in each two-year period of service. We also affirm that any waiver must be granted for similarly situated service, which would include conditional firm or planning redispatch service that is limited because of the same constraints or general system limitations. Such a waiver would be an act of discretion that must be posted on OASIS. Waiver of the reassessment presents an opportunity for discrimination among classes of customers on the part of the transmission provider and posting will provide eligible customers with an indicator of how often conditions or redispatch requirements have been reassessed. Transmission providers are directed to develop uniform OASIS posting standards, in coordination with NAESB, for transmission providers to post information regarding waivers of the biennial reassessment for planning redispatch and conditional firm service.

587. With regard to the conditional firm option, we continue to require that transmission providers study and offer service based on both system conditions and annual curtailment hours. The Commission introduced the concept of conditional curtailment based on system conditions in its request for supplemental comments issued on November 15, 2006. MidAmerican and other industry participants were therefore provided adequate notice and opportunity to comment on the potential for the Commission to expand the scope of the required offerings for conditional firm service. Upon review of these comments, the Commission allowed transmission providers to determine system conditions and conditional curtailment hours through different means, implicitly recognizing that system conditions are not exactly interchangeable with conditional curtailment hours. Modeling of conditional curtailment hours entails difficulties beyond those encountered in modeling ATC. Transmission providers have therefore been granted flexibility in making these determinations and are allowed to use an additional risk factor in calculating conditional hours. In light of the flexibility provided to transmission providers, we reject as unsupported petitioners’ requests to eliminate or limit the requirement to offer conditional firm service based on the number of hours in which service may be conditional.

590. We clarify in response to Constellation and EPSA that, when a transmission provider is evaluating its continued ability to provide conditional firm service during a biennial reassessment, the transmission provider is not limited to the specific conditions previously agreed to by the transmission customer in the initial service agreement or a prior reassessment. The purpose of the biennial reassessment is to allow the transmission provider to adjust the conditions or number of hours during which conditional firm service will be conditional in order to ensure that continued provision of the service does not impair reliability. Thus, the Commission does not impose upon the transmission provider the obligation to plan its system to keep firm the part of the conditional firm service that is firm when service was initiated. Although this may increase (or decrease) the number of hours in which service is conditional, the transmission provider may not entirely terminate service to the conditional firm customer.

591. We affirm our decision to assign conditional firm service the same curtailment priority as secondary network service for periods when the service is conditional. EEI’s argument that customers use secondary network service to meet the reliability needs of their loads is inapposite. Secondary network service is a non-firm service for which requests are made in the same time-frame as other non-firm service. While the Commission recognized that network customers may use secondary network service on an “as available” basis to meet peak native load, and in this way meet the reliability needs of loads, this is not the purpose of secondary network service. Network customers that rely upon secondary network service to meet their peak native load are already lessening the reliability of their service by taking non-firm service. The fact that conditional firm service will compete with secondary network service when curtailments are ordered is irrelevant.

592. We agree with petitioners that the NAESB rules regarding tagging do not allow a transmission provider to change the tag of a transmission customer. That is why, in Order No. 890, the Commission directed transmission providers to coordinate with other transmission providers in their regions to develop their own business practices to implement the tagging and tracking of conditional firm service. Upon consideration of petitioners’ concerns, we grant rehearing to require transmission providers, in coordination with NERC and NAESB, to develop within 180 days of publication of this order in the Federal Register a consistent set of tracking capabilities and business practices for tagging for implementation of conditional firm service. We agree with petitioners that a consistent set of practices followed by the industry will reduce transmission provider discretion and bring uniformity in implementing conditional firm service. In the interim, the existing business practices of each transmission provider for tracking and tagging conditional firm service shall remain in effect.

The following additional Commission determinations were issued in FERC Order 890-B under Docket Nos. RM05-17-003 and RM05-25-003 issued June 23, 2008:

133. With regard to our determination that unexpected events should not be incorporated into the analysis to determine the number of annual curtailment hours applying in any transmission service agreement, we clarify that whether such events impact the accounting for annual curtailment hours depends on the curtailment priority of the service at the time of the event. If an unexpected event occurs when the conditional firm customer is curtailed pursuant to a firm curtailment priority, then the curtailment will not count against the annual hours. In determining whether the annual conditional curtailments are met, transmission providers should count curtailments made when the service is otherwise conditional, i.e., tagged with a secondary network curtailment priority, regardless of whether the curtailment occurred during an unexpected event.

172. In Order No. 890-A, the Commission affirmed the decision to allow off-system resources supported by conditional firm point-to-point service to be designated as network resources. The Commission declined to require a network customer with a designated off-system resource supported by conditional firm service to obtain reserves or backup resources to cover the periods when the resource supported with conditional firm point-to-point transmission service might not be delivered. The Commission explained that, in the event conditional firm service is curtailed, the network customer would be required to serve its network load from other resources, just as when the transmission provider curtails the network customer’s use of secondary network service. The Commission reiterated that it is not the responsibility of the transmission provider to ensure that the network customer has sufficient resources to meet its load.

The following is an excerpt from the Order 890-B Pro Forma Tariff Section 15.4:

(c) If the Transmission Provider determines that it cannot accommodate a Completed Application for Long-Term Firm Point-To-Point Transmission Service because of insufficient

capability on its Transmission System, the Transmission Provider will offer the Firm Transmission Service with the condition that the Transmission Provider may curtail the service prior to the curtailment of other Firm Transmission Service for a specified number of hours per year or during System Condition(s). If the Transmission Customer accepts the service, the Transmission Provider will use due diligence to provide the service until (i) Network Upgrades are completed for the Transmission Customer, (ii) the Transmission Provider determines through a biennial reassessment that it can no longer reliably provide such service, or (iii) the Transmission Customer terminates the service because the reassessment increased the number of hours per year of conditional curtailment or changed the System Conditions.

2008 WEQ Annual Plan Item 2.a.i.2

Conditional Firm Business Practice Standards associated with S&CP Requirements completed in 2(a)(i)(1).

2008 WEQ Annual Plan Item 2.a.vii.1

Tagging for Conditional Firm Service.

b.  Description of Recommendation:

c.  Business Purpose:

Implementation of FERC Orders 890, 890-A, and 890-B.

- Commentary/Rationale of Subcommittee(s)/Task Force(s):

Please review the following ESS/ITS Subcommittee meeting minutes:

- April 3-4, 2008

- April 15, 2008 (unavailable- DC)

- April 29-30, 2008 (unavailable- DC)

- May 7, 2008 (unavailable- DC)

- May 21-22, 2008 (unavailable- DC)

- May 27-28, 2008 (unavailable- DC)

- June 10-11, 2008 (unavailable- DC)

- June 23-24, 2008 (unavaiable- DC)

Please review the Motions Document:

- CCO Motions Document