Dear Wholesale Electric Quadrant Members and Interested Industry Participants –

For the attached recommendation for standards that are linked to NERC standards development for FERC Order No. 890, our subcommittees have accepted these proposed standards and are asking you to comment on them in a formal comment period.  They are also asking the WEQ Executive Committee to consider this recommendation and comments that are submitted during the formal comment period, and vote on this recommendation.

The subcommittees understand that the proposed standards include references to NERC reliability standards that have not yet been approved through the NERC process, but are based on their current work products.  The proposed standards include placeholders for the NERC references, which will be added once they are approved by NERC.  The subcommittees do not expect that this recommendation will require significant rework by the subcommittees once NERC adopts its related reliability standards.  The subcommittees’ chairs will review the final NERC standards once adopted and will identify if any changes are needed to recommendations that have already been processed through commenting and EC consideration.  If changes are needed, a determination will be made whether the changes can be processed as minor actions, or for more substantive changes, the standards modification process will be used.

We are taking these steps and offering this recommendation for your comment and for EC consideration for vote rather than waiting until NERC completes its full process because:

- NERC has developed draft standards for the referenced items which have been processed through at least one ballotting period.

- The NAESB subcommittees do not expect the changes to be made by NERC in its current efforts to yield substantive changes to the NAESB related standards.

- The progress being made by the subcommittees now on Order 890 towards meeting the August deadline will be adversely affected if the all NAESB Order No. 890 recommendations with NERC references are held in abeyance until NERC has concluded its efforts.

- The interested industry participants and the WEQ EC will have the opportunity to review the  NAESB Order No. 890 recommendations with NERC references for formal comment and consideration as NAESB completes its efforts on each recommendation.  In this manner, the industry and the EC are asked to consider proposed standards within a reasonable workflow.  The alternative is to hold all NAESB Order No. 890 recommendations with NERC references and then submit them all for industry comment and EC consideration which would provide a significant amout of documents for review, comment and consideration.

- Last but certainly not least, submitting the recommendations now to the industry and to the WEQ EC will provide the necessary signals from the industry through formal comments and through EC actions.  If corrective action is needed, NAESB would have the opportunity to meet or come closer to the deadlines set by the FERC.

Please note that the subcommittees have worked diligiently on this recommendation along with the work products available from NERC and that all steps outlined conform with NAESB operating procedures.  We appreciate your consideration and your comments.

With Best Regards,

Marcie Otondo, Co-Chair, NAESB Electronic Scheduling Subcommittee/Information Technology Subcommittee

Ed Skiba, Co-Chair, NAESB Business Practices Subcommittee

Paul Sorenson, Co-Chair, NAESB Electronic Scheduling Subcommittee/Information Technology Subcommittee

J.T. Wood, Co-Chair, NAESB Business Practices Subcommittee and Co-Chair,  NAESB Electronic Scheduling Subcommittee/Information Technology Subcommittee

1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

Accept as requested				 X   Change to Existing Practice

X Accept as modified below			      Status Quo

Decline

2.  TYPE OF DEVELOPMENT/MAINTENANCE

Per Request:					Per Recommendation:

Initiation					      Initiation

X Modification					  X  Modification

Interpretation				      Interpretation

Withdrawal					      Withdrawal

Principle 					      Principle

Definition 					      Definition

Business Practice Standard 			      Business Practice Standard

Document 					      Document

Data Element 				      Data Element

Code Value 					      Code Value

X12 Implementation Guide			      X12 Implementation Guide

Business Process Documentation		      Business Process Documentation

3.  RECOMMENDATION

SUMMARY:

The recommendation is intended to address the NAESB Existing Transmission Commitments requirements in FERC Order 890 under Docket numbers RM05-17 and RM05-25 in paragraphs 243, 244, and 246 and in FERC Order 890-A under Docket number RM05-17 and RM05-25 in paragraphs 63 and 151.

This recommendation addresses the WEQ 2008 Annual Plan Items:

2.a.iv.4	Posting of Existing Transmission Commitments (ETC)

2.a.vi.4	Posting of Existing Transmission Commitments (ETC)

2.b.ii.1	Develop the Business Practice Standards complementary to NERC Reliability Standards for Existing Transmission Commitments (ETC) to create a “consistent approach for determining the amount of transfer capability a transmission provider may set aside for its native load and other committed uses”, including the elements of ETC for full implementation of the NERC MOD-001 reliability standard.

The business practices developed to address this item will require coordination with the NERC Order 890 reliability standards development.

The Joint ESS/ITS and BPS Subcommittee reviewed the NERC MOD Standards MOD-028 (Area Interchange Methodology), MOD-029 (Rated System Path Methodology) and MOD-030 (Flowgate Methodology) and determined that scope of this recommendation would be limited to those ETC components, which they agreed were not addressed adequately in the existing standards or included in the scope of work assigned to the ESS/ITS Subcommittee.  As a result of this decision the scope of the recommendation has been limited to the Grandfathered Agreement and Rollover Rights components of the ETC. There will not be a separate section for ETC.

Recommended Standards:

- Modifications to WEQ-001 (New Requirements)

- Business Practice Standards for Posting the ETC Components

- NOTE: NAESB Staff will assign appropriate enumeration to this standard to replace “x” and “y” with next sequential standard number within WEQ-001

- 001-x	Grandfathered Agreements

The Transmission Provider shall identify in the ATCID the treatment of grandfathered agreements in the ATC/AFC calculations.

- 001-x.1	The Transmission Provider shall post on OASIS the aggregate MW value for the grandfathered agreements component of ETC associated with the ATC value posted on OASIS. The aggregate MW value shall be posted such that it can be viewed and queried using the systemdata template (see WEQ-013.3.3.1).

- 001-x.1.1	If the Transmission Provider has already included the MW value of a grandfathered agreement in another ETC component, the Transmission Provider should not include such value in the aggregate MW value for grandfathered agreements.

- 001-x.1.2	If the Transmission Provider is using the Flowgate Methodology, the Transmission Provider shall not be required to post an aggregate MW value that can be viewed and queried using the systemdata template.  The Transmission Provider shall post a list of Grandfathered Agreements with MW values that are expected to be scheduled or expected to flow.  This posting shall be included via a link titled “Flowgate Methodology Grandfathered Agreements” under the ATC Information Link.

001-y	Rollover Rights

001-y.1	The Transmission Provider upon approving a Long Term Firm Point-to-Point request with rollover rights, shall post on OASIS the information relevant to the rollover rights. Such information shall be posted such that it can be viewed and queried using the transtatus and rollover templates (see WEQ-002 and WEQ-013).

001-y.2	Upon confirmation of a renewal Long Term Firm Point-to-Point request by a Transmission Customer who exercises rollover rights, the Transmission Provider shall reduce the rollover capacity in the parent reservation’s rollover template by the capacity granted of the renewal reservation.

001-y.3	Once the deadline for the Transmission Customer to submit a renewal request has passed for a Long Term Firm Point-to-Point reservation, the Transmission Provider shall set the rollover capacity to zero.

- Modifications to WEQ-001 (Changed Requirements Denoted in Red-line)

      - ATC INFORMATION LINK

If the Transmission Provider does not use CBM or TRM in their assessment of ATC, that information shall  be found in the CBM Implementation Document – CBMID or TRM Implementation Document – TRMID link below.

The information posted at “ATC Information” should include the following links:

- Available Transfer Capability Implementation Document - ATCID (as specified in NERC [MOD-001-?])

- CBM Implementation Document - CBMID (as specified in NERC [MOD-004-?])

- TRM Implementation Document - TRMID (as specified in NERC [MOD-008-?])

    - ATC Methodology Contact (as required by WEQ-001-xx)

    - Load Forecast Descriptive Statement (as specified in WEQ-001-n.6.5 [“n” represents 2008 WEQ Annual Plan Item 2.b.vii])

    - Flowgate Methodology Grandfathered Agreements (only required for Transmission Providers using the Flowgate Methodology)

The posting of this information would be subject to the Transmission Provider’s ability to redact certain provisions due to market, security or reliability sensitivity concerns. Any section that has been redacted shall retain the heading and will include the reason for the redaction. Appropriate reasons for redaction are:  “Market Sensitive Information”, “CEII Information”, “Security Sensitive Information” or “Reliability Sensitive Information”.

These items shall appear in the order specified above and before any other items which may be required as per specific FERC direction or local business practice related to ATC Information. Posting of the cites noted in the parentheses is optional. Access to some of the information found under the ATC Information Link above may require the user to register with the individual OASIS sites according to WEQ-002-3.1.

Modifications to WEQ-003 (Changed Requirements Denoted in Red-line)

WEQ-003 Data Dictionary Changes

| 003-0 OASIS DATA DICTIONARY, Version 1.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Data Dictionary Element Name | Alias | Field Format : minimum characters {type of ASCII} maximum characters | Restricted Values | Definition of Data Element |
| SYSTEM_ATTRIBUTE | SYSATTR | 0{ALPHANUMERIC}20 | Valid Values: CBM FTRM NFTRM TTC FATC NFATC FGF NFGF ATC_ANNOTATION ZONE_FORECASTED_LOAD SYSTEM_FORECASTED_LOAD NATIVE_FORECASTED_LOAD ZONE_ACTUAL_LOAD SYSTEM_ACTUAL_LOAD NATIVE_ACTUAL_LOAD {Registered} | Type of system data viewed by SYSTEMDATA Template: CBM – Capacity Benefit Margin FTRM – Transmission Reliability Margin for use in FATC NFTRM – Transmission Reliability Margin for use in NFATC TTC – Total Transmission Capability FATC –Firm Available Transmission Capability NFATC – Non-firm Available Transmission Capability ATC_ANNOTATION – Annotation for a change in monthly or yearly posted ATC. FGF – Firm Grandfathered Firm Transmission Service NFGF – Non-Firm Grandfathered Non-Firm Transmission Service ZONE_FORECASTED_LOAD – Anticipated Forecasted Daily Zonal Load SYSTEM_FORECASTED_LOAD – Anticipated Forecasted Daily System-Wide Peak Load NATIVE_FORECASTED_LOAD – Anticipated portion of the Forecasted Daily System-Wide Peak Load for the native load ZONE_ACTUAL_LOAD – Actual Daily Zonal Load SYSTEM_ACTUAL_LOAD – Actual Daily System-Wide Peak Load NATIVE_ACTUAL_LOAD – Actual portion of Daily System-Wide Peak Load for the native load {registered} – Provider specific registered name for the data posted |

Modifications to WEQ-013 (Changed Requirements Denoted in Red-line)

013-3.3.1	ATC Related Query/Response Requirements

Transmission Providers shall provide applicable data associated with the following SYSTEM_ATTRIBUTE values:

- TTC – Total Transfer Capability

- CBM – Capacity Benefit Margin

- FTRM –  Transmission Reliability Margin for use in FATC

- NFTRM –  Transmission Reliability Margin for use in NFATC

- FATC –Firm Available Transfer Capability

- NFATC –Non-Firm Available Transfer Capability

- FGF – Firm Grandfathered Transmission Service

- NFGF – Non-Firm Grandfathered Transmission Service

- ZERO_FATC_NARRATIVE – Narrative when a posted firm monthly or yearly ATC value remains unchanged at a value of zero consistent with WEQ-001-Y

- ZERO_NFATC_NARRATIVE – Narrative when a posted non-firm monthly or yearly ATC value remains unchanged at a value of zero consistent with WEQ-001-Y

- FATC_CHANGE_NARRATIVE – Narrative when a posted firm monthly or yearly ATC changes as a result of a 10 percent change in TTC/TFC consistent with WEQ-001-Z

- NFATC_CHANGE_NARRATIVE – Narrative when a posted non-firm monthly or yearly ATC changes as a result of a 10 percent change in TTC/TFC consistent with WEQ-001-Z

- ATC_ANNOTATION – Annotation for a change in monthly or yearly posted ATC (no longer used replaced by ZERO_FATC_NARRATIVE, ZERO_NFATC_NARRATIVE, FATC_CHANGE_NARRATIVE, NFATC_CHANGE_NARRATIVE)

- {registered} – Provider defined attributes necessary for posting required data

The SYSTEM_ELEMENT_TYPE and SYSTEM_ELEMENT shall be used as follows:

- SYSTEM_ELEMENT_TYPE = PATH

- SYSTEM_ELEMENT = Name of the path associated with the posted ATC component value

Posting of ATC component data and narratives via the systemdata template shall comply with all applicable regulations and business practices.

The following are the systemdata template Data Element requirements for providing ATC component data, i.e., TTC, CBM, FTRM, NFTRM, FATC, NFATC, FGF, or NFGF to the user:

| Data Element | Restriction/Requirement |
| --- | --- |
| POSTING_REF | The unique OASIS identifier assigned to the posting |
| SYSTEM_ELEMENT_TYPE | “PATH” |
| SYSTEM_ELEMENT | Name of the path associated with the posted ATC component value |
| SYSTEM_ATTRIBUTE | “TTC”, “CBM”, “FTRM”, “NFTRM”, “FATC”, “NFATC,” “FGF”, or “NFGF” |
| START_TIME | Beginning of interval associated with the posted value |
| STOP_TIME | End of interval associated with the posted value |
| ATTRIBUTE_VALUE | Value of the posted ATC component over the interval of START_TIME/STOP_TIME |
| ATTRIBUTE_UNITS | Units of ATTRIBUTE_VALUE; typically MW |
| ANNOTATION | Optional at discretion of the Provider; typically null |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

The following are the systemdata template Data Element requirements for providing a brief, but specific, narrative explanation of the reason for a change in the monthly or yearly firm or non-firm ATC value on a constrained Posted Path when the monthly or yearly ATC value changes as a result of a 10 percent or greater change in the related posted TTC or TFC.

| Data Element | Restriction/Requirement |
| --- | --- |
| POSTING_REF | The unique OASIS identifier assigned to the posting |
| SYSTEM_ELEMENT_TYPE | “PATH” |
| SYSTEM_ELEMENT | Name of the path associated with the posted ATC component value |
| SYSTEM_ATTRIBUTE | ”FATC_CHANGE_NARRATIVE” or “NFATC_CHANGE_NARRATIVE” |
| START_TIME | Date and time that the Transmission Provider’s most limiting TTC path’s TTC or TFC flowgate’s TFC changed consistent with the criteria specified in Z.1.1 |
| STOP_TIME | Date and time of the end of the reason for the change in the Transmission Provider’s most limiting TTC path’s TTC or TFC flowgate’s TFC, consistent with the criteria specified in Z.1.1, if a reasonable estimate is known at the time of the posting. . If the date and time for the end of the reason for the change is (1) unknown or is indeterminate, (2) beyond the OASIS posting horizon, or (3) the change is permanent then a null value shall be returned for STOP_TIME. |
| ATTRIBUTE_VALUE | The maximum change in TTC/TFC over the start /stop time interval rounded to the nearest percent (e.g., an increase of 13.7% in TTC would be posted as +14% and a decrease of 13.7% in TTC would be posted as -14%) |
| ATTRIBUTE_UNITS | % |
| ANNOTATION | Narrative text fields shall be delimited by semicolons “;” (see standard Z.2.3) |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

The following are the systemdata template Data Element requirements for providing a brief, but specific, narrative associated with yearly or monthly ATC values when either the posted firm or non-firm ATC remains unchanged at a value of zero (0) for six (6) months or longer.

| Data Element | Restriction/Requirement |
| --- | --- |
| POSTING_REF | The unique OASIS identifier assigned to the posting |
| SYSTEM_ELEMENT_TYPE | “PATH” |
| SYSTEM_ELEMENT | Name of the path associated with the posted ATC component value |
| SYSTEM_ATTRIBUTE | “ZERO_FATC_NARRATIVE” or “ZERO_NFATC_NARRATIVE” |
| START_TIME | Date and time that the posted ATC initially became zero, which may be prior to the date of the zero ATC narrative posting |
| STOP_TIME | Date and time when the posted ATC is expected to be greater than zero. If such date and time is unknown, then a null value shall be returned for STOP_TIME. |
| ATTRIBUTE_VALUE | 0 |
| ATTRIBUTE_UNITS | “MW” |
| ANNOTATION | Narrative text fields shall be delimited by semicolons “;”(see standard Y.2.3) |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

4.  SUPPORTING DOCUMENTATION

a.  Description of Request:

In the FERC Order 890 under Docket Nos. RM05-17-000 and RM05-25-000 dated February 16, 2007, the Commission made the following conclusions about this recommendation:

- Paragraph 243 - To achieve greater consistency in ETC calculations and further reduce the potential for undue discrimination, the Commission adopts the NOPR proposal and directs public utilities, working through NERC and NAESB, to develop a consistent approach for determining the amount of transfer capability a transmission provider may set aside for its native load and other committed uses.  We expect that NERC will address ETC through the MOD-001 reliability standard rather than through a separate reliability standard.  By using MOD-001, the ETC calculation can be adjusted to be applicable to each of the three ATC methodologies under development by NERC.

Paragraph 244 - In order to provide specific direction to public utilities and NERC, we determine that ETC should be defined to include committed uses of the transmission system, including (1) native load commitments (including network service), (2) grandfathered transmission rights, (3) appropriate point-to-point reservations, (4) rollover rights associated with long-term firm service, and (5) other uses identified through the NERC process.  ETC should not be used to set aside transfer capability for any type of planning or contingency reserve, which are to be addressed through CBM and TRM.  In addition, in the short-term ATC calculation, all reserved but unused transfer capability (non-scheduled) shall be released as non-firm ATC.

Paragraph 246 - We agree with NERC that some elements of ETC are candidates for business practices rather than reliability standards.  Accordingly, we direct public utilities, working through NAESB, to develop business practices necessary for full implementation of the developed MOD-001 reliability standard.

- In the FERC Order 890-A under Docket Nos. RM05-17-001, 002 and RM05-25-001, 002 dated December 28, 2007, the Commission made the following conclusions about this recommendation:

Paragraph 63 - The Commission also found that inclusion of all requests for transmission service in ETC would likely overstate usage of the system and understate ATC.  The Commission therefore found that reservations that have the same point of receipt (POR) (generator) but different point of delivery (POD) (load), for the same time frame, should not be modeled in the ETC calculation simultaneously if their combined reserved transmission capacity exceeds the generator’s nameplate capacity at the POR.  The Commission directed public utilities, working through NERC, to develop requirements in MOD-001 that lay out clear instructions on how these reservations should be modeled.  The Commission also concluded that some elements of ETC are candidates for business practices instead of reliability standards and directed public utilities, working through NAESB, to develop business practices necessary for full implementation of the MOD-001 reliability standard.

Paragraph 151 - We decline to impose additional posting requirements regarding ETC uses, as requested by EPSA and Powerex.  In Order No. 890, the Commission required transmission providers to make available all data used to calculate ATC for constrained paths and any system planning studies or specific network impact studies performed for customers.  This would include information regarding ETC uses, including grandfathered agreements, that affect ATC calculations or study results.  EPSA and Powerex fail to demonstrate that it is necessary to require the posting of additional information regarding ETC uses to verify the accuracy of the transmission provider’s ATC calculations.  We note in response to Powerex that, if any new service taken upon expiration of a pre-Order No. 888 contract, the terms and conditions of the transmission provider’s OATT would apply.

This recommendation addresses the WEQ 2008 Annual Plan Items:

2.a.iv.4	Posting of Existing Transmission Commitments (ETC)

2.a.vi.4	Posting of Existing Transmission Commitments (ETC)

2.b.ii.1	Develop the Business Practice Standards complementary to NERC Reliability Standards for Existing Transmission Commitments (ETC) to create a “consistent approach for determining the amount of transfer capability a transmission provider may set aside for its native load and other committed uses”, including the elements of ETC for full implementation of the NERC MOD-001 reliability standard.

The business practices developed to address this item will require to coordination with the NERC Order 890 reliability standards development.

b.  Description of Recommendation:

c.  Business Purpose:

Implementation of FERC Orders 890 and 890-A.

- Commentary/Rationale of Subcommittee(s)/Task Force(s):

Discussion of Scope

During the Joint ESS/ITS and BPS meetings held in Washington D.C. on May 14-16, 2008, the Joint Subcommittee reviewed the ETC Background document.  The scope of the Annual Plan Item 2.b.ii.1 “… including the elements of ETC for full implementation of the NERC MOD-001 reliability standard” was interpreted to include addressing the ETC components documented in MOD Standards MOD-028 (Area Interchange Methodology), MOD-029 (Rated System Path Methodology) and MOD-030 (Flowgate Methodology).  The Joint Subcommittee reviewed each component and made the following determination:

- Network Integration Transmission Service (NITS) – The joint subcommittee decided not to address this component since NITS is currently within the scope of the ESS/ITS subcommittee and the ESS/ITS has already begun working on this area.

- Grandfathered Agreements – The joint subcommittee determined this ETC component should be within scope and has included standards in the recommendation.

- Rollover Rights – The joint subcommittee limited its scope for this ETC component to standards within WEQ-001 (Business Practices for Open Access Same-Time Information Systems (OASIS)).  The joint subcommittee determined that it would not address WEQ-002 Business Practices for Open Access Same-Time Information Systems (OASIS) Standards and Communication Protocols), WEQ-003 Business Practices for Open Access Same-Time Information Systems (OASIS) Data Dictionary), and WEQ-013 Business Practices for Open Access Same-Time Information Systems (OASIS) Implementation Guide) since they are under the control of the ESS/ITS Subcommittee.  The ESS/ITS has submitted recommendations that include rollover rights for these business practices, but have left the WEQ-001 changes to the Joint ESS/ITS and BPS Subcommittee.

- Point-to-Point Service – The Joint ESS/ITS and BPS determined that the requirements for point-to-point service have been documented sufficiently in the NAESB business practices such that no new business practices are required for the point-to-point service component of ETC.

- Other Services – The Joint ESS/ITS and BPS had considerable discussion on whether or not NAESB should develop requirements for documenting the Other Services component of the ETC in the ATCID, beyond what is included in the NERC MOD standards.  The Joint ESS/ITS and BPS approved a motion on June 17, 2008 using a balanced vote which indicated they felt the NERC requirements appear to document sufficiently the Other Services description for inclusion in the ATCID.

At the June 13, 2008, Joint ESS/ITS and BPS Subcommittee meeting there was considerable discussion as to whether a separate ETC section should be created in WEQ-001 documenting references for all ETC components including those that are already addressed by the existing business pratices for point-to-point.  A motion was approved using a “balanced vote” that stated:

The joint subcommittee will enhance the summary and justification sections of the recommendation to explain why we are not documenting the point-to-point and NITS components in the recommendation and remove the heading for the ETC section with the grandfathered,  rollover, and potentially the other services being stand-alone sections.

As a result of this motion, those ETC components for which new standards are being developed under this recommendation will be documented in separate sections in WEQ-001, such as Grandfathered Agreements and Rollover Rights, rather than creating a specific ETC section in WEQ-001.

Supporting Documentation:

Please review the following Joint BPS and ESS/ITS ATC/AFC Subcommittee meeting minutes:

- May 14 -16, 2008 (unavailable- DC)

- May 29, 2008 (unavailable- DC)

- June 12 -13, 2008 (unavailable- DC)

- June 17-18, 2008 (unavailable)- DC)

Please review the Motions Document:

- ETC Motions Document

Please review the other interim work products developed by the Joint BPS and ESS/ITS  Subcommittee including the:

- ETC Background Document

- ETC Concepts

- ETC Grandfathered Component

- ETC Rollover Rights Component

- ETC Other Services Component

The purpose of MOD-001 is to promote the consistent and uniform application of transfer capability calculations among the transmission system users. (NOTE: this is footnote 169)

By “appropriate,” we mean that reservations accounted for under ETC depend on the firmness and duration of the reservation.  The specific characteristics should be developed in the reliability standard. (Note: this is footnote 170)

TRM also includes such things as loop flow and parallel path flow. (Note: this is footnote 171)

See id. at P 348. (NOTE: this is footnote 64)

See Order No. 888 at 31,655. (NOTE: this is footnote 65)