1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

X  Accept as requested			      Change to Existing Practice

Accept as modified below			      Status Quo

Decline

2.  TYPE OF DEVELOPMENT/MAINTENANCE

Per Request:					Per Recommendation:

X  Initiation					   X  Initiation

Modification					  _   Modification

Interpretation				      Interpretation

Withdrawal					      Withdrawal

Principle 					      Principle

Definition 					      Definition

Business Practice Standard 			  X  Business Practice Standard

Document 					      Document

Data Element 				      Data Element

Code Value 					      Code Value

X12 Implementation Guide			     _X12 Implementation Guide

Business Process Documentation		      Business Process Documentation

3.  RECOMMENDATION

SUMMARY:

In FERC Order 890 under Docket numbers RM05-17 and RM05-25, the Commission made conclusions regarding posting load forecast and actual peak load in paragraphs 413 and 416.  In FERC Order 890-A under Docket number RM05-17 and RM05-25 the Commission expanded on their requirements for posting load forecast under paragraph 43.  The following recommended Standards address these determinations.

- Recommended Standards:

- Modifications to WEQ-001 (New Requirements)

- Business Practice Standards for Posting the Forecast and Actual Loads

- NOTE: NAESB Staff will assign appropriate enumeration to this standard to replace “n” with next sequential standard number within WEQ-001

- 001-n	ACTUAL and FORECASTED LOAD

The Transmission Provider or applicable RTO/ISO shall post on OASIS the actual daily peak load , as well as  the final forecasted system–wide load, including forecasted native-load, Control Area, or individual Load Serving Entity (LSE) load, as applicable according to the following standards. If the Transmission Provider is a member of an approved Independent System Operator (ISO) or Regional Transmission Organization (RTO), their obligations under this standard are satisified by the obligations of the ISO/RTO.

- 001-n.1	Transmission Provider Obligations – Actual Load Postings

- 001-n.1.1	The Transmission Provider shall post the Actual Daily System-Wide Peak Load, which is the single maximum integrated clock-hour megawatt (MW) value as measured for the prior day using the best available information for the transmission provider’s service territory.  System-wide load includes native load.

- 001-n.1.2	The Transmission Provider shall post the Actual Daily Native Peak Load, which is the total load of Native Load Customers as defined in the approved Tariff or as defined by NERC, and shall be determined as the Actual Daily System-Wide Peak Load for the prior day minus the best available information for the coincident hourly integrated MW value for measured loads within the Transmission Provider’s service territory not considered Native Load.

- 001-n.2	Transmission Provider Obligations – Forecasted Load Postings

- 001-n.2.1	The Transmission Provider shall post the Forecasted Daily System-Wide Peak Load which is the single maximum hourly MW value forecasted for the next pre-schedule day(s) (Western Interconnection) or the next day(s) (Eastern Interconnection), as applicable.

001-n.2.2	The Transmission Provider shall determine the Forecasted Daily System-Wide Peak Load on a comparable basis to the Actual Daily System-Wide Peak Load (e.g., inclusion or exclusion of particular loads).

- 001-n.2.3	The Transmission Provider shall post the Forecasted Daily Native Peak Load which is the Forecasted Daily System-Wide Peak Load minus the coincident hourly MW value of all forecasted loads within the Transmission Provider’s service territory not considered to be Native Load.  The Forecasted Daily Native Peak Load shall be posted for the next pre-schedule day(s) (Western Interconnection) or the next day(s) (Eastern Interconnection), as applicable.

001-n.2.4	The Transmission Provider shall determine the Forecasted Daily Native Peak Load on a comparable basis to the Actual Daily Native Peak Load (e.g., inclusion or exclusion of particular loads).

- 001-n.3	ISO/RTO Obligations – Actual Load Postings

- 001-n.3.1	The ISO/RTO shall post total Actual Daily System-Wide Peak Load, which is defined as the single maximum hourly integrated MW value for the prior day as measured for the entire ISO/RTO tariff footprint.

- 001-n.3.2	If the ISO/RTO is currently creating their tariff footprint Actual Daily System-Wide Peak Load values based on an aggregation of the area values that are readily available (historical Control Area or Balancing Authority Area, or LSE), then those coincident area values shall also be posted for the prior day, as Actual Daily Zonal Load values, in addition to the Actual Daily System-Wide Peak Load.

- 001-n.3.2.1	If the entire ISO/RTO tariff footprint is comprised of a single Balancing Authority Area and was historically a single Control Area prior to ISO/RTO formation, the granularity obligation for posting Actual Daily Zonal Load information has been met by posting the Actual Daily System-Wide Peak Load.

- 001-n.3.3	Posting of Actual Daily System-Wide Peak Load shall meet the ISO/RTO native load posting requirement.

- 001-n.4.	ISO/RTO Obligations – Forecasted Load Postings

- 001-n.4.1	The ISO/RTO shall post the Forecasted Daily System-Wide Peak Load which is the single maximum hourly MW value for the entire ISO/RTO tariff footprint forecasted for the next pre-schedule day(s) (Western Interconnection) or the next day(s) (Eastern Interconnection), as applicable.

- 001-n.4.2	If the ISO/RTO is currently creating their tariff footprint Load Forecast values based on an aggregation of the area values that are readily available (historical Control Area  or Balancing Authority Area, or LSE), then those coincident area values shall also be posted, as Forecasted Daily Zonal Load data, in addition to the Forecasted Daily System-Wide Peak Load value.   The Forcasted Daily Zonal Load shall be posted for the next pre-schedule day(s) (Western Interconnection) or the next day(s) (Eastern Interconnection), as applicable.

- 001-n.4.2.1	If the entire ISO/RTO tariff footprint is comprised of a single Balancing Authority Area and was historically a single Control Area prior to ISO/RTO formation, the granularity obligation for posting load forecast information has been met by posting the Forecasted Daily System-Wide Peak Load.

- 001-n.4.2.2	If the ISO/RTO is providing Actual Daily Zonal Load values, the ISO/RTO shall provide comparable Forecasted Daily Zonal Load values.

- 001-n.4.3	Posting of Forecasted Daily System-Wide Peak Load shall meet the ISO/RTO native load posting requirement.

- 001-n.5	OASIS Information Posting Requirements - Actual Load

- All actual load information required to be posted on OASIS by the Transmission Provider or applicable ISO/RTO shall be made available for query through the systemdata template and in compliance with standards WEQ-002, WEQ-003 and WEQ-013.

- 001-n.5.1	Actual Daily System-Wide Peak Load shall be posted on OASIS and made available for query using the following systemdata template data elements:

- SYSTEM_ELEMENT = Name associated with the Transmission Provider’s service territory or ISO/RTO footprint

- SYSTEM_ELEMENT_TYPE = “LOAD_ZONE”

- SYSTEM_ATTRIBUTE = “SYSTEM_ACTUAL_LOAD”

- 001-n.5.2	Actual Daily Native Peak Load shall be posted on OASIS and made available for query using the following systemdata template data elements:

- SYSTEM_ELEMENT = Name associated with the Transmission Provider’s service territory

- SYSTEM_ELEMENT_TYPE = “LOAD_ZONE”

- SYSTEM_ATTRIBUTE = “NATIVE_ACTUAL_LOAD”

- 001-n.5.3	Actual Daily Zonal (Control Area or Balancing Authority Area, or LSE) Load shall be posted on OASIS and made available for query using the following systemdata template data elements:

- SYSTEM_ELEMENT = Name associated with the metered Balancing Authority Area or former Control Area zone within the ISO/RTO footprint

- SYSTEM_ELEMENT_TYPE = “LOAD_ZONE”

- SYSTEM_ATTRIBUTE = “ZONE_ACTUAL_LOAD”

- 001-n.5.4	The hour corresponding to the Actual Daily Peak Load values shall be represented in the systemdata template START_TIME and STOP_TIME data elements.

- 001-n.6	OASIS Information Posting Requirments - Forecasted Load

- All load forecast information required to be posted on OASIS by the Transmission Provider or applicable ISO/RTO shall be made available for query through the systemdata template and in compliance with standards WEQ-002, WEQ-003 and WEQ-013.

- 001-n.6.1	Forecasted Daily System-Wide Peak Load shall be posted on OASIS and made available for query using the following systemdata template data elements:

- SYSTEM_ELEMENT = Name associated with the Transmission Provider’s service territory or ISO/RTO footprint

- SYSTEM_ELEMENT_TYPE = “LOAD_ZONE”

- SYSTEM_ATTRIBUTE = “SYSTEM_FORECASTED_LOAD”

- 001-n.6.2	Forecasted Daily Native Peak Load shall be posted on OASIS and made available for query using the following systemdata template data elements:

- SYSTEM_ELEMENT = Name associated with the Transmission Provider’s service territory

- SYSTEM_ELEMENT_TYPE = “LOAD_ZONE”

- SYSTEM_ATTRIBUTE = “NATIVE_FORECASTED_LOAD”

- 001-n. 6.3	Forecasted Daily Zonal (Control Area or Balancing Authority Area, or LSE) Load shall be posted on OASIS and made available for query using the following systemdata template data elements:

- SYSTEM_ELEMENT = Name associated with the metered Balancing Authority Area or former Control Area zone within the ISO/RTO footprint

- SYSTEM_ELEMENT_TYPE = “LOAD_ZONE”

- SYSTEM_ATTRIBUTE = “ZONE_FORECASTED_LOAD”

- 001-n.6.4	The time period for which the load forecast value applies shall be represented in the systemdata template START_TIME and STOP_TIME data elements.

- 001-n.6.5	The Transmission Provider or applicable ISO/RTO  shall  post a document on the ATC Information link described in WEQ-001-13.1 that shall contain the current underlying load forecast assumptions. The document shall be a descriptive statement which includes the load forecaster (methodology), all weather variables used (e.g., temperature, humidity, wind speed, number of measuring points), actual load assumptions (e.g., system-wide load, native load, historical loads, existing contracts) and economic assumptions, as applicable (e.g., assumptions for long term ATC calculations may differ from assumptions for short term ATC calculations).

- 001-n.6.6 	When there are differences or deviations in the inputs to the load forecast other than as documented in the descriptive statement specified in 001-n.6.5, a brief specific narrative explanation of such difference or deviation shall be posted with the load forecast numbers using the systemdata template ANNOTATION data element.

- Modifications to WEQ-013 (Revisions noted with redlines)

Business Practices for Open Access Same-Time Information Systems (OASIS) Implementation Guide

Version 1.5

013-3.3.2	System Load Related Query/Response Requirements

Transmission Providers or applicable ISO/RTOs shall provide data for the following SYSTEM_ATTRIBUTE values related to System load postings as required by all applicable regulations and business practices:

- ZONE_FORECASTED_LOAD – The applicable ISO/RTO’s anticipated Forecasted Daily Zonal Load as specified in WEQ-001-n.4

- SYSTEM_FORECASTED_LOAD – The Transmission Provider’s or applicable ISO/RTO’s anticipated Forecasted Daily System-Wide Peak Load as specified in WEQ-001-n.2 and WEQ-001-n.4

- NATIVE_FORECASTED_LOAD – The Transmission Provider’s anticipated portion of the Forecasted Daily System-Wide Peak Load for the native load as specified in WEQ-001-n.2

- ZONE_ACTUAL_LOAD – The applicable ISO/RTO’s Actual Daily Zonal Load as specified in WEQ-001-n.3

- SYSTEM_ACTUAL_LOAD – The Transmission Provider’s or applicable ISO/RTO’s Actual Daily System-Wide Peak Load as specified in WEQ-001-n.1 and WEQ-001-n.3

- NATIVE_ACTUAL_LOAD – The Transmission Provider’s portion of the Actual Daily System-Wide Peak Load for the native load as specified in WEQ-001-n.1

The SYSTEM_ELEMENT_TYPE and SYSTEM_ELEMENT shall be used as follows:

- SYSTEM_ELEMENT_TYPE = LOAD_ZONE

- SYSTEM_ELEMENT = Name associated with the Transmission Provider’s services territory or ISO/RTO footprint or name associated with the metered Balancing Authority Area or former Control Area zone within the ISO/RTO footprint whose forecast and actual load data is being posted; for ZONE_FORECASTED_LOAD, SYSTEM_FORECASTED_LOAD,  NATIVE_FORECASTED_LOAD, ZONE_ACTUAL_LOAD,  SYSTEM_ACTUAL_LOAD, or NATIVE_ACTUAL_LOAD, values shall be the registered NERC entity code for the Transmission Provider or ISO/RTO, as applicable.SYSTEM_LOAD_FORECAST

The following table shows an example of the systemdata template Data Element usage for providing information on load related data:

| Data Element | Restriction/Requirement |
| --- | --- |
| POSTING_REF | The unique OASIS identifier assigned to the posting |
| SYSTEM_ELEMENT_TYPE | “LOAD_ZONE” |
| SYSTEM_ELEMENT | The name of a transmission system element associated with the SYSTEM_ATTRIBUTE being posted. |
| SYSTEM_ATTRIBUTE | “ZONE_FORECASTED_LOAD”, “SYSTEM_FORECASTED_LOAD”, “NATIVE_FORECASTED_LOAD”, “ZONE_ACTUAL_LOAD”, “SYSTEM_ACTUAL_LOAD”, or “NATIVE_ACTUAL_LOAD” |
| START_TIME | Beginning of date/time interval associated with the posted value |
| STOP_TIME | End of date/time interval associated with the posted value |
| ATTRIBUTE_VALUE | Value of the actual or forecasted load over the interval of START_TIME/STOP_TIME |
| ATTRIBUTE_UNITS | “MW” |
| ANNOTATION | Corresponds to forecasted load postings only, as applicable |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

- Modifications to WEQ-003 (Revisions noted with redlines)

003-0	OASIS DATA DICTIONARY Version 1.5  Modifications

Data Dictionary Element Name - SYSTEM_ATTRIBUTE

Alias - SYSATTR

Field Format: minimum characters {type of ASCII} maximum charaters – 0{ALPHANUMERIC}20

Restricted Values – Valid Values:

CBM

FTRM

NFTRM

TTC

FATC

NFATC

ATC_ANNOTATION

ZONE_FORECASTED_LOAD

SYSTEM_FORECASTED_LOAD

NATIVE_FORECASTED_LOAD

ZONE_ACTUAL_LOAD

SYSTEM_ACTUAL_LOAD

NATIVE_ACTUAL_LOAD

{Registered}

Definition of Data Element – Type of system data viewed by systemdata template :

CBM – Capacity Benefit Margin

FTRM – Transmission Reliability Margin for use in FATC

NFTRM – Transmission Reliability Margin for use in NFATC

TTC – Total Transmission Capability

FATC –Firm Available Transmission Capability

NFATC – Non-firm Available Transmission Capability

ATC_ANNOTATION – Annotation for a change in monthly or yearly posted ATC.

ZONE_FORECASTED_LOAD – Anticipated Forecasted Daily Zonal Load

SYSTEM_FORECASTED_LOAD – Anticipated Forecasted Daily System-Wide Peak Load

NATIVE_FORECASTED_LOAD – Anticipated portion of the Forecasted Daily System-Wide Peak Load for the native load

ZONE_ACTUAL_LOAD – Actual Daily Zonal Load

SYSTEM_ACTUAL_LOAD – Actual Daily System-Wide Peak Load

NATIVE_ACTUAL_LOAD – Actual portion of Daily System-Wide Peak Load for the native load

{registered} – Provider specific registered name for the data posted

4.  SUPPORTING DOCUMENTATION

a.  Description of Request:

In the FERC Order 890 under Docket Nos. RM05-17-000 and RM05-25-000 dated February 16, 2007, the Commission made the following conclusions about the requirement for the posting of load forecasts and actual load:

- Paragraph 413 - The Commission adopts the proposed requirement to post on OASIS metrics related to the provision of transmission service under the OATT.  Specifically, transmission providers must post (1) the number of affiliate versus non-affiliate requests for transmission service that have been rejected and (2) the number of affiliate versus non-affiliate requests for transmission service that have been made.  This posting must detail the length of service request (e.g., short-term or long-term) and the type of service requested (e.g., firm point-to-point, non-firm point-to-point or network service).  The Commission also will require transmission providers to post their underlying load forecast assumptions for all ATC calculations and, to post on a daily basis, their actual daily peak load for the prior day.  The Commission directs transmission providers to work through NAESB to develop standards for consistent methods of posting the new requirements on OASIS.

Paragraph 416 - With regard to posting of load forecasts and actual daily peak load, we conclude that such postings are necessary to provide transparency for transmission customers.  We agree with E.ON that RTO and ISO load data needs to be posted at a sufficient granularity to allow for meaningful comparison of control area and LSE load levels.  Most RTOs and ISOs post load data for the entire footprint, but few post it on an LSE or control area basis.  We therefore direct ISOs and RTOs to post load data for the entire ISO/RTO footprint and for each LSE or control area footprint within the ISO/RTO.  This will not create an undue burden on ISOs and RTOs, since the load data for the entire footprint is an aggregation of load data across the LSEs or control areas in the footprint.  We also agree with EEI that the peak load applies to system-wide load, including native load.  We direct transmission providers to post load forecasts and actual daily peak load for both system-wide load (including native load) and native load, as this data will be useful to customers and regulators.  We deny EEI’s request for a guarantee that transmission providers will not be held accountable for producing a reasonable load forecast.  While we do not intend to penalize transmission providers for failing to account for unforeseen circumstances, we retain our ability to investigate any allegations of manipulation of load forecasts, as this could be used as a means of inappropriately denying requested transmission service.

- In the FERC Order 890-A under Docket Nos. RM05-17-001, 002 and RM05-25-001, 002 dated December 28, 2007, the Commission made the following conclusions about this recommendation:

Paragraph 143 - In response to Constellation, we clarify that underlying load forecast assumptions should include economic and weather-related assumptions.  We revise our regulations to clearly state the obligation to post both actual daily peak load and load forecast data, as required in Order No. 890.  We decline to adopt E.ON LSE’s request to delay release of load data required to be posted in Order No. 890.  Posting load forecast and actual load data on a control area and LSE level provides necessary transparency to transmission customers and does not, in our view, raise serious competitive implications.  If there is customer-specific information deemed confidential by the affected customer that impedes the ability of the transmission provider to post this data, we will consider requests for exemption from the posting requirement on a case-by-case base.

The following is the excerpt from Commission regulations related to the above determinations:

890 18 CFR 37.6 Paragraph (b)(3)(iv);

(iv) Daily load. The Transmission Provider must post on a daily basis, its actual daily peak load for the prior day.

890-A 18 CFR 37.6 Paragraph (b)(3)(iv) revised:

(iv) Daily load. The Transmission Provider must post on a daily basis, its load forecast, including underlying assumptions, and actual daily peak load for the prior day.

2008 WEQ Annual Plan Item 2.a.i.4

Load Forecast and Actual Load Business Practice Standards associated with S&CP changes.

b.  Description of Recommendation:

c.  Business Purpose:

Implementation of FERC Orders 890 and 890-A.

- Commentary/Rationale of Subcommittee(s)/Task Force(s):

Note: Reference in section WEQ-001-n.4.5 to the ATC Information link is dependent on ratificatication of the recommendation number “2008 AP Item 2.a.i.5, 2008 AP Item 2.a.i.6, and 2008 AP Items 2.a.ii.1-3” http://www.naesb.org/pdf3/2008_weq_ap_2ai5_2ai6_2aii1-3_rec_clean.doc.

Note: Redline changes to WEQ-013-3.3.2 are based on the membership ratification of the recommendation number “2007 AP item 2(ii)” http://www.naesb.org/doc_view2.asp?doc=weq_rat013108_2007_weq_ap_2ii_rec_attach013.doc, which was ratified on February 29, 2008.

Note: Redline changes to WEQ-003-0 are based on the membership ratification of the recommendation number “2007 AP item 2(ii)” http://www.naesb.org/doc_view2.asp?doc=weq_rat013108_2007_weq_ap_2ii_rec_attach003.doc, which was ratified on February 29, 2008.

Please review the following Joint BPS and ESS/ITS ATC/AFC Subcommittee meeting minutes:

- February 13, 2008

- February 21-22, 2008

- February 27, 2008

- March 10, 2008

Please review the Motions Document:  http://naesb.org/pdf3/weq_atc_afc031008a2.doc

See Order No. 890 at P 416.

See id. at P 417.