Dear Wholesale Electric Quadrant Members and Interested Industry Participants –

For the attached recommendation for standards that are linked to NERC standards development for FERC Order No. 890, our subcommittees have accepted these proposed standards and are asking you to comment on them in a formal comment period.  They are also asking the WEQ Executive Committee to consider this recommendation and comments that are submitted during the formal comment period, and vote on this recommendation.

The subcommittees understand that the proposed standards include references to NERC reliability standards that have not yet been approved through the NERC process, but are based on their current work products.  The proposed standards include placeholders for the NERC references, which will be added once they are approved by NERC.  The subcommittees do not expect that this recommendation will require significant rework by the subcommittees once NERC adopts its related reliability standards.  The subcommittees’ chairs will review the final NERC standards once adopted and will identify if any changes are needed to recommendations that have already been processed through commenting and EC consideration.  If changes are needed, a determination will be made whether the changes can be processed as minor actions, or for more substantive changes, the standards modification process will be used.

We are taking these steps and offering this recommendation for your comment and for EC consideration for vote rather than waiting until NERC completes its full process because:

- NERC has developed draft standards for the referenced items which have been processed through at least one ballotting period.

- The NAESB subcommittees do not expect the changes to be made by NERC in its current efforts to yield substantive changes to the NAESB related standards.

- The progress being made by the subcommittees now on Order 890 towards meeting the August deadline will be adversely affected if the all NAESB Order No. 890 recommendations with NERC references are held in abeyance until NERC has concluded its efforts.

- The interested industry participants and the WEQ EC will have the opportunity to review the  NAESB Order No. 890 recommendations with NERC references for formal comment and consideration as NAESB completes its efforts on each recommendation.  In this manner, the industry and the EC are asked to consider proposed standards within a reasonable workflow.  The alternative is to hold all NAESB Order No. 890 recommendations with NERC references and then submit them all for industry comment and EC consideration which would provide a significant amout of documents for review, comment and consideration.

- Last but certainly not least, submitting the recommendations now to the industry and to the WEQ EC will provide the necessary signals from the industry through formal comments and through EC actions.  If corrective action is needed, NAESB would have the opportunity to meet or come closer to the deadlines set by the FERC.

Please note that the subcommittees have worked diligiently on this recommendation along with the work products available from NERC and that all steps outlined conform with NAESB operating procedures.  We appreciate your consideration and your comments.

With Best Regards,

Marcie Otondo, Co-Chair, NAESB Electronic Scheduling Subcommittee/Information Technology Subcommittee

Ed Skiba, Co-Chair, NAESB Business Practices Subcommittee

Paul Sorenson, Co-Chair, NAESB Electronic Scheduling Subcommittee/Information Technology Subcommittee

J.T. Wood, Co-Chair, NAESB Business Practices Subcommittee and Co-Chair,  NAESB Electronic Scheduling Subcommittee/Information Technology Subcommittee

1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

X  Accept as requested				  X  Change to Existing Practice

Accept as modified below			       Status Quo

Decline

2.  TYPE OF DEVELOPMENT/MAINTENANCE

- Per Request:					Per Recommendation:

X Initiation					 X  Initiation

Modification				 	      Modification

Interpretation					      Interpretation

Withdrawal					      Withdrawal

Principle 					      Principle

Definition 					      Definition

X Business Practice Standard 			 X  Business Practice Standard

Document 					      Document

Data Element 				      Data Element

Code Value 					      Code Value

X12 Implementation Guide			     X12 Implementation Guide

Business Process Documentation		     Business Process Documentation

3.  RECOMMENDATION

BACKGROUND:  The Joint ESS/ITS and BPS subcommittee reviewed the 2008 WEQ Annual Plan Item 2.c: Develop version 1 business practice standards to support transparency reporting and related functions that may be required as a result of the final order.

Upon review of this annual plan item and the requirements developed by NERC under MOD-001, MOD-004, MOD-008, MOD028, MOD-029, and MOD-030 the joint subcomittee is recommending the following NAESB Business Practice Standards be developed for this annual plan item.

- SUMMARY:

The WEQ EC ad hoc group met on June 6, 2008 and determined to pursue the list, not through the proscriptive means of detailing each piece of information that should be made available or not made available publicly, and then uniformly apply such requirements to all transmission providers.  Instead the group determined to pursue the list through a standardized report that the TPs would post on OASIS.  The report would be standardized, and each TP would complete the report specific to its own operations, and would maintain the currency of the information posted.  In this approach, the TPs would not have a “one size fits all” approach to the information to be made public or to remain not-public, and the market participants would have access through a standardized report to the TPs decisions on acess to ATC related information. The ad hoc group met also on July 18 and August 14, 2008 to finalize this approach and draft the needed business practices.

- Recommended Standard:

- NOTE: NAESB Staff will assign appropriate enumeration to this standard to replac “n” with next sequential standard number within WEQ-001

WEQ-001.n	Each Transmission Provider (TP) shall prepare a report according to the format given herein,completing columns and will post such report on OASIS in accordance with WEQ-001.13.1.5 Report on the List of ATC Related Information Report.

- LIST OF ATC RELATED INFORMATION

This List identifies the data associated with ATC calculations, as addressed in NERC ATC Standards (MOD - 001, MOD – 004, MOD – 008, MOD – 028, MOD – 029 and MOD – 030).  The Transmission Provider shall indicate whether or not the data will be publicly available.

This standard and table address the provision of ATC-related data to the industry at large, while the NERC MOD standards address the exchange of data amongst those entities that require the data for reliability purposes.  Compliance with the NERC MOD standards should not imply compliance with the NAESB standard or vice versa.  Nor should non-compliance with the NERC standards imply non-compliance with the NAESB standards or vice versa.

| Item # | NERC Reference | ATC-Related Item2 |  | Available3 | Comments4 |
| --- | --- | --- | --- | --- | --- |
| 1 | MOD-001 | The Available Transfer Capability Implementation Document (ATCID) to include the following: How the methodology has been implemented How counterflows are accounted for: - How confirmed transmission reservations, expected Interchange, and internal counterflow are addressed, including rationale for that accounting A description of the allocation processes listed below that are applicable to the Transmission Service Provider: - Processes used to allocate transfer or Flowgate capability among multiple lines or sub-paths within a larger ATC Path or Flowgate. - Processes used to allocate transfer or Flowgate capabilities among multiple owners or users of an ATC Path or Flowgate. - Processes used to allocate transfer or Flowgate capabilities between Transmission Service Providers to address issues such as forward looking congestion management or seams coordination - A description of how partial period outages or outages from other providers that cannot be directly mapped are handled |  |  |  |
| 2 | MOD-001 | Expected generation outages |  |  |  |
| 3 | MOD-001 | Expected generation additions |  |  |  |
| 4 | MOD-001 | Expected generation retirements |  |  |  |
| 5 | MOD-001 | Expected Transmission outages |  |  |  |
| 6 | MOD-001 | Expected Transmission additions |  |  |  |
| 7 | MOD-001 | Expected Transmission retirements |  |  |  |
| 8 | MOD-001 | Load forecasts |  |  |  |
| 9 | MOD-001 | Unit commitments and order of dispatch, to include all designated network resources and other resources that are committed or have the legal obligation to run, as they are expected to run, in one of the following formats chosen by the data provider: - Dispatch Order - Participation Factors - Block Dispatch |  |  |  |
| 10 | MOD-001 | Aggregated firm capacity set-aside for Network Integration Transmission Service |  |  |  |
| 11 | MOD-001 | Aggregated non-firm capacity set-aside for Network Integration Transmission Service (i.e., Secondary Service) |  |  |  |
| 12 | MOD-001 | Firm Transmission reservations |  |  |  |
| 13 | MOD-001 | Non-firm Transmission reservations |  |  |  |
| 14 | MOD-001 | Aggregated capacity set-aside for Grandfathered obligations |  |  |  |
| 15 | MOD-001 | Firm roll-over rights |  |  |  |
| 16 | MOD-001 | Any firm adjustments applied by the Transmission Service Provider to reflect parallel path impacts |  |  |  |
| 17 | MOD-001 | Any non-firm adjustments applied by the Transmission Service Provider to reflect parallel path impacts |  |  |  |
| 18 | MOD-001 | Power flow models and underlying assumptions |  |  |  |
| 19 | MOD-001 | Contingencies, provided in one or more of the following formats: - A list of Elements - A list of Flowgates - A set of selection criteria that can be applied to the Transmission model used by the Transmission Operator and/or Transmission Service Provider |  |  |  |
| 20 | MOD-001 | Facility Ratings |  |  |  |
| 21 | MOD-001 | Any other services that impact Existing Transmission Commitments (ETCs) |  |  |  |
| 22 | MOD-001 | Values of Capacity Benefit Margin (CBM), Transmission Reliability Margin (TRM) for all ATC Paths or Flowgates |  |  |  |
| 23 | MOD-001 | Values of Total Flowgate Capability (TFC) and (AFC) for any Flowgates considered by the Transmission Service Provider selling the Transmission Service |  |  |  |
| 24 | MOD-001 | Values of Total Transfer Capability (TTC) and Available Transfer Capability (ATC) for all ATC Paths or Flowgates |  |  |  |
| 25 | MOD-001 | Source and sink identification and mapping to the model |  |  |  |
| 26 | MOD004 | The Capacity Benefit Margin Implementation Document (CBMID) including the procedures and assumptions for establishing CBM for each ATC Path or Flowgate |  |  |  |
| 27 | MOD 004 | The amount of CBM set aside for Load-Serving Entities and Resource Planners over each ATC Path or Flowgate |  |  |  |
| 28 | MOD 004 | Applicable supporting data, including any models, used for determining CBM or allocating CBM over each ATC Path or Flowgate |  |  |  |
| 29 | MOD 008 | The Transmission Reliability Margin Implementation Document (TRMID), including a list of the components of uncertainty used in calculating TRM for each ATC Path or Flowgage and a description of how the component is used and of the method used to allocate TRM across ATC Paths or Flowgates. |  |  |  |
| 30 | MOD 028 | Each Transmission Service Provider shall include in its ATCID, at a minimum, the following information relative to its methodology for determining TTC: - Information describing how the selected methodology has been implemented - A description of the manner in which the Transmission Operator will account for Interchange Schedules in the calculation of TTC - Any contractual obligations for allocation of TTC - A description of the manner in which Contingencies are identified for use in the TTC process - The following information on how source and sink for transmission service is accounted for in ATC calculations including: - Define if the source used for ATC calculations is obtained from the source field or the POR field of the transmission reservation - Define if the sink used for ATC calculations is obtained from the sink field or the POD field of the transmission reservation - The source/sink or POR/POD identification and mapping to the model - If the Transmission Service Provider’s ATC calculation process involves a grouping of generation, the ATCID must identify how these generators participate in the group |  |  |  |
| 31 | MOD 029 | The TTC study report documenting the assumptions used and steps taken in determining the current value for TTC for that ATC Path. |  |  |  |
| 32 | MOD 029 | The firm capacity set aside for Native Load commitments, including losses and native load growth |  |  |  |
| 33 | MOD 030 | The Transmission Service Provider shall include in its ATCID: - The criteria used by the Transmission Operator to identify sets of Transmission Facilities as Flowgates that are to be considered in Available Flowgate Capability (AFC) calculations - The following information on how source and sink for transmission service is accounted for in AFC calculations including: - Define if the source used for AFC calculations is obtained from the source field or the Point of Receipt (POR) field of the transmission reservation. - Define if the sink used for AFC calculations is obtained from the sink field or the Point of Delivery (POD) field of the transmission reservation - The souce/sink or POR/POD identification and mapping to the model If the Transmission Service Provider’s AFC calculation process involves a grouping of generators, the ATCID must identify how these generators participate in the group |  |  |  |
| 34 | MOD 030 | Regarding the firm and non-firm ETC calculations, identify the elements of the calculation as follows: - The impact of firm and non-firm NITS including the impacts of generation to load and based on the load forecast including native load and network service load and unit commitment and dispatch order as specified in the ATCID for the TSPs area. - The impact of firm or non-firm NITS including the impacts of generation to load for the areas of adjacent TSPs and any other TSPs with coordination agreements where the distribution factor is equal to or greater than DF%, where DF% is the percentage distribution factor used to determine which are included in the AFC analysis for ETC calculations. - The impact of confirmed firm and non-firm point-to-point service for the TSPs area, expected to be scheduled including roll-over rights - The impact of confirmed firm and non-firm point-to-point service for the TSPs area, expected to be scheduled with duplicate impacts eliminated, including roll-over rights using service from multiple TSPs for the areas of adjacent TSPs and any other TSPs with coordination agreements where the distribution factor is equal to or greater than DF%, where DF% is the percentage distribution factor used to determine which are included in the AFC analysis for ETC calculations. - The impact of any firm or non-firm Grandfathered obligations, expected to be scheduled or expected to flow in the TSPs area. |  |  |  |
|  |  | - The impact of any firm or non-firm Grandfathered obligations, expected to be scheduled or expected to flow for the areas of adjacent TSPs and any other TSPs with Coordination Agreements where the distribution factor is equal to or greater than DF%, where DF% is the percentage distribution factor used to determine which are included in the AFC analysis for ET C calculations. - The impact of any firm or non-firm Other Services as determined by the TSP |  |  |  |

Modifications to WEQ-001 (Revisions noted with redlines)

Additions to WEQ-001-13.1.5:

001-13.1.5	ATC INFORMATION LINK

If the Transmission Provider does not use CBM or TRM in their assessment of ATC, that information shall  be found in the CBM Implementation Document – CBMID or TRM Inplementation Document – TRMID link below.

The information posted at “ATC Information” should include the following links (as applicable to the Transmission Provider’s methodologies):

- Available Transfer Capability Implementation Document - ATCID (as specified in NERC [MOD-001-?])

- CBM Implementation Document - CBMID (as specified in NERC [MOD-004-?])

- TRM Implementation Document - TRMID (as specified in NERC [MOD-008-?])

    - ATC Methodology Contact (as required by WEQ-001-xx)

    - Report on the List of ATC Related Information Report (as required by WEQ-001-n)

The posting of this information would be subject to the Transmission Provider’s ability to redact certain provisions due to market, security or reliability sensitivity concerns. Any section that has been redacted shall retain the heading and will include the reason for the redaction. Appropriate reasons for redaction are:  “Market Sensitive Information”, “CEII Information”, “Security Sensitive Information” or “Reliability Sensitive Information”.

These items shall appear in the order specified above and before any other items which may be required as per specific FERC direction or local business practice related to ATC Information. Posting of the cites noted in the parentheses is optional. Access to some of the information found under the ATC Information Link above may require the user to register with the individual OASIS sites according to WEQ-002-3.1.

4.  SUPPORTING DOCUMENTATION

a.  Description of Request:

The request can be inferred from the FERC Order No. 890 and 890-A requirements for transparency of ATC/TTC related information.  A recitation of the relevant cites from the orders can be found at the following link:  http://www.naesb.org/pdf3/weq_ec_atc060608w1.doc.

b.  Description of Recommendation:

The recommendation provides a reporting tool to aid in transparency as referenced in FERC Order Nos. 890, 890-A, and 890-B.  It identifies the information for which each Transmission Provider (TP) will designate the access as public or non-public with reasons given for the access designation.

c.  Business Purpose:

The purpose served by providing the report is market transparency.  In providing the report, the market participants can easily identify which ATC related information is publicly available versus that information that requires restricted or no access.

d.  Commentary/Rationale of Subcommittee(s)/Task Force(s):

Joint BPS & ESS/ITS Final Meeting Minutes March 10-13, 2008: http://naesb.org/pdf3/weq_atc_afc031008fm.doc

ATC-AFC Motions (from March 10-13, 2008 meeting):  http://naesb.org/pdf3/weq_atc_afc031008a8.doc

WEQ EC Meeting 5-13-08:  http://naesb.org/pdf3/weq_ec051308dm.doc

WEQ EC ATC Information List Task Force Meeting 6-6-08:  Add Link

WEQ EC ATC Information List Task Force Meeting 7-18-08: Add Link

WEQ EC ATC Information List Task Force Meeting 8-14-08: Add Link

Note: The revisions to WEQ001-13.1.5 were applied to the current version, which has been voted out of subcommittee pending EC approval and membership ratification.

2008 WEQ Annual Plan Item 2.b.vii Final Action: http://naesb.org/member_login_form.asp?doc=fa_2008_weq_api_2bvii.doc

The Transmission Provider should enter a response of “Yes” or “No” for each data element.

All “No” responses should contain a reason in the Comment field.

Potential “No” Comments:

- Market Sensitive Information

- CEII Information

- Security Sensitive Information

- Reliability Sensitive Information

- Data not created by the Transmission Provider but provided to the Transmission Provider from xxx (include entity identity of information provider)

- Data not utilized by the Transmission Provider

Potential “Yes” Comments:

- Available on OASIS

- Available upon request (in the electronic format in which it was created along with any necessary decoding instructions)

- Available upon request (in the electronic format in which it was created along with any necessary decoding instructions) but may be subject to a confidentiality agreement