1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| X | Accept as requested | X | Change to Existing Practice |
| --- | --- | --- | --- |
|  | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

2.  TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
|  | Initiation |  | Initiation |
|  | Modification |  | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
|  | Principle |  | Principle |
|  | Definition |  | Definition |
| X | Business Practice Standard | X | Business Practice Standard |
|  | Document |  | Document |
|  | Data Element |  | Data Element |
|  | Code Value |  | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
|  | Business Process Documentation |  | Business Process Documentation |

3.  RECOMMENDATION

SUMMARY:

Establish default scheduling intervals if intervals have not been defined in a Balancing Authority’s or Transmission Service Provider’s Governing Documents. Governing Documents are defined in WEQ-000 as “documents that determine the interactions between a System Operator and other parties, including but not limited to, applicable law, regulatory documents (e.g. tariffs, rules, regulations), contractual agreements, operational manuals, and other relevant models and operational procedures.”

Recommended Standards:

WEQ Coordinate Interchange Business Practice Standards – WEQ-004

004-xx 	DEFAULT SCHEDULING INTERVALS

Unless otherwise stated in applicable Governing Documents, each Balancing Authority and Transmission Service Provider shall support default transaction start and transaction stop times as follows: xx:00, xx:15, xx:30, and xx:45.

4.  SUPPORTING DOCUMENTATION

a.  Description of Request:

FERC order 764 directs transmission entities to accept scheduling interchange in 15 minute intervals within the operating hour. Currently no NAESB standards require, state, or give guidance that the default for these intervals should be on the quarter hour increment (i.e. 0 (top of the hour), 15, 30, and 45, etc.). In order to ensure a smooth, interconnection wide, transition to 15 minute scheduling a default definition of 15 minute schedules needs to be developed.

b.  Description of Recommendation:

Unless otherwise stated in applicable Governing Documents, each Balancing Authority and Transmission Service Provider shall support default transaction start and transaction stop times as follows: xx:00, xx:15, xx:30, and xx:45.

c.  Business Purpose:

Provide a default of 15 minute scheduling intervals in the event scheduling intervals are not defined in a Transmission System Provider’s or Balancing Authority’s Governing Documents.  Governing Documents are defined in WEQ-000.

d.  Commentary/Rationale of Subcommittee(s)/Task Force(s):

Supporting Documentation

FERC Order 764

R12010

February 5-6, 2013, Joint Electric Scheduling Subcommittee Meeting Minutes

March 5-6, 2013, Joint Electric Scheduling Subcommittee Meeting Minutes, when available