- 1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| x | Accept as requested | x | Change to Existing Practice |
| --- | --- | --- | --- |
|  | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

- 2.  TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
| x | Initiation | x | Initiation |
|  | Modification |  | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
| x | Principle | x | Principle |
| x | Definition | x | Definition |
| x | Business Practice Standard | x | Business Practice Standard |
|  | Document |  | Document |
|  | Data Element |  | Data Element |
|  | Code Value |  | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
|  | Business Process Documentation |  | Business Process Documentation |

Note: This does not include Technical data

- 3.  RECOMMENDATION

- SUMMARY:

The Retail Electric Quadrant (REQ) DSM-EE Subcommittee submits this Recommendation for 2012 Retail Annual Plan Item No. 3(b) “Develop business practice standards used to measure and verify reductions in energy and demand from energy efficiency in wholesale and retail markets. This includes developing business practices to measure and verify energy reductions for energy efficiency or a stand-alone Energy Efficiency Portfolio Standard.”

DISCLAIMER:  This document contains Model Business Practices for the Measurement and Verification of electrical energy and demand impacts of Energy Efficiency Programs implemented within any regulated or unregulated retail arena.  The information and practices contained within this document are subject to the Governing Documents and the requirements of the Applicable Regulatory Authority, and in the event of a conflict, the latter should have precedence over these Model Business Practices.

REQ.19	Measurement & Verification of Energy Efficiency Programs

Executive Summary

This document contains the Model Business Practices for the Measurement & Verification (“M&V”) of Energy Efficiency programs.  These Model Business Practices are applicable to the M&V of electrical Energy (kWh) and Demand (kW) impacts, also referred to as reductions or savings, of Energy Efficiency programs offered to Retail Customers.

These Model Business Practices may be applied within the context of regulatory or other market requirements and agreements. The information contained in these Model Business Practices does not replace the Governing Documents or the requirements of the Applicable Regulatory Authority.  In the event of a conflict, the Governing Documents and the requirements of the Applicable Regulatory Authority should have precedence over these Model Business Practices.

Model Business Practices for M&V of Energy Efficiency programs have the potential to broaden implementation and acceptance of energy reduction measures and practices in both retail and wholesale markets.  Retail Energy Efficiency in retail electricity markets should provide consistent and reliable evidence of reductions in electrical usage for qualification and performance.  Methodologies for qualifying and demonstrating energy and demand reductions should be specified in the Governing Documents. These Model Business Practices are not intended to replace the existing rules and tariffs stipulated within each market or to establish or support any policy.

Introduction

The North American Energy Standards Board (NAESB) is a voluntary non-profit organization comprised of members from all aspects of the natural gas and electric industries. Within NAESB, the Retail Electric Quadrant (REQ) and the Retail Gas Quadrant (RGQ) focus on issues impacting the retail sale of energy to Retail Customers. REQ / RGQ Model Business Practices are intended to provide guidance to Distribution Companies, other Market Participants, and Applicable Regulatory Authorities involved in providing energy service to Retail Customers. The focus of these Model Business Practices is the Measurement & Verification of Energy Efficiency programs.

These Model Business Practices are voluntary and do not address policy issues that are the subject of state legislation or regulatory decisions. These voluntary Model Business Practices have been adopted by NAESB with the realization that as the industry evolves, additional and amended voluntary Model Business Practices may be necessary. Any industry participant seeking additional or amended voluntary Model Business Practices (including principles, definitions, data elements, process descriptions, and technical implementation instructions) should submit a request to the NAESB office, detailing the change, so that the appropriate process may take place to amend the voluntary Model Business Practices.

Business Processes and Practices

REQ.19.1	   Principles

REQ.19.1.1	These Model Business Practices pertain to M&V of retail Energy Efficiency projects and programs. These Model Business Practices are intended to be applicable in any regulated or unregulated retail arena. The information contained within these Model Business Practices is not intended to replace the Governing Documents or the requirements of the Applicable Regulatory Authority.  In the event of a conflict between these Model Business Practices and the Governing Documents or the requirements of the Applicable Regulatory Authority, the latter two should have precedence.

REQ.19.1.2	This document is intended to provide general M&V guidance, and is intended to create consistency across retail and wholesale markets, where appropriate and applicable.  These Model Business Practices do not establish practices or provide guidance related to the compensation, design, operation, or use of Energy Efficiency.  These Model Business Practices do not establish practices or provide guidance related to how the results are used.  They do not establish practices or provide guidance related to the evaluation of program design, cost effectiveness (cost-benefit analysis), implementation (process evaluation) or market assessments (market evaluations).

REQ 19.1.3	These Model Business Practices include recognition that Energy Efficiency is an evolving practice within the energy service industry with increased penetration across wholesale and retail markets.   As such, terminology used in the energy service industry to define approaches to quantifying energy savings and Demand reductions from Energy Efficiency investments vary.  For the purposes of these Model Business Practices, the term M&V refers to a range of activities that are used to estimate savings from Energy Efficiency projects or programs.  Such activities not only include M&V of site-specific project savings, but also include statistical sampling and analysis to estimate program level savings, measure life and persistence, and use of deemed savings and large scale billing analysis.  In these Model Business Practices, the term M&V covers this range of activities which are sometimes referred to as “impact evaluation” activities in the retail industry and relevant guidance documents.

REQ.19.2	     Definition of Terms

REQ.19.2.A	Business Definitions

RXQ.0.2.1	Applicable Regulatory Authority:  The state regulatory agency or other local governing body that provides oversight, policy guidance, and direction to any parties involved in the process of providing energy to Retail Customers through regulations and orders.

REQ.0.2.153	Demand:  The rate at which energy is delivered to or by a system or part of a system, generally expressed in kilowatts or megawatts, at a given instant or averaged over any designated interval of time; and the rate at which energy is being used by the Retail Customer.

REQ.0.2.154	Demand Reduction Value:  Measurement of reduced electricity usage by a Demand Resource during a Demand Response Event or Energy Efficiency performance hours, generally expressed in kilowatts or megawatts.

REQ.0.2.xx	Energy Efficiency:  Installed measures (e.g. products, equipment, systems, services, practices and/or strategies) that reduce the total amount of electrical energy needed, while delivering a comparable or improved level of end-use service.  Such measures include, but are not limited to, the installation of more energy efficient lighting, motors, refrigeration, HVAC equipment and control systems, envelope measures, operations and maintenance procedures, and industrial process equipment.

RXQ.0.2.22	Governing Documents:  Documents that determine the interactions among parties, including but not limited to: applicable law, regulatory documents (e.g. tariffs, rules, regulations), contractual agreements, Distribution Company Operational Manuals, and other relevant models and operational procedures.

REQ.0.2.168	Load:  An end-use device or Retail Customer that receives power from the electric system.

REQ.0.2.xx	Measurement & Verification (M&V):  The process of determining reductions in usage and/or Demand resulting from Demand Response or Energy Efficiency.

RXQ.0.2.208	Model Business Practices:  Electric and gas industry processes and procedures developed by interested parties representing the NAESB Retail Gas and Retail Electric Quadrants’ segments and ratified by the NAESB Retail Gas and Retail Electric Quadrants’ members.

RXQ.0.2.207	Retail Customer:  Any Entity that takes or is applying to take gas and/or electric service for its own consumption.

REQ.0.2.192	Validating, Editing, and Estimation (VEE):  The process of confirming the accuracy of raw meter data and, if necessary, replacing corrupt or missing data.  VEE guidelines are published in the Edison Electric Institute’s Uniform Business Practices for Unbundled Electricity Metering.

REQ.19.2.B	Technical Definitions - Reserved

REQ.19.2.C	Abbreviations and Acronyms

ANSI	American National Standards Institute

HVAC	Heating, Ventilation & Air Conditioning

M&V	Measurement & Verification

NIST	National Institute of Standards & Technology

VEE	Validating, Editing and Estimation

REQ.19.3	Model Business Practices

REQ.19.3.1		Measurement and Verification Methodologies

REQ.19.3.1.1	M&V Methodologies:  M&V methodologies should be appropriate to the measure type and sensitivity of the measurement techniques.  These methods are commonly applied to analyzing measure or project level savings. A representative sample of projects in the program can be selected and the savings from those selected projects are determined and may be applied to the entire population of projects.

Acceptable methods can include, but are not limited to, the following  options.

REQ.19.3.1.1.1	Option A: Partially Measured Retrofit Isolation/Stipulated Measurement:  Option A may involve an equipment specific retrofit or replacement, new installation or a system level M&V assessment. The approach is intended for measures where either performance factors (such as lighting wattage) or operational factors (such as operating hours) can be measured on a spot or short-term, or for measures for which a measured proxy variable and/or stipulated factors, can provide an accurate estimate of energy and demand savings.

REQ.19.3.1.1.2	Option B: Retrofit Isolation/Metered Equipment:
Option B involves a retrofit or system-level M&V assessment. The approach is intended for retrofits with performance factors and operational factors that can be measured at the component or system level using interval electrical Demand meters installed on the affected end-use.

REQ.19.3.1.1.3	Option C: Whole Facility/Regression Analysis:
Option C estimates energy and Demand by analyzing the overall energy use in a facility and identifying the impact of the implemented measures on the total building or facility energy use patterns. The analysis of whole-building or facility level metered data may be completed using techniques ranging, for example, from billing comparisons to multivariate regression analysis.

REQ.19.3.1.1.4   Option D: Calibrated Simulation:  Option D involves calibrated computer simulation models of component or whole-building Demand and energy usage to measure Demand and energy savings.

REQ.19.3.1.2	Alternative M&V Methodologies:  Alternative or supplemental methodologies should be appropriate to the measure type and sensitivity of the estimation techniques.  These alternative methodologies are commonly applied to program level savings, and may include, but are not limited to:

REQ.19.3.1.2.1	Deemed savings: Deemed savings are stipulated values based on historical savings values of like measures directly or indirectly measured, determined through engineering calculations or based on previous studies. As with the  M&V options described in REQ.19.3.1.1, the savings determined for a sample of projects may be applied to all the measures or projects in the program. This approach is best suited for projects with predictable operating conditions and documented stipulated values such as energy-efficient appliances.

REQ.19.3.1.2.2	Large-scale billing analysis: Statistical analyses are conducted on the energy usage data collected from revenue meters or equivalent for all or most of the participants in an Energy Efficiency program and either non-participants (a control group) or a baseline condition. This approach is primarily used for residential programs with homogeneous participants, load characteristics and measures.  Billing analysis may be appropriate when project-specific analyses are not practical.  Billing analysis may only be useful for quantification of energy use rather than Demand use, unless interval meter data is available.

REQ.19.3.1.3	Verification:  For projects or programs involving installation of measures, methodologies should include a verification component for each project or a sample of projects that verifies Energy Efficiency Baseline conditions, measures were actually installed, and/or measures were installed and are operating correctly.

REQ.19.3.1.4	Measure Life and Persistence:  Methodologies should include mechanisms for estimating measure life and persistence of measures.

REQ.19.3.2		Energy Efficiency Baselines

REQ.19.3.2.1	Underlying Assumptions:  Energy Efficiency baseline definitions should include a description of underlying assumptions used for establishing the Energy Efficiency baseline conditions that would have occurred in the absence of the program (i.e., the counterfactual).

REQ.19.3.2.2	Energy Efficiency Baseline Conditions:  The Energy Efficiency baseline should reflect the conditions under which new energy efficient equipment or processes are installed to provide a service function.  The four primary conditions are as follows:

(a) Replacement or retrofit of functional equipment still within its current useful life or process improvements.

(b) Replacement of functional equipment beyond its current useful life.

(c) Unplanned replacement for (of) failed equipment.

(d) New construction.

REQ.19.3.2.3	Standard Energy Efficiency Baseline:  The standard Energy Efficiency baseline should be the nameplate rating of the equipment meeting the more stringent level of Energy Efficiency required by applicable state code, the federal or state (as applicable) product Energy Efficiency standard, or standard practice.  The standard Energy Efficiency baseline should be determined at the time of installation or as set forth in the Governing Documents or as established by the Applicable Regulatory Authority.

REQ.19.3.2.4	Current Load Energy Efficiency Baseline.  The current Load Energy Efficiency baseline should be the current Load of the existing operating equipment or facility.  The current Load Energy Efficiency baseline should be determined at the time of installation or as set forth in the Governing Documents or as established by the Applicable Regulatory Authority.

REQ.19.3.2.5	The application of the Energy Efficiency baseline conditions described in REQ 19.3.2.2 applicable to the two Energy Efficiency baselines in REQ 19.3.2.3 and REQ 19.3.2.4 is summarized below in Table REQ.19.3.2.6.

Table REQ.19.3.2.6.

|  | Primary Condition | Standard EE Baseline | Current Load EE Baseline |
| --- | --- | --- | --- |
| A | Replacement or retrofit of functional equipment still within its current useful life or process improvements |  | X |
| B | Replacement of functional equipment beyond its current useful life | Depends on Governing Documents or Applicable Regulatory Authority | Depends on Governing Documents or Applicable Regulatory Authority |
| C | Unplanned replacement for (of) failed equipment | X |  |
| D | New construction | X |  |

REQ.19.3.4	Statistical Significance

REQ.19.3.4.1	General:  M&V of Energy Efficiency programs may include measurement methodologies utilizing statistical estimation techniques for estimating energy and Demand savings.  In the event that statistical methods are used, the following expectations for statistical significance should be met:

REQ.19.3.4.1.1	Specification for Statistical Error and Precision when Sampling is Used:  Sample error and precision used should be suited to the provisions of the program (e.g. at least 80/10 using a two-tailed test or 90/10 using a one-tailed test), subject to the Governing Documents and the requirements of the Applicable Regulatory Authority.

REQ.19.3.4.1.2	Sample Size Calculation:  The sample size should reflect a population coefficient of variation (c.v.), which may not be known at the time of sample design.  The desired error and precision level are also inputs into sample size calculation.  The sample size may be established using an estimate of the c.v.  For example, the estimated c.v. should be not less than 0.5 for homogeneous populations and not less than 1.0 for heterogeneous populations.

REQ.19.3.4.1.3	Sample Size Recalibration:  Population sampling error and precision should be recalibrated using a measured c.v.  The sample c.v. used should be calculated from the measured sample population and applied to the reported estimate of sampling error and precision.

REQ.19.3.4.1.4	Sampling Over Heterogeneous Geographically Areas:  Sampling used over heterogeneous geographical areas and/or application of sampling studies from one area to another should help control for potential bias and help meet the overall error and precision requirements.

REQ.19.3.4.1.5	Sample Bias Control:  Sampling bias control should mitigate sources of bias including, but not limited to:

(a) Construct validity;

(b) Sampling frame versus population;

(c) Selection bias (for a sample and for a census attempt where not all sites within the census received usable data);

(d) Non-response bias;

(e) Error in measuring variables;

(f)	Sample homogeneity relative to project (external validity);

(g) Outlier data points; and

(h) Missing data.

REQ.19.3.5	Energy Efficiency Value Savings Calculations:  Energy Savings and Demand Reductions Calculations

REQ.19.3.5.1	Energy Efficiency Savings Value Calculation Variables:  Calculation of energy and Demand Reduction Values for equipment, measures and practices should be performed using energy (kWh) or Demand (kW) values calculated according to M&V methodologies provided herein.  Calculation of Demand Reduction Values for equipment, measures and practices, including weather sensitive Loads, may include estimated modifiers or proxy variables.  Estimated modifiers and proxy variables used in the calculation of the Demand Reduction Value should include, but are not limited to the following: coincidence factor, realization rate, equipment failure rate, weather normalization for weather sensitive loads, temperature, humidity, flow, concentration, volts, amps, lumens, and quantity.

REQ.19.3.6	Measurement and Monitoring

REQ.19.3.6.1	Measurement and Monitoring Parameters and Variables Requirements:  Measurement and monitoring involve the collection of data of various types from equipment, measures and practices.   Monitoring parameters and variables should be used in the calculation of the energy savings and Demand reductions.

REQ.19.3.6.1.1 All measured monitoring parameters and variables used in calculation of the energy savings and Demand reductions should be documented.

REQ.19.3.6.1.2 All measured monitoring parameters and variables used in the calculation of the energy savings and Demand reductions should be applicable to the category of equipment, measure or practice, including but not limited to: heating ventilating and air conditioning (HVAC) equipment, HVAC controls, building envelope, interior/exterior lighting, major electric consuming equipment and weather sensitive loads.

REQ.19.3.6.2	Monitoring Frequency and Duration:  Monitoring frequency and duration of sampling should be managed so as to reduce measurement error and biases. It should help ensure accurate representation of the energy savings and Demand reductions during the performance hours or minimum reporting interval, whichever is less, consistent with statistical significance as called for in REQ.19.3.4

REQ.19.3.7      Measurement Equipment Specification

REQ.19.3.7.1 Meter accuracy should meet or exceed industry standards or as specified by the Applicable Regulatory Authority.

REQ.19.3.7.2 Meters and other equipment should meet or exceed industry standards equivalent to ANSI C12 or as specified by the Applicable Regulatory Authority.

REQ.19.3.7.3 The meter clock / time accuracy should meet or exceed industry standards equivalent to ANSI C12 or as specified by the Applicable Regulatory Authority.

REQ.19.3.8	Data Validation

All measured data used in the energy savings and Demand reduction calculations should be validated using standard Validating, Editing and Estimation (VEE) procedures.  The following checks on any interval data from an individual facility or energy consuming equipment should be made:

REQ.19.3.8.1	Time Check:  Validate that the measurement devices time clock is within ± two minutes of the true time as defined by NIST.

REQ.19.3.8.2	Sum Check:  Validate that the difference between the sum of the values recorded over the intervals and the value recorded by the meter over the same time-period is within ± 2%. This check may be performed on either usage or pulse data, provided the data scaling is consistent throughout the period.

REQ.19.3.8.3	High/Low Check:  Establish minimum and maximum expected values for each parameter, project, facility, or measure.  The minimum and maximum values should be based on equipment ratings or historical equipment and/or facility usage data.  All interval data that is greater than the maximum expected value or less than the minimum expected value should be identified. Any such interval data should be deemed to fail validation.

REQ.19.3.8.4	Zero Value Check:  Identify any interval data with a value equal to zero.  Verification of whether the zero value is the correct value for that interval should be made.  If it is determined that the zero value is incorrect, substitution of a corrected or estimated non-zero value for the zero value may be allowed.  A zero value should not be substituted for missing interval data.

REQ.19.3.8.5	Identification of Estimated Data:  All estimated data used in the energy savings and Demand reduction calculations, as well as the methodology used to develop the estimate, should be identified.

REQ.19.3.8.6	Identification of Data Classifications:  All data that have passed VEE and used in the energy savings and Demand reduction calculations should be classified as (i) actual data, (ii) estimated data or (iii) missing data.  The data classification should be stored along with the data values in a data retention and management system consistent with the Governing Documents.

REQ.19.3.8.7 The method of Validating, Editing and Estimation (VEE) should conform to an accepted methodology (such as the guidelines published in the current edition of the Edison Electric Institute’s Uniform Business Practices for Unbundled Electricity Metering), and should be specified in the Governing Documents.

REQ.19.4	Models – Reserved

4.  SUPPORTING DOCUMENTATION

a.  Description of Request:

2012 Retail Annual Plan Item No. 3(b) “Develop business practice standards used to measure and verify reductions in energy and demand from energy efficiency in wholesale and retail markets. This includes developing business practices to measure and verify energy reductions for energy efficiency or a stand-alone Energy Efficiency Portfolio Standard.”

b.  Description of Recommendation:

This document contains the Model Business Practices for the Measurement & Verification (“M&V”) of Energy Efficiency programs.  These Model Business Practices are applicable to the M&V of electrical Energy (kWh) and Demand (kW) impacts, also referred to as reductions or savings, of Energy Efficiency programs offered to Retail Customers.

# As part of the development of these Model Business Practices, the methodological options were developed using references such as the International Performance Measurement and Verification Protocol (IPMVP) and US DOE documents.  For additional information, refer to: IPMVP Concepts and Options for Determining Energy and Water Savings, Volume I, 2012: English  (http://www.evo-world.org) and US DOE guidance documents (http://www1.eere.energy.gov/seeaction/evaluation.html)

c.  Business Purpose:

See above

d.  Commentary/Rationale of Subcommittee(s)/Task Force(s):

These Model Business Practices were discussed by the Retail Demand Response Work Group during the following conference calls and/or face-to-face meetings:

January 27, 2010

February 23, 2010

April 15, 2010

July 1, 2010

July 22, 2010

August 12, 2010

September 2, 2010

September 24, 2010

September 29, 2010

October 15, 2010

November 5, 2010

November 19, 2010

February 7, 2011

February 28, 2011

March 28, 2011

April 8, 2011

April 29, 2011

July 18, 2011

August 8, 2011

August 22, 2011

November 7, 2011

November 30, 2011

December 21, 2011

January 26, 2012

March 7, 2012

April 19, 2011

The Meeting Notes of all Retail Demand Response Work Group meetings and conference calls as well as all work papers are posted on the NAESB REQ Demand Side Management and Energy Efficiency (DSM-EE) web page.

On its April 19, 2012 Retail Demand Response Work Group conference call, these Model Business Practices were voted out of the Work Group by a vote of 6 In Favor, 0 Opposed, and 0 Abstentions.  The Model Business Practices were presented to the DSM-EE Subcommittee on May 30, 2012 and voted out of Subcommittee without opposition by the majority of those present, but with the abstentions by the following:

Steve Krommer

Rodney Sobin

Julie Michaels

Kevin Warren

Mark Smith