- 1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| X | Accept as requested | X | Change to Existing Practice |
| --- | --- | --- | --- |
|  | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

- 2.  TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
| X | Initiation | X | Initiation |
|  | Modification |  | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
| X | Principle | X | Principle |
| X | Definition | X | Definition |
| X | Business Practice Standard | X | Business Practice Standard |
| X | Document | X | Document |
| X | Data Element | X | Data Element |
| X | Code Value | X | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
| X | Business Process Documentation | X | Business Process Documentation |

- 3.  RECOMMENDATION

- SUMMARY:

The Joint Retail Electric (REQ) and Wholesale Electric (WEQ) Quadrants’ PAP 10 Smart Grid Subcommittee submit this Recommendation for 2010 Retail Annual Plan Item No. 9d and WEQ Annual Plan Item No. 6d – Business Practices and Information Models to support Priority Action Plan 10, “Standardized Energy Usage Information,” based on the Tiger Team Report issued on June 22, 2010 by the NIST SGIP PAP10 Committee.

In initiating this standards development, NAESB agreed to by year-end 2010 develop an energy use information model standard defining a common data format that may be used when information is communicated between utilities, third parties and energy use customers, via customer devices and/or third party energy services providers.  The energy usage information model standard will enable the exchange of detailed energy information in a consistent format for use by customers, utilities, service providers, consumer devices, and energy applications.  Doing so will let Retail Customers track their power use and help them manage energy consumption and cost.  Without a standardized format for representing energy use data, a variety of approaches could emerge, leading to incompatibilities among energy management products and services.  It is planned that the American Society of Heating, Refrigerating and AC Engineers (ASHRAE) will extend the NAESB standard to create a facilities data model providing additional energy use data elements for facility energy management including buildings.

- RECOMMENDED STANDARDS:

# REQ.18	CUSTOMER ENERGY USAGE INFORMATION COMMUNICATION

- EXECUTIVE SUMMARY

This standard establishes the model business practices for Retail Customer energy usage information communication.  Specifically, these model business practices establish  a data model for energy usage information.  The standard does not limit the form or function of the data model and is inclusive, but not limited to, information  that may be communicated in a consistent format among a variety of Entities, including but not limited to Distribution Companies, energy service providers, meter-reading entities, and Retail Customers.  Such communication may occur via multiple systems and devices.  Establishment of this energy usage information model will standardize a common data format which may be used when information is communicated between utilities, third parties and Retail Customers which may aid Retail Customers in tracking and managing their energy use.

The model business practices in this standard do not require that wholesale electricity markets administered by system operators adopt this model since system operators generally do not maintain or have access to the system of record for individual Retail Customer energy usage information and load data or individual Retail Customer forecasted usage and load data.  These model business practices are not intended to replace applicable Governing Documents, and in the event of a conflict, the latter documents shall have precedence over these standards. Without limiting the foregoing, these model business practices are only applicable to the extent the information covered by this model is collected, managed or communicated pursuant to the applicable Governing Documents.Retail Customer energy usage information communication encompasses a variety of interactions between Distribution Companies, Retail Customers and energy services providers.  In a business environment where best practices are voluntary, model business practices such as those in this document may be applied within the context of regulatory or other market requirements and agreements.

- INTRODUCTION

The North American Energy Standards Board (NAESB) is a voluntary, non-profit organization comprised of members from all aspects of the natural gas and electric industries.  Within NAESB, the Retail Electric Quadrant (REQ) and the Retail Gas Quadrant (RGQ) focus on issues impacting the retail sale of energy to end-use Retail Customers.  REQ / RGQ model business practices are intended to provide guidance to retail energy market participants not limited to: Distribution Companies, energy Suppliers, and energy service providers involved in providing competitive energy service to end-use Retail Customers.  The focus of these model business practices is the representation of Retail Customer energy usage information.  The scope of the energy usage information model is not intended to characterize the data information model for communication of  billing information.

These model business practices are voluntary and do not address policy issues that are the subject of state legislation or regulatory decisions.  These model business practices have been adopted with the realization that as the industry evolves, additional and amended model business practices may be necessary.  Any industry participant seeking additional or amended model business practices (including principles, definitions, data elements, process descriptions, and technical implementation instructions) should submit a request to the NAESB office, detailing the change, so that the appropriate process may take place to amend the model business practice.

- BUSINESS PROCESSES AND PRACTICES

Overview

This standard establishes the model business practices for Retail Customer energy usage information communication.  Specifically, these model business practices establish  a data model for energy usage information.  The standard does not limit the form or function of the data model and is inclusive, but not limited to, information   that may be communicated in a consistent format among a variety of Entities, including but not limited to Distribution Companies, energy service providers, meter-reading entities, and Retail Customers.  Such communication may occur via multiple systems and devices.  Establishment of this energy usage information model will standardize a common data format which may be used when information is communicated between utilities, third parties and energy use customers which may aid Retail Customers in tracking and managing their energy use.

The business processes and practices described below are not presently applicable to wholesale markets because wholesale markets do not generally communicate directly with Retail Customers and are not the system of record for individual Retail Customer energy usage information or individual Retail Customer load forecast. The energy usage information model and these model business processes and practices  are not required of system operators.  As the energy usage information model and these business processes and practices evolve, system operators may determine that use of the energy usage information model in these model business practices   can be applied to other information.  However, such use is not intended to replace or supplant applicable Governing Documents. Without limiting the foregoing, these model business practices are only applicable to the extent the information covered by this model is collected, managed or communicated pursuant to the applicable  Governing Documents.

## REQ.18.1	Principles

### REQ.18.1.1	Overall Principles

#### These model business practices provide an energy usage information model, defining a collection of structured energy usage information elements that may be needed to allow customer management of their energy usage and costs, including environmental impacts.

#### The energy usage information model is specified in UML, as a syntax neutral notation, so that it may be used within exchange protocols using a variety of specific representation syntax and exchange mechanisms, specified outside this standard.

#### The recommended use of the energy usage information model is in implementation specifications exposing Customer energy usage information. Specifications that conform to the model shall contain equivalent required and optional classes and attributes, thus resulting in straightforward (preferably lossless) transformations between conformant specifications.

#### Neither the energy usage information model, nor these model business practices establish or govern ownership or any other rights in any information or data; as such ownership and other rights are subject to and governed by the Governing Documents and/or applicable laws and regulations.

#### Neither the energy usage information model nor these Model Business Practices create any requirement to collect, manage or communicate any information.

#### While this standard defines a data model to be used when energy usage information is communicated, the Governing Documents determine the ownership of the data, the access to the data, what systems and hardware are required to comply with providing this information, and how it is paid for. There are no assumed or implied regulations in this standard.

#### These principles are deleted as they referred to Appendix A which has also been deleted. Note: We support substituting a statement that points to the PAP 10 charter  – the text of the abstract, description, and objectives from the NIST Framework Release 1.0 as a document that formed the basis of this standard.

## Energy Usage Abbreviations, Acronyms and Definition of Terms

### Business Definitions Note: We should look to the NASEB glossary committee to provide direction on how we treat definitions and then attempt to coordinate across all PAPs definitions and terminology wherever possible.

#### Applicable Regulatory Authority

The state regulatory agency or other governing body that provides oversight, policy guidance, and direction to any parties involved in the process of providing energy to Retail Customers through regulations and orders.

#### Retail Customer

Any Entity that takes gas and/or electric service for its own consumption.

#### Distribution Company

A regulated Entity which provides distribution services and may provide energy and/or transmission/transportation services in a given area.

#### Entity

A person or organization with sufficient legal standing to enter into a contract or arrangement with another such person or organization (as such legal standing may be determined by those parties) for the purpose of conducting and/or coordinating energy transactions.

#### Governing Documents

Documents that govern the interactions among parties, including but not limited to: regulatory documents (e.g. tariffs, rules, regulations), contractual agreements, and Distribution Company Operational Manuals.

#### Market Participant

A party engaged in the process of providing competitive retail energy to end-use Customers including but not limited to the Distribution Company, the Supplier, the Registration Agent, the settlement agent, and the meter reading Entity.

#### Supplier

Persons engaged in the competitive sale of energy to end-users.

### Technical Definitions

This section contains technical terms and abbreviations used in this recommendation.

#### Fine Grained

Characterized by abundant use of detail or thoroughness of treatment.

#### Operations

### One of the seven domains identified in the NIST Framework and Roadmap, defined there as “The managers of the movement of electricity”. This could apply to operators of equipment within any of the other domains. Abbreviations and Acronyms

| Abbreviation / Acronym | Meaning |
| --- | --- |
| UML | Unified Modeling Language |
| EMS | Energy Management System |
| ESI | Energy Services Interface |
| PEV | Plug-in Electric Vehicle |
| EISA | Energy Independence and Security Act |
| DOE | Department of Energy |
| HAN | Home Area Network |

## Energy Usage Information Model Business Practices

### Introduction

The focus of these model business practices is the representation of energy usage information. As defined in [PAP10 Requirements] the energy usage information includes historic, present, and future projected usage and load together with the time period(s) for that information.

These model business practices draw on actors and use cases defined by the following groups:

- Energy Information Standards Alliance (the EIS Alliance) [EIS]

- NAESB Survey and Consolidation of PAP10 Use Cases [NAESB PAP10]

- UCAIug OpenADE [ADE]

- ZigBee/Home Plug Smart Energy Profile 2.0 Market Requirements [SEP MRD]

The relevant use cases are summarized as follows:

#### The energy service provider and/or Distribution Company communicate historic and present energy usage information and load information to the Retail Customer or facility.

#### The Retail Customer or facility communicates future projected usage and load information to the energy service provider, Distribution Company, or grid Operations.

#### The energy service provider and/or Utility communicates their projection of usage and load to the Retail Customer or facility.

#### Devices within a facility communicate their present and future projected usage and load to controllers or facility EMS for aggregation and to be a component of facility aggregated future projected usage and load.

#### Devices, business processes, EMS, ESI, and other functional units within the facility communicate usage and load information amongst themselves.

#### These model business practices are limited to the seed specification which shall be usable by others to build standards and/or specification for exchange of energy usage information and load information appropriate to their needs without overly constraining those uses or including information that is not required in all implementations of specifications for exchanging load and usage.

### Energy Usage Information Model Format and Use

The energy usage information model is developed using a UML modeling tool.  The model classes, attributes, types and descriptions are included in REQ 18.4.  The model is made available as XMI, which is the standard XML import/export format for UML.  The model is exported as HTML, and made available as a downloadable archive viewable with a web browser.

### Energy Usage Information Model Technical Considerations

#### The energy usage information model will be used as the basis for smart grid interfaces exchanging customer usage information between energy services providers, consumers, and others.

#### The energy usage information model permits schemas to be generated from it, using XML Schema Definition Language (XSD), and other format specification languages. Profiles may be constructed from the energy usage information model for this purpose.

#### Implementations may include all or a subset of the elements defined in the model, possibly using a profile of the model.

#### The informative example XSD shall conform to Naming and Design Rules best practices as described by IEC 62361-100, Naming and Design Rules for CIM Profiles to XML Schema Mapping.

#### The model facilitates multiple information exchange standards to be used, including REST and Web Services. The specifics are left to implementing specifications to define.

#### Though there may be elements useful for transfer of security-related information elements in the model, the specific details related to how to protect sensitive information, and how to authorize specific roles or identities to have access are not defined in this recommendation.

### Conformance

#### A conformant specification that refines or extends this standard shall produce information for exchange that can be transformed algorithmically (that is based on the standard alone) into a form that can be validated through the method described in REQ.18.3.4.2.

This requirement means that various formats for representation and exchange and various subsets and potentially supersets of information content are envisioned based on this standard. The use of the informative example schema is not to impose its direct use in message validation. Rather, its use is intended to facilitate verification of conformance to the information model with respect to message content and semantics without imposing constraints on specific message payload schemas and data element representations. Some representations are anticipated to be entirely binary in nature. Others will trade off strings for integer representations of information contents.  Regardless of the means, the information should be convertible to be testable as described herein.

#### A specification that claims conformance to this standard shall describe and define an automatable transformation between that specification’s model to and from this UML Model, including indicating attributes used and not used.

#### Conformant specifications shall map corresponding model components to and from at least the following required core model attributes as exchanged between data provider and data consumer in their defined messages:

- One or more measurement or summary containers: IntervalReading, Reading, PowerQualitySummary, UsageSummary

- At least two of the following attributes, for each IntervalReading: timeStamp, endTimeStamp, duration

- value (the value of the measurement, from IntervalReading or Reading)

- ReadingType – ID, defaultQuality, direction, kind, multiplier, name, unit

- Association to ReadingType for each measurement (IntervalReading or Reading) (exists in model through MeterReading)

- Measurement source / location – ServiceDeliveryPoint.ID or MeterAsset.ID and association to measurements or summary

## Energy Usage Information Model

The energy usage information model herein is organized consistent with several related and well established models including the IEC TC57 Common Information Model [IEC 61968 Part 9], ZigBee Smart Energy Profile 2.0 [SEP2.0],that defined by the Energy Information Standards Alliance [EIS Alliance], and Open Automated Data Exchange [OpenADE]. New classes and attributes identified in the model in this standard will be proposed to IEC TC57 for extension of a future release of the CIM.  The energy usage information model, where possible, uses classes, information elements and attribute names drawn from the CIM and the cited references.

The starting point for the energy usage information model is the ServiceDeliveryPoint. ServiceDeliveryPoints identifies key references for the information set optionally including identification of the customer, the location, and the physical asset. ServiceDeliveryPoint is associated in turn with zero or more MeterReadings. A MeterReading composes information about a particular measurement such as kWH or kW. A MeterReading has a ReadingType which describes the nature of the measurement including it units of measure, and zero or more IntervalReadings or Readings and associated quality information. ServiceDeliveryPoint may also be associated with summary information on load and usage, and optionally, power quality.  For applications requiring third party access to this information, additional classes are identified to facilitate associating customer and customer agreement information with the measurements available at a ServiceDeliveryPoint.

To find the usage or load in a particular interval, identify the appropriate ServiceDeliveryPoint, select the MeterReading of interest (measurement) and then select the IntervalReading or Reading associated with the given interval.

The energy usage information model includes many optional components. The complete set of information expressable using the model satisfies a wide range of applicability requirements identified by industry. Users of this standard may optionally take advantage of these extended definitions based on need without requiring them. Applications built on the the energy usage information model may elect which optional components to present. However, clients of this information can be expected to recognize all components provided in the application.

Section REQ.18.3.4.3 idenitifies the set of core model elements that shall be supported by specifications claiming conformance to this standard. The following class diagram illustrates a view of this core of the energy usage information model:

Figure 1: Energy Usage Information Model – Minimal View

The full energy usage information model, Illustrated in Figure 2,  forms the basis of the required standard.

Figure 2: Full Energy Usage Information Model

### Energy Usage Information Model Details

The following sections contain the classes and attributes defined in the model, along with their descriptions. Elements tagged with <<enumeration>> define the valid values for an enumerated data type.

#### Customer

Organisation receiving services from ServiceSupplier.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |
| name | string | Name of an attribute. |

#### CustomerAgreement

Agreement between the Customer and the ServiceSupplier to pay for service at a specific ServiceLocation. It specifies the type of service provided at the ServiceLocation and the rate, which determines how the customer bill is calculated.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |

#### DateTimeInterval

Interval of date and time.

| Name | Type | Description |
| --- | --- | --- |
| duration | int | The duration of this interval, in seconds |
| end | dateTime | Date and time that this interval ended. |
| start | dateTime | Date and time that this interval started. |

#### EnergyUsageInformation

A collection of customer energy usage information

#### IntervalReading

Data captured over a specific interval of time. If not specified, the duration is the intervalLength of the associated ReadingType, where the full definition of the units of measure is located.

| Name | Type | Description |
| --- | --- | --- |
| cost | float | The cost associated with this reading for this interval. |
| duration | int | The duration of the interval, in seconds. |
| endTimeStamp | dateTime | End interval timestamp |
| timeStamp | dateTime | The start date and time of an interval reading |
| value | float | The value of the reading for this interval in the unit of measure defined by the associated ReadingType. |

#### MeterAsset

Physical asset that performs the metering role of the ServiceDeliveryPoint. Used for measuring consumption and detection of events.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |
| name | string | Meter name |

#### MeterReading

Set of values obtained from the meter.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |
| valuesInterval | DateTimeInterval | Interval in date time (start & end) |

#### PowerQualitySummary

A summary of power quality events. Most of the power quality data described in the PowerQualitySummary table is currently not collected or stored in meters commonly in use today.  Smart meters may be able to flag high and low voltages but are not equipped to provide a cycle by cycle analysis of transients, harmonics, etc. Additional clarity is needed here.

| Name | Type | Description |
| --- | --- | --- |
| frequencyVariations | int | Number of frequency variations |
| interruptions | int | Number of interruptions |
| sags | int | Number of sags |
| summaryInterval | DateTimeInterval | Interval of summary period |
| swells | int | Number of swells |
| transients | int | Number of transients |
| voltageImbalances | int | Number of voltage imbalances |

#### QualityOfReading  «enumeration»

List of codes indicating the quality of the reading

| Name | Type | Description |
| --- | --- | --- |
| estimated |  |  |
| forecast |  |  |
| mixed |  |  |
| raw |  |  |
| validated |  |  |
| normalizedForWeather |  |  |
| other |  |  |

#### Reading

Specific value measured by a meter or other asset. Each Reading is associated with a specific ReadingType and was taken at the specified time.

| Name | Type | Description |
| --- | --- | --- |
| cost | float | Cost in a currency |
| timeStamp | dateTime | The date and time of a reading |
| value | float | The value of the reading in the unit of measure defined by the associated ReadingType. |

#### ReadingDirection  «enumeration»

| Name | Type | Description |
| --- | --- | --- |
| delivered |  |  |
| received |  |  |
| net |  |  |

#### ReadingKind  «enumeration»

Kind of reading.

| Name | Type | Description |
| --- | --- | --- |
| current |  |  |
| currentAngle |  |  |
| date |  |  |
| demand |  |  |
| energy |  |  |
| other |  |  |
| phaseAngle |  |  |
| power |  |  |
| powerFactor |  |  |
| pressure |  |  |
| time |  |  |
| voltage |  |  |
| voltageAngle |  |  |
| volume |  |  |
| carbonDioxide |  |  |
| NOx |  |  |
| carbon |  |  |
| SO2 |  |  |
| methane |  |  |
| HCH |  |  |
| perfluorocarbons |  |  |
| sulfurHexafluoride |  |  |
| phasorPower |  |  |
| quantityPowerQ60 |  |  |
| quantityPowerQ45 |  |  |
| distortionPower |  |  |
| voltageRMS |  |  |
| voltageAverage |  |  |
| currentRMS |  |  |
| currentAverage |  |  |
| voltageTHD |  |  |
| currentTHD |  |  |

#### ReadingQuality

Quality of a specific reading value or interval reading value. If not present, then is assumed to be the defaultQuality indicated by the ReadingType.

| Name | Type | Description |
| --- | --- | --- |
| quality | QualityOfReading | Quality, to be specified if different than defaultQuality |

#### ReadingType

Type of data conveyed by a specific Reading.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |
| aliasName | string | The aliasName is free text human readable name of the object alternative to IdentifiedObject.name. It may be non unique and might not correlate to a naming hierarchy. |
| channelNumber | integer | Logical positioning of this measurement data. |
| defaultQuality | QualityOfReading | Characteristics of a data value conveyed by a specific Reading, which allow an application to understand how a specific Reading is to be interpreted. |
| description | string | The description is a free human readable text describing or naming the object. It may be non unique and may not correlate to a naming hierarchy. |
| direction | ReadingDirection | Specifies the direction of flow of the measurement. |
| intervalLength | int | (if incremental reading value) Length of increment interval, in seconds. Interval duration specified at the IntervalReading overrides this default. |
| kind | ReadingKind | Kind of reading. |
| multiplier | UnitMultiplier | Multiplier for 'unit'. |
| name | string | Name of an attribute. |
| unit | UnitSymbol | Unit in symbol |

#### ServiceDeliveryPoint

Logical point on the network where the ownership of the service changes hands. It is one of potentially many service points within a ServiceLocation, delivering service in accordance with a CustomerAgreement. Used at the place where a meter may be installed.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |
| description | string | The description is a free human readable text describing or naming the object. It may be non unique and might not correlate to a naming hierarchy. |
| lat | float | Latitude of the location |
| long | float | Longitude of the location |
| name | string | Name of an attribute. |

#### SummaryMeasurement

An aggregated summary measurement reading.

| Name | Type | Description |
| --- | --- | --- |
| multiplier | UnitMultiplier | The multiplier part of the unit of measure, e.g. "kilo" (k) |
| timeStamp | dateTime | The date and time (if needed) of the summary measurement. |
| unit | UnitSymbol | The units of the reading, e.g. "Wh" |
| value | float | The value of the summary measurement. |

#### TariffProfile

A schedule of charges; structure associated with Tariff that allows the definition of complex tarif structures such as step and time of use when used in conjunction with TimeTariffInterval and Charge. Inherited 'status.value' is defined in the context of the utility's business rules, for example: active, inactive, etc.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |
| name | string | Name of an attribute. |

#### UnitMultiplier  «enumeration»

The unit multipliers defined for the CIM

| Name | Type | Description |
| --- | --- | --- |
| c |  | Centi 10**-2 |
| d |  | Deci 10**-1 |
| G |  | Giga 10**9 |
| k |  | Kilo 10**3 |
| m |  | Milli 10**-3 |
| M |  | Mega 10**6 |
| micro |  | Micro 10**-6 |
| n |  | Nano 10**-9 |
| none |  |  |
| p |  | Pico 10**-12 |
| T |  | Tera 10**12 |

#### UnitSymbol  «enumeration»

The units defiend for usage in the CIM

| Name | Type | Description |
| --- | --- | --- |
| A |  | Current in ampere |
| deg |  | Plane angle in degrees |
| F |  | Capacitance in farad |
| g |  | Mass in gram |
| h |  | Time in hours |
| H |  | Inductance in henry |
| Hz |  | Frequency in hertz |
| Hz-1 |  | per Hertz |
| J |  | Energy in joule |
| J/s |  | Joule per second |
| kg/J |  | Mass per energy |
| m |  | Length in meter |
| m2 |  | Area in square meters |
| m3 |  | Volume in cubic meters |
| min |  | Time in minutes |
| N |  | Force in newton |
| none |  | Dimension less quantity, e.g. count, per unit, etc. |
| C |  | Relative temperature in degrees Celsius |
| ohm |  | Resistance in ohm |
| Pa |  | Pressure in pascal (n/m2) |
| rad |  | Plane angle in radians |
| S |  | Conductance in siemens |
| s |  | Time in seconds |
| s-1 |  | per second |
| V |  | Voltage in volt |
| V/VAr |  | Volt per volt ampere reactive |
| VA |  | Apparent power in volt ampere |
| VAh |  | Apparent energy in volt ampere hours |
| VAr |  | Reactive power in volt ampere reactive |
| VArh |  | Reactive energy in volt ampere reactive hours |
| W |  | Active power in watt |
| W/Hz |  | Watt per hertz |
| W/s |  | Watt per second |
| Wh |  | Real energy in Watt hours |
| pct |  | Percent |

#### UsageSummary

Summary of usage for a billing period

| Name | Type | Description |
| --- | --- | --- |
| billingPeriod | DateTimeInterval | The billing period to which the included measurements apply |
| billLastPeriod | float | The amount of the bill for the previous period |
| billToDate | float | The bill amount related to the billing period as of the date received |
| costAdditionalLastPeriod | float | Additional charges from the last billing period |
| currency | string | The ISO 4217 code indicating the currency applicable to the bill amounts in the summary. See list at http://www.unece.org/cefact/recommendations/rec09/rec09_ecetrd203.pdf |
| currentBillingPeriodOverAllConsumption | SummaryMeasurement | The total consumption for the billing period |
| currentDayLastYearNetConsumption | SummaryMeasurement | The amount of energy consumed one year ago |
| currentDayNetConsumption | SummaryMeasurement | Net consumption for the current day |
| currentDayOverallConsumption | SummaryMeasurement | Overall energy consumption for the current day |
| peakDemand | SummaryMeasurement | Peak demand recorded for the current period |
| previousDayLastYearOverallConsumption | SummaryMeasurement | The amount of energy consumed on the previous day one year ago |
| previousDayNetConsumption | SummaryMeasurement | Net consumption for the previous day |
| previousDayOverallConsumption | SummaryMeasurement | The total consumption for the previous day |
| qualityOfReading | QualityOfReading | Indication of the quality of the summary readings |
| ratchetDemand | SummaryMeasurement | The current ratchet demand value for the ratchet demand period |
| ratchetDemandPeriod | DateTimeInterval | The period over which the ratchet demand applies |

#### ServiceKind  «enumeration»

Kind of service.

| Name | Type | Description |
| --- | --- | --- |
| electricity |  |  |
| gas |  |  |
| water |  |  |
| time |  |  |
| heat |  | Includes hot water and steam |
| refuse |  |  |
| sewerage |  |  |
| rates |  |  |
| tvLicence |  |  |
| internet |  |  |
| other |  |  |
| cold |  | Includes chilled water and ice |

#### CustomerAuthorisation

Holds an authorisation for access to specific user-private data granted to a 3rd Party service provider. [OpenADE Extension - Specialization of "Agreement"]

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Unique identifier for this authorisation |
| validityInterval | DateTimeInterval | Date and time interval this agreement is valid (from going into effect to termination). |

#### ServiceCategory

Category of service provided to the customer.

| Name | Type | Description |
| --- | --- | --- |
| kind | ServiceKind | Kind of service. |

#### SupplierKind  «enumeration»

Kind of supplier.

| Name | Type | Description |
| --- | --- | --- |
| district |  |  |
| intermediary |  |  |
| local |  |  |
| microgrid |  |  |
| other |  |  |
| retailer |  |  |
| utility |  |  |

#### ServiceSupplier

Organisation that provides services to Customers.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Unique identifier of this service supplier. |
| kind | SupplierKind | Kind of supplier. |
| name | string | The human-readable name for the service supplier. |

# This section is recommended to be deleted since these requirements were not subject to vetting via an ANSI accredited process. Also  The National Technology Transfer and Advancement Act of 1995 and a related document OMB Circular A-119 sets forth the principles for adopting consensus based standards.

The additional considerations are recommended to be deleted since they have been treated largely like the ones handed down from the SGIP and are out of scope, regulatory in nature, optional, not needed or could be part of a follow – on specification that emerges from this work.

Exelon would support a statement in the Principles section that pointed to the PAP10 charter charter – the text of the abstract, description, and objectives from the NIST Framework Release 1.0 as the basis for this standard development.

#### 4.  SUPPORTING DOCUMENTATION

- Description of Request:

- Description of Recommendation:

- Business Purpose:

- Commentary/Rationale of Subcommittee(s)/Task Force(s):

NAESB Process for Standards Development - http://www.naesb.org//misc/naesb_process_for_standards_dev.doc

See e.g. EIS Alliance Use Cases v2 including UC-9, UC-11, UC-12, UC-14, UC-15, ZBHP_SE_MRD 3.1 "HAN Device Information Retrieval Request", and OpenADE use case “Publication”.

See e.g. EIS Alliance Use Cases v2 including UC-3, UC-11, UC-14, UC-15, and OpenADE use case “Publication”

Needs refs. This is an addition to EIS and OpenADE.

See e.g. EIS Alliance Use Cases v2 including UC-1, UC-2, UC-3, UC-8, UC-9.

See e.g. EIS Alliance Use Cases v2 including UC-1, UC-2, UC-3

http://www.nist.gov/public_affairs/releases/upload/smartgrid_interoperability_final.pdf