- 1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| X | Accept as requested | X | Change to Existing Practice |
| --- | --- | --- | --- |
|  | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

- 2.  TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
| X | Initiation | X | Initiation |
|  | Modification |  | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
| X | Principle | X | Principle |
| X | Definition | X | Definition |
| X | Business Practice Standard | X | Business Practice Standard |
| X | Document | X | Document |
| X | Data Element | X | Data Element |
| X | Code Value | X | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
| X | Business Process Documentation | X | Business Process Documentation |

- 3.  RECOMMENDATION

- SUMMARY:

The Joint Retail Electric (REQ) and Wholesale Electric (WEQ) Quadrants’ PAP 10 Smart Grid Subcommittee submit this Recommendation for 2010 Retail Annual Plan Item No. 9d and 2010 WEQ Annual Plan Item No. 6d – Business Practices and Information Models to support Priority Action Plan 10, “Standardized Energy Usage Information,” based on the Tiger Team Report issued on June 7, 2010 by the NIST SGIP PAP10 Committee.

In initiating this standards development, NAESB agreed to by year-end 2010 develop an energy use information model standard defining a common data format that may be used when information is communicated between utilities, third parties and energy use customers, via customer devices and/or third party energy services providers.  The energy usage information model standard will enable the exchange of detailed energy information in a consistent format for use by customers, utilities, service providers, consumer devices, and energy applications.  Doing so will let Customers track their power use and help them manage energy consumption and cost.  Without a standardized format for representing energy use data, a variety of approaches could emerge, leading to incompatibilities among energy management products and services.  It is planned that the American Society of Heating, Refrigerating and AC Engineers (ASHRAE) will extend the NAESB standard to create a facilities information model providing additional energy use data elements for facility energy management including buildings.

- RECOMMENDED STANDARDS:

# REQ.18	CUSTOMER ENERGY USAGE INFORMATION COMMUNICATION

- EXECUTIVE SUMMARY

This document establishes  the Model Business Practices for Retail Customer energy usage information communication.  Specifically, these Model Business Practices establish  an information model for energy usage information.  These Model Business Practices do not limit the form or function of the information model and is exemplary, but not exclusive, of the information   that may be communicated in a consistent format among a variety of Entities, including, but not limited to, Distribution Companies, energy service providers, meter-reading entities, and Retail Customers.  Such communication may occur via multiple systems and devices.  Establishment of this energy usage information model will standardize a common data format which may be used when information is communicated between Distribution Companies, third parties and energy use Retail Customers which may aid Retail Customers in tracking and managing their energy use.

These Model Business Practices do not require that wholesale electricity markets administered by system operators adopt this energy usage information model since system operators generally do not maintain or have access to the system of record for individual Retail Customer energy usage information and load data or individual Retail Customer forecasted usage and load data.  These Model Business Practices are not intended to replace applicable Governing Documents, and, in the event of a conflict, the latter documents shall have precedence over these Model Business Practices. Without limiting the foregoing, these Model Business Practices are only applicable to the extent the information covered by this energy usage information model is collected, managed or communicated pursuant to the applicable Governing Documents.  Retail Customer energy usage information communication encompasses a variety of interactions between Distribution Companies, Retail Customers and energy services providers.  In a business environment where best practices are voluntary, these Model Business Practices may be applied within the context of regulatory or other market requirements and agreements.

- INTRODUCTION

The North American Energy Standards Board (NAESB) is a voluntary, non-profit organization comprised of members from all aspects of the natural gas and electric industries.  Within NAESB, the Retail Electric Quadrant (REQ) and the Retail Gas Quadrant (RGQ) focus on issues impacting the retail sale of energy to end-use Retail Customers.  REQ / RGQ Model Business Practices are intended to provide guidance to Market Participants. The focus of these Model Business Practices is the representation of Retail Customer energy usage information.  The scope of the energy usage information model is not intended to characterize the data information model for communication of  billing or related information.

These Model Business Practices are voluntary and do not address policy issues that are the subject of state legislation or regulatory decisions.  These Model Business Practices have been adopted with the realization that as the industry evolves, additional and amended Model Business Practices may be necessary.  Any industry participant seeking additional or amended Model Business Practices (including principles, definitions, data elements, process descriptions, and technical implementation instructions) should submit a request to the NAESB office, detailing the change, so that the appropriate process may take place to amend the Model Business Practice.

- BUSINESS PROCESSES AND PRACTICES

Overview

This Model Business Practice establishes  the Model Business Practices for Retail Customer energy usage information communication.  Specifically, these Model Business Practices establish an information model for energy usage information.  The Model Business Practice does not limit the form or function of the information model and is exemplary, but not exclusive, of the information that may be communicated in a consistent format among a variety of Entities, including, but not limited to, Distribution Companies, energy service providers, meter-reading entities, and Retail Customers.  Such communication may occur via multiple systems and devices.  Establishment of this energy usage information model will standardize a common data format which may be used when information is communicated between Distribution Companies, third parties and energy use customers which may aid Retail Customers in tracking and managing their energy use.

The business processes and practices described below are not presently applicable to wholesale markets because wholesale markets do not generally communicate directly with Retail Customers and are not the system of record for individual Retail Customer energy usage information or individual Retail Customer load forecast. The energy usage information model and these model business processes and practices are not required of system operators.  As the energy usage information model and these business processes and practices evolve, system operators may determine that use of the energy usage information model in these Model Business Practices can be applied to other information.  However, such use is not intended to replace or supplant applicable Governing Documents. Without limiting the foregoing, these Model Business Practices are only applicable to the extent the information covered by this energy usage information model is collected, managed or communicated pursuant to the applicable  Governing Documents.

## REQ.18.1	Principles

### REQ.18.1.1	Overall Principles

#### These Model Business Practices provide an energy usage information model, which defines a collection of structured energy usage information classes and attributes that may be used to enable Retail Customer management of their energy usage and costs.

#### The energy usage information model is specified in UML, as a syntax neutral notation, so that it may be used within exchange protocols using a variety of specific representation syntax and exchange mechanisms, specified outside these Model Business Practices.

#### The recommended use of the energy usage information model is in implementation specifications exposing Retail Customer energy usage information. Specifications that conform to the energy usage information model shall contain equivalent required and optional classes and attributes, thus resulting in straightforward (preferably lossless) transformations between conformant specifications.

#### Neither the energy usage information model, nor these Model Business Practices establish or govern ownership or any other rights in any information or data; such ownership and other rights are subject to and governed by the Governing Documents and/or Applicable Regulatory Authority.

#### Neither the energy usage information model nor these Model Business Practices create any requirement to collect, manage or communicate any information.

#### While these Model Business Practices define an information model which may be used when energy usage information is communicated, the Governing Documents and/or Applicable Regulatory Authority determine the ownership of the data, the access to the data, what systems and hardware are necessary to provide this information, and how it is paid for. There are no assumed or implied regulations in these Model Business Practices.

#### Appendix A describes the principles used in developing the energy usage information model and includes an explanatory verification paragraph describing how the energy usage information model satisfies each requirement. The Appendix A requirements up through Additional Requirements were provided by the Smart Grid Interoperability Panel Priority Action Plan 10 Working Group to the NAESB Priority Action Plan 10 Subcommittee.

#### Appendix A represents a series of capabilities for the expressiveness of these Model Business Practices and are specifically not intended to be requirements for the use of or on users of these Model Business Practices.

## Energy Usage Abbreviations, Acronyms and Definition of Terms

### Business Definitions

#### RXQ.00.xx		Applicable Regulatory Authority

The state regulatory agency or other governing body that provides oversight, policy guidance, and direction to any parties involved in the process of providing energy to Retail Customers through regulations and orders.

#### RXQ.00.xx		Retail Customer

Any Entity that takes gas and/or electric service for its own consumption.

#### RXQ.00.xx		Distribution Company

A regulated Entity which provides distribution services and may provide energy and/or transmission/transportation services in a given area.

#### RXQ.00.xx		Entity

A person or organization with sufficient legal standing to enter into a contract or arrangement with another such person or organization (as such legal standing may be determined by those parties) for the purpose of conducting and/or coordinating energy transactions.

#### RXQ.00.xx		Governing Documents

Documents that govern the interactions among parties, including but not limited to: applicable law, regulatory documents (e.g. tariffs, rules, regulations), contractual agreements, and Distribution Company Operational Manuals and other relevant models and procedures.

#### RXQ.00.xx		Market Participant

A Distribution Company, Supplier, Registration Agent, settlement agent, meter reading Entity or other party engaged in the process of providing competitive retail energy or another specified program to Retail Customers.

RXQ.00.xx		Model Business Practices

Electric and gas industry processes and procedures developed by interested parties representing the NAESB Retail Gas and Electric Quadrants’ segments and ratified by the NAESB Retail Gas and Electric Quadrants’ members

#### RXQ.00.xx		Supplier

Entities engaged in the competitive sale of energy to end-users.

### Technical Definitions

This section contains technical terms and abbreviations used in this recommendation.  Additional Technical Definitions are located in REQ.18.4.1.

#### Fine Grained

Characterized by abundant use of detail or thoroughness of treatment.

### REQ.18.2.3	Abbreviations and Acronyms

| Abbreviation / Acronym | Meaning |
| --- | --- |
| UML | Unified Modeling Language |
| EMS | Energy Management System |
| ESI | Energy Services Interface |
| PEV | Plug-in Electric Vehicle |
| EISA | Energy Independence and Security Act |
| DOE | Department of Energy |
| HAN | Home Area Network |
| XMI | XML Media Exchange |
| XML | Extensible Markup Language |
| HTML | HyperText Markup Language |
| XSD | XML Schema Definition Language |
| CIM | Common Information Model |
| IEC | Internationl Electrotechnical Commission |

## Energy Usage Information Model Business Practices

### Introduction

The focus of these Model Business Practices is the representation of energy usage information, which includes historic, present, and future projected usage and load together with the time period(s) for that information.

These Model Business Practices drew upon actors and use cases defined by the following groups:

- Energy Information Standards Alliance (the EIS Alliance) [EIS]

- NAESB Survey and Consolidation of PAP10 Use Cases [NAESB PAP10]

- UCAIug OpenADE [ADE]

- ZigBee/Home Plug Smart Energy Profile 2.0 Market Requirements [SEP MRD]

The relevant use cases are summarized as follows:

#### The energy service provider and/or Distribution Company communicates historic and present energy usage information and load information to the Retail Customer or facility.

#### The Retail Customer or facility communicates future projected usage and load information to the energy service provider, Distribution Company, or grid operations.

#### The energy service provider and/or Distribution Company communicates their projection of usage and load to the Retail Customer or facility.

#### Devices within a facility communicate their present and future projected usage and load to controllers or facility EMS for aggregation and to be a component of facility aggregated future projected usage and load.

#### Devices, business processes, EMS, ESI, and other functional units within the facility communicate usage and load information among themselves.

#### These Model Business Practices are limited to the seed specification which shall be usable by others to build standards, or additional Model Business Practices, and/or specification for exchange of energy usage information and load information appropriate to their needs without overly constraining those uses or including information that is not required in all implementations of specifications for exchanging load and usage.

### Energy Usage Information Model Format and Use

The energy usage information model is developed using a UML modeling tool.  The model classes, attributes, types and descriptions are included in REQ 18.4.  The model is made available as XMI, which is the standard XML import/export format for UML.  The model is exported as HTML and made available as a downloadable archive viewable with a web browser.

### Energy Usage Information Model Technical Considerations

#### The energy usage information model may be used as the basis for smart grid interfaces between parties exchanging customer energy usage information between energy services providers, consumers, and others.

#### The energy usage information model permits schemas to be generated from it using XSD and other format specification languages. Profiles may be constructed from the energy usage information model for this purpose.

#### Implementations may include all or a subset of the elements defined in the model, possibly using a profile of the energy usage information model.

#### The informative example XSD should conform to Naming and Design Rules best practices as described by IEC 62361-100, Naming and Design Rules for CIM Profiles to XML Schema Mapping.

#### The model facilitates the use of multiple information exchange standards. The specifics are left to implementing specifications to define.

#### Though there may be elements useful for the transfer of security-related information elements in the energy usage information model, the specific details related to how to protect sensitive information, and how to authorize specific roles or identities to have access are not defined in these Model Business Practices.

### Conformance

#### A specification that claims conformance and refines or extends these Model Business Practices shall produce information for exchange that can be transformed algorithmically (that is based on the Model Business Practices alone) into a form that can be validated through the method described in REQ.18.3.4.2.

This requirement means that various formats for representation and exchange and various subsets and potentially supersets of information content are envisioned based on these Model Business Practices. The use of the informative example schema is not to impose its direct use in message validation. Rather, its use is intended to facilitate verification of conformance to the energy usage information model with respect to message content and semantics without imposing constraints on specific message payload schemas and data element representations. Some representations are anticipated to be entirely binary in nature. Others will trade off strings for integer representations of information contents.  Regardless of the means, the information should be convertible to be testable as described herein.

#### A specification that claims conformance to these Model Business Practices shall describe and define an automatable transformation between that specification’s model to and from this UML model, including indicating attributes used and not used, as well as the version of the model stated in NAESB_EUI_Version.

#### A specification that claims conformance to these Model Business Practices shall map corresponding energy usage information model components to and from at least the following required core model attributes as exchanged between data provider and data consumer in their defined messages (Refer to REQ.18.4.1 for definitions of capitalized terms):

- One or more measurement or summary containers: IntervalReading, Reading, PowerQualitySummary, UsageSummary

- At least two of the following attributes, for each IntervalReading: timeStamp, endTimeStamp, duration

- The attribute “value” (the value of the measurement, from IntervalReading or Reading)

- ReadingType – ID, defaultQuality, direction, kind, multiplier, name, unit

- Association to ReadingType for each measurement (IntervalReading or Reading) (exists in model through MeterReading)

- Measurement source / location – ServiceDeliveryPoint.ID or MeterAsset.ID and association to measurements or summary

## Energy Usage Information Model

The energy usage information model herein is organized consistent with several related models, including the IEC TC57 Common Information Model [IEC 61968 Part 9], ZigBee Smart Energy Profile 2.0 [SEP2.0], that are defined by the Energy Information Standards Alliance [EIS Alliance] and Open Automated Data Exchange [OpenADE]. The energy usage information model, where possible, uses classes, information elements and attribute names drawn from the CIM and the cited references.

The starting point for the energy usage information model is the ServiceDeliveryPoint. ServiceDeliveryPoints identify key references for the information set optionally including identification of the Retail Customer, the location, and the physical asset. ServiceDeliveryPoint is associated in turn with zero or more MeterReadings. A MeterReading composes information about a particular measurement such as kWh or kW. A MeterReading has a ReadingType which describes the nature of the measurement including its units of measure, and zero or more IntervalReadings or Readings and associated quality information. ServiceDeliveryPoint may also be associated with summary information on load and usage, and optionally, power quality.  For applications requiring third party access to this information, additional classes are identified to facilitate associating Retail Customer and Retail Customer agreement information with the measurements available at a ServiceDeliveryPoint.

To find the usage or load in a particular interval, identify the appropriate ServiceDeliveryPoint, select the MeterReading of interest (measurement) and then select the IntervalReading or Reading associated with the given interval.

The energy usage information model includes many optional components. The complete set of information expressable using the energy usage information model satisfies a wide range of applicability requirements identified by the industry. Users of these Model Business Practices may optionally take advantage of these extended definitions based on need without requiring them. Applications built on the energy usage information model may elect which optional components to present. However, clients of this information can be expected to recognize all components provided in the application.

REQ.18.3.4.3 idenitifies the set of core model elements that shall be supported by specifications claiming conformance to these Model Business Practices. The following class diagram illustrates a view of this core of the energy usage information model:

Figure 1: Energy Usage Information Model – Minimal View

The full energy usage information model, illustrated in Figure 2,  forms the basis of the required Model Business Practices.  Note, some minor classes detailed in REQ.18.4.1 are omitted from the diagram to aid in readability (e.g. DateTimeInterval).

Figure 2: Full Energy Usage Information Model

### Energy Usage Information Model Details

The following sections contain the classes and attributes defined in the energy usage information model, along with their descriptions. Elements tagged with <<enumeration>> define the valid values for an enumerated data type and so do not have their own data type, and should be self explanatory, not requiring a definition.

Terms in this section may be based upon IEC 61968 classes and their descriptions.

The following sections contain the classes and attributes defined in the model, along with their descriptions.  Elements tagged with <<enumeration>> define the valid values for an enumerated data type.

#### Organisation receiving services from ServiceSupplier.

#### Agreement between the Customer and the ServiceSupplier to pay for service at a specific ServiceLocation. It specifies the type of service provided at the ServiceLocation and the rate, which determines how the customer bill is calculated.

#### Interval of date and time.

#### Data captured over a specific interval of time. If not specified, the duration is the intervalLength of the associated ReadingType, where the full definition of the units of measure is located.

#### Physical asset that performs the metering role of the ServiceDeliveryPoint. Used for measuring consumption and detection of events.

#### Set of values obtained from the meter.

#### List of codes indicating the quality of the reading

#### Specific value measured by a meter or other asset. Each Reading is associated with a specific ReadingType and was taken at the specified time.

#### Kind of reading.

#### Quality of a specific reading value or interval reading value. If not present, then is assumed to be the defaultQuality indicated by the ReadingType.

#### Type of data conveyed by a specific Reading.

#### Logical point on the network where the ownership of the service changes hands. It is one of potentially many service points within a ServiceLocation, delivering service in accordance with a CustomerAgreement. Used at the place where a meter may be installed.

#### An aggregated summary measurement reading.

#### A schedule of charges; structure associated with Tariff that allows the definition of complex tarif structures such as step and time of use when used in conjunction with TimeTariffInterval and Charge. Inherited 'status.value' is defined in the context of the utility's business rules, for example: active, inactive, etc.

#### The unit multipliers defined for the CIM

#### d for usage in the CIM

#### Summary of usage for a billing period

#### Kind of service.

#### service provider. [OpenADE Extension - Specialization of "Agreement"]

#### Category of service provided to the customer.

#### Kind of supplier.

#### Organisation that provides services to Customers.

#### AbsoluteDateTime  «Datatype»

Date and time as specified in ISO 8601.

#### Boolean  «Primitive»

A type with the value space "true" and "false".

#### Customer

Organisation receiving services from ServiceSupplier.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

#### CustomerAgreement

Agreement between the customer and the service supplier to pay for service at a specific service location. It records certain billing information about the type of service provided at the service location and is used during charge creation to determine the type of service.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

#### CustomerAuthorisation

Holds an authorisation for access to specific user-private data granted to a third party service provider.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |
| validityInterval | DateTimeInterval | Date and time interval this agreement is valid (from going into effect to termination). |

#### DateTimeInterval  «Compound»

Interval of date and time.

| Name | Type | Description |
| --- | --- | --- |
| start | AbsoluteDateTime | Date and time that this interval started. |
| end | AbsoluteDateTime | Date and time that this interval ended. |
| duration | Duration | The duration of this interval, in seconds |

#### Duration  «CIMDatatype»

An interval of time, specified in ISO 8601 compatible format.

#### ElectricPowerQualitySummary

A summary of power quality events. This information represents a summary of power quality information required by customer facility energy management systems. It is not intended to satisfy the detailed requirements of power quality monitoring. All values are as defined by measurementProtocol during the period. The standards typically also give ranges of allowed values; the information attributes are the raw measurements, not the "yes/no" determination by the various standards. See referenced standards for definition, measurement protocol  and period.

| Name | Type | Description |
| --- | --- | --- |
| flickerPlt | Float | A measurement of long term Rapid Voltage Change |
| flickerPst | Integer | A count of Rapid Voltage Change events during the summary interval period |
| harmonicVoltage | Float | A measurement of the Harmonic Voltage during the period. For DC, distortion is with respect to a signal of zero Hz. |
| longInterruptions | Integer | A count of Long Interruption events (as defined by measurementProtocol) during the summary interval period. |
| mainsVoltage | Float | A measurement of the Mains [Signaling] Voltage during the summary interval period. |
| measurementProtocol | String | A reference to the source used as the measurement protocol definition. e.g. "IEEE1519-2009", "EN50160" |
| powerFrequency | Float | A measurement of the power frequency during the summary interval period. |
| rapidVoltageChanges | Integer | A count of Rapid Voltage Change events during the summary interval period |
| shortInterruptions | Integer | A count of Short Interruption events during the summary interval period |
| summaryInterval | DateTimeInterval | Interval of summary period |
| supplyVoltageDips | Integer | A count of Supply Voltage Dip events during the summary interval period |
| supplyVoltageImbalance | Integer | A count of Supply Voltage Imbalance events during the summary interval period |
| supplyVoltageVariations | Integer | A count of Supply Voltage Variations during the summary interval period |
| tempOvervoltage | Integer | A count of Temporary Overvoltage events (as defined by measurementProtocol) during the summary interval period |

#### EndDeviceAsset

AssetContainer that performs one or more end device functions. One type of EndDeviceAsset is a MeterAsset which can perform metering, load management, connect/disconnect, accounting functions, etc. Some EndDeviceAssets, such as ones monitoring and controlling air conditioner, refrigerator, pool pumps may be connected to a MeterAsset. All EndDeviceAssets may have communication capability defined by the associated ComFunction(s). An EndDeviceAsset may be owned by a consumer, a service provider, utility or otherwise.

There may be a related end device function that identifies a sensor or control point within a metering application or communications systems (e.g., water, gas, electricity).

Some devices may use an optical port that conforms to the ANSI C12.18 standard for communications.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

#### EnergyUsageInformation

A collection of customer energy usage information. This class is a container, and has no attributes.

#### Float  «Primitive»

A floating point number. The range is unspecified and not limited.

#### IdentifiedObject

This is a root class to provide common identification for all classes needing identification and naming attributes

#### Integer  «Primitive»

An integer number. The range is unspecified and not limited.

#### IntervalBlock

Time sequence of Readings of the same ReadingType. Contained IntervalReadings may need conversion through the application of an offset and a scalar defined in associated Pending.

#### IntervalReading

Data captured over a specific interval of time. If not specified, the duration is the intervalLength of the associated ReadingType, where the full definition of the units of measure is located.

| Name | Type | Description |
| --- | --- | --- |
| cost | Float | The cost associated with this reading for this interval. |
| interval | DateTimeInterval | The time interval associated with the reading. |
| value | Float | Value of this interval reading. |

#### MeterReading

Set of values obtained from the meter.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |
| valuesInterval | DateTimeInterval | Date and time interval of the data items contained within this meter reading. |

#### Name

The Name class provides the means to define any number of human readable  names for an object. A name is not to be used for defining inter-object relationships. For inter-object relationships instead use the object identification 'mRID'.

| Name | Type | Description |
| --- | --- | --- |
| name | String | Any free text that name the object. |

#### NameType

Type of name. Possible values for attribute 'name' are implementation dependent but standard profiles may specify types. An enterprise may have multiple IT systems each having its own local name for the same object, e.g. a planning system may have different names from an EMS. An object may also have different names within the same IT system, e.g. localName and aliasName as defined in CIM version 14. Their definitions from CIM14 are

The localName is a human readable name of the object. It is only used with objects organized in a naming hierarchy. localName: A free text name local to a node in a naming hierarchy similar to a file directory structure. A power system related naming hierarchy may be: Substation, VoltageLevel, Equipment etc. Children of the same parent in such a hierarchy have names that typically are unique among them.

aliasName: A free text alternate name typically used in tabular reports where the column width is limited.

| Name | Type | Description |
| --- | --- | --- |
| name | String | Name of the name type. |

#### NameTypeAuthority

Authority responsible for creation and management of names of a given type; typically an organization or an enterprise system.

| Name | Type | Description |
| --- | --- | --- |
| name | String | Name of the name type authority. |

#### PositionPoint

Set of spatial coordinates that determine a point, defined in coordinate system specified in 'Location.CoordinateSystem', or "WGS 84" if not specified otherwise. Use a single position point instance to desribe a point-oriented location. Use a sequence of position points to describe a line-oriented object (physical location of non-point oriented objects like cables or lines), or area of an object (like a substation or a geographical zone - in this case, have first and last position point with the same values).

| Name | Type | Description |
| --- | --- | --- |
| xPosition | String | X axis position. |
| yPosition | String | Y axis position. |
| zPosition | String | (if applicable) Z axis position. |

#### QualityOfReading  «enumeration»

List of codes indicating the quality of the reading

| Name | Type | Description |
| --- | --- | --- |
| estimated |  |  |
| forecast |  |  |
| mixed |  |  |
| raw |  |  |
| validated |  |  |
| normalizedForWeather |  |  |
| other |  |  |

#### Reading

Specific value measured by a meter or other asset. Each Reading is associated with a specific ReadingType.

| Name | Type | Description |
| --- | --- | --- |
| cost | Float | Cost in a currency |
| timeStamp | AbsoluteDateTime | The time when the value was last updated |
| value | Float | Value of this reading. |

#### ReadingDirection  «enumeration»

| Name | Type | Description |
| --- | --- | --- |
| forward |  |  |
| reverse |  |  |
| net |  |  |
| total |  |  |

#### ReadingKind  «enumeration»

Kind of reading.

| Name | Type | Description |
| --- | --- | --- |
| energy |  |  |
| power |  |  |
| demand |  |  |
| voltage |  |  |
| current |  |  |
| voltageAngle |  |  |
| currentAngle |  |  |
| phaseAngle |  |  |
| powerFactor |  |  |
| pressure |  |  |
| volume |  |  |
| date |  |  |
| time |  |  |
| frequency |  |  |
| other |  |  |
| carbon |  |  |
| carbonDioxide |  |  |
| currentAverage |  |  |
| currentRMS |  |  |
| currentTHD |  |  |
| distortionPower |  |  |
| HCH |  |  |
| methane |  |  |
| NOx |  |  |
| perfluorocarbons |  |  |
| phasorPower |  |  |
| quantityPowerQ45 |  |  |
| quantityPowerQ60 |  |  |
| SO2 |  |  |
| sulfurHexafluoride |  |  |
| voltageAverage |  |  |
| voltageRMS |  |  |
| voltageTHD |  |  |
| relativeHumidity |  |  |
| volumetricFlow |  |  |

#### ReadingQuality

Quality of a specific reading value or interval reading value. Note that more than one quality may be applicable to a given reading. Typically not used unless problems or unusual conditions occur (i.e., quality for each reading is assumed to be 'good' unless stated otherwise in associated reading quality).

| Name | Type | Description |
| --- | --- | --- |
| quality | QualityOfReading | Quality, to be specified if different than ReadingType.defaultQuality. |

#### ReadingType

Type of data conveyed by a specific Reading.

| Name | Type | Description |
| --- | --- | --- |
| defaultQuality | QualityOfReading | Characteristics of a data value conveyed by a specific Reading, which allow an application to understand how a specific Reading is to be interpreted. |
| direction | ReadingDirection | Specifies the direction of flow of the measurement. |
| intervalLength | Duration | (if incremental reading value) Length of increment interval. |
| kind | ReadingKind | Kind of reading. |
| multiplier | UnitMultiplier | Multiplier for 'unit'. |
| name | String | The name is any free human readable and possibly non unique text naming the object. |
| unit | UnitSymbol | Unit for the reading value. |

#### Seconds  «Datatype»

Time, in seconds

| Name | Type | Description |
| --- | --- | --- |
| value | Float | Time, in seconds |
| unit | UnitSymbol |  |
| multiplier | UnitMultiplier |  |

#### ServiceCategory

Category of service provided to the customer.

| Name | Type | Description |
| --- | --- | --- |
| kind | ServiceKind | Kind of service. |

#### ServiceDeliveryPoint

Logical point on the network where the ownership of the service changes hands. It is one of potentially many service points within a service location, delivering service in accordance with a customer agreement. Used at the place where a meter may be installed.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

#### ServiceKind  «enumeration»

Kind of service.

| Name | Type | Description |
| --- | --- | --- |
| electricity |  |  |
| gas |  |  |
| water |  |  |
| time |  |  |
| heat |  | Includes hot water and steam |
| refuse |  |  |
| sewerage |  |  |
| rates |  |  |
| tvLicence |  |  |
| internet |  |  |
| other |  |  |
| cold |  | Includes chilled water and ice |

#### ServiceSupplier

Organisation that provides services to customers.

| Name | Type | Description |
| --- | --- | --- |
| kind | SupplierKind | Kind of supplier. |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

#### String  «Primitive»

A string consisting of a sequence of 8 bit characters. The character encoding is UTF-8. The string length is unspecified and unlimited.

#### SummaryMeasurement

An aggregated summary measurement reading.

| Name | Type | Description |
| --- | --- | --- |
| multiplier | UnitMultiplier | The multiplier part of the unit of measure, e.g. "kilo" (k) |
| timeStamp | AbsoluteDateTime | The date and time (if needed) of the summary measurement. |
| unit | UnitSymbol | The units of the reading, e.g. "Wh" |
| value | Float | The value of the summary measurement. |

#### SummaryQuality  «enumeration»

List of codes indicating the quality of the summary.

| Name | Type | Description |
| --- | --- | --- |
| estimated |  |  |
| forecast |  |  |
| mixed |  |  |
| validated |  |  |
| raw |  |  |
| normalizedForWeather |  |  |
| other |  |  |

#### SupplierKind  «enumeration»

Kind of supplier.

| Name | Type | Description |
| --- | --- | --- |
| utility |  |  |
| retailer |  |  |
| other |  |  |
| district |  |  |
| intermediary |  |  |
| local |  |  |
| microgrid |  |  |

#### TariffProfile

A schedule of charges; structure associated with Tariff that allows the definition of complex tariff structures such as step and time of use when used in conjunction with TimeTariffInterval and Charge. Inherited 'status.value' is defined in the context of the utility's business rules, for example: active, inactive, etc.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

#### UnitMultiplier  «enumeration»

The unit multipliers defined for the CIM

| Name | Type | Description |
| --- | --- | --- |
| p |  | Pico 10**-12 |
| n |  | Nano 10**-9 |
| micro |  | Micro 10**-6 |
| m |  | Milli 10**-3 |
| c |  | Centi 10**-2 |
| d |  | Deci 10**-1 |
| k |  | Kilo 10**3 |
| M |  | Mega 10**6 |
| G |  | Giga 10**9 |
| T |  | Tera 10**12 |
| none |  |  |

#### UnitSymbol  «enumeration»

The units defined for usage in the CIM

| Name | Type | Description |
| --- | --- | --- |
| VA |  | Apparent power in volt ampere |
| W |  | Active power in watt |
| VAr |  | Reactive power in volt ampere reactive |
| VAh |  | Apparent energy in volt ampere hours |
| Wh |  | Real energy in what hours |
| VArh |  | Reactive energy in volt ampere reactive hours |
| V |  | Voltage in volt |
| ohm |  | Resistance in ohm |
| A |  | Current in ampere |
| F |  | Capacitance in farad |
| H |  | Inductance in henry |
| C |  | Relative temperature in degrees Celsius |
| degC |  | Relative temperature in degrees Celsius. In the SI unit system the symbol is ºC. Electric charge is measured in coulomb that has the unit symbol C. To destinguish degree Celsius form coulomb the symbol used in the UML is degC. Reason for not using ºC is the special character º is difficult to manage in software. |
| s |  | Time in seconds |
| min |  | Time in minutes |
| h |  | Time in hours |
| deg |  | Plane angle in degrees |
| rad |  | Plane angle in radians |
| J |  | Energy in joule |
| N |  | Force in newton |
| S |  | Conductance in siemens |
| none |  | Dimension less quantity, e.g. count, per unit, etc. |
| Hz |  | Frequency in hertz |
| g |  | Mass in gram |
| Pa |  | Pressure in pascal (n/m2) |
| m |  | Length in meter |
| m2 |  | Area in square meters |
| m3 |  | Volume in cubic meters |
| thm |  | Energy, in therms |
| m3/h |  | Volumetric flow, in cubic meters per hour |
| ft3/h |  | Volumetric flow, in cubic feet per hour |

#### UsagePoint

Logical point on a network at which consumption or production is either physically measured (e.g. metered) or estimated (e.g. unmetered street lights).

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

#### UsageSummary

Summary of usage for a billing period

| Name | Type | Description |
| --- | --- | --- |
| billingPeriod | DateTimeInterval | The billing period to which the included measurements apply |
| billLastPeriod | Float | The amount of the bill for the previous period |
| billToDate | Float | The bill amount related to the billing period as of the date received |
| costAdditionalLastPeriod | Float | Additional charges from the last billing period |
| currency | String | The ISO 4217 code indicating the currency applicable to the bill amounts in the summary. See list at http://www.unece.org/cefact/recommendations/rec09/rec09_ecetrd203.pdf |
| currentBillingPeriodOverAllConsumption | SummaryMeasurement | The total consumption for the billing period |
| currentDayLastYearNetConsumption | SummaryMeasurement | The amount of energy consumed one year ago |
| currentDayNetConsumption | SummaryMeasurement | Net consumption for the current day (delivered - received) |
| currentDayOverallConsumption | SummaryMeasurement | Overall energy consumption for the current day |
| peakDemand | SummaryMeasurement | Peak demand recorded for the current period |
| previousDayLastYearOverallConsumption | SummaryMeasurement | The amount of energy consumed on the previous day one year ago |
| previousDayNetConsumption | SummaryMeasurement | Net consumption for the previous day |
| previousDayOverallConsumption | SummaryMeasurement | The total consumption for the previous day |
| qualityOfReading | QualityOfReading | Indication of the quality of the summary readings |
| ratchetDemand | SummaryMeasurement | The current ratchet demand value for the ratchet demand period |
| ratchetDemandPeriod | DateTimeInterval | The period over which the ratchet demand applies |

# APPENDIX A		Principle Verifications Against The Energy Information Usage Model

### Requirements validation method

Appendix A is provided for reference purposes only and is not intended to be construed as Business Practice Standards or Model Business Practices.

A complete requirements process involves a sequential breakdown of a problem starting with the collection of high level functional user requirements. The high level requirements are broken down into ever more specific derived requirements. Eventually to design requirements are achieved. At the bottom of the requirements tree are simple, testable, atomic requirements. When the atomic requirements are met, the tracing to the higher level requirements allows them to be met by definition.

The Priority Action Plan 10 charter – the text of the abstract, description, and objectives from the National Institute of Standards and Technology Framework and Roadmap for Smart Grid Interoperability Standards, Release 1.0 – contains high level user requirements  . The requirements presented in REQ.18.3 can be considered the first level requirements breakdown of derived requirements.

These Model Business Practices do not go to the final level of detailed design requirements. Instead, a parallel assessment was performed of the “derived requirements” against the evolving energy usage informationmodel, which itself was initially derived from detailed requirements identified in activities external to these  Model Business Practices.

- General

- Facilities shall include residences, buildings, and industrial installations

No specific limit as to the nature of the facility appears in the energy usage information model. Identifiers are for user, location, and device only.

- Usage and load information shall be readily available

While the availability of information is the realm of the Distribution Company and its Retail Customer (since the Distribution Company generally owns the billing meter), this energy usage information model facilitates availability by providing a single simple information model for client applications

- PAP 10 is an information model concept which is transport agnostic.

The information, classes, and attributes that are contained in the energy usage information model and concepts described in these Model Business Practices are data encoding and communication protocol agnostic.

#### Timeliness of Delivery

- Customers shall be able to use the information defined in these standards for real-time feedback on present and projected performance

There are no constraints on time stamps, latency or performance imposed by the energy usage information model, so it is possible to represent past, present, or future usage.

- Premises based systems (e.g. EMS/ESI) shall be able to use the information defined in these standards for real-time feedback on present and projected performance

There are no constraints on time stamps, latency or performance imposed by the energy usage information model, so it is possible to represent past, present, or future usage.

- Information exchanged shall be delivered in sufficient time to affect usage

There are no constraints on time stamps, latency or performance imposed by the energy usage information model, so it is possible to represent past, present, or future usage.

- Operations, Distribution, and Service Providers shall be able to use the information defined in these standards with the facility in near-real-time

There are no constraints on time stamps, latency or performance imposed by the energy usage information model, so it is possible to represent past, present, or future usage.

- Customers and premises-based systems shall be able to use the information defined in these standards to provide real-time feedback on present and projected performance

There are no constraints on time stamps, latency or performance imposed by the energy usage information model, so it is possible to represent past, present, or future usage.

#### Benefits to Facilities

- Standard load and usage information shall enable improved energy efficiency by defining a consistent way to communicate usage information.

By minimizing the number of different physical representations of usage information, and aligning the logical elements included in the definition of this information, it will allow for the development of applications that require this information to provide energy efficiency services and functionality.

- Standard load and usage information shall enable helping [all] Customers and operations manage their energy usage [and load]

The energy usage information model standardizes load and usage information so Retail Customer usage data provider services may be developed to interface with systems that help customers and operations with energy management.

- Standard load and usage information shall enable improved [facility] energy usage by availability of fine grained and timely information

The energy usage information model provides for measurement intervals enabling the availability of fine grained and timely load and usage information.

- Availability of fine grained and timely information will enable better decisions about energy usage and conservation

The energy usage information model enables a standard approach for Fine Grained and timely load and usage information so Retail Customer usage data provider services may interface to systems that help Retail Customers and/or operations with energy conservation.

- Facilities will benefit from consistent usage information exchange inside the facility, including meeting the energy efficiency goals of EISA 2007 and DOE initiatives

The energy usage information model provides consistent energy usage data representation so information exchanges are uniformly understood.  This may help ensure facility energy efficiency systems using this data are acting on correct and consistent information.

- Standard model shall support aggregated projections that can be passed on to operations [as defined in the NIST Smart Grid Framework] to make forecasting and management better and increase the value of a facility to smart grid

Aggregated projections are supported via future timestamps in interval and other data. The energy usage information model supports aggregation based on topology, device, and owner.

Sharing usage and load information inside a facility makes that facility more valuable to smart grid

The energy usage information model provides usage, load, and pricing information from which a facility manager or system may take optimization actions.

- The information exchanged shall allow integration of usage information throughout facility decision processes

The energy usage information model standardizes usage information so Retail Customer usage data provider services may be developed to integrate with systems handling facility decision processes.

#### Benefits to Grid and Service Provider Operations

- Standard load and usage information will improve forecasting and grid management by delivering aggregated projections to operations.

By specifying future intervals, which could be larger than measured intervals, it is possible to represent aggregated projections.

- Standard load and usage information will improve forecasting and grid management by delivering aggregated projections to service providers.

By specifying future intervals, which could be larger than measured intervals, it is possible to represent aggregated projections.

- Standard load and usage information will enable more responsive facilities.

Interoperable models of load and usage information may allow coordination of response capability, allowing those capabilities to be utilized more efficiently.

- Standard load and usage information will enable early deployment of devices that deliver and understand usage information.

By standardizing the information to be made available, devices and applications may  be able to obtain access to that information.

#### Internet-Like Future

- Standard usage and load information enables innovation in novel ways to help customers manage energy usage.

By standardizing on simple data sets that many energy service providers and Retail Customers can utilize, this information may be combined with additional information to find the specific way that will help Retail Customers the most.

- Standard usage and load information enables innovation by third party service and software providers.

By standardizing on simple data sets that many energy service providers and Retail Customers may utilize, this information may be combined with additional information to find the specific way that will help Retail Customers the most.

#### Improved Collaboration

- The Information model shall not restrict two way flows of information.

Subject to the Governing Documents and Applicable Regulatory Authority, Retail Customers, Distribution Companies and third parties may have access to the energy usage information model in real time with the appropriate usage information and Retail Customer and Distribution Company permissions/allowances.

- By standardizing usage and load information exchange, devices that deliver and understand usage and load can be deployed more quickly, contributing to achieving the energy efficiency goals of EISA 2007 and DOE .

Uniformity among signals as defined in the energy usage information model may lead to greater acceptance and adoption.

#### Information Sources and Accessibility

- Usage and load information shall be provided by utilities and aggregating service providers and may be provided by others.

Subject to the Governing Documents and Applicable Regulatory Authority, with consent from the Retail Customer, Distribution Companies and aggregators may provide access to usage and load information in the provided format for model integration.

- Usage and load information shall be provided by devices and EMS/ESI implementations.

Subject to the Governing Documents and Applicable Regulatory Authority, with consent from the Retail Customer and the Distribution Company, devices and EMS/ESI implementations may provide usage and load information in the provided format for model integration.

- Usage and load information can be accessed from the meter.

Subject to the Governing Documents and Applicable Regulatory Authority, with consent from the Retail Customer and the Distribution Company, usage and load information may be provided by the meter to accredited sources for purposes of running the energy usage information model.

- Usage and load information can be accessed from smart grid.

Subject to the Governing Documents and Applicable Regulatory Authority, with consent from the Retail Customer and the Distribution Company, usage and load information may be made available by any smart grid services provider via the public Internet.

#### Interactions and Information Exchanges Supported

- Interactions supported shall include those between Distribution [and Operations] and the industrial, commercial, and residential premises.

The enery usage information model does not exclude/include specific actors or potential exchange points, it merely describes data and format of information.

- Standard information models and understanding of usage and load are essential to cross domain interactions between Distribution [and Operations]: and Industrial, Commercial, Residential, and PEVs.

The standard information format does not exclude potential actors or exchange points including potential cross domain interaction including those between Distribution Companies and other actors.

- Interactions supported shall include those between Distribution [and Operations] and the industrial, commercial, residential premises, and plug-in electric vehicles.

The energy usage information model does not exclude/include specific actors or potential exchange points, it merely describes data and format of information.

- Standard information models and understanding of usage and load are essential to cross domain interactions between Service Providers: and Industrial, Commercial, Residential, and PEVs.

The standard information format does not exclude potential actors or exchange points including potential cross domain interaction including those between energy service providers and other actors.

- Interactions supported shall include those between Service Providers and the industrial, commercial, residential premises, and plug-in electric vehicles.

The energy usage information model does not exclude/include specific actors or potential exchange points, it merely describes data and format of information therefore by not excluding these actors.

- This effort shall support information standards for load curtailment, load shaping, and energy market operations, hence load and usage must be supported (see PAP09, PAP03, and PAP04).

Load and usage information are part of the energy usage information model.

- Information exchanges shall include to, from, and within facilities.

The energy usage information model does not preclude any potential exchange points therefore the requirement is met.

#### Information Characteristics

- Information model shall support exchange of both Fine Grained and summary information.

Summary information is available at the top level through PowerQualitySummary and UsageSummary components. Fine Grained information is available through the list of MeterReading measurement structures which in turn have both summary, Reading, and interval level detail, IntervalReading components. Additionally, the energy usage information data structures may be associated by device, location, and owner facilitating variable aggregations.

- Fine grained means that there is disaggregated information. Disaggregated information can include for example load, subsystem, premise, and variable time interval.

Fine Grained (disaggregated) information is described throughout the energy usage information model.  From CustomerEndDeviceAsset, ReadingKind, etc. there are enough detailed data elements to support Fine Grained information.

- Support for exchanging standard historical, present, and projected load information is required.

Historical information seems to be supported by the UsageSummary attributes for billing items only. Present (meter reading) information has Fine Grained as well as billing attributes defined.  Projected usage may be supported in the QualityOfReading <<enumeration>> attribute, which delineates the “estimated” and “forecast” attribute names. Historical, present, and projected load information is implicitly available at the interval and reading level based on the time stamps utilized -- past/present/future.

- Energy usage information shall  include usage, usage profile, and some component of cost (consistent with PAP03, PAP04)

Energy usage information, including costs is contained in the energy usage information model.  Interval data is supportive of usage profiles PAP03 and PAP04.

The PAP 10 energy usage information model shall allow for exchange of greater or lesser detail.

Summary information is available at the top level through PowerQualitySummary and UsageSummary components. Fine Grained information is available through the list of MeterReading measurement structures which in turn have both summary, Reading, and interval level detail, IntervalReading components. Additionally, the energy usage information data structures may be associated by device, location, and owner facilitating variable aggregations.

#### Timing and Goals

- Initial steps include making usage information more readily available by defining and standardizing usage information [delivered] through existing SG infrastructure.

The energy usage information data structures enable a diverse range of technologies to deliver energy usage information in a timely and protocol agnostic manner.  Examples of this might be a premise ESI or through services provided by a Distribution Company, facility, aggregator, energy services provider, or others.

- Information on device and facility usage is a primary goal in the initial focus.

As the energy usage information data structures cover summary and device level information, device and facility (aggregated) information may be delivered.

- Standard load and usage information will enable early deployment of devices that deliver and understand usage information.

As the energy usage information models are standardized, producers may deliver usage based devices with confidence in interoperability.

#### Requirements on Quality of Consensus Standard

The information model shall support:

- Consistent data representation for REST & Web Services.

This requirement will support REST and Web Services. However, other transports may be supported such as, but not limited to, IEEE 1703/ANSI C12.22 architecture per AEIC Guidelines V2.0 . See REQ.18.3.3 which implies support for alternative syntaxes.

- Specification of transactional exchange, syntax, and required population of the information model are beyond the scope of these requirements.

No transactional syntaxes or message services are defined in these Model Business Practices.

- an extensible information model – e.g. ability to add custom extensions as needed

This energy usage information model may be extended by adding new elements (classes, attributes, and associations). Implementations must utilize conventions to ignore any elements not understood by components, to allow them to continue to operate when future extensions are added.

- have an evolvable information model – e.g. the standards process supports future revisions

All NAESB Model Business Practices may be modified through the NAESB process. (Please see: http://www.naesb.org//misc/naesb_process_for_standards_dev.doc)

Forward compatible as we evolve

To maintain forward and backward compatibility, best practicies on extensibility, which also includes evolvability through Model Business Practice versions, may be followed.

- Supportive of versioning

Versioning in the UML Model is addressed by a tagged value.

- Usable without “knowing all the details”

Information in the energy usage information model is arranged hierarchically. Greater detail can be understood the deeper into this hierarchy the reader goes. Summary information is exposed near the top of the hierarchy. These arrangements make it straightforward to accessing applications to ignore the level of detail they do not plan to use or comprehend, while getting value out of higher level information.

More flexibility for independent innovation shall be achieved through:

- Focus on information exchanged

The specification of an information-only model allows the Model Business Practices to focus on the information exchanged.

- Agreed upon interfaces are maintained over time

A profile of this energy usage information model supports the exposure of an interface. One is provided as an example in this Model Business Practice Standard. All NAESB Model Business Practices may be modified through the NAESB process. (Please see: http://www.naesb.org//misc/naesb_process_for_standards_dev.doc). See REQ 18.3.3 which describes how the energy usage information model may be used as the basis for smart grid interfaces.

- Minimal details = maximum interoperation

The energy usage information model provides a hierarchy of summary and detailed information. A balance is achieved by the definition of minimal set of details necessary to satisfy the cumulative set of use cases of the energy usage information and allocated to this model.

- Intellectual Property Rights shall be clear & clean

The NAESB IPR Policy may be found through the following link: http://www.naesb.org/pdf4/naesb_ipr_policy.doc.  Information in the energy usage information model is arranged hierarchically. Greater detail can be understood the deeper into this hierarchy the reader goes. Summary information is exposed near the top of the hierarchy. These arrangements make it straightforward to accessing applications to ignore the level of detail they do not plan to use or comprehend, while getting value out of higher level information.  NAESB has agreed to make the energy usage information model included in REQ.18.4 and REQ.18.4.1 available to the public without charge. The use of the energy usage information model included in REQ.18.4 and REQ.18.4.1 for the production of derivative work products is not prohibited. All NAESB Model Business Practices may be modified through the NAESB process. (Please see: http://www.naesb.org//misc/naesb_process_for_standards_dev.doc). The use of the energy usage information model included in REQ.18.4 and REQ.18.4.1 may be distributed in any manner, however, such model must be made available without charge.

Results of PAP 10 shall produce:

- Information model and XML schema

There is no normative schema in these Model Business Practices. There is an "example" non-normative schema to demonstrate how to take the  energy usage information model and create a profile that can generate an XML schema or other descirption.

- At information exchange level

The energy usage information model is designed for semantic exchange between participants. The energy usage information model serves to define an information exchange (see REQ.18.3.3)

The information model shall be:

- Readable without charge

NAESB has agreed to make the energy usage information model included in REQ.18.4 and REQ.18.4.1 available to the public without charge.

- Reusable without restriction or charge

The use of the energy usage information model included in REQ.18.4 and REQ.18.4.1 for the production of derivative work products is not prohibited.

- Adaptable without restriction or charge

All NAESB Model Business Practices may be modified through the NAESB process. (Please see: http://www.naesb.org//misc/naesb_process_for_standards_dev.doc)

- Usable for open source

The use of the energy usage information model included in REQ.18.4 and REQ.18.4.1 may be distributed in any manner, however, such model must be made available without charge.

#### Additional Considerations (not provided by SGIP PAP10 Working Group)

- In communicating energy usage information, the energy usage information model should be used and the information outlined within the model should be available.

The energy usage information model provides a common, consensus-based, vendor-neutral model to represent usage information, to be used in exchange scenarios where it is applicable and desired by usage information providers and Retail Customers. REQ.18.3.3 elaborates the technical considerations for constructing implementing Model Business Practices which do specify syntax.

- The energy usage information model data set has three unique identifiers, which serve the purpose of allowing various aggregations of data sets.

The energy usage information model has the class ServiceDeliveryPoint which has an ID. Additionally, it has an association with Retail Customer and MeterAsset which support aggregation by location, owner, and device.

Owner (Retail Customer): This is represented by the Retail Customer class, provided the use of the term “owner” or similar verbiage is not indicative or dispositive of any ownership or other rights under applicable law in the energy usage information model or any data.

Location: This is represented by ServiceDeliveryPoint.

Device: This is represented by MeterAsset.

- The TariffProfile component should be included in the energy information usage model.

TariffProfile is included in the energy usage information model as a reference. This reference supports the potential acquisition of details on the agreements between energy service provider and Retail Customer of energy when this optional reference is included and populated.

- UsageSummary should include an optional cost attribute to interval and reading class.

UsageSummary has a cost roll up and optional cost attributes in IntervalReading and Reading classes are included in the energy usage information model.

- A known base currency should be included in the top level class associated with the MeterAsset for the energy usage information model, which would be applicable to all instances

A base currency attribute is part of the UsageSummary class based on ISO4217.

- The energy usage information model should be compliant with ISO8601.

The current energy usage information model does not define the primitive to be used to represent the dateTime, however the default xs:dateTime is ISO8601 compliant.

- Optionally, both start and end of interval can be defined in order to support non-uniform interval information, which is accomplished in the energy information usage model through endTimestamp to IntervalReading.

The endTimeStamp element is included as an optional component of IntervalReading.

- Demand based elements are maintained in the TariffProfile including common demand and demand ratchets.

The energy usage information model does not contain the full definition of TariffProfile, but this is the CIM element where thresholds associated with the rate are specified.

- The energy information usage model status structure includes a named pair of QualityOfReading and values of raw, forecast, validated, estimated, mixed, and other for qualifying the associated data set, as an explicit representation of these name value pairs can be processed in implementations by sending and receiving actors. The UsageSummary class also includes such summary information.

The energy usage information model is specified as described.

- MeterEvent should be associated with MeterAsset, not MeterReading.

- MeterEvent is not part of the energy usage information model. This will be forwarded to IEC 61968 Committtee for resolution.

- PowerQualitySummary as events of enumerated type are provided in the energy information usage model to represent a simple exposure of summary information.

The energy usage information model exposes these events in summary form as described.

- With respect to devices monitoring or capable of monitoring point source emissions, the energy usage information model should represent and take into account pollutant emissions (e.g. SO2, NOX, etc.).

Detailed information about these emissions is possible through definition of a ReadingType specifying the appropriate kindReading values as enumerated in ReadingKind.

- energy usage information model should include a top level cost summary – billStart, billEnd, billToDate, lastPeriod, and costAdditional, so that the bill to date and bill as of the last billing period could be conveyed through a formula:

billLastPeriod = costAdditionalLastPeriod + ∑MeterReading[i].Reading.cost (constrained by datetime)

The model represents these elements in the UsageSummary class.

- 4.  SUPPORTING DOCUMENTATION

- Description of Request:

- Description of Recommendation:

- Business Purpose:

- Commentary/Rationale of Subcommittee(s)/Task Force(s):

NAESB Process for Standards Development - http://www.naesb.org//misc/naesb_process_for_standards_dev.doc

See e.g. EIS Alliance Use Cases v2 including UC-9, UC-11, UC-12, UC-14, UC-15, ZBHP_SE_MRD 3.1 "HAN Device Information Retrieval Request", and OpenADE use case “Publication”.

See e.g. EIS Alliance Use Cases v2 including UC-3, UC-11, UC-14, UC-15, and OpenADE use case “Publication”

Needs refs. This is an addition to EIS and OpenADE.

See e.g. EIS Alliance Use Cases v2 including UC-1, UC-2, UC-3, UC-8, UC-9.

See e.g. EIS Alliance Use Cases v2 including UC-1, UC-2, UC-3

This is but one of several views that might result from choices permitted in the context of the conformance paragraph.

http://www.nist.gov/public_affairs/releases/upload/smartgrid_interoperability_final.pdf