- 1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| X | Accept as requested | X | Change to Existing Practice |
| --- | --- | --- | --- |
|  | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

- 2.  TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
| X | Initiation | X | Initiation |
|  | Modification |  | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
| X | Principle | X | Principle |
| X | Definition | X | Definition |
| X | Business Practice Standard | X | Business Practice Standard |
| X | Document | X | Document |
| X | Data Element | X | Data Element |
| X | Code Value | X | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
| X | Business Process Documentation | X | Business Process Documentation |

- 3.  RECOMMENDATION

- SUMMARY:

The Joint Retail Electric (REQ) and Wholesale Electric (WEQ) Quadrants’ PAP 10 Smart Grid Subcommittee submit this Recommendation for 2010 Retail Annual Plan Item No. 9d and WEQ Annual Plan Item No. 6d – Business Practices and Information Models to support Priority Action Plan 10, “Standardized Energy Usage Information,” based on the Tiger Team Report issued on June 22, 2010 by the NIST SGIP PAP10 Committee.

In initiating this standards development, NAESB agreed to by year-end 2010 develop an energy use information model standard defining a common data format that may be used when information is communicated between utilities, third parties and energy use customers, via customer devices and/or third party energy services providers.  The energy usage information model standard will enable the exchange of detailed energy information in a consistent format for use by customers, utilities, service providers, consumer devices, and energy applications.  Doing so will let Customers track their power use and help them manage energy consumption and cost.  Without a standardized format for representing energy use data, a variety of approaches could emerge, leading to incompatibilities among energy management products and services.  It is planned that the American Society of Heating, Refrigerating and AC Engineers (ASHRAE) will extend the NAESB standard to create a facilities data model providing additional energy use data elements for facility energy management including buildings.

- RECOMMENDED STANDARDS:

# REQ.18	CUSTOMER ENERGY USAGE INFORMATION COMMUNICATION

- EXECUTIVE SUMMARY

This standard establishes the model business practices for Retail Customer energy usage information communication.  Specifically, these model business practices establish  a data model for energy usage information.  The standard does not limit the form or function of the data model and is exemplary, but not exclusive, of the information that may be communicated in a consistent format among a variety of Entities, including, but not limited to, Distribution Companies, energy service providers, meter-reading entities, and Retail Customers.  Such communication may occur via multiple systems and devices.  Establishment of this energy usage information model will standardize a common data format which may be used when information is communicated between utilities, third parties and energy use customers which may aid Retail Customers in tracking and managing their energy use.

The model business practices in this standard do not require that wholesale electricity markets administered by System Operators adopt this model since System Operators generally are not the system of record for individual Retail Customer energy usage information and load data or individual Retail Customer forecasted usage and load data.  These model business practices are not intended to replace applicable Governing Documents, and, in the event of a conflict, the latter documents shall have precedence over these standards. Without limiting the foregoing, these model business practices are only applicable to the extent the information covered by this model is collected, managed or communicated pursuant to the applicable Governing Documents.  Retail Customer energy usage information communication encompasses a variety of interactions between Distribution Companies, Retail Customers and energy services providers.  In a business environment where best practices are voluntary, model business practices such as those in this document may be applied within the context of regulatory or other market requirements and agreements.

- INTRODUCTION

The North American Energy Standards Board (NAESB) is a voluntary, non-profit organization comprised of members from all aspects of the natural gas and electric industries.  Within NAESB, the Retail Electric Quadrant (REQ) and the Retail Gas Quadrant (RGQ) focus on issues impacting the retail sale of energy to end-use Retail Customers.  REQ / RGQ model business practices are intended to provide guidance to retail energy market participants not limited to: Distribution Companies, energy Suppliers, and energy service providers involved in providing competitive energy service to end-use Retail Customers.  The focus of these model business practices is the representation of Retail Customer energy usage information.  The scope of the energy usage information model is not intended to characterize the data information model for communication of  billing or related information.

These model business practices are voluntary and do not address policy issues that are the subject of state legislation or regulatory decisions.  These model business practices have been adopted with the realization that as the industry evolves, additional and amended model business practices may be necessary.

- BUSINESS PROCESSES AND PRACTICES

Overview

This standard establishes the model business practices for Retail Customer energy usage information communication.  Specifically, these model business practices establish  a data model for energy usage information.  The standard does not limit the form or function of the data model and is exemplary, but not exclusive, of the information that may be communicated in a consistent format among a variety of Entities, including, but not limited to, Distribution Companies, energy service providers, meter-reading entities, and Retail Customers.  Such communication may occur via multiple systems and devices.  Establishment of this energy usage information model will standardize a common data format which may be used when information is communicated between utilities, third parties and energy use customers which may aid Retail Customers in tracking and managing their energy use.

The business processes and practices described below are not presently applicable to wholesale markets because wholesale markets do not generally communicate directly with Retail Customers and are not the system of record for individual Retail Customer energy usage information or individual Retail Customer load forecast. The energy usage information model and these model business processes and practices  are not required of System Operators.  As the energy usage information model and these business processes and practices evolve, System Operators may determine that use of the energy usage information model in these model business practices can be applied to other information.  However, such use is not intended to replace or supplant applicable Governing Documents. Without limiting the foregoing, these model business practices are only applicable to the extent the information covered by this model is collected, managed or communicated pursuant to the applicable  Governing Documents.

## REQ.18.1	Principles

### REQ.18.1.1	Overall Principles

#### These model business practices provide an energy usage information model, defining a collection of structured energy usage information elements that may be needed to allow customer management of their energy usage and costs, including environmental impacts.

#### The energy usage information model is specified in UML, as a syntax neutral notation, so that it may be used within exchange protocols using a variety of specific representation syntax and exchange mechanisms, specified outside this standard.

#### The recommended use of the energy usage information model is in implementation specifications exposing Customer energy usage information. Specifications that conform to the model shall contain equivalent required and optional classes and attributes, thus resulting in straightforward (preferably lossless) transformations between conformant specifications.

#### Neither the energy usage information model nor these model business practices establish or govern ownership or any other rights in any information or data; such ownership and other rights are subject to and governed by the Governing Documents and/or Applicable Regulatory Authority.

#### Neither the energy usage information model nor these model business practices create any requirement to collect, manage or communicate any information.

#### While this standard defines a data model which may be used when energy usage information is communicated, the Governing Documents and/or Applicable Regulatory Authority determine the ownership of the data, the access to the data, what systems and hardware are required, and how it is paid for. There are no assumed or implied regulations in this standard.

Delete the following two items if Appendix A is deleted.  If Appendix A is not deleted, see comments below:

#### Appendix A describes the principles used in developing the energy usage information model and includes an explanatory verification paragraph describing how the energy usage information model satisfies each requirement. The Appendix A requirements up through REQ.18.4.2.11 were provided by the SGIP PAP10 Working Group to the NAESB PAP 10 Subcommittee.

#### The potential requirements in Appendix A represent a series of potential capabilities for the expressiveness of this standard and are specifically not intended to be requirements for the use of or on users of this standard.

## Energy Usage Abbreviations, Acronyms and Definition of Terms

### Business Definitions

#### Applicable Regulatory Authority

The state regulatory agency or other governing body that provides oversight, policy guidance, and direction to any parties involved in the process of providing energy to Retail Customers through regulations and orders.

#### Retail Customer

Any Entity that takes gas and/or electric service for its own consumption.

#### Distribution Company

A regulated Entity which provides distribution services and may provide energy and/or transmission/transportation services in a given area.

#### Entity

A person or organization with sufficient legal standing to enter into a contract or arrangement with another such person or organization (as such legal standing may be determined by those parties) for the purpose of conducting and/or coordinating energy transactions.

#### Governing Documents

Documents that govern the interactions among parties, including but not limited to: regulatory documents (e.g., tariffs, rules, regulations), contractual agreements, and Distribution Company Operational Manuals and other relevant models and procedures.

#### Market Participant

A party engaged in the process of providing competitive retail energy to end-use Customers including, but not limited to, the Distribution Company, the Supplier, the Registration Agent, the settlement agent, and the meter reading Entity.

#### Supplier

Persons engaged in the competitive sale of energy to end-users.

### Technical Definitions

This section contains technical terms and abbreviations used in this recommendation.

#### Fine Grained

Characterized by abundant use of detail or thoroughness of treatment.

Comment:  Do you want to add definitions for Energy Management System (EMS) and Energy Services Interface (ESI)?

#### Operations

One of the seven domains identified in the NIST Framework and Roadmap, defined therein as “The managers of the movement of electricity”. This could apply to operators of equipment within any of the other domains.

### Abbreviations and Acronyms

| Abbreviation / Acronym | Meaning |
| --- | --- |
| UML | Unified Modeling Language |
| EMS | Energy Management System |
| ESI | Energy Services Interface |
| PEV | Plug-in Electric Vehicle |
| EISA | Energy Independence and Security Act |
| DOE | Department of Energy |
| HAN | Home Area Network |

## Energy Usage Information Model Business Practices

### Introduction

The focus of these model business practices is the representation of energy usage information.  The energy usage information includes historic, present, and future projected usage and load together with the time period(s) for that information.

These model business practices draw on actors and use cases defined by the following groups:

- Energy Information Standards Alliance (the EIS Alliance) [EIS]

- NAESB Survey and Consolidation of PAP10 Use Cases [NAESB PAP10]

- UCAIug OpenADE [ADE]

- ZigBee/Home Plug Smart Energy Profile 2.0 Market Requirements [SEP MRD]

The relevant use cases are summarized as follows:

#### The energy service provider and/or Distribution Company communicates historic and present energy usage information and load information to the Retail Customer or facility.

#### The Retail Customer or facility communicates future projected usage and load information to the energy service provider, Distribution Company, or grid Operations.

#### The energy service provider and/or Utility communicates their projection of usage and load to the Retail Customer or facility.

#### Devices within a facility communicate their present and future projected usage and load to controllers or facility EMS for aggregation and to be a component of facility aggregated future projected usage and load.

#### Devices, business processes, EMS, ESI, and other functional units within the facility communicate usage and load information among themselves.

#### These model business practices are limited to the seed specification which shall be usable by others to build standards and/or specification for exchange of energy usage information and load information appropriate to their needs without overly constraining those uses or including information that is not required in all implementations of specifications for exchanging load and usage.

### Energy Usage Information Model Format and Use

The energy usage information model is developed using a UML modeling tool.  The model classes, attributes, types and descriptions are included in REQ 18.4.  The model is made available as XMI, which is the standard XML import/export format for UML.  The model is exported as HTML and made available as a downloadable archive viewable with a web browser.

### Energy Usage Information Model Technical Considerations

#### The energy usage information model may be used as the basis for smart grid interfaces between parties exchanging customer energy usage information between energy services providers, consumers, and others.

#### The energy usage information model permits schemas to be generated from it, using XML Schema Definition Language (XSD) and other format specification languages. Profiles may be constructed from the energy usage information model for this purpose.

#### Implementations may include all or a subset of the elements defined in the model, possibly using a profile of the model.

#### The informative example XSD shall conform to Naming and Design Rules best practices as described by IEC 62361-100, Naming and Design Rules for CIM Profiles to XML Schema Mapping.

#### The model facilitates multiple information exchange standards to be used, including REST and Web Services. The specifics are left to implementing specifications to define.

#### Though there may be elements useful for transfer of security-related information elements in the model, the specific details related to how to protect sensitive information, and how to authorize specific roles or identities to have access are not defined in this recommendation.

### Conformance

#### A conformant specification that refines or extends this standard shall produce information for exchange that can be transformed algorithmically (that is based on the standard alone) into a form that can be validated through the method described in REQ.18.3.4.2.

This requirement means that various formats for representation and exchange and various subsets and potentially supersets of information content are envisioned based on this standard. The use of the informative example schema is not to impose its direct use in message validation. Rather, its use is intended to facilitate verification of conformance to the information model with respect to message content and semantics without imposing constraints on specific message payload schemas and data element representations. Some representations are anticipated to be entirely binary in nature. Others will trade off strings for integer representations of information contents.  Regardless of the means, the information should be convertible to be testable as described herein.

#### A specification that claims conformance to this standard shall describe and define an automatable transformation between that specification’s model to and from this UML Model, including indicating attributes used and not used.

#### Conformant specifications shall map corresponding model components to and from at least the following required core model attributes as exchanged between data provider and data consumer in their defined messages:

- One or more measurement or summary containers: IntervalReading, Reading, PowerQualitySummary, UsageSummary

- At least two of the following attributes, for each IntervalReading: timeStamp, endTimeStamp, duration

- value (the value of the measurement, from IntervalReading or Reading)

- ReadingType – ID, defaultQuality, direction, kind, multiplier, name, unit

- Association to ReadingType for each measurement (IntervalReading or Reading) (exists in model through MeterReading)

- Measurement source / location – ServiceDeliveryPoint.ID or MeterAsset.ID and association to measurements or summary

## Energy Usage Information Model

The energy usage information model herein is organized consistent with several related and well established models, including the IEC TC57 Common Information Model [IEC 61968 Part 9], ZigBee Smart Energy Profile 2.0 [SEP2.0], that are defined by the Energy Information Standards Alliance [EIS Alliance] and Open Automated Data Exchange [OpenADE]. New classes and attributes identified in the model in this standard will be proposed to IEC TC57 for extension of a future release of the CIM.  The energy usage information model, where possible, uses classes, information elements and attribute names drawn from the CIM and the cited references.

The starting point for the energy usage information model is the ServiceDeliveryPoint. ServiceDeliveryPoints identify key references for the information set optionally including identification of the customer, the location, and the physical asset. ServiceDeliveryPoint is associated in turn with zero or more MeterReadings. A MeterReading composes information about a particular measurement such as kWH or kW. A MeterReading has a ReadingType which describes the nature of the measurement including it units of measure, and zero or more IntervalReadings or Readings and associated quality information. ServiceDeliveryPoint may also be associated with summary information on load and usage, and optionally, power quality.  For applications requiring third party access to this information, additional classes are identified to facilitate associating customer and customer agreement information with the measurements available at a ServiceDeliveryPoint.

To find the usage or load in a particular interval, identify the appropriate ServiceDeliveryPoint, select the MeterReading of interest (measurement) and then select the IntervalReading or Reading associated with the given interval.

The energy usage information model includes many optional components. The complete set of information expressable using the model satisfies a wide range of applicability requirements identified by industry. Users of this standard may optionally take advantage of these extended definitions based on need without requiring them. Applications built on the energy usage information model may elect which optional components to present. However, clients of this information can be expected to recognize all components provided in the application.

REQ.18.3.4.3 idenitifies the set of core model elements that shall be supported by specifications claiming conformance to this standard. The following class diagram illustrates a view of this core of the energy usage information model:

Figure 1: Energy Usage Information Model – Minimal View

The full energy usage information model, Illustrated in Figure 2,  forms the basis of the required standard.

Figure 2: Full Energy Usage Information Model

### Energy Usage Information Model Details

The following sections contain the classes and attributes defined in the model, along with their descriptions. Elements tagged with <<enumeration>> define the valid values for an enumerated data type.

#### Customer

Organisation receiving services from ServiceSupplier.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |
| name | string | Name of an attribute. |

#### CustomerAgreement

Agreement between the Customer and the ServiceSupplier to pay for service at a specific ServiceLocation. It specifies the type of service provided at the ServiceLocation and the rate, which determines how the customer bill is calculated.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |

#### DateTimeInterval

Interval of date and time.

| Name | Type | Description |
| --- | --- | --- |
| duration | int | The duration of this interval, in seconds |
| end | dateTime | Date and time that this interval ended. |
| start | dateTime | Date and time that this interval started. |

#### EnergyUsageInformation

A collection of customer energy usage information

#### IntervalReading

Data captured over a specific interval of time. If not specified, the duration is the intervalLength of the associated ReadingType, where the full definition of the units of measure is located.

| Name | Type | Description |
| --- | --- | --- |
| cost | float | The cost associated with this reading for this interval. |
| duration | int | The duration of the interval, in seconds. |
| endTimeStamp | dateTime | End interval timestamp |
| timeStamp | dateTime | The start date and time of an interval reading |
| value | float | The value of the reading for this interval in the unit of measure defined by the associated ReadingType. |

#### MeterAsset

Physical asset that performs the metering role of the ServiceDeliveryPoint. Used for measuring consumption and detection of events.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |
| name | string | Meter name |

#### MeterReading

Set of values obtained from the meter.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |
| valuesInterval | DateTimeInterval | Interval in date time (start & end) |

#### PowerQualitySummary

A summary of power quality events.

| Name | Type | Description |
| --- | --- | --- |
| frequencyVariations | int | Number of frequency variations |
| interruptions | int | Number of interruptions |
| sags | int | Number of sags |
| summaryInterval | DateTimeInterval | Interval of summary period |
| swells | int | Number of swells |
| transients | int | Number of transients |
| voltageImbalances | int | Number of voltage imbalances |

#### QualityOfReading  «enumeration»

List of codes indicating the quality of the reading

| Name | Type | Description |
| --- | --- | --- |
| estimated |  |  |
| forecast |  |  |
| mixed |  |  |
| raw |  |  |
| validated |  |  |
| normalizedForWeather |  |  |
| other |  |  |

#### Reading

Specific value measured by a meter or other asset. Each Reading is associated with a specific ReadingType and was taken at the specified time.

| Name | Type | Description |
| --- | --- | --- |
| cost | float | Cost in a currency |
| timeStamp | dateTime | The date and time of a reading |
| value | float | The value of the reading in the unit of measure defined by the associated ReadingType. |

#### ReadingDirection  «enumeration»

| Name | Type | Description |
| --- | --- | --- |
| delivered |  |  |
| received |  |  |
| net |  |  |

#### ReadingKind  «enumeration»

Kind of reading.

| Name | Type | Description |
| --- | --- | --- |
| current |  |  |
| currentAngle |  |  |
| date |  |  |
| demand |  |  |
| energy |  |  |
| other |  |  |
| phaseAngle |  |  |
| power |  |  |
| powerFactor |  |  |
| pressure |  |  |
| time |  |  |
| voltage |  |  |
| voltageAngle |  |  |
| volume |  |  |
| carbonDioxide |  |  |
| NOx |  |  |
| carbon |  |  |
| SO2 |  |  |
| methane |  |  |
| HCH |  |  |
| perfluorocarbons |  |  |
| sulfurHexafluoride |  |  |
| phasorPower |  |  |
| quantityPowerQ60 |  |  |
| quantityPowerQ45 |  |  |
| distortionPower |  |  |
| voltageRMS |  |  |
| voltageAverage |  |  |
| currentRMS |  |  |
| currentAverage |  |  |
| voltageTHD |  |  |
| currentTHD |  |  |

#### ReadingQuality

Quality of a specific reading value or interval reading value. If not present, then is assumed to be the defaultQuality indicated by the ReadingType.

| Name | Type | Description |
| --- | --- | --- |
| quality | QualityOfReading | Quality, to be specified if different than defaultQuality |

#### ReadingType

Type of data conveyed by a specific Reading.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |
| aliasName | string | The aliasName is free text human readable name of the object alternative to IdentifiedObject.name. It may be non unique and might not correlate to a naming hierarchy. |
| channelNumber | integer | Logical positioning of this measurement data. |
| defaultQuality | QualityOfReading | Characteristics of a data value conveyed by a specific Reading, which allow an application to understand how a specific Reading is to be interpreted. |
| description | string | The description is a free human readable text describing or naming the object. It may be non unique and may not correlate to a naming hierarchy. |
| direction | ReadingDirection | Specifies the direction of flow of the measurement. |
| intervalLength | int | (if incremental reading value) Length of increment interval, in seconds. Interval duration specified at the IntervalReading overrides this default. |
| kind | ReadingKind | Kind of reading. |
| multiplier | UnitMultiplier | Multiplier for 'unit'. |
| name | string | Name of an attribute. |
| unit | UnitSymbol | Unit in symbol |

#### ServiceDeliveryPoint

Logical point on the network where the ownership of the service changes hands. It is one of potentially many service points within a ServiceLocation, delivering service in accordance with a CustomerAgreement. Used at the place where a meter may be installed.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |
| description | string | The description is a free human readable text describing or naming the object. It may be non unique and might not correlate to a naming hierarchy. |
| lat | float | Latitude of the location |
| long | float | Longitude of the location |
| name | string | Name of an attribute. |

#### SummaryMeasurement

An aggregated summary measurement reading.

| Name | Type | Description |
| --- | --- | --- |
| multiplier | UnitMultiplier | The multiplier part of the unit of measure, e.g., "kilo" (k) |
| timeStamp | dateTime | The date and time (if needed) of the summary measurement. |
| unit | UnitSymbol | The units of the reading, e.g. "Wh" |
| value | float | The value of the summary measurement. |

#### TariffProfile

A schedule of charges; structure associated with Tariff that allows the definition of complex tarif structures such as step and time of use when used in conjunction with TimeTariffInterval and Charge. Inherited 'status.value' is defined in the context of the utility's business rules, for example: active, inactive, etc.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Object identifier |
| name | string | Name of an attribute. |

#### UnitMultiplier  «enumeration»

The unit multipliers defined for the CIM

| Name | Type | Description |
| --- | --- | --- |
| c |  | Centi 10**-2 |
| d |  | Deci 10**-1 |
| G |  | Giga 10**9 |
| k |  | Kilo 10**3 |
| m |  | Milli 10**-3 |
| M |  | Mega 10**6 |
| micro |  | Micro 10**-6 |
| n |  | Nano 10**-9 |
| none |  |  |
| p |  | Pico 10**-12 |
| T |  | Tera 10**12 |

#### UnitSymbol  «enumeration»

The units defiend for usage in the CIM

| Name | Type | Description |
| --- | --- | --- |
| A |  | Current in ampere |
| deg |  | Plane angle in degrees |
| F |  | Capacitance in farad |
| g |  | Mass in gram |
| h |  | Time in hours |
| H |  | Inductance in henry |
| Hz |  | Frequency in hertz |
| Hz-1 |  | per Hertz |
| J |  | Energy in joule |
| J/s |  | Joule per second |
| kg/J |  | Mass per energy |
| m |  | Length in meter |
| m2 |  | Area in square meters |
| m3 |  | Volume in cubic meters |
| min |  | Time in minutes |
| N |  | Force in newton |
| none |  | Dimension less quantity, e.g., count, per unit, etc. |
| C |  | Relative temperature in degrees Celsius |
| ohm |  | Resistance in ohm |
| Pa |  | Pressure in pascal (n/m2) |
| rad |  | Plane angle in radians |
| S |  | Conductance in siemens |
| s |  | Time in seconds |
| s-1 |  | per second |
| V |  | Voltage in volt |
| V/VAr |  | Volt per volt ampere reactive |
| VA |  | Apparent power in volt ampere |
| VAh |  | Apparent energy in volt ampere hours |
| VAr |  | Reactive power in volt ampere reactive |
| VArh |  | Reactive energy in volt ampere reactive hours |
| W |  | Active power in watt |
| W/Hz |  | Watt per hertz |
| W/s |  | Watt per second |
| Wh |  | Real energy in Watt hours |
| pct |  | Percent |

#### UsageSummary

Summary of usage for a billing period

| Name | Type | Description |
| --- | --- | --- |
| billingPeriod | DateTimeInterval | The billing period to which the included measurements apply |
| billLastPeriod | float | The amount of the bill for the previous period |
| billToDate | float | The bill amount related to the billing period as of the date received |
| costAdditionalLastPeriod | float | Additional charges from the last billing period |
| currency | string | The ISO 4217 code indicating the currency applicable to the bill amounts in the summary. See list at http://www.unece.org/cefact/recommendations/rec09/rec09_ecetrd203.pdf |
| currentBillingPeriodOverAllConsumption | SummaryMeasurement | The total consumption for the billing period |
| currentDayLastYearNetConsumption | SummaryMeasurement | The amount of energy consumed one year ago |
| currentDayNetConsumption | SummaryMeasurement | Net consumption for the current day |
| currentDayOverallConsumption | SummaryMeasurement | Overall energy consumption for the current day |
| peakDemand | SummaryMeasurement | Peak demand recorded for the current period |
| previousDayLastYearOverallConsumption | SummaryMeasurement | The amount of energy consumed on the previous day one year ago |
| previousDayNetConsumption | SummaryMeasurement | Net consumption for the previous day |
| previousDayOverallConsumption | SummaryMeasurement | The total consumption for the previous day |
| qualityOfReading | QualityOfReading | Indication of the quality of the summary readings |
| ratchetDemand | SummaryMeasurement | The current ratchet demand value for the ratchet demand period |
| ratchetDemandPeriod | DateTimeInterval | The period over which the ratchet demand applies |

#### ServiceKind  «enumeration»

Kind of service.

| Name | Type | Description |
| --- | --- | --- |
| electricity |  |  |
| gas |  |  |
| water |  |  |
| time |  |  |
| heat |  | Includes hot water and steam |
| refuse |  |  |
| sewerage |  |  |
| rates |  |  |
| tvLicence |  |  |
| internet |  |  |
| other |  |  |
| cold |  | Includes chilled water and ice |

#### CustomerAuthorisation

Holds an authorisation for access to specific user-private data granted to a 3rd Party service provider. [OpenADE Extension - Specialization of "Agreement"]

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Unique identifier for this authorisation |
| validityInterval | DateTimeInterval | Date and time interval this agreement is valid (from going into effect to termination). |

#### ServiceCategory

Category of service provided to the customer.

| Name | Type | Description |
| --- | --- | --- |
| kind | ServiceKind | Kind of service. |

#### SupplierKind  «enumeration»

Kind of supplier.

| Name | Type | Description |
| --- | --- | --- |
| district |  |  |
| intermediary |  |  |
| local |  |  |
| microgrid |  |  |
| other |  |  |
| retailer |  |  |
| utility |  |  |

#### ServiceSupplier

Organisation that provides services to Customers.

| Name | Type | Description |
| --- | --- | --- |
| ID | string | Unique identifier of this service supplier. |
| kind | SupplierKind | Kind of supplier. |
| name | string | The human-readable name for the service supplier. |

# APPENDIX A		[Comment:  Delete Appendix A and related references.  If Appendix A is not deleted, see comments below.]  Principle Verifications Against The Energy Information Usage Model

### Requirements validation method

A complete requirements process involves a sequential breakdown of a problem starting with the collection of high level functional user requirements. The high level requirements are broken down into ever more specific derived requirements. Eventually you get to design requirements. At the bottom of the requirements tree are simple, testable, atomic requirements. When the atomic requirements are met, the tracing to the higher level requirements allows them to be met by definition.

The PAP10 charter – the text of the abstract, description, and objectives from the NIST Framework and Roadmap for Smart Grid Interoperability Standards, Release 1.0 – contains high level user requirements. [Comment:  Make REQ and WEQ consistent here.]  The requirements presented in REQ.18.3 can be considered the first level requirements breakdown of derived requirements.

This standard does not go to the final level of detailed design requirements. Instead, a parallel assessment was performed of the “derived requirements” against the evolving model, which itself was initially derived from detailed requirements identified in activities external to this standard.

Here is a sample requirement and how it is verified:

REQ 18.4.2.1.2			Usage and load information shall be readily available

While the availability of information is the realm of the utility and its customer, this model facilitates availability by providing a single simple information model for client applications.

#### General

        - Facilities may include residences, buildings, and industrial installations

No specific limit as to the nature of the facility appears in the energy usage information model. Identifiers are for user, location, and device only.

        - Usage and load information may be readily available

While the availability of information is the realm of the utility and its customer, this model facilitates availability by providing a single simple information model for client applications

        - PAP 10 is an information model concept which is transport agnostic.

The information, classes, and attributes that are contained in the energy usage information model and concepts described in this standard are data encoding and communication protocol agnostic.

#### Timeliness of Delivery

        - Customers may be able to use the information defined in these standards for real-time feedback on present and projected performance

There are no constraints on time stamps, latency or performance imposed by the model, so it is possible to represent past, present, or future usage.

        - Premises based systems (e.g., EMS/ESI) may be able to use the information defined in these standards for real-time feedback on present and projected performance

There are no constraints on time stamps, latency or performance imposed by the model, so it is possible to represent past, present, or future usage.

        - Information exchanged may be delivered in sufficient time to affect usage

There are no constraints on time stamps, latency or performance imposed by the model, so it is possible to represent past, present, or future usage.

        - Operations, Distribution, and Service Providers may be able to use the information defined in these standards with the facility in near-real-time

There are no constraints on time stamps, latency or performance imposed by the model, so it is possible to represent past, present, or future usage.

          - Customers and premises-based systems may be able to use the information defined in these standards to provide real-time feedback on present and projected performance

There are no constraints on time stamps, latency or performance imposed by the model, so it is possible to represent past, present, or future usage.

#### Benefits to Facilities

        - Standard load and usage information may enable improved energy efficiency by defining a consistent way to communicate usage information.

By minimizing the number of different physical representations of usage information, and aligning the logical elements included in the definition of this information, it will allow for the development of applications that require this information to provide energy efficiency services and functionality.

        - Standard load and usage information may enable helping [all] Customers and operations manage their energy usage [and load]

The data model standardizes load and usage information so customer usage data provider services may be developed to interface with systems that help customers and operations with energy management.

        - Standard load and usage information may enable improved [facility] energy usage by availability of Fine Grained and timely information

The data model standard provides for measurement intervals enabling the availability of Fine Grained and timely load and usage information.

        - Availability of Fine Grained and timely information may enable better decisions about energy usage and conservation

The data model enables a standard approach for Fine Grained and timely load and usage information so customer usage data provider services may interface to systems that help customers and/or operations with energy conservation.

        - Facilities may benefit from consistent usage information exchange inside the facility, including meeting the energy efficiency goals of EISA 2007 and DOE initiatives

The data model provides consistent energy usage data representation so information exchanges are uniformly understood.  This may help ensure facility energy efficiency systems using this data are acting on correct and consistent information.

        - Standard model may support aggregated projections that can be passed on to help make forecasting and management better and increase the value of a facility to smart grid

Aggregated projections are supported via future timestamps in interval and other data. The model supports aggregation based on topology, device, and owner.

REQ.18.4.2.3.7	Sharing usage and load information inside a facility can make that facility more valuable to smart grid

The data model provides usage, load, and pricing information from which a facility manager or system may take optimization actions.

- REQ.18.4.2.3.8	The information exchanged may allow integration of usage information throughout facility decision processes

The data model standardizes usage information so customer usage data provider services may be developed to integrate with systems handling facility decision processes.

#### Benefits to Grid and Service Provider Operations

        - Standard load and usage information may improve forecasting and grid management by delivering aggregated projections to operations.

By specifying future intervals, which could be larger than measured intervals, it is possible to represent aggregated projections.

        - Standard load and usage information may improve forecasting and grid management by delivering aggregated projections to service providers.

By specifying future intervals, which could be larger than measured intervals, it is possible to represent aggregated projections.

        - Standard load and usage information may enable more responsive facilities.

Interoperable models of load and usage information can allow coordination of response capability, allowing those capabilities to be utilized more efficiently.

        - Standard load and usage information may enable early deployment of devices that deliver and understand usage information.

By standardizing the information to be made available, devices and applications may be able to obtain access to that information.

#### Internet-Like Future

        - Standard usage and load information may enable innovation in novel ways to help customers manage energy usage.

By standardizing on simple data sets that many providers and consumers can utilize, this information may be combined with additional information to find the specific way that will help customers the most.

        - Standard usage and load information may enable innovation by third party service and software providers.

By standardizing on simple data sets that many providers and consumers may utilize, this information may be combined with additional information to find the specific way that will help customers the most.

#### Improved Collaboration

        - Subject to the Governing Documents and Applicable Regulatory Authority, the Information model should not restrict two way flows of information.

Subject to the Governing Documents and Applicable Regulatory Authority, customers, utilities and third parties may have access to the model in real time with the appropriate usage information and customer and utility permissions/allowances.

        - By standardizing usage and load information exchange, devices that deliver and understand usage and load may be deployed more quickly, contributing to achieving the energy efficiency goals of EISA 2007 and DOE.

Uniformity among signals may lead to greater acceptance and adoption.

#### Information Sources and Accessibility

        - Subject to the Governing Documents and Applicable Regulatory Authority, usage and load information may be provided by utilities and aggregating service providers and may be provided by others with the consent of the customer and the utility.

Subject to the Governing Documents and Applicable Regulatory Authority,with consent from the customer and the utility, utilities and aggregators may provide access to usage and load information in the provided format for model integration.

        - Subject to the Governing Documents and Applicable Regulatory Authority,usage and load information may be provided by devices and EMS/ESI implementations with the consent of the customer and the utility.

Subject to the Governing Documents and Applicable Regulatory Authority,with consent from the customer and the utility, devices and EMS/ESI implementations may provide usage and load information in the provided format for model integration.

        - Subject to the Governing Documents and Applicable Regulatory Authority,usage and load information may be accessed from the meter with the consent of the customer and the utility.

Subject to the Governing Documents and Applicable Regulatory Authority, with consent from the customer and the utility, usage and load information may be provided by the meter to accredited sources for purposes of running the model.

        - Subject to the Governing Documents and Applicable Regulatory Authority,usage and load information may be accessed from smart grid with the consent of the customer and the utility.

Subject to the Governing Documents and Applicable Regulatory Authority, with consent from the customer and the utility, usage and load information may be made available by any Smart Grid services provider via the public Internet.

#### Interactions and Information Exchanges Supported

        - Interactions supported may include those between Distribution [and Operations] and the industrial, commercial, and residential premises.

The model does not exclude/include specific actors or potential exchange points, it merely describes data and format of information.

        - Standard information models and understanding of usage and load are essential to cross domain interactions between Distribution [and Operations] and Industrial, Commercial, Residential, and PEVs.

The standard information format does not exclude potential actors or exchange points including potential cross domain interaction including those between Distribution companies and other actors.

        - Interactions supported may include those between Distribution [and Operations] and the industrial, commercial, residential premises, and plug-in electric vehicles.

The model does not exclude/include specific actors or potential exchange points, it merely describes data and format of information.

        - Standard information models and understanding of usage and load are important to cross domain interactions between Service Providers and Industrial, Commercial, Residential, and PEVs.

The standard information format does not exclude potential actors or exchange points including potential cross domain interaction including those between Service Providers and other actors.

        - Interactions supported may include those between Service Providers and the industrial, commercial, residential premises, and plug-in electric vehicles.

The model does not exclude/include specific actors or potential exchange points, it merely describes data and format of information therefore by not excluding these actors.

        - This effort may support information standards for load curtailment, load shaping, and energy market operations, hence load and usage may be supported (see PAP09, PAP03, and PAP04).

Load and usage information are part of the model.

        - Information exchanges may include to, from, and within facilities.

The model does not preclude any potential exchange points therefore the requirement is met.

#### Information Characteristics

        - Information model may support exchange of both Fine Grained and summary information.

Summary information is available at the top level through PowerQualitySummary and UsageSummary components. Fine Grained information is available through the list of MeterReading measurement structures which in turn have both summary, Reading, and interval level detail, IntervalReading components. Additionally, the energy usage information data structures may be associated by device, location, and owner facilitating variable aggregations.

        - Fine Grained means that there is disaggregated information. Disaggregated information may include for example load, subsystem, premise, and variable time interval.

Fine Grained (disaggregated) information is described throughout the information model.  From CustomerAccount, EventType, MeterAsset, ReadingKind, etc. there seems to be enough detailed data elements to support “Fine Grained” information.

        - Support for exchanging standard historical, present, and projected load information is possible.

Historical information seems to be supported by the UsageSummary attributes for billing items only. Present (meter reading) information has Fine Grained as well as billing attributes defined.  Projected usage may be supported in the QualityOfReading <<enumeration>> attribute, which delineates the “estimated” and “forecast” attribute names. Historical, present, and projected load information is implicitly available at the interval and reading level based on the time stamps utilized -- past/present/future.

        - Energy usage information may include usage, usage profile, and some component of cost (consistent with PAP03, PAP04).

Energy usage information, including costs is contained in the data model.  Interval data is supportive of usage profiles (PAP05 and PAP04).

REQ.18.4.2.9.5	The PAP 10 energy usage information model may allow for exchange of greater or lesser detail.

Summary information is available at the top level through PowerQualitySummary and UsageSummary components. Fine Grained information is available through the list of MeterReading measurement structures which in turn have both summary, Reading, and interval level detail, IntervalReading components. Additionally, the energy usage information data structures may be associated by device, location, and owner facilitating variable aggregations.

#### Timing and Goals

        - Initial steps may include making usage information more readily available by defining and standardizing usage information [delivered] through existing Smart Grid infrastructure.

The energy usage information data structures enable a diverse range of technologies to deliver usage information in a timely and protocol agnostic manner.  Examples of this might be a premise ESI or through services provided by a utility,  facility, aggregator, energy services provider, or others.

        - Information on device and facility usage may be a primary goal in the initial focus.

As the energy usage information data structures cover summary and device level information, device and facility (aggregated) information may be delivered.

        - Standard load and usage information may enable early deployment of devices that deliver and understand usage information.

As the energy usage information data models are standardized, producers may deliver usage based devices with confidence in interoperability.

#### Requirements on Quality of Consensus Standard

The information model can support:

        - Consistent data representation for REST & Web Services.

This requirement will support REST and Web Services. However, other transports may be supported such as, but not limited to, IEEE 1703/ANSI C12.22 architecture per AEIC Guidelines V2.0 . See REQ.18.3.3 which implies support for alternative syntaxes.

        - Specification of transactional exchange, syntax, and required population of the information model are beyond the scope of these requirements.

No transactional syntaxes or message services are defined in this standard.

        - An extensible information model – e.g., ability to add custom extensions as needed

This model may be extended by adding new elements (classes, attributes, and associations). Implementations must utilize conventions to ignore any elements not understood by components, to allow them to continue to operate when future extensions are added.

        - Having an evolvable information model – e.g., the standards process supports future revisions

REQ.18.3.3 identifies how the model is evolvable. The reference in REQ.18.4.2.11.16 defines how the standard version can be revised.

        - Forward compatibility as we evolve

To maintain forward and backward compatibility, best practicies on extensibility, which also includes evolvability through standards versions, may be followed.

        - Versioning

Versioning in the UML Model is addressed by a tagged value.

        - Usability without “knowing all the details”

Information in the model is arranged hierarchically. Greater detail can be understood the deeper into this hierarchy the reader goes. Summary information is exposed near the top of the hierarchy. These arrangements make it straightforward to accessing applications to ignore the level of detail they do not plan to use or comprehend, while getting value out of higher level information.

More flexibility for independent innovation can be achieved through:

        - Focus on information exchanged

The specification of an information-only model allows the standard to focus on the information exchanged.

        - Agreed upon interfaces are maintained over time

A profile of this energy usage information model supports the exposure of an interface. One is provided as an example in this standard. REQ.18.4.2.11.16 verification references how the information model may be maintained. See REQ 18.3.3 which describes how the energy usage information model may be used as the basis for Smart Grid interfaces.

        - Minimal details = maximum interoperation

The model provides a hierarchy of summary and detailed information. A balance is achieved by the definition of minimal set of details necessary to satisfy the cumulative set of use cases of the energy usage information and allocated to this model.

        - Intellectual Property Rights

REQ.18.4.2.11.7,  REQ.18.4.2.11.14, REQ.18.4.2.11.15, REQ.18.4.2.11.16, and REQ.18.4.2.11.17 describe the IPR rights to use of the energy usage information model defined by this standard.

Results of PAP 10 can produce:

        - Information model and XML schema

There is no normative schema in this standard. There is an "example" non-normative schema to demonstrate how to take the energy usage information model and create a profile that can generate an XML schema or other descirption.

        - At information exchange level

The information model is designed to be exchanged between participants. The information model and XML Schema provide the specificity to define an information exchange, but stops short of mandating one (see REQ.18.3.3).

The information model should be:

        - Readable without charge

NAESB has agreed to make the energy usage information model included in REQ.18.4 and REQ.18.4.1 available to the public without charge.

        - Reusable without restriction or charge

The use of the energy usage information model included in REQ.18.4 and REQ.18.4.1 for the production of derivative work products is not prohibited.

        - Adaptable without restriction or charge

All NAESB standards may be modified through the NAESB process. (Please see: http://www.naesb.org//misc/naesb_process_for_standards_dev.doc)

        - Usable for open source

The use of the energy usage information model included in REQ.18.4 and REQ.18.4.1 may be distributed in any manner, however, such model must be made available without charge.

#### Additional Considerations (not provided by SGIP PAP10 Working Group)

        - In communicating energy usage information, the energy usage information model may be used and the information outlined within the model may be available.

The model provides a common, consensus-based, vendor-neutral model to represent usage information, to be used in exchange scenarios where it is applicable and desired by usage information providers and Customers. REQ 18.3.3 elaborates the technical considerations for constructing implementing standards which do specify syntax.

        - The energy usage information model data set has three unique identifiers, which serve the purpose of allowing various aggregations of data sets.

The model has the class ServiceDeliveryPoint which has an ID. Additionally, it has an association with Customer and MeterAsset which support aggregation by location, owner, and device.

Owner (customer): This is represented by the Customer class, provided the use of the term “owner” or similar verbiage is not indicative or dispositive of any ownership or other rights under applicable law in the model or any data.

Location: This is represented by ServiceDeliveryPoint.

Device: This is represented by MeterAsset.

        - The TariffProfile component may be included in the energy information usage model.

TariffProfile may be included in the energy usage information model  as a reference. This reference supports the potential acquisition of details on the agreements between supplier and consumer of energy when this optional reference is included and populated.

        - UsageSummary may include an optional cost attribute to interval and reading class.

UsageSummary has a cost roll up and optional cost attributes in interval and reading classes and may be included in the model.

        - A known base currency may be included in the top level class associated with the MeterAsset for the energy usage information model, which would be applicable to all instances.

A base currency attribute is part of the UsageSummary class based on ISO 4217.

        - The energy usage information model may be compliant with ISO8601.

The current model does not define the primitive to be used to represent the dateTime, however the default xs:dateTime is ISO8601 compliant.

        - Optionally, both start and end of interval can be defined in order to support non-uniform interval information, which is accomplished in the energy information usage model through endTimestamp to IntervalReading.

The endTimeStamp element is included as an optional component of IntervalReading.

        - Demand based elements are maintained in the TariffProfile including common demand and demand ratchets.

The model does not currently contain the full definition of TariffProfile, but this is the CIM element where thresholds associated with the rate are specified.

        - The energy information usage model status structure includes a named pair of QualityOfReading and values of raw, forecast, validated, estimated, mixed, and other for qualifying the associated data set, as an explicit representation of these name value pairs can be processed in implementations by sending and receiving actors. The UsageSummary class also includes such summary information.

Model is specified as described.

        - MeterEvent can be associated with MeterAsset.

- MeterEvent is not part of the energy usage information model. This will be forwarded to IEC 61968 Committtee for resolution.

        - PowerQualitySummary as events of enumerated type are provided in the energy information usage model to represent a simple exposure of summary information.

The model exposes these events in summary form as described.

        - With respect to devices monitoring or capable of monitoring point source emissions, the energy usage information model may represent and take into account pollutant emissions (e.g., SO2, NOX, etc.).

Detailed information about these emissions is possible through definition of a ReadingType specifying the appropriate kindReading values as enumerated in ReadingKind.

        - Energy usage information model may include a top level cost summary – billStart, billEnd, billToDate, lastPeriod, and costAdditional, so that the bill to date and bill as of the last billing period could be conveyed through a formula:

billLastPeriod = costAdditionalLastPeriod + ∑MeterReading[i].Reading.cost (constrained by datetime)

The model represents these elements in the UsageSummary class.

- 4.  SUPPORTING DOCUMENTATION

- Description of Request:

- Description of Recommendation:

- Business Purpose:

- Commentary/Rationale of Subcommittee(s)/Task Force(s):

NAESB Process for Standards Development - http://www.naesb.org//misc/naesb_process_for_standards_dev.doc

- PAP 10 Charter

See e.g., EIS Alliance Use Cases v2 including UC-9, UC-11, UC-12, UC-14, UC-15, ZBHP_SE_MRD 3.1 "HAN Device Information Retrieval Request", and OpenADE use case “Publication”.

See e.g., EIS Alliance Use Cases v2 including UC-3, UC-11, UC-14, UC-15, and OpenADE use case “Publication”

Needs refs. This is an addition to EIS and OpenADE.

See e.g., EIS Alliance Use Cases v2 including UC-1, UC-2, UC-3, UC-8, UC-9.

See e.g., EIS Alliance Use Cases v2 including UC-1, UC-2, UC-3

http://www.nist.gov/public_affairs/releases/upload/smartgrid_interoperability_final.pdf