Some of the participants in the PAP10 NAESB standards effort have been meeting with some of the members of the IEC 61968 Part 9 team this past month. We had identified a series of topics where, it was suggested, improvements in the compatibility of the NAESB standard with CIM could be achieved. Given the rather short timescale, progress on these topics was made through the serious work of both teams. However, we could not succeed in addressing all issues. We are, as a result, recommending the following modifications to the NAESB PAP10 Standard information model. There may be additional recommendations coming from the CIM team:

ID

IdentifiedObject.name should replace ID, as recommended by the IEC. The extended IdentifiedObject class hierarchy, including Name, NameType, and NameAuthority, should also be included to allow full qualification of identifiers.

UsagePoint

UsagePoint class will be added to CIM and should replace ServiceDeliveryPoint in the NAESB PAP10 model. It should retain the same associations we have to SDP, as well as an association to SDP. This is recommended because ServiceDeliveryPoint has a description that implies it is primarily for the boundary of ownership of the customer site, which is too limiting. UsagePoint is a more generalized class appropriate for representing any point in the power chain through to an appliance, and could be used to refer to virtual meter points, such as street lights.

CIM Extension

CIM evolution style guides provided by CIM team should be used as a guide in the construction of the final NAESB model. A prototype model is attached in this document representing a starting point for the updated model, which was built according to the CIM modeling team recommendations.

IntervalBlock and CIM Part 9 Associations

CIM IntervalBlock should be included between MeterReading, and IntervalReading, and ReadingType, for compatibility with Part 9, and ability to include multiple types of readings in one MeterReading.

To Do:

ReadingType

CIM Part 9 team to incorporate C12.19 enumerated values to harmonize with C12.19 as far as possible.

Association from MeterReading to ReadingType

Further discussion is needed to determine whether this association needs to be a NAESB PAP10 extension or can be migrated back into the 61968 Part 9 base.

Summary Reading Types

The attached model retains the recommendation to represent summary measurements (e.g. currentDayLastYearNetConsumption) as ReadingTypes, however this will need to be evaluated for feasibility and coherence once the ReadingType lists are finalized.