1.	RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| X | Accept as requested | X | Change to Existing Practice |
| --- | --- | --- | --- |
| X | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

2.	TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
|  | Initiation |  | Initiation |
| X | Modification | X | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
| X | Principle | X | Principle |
| X | Definition | X | Definition |
| X | Business Practice Standard | X | Business Practice Standard |
|  | Document |  | Document |
|  | Data Element |  | Data Element |
|  | Code Value |  | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
|  | Business Process Documentation |  | Business Process Documentation |

3.	RECOMMENDATION

SUMMARY:

The business process flows, use cases and data requirements presented in this document illustrate the standard interactions between Distribution Company Operator, Service Provider (Demand Response) and Retail Customer for the administration and deployment of Demand Response Resources for retail markets.

(Isn’t this redundant??)

In response to NIST’s Priority Action Plan 9, this document provides requirements specification, in the form of business process flows, use cases and data requirements in certain areas, to support the standardization of the information exchange between Distribution Company Operator, Service Provider (Demand Response) and Retail Customer. This document shares a standard set of actors and terminologies with the companion Requirements Specification for Wholesale Electric Quadrant Standard Demand Response Signals.

(Moved to appropriate location in the document)

Recommended Standards:

# Specifications for Retail Standard Demand Response Signals

# Executive Summary

# The North America Energy Standards Board (NAESB) Smart Grid Standards Subcommittee  took the responsibility of consolidating and developing Demand response (DR) use cases that provide requirements for developing DR control and pricing signal standards, which is called for by National Institute of Standards and Technology (NIST) Priority Action Plans (PAP) 03 and 09.  This document addresses the business and data requirements for standardizing control and pricing signals for Retail level DR and Distributed Energy Resources (DER) as part of the Smart Grid implementation.   Wholesale Electric market DR and DER information exchanges are outside the scope of this document and are addressed in the Requirements Specification for Wholesale Electric DR signals. The first step of use case development is the development of the Framework for Integrated DR and DER Models document, which provides an overall business context for DR and DER Models.

According to the Framework Document findings:

- DR signals standardization must support all four market types; i.e. regions with a) no open wholesale and no retail competition, b) open wholesale market only, c) open retail competition only, d) open wholesale and open retail competition. It must also consider key differences that exist and will continue to exist in all four market types.

- Wholesale market DR and pricing signals have different characteristics than retail market DR and pricing signals, although commonality in format is feasible.

- Most Retail Customers (with a few exception of Commercial and Industrial (C&I) Retail Customers) will not interact directly with wholesale market when it comes to DR and pricing signals.

- Retail pricing models are complex, due to the numerous tariff rate structures that exist in both regulated and un-regulated markets.  Attempts to standardize DR control and pricing signals must not hinder regulatory changes or market innovations when it comes to future tariff or pricing models.

- New business Entities (Energy Service Providers (ESP), Demand Response Providers (DRP), Service Providers, and Energy Information Service Providers (ESIP)) as well as new Retail Customer energy management systems will play an increasing role in DR implementation.

- DER may play an increasingly important role in DR, yet the development of tariff and/or pricing models that support DER’s role in DR is still in its infancy.

- The Retail Customer’s perspective and ability to react to DR control and pricing signals must be a key driver during the development of DR standards.

The following deals exclusively with the retail level Demand Response signals requirements.  The requirements are captured in the form of UML models, where business requirements are captured in use case narrative format, data requirements are captured in tabular format, and information exchange flows are captured in sequence diagram format.  The top level use cases for retail DR are:

- Administrate DR program

- Administrate Retail Customer for DR

- Administrate Demand Response Resource

- Execute Demand Response Event

- Post Demand Response Event management

# Introduction

# (Redundant)

These Model Business Practices and the data elements included herein are not intended to replace applicable Governing Documents, and in the event of a conflict, the latter documents shall have precedence over these Model Business Practices.

Purpose: The purpose of the following Model Business Practices is to capture business and data requirements for Retail Level DR signals between Entities that control and manage the DR programs and Entities that provide Demand Response with Demand Response Resources and assets.  This is to fulfill the NAESB SGTF responsibility under the NIST Smart Grid Interoperability Standards development PAP09.  This requirements document will then be used by other standards development organizations identified by NIST PAP09 for further development of DR signals technical specifications for all Retail Customer markets.

Scope: The scope of these Model Business Practices is to provide business and data requirements for standardizing Demand Response signals at the retail (distribution) level.

### (Shouldn’t these be a definitions???)

### Commentary on Distributed Energy Resources:  In the context of this document, Distributed Energy Resources (DER) are dispatchable energy generation and storage technologies, typically up to ten MWs in size, that are interconnected to the distribution grid to provide electric capacity and/or energy to a Retail Customer or a group of Retail Customers and potentially export the excess to the grid for economical purposes. DER systems can generate electric power, as opposed to curtailable or interruptible loads that can just reduce electric loading on the grid.

DER may be subdivided into distributed generation (DG), distributed storage, and plug-in electric vehicles (PEVs):

Distributed generation (DG) technologies can be divided into a) Renewable Generation, e.g., small wind power systems, solar photovoltaic (PV), Fuel Cells, Bio-fuel generators and Digesters, and other generation resources using renewable fuel, and b) Non-Renewable Generation, e.g., micro-turbines, small combustion turbines, diesel, natural gas and dual-fueled engines, etc.

Storage resources could be divided into a) Electric Storage, e.g., battery systems and uninterruptible power supplies (UPS), flywheel, superconducting magnetic energy storage (SMES), etc. and b) Thermal Storage – These convert electric power to thermal (cooling/heating) energy for later use, e.g., Ice-based air cooling system, and high-capacity brick-based air heating system.

Plug-in electric and hybrid vehicles (PEV/PHEV) may also be considered as DER, The electric vehicles may be used to supply stored electric energy back to the grid, a.k.a. Vehicle-to-grid (V2G).

Behind-the-meter DER may be bundled with regular load and managed alongside the Demand Response Resources – such as a residential roof-top PV solar panel. But often, DER is treated separately in part due to its control capabilities.  In addition to a regular retail tariff, behind-the-meter DER may be subject to net-metering or feed-in tariff, where excess generation can be exported to the grid at an established or a dynamic price.

Similar to Demand Response Resources, DER assets need to be registered and enrolled into a DR program.  Furthermore, DER assets are typically required to meet additional technical requirements and certification for grid interconnection. Depending on the size of a DER and its export capabilities, sub-metering and telemetry capabilities may be required to monitor the impact of the DER operation on the distribution grid reliability and power quality.   Also, renewable resources may receive Renewable Energy Credits (RECs) and may also qualify as a must-run resource, e.g., wind power in some regions.  These need to be incorporated into pricing and control signals associated with DER operation.

(Need to add the boiler-plate language)

The North American Energy Standards Board (NAESB) is a voluntary non-profit organization comprised of members from all aspects of the natural gas and electric industries. Within NAESB, the Retail Electric Quadrant (REQ) and the Retail Gas Quadrant (RGQ) focus on issues impacting the retail sale of energy to Retail Customers. REQ / RGQ Model Business Practices are intended to provide guidance to Distribution Companies, and other Market Participants involved in providing energy service to Retail Customers.

These Model Business Practices are voluntary and do not address policy issues that are the subject of state legislation or regulatory decisions. These Model Business Practices have been adopted with the realization that as the industry evolves, additional and amended Model Business Practices may be necessary. Any industry participant seeking additional or amended Model Business Practices (including principles, definitions, data elements, process descriptions, and technical implementation instructions) should submit a request to the NAESB office, detailing the change, so that the appropriate process may take place to amend the Model Business Practice.

REQ.17.1	Principles

REQ.17.2	Definitions

REQ.17.2.A	Business Definitions (Reserved)

REQ.17.2.B	Techincal Definitions

New REQ.17.2.B Defined Terms to be added to REQ.17 are included in the Recommendation Request Titled Phase Two Requirements Specification for Common Electricity Product and Pricing Definition – for NIST PAP03 (REQ.15) to avoid redundancy between Model Business Practices.

Note that beginning at REQ.17.3.2 additional terms are defined.

REQ.17.3	Model Business Practices

REQ.17.3.1	Genral Practices for Specifications for Retail Standard Demand Response Signals

The following describes general business processes for implementing Demand Response capacities at the retail market level.  The following principles apply to the development and application of these business process requirements.

| # | Name | Description | Rationale | Implication |
| --- | --- | --- | --- | --- |
| 1 | Scalability | The architecture should allow for the management of millions of Retail Customer premises devices | Scale-out, adding more low-cost machines, has proven to be superior to scale-up, using larger single machines, as a way to handle large loads. For example, high traffic web sites use farms of low-cost machines to serve pages to millions of simultaneous web users. | Architecture should allow the solution to scale-out as more devices are added. |
| 2 | Actionable, testable, and transferable work products | Any work (artifacts) that are created can be used by the audience for this work, e.g., Distribution Companies, vendors, regulators, or other Market Participants. There needs to be clear, explicit guidance for how to use the artifacts. There is an expectation that the work products are useful at lower levels of design. | Such work products will promote market adoption and minimize the cost and risk of adoption, leverage open and best practices, and establish repeatable processes, patterns, and templates for all work products. | The use of common tools and methods will be fostered. Organizations that do not follow the use of the common tools and methods may have more difficulty implementing the artifacts. |
| 3 | Platform Independence, Vendor Neutral | Requirements and design artifacts shall be platform independent. Implementation technology shall be chosen due to its level of acceptance at the marketplace as open standards. | There is an expectation of differentiation and innovation in the marketplace. With greater dependence on a specific platform, there may be less architectural flexibility. | To achieve both technical and semantic interoperability, certain lower level technologies will need to be chosen. For example, Web Services technology is widely used for integration, and UML is widely used for modeling. |
| 4 | Cyber Secure | Architecture must incorporate latest secure computing techniques. | The integrity of the electric grid and market depend on preventing unauthorized actors from manipulating the DR and DER system. | Architecture must comply with the latest security guidance from the NIST SGIP Architecture Committee and CyberSecurity Task Group, when it comes to Web Services security related implementation. |
| 5 | Interoperability | DR signaling standards should utilize open, interoperable industry standard control and communication technologies to integrate with both common energy management and control systems that can conduct DR activities | Interoperability across different domains of Smart Grid (Not defined) is key to the success of DR signaling standards as it ensures level playing field for innovative solutions for all participants of the Smart Grid. | DR signaling standards must be developed in harmonization with others relevant standards as identified by the NIST SGIP. |
| 6 | Automatic Demand Response | The DR program (analysis, notification, event, action, monitoring, verification, settlement) should be automatic. DR signaling standards should provide secure, reliable communications with consumers for DR operations | Automation is the only way to ensure large scale and repeatable Demand Response implementation. | Security, reliability and testability must be ensured for DR automation. |
| 7 | Cover All Market Types | DR signaling standards should be supported and implementable in all four market types listed in this document | Standards development shall not get in the way of regulatory policy decisions. | DR signals design need to support a wide range of programs and tariff/pricing structures. |

## REQ.17.3.1.1	Conventions

The use case modeling technique is used for capturing business and date requirements relative to Retail Demand Response.  There are two basic diagrams used in this document.  The first one is the use case diagram, which shows the scope of business use cases for a given business scenario.   The relationships used in the use case diagrams are as follows:

- Include:  this indicates additional scenarios

- Extend: this indicates alternatives scenarios

- Generalization (line with triangle head): this indicates variations (sub-types) of the main use case.

- Precedes: this indicates a time sequence

- Invokes: this indicates a pre-condition or dependency

The second is the sequence diagram, which shows the information exchange flows between actors for a given use case.  The message flows are shown in one way with the convention of verb and noun, verb indicating the action being taken, noun indicating the name of the information being exchanged.  The return message flow is the acknowledgement of the main message flow.

REQ.17.3.1.2	Actors

The actors listed in the table below are specific to these Model Business Practices and are a subset of the actors and related objects defined in REQ.15. Details on the relationships between the actors and related objects are further defined in Appendix A (Question whether this should be here or only listed in REQ.15).

Table 1 - Actor Roles

| Term | Actor ID |
| --- | --- |
| Communication Method | 5.5 |
| Control | 5.2 |
| Designated Dispatch Entity | 3.4 |
| Designated Dispatch Entity (DDE) | 3.4 |
| End Device (ED) | 4.5 |
| Enrollment | 5.3 |
| Environmental Authority (EA) | 1.3 |
| Federal Regulator (FR) | 1.1 |
| Load Serving Entity | 3.2 |
| Load Serving Entity (LSE) | 3.2 |
| Local Authority (LA) | 1.5 |
| Market Participant (MP) | 2.2 |
| Measurement | 5.1 |
| Metering Authority | 3.6 |
| Metering Authority (MA) | 3.6 |
| P-Node | 5.6 |
| PremiseService Location | 4.3 |
| Reliability Authority (RA) | 1.2 |
| Resource | 4.1 |
| Scheduling Entity | 3.3 |
| Scheduling Entity (SE) | 3.3 |
| Service Provider | 3.1 |
| Service Provider (SP) | 3.1 |
| State Regulator (SR) | 1.4 |
| System Operator | 2.1 |
| Transmission/Distribution Service Provider | 3.5 |
| Transmission/Distribution Service Provider (TDSP) | 3.5 |
| Retail Customer (RC) | 2.4 |
| Distribution Company Operator (DCO) | 2.3 |
| Zone | 5.4 |

## REQ.17.3.1.3	Use Case Overview

Similar to the Wholesale Electric Demand Response use cases, Retail Electric Demand Response use cases generally follow five main steps, see the Figure 1.

- Administrate DR program

- Administrate Retail Customer for DR

- Administrate Demand Response Resource

- Execute Demand Response Event

- Post Demand Response Event management

There are many programs that exist in today’s retail markets with or without open competition.  New programs will be defined and deployed in the future.  The goal of this use case analysis is to find commonalities among these programs so that the derived DR signal messaging specification can be implemented to support a large number of DR programs.

The focus of the standardized DR signals is the step 4, execute DR event, where the DR signals will be communicated between the DR providers and the Demand Response Resources.  All other use cases are supporting materials to ensure that DR use case requirements are documented and understood in a complete picture.

The use cases included in this document are not intended to be required or exhaustive and are provided for clarification purposes.

Figure 1. Retail Market DR Use Case Overview

### REQ.17.3.2	Specific Use Cases (Note: the following terms are defined herein and are in addition to those listed as Technical definitions in REQ.17.2.B)

###### REQ.17.3.2.1	Administrate DR Program

DR program administration related use cases are determined by the SGTF as out of scope for the NIST PAP09 purpose, and therefore the original content has been removed from these Model Business Practices.  NAESB may start a new project to deal with the Retail Customer Enrollment and DR program administration related business practices.

###### REQ.17.3.2.2	Administrate Customer for DR

DR program Retail Customer Enrollment related use cases are determined by the SGTF as out of scope for the NIST PAP09 purpose, and therefore the original content has been removed from these Model Business Practices.  NAESB may start a new project to deal with the Retail Customer Enrollment and DR program administration related business practices.

###### REQ.17.3.2.3	Administrate DR Resource

DR Resource is an equivalent of Resource defined in the actor list table.

As part of Enrolling Retail Customers in a DR program, specific Demand Response Resources and assets that are associated with Retail Customers’ accounts and premises will also need to be registered.  Figure 4 shows the actors and use cases involved in administrating Demand Response Resources and/or assets into DR programs. The following use cases deal with the registration, update, and removal of Demand Response Resources and assets associated with a Retail Customer.  When updating Demand Response Resources, there will also be potential use cases of DR bidding process, either for supplying or for buying power.  There are also cases where Distributed Energy Resources will act as DR assets.

Figure 2. Administrate DR Resources and/or Assets into DR Programs

###### REQ.17.3.2.3.1	Administrate Distribution DR Resource

These activities are used to administer all of the DR Resources and Assets, including a variety of activities surrounding the registration of DR Resources and the management of information surrounding the DR Resources that a Service Provider may call upon during a DR Event. Before a DR Resource may be called upon, it must be registered with the Service Provider.  The administration of a DR Resource covers the following requirements:

- The need of the Service Provider to identify a DR Resource for communications purposes;

- The need of the Service Provider to establish DR Resource accounts for accounting purposes;

- The need of the Service Provider to collect information from DR Resources prior to DR Events for the purposes of determining which DR Resources to call upon during a DR Event; and

- The need of the Service Provider to collect information from DR Resources prior to DR Events for the purposes of determining the nature of the DR signals to send to the DR Resources.

###### REQ.17.3.2.3.2	Update DR Resource

REQ.17.3.2.3.2.1	Description

When the base capacity of a DR Resource changes from when it was last registered with the DR provider, Retail Customer needs to provide updated information to the DR provider so that such information can be factored into the consideration for DR Event planning.

REQ.17.3.2.3.2.2	Data Requirements

| Attribute Name | Description |
| --- | --- |
| Resource ID | Identifier assigned to the Resource |
| Resource Type | Type of Resource. Examples are: Load reduction (curtailable or interruptible load), generation, combination, DG, renewable, and storage. |
| Transmission/Distribution Service Provider Account Number | Transmission/Distribution Service Provider's (Distribution Company) account number for the Resource. |
| Demand Response Provider | The Entity that is responsible for delivering Demand reductions from Demand Resources and is compensated for providing such Demand Response products. |
| DR Resource Group ID | Grouping of Resources that can respond to the same DR Signal. |
| Program Identifier | The identifier of the program |
| Constraint Type | Type of operational, schedule or offer constraint |
| Constraint Interval | The timeframe over which the constraint type applies. |
| Constraint Value | Value of the Constraint Type and Interval |
| Effective Enrollment Date | Effective Start Date for the Enrollment (This is a defined term. The correct definition should be used) |
| Location | The location of a Resource defined by one or more of the following attributes: |
| Location Attributes |  |
| Service Location ID | Identifier assigned to the Service Location |
| Address1 | Address line 1 |
| Address2 | Address line 2 |
| City | City |
| State/Province | State or Province two-letter code |
| Zip/Postal Code | Zip or Postal Code |
| Country | Country |
| GPS Coordinates | Latitide and longitude |
| Weather Station | Weather Station code associated with the Service Location |
| Zone ID | Identifier assigned to the Zone in which the Service Location is located |
| Zone | Name of the Zone in which the Service Location is located |
| Electrical Node ID | Identifier assigned to the Electrical Node at which the Service Location is connected |
| Electrical Node Name | Name of the Electrical Node at which the Service Location is connected |
| Electrical Node Type | Type of Electrical Node at which the Service Location is attached |
| PNode | Name of the Price Node associated with the Service Location |
| PNode ID | Identifier assigned to the Price Node associated with the Service Location |
| End of Location Attributes |  |
| Response Time | The amount of time before an asset/Resource is capable of meeting its full performance, in response to a request by a Service Provider to shed load. |
| Capacity Type | Type of Capacity |
| Capacity Type description | Description of the Type of Capacity |
| Nominal Capacity | Nominated load |
| Offer Commit Status. | Commitment status of offer Product-specific commitment status types included with offer. Examples include: economic, emergency, must-run, not participating, outage run status, override status, etc. Also referred to as "opt-in" or "opt-out" for existing Demand Response Programs. |
| Resource Type | Type of Resource Examples are: load reduction (curtailable or interruptible load), generation, combination, DG, renewable, storage, etc. |
| Resource Qualification Test Date | Date the Resource demonstrated its ability to deliver a product or service |
| Requalification Test Date | Date the Resource will retest its ability to deliver a product or service |
| Enrollment End Date | Date of Termination of Enrollment (There is a defined term – Drop Effective Date – which covers this) |
| Enrollment Status | Status of the Enrollment for the Resource |

Sequence Diagram

###### REQ.17.3.2.3.3	Register DR Resource

REQ.17.3.2.3.3.1	Description

A party with ownership, controlling interest, or administrative responsibilities for a DR Asset or DR Resource that communicates status-related operational information about the DR Asset or DR Resource to a Service Provider.   For example, the owner of a DR Resource or Asset may wish to declare their inability to shed load (an outage) due to summer shutdown or may wish to reduce the available capacity of a resource/asset due to equipment maintenance or other causes.

REQ.17.3.2.3.3.2	Data Requirements

The Data Requirements are the same for Register / Enroll DR Resource and Update DR Resource.

Sequence Diagram

###### REQ.17.3.2.3.4	Remove DR Resource

REQ.17.3.2.3.4.1	Description

When a DR resource is no longer available permanently for DR purpose, it needs to be removed from the DR program for that customer so that it will be accounted for DR Event planning.

REQ.17.3.2.3.4.2	Data Requirements

| Attribute Name | Description |
| --- | --- |
| Resource ID | Identifier assigned to the Resource that is removed. |
| Program Identifier | The identifier of the program |
| Enrollment End Date | Date of Termination of Enrollment Should use Drop Effective Date) |
| Enrollment Status | Status of the Enrollment for the Resource (Set to ‘REMOVE’) |

Sequence Diagram

###### REQ.17.3.2.4	Administrate DR Asset (Direct)

###### REQ.17.3.2.4.1	Register DR Asset

REQ.17.3.2.4.1.1	Description

The DR Asset registration process must capture key identifiers to enable accurate tracking of DR assets and their capabilities. A requirement for a System and Market Operator, Load Serving Entity, Service Provider, or other Entity facilitating the registration process (hereinafter referred to as DR Asset Administrator (Defined ??)) is to track assets. This is done through DR asset registration and association of physical DR assets to DR Resources to recognize the asset and its potential contribution as part of a DR Resource.  The DR Asset Administrator ultimately administers the DR Asset registration process to recognize DR Assets that can serve as part of a DR Resource, although the Retail Customer may be party to the registration process.

REQ.17.3.2.4.1.2	Data Requirements

| Attribute Name | Description |
| --- | --- |
| Asset ID | The unique identifer of the asset |
| Program Identifier | The identifier of the program |
| DR Asset Group ID | Grouping of Assets that can respond to the same DR Signal within a DR Resource. (See DR Resource Specification) |
| Asset Operator ID | The identifier of the business Entity that operates the DR assets. This is the Entity that has physical control of the asset and control of the capability to participate in DR Events. |
| Asset Operator Name | The name of the business Entity that operates the DR assets. This is the Entity that has physical control of the asset and control of the capability to participate in DR Events. |
| Asset Owner ID | The Identifier of the business Entity that owns the DR assets (Entity that has the right to use or transfer use of the asset). |
| Asset Owner Name | The Name of business Entity that owns the DR assets (Entity that has the right to use or transfer use of the asset). |
| Location | The location of a Resource defined by one or more attributes: |
| Service Location ID | Identifier assigned to the Service Location |
| Address1 | Address line 1 |
| Address2 | Address line 2 |
| City | City |
| State/Province | State or Province two-letter code |
| Zip/Postal Code | Zip or Postal Code |
| Country | Country |
| GPS Coordinates | Latitide and longitude |
| Weather Station | Weather Station code associated with the Service Location |
| Zone ID | Identifier assigned to the Zone in which the Service Location is located |
| Zone | Name of the Zone in which the Service Location is located |
| Electrical Node ID | Identifier assigned to the Electrical Node at which the Service Location is connected |
| Electrical Node Name | Name of the Electrical Node at which the Service Location is connected |
| Electrical Node Type | Type of Electrical Node at which the Service Location is attached |
| PNode | Name of the Price Node associated with the Service Location |
| PNode ID | Identifier assigned to the Price Node associated with the Service Location |
| Effective Enrollment Date | Effective Start Date for the Enrollment (Registration) (Existing definition should be used – Enrollment Effective Date) |
| Enrollment Status | Status of the Enrollment for the Asset |
| Offer Commit Status | Commitment status of offer Product-specific commitment status types included with offer. Examples include: economic, emergency, must-run, not participating, outage run status, override status, etc. Also referred to as "opt-in" or "opt-out" for existing Demand Response Programs. |
| DR Asset Physical Capabilities | Asset Capabilities include Ramp Rates and Capacity. |
| Ramp Rate Value | Ramp Rate associated with the selected Ramp Rate type for the selected Ramp Rate segment and Ramp Rate direction |
| Capacity Type | Type of Capacity |
| Capacity Type description | Description of the type of capacity |
| Nominal Capacity | Nominated Load |
| Asset Manufacturer | Manufacturer of the Asset |
| Asset Model | Model identifier of the Asset assigned by the manufacturer |
| Asset Version | Model versioniIdentifier assigned by the manufacturer. |
| Asset Manufacture Date | Date of manufacture of the Asset |
| Asset Type | Type of Asset. Examples are: Load reduction (curtailable or interruptible Load), generation, combination, DG, renewable, storage, etc. |
| DR Resources | The identifier of DR Resources that the DR Assets belong to. An Asset can associate with multiple Resources, but with only one Resource for a DR program. |

Sequence Diagram

###### REQ.17.3.2.4.2	Update DR Asset

REQ.17.3.2.4.2.1	Description

Once a DR Asset registration is completed, changes to elements of the registration may be required or desired. After identifying the specific required modification and verifying the authorization to perform the update, each of the considerations identified in the DR Asset registration step should be evaluated and utilized, where necessary, during the update of the DR asset information. The ability to update DR asset information helps to ensure the asset information on record is current and complete.

Parties that are privy to and rely on the update should be notified of updates as soon as practicable after they are made.  Additionally, these parties should have access to review and, in some cases, to approve/disapprove the update(s).

REQ.17.3.2.4.2.2	Data Requirements

The Data Requirements are the same for register / Enroll DR Asset and update DR Asset.

Sequence Diagram

###### REQ.17.3.1.4.4.3	Remove DR Asset

Description

Removing a DR Asset must take into account any DR Resource long term commitments in Program Administrator programs and contractual obligations with Service Providers or Load Serving Entities.  In addition, Applicable Regulatory Authority filings and approvals must be considered prior to removing a DR Asset from a DR program.

Data Requirements

| Attribute Name | Description |
| --- | --- |
| Asset ID | The unique identifer of the asset |
| Program Identifier | The identifier of the program |
| Enrollment End Date | Date of Termination of Enrollment (Should use Drop Effective Date) |
| Enrollment Status | Status of the Enrollment for the Asset (‘REMOVE’) |

Sequence Diagram

###### REQ.17.3.2.5	DR Bidding

Description

Demand Response Bidding occurs after the DR Assets and Resources that are to comprise the desired bid have been registered and a bid or an offer is to be formed. The DR Bidding use case has two sub-use cases, depending on the type of bid to be developed for submission: a bid to supply (i.e., an offer), or a bid to buy. The Entity operating the DR Bidding program (the Bid Receiver) receives bids from participants (the Bidders). The Bidder may be a Retail Customer bidding with either an energy retailer (e.g., LSE, ESP), a Service Provider (e.g., CSP, DRP), or a power marketer (i.e., wholesale market participant). The Bidder may also be a Service Provider bidding up to a LSE or power marketer.

Before the start of the bidding process, it is assumed that DR Resources available for selection are already registered to provide the physical resource participating in the DR Bidding program. It is also assumed that the Bidder has already registered to bid with the Bid Receiver and has been informed on the appropriate timing and parameters for bid participation. The DR Bidding process starts with the decision of the Bidder to develop a bid for submission. The Bidder submits its bid to the Bid Receiver within the appropriate time window and following set guidelines for bidding. The Bidder receives information on the outcome of the submitted bid from the Bid Receiver, who notifies of the bid outcome (e.g., a resource award or other DR commitment).

Examples of retail DR Bidding programs include:

- Demand Bidding

- Capacity Bidding

###### REQ.17.3.2.5.1	DR Bid to Supply (Retail Offers)

REQ.17.3.2.5.1.1	Description

The Bid Receiver opens the forum/venue for receiving an offer. (This may include initiating the bidding process by sending initial information to Bidders indicating parameters of acceptability for bids to be made). The Bidder communicates an offer to supply a specified product/service and the pricing or other incentive structure the bidder is willing to accept for DR. (This may be as simple as the Retail Customer sending a response back to the Bid Receiver that accepts or rejects the initial bid sought, or may involve more complex bid structuring).

Each offer will have a clearly specified product or service that is being bid and the pricing/incentive structure being offered. The Bidder also nominates the amount of and nature of supply being offered, the DR Resource to physically provide the service, and other physical parameters affecting Resource availability and capability, in addition to the identity (or ID) of the Bidder and Resource(s) that comprise the bid.

The Bid Receiver reviews bids (offers) received and clears them, notifying the bidders of the outcome.

The Bidder receives the notification of awards or Resource commitments and uses the information internally in preparation for DR Resource dispatch.

REQ.17.3.2.5.1.2	Sequence Diagram

(Note, the Bid in the following sequence diagram is an offer to supply)

###### REQ.17.3.2.5.2	DR Bid to Buy

REQ.17.3.2.5.2.1	Description

The Bid Receiver opens the forum/venue for receiving a bid. (This may include initiating the bidding process by sending initial information to Bidders indicating parameters of acceptability for bids to be made). The Bidder communicates a bid to buy a specified product/service and the pricing or other incentive structure the bidder is willing to pay to avoid DR. (This may be as simple as the Retail Customer sending a response back to the Bid Receiver that accepts or rejects the initial bid sought, or may involve more complex bid structuring).

Each bid will have a clearly specified product or service that is being bid for purchase and the corresponding pricing structure that is acceptable to the Bidder. The Bidder also includes the quantity of product/service in the bid to buy, and specifies any DR Resource(s) that can physically substitute for the product/service in the event the bid to buy is not cleared (i.e., the Bidder is unwilling to pay more than the going/market outcome price for the product/service). Other information submitted include physical parameters affecting Resource availability and capability, in addition to the identity (or ID) of the Bidder and Resource(s) that comprise the bid.

The Bid Receiver reviews bids received and clears all bids, notifying the Bidders of the outcome.  The Bidder receives the notification of resource commitments and uses the information internally in preparation for DR Resource dispatch.

REQ.17.3.2.5.2.2	Sequence Diagram

###### REQ.17.3.2.6	Execute DR Event

The execution of a DR Event takes different forms depending on operational situation and the type of DR programs being executed.  Typically, they fall into one of the following four use cases, see Figure 5:

- Advanced DR notification

- Broadcast DR message (price plus information)

- DR dispatch instruction

- DR direct load control.

Operational considerations for DR Event execution are also associated with DR programs where direct load control or dispatching instructions are carried out.

Figure 5. Execute DR Events

###### REQ.17.3.2.7	Notify DR Event (Retail)

###### REQ.17.3.2.7.1	Advanced Notification for DR (Retail)

REQ.17.3.2.7.1.1	Description

This is used by the Service Provider to send the DR Event notification (i.e., DR signal) to the DR Resource.

The Notify DR Event message applies to one of three types:

- Broadcast DR message (price plus information)

- Dispatch DR instructions for objectives (Retail)

- DR direct Load control (Retail).

The message content specific to each of these types is defined in their respective sections below.

REQ.17.3.2.7.1.2	Data Requirements

The following are the data requirements that are common to all of the Event message types.

| Attribute Name | Description |
| --- | --- |
| Program ID | Program Identifier |
| Program Name | Name of the Program |
| Service Provider ID | Identifier assigned to the Service Provider |
| Event ID | Identifier assigned to the DR Event |
| Event Modification Number | A modification number for the DR Event. This is used to indicate if the DR Event has been modified by the Distribution Company. Each time it is modified, this number is incremented. |
| Area of Event Notice Applicability: The area affected by the Demand Response Event can be defined by a location attribute (point location or area) or is applicable to a Resource or to an Asset for Direct Load Control. |  |
| Resource ID | Identifier assigned to the Resource |
| Asset ID | The unique identifer of the Asset |
| Location can be identified by any of the following: |  |
| Service Location ID | Identifier assigned to the Service Location |
| Address1 | Address line 1 |
| Address2 | Address line 2 |
| City | City |
| State/Province | State or Province two-letter code |
| Zip/Postal Code | Zip or Postal Code |
| Country | Country |
| GPS Coordinates | Latitide and longitude |
| Weather Station | Weather Station code associated with the Service Location |
| Zone ID | Identifier assigned to the Zone in which the Service Location is located |
| Zone | Name of the Zone in which the Service Location is located |
| Electrical Node ID | Identifier assigned to the Electrical Node at which the Service Location is connected |
| Electrical Node Name | Name of the Electrical Node at which the Service Location is connected |
| Electrical Node Type | Type of Electrical Node at which the Service Location is attached |
| PNode | Name of the Price Node associated with the Service Location |
| PNode ID | Identifier assigned to the Price Node associated with the Service Location |
| The following are other attributes applicable to all DR Event Messages. |  |
| Deployment Type | Type of Deployment. Examples are: Advanced (Normal), Immediate, Audit/Test. |
| Simple Signal Levels | Used as an alternate and simplified representation of the DR signal, whether it be price based or a dispatch. Takes on a small number of finite levels such as NORMAL, MODERATE, and HIGH, SPECIAL |
| Criticality Level | This field defines the level of criticality of this event. The action taken by load control devices for an Event can be solely based on this value, or combination with other Load Control Event fields supported by this device. For example, additional fields such as Average Load Adjustment Percentage, Duty Cycle, Cooling Temperature Offset, Heating Temperature Offset, Cooling Temperature Set Point or Heating Temperature Set Point can be used in combination with the Criticality level. Criticality Level Level Description Participation 0 Reserved 1 Green Voluntary 2 1 Voluntary 3 2 Voluntary 4 3 Voluntary 5 4 Voluntary 6 5 Voluntary 7 Emergency Mandatory 8 Planned Outage Mandatory 9 Service Disconnect Mandatory 0x0A to 0x0F Utility Defined Utility Defined |
| Baseline Dates | Dates of days used to calculate the Energy Baseline |
| Baseline Exclusion Dates | Dates of days Excluded from the calculation of the Energy Baseline |
| Energy Baseline Value | Calculated Energy Baseline |
| Energy Baseline Timestamp | Timestamp of Energy Baseline |
| Deployment Period (Interval): The deployment period that is communicated to a Resource or Asset is an interval contained within the Start or End Time of the Event, but not necessarily for the entire Event. |  |
| Event Start Time | Time of Start of deployment period for Event |
| Event End Time | Time of End of deployment period |

REQ.17.3.2.7.1.3	Sequence Diagram

###### REQ.17.3.2.7.2	Update a DR Event (Retail)

REQ.17.3.2.7.2.1	Description

This is used to update a DR Event that has been previously notified.  The requirements for this use case are the same as those for the "Advanced Notification for DR" use case.

Data Requirements

| Attribute Name | Description |
| --- | --- |
| Event | See the "Advanced Notification for DR" use case. |
| Event Modification Number | This is Modification number of the DR event. It is used to indicate that the DR Event has been modified by the Utility or Service Provider. Each time a DR Event is modified, this number is incremented. |
| Modification reason code | The reason the event was modified. |

REQ.17.3.2.7.2.2	Sequence Diagram

###### REQ.17.3.2.7.3	Cancel a DR Event (Retail)

REQ.17.3.2.7.3.1	Description

This is used by the Service Provider to notify the DR Resource that a DR Event is being cancelled. It should only be executed if a DR Resource has previously received notification of a DR Event and that event has subsequently been cancelled.

Data Requirements

| Attribute Name | Description |
| --- | --- |
| Modification reason code | The reason the Event is being cancelled. |
| Event Status | Status of the Event |
| DR Event Identifier | An identifier for the Event that is being cancelled. |
| Effective date/time | The date and time a cancellation takes effect. |

REQ.17.3.2.7.3.2	Sequence Diagram

###### REQ.17.3.2.7.4	DR Resource Confirmation (Retail)

REQ.17.3.2.7.4.1	Description

This is a confirmation message that is sent by a DR Resource to the Service Provider as a result of receiving a DR Signal form the Service Provider. It may be used as an acknowledgement of the receipt of the DR signal, but it may also contain various information used to signify how the DR Resource will respond to the DR Signal.

Any changes to the Resource or Asset that may impact participation or performance for the Event (such as Location and Load Profile) shall be provided through the Update Resource or Update Asset Messages.

REQ.17.3.2.7.4.2	Data Requirements

| Attribute Name | Description |
| --- | --- |
| Resource ID | Identifier assigned to the Resource |
| Asset ID | The unique identifer of the Asset |
| Exception Conditions | This is used to report that the Load controller may not behave as commanded because of a variety of conditions including: • Faults in device • Retail Customer override. |
| Load Control State | The state of the Load, which includes both commanded states and user settings. This may include a schedule of future states if a particular control algorithm for the Load controller is being executed. |
| Operational Constraints | Operational Constraints at run time may differ from those provided at Enrollment, and will be applicable only to the time interval of the Event for this Resource or Asset. |
| Constraint Type | Type of operational, schedule or offer constraint |
| Constraint Interval | The timeframe over which the constraint type applies. |
| Constraint Value | Value of the Constraint Type and Interval |
| Offer Dispatch Status | Dispatch Status of Offer a.k.a. as “Opt-in” or “Opt-out” in existing programs. |

REQ.17.3.2.7.4.3	Sequence Diagram

###### REQ.17.3.2.8	Broadcast DR Message (Price Plus Information)

REQ.17.3.2.8.1	Description

Demand Response programs are designed to address different power system concerns caused by the imbalance between generation, transmission and consumption, or environmental concerns. Some DR programs are conducted on a voluntary basis, where the Retail Customer can opt to maintain the level or Load.  Some DR programs are mandatory, where either the Retail Customer Loads will be curtailed under certain conditions or the Retail Customer will incur financial penalties for noncompliance.

In retail markets, certain “DR Messages” can be broadcasted to DR Resources either directly or through Service Providers.  DR Message is defined as information about the DR Signals that may affect the Demand behaviors of the Retail Customers.  Obviously, the main component of the DR Message is the pricing information.  However, there are other attributes of energy being delivered that may be of interest to Retail Customers and could be included in the DR Message.

Based on the OpenADR discussion, the broadcasting function is mainly used for volunteer or non-mission critical DR Messages. Therefore, DR Message acknowledgement or confirmation is optional.

REQ.17.3.2.8.2	DR Source of Power/Generation

“DR Message” also can be source of power or generation. ANSI C12.19-2008 defined the following source of power/generation in Standard Table 12.

Precondition:

The DR Service Provider sends the DR message to service providers. In the case of DR pricing, the DR Service Provider calculates the price per location and communicates the appropriate price associated with the DR product to service providers.

Post Condition:

DR Resources received the DR pricing information, and may or may not act based on the DR agreement or contract.

REQ.17.3.2.8.3	Data Requirements

| Attribute Name | Description |
| --- | --- |
| Currency | Economic unit of exchange in which the total price and price components are stated (e.g. dollars, euros) |
| Product Price | The sum of all Price Component Values for a given product (Total Price). |
| Product Price Factor | Terms of use attribute of Product Price Examples: Price absolute, Price relative, Price multiplier |
| Unit of Measure | Attribute of Product Identifier |
| Delivery Time | Time at which the product is available for physical consumption |
| Delivery Interval | The duration over which the product is available for physical consumption |
| Market Product (Product Type) | Market Product Product types include the following: energy, capacity, regulation, operating reserves (spinning or non-spin). |

REQ.17.3.2.8.4	Sequence Diagram

###### REQ.17.3.2.9	Dispatch DR Instructions (Retail)

REQ.17.3.2.9.1	Description

This interaction is used to dispatch DR Resources.  This type of interaction is used when the Service Provider needs to achieve specific objectives from the DR Resources during DR Events. This means that specific instructions will be given to the DR Resource with objectives (e.g. shed 100 kW) for the Load profile of the DR Resource.

Note that for the retail use cases, a Dispatch is considered a special type of DR Notification, and thus, it is very similar in nature to the requirements given for that interaction.

REQ.17.3.2.9.2	Data Requirements

|  | Attribute Name | Description |
| --- | --- | --- |
| DR Dispatches for DR Objective based messages are of three types. • Load Level • Load Amount • Load Percentage |  |  |
|  | For Load Level: Load Level Value | Value of the load level to be achieved based on a set of enumerated values. (e.g. moderate, high, etc) |
|  | For Load Amount: Deployment MW | Deployment quantity expressed as an absolute |
|  | For Load Percentage: Deployment MW | Relative Deployment quantity expressed as %. |

REQ.17.3.2.9.3	Sequence Diagram

###### REQ.17.3.2.10	DR Direct Load Control (Retail)

REQ.17.3.2.10.1	Description

This use case covers direct interactions between the DR Service Provider and a specific DR Asset for the purposes of putting that Asset into a specific Load control state (e.g., to turn it on or off). In this case, the DR Asset is a DR Resource with only one DR Asset, and thus, the interaction is directly with the DR Asset.

See REQ.17.3.2.11.3, DR Execution - Direct Load Control (DLC) for more details.

REQ.17.3.2.10.2	Data Requirements

| Attribute Name | Description |
| --- | --- |
| Asset ID | The unique identifer of the Asset (Note: Direct Load Control messages are applicable only to Assets) |
| Status Check | A signal to require the DR Resource status to be sent back. (Only applicable to Direct Load Control) |
| DR Control Command: Direct Load Control Type | The type of DR Direct Load Control Command: e.g. Set Point, Open/Close, Heating Temperature -offset/setpoint, Cooling Temperature -offset/setpoint, Load adjustment offset |
| DR Control Command: Direct Load Control Value | Value associated with the Direct Load Control Type. |
| Event Start Time | Time of Start of deployment period for Event |
| Event End Time | Time of End of deployment period |
| Duty Cycle | “Duty Cycle : Defines the maximum On state duty cycle as a percentage of time. Example, if the value is 80, the device would be in an “on state” for 80% of the time for the duration of the event. Range of the value is 0 to 100. A value of 0xFF indicates the field is not used.” (from SEP2.0) Error: Reference source not found |
| Event Control | Event Control options for randomized start or end times: 1= Randomize Start time, 0=Randomized Start not Applied 1= Randomize End time, 0=Randomized End not Applied. (from SEP2.0) Error: Reference source not found |
| Device Class | Enumeration representing the Device Class to apply the current Load Control Event.Error: Reference source not found |

REQ.17.3.2.10.3	Sequence Diagram

###### REQ.17.3.2.11	Monitor DR Event (DR Resource)

REQ.17.3.2.11.1	Description

This interaction is used to monitor a DR Resource's behavior. It may be executed as a result of the DR Resource receiving a DR signal or it may be conducted continuously.

REQ.17.3.2.11.2	Sequence Diagram

###### REQ.17.3.2.12	Monitor DR Event (DR Asset)

REQ.17.3.12.1	Description

This interaction is used to monitor the state of a DR Asset.  It may be executed as a result of the DR Asset receiving a DR Signal or it may be conducted continuously.

REQ.17.3.12.2	Sequence Diagram

###### REQ.17.3.2.13	DR Event Execution

Demand Response is deployed for economic or a reliability purposes.  At high penetration levels, Demand Response has to be integrated with the power supply scheduling process, and it may impact the distribution grid operation and reliability, and perhaps the transmission grid operation and reliability. Thus it is important to include the required considerations for the grid and system level operations when developing DR use cases and DR Signal Model Business Practices.  Such impacts may not be significant at low penetration levels; however, proper considerations must be given to support scalability and expandability for future deployments.

Power system operates based on a real-time balancing of supply and Demand.  To economically schedule the supply, an accurate load forecast is needed.   Traditionally, Load forecast was generated using the historical usage patterns, weather forecast and other similar parameters.  With the potential of high-penetration of DR, it will be necessary to incorporate the planned or forecasted DR levels into the Load forecast.   Also, DR capabilities can be used to support the power system capacity or emergency supply (ancillary service) needs.  Such services require proper scheduling and monitoring capabilities.

A Demand Response Event may be initiated by a power system operator in response to a reliability event, e.g., loss of a generating station causing a lack of supply to meet Demand, or an uncontrolled set of PEVs overloading of a distribution transformer.  A Demand Response Event may also be initiated by Distribution Company operational systems based on economic considerations, e.g., reducing or shifting peak Load at the system level or at specific feeder or facility.  Demand Response may also be initiated by a Retail Customer in response to a market pricing signal

Considering that the distribution grid in the US is typically a three-phase imbalance circuit, i.e., many retail Customers are on a single-phase or two-phase of the three-phase system.  Thus, significant changes in Retail Customer Load patters could result in undesirable imbalanced conditions on a feeder.   Also, Load pickup following the termination of a major DR Event could possibly cause overloads and other operational problems, if the Load pickup is not properly scattered/managed.

The following provides three representative use cases to highlight some the key interactions.  These include:

- Dynamic price-based DR

- Notification-based DR

- Direct Load Control

###### REQ.17.3.12.13.1	DR Execution – Retail Time Pricing (RTP)

REQ.17.3.2.13.1.1	Description

Provide RealTime or dynamic Pricing (RTP) to Retail Customers so that Retail Customers can vary their Demand based on pricing signals.   (Defined term – no explanation necessary)

Traditionally, retail tariff provisions are based on a fixed or tiered rate structure with possible considerations of static rates for pre-established time of use (TOU) conditions.  RTP represents retail electricity rates that could vary as a function of time and is intended to modify Demand.  It requires interval metering for accurate billing based on time-varying prices.  A number of states have implemented RTP or other dynamic pricing tariffs for large Retail Customers. With a broader deployment of advanced meters, RTP rates may become more common nation-wide.

The scenario presented here represents a case where a location-dependent Real Time Pricing for energy is established based on the Locational Marginal Price (LMP) for that location.  LMP values are typically established by an ISO (for regions covered by an ISO/RTO) on a day-ahead (hourly resolution) and real-time basis, typically on a five minute resolution.  For the RTP, it may be appropriate to use the real-time LMP as the basis for computing the retail dynamic tariff.  However, decisions should be made on the time and the spatial resolution of the RTP.   For example, an hourly RTP averaging the five minute pricing values, or a Critical Peak Pricing model to reflect the extreme conditions only, may be adopted.   In addition to the energy price, the retail RTP rate may also include the appropriate uplift charges to cover for distribution wire/services charges and for the power loss compensations.  Note that the uplift charges are typically subject to a regulatory review and approval.

Special care must be given to the RTP rate design to ensure Retail Customer acceptance and adaptation.  Also grid operational issues may have to be addressed.  For example, during low LMP (RTP) periods (e.g., at night), certain distribution circuits may get over loaded (congested), with Retail Customers shifting usage, e.g., charging PEVs, to that period.  This improves the overall system economy, but may cause circuit congestion. To combat this, some have proposed use of Demand charges or an additional incentive payment for Load reduction, a locational incentive to relief congestion.

REQ.17.3.2.13.1.2	Sequence Diagram

###### REQ.17.3.2.13.2	DR Execution – Notification Based

REQ.17.3.2.13.2.1	Description

Notification-based DR is mostly used for economic purposes based on a day-ahead or hours or hour-ahead basis.   Notification-based DR may also be used for reliability events when the Program Administrator expects a contingency or operating condition (e.g., congestion or planned outage) on the distribution or the transmission grid that requires a reduction (or an increase) on the load at a given location.

Considering that the business processes for Demand Response have not yet been standardized across the Country, the following is a representative scenario that captures some of the interactions between the key stakeholders (Actors).

At high penetration levels, the DR operation, especially on a day-ahead or hour ahead basis, need to be coordinated with the overall operations and supply scheduling process.   As is shown in the Sequence Diagram below, this may require a timely update of the “locational” Load forecast and an up-to-date nomination of the DR capabilities.  This information may be supplied to the “system” and/or market operator to be incorporated in the overall supply and Demand scheduling process.   The DR capabilities are specified by the DR Provider to the Program Administrator based on a DR Program, a price curve, or other nomination protocol.  The DR Provider may also inform the Load Serving Entity (LSE) of the locational available DR capabilities.

The notification-based DR dispatch process is typically initiated by the Program administrator; the process may also be initiated by an LSE or UDO.  It is also possible that the DR Provider be the initiator of the DR process based on a market opportunity (economic operation) or based on a pre-established program.

At high DR penetration levels, it is expected that the DR provider will need to clear the DR schedule with the Distribution Company Grid Operator.  This is to insure that the high penetration DR has no adverse impact on the distribution grid reliability and power quality, e.g., cause of excessive imbalances, voltage violation, or an overload during Load pickup period.   The Distribution Company Grid Operator, in a timely fashion, will inform the DR Provider, if the schedule is cleared or requires an adjustment.   Please note that if the DR deployed by a Distribution Company that includes both LSE and Distribution Company Grid Operator functions, such coordination is performed within the Distribution Company’s operational systems (e.g., DRMS, DMS, etc.), or it can be an integral part of the DR scheduling application.

Following the clearing the DR schedule, notifications are sent to Retail Customers for DR operation.  Under this scenario, the Market Operator and the LSE are also informed of the final DR schedule.

REQ.17.3.2.13.2.2	Sequence Diagram

###### REQ.17.3.2.13.3	DR Execution – Direct Load Control (DLC)

REQ.17.3.2.13.3.1	Description

Direct-Load Control is typically used for reliability-based events such as contingency and emergency support, supply of balancing energy or other ancillary service. Many of these programs require quick response time, e.g., five minutes or faster, that is practically only possible through a DLC capability. Direct-Load Control may also be used for economic operations, e.g., water heater programs to reduce or shift peak load.

Considering that the business processes for Demand Response have not yet been standardized across the Country, the following is a representative DLC scenario that captures the interactions between the key stakeholders (actors).

Direct Load Control is typically called for by a Program Administrator.  It could be also issued by a Market Operator based on an ancillary service DR award, or by an LSE or DRP for an economic purpose.   At high penetration levels, DLC operation needs to be coordinated with the Distribution Company Grid Operator.

As is shown in the sequence diagram below, the Retail Customer DR capabilities are aggregated by location and specified/nominated to the Distribution Company Grid Operator, Load Serving Entity, and/or to the Program Administrator.   In addition to location, the capabilities may also be aggregated by the DR response time, e.g., four seconds, five minutes, 30 minutes, etc.

The DLC Dispatch process is typically initiated by the Program Administrator..  The actual control of the DR resources may be done by the DR provider based on the dispatch signal received.

At high DR penetration levels, it is expected that the DLC activation schedule to be coordinated with the Distribution Company Grid Operator.  This is to insure that there are no adverse impacts on the distribution grid reliability and power quality, e.g., voltage violation, excessive phase imbalances, or an overload during load pickup period.   If the DLC deployed by a Distribution Company such coordination can be accomplished within the Distribution Company’s DR and distribution management systems, or performed as an integral part of the DR control functions.

For a “fast” DR, e.g., provision of ancillary services, a telemetry capability is required to enable the real-time monitoring of the Resource condition and its response to the DR control signal.

REQ.17.3.2.13.3.2	Sequence Diagram

###### REQ.17.3.2.14	Operational Coordination

Due to the existing differences in the regional retail energy market regulations, wholesale market structure and protocols, and supply and Demand conditions, currently there are regional differences in business practices and processes governing Demand Response.   As a result, operational coordination requirements for Demand Response somewhat vary from region to region, depending on the available retail tariff, retail market structure, available wholesale market for DR and the market mechanics.

The following table summarizes the major roles and responsibilities of the key stakeholders with respect to Demand Response operation.     As can be seen in this table, a significant level of stakeholder coordination is required for an end-to-end DR operation spanning from the wholesale markets to the end-use customers.

(Why are Wholesale stakeholders included in Retail Model Business Practices???)

| Stake Holder | Key Responsibilities |
| --- | --- |
| Wholesale Market Operator | - Establish, administer, and operate wholesale Demand Response programs and markets; - Determine and publish locational prices for wholesale Demand Response products; - Improve energy market efficiencies (economics) by integrating Demand Response products and capabilities; - Monitor compliance to wholesale DR instructions and financially settle with the participating stakeholders. |
| Transmission Grid / System Operator - Balancing Authority | - Monitor and maintain transmission grid reliability, and call upon wholesale Demand Response capabilities to address contingency/emergency conditions; - Dispatch Demand Response capabilities to balance supply and Demand, e.g., mitigating impact of variable generation (e.g., wind). - In regions with no organized wholesale energy market, economically dispatch available generation and demand resources including demand response to optimize supply economics. |
| Distribution Company Grid Operator | - Monitor and maintain the distribution grid reliability and the quality of power supply (e.g., voltage levels). Call upon DR to mitigate the distribution contingencies and reliability concerns. - Review and approve the integration of distributed energy resources with the distribution grid. - Assess the operational impact of distributed resources and Demand Response (DR/DER) on power quality and reliability of the distribution grid, approve high-penetration DR schedules (especially cold-load pickup operation). - Coordinate with the respective Load Serving Entity and Energy Service Providers for the assessment of distribution service and loss compensation charges to retail customers. |
| Load Serving Entity | - Forecast load and economically schedule supply to meet Demand at all times. Incorporate Demand Response capabilities to improve load factor, reducing/shifting peak load. - Establish and obtain regulatory approval for a fair and unbiased retail tariff for all Retail Customer classes. For Real Time Pricing (RTP) tariff, apply distribution service, loss compensation and other uplifts to the wholesale locational prices. - Establish and administer Demand Response and distributed resource management programs including DR tariff. - Meet regional requirements for Renewable Portfolio Standards compliance for power delivery. - Aggregate and schedule DR capabilities in wholesale markets, manage DR programs. - Manage Retail Customer metering and billing process. |
| Power Marketer | - Forecast Load and economically schedule supply to meet Demand at all times. Incorporate Demand Response capabilities to improve load factor, reducing/shifting peak Load. - Establish and obtain regulatory approval for a fair and unbiased retail tariff for all Retail Customer classes. For Real Time Pricing (RTP) tariff, apply distribution service, loss compensation and other uplifts to the wholesale locational prices. - Operate a wholesale and/or retail Demand Response programs with wholesale and/or Retail Customers. |
| Energy Service Provider | - Operate only in regions with competitive retail market - Be an alternative Supplier of electric power to Retail Customers by offering competitive products, services and prices. - Forecast aggregated Load of the Enrolled Retail Customers by location as required. - Coordinate the power supply with the respective LSE and UDO. - Support Demand Response programs (DR/DER) for Enrolled Retail Customers. - Aggregate Demand and DemandResponse capabilities; schedule the aggregated Load and DR capabilities in wholesale markets. - Provide for DR telemetry and other monitoring requirements, on a Resource or aggregated basis. - Apply the necessary uplifts (e.g., distribution service and loss compensation charges) to the wholesale locational prices. - Meet regional requirements for Renewable Portfolio Standards compliance for power delivery. - Bill and settle with Retail Customers for energy supply and DR operation. |
| Demand Response Provider | - Offer competitive Demand Response services to the Retail Customers. - Design and manage DR programs, establish pricing structure and signals for DR operation - Aggregate Demand and Demand Response capabilities; and offer/schedule these capabilities with the respective LSE or the wholesale market. - Provide for DR telemetry and other monitoring requirements, on a resource or aggregated basis. - Bill and settle with Retail Customers for energy supply and DR operation. |
| Retail Customer | - Subscribe to a DR program, and update DR availability and capability information as appropriate - Maintain DR resources, data communications, telemetry, metering and control devices as required. - Respond to pricing signals, DR notifications, or control signals as appropriate. |

###### REQ.17.3.2.15	Post DR Event Management

Post DR Event management mainly concerns with the reconciliation of the actual usage of energy within the duration of a DR Event with the expected behavior so that proper financial reward or penalties could be carried out.  The following use cases describe post DR Event management for three different market types.

###### REQ.17.3.2.15.1	Post DR Event M&V / Settlement (No Open Retail)

REQ.17.3.2.15.1.1	Description

After DR Event has been completed, then Measurement and Verification (M&V) can confirm whether action has been taken or to measure the impact of the response.  After the M&V, then the financial settlement is calculated and sent to the Retail Customer.   This use case covers the scenarios with no Open Retail Markets.

REQ.17.3.2.15.1.2	Sequence Diagram

###### REQ.17.3.2.15.2	Post DR Event M&V / Settlement (Open Retail)

REQ.17.3.2.15.2.1	Description

After DR Event has been completed, Measurement and Verification (M&V) can confirm action has been taken or to measure the impact of the response.  After the M&V, the financial settlement is calculated and sent to the Retail Customer.   This use case covers the scenarios with open retail markets.

REQ.17.3.2.15.2.2	Sequence Diagram

(Repeating the Appendix is redundant – refer to the Appendix in REQ.15.  This also avoids the maintenance issue of havinf to update 3 different areas.)

# 4.	Supporting Documentation

a.  Description of Request:

- REQ Annual Plan Item 9(c) – “Develop Requirements and Use Cases for PAP 09 – Demand Response/Distributed Energy Resources”

b.  Description of Recommendation:

This recommendation provides the data requirements related to the use cases developed as part of the phase one effort

c.  Business Purpose:

This recommendation has been developed in response to a request from NIST to provide use cases and requirements germane to the development of standard DR and DER signals.

d.  Commentary/Rationale of Subcommittee(s)/Task Force(s):

NAESB Smart Grid Standards Subcommittee Meeting Notes/Documents:

- March 11, 2010 Meeting Notes – To be posted

- March 18, 2010 Meeting Notes – To be posted

- March 25, 2010 Meeting Notes – To be posted

- April 1, 2010 Meeting Notes – To be posted

- April 8, 2010 Meeting Notes – To be posted

- April 29, 2010 Meeting Notes – To be posted

- May 13, 2010 Meeting Notes – To be posted

- May 20, 2010 Meeting Notes – To be posted

- June 10, 2010 Meeting Notes – To be posted

- June 17, 2010 Meeting Notes –  http://www.naesb.org/pdf4/smart_grid_ssd061710notes.doc

- July 1, 2010 Meeting Notes – To be posted

- July 13-14, 2010 Meeting Notes – To be posted

- July 22, 2010 Meeting Notes – To be posted

- August 5, 2010 Meeting Notes –http://www.naesb.org/pdf4/smart_grid_ssd080510notes.doc

- August 12, 2010 Meeting Notes – http://www.naesb.org/pdf4/smart_grid_ssd081210notes.doc

- August 26, 2010 Meeting Notes – http://www.naesb.org/pdf4/smart_grid_ssd082610notes.doc

- September 9, 2010 Meeting Notes – http://www.naesb.org/pdf4/smart_grid_ssd090910notes.doc

- September 16, 2010 Meeting Notes – http://www.naesb.org/pdf4/smart_grid_ssd091610notes.doc

- September 23, 2010 Meeting Notes – To be posted

The Actor ID shown in the table refers to the item number of the corresponding actor or related object in the Entity-relationship diagram provided in Appendix A. Definitions of the actors and related objects are included in REQ.15.2

DR program administration related use cases are determined by the SGTF as out of scope for the NIST PAP09 purpose, and therefore the original content has been removed from the final recommendation and resulting Model Business Practices presented here.  NAESB may start a new project to deal with the Retail Customer Enrollment and DR program administration related business practices.

Requirement as defined by the ZigBee Smart Energy Profile™ 2.0 Technical Requirements Document

.