Additions to WEQ-000-1 (Acronym)

SAMTS – Service Across Multiple Transmission Systems

Additions to WEQ-000-2 (Glossary)

Coordinated Group – Two or more Transmission Service and/or NITS requests that are linked together for the purpose of procuring SAMTS on a Transmission path. The Coordinated Group may be made up of requests and reservations such that there is  no gap in service over a given path to meet continuity requirements.

Coordinated Request – A Transmission Service request or NITS request that is part of a Coordinated Group.

001-4	On-line Negotiation and Confirmation Process

001-4.1	All reservations and price and/or capacity negotiations shall be conducted on OASIS.

001-4.2	RESERVED

001-4.3	RESERVED

Phase IA Negotiation Process State Transition Diagram

The NAESB OASIS Implementation Guide (Standard WEQ-013) provides a process state diagram to define the Transmission Customer and Transmission Provider interactions for negotiating transmission service.  This diagram defines allowable steps in the reservation request, negotiation, approval confirmation and post-confirmation processes.

001-4.4	RESERVED

001-4.5	RESERVED

Negotiations Without Competing Bids

The following practices are defined in order to enhance consistency of the reservation process across OASIS Phase IA nodes.

001-4.6	A Transmission Provider/Seller shall respond to a Transmission Customer’s service request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 4-2 Reservation Timing Requirements.  The time limit is measured from the time the OASIS Transmission request is QUEUED.  A Transmission Provider may respond by setting the state of the reservation request to one of the following:

I.	INVALID

II.	DECLINED

III.	REFUSED

IV.	COUNTEROFFER or CR_COUNTEROFFER

V.	ACCEPTED or CR_ACCEPTED

VI.	RECEIVED or STUDY (leading to REFUSED, COUNTEROFFER, CR_COUNTEROFFER, CR_ACCEPTED or ACCEPTED).

001-4.7	Prior to setting a request to ACCEPTED, CR_ACCEPTED, COUNTEROFFER, CR_COUNTEROFFER, or REFUSED a Transmission Provider shall evaluate the appropriate resources and ascertain that the requested transfer capability is (or is not) available.

001-4.7.1	If the Transmission Provider determines there is insufficient transfer capability available to grant the Transmission Customer’s request and there is no obligation to provide Partial Service (or Partial Service is also not available in cases where the Transmission Provider is obligated to provide Partial Service), the Transmission Provider may respond by setting the request status to REFUSED.

001-4.7.2	If the Transmission Provider determines there is sufficient transfer capability available to grant the Transmission Customer’s request and the customer’s bid price is at least equal to the provider’s current posted offer price over time and all other Transmission Provider requirements have been met, the Transmission Provider shall respond by setting the request status to ACCEPTED or for a Coordinated Request the Transmission Provider shall respond by setting the request to CR_ACCEPTED.

001-4.7.2.1	If the Transmission Customer does not specify a bid price, the Transmission Provider shall consider the bid price to be equal to the current posted offer price when evaluating the request.

001-4.7.3	The Transmission Provider shall use the status of COUNTEROFFER, or for a Coordinated Request the Transmission Provider shall use the status of CR_COUNTEROFFER, to initiate the negotiation of requested capacity (Partial Service) and/or price.

001-4.7.3.1	The Transmission Provider is not required to provide short-term Partial Service in response to a Long-Term request.

001-4.7.3.2	If the available transfer capability can support only a portion of the total capacity requested by the Transmission Customer and the Transmission Provider is obligated or elects to offer what limited capability is available to the customer, the Transmission Provider shall notify the Transmission Customer on OASIS by setting the amount of capacity being offered over time in the transmission request and updating the request status to COUNTEROFFER or for a Coordinated Request the Transmission Provider shall update the request status to CR_COUNTEROFFER.

001-4.7.3.3	If the Transmission Customer is attempting to negotiate price by submission of a bid price that is lower or higher than the current posted offer price, and the Transmission Provider elects to not negotiate price, the Transmission Provider shall notify the customer by setting the request status to DECLINED.

001-4.7.3.4	If the Transmission Provider elects to continue to negotiate price, the Transmission Provider shall update the offer price over time in the transmission request, indicate whether the price being negotiated is higher or lower than the posted price, and set the request status to COUNTEROFFER or for a Coordinated Request the Transmission Provider shall set the request status to CR_COUNTEROFFER.

001-4.7.3.4.1	If negotiation of price on one Coordinated Request is continued by the Transmission Provider,, through CR_COUNTEROFFER, such negotiation of price only does not automatically initiate negotiation of price or capacity on other Coordinated Requests in the Coordinated Group.

001-4.7.3.5	If both capacity and price are being negotiated, that negotiation shall be performed simultaneously.

001-4.8	Consistent with the requirements in Standard WEQ-013-2.3, Transmission Providers are required to post a reason for certain STATUS changes.

001-4.9	The Transmission Customer may change a request from QUEUED, RECEIVED, STUDY, COUNTEROFFER, CR_COUNTEROFFER, REBID, CR_ACCEPTED or ACCEPTED to WITHDRAWN at any time prior to CONFIRMED except as follows:

001-4.9.1	Preconfirmed requests for short-term firm and non-firm point-to-point service shall not be allowed to be WITHDRAWN by the Transmission Customer unless counteroffered by the Transmission Provider, after which the Transmission Customer shall be allowed to withdraw the request.

001-4.9.2	When the CAPACITY_GRANTED is less than the original CAPACITY_REQUESTED for any Coordinated Request in a Coordinated Group, the Transmission Customer is permitted, but not required, to withdraw any Coordinated Request in the Coordinated Group.

001-4.9.2.1	Prior to withdrawing a Coordinated Request with a status of CR_ACCEPTED, the Transmission Customer shall demonstrate that one or more of the Coordinated Requests in the Coordinated Group was not granted the original CAPACITY_REQUESTED and is assigned CR_ACCOMMODATED value of “N”.

001-4.9.2.1.1	For a Coordinated Request, where information has been provided as required in Business Practice Standard WEQ 001-xx.3.3.2, and CAPACITY_GRANTED equals the original CAPACITY_REQUESTED, REBID does not apply and the Transmission Customer shall set the value of CR_ACCOMMODATED to “Y”.

001-4.9.2.1.2	For a Coordinated Request, where information has been provided as required in Business Practice Standard WEQ 001-xx.3.3.2, and the state of the request has been set to WITHDRAWN prior to TP setting the state to CR_ACCEPTED the Transmission Customer shall set the value of CR_ACCOMMODATED to “Y” for all Coordinated Requests in the same Coordinated Group.

001-4.9.2.1.3	Withdrawal of one Coordinated Request prior to the Transmission Provider setting the state to CR_ACCEPTED does not justify the withdrawal of any other Coordinated Request in the same Coordinated Group.

001-4.9.2.1.4	For a Coordinated Request where information has been provided as required in Business Practice Standard WEQ 001-xx.3.3.2 and CAPACITY_GRANTED is less than the original CAPACITY_REQUESTED, the Transmission Customer shall set the value of CR_ACCOMMODATED to “N” shall set CR_GRANTED equal to the request’s CAPACITY_GRANTED and shall set the value of CR_REQUESTED equal to the request’s CAPACITY_REQUESTED.

001-4.9.2	For a preconfirmed request for short-term firm point-to-point service, if the Transmission Provider has notified the Transmission Customer that a System Impact Study is needed, the Transmission Customer may, prior to execution of the System Impact Study Agreement, request that the Transmission Provider void (status of DECLINED) the request.  The Transmission Provider must honor such request.

001-4.9.3	In cases of inadvertent error, the Transmission Customer may in the very near term request that the Transmission Provider void (status of ANNULLED) a pre-confirmed request/reservation for short-term firm or non-firm point-to-point service.  Honoring such a request is at the discretion of the Transmission Provider.

001-4.10	From ACCEPTED or COUNTEROFFER, a Transmission Customer may change the status to CONFIRMED, REBID or WITHDRAWN.  The Transmission Customer has the amount of time designated as Customer Confirmation Time Limit in Table 4-2 Request Timing Requirements to change the state of the request to CONFIRMED.  The Customer Confirmation Time Limit is measured from the first time the request is moved to ACCEPTED or COUNTEROFFER, and is not reset with subsequent iterations of negotiation.

001-4.10.1	The Transmission Customer shall use the request status of REBID to initiate or continue the negotiation of price and/or capacity.

001-4.10.2	The Transmission Customer may negotiate for a lower amount of transmission capacity by updating the capacity requested over time and changing the request status from ACCEPTED or COUNTEROFFER to REBID.  For example, the Transmission Customer may choose to coordinate the level of transmission service purchased across multiple Transmission Providers or other such purposes.  Negotiation for a higher amount of transmission capacity shall not be allowed. A Transmission Customer seeking additional capacity must submit a new request which will be assigned a new queue position.

001-4.10.3	The Transmission Customer may negotiate for a higher or lower price by updating the bid price requested over time and changing the request status from ACCEPTED or COUNTEROFFER to REBID.

001-4.10.3.1	For a Coordinated Request where the CAPACITY_GRANTED is less than the original CAPACITY_REQUESTED, a Transmission Customer may reduce the amount of CAPACITY_REQUESTED on any other Coordinated Request in the same Coordinated Group.

001-4.10.3.1.1	Prior to changing the status of the Coordinated Request from CR_ACCEPTED or CR_COUNTEROFFER to REBID, the Transmission Customer shall set the CR_ACCOMMODATED value to “N” for any Coordinated Request in the same Coordinated Group with a status of CR_ACCEPTED or CR_COUNTEROFFER which was not granted its original CAPACITY_REQUESTED.

001-4.10.3.1.1.1	For a Coordinated Request where information has been provided as required in Business Practice Standard WEQ 001-xx.3.3.2 and CAPACITY_GRANTED equals the original CAPACITY_REQUESTED, the Transmission Customer shall set the value of CR_ACCOMMODATED to “Y”.

001-4.10.3.1.1.2	For a Coordinated Request where information has been provided as required in Business Practice Standard WEQ 001-xx.3.3.2 and the state of the request has been set to WITHDRAWN prior to TP setting the state to CR_ACCEPTED the Transmission Customer shall set the value of CR_ACCOMMODATED to “Y” for all Coordinated Requests in the same Coordinated Group.

001-4.10.3.1.1.3	Withdrawal of one Coordinated Request prior to the Transmission Provider setting the state to CR_ACCEPTED does not justify the withdrawal of any other Coordinated Request in the same Coordinated Group.

001-4.10.3.1.1.4	For a Coordinated Request where information has been provided as required in Business Practice Standard WEQ 001-xx.3.3.2 and CAPACITY_GRANTED is less than the original CAPACITY_REQUESTED, the Transmission Customer shall set the value of CR_ACCOMMODATED to “N” and shall set the value of CR_GRANTED equal to the request’s CAPACITY_GRANTED and shall set the value of CR_REQUESTED equal to the request’s CAPACITY_REQUESTED.

001-4.10.4	The Transmission Customer shall positively acknowledge and agree to both the price and capacity over time by setting the bid price and requested capacity equal to the Transmission Provider’s offer price and granted capacity prior to confirmation of the transmission service request.

001-4.11	After expiration of the Customer Confirmation Time Limit, specified in Table 4-2 Request Timing Requirements, the Transmission Provider has a right to move the request to the RETRACTED state, unless the request is a Coordinated Request.

001-4.12	Should the Customer elect to initiate or continue the negotiation of price and/or capacity by moving a transmission service request to REBID, the Transmission Provider shall respond by taking the request to a DECLINED, SUPERSEDED, ACCEPTED, or COUNTEROFFER state within the Provider Counter Time Limit, specified in Table 4-2 Request Timing Requirements.  The Transmission Provider response time is measured from the most recent REBID time.

001-4.12.1	The Transmission Provider shall positively acknowledge and agree to both the price and capacity over time by setting the offer price and granted capacity equal to the Transmission Customer’s bid price and requested capacity prior to acceptance of the transmission service request.

001-4.13	The following timing requirements shall apply to all Point-to-Point transmission service requests:

Table 4-2

REQUEST Timing Requirements

| Class | Service Increment | Time QUEUED Prior to Start | Provider Evaluation Time Limit1 | Customer Confirmation Time Limit2 after ACCEPTED or COUNTEROFFER3 | Customer Confirmation Time Limit2,8 after CR_ACCEPTED or CR_COUNTER-OFFER | Provider Counter Time Limit after REBID4 |
| --- | --- | --- | --- | --- | --- | --- |
| Non-Firm | Hourly | <1 hour | Best effort | 5 minutes | N/A | 5 minutes |
| Non-Firm | Hourly | >1 hour | 30 minutes | 5 minutes | N/A | 5 minutes |
| Non-Firm | Hourly | Day ahead | 30 minutes | 30 minutes | N/A | 10 minutes |
| Non-Firm | Daily | N/A | 30 minutes | 2 hours | N/A | 10 minutes |
| Non-Firm | Weekly | N/A | 4 hours | 24 hours | N/A | 4 hours |
| Non-Firm | Monthly | N/A | 2 days 5 | 24 hours | As specified in WEQ 001-xx? | 4 hours |
| Firm | Daily | < 24 hours | Best effort | 2 hours | N/A | 30 minutes |
| Firm | Daily | N/A | 30 days6 | 24 hours | N/A | 4 hours |
| Firm | Weekly | N/A | 30 days6 | 48 hours | N/A | 4 hours |
| Firm | Monthly | N/A | 30 days6 | 4 days | As specified in WEQ 001-xx? | 4 hours |
| Firm | Yearly | 60 days 7 | 30 days | 15 days | As specified in WEQ 001-xx? | 4 hours |

Notes for Table 4-2:

1	Consistent with regulations and filed tariffs, measurement starts at the time the request is QUEUED.

2	Confirmation time limits are not to be interpreted to extend scheduling deadlines or to override pre-exemption deadlines.

3	Measurement starts at the time the request is first moved to either ACCEPTED or COUNTEROFFER.  The time limit does not reset on subsequent changes of state.

4	Measurement starts at the time the Transmission Customer changes the state to REBID.  The measurement resets each time the request is changed to REBID.

5	Days are defined as calendar days.

6	Subject to expedited time requirements of Section 17.1 of the pro forma tariff.  Transmission Providers shall make best efforts to respond within 72 hours, or prior to the scheduling deadline, whichever is earlier, to a request for Daily Firm Service received during period 2-30 days ahead of the service start time.

7	Subject to Section 17.1 of the pro forma tariff, whenever feasible and on a nondiscriminatory basis, Transmission Providers should accommodate requests made with less than 60 days notice.

8	Measurement starts at the time all Coordinated Requests in the Coordinated Group are first moved to CR_ACCEPTED, CR_COUNTEROFFERED or a final state.  The time limit does not reset on subsequent changes of state.

START HERE on 3/1/11

JT note to group – These are the transmission service definitions. Look them so that we can use them correctly in the standard

Transmission Service - Services needed to move energy from a POR to a POD provided to Transmission Customers by Transmission Providers.

Point-to-Point Transmission Service (PTP) - The reservation and transmission of capacity and energy on either a firm or non-firm basis from the PORs to the PODs as specified in the Transmission Provider's tariff.

Network Integration Transmission Service (NITS) - As specified in the Transmission Provider’s tariff, service that allows an electric Transmission Customer to integrate, plan, economically dispatch and regulate its network resources in a manner comparable to that in which the Transmission Provider serves Native Load Customers.

- .

- 001-xx	COORDINATION OF REQUESTS FOR SERVICE ACROSS MULTIPLE TRANSMISSION SYSTEMS (SAMTS)

The following requirements provide consistency in processing Coordinated Requests.

001-xx.1	General Requirements

001-xx.1.1	OASIS shall support the queuing and processing of all Coordinated Requests, as specified in the technical requirements of Business Practice Standards WEQ-002, WEQ-003 and WEQ-013.

001-xx.1.2	If the Transmission Provider determines that only a portion of the requested capacity can be accommodated on a Coordinated Request, a Transmission Customer or Network Customer will not be required to reserve the full requested capacity on the remaining Coordinated Request(s) in a Coordinated Group.

001-xx.1.3	A request for SAMTS shall be queued and evaluated in the same manner (i.e., same service priority) as any other Transmission Service or NITS request, subject to the other requirements of Business Practice Standard WEQ-001, as if the Coordinated Request were not part of a Coordinated Group (timing tables do not need to be changed)

001-xx.1.4	A Transmission Provider shall not be required to review or evaluate requests on other Transmission Providers’ systems.

001-xx.1.4.1	A Transmission Provider shall not be required to validate consistency of product names and/or attributes across Coordinated Requests which are part of a Coordinated Group.

001-xx.1.5	A Transmission Customer shall not be required to ultimately reserve Transmission Service or NITS in excess of the lowest capacity granted by any Transmission Provider.

001-xx.1.6	This Business Practice Standard shall only be for evaluation and processing of Coordinated Requests in a Coordinated Group.

001-xx.1.6.1	Once all Coordinated Requests are in a final state, the coordination process is complete and the Coordinated Group is no longer linked for any further coordination processing.

001-xx.1.6.2	After the coordination process is completed each of the resulting reservations shall be treated as any other reservation.

001-xx.1.7	This Business Practice Standard shall permit regional alliances of Transmission Providers to establish a mechanism wherein a single request is processed for Transmission Service or NITS on multiple Transmission Providers.  Such a mechanism shall allow Coordinated Requests that extend beyond the boundaries of the alliance.

001-xx.1.8	This Business Practice Standard shall not require any Transmission Provider to offer additional products in order to establish an identical hour-by-hour match of reservations on OASIS systems that serve different time zones.

001-xx.1.9	This Business Practice Standard shall not require the Transmission Customer or Network Customer to procure additional products in order to establish an identical hour-by-hour match of reservations on OASIS systems that serve different time zones.

001-xx.2	Eligibility

001-xx.2.1	The Coordinated Group shall be contiguous. This requirement shall be fulfilled when  the Coordinated Group is made up of requests and reservations such that there is  no gap in service over a given path from the earliest start time of any Coordinated Request in the Coordinated Group to the latest stop time of any Coordinated Request in the Coordinated Group.

001-xx.2.1.1	A Coordinated Group shall permit time zone differences to exist. (e.g., a start time of midnight EST shall be deemed contiguous with a start time of midnight CST.)

001-xx.2.1.2	The Transmission Customer or Network Customer shall attest that sufficient requests and reservations are under said customer’s control to meet the contiguity requirements.

001-xx.2.2	A Coordinated Group shall combine one or more Coordinated Requests on a Transmission Provider’s system with one or more Coordinated Requests another Transmission Provider’s system.

001-xx.2.3	Requests for Concomitant Evaluation may be included in a Coordinated Group.

001-xx.2.4	Coordinated Requests within a Coordinated Group may have different durations (start/stop times).

001-xx.2.5	Unless otherwise prohibited, Transmission Customer or Network Customer may submit any combination and any number of requests of the following PTP or NITS services to form a Coordinated Group:

- Yearly firm PTP

- Monthly firm PTP

- Monthly non-firm PTP

- Firm Network Service with a minimum duration of one month

- Monthly secondary Network Service

001-xx.2.6	.Requests for Coordinated Requests shall be submitted by the Transmission Customer or Network Customer as pre-confirmed.

001-xx.2.7	A Coordinated Request is subject to all request timing requirements consistent with requests for Transmission Service or NITS of similar duration. These pre-confirmed requests shall be treated as any other pre-confirmed Transmission Service request with respect to queue position and preemption (Business Practice Standard WEQ-013 Table 4-3)

001-xx.3	Treatment of Coordinated Requests

001-xx.3.1	A Transmission Customer shall be permitted to submit a set of requests on multiple transmission systems and group them together.

001-xx.3.2	Requests may be submitted by multiple Transmission Customers and Network Customers  and may be combined into a Coordinated Group.

001-xx.3.3	The Transmission Customer or Network Customer shall provide the following for each Coordinated Request:

001-xx.3.3.1	A unique identifier that is assigned to the Coordinated Group (similar to the way a unique identifier is established for each e-tag) The CG_FLAG shall be set to “Y”.

001-xx.3.3.2	The following information shall be provided for each of the Coordinated Requests for Transmission Service or NITS that are part of the Coordinated Group:

- CR_PRIMARY_PROVIDER_CODE - Unique code for each Transmission Provider. Used by PATH_NAME and in URL. Registered as part of URL at www.tsin.com.

- CR_ASSIGNMENT_REF -The ASSIGNMENT_REF number of the Coordinated Request for Transmission Service or NITS on the CR_PRIMARY_PROVIDER_CODE’s system.

- CR_TYPE shall be set to NETWORK or POINT-TO-POINT

- CR_CLASS shall be set to FIRM, NON-FIRM, or SECONDARY

- CR_INTERVAL shall be set to MONTH or YEAR

- CR_ACCOMMODATED shall be set to “Y” or “N”

001-xx.3.3.3	The Transmission Customer or Network Customer shall attest that sufficient requests and reservations are under said customer’s control to meet the contiguity requirements by setting the CG_ATTEST value to “Y”.

001-xx.3.4	Upon receipt of a queued Coordinated Request, the Transmission Provider shall set the CR_SUBMIT_DEADLINE to a date and time that is 24 hours after the TIME_QUEUED

001-xx.3.4.1	The Transmission Customer or Network Customer shall be permitted to amend the Coordinated Request to permit adding the information as described in WEQ-001-xx.3.3.2 until the CR_SUBMIT_DEADLINE.

.

001-xx.3.4.2	The Transmission Customer or Network Customer shall be permitted to change the CG_FLAG value to “N” prior to the CR_SUBMIT_DEADLINE.  Upon making these changes, the request will no longer be considered a part of a Coordinated Group.

001-xx-3.4.3	The Transmission Customer or Network Customer shall be permitted to remove, insert and modify the information described in WEQ 001-xx.3.3.2 any time prior to the CR_SUBMIT_DEADLINE.

001-xx.3.5	Each Transmission Provider shall evaluate and respond to the Coordinated Request on its OASIS in accordance with WEQ 001-4 as if it were not part of a Coordinated Group with the following exceptions:

001-xx.3.5.1	If the Transmission Provider determines there is sufficient transfer capability available to grant the Transmission Customer’s request and the customer’s bid price is at least equal to the provider’s current posted offer price over time and all other Transmission Provider requirements have been met, the Transmission Provider shall respond by setting the request status to CR_ACCEPTED.

001-xx.3.5.3	The Transmission Provider shall use the status of CR_COUNTEROFFER to initiate the negotiation of requested capacity (Partial Service) and/or price.

001-xx.3.5.3.1	If the available transfer capability can support only a portion of the total capacity requested by the Transmission Customer and the Transmission Provider is obligated or elects to offer what limited capability is available to the customer, the Transmission Provider shall notify the Transmission Customer on OASIS by setting the amount of capacity being offered over time in the transmission request and updating the request status to CR_COUNTEROFFER.

001- xx.3.5.3.2	If the Transmission Provider elects to continue to negotiate price, the Transmission Provider shall update the offer price over time in the transmission request, indicate whether the price being negotiated is higher or lower than the posted price, and set the request status to CR_COUNTEROFFER.

001-xx.3.5.4	Transmission Providers shall not deny a Coordinated Request for any of the following reasons:

- A Coordinated Request has a different time zone than another Coordinated Request in the Coordinated Group

- A Coordinated Request has a different CAPACITY_REQUESTED value than another Coordinated Request in the Coordinated Group.

- A Coordinated Request has a different SERVICE_INCREMENT than another Coordinated Request in the Coordinated Group.

- A Coordinated Request has a different START_TIME or a different STOP_TIME than another Coordinated Request in the Coordinated Group

001- xx.3.6	The Transmission Provider, upon setting a request status to CR_ACCEPTED or CR_COUNTEROFFER, shall review information on all Coordinated Requests in the Coordinated group and shall set the CR_CONFIRMATION_LIMIT.  If any of the requests is yearly, set the value to 15 days. If none of the requests are yearly and any of the requests is monthly, set the value to 4 days.  If neither of these conditions are met, set the value to 24 hours.

| CR_INTERVAL | CR_CLASS |  | Confirmation Time Limit |
| --- | --- | --- | --- |
| YEAR | FIRM |  | 15 days |
| MONTH | FIRM |  | 4 days |
| MONTH | NON-FIRM |  | 24 hours |
| MONTH | SECONDARY |  | 24 hours |

001-xx.3.7	From CR_ACCEPTED or CR_COUNTEROFFER, a Transmission Customer may change the status to CONFIRMED prior to knowing the final outcome of all Coordinated Requests in a Coordinated Group.  Once the Coordinated Request is set to CONFIRMED, no adjustment in capacity of that Coordinated Request shall be permitted.

001-xx.3.8	When each of the Coordinated Requests of the Coordinated Group is in a pending or final state, the Transmission Customer shall determine the final requested capacity of each of the Coordinated Request in accordance with Business Practices in WEQ 001-4.7 and the following:

001-xx.3.8.1	From CR_ACCEPTED or CR_COUNTEROFFER, a Transmission Customer may change the status to CONFIRMED, REBID or WITHDRAWN.  The Transmission Customer has the amount of time designated as Customer Confirmation Time Limit in Table 4-2 Request Timing Requirements to change the state of the request to CONFIRMED.  The Customer Confirmation Time Limit is measurement starts at the time all Coordinated Requests in the Coordinated Group are first moved to CR_ACCEPTED, CR_COUNTEROFFERED or a final state.  The time limit does not reset on subsequent changes of state

001-xx.3.8.2	From CR_ACCEPTED or CR_COUNTEROFFER, a Network Customer may change the status to CONFIRMED, REBID or WITHDRAWN.  The Transmission Customer has the amount of time designated as Customer Confirmation Time Limit in Table 105 Request Timing Requirements to change the state of the request to CONFIRMED.  The Customer Confirmation Time Limit is measurement starts at the time all Coordinated Requests in the Coordinated Group are first moved to CR_ACCEPTED, CR_COUNTEROFFERED or a final state.  The time limit does not reset on subsequent changes of state

001-xx.3.8.3	After expiration of the Customer Confirmation Time Limit, specified in Table 4-2 Request Timing Requirements or Table 105 Request Timing Requirements, the Transmission Provider has a right to move a request with the status of CR_ACCEPTED to the CONFIRMED state and to move a request with the status of CR_COUNTEROFFERED to the RETRACTED state.

001-xx.4.3	Each Transmission Provider shall accept any rebid capacity that was set to the status of REBID by the Transmission Customer for each of the Coordinated Request(s).

Additional requirements:

d) 02/09/2011 Subcommittee discussed and decided that theTP is not required to monitor these states but if observed that the customer has not responded within the deadline then the TP may set the state to RETRACTED.

a)02/09/2011 Subcommittee discussed and decided that TP negotiation of price,

Additions to WEQ-002

Additions to WEQ-003

New items:

- Incorporate CR_ACCEPTED and CR_CCOUNTEROFFER as valid Status values.

- CG_FLAG (to announce that this is a Coordinated Request)

- CR_PRIMARY_PROVIDER_CODE (Primary Provider Code for additional Coordinated Requests)

- CR_ASSIGNMENT_REF CODE (Aref for additional Coordinated Requests)

- CR_TYPE CODE (TS Type for additional Coordinated Requests)

- CR_CLASS (TS Class for additional Coordinated Requests)

- CR_INTERVAL (Interval or Service Increment for additional Coordinated Requests)

- CG_ATTEST (Customer attestation that this request is part of a valid Coordinated Group)

- CR_SUBMIT_DEADLINE (Deadline for submission of all info for additional Coordinated Request data)

- CR_CONFIRMATION_LIMIT (Window of time available for customer confirmation after all statuses are known…e.g., 15 days for yearly)

- CR_GRANTED applies to the amount of capacity that can be provide by the other TP

- CR_REQUESTED applies to the other amount of capacity that has been requested by the TC

Additions to WEQ-013

013-2.2 	TRANSACTION STATUS

CR_COUNTEROFFER = applies to a Coordinated Request in a Coordinated Group when assigned by the Transmission Provider or Reseller to indicate that a new OFFER_PRICE and/or CAPACITY_GRANTED over time is being proposed in the negotiation of requested service, (i.e., offering of Partial Service or negotiation of price).

CR_ACCEPTED = applies to a Coordinated Request in a Coordinated Group when assigned by the Transmission Provider or Reseller to indicate the service request at the designated BID_PRICE and CAPACITY_REQUESTED has been approved/accepted. Depending upon the type of ancillary services required, the Seller may or may not require all ancillary service reservations to be completed before accepting a request.