1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| X | Accept as requested | X | Change to Existing Practice |
| --- | --- | --- | --- |
|  | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

2.  TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
| X | Initiation | X | Initiation |
| X | Modification | X | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
| X | Principle | X | Principle |
| X | Definition | X | Definition |
| X | Business Practice Standard | X | Business Practice Standard |
| X | Document | X | Document |
|  | Data Element |  | Data Element |
|  | Code Value |  | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
| X | Business Process Documentation | X | Business Process Documentation |

- 3.  RECOMMENDATION

- SUMMARY:

- Recommended Standards:

- 4.  SUPPORTING DOCUMENTATION

- a. Description of Request:

- b. Description of Recommendation:

- c. Business Purpose:

- d. Commentary/Rationale of Subcommittee(s)/Task Force(s):

e. Additional Background documentation

# Business Practices for Measurement and Verification Business Practices for Measurement and Verification of Demand Response Programs in the Electric Retail Market

Introduction

- Measurement and Verification

(Note: Nos. 1 and 2 were changed to match standard NAESB Retail language.  The concepts should have carried forward.)

This section provides a common framework of the Model Business Practices for Measurement and Verification (M&V) of Demand Response programs in the retail energy markets.   The purpose of these Model Business Practices is to provide:

- Transparency: accessible and understandable M&V requirements for Demand Response programs

- Accountability: criteria that will enable the Program Administrator  to accurately measure performance of Demand Response Resources; and

- Consistency: a process or protocol that will allow Program Administrators, Applicable Regulatory Authorities, or program participants to agree on the required steps to take to verify demand reductions resulting from Demand Response programs in retail energy markets.

- Comprehensive:  strives to cover all forms of Demand Response

- Applicability of Measurement and Verification Standards:

The North American Energy Standards Board (NAESB) is a voluntary non-profit organization comprised of members from all aspects of the natural gas and electric industries. Within NAESB, the Retail Electric Quadrant (REQ) and the Retail Gas Quadrant (RGQ) focus on issues impacting the retail sale of energy to end-use customers. REQ / RGQ Model Business Practices are intended to provide guidance to Distribution Companies, Suppliers, and other Market Participants involved in providing energy service to end-use Customers. The focus of these Model Business Practices is performing M&V for Demand Response programs.  These Model Business Practices are intended to be consistent with the Wholesale Electric Standards, but also acknowledge differences in product and program types between the two markets.

These Model Business Practices are voluntary and do not address policy issues that are the subject of state legislation or regulatory decisions. These Model Business Practices have been adopted with the realization that as the industry evolves, additional and amended Model Business Practices may be necessary. Any industry participant seeking additional or amended Model Business Practices (including principles, definitions, data elements, process descriptions, and technical implementation instructions) should submit a request to the NAESB office, detailing the change, so that the appropriate process may take place to amend the Model Business Practice.

# Retail Demand Response Program Classifications

# Definition of Demand Response

# A change, temporary or recurring, in electricity consumption by a Demand Resource in response to market or reliability conditions.  For purposes of these Model Business Practices, this definition does not include energy efficiency or permanent Load reduction.

Comment:  Is the language above brad enough to be considered commodity neutral as to apply also to the retail gas markets? (BGE)

Definition of Program Administrator

An investor owned utility, governmental or cooperative utility or independent aggregator of demand response programs who has responsibility for developing and operating demand response programs identified in this standard. The Program Administrator shall have regulatory reporting responsibility to the utility regulatory body in the jurisdiction of the retail load and reporting responsibility to NERC.

Demand Response Provider

The entity that is responsible for delivering Demand reductions from Demand Resources.

Comment:  The group will work to ensure consistency between these two definitions.

Demand Response programs can be classified into two major categories, dispatchable and nondispatchable programs:

Dispatchable Programs

Dispatchable programs are programs that allow a system operator (RTO system operator or a utility system operator) to declare a load response event that has a specific start time and end time. The measurement and verification process would determine the load reductions attributable to Demand Response programs over this specified time period.  Examples of programs in this classification would be:

Comments:  This is a working definition and will be reviewed for other changes.

Capacity Programs: (Definition exists)

(Need to define all terms)

Direct Load Control

Firm Service Level

Guaranteed Load Drop

Critical Peak Pricing

Ancillary Service Programs:

Spinning Reserve

Non Spinning Reserve

Regulation Services (Definition exists)

Energy Service Programs – Dispatched for economic reasons

Non Dispatchable Programs

(Definition)  Non Dispatchable programs include programs that have predetermined time periods where consumption patters are expected to change due to price signals or other incentives designed to induce lower electricity consumption, Examples of programs in this classification would be:

Time Sensitive Pricing Programs

Time of use

Critical Peak Pricing

Real Time Pricing

For purposes of these Measurement and Verification standards, Demand Response does not include Measurement and Verification of energy efficiency or permanent Load reduction.

Business Practice Conflicts with the wholesale DR standard and NERC Standards:

This standard is designed to provide a framework for measurement and verification of demand response programs in the retail sector. It was designed in concert with NAESB standards covering demand response programs operating in the wholesale markets. In the event of a conflict between these business practices and business practices developed for the wholesale markets the wholesale market standard shall have precedence.    Additionally, all entities supplying Demand Response Services shall comply with applicable NERC reliability standards.

- Overview of the Standards

These M&V standards establish criteria for the use of equipment, technology, and procedures to quantify the Demand Reduction Value delivered. Standards developed may include commonalities among product types.  The following outline of standards is applicable to the two Demand Response product categories.

| General | Advance Notification |
| --- | --- |
|  | Deployment Time |
|  | Reduction Deadline |
|  | Release/Recall |
|  | Normal Operations |
|  | Demand Resource Availability Measurement |
|  | Aggregation |
|  | Transparency of Requirements |
| Telemetry | Telemetry Requirement |
|  | Telemetry Accuracy |
|  | Telemetry Interval |
|  | Other Telemetry Measurements |
|  | Communication Protocol |
|  | Governor Control Equivalent |
|  | On-Site Generation Telemetry Requirement |
| After-The-Fact Metering | After-the-Fact Metering Requirement |
|  | Meter Accuracy |
|  | Details of Meter/Equipment Standards |
|  | Meter Data Reporting Deadline |
|  | Meter Data Reporting Interval |
|  | Clock / Time Accuracy |
|  | Validating, Editing & Estimating (VEE) Method |
|  | On-Site Generation Meter Requirement |
| Performance Evaluation | Rules for Performance Evaluation |

Performance Evaluation Methodology

For each Demand Response service, a performance evaluation methodology is used to determine the Demand Reduction Value provided by a Demand Resource. The standards include descriptions of acceptable Baselines and alternative performance measurements that are appropriate for each of the four types of Demand Response services.  The table below provides an outline of the applicable criteria for performance evaluation methodologies.

| Baseline Information | Baseline Window |
| --- | --- |
|  | Calculation Type |
|  | Sampling Precision and Accuracy |
|  | Exclusion Rules |
|  | Baseline Adjustments |
|  | Adjustment Window |
| Event Information | Use of Real-Time Telemetry |
|  | Use of After-The-Fact Metering |
|  | Performance Window |
|  | Measurement Type |
| Special Processing | Highly-Variable Load Logic |
|  | On-Site Generation Requirements |

These standards do not specify detailed characteristics of performance evaluation methodologies, but rather provide a framework that may be used to develop performance evaluation methodologies for specific Demand Response services.  This approach is believed to be most appropriate at this time as development of performance evaluation methodologies and baseline calculations continues to mature. The following methodology types are applicable to wholesale Demand Response Services:

(Comment, Dominion – Additional specificity should be considered.  While performance methodologies are maturing, there are some reliable methodologies that we may want to identify and recommend)

Maximum Base Load: A performance evaluation methodology based solely on a Demand Resource’s ability to  not exceeda specified level of electricity demand, regardless of its electricity consumption or demand at Deployment. (Example?)

Meter Before / Meter After:  A performance evaluation methodology where electricity consumption or demand over a prescribed period of time prior to Deployment is compared to similar readings during the Sustained Response Period. (Example?)

Baseline Type-I: A Baseline performance evaluation methodology based on a Demand Resource’s historical interval meter data which may also include other variables such as weather and calendar data. (example?)

Baseline Type-II: A Baseline performance evaluation methodology that uses statistical sampling to estimate the electricity consumption of an Aggregated Demand Resource where interval metering is not available on the entire population. (example ?)

Metering Generator Output: A performance evaluation methodology, used when a generation asset is located behind the Demand Resource’s revenue meter, in which the Demand Reduction Value is based on the output of the generation asset.

Applicability of Performance Evaluation Methodology to Program Type

| Performance Evaluation Type | Maximum Base Load | Meter Before/Meter After | Baseline Type I | Baseline Type II | Metering Generator Output |
| --- | --- | --- | --- | --- | --- |
| Capacity Program |  |  |  |  |  |
| Direct Load Control |  |  |  | X |  |
| Firm Service Load | X |  |  |  |  |
| Guaranteed Load Drop |  | X |  |  |  |
| Critical Peak Pricing |  |  | X |  |  |
| Ancillary Service Programs |  |  |  |  |  |
| Spinning Reserve |  |  |  |  | X |
| Non-Spinning Reserves |  | X |  |  | X |
| Regulation Services |  | X |  |  | X |
| Time Sensitive Pricing Programs |  |  |  |  |  |
| Time of Use |  |  | X |  |  |
| Critical Peak Pricing |  |  | X |  |  |
| Real Time Pricing |  |  | X |  |  |

Definition of Terms

Dispatchable DEMAND RESPONSE EVENT TERMS

Illustration of Timing of a Demand Response Event

The illustration below represents the terms for timing events and time durations applicable to the characteristics of a dispatchable Demand Response Event.  The definitions of the ten elements in the illustration are the basis for describing the Timing of a Demand Response Event.

Figure 1. Timing of a Demand Response Event

# The following terms refer to the above Figure 1.

# Advance Notification(s)

One or more communications to Demand Resources of an impending Demand Response Event in advance of the actual event.

# Demand Response Event

The time periods, deadlines and transitions during which Demand Resources perform.  The System Operator shall specify the duration and applicability of a Demand Response Event.  All deadlines, time periods and transitions may not be not applicable to all Demand Response products or services.

# Deployment

The time at which a Demand Resource begins reducing Demand on the system in response to an instruction.

# Deployment Period

The time in a Demand Response Event beginning with the Deployment and ending with the Release/Recall.

# Normal Operations

The time following Release/Recall at which a System Operator may require a Demand Resource to have returned its Load consumption to normal levels, and to be available again for Deployment.

# Ramp Period

The time between Deployment and Reduction Deadline, representing the period of time over which a Demand Resource is expected to achieve its change in Demand.

# Recovery Period

The time between Release/Recall and Normal Operations, representing the window over which Demand Resources are required to return to their normal Load.

# Reduction Deadline

The time at the end of the Ramp Period when a Demand Resource is required to have met its Demand Reduction Value obligation.

# Release/Recall

The time when a System Operator or Demand Response Provider notifies a Demand Resource that the Deployment Period has ended or will end.

# Sustained Response Period

The time between Reduction Deadline and Release/Recall, representing the window over which a Demand Resource is required to maintain its reduced net consumption of electricity.

Non Dispatchable Demand Response Terms

(Comment, Dominion:  –Additional text describing non dispatchable programs should be included and the group will add at a subsequent working session)

GENERAL TERMS

# Adjustment Window

The period of time prior to a Demand Response Event used for calculating a Baseline adjustment.

# After-the-Fact Metering

Interval meter data separate from Telemetry that is used to measure Demand Response.  May not apply to Demand Resources under Baseline Type II (Non-Interval Meter).

# Aggregated Demand Resource

A group of independent Load facilities that provide Demand Response services as a single Demand Resource.

Applicable Regulatory Authority

The state regulatory agency or other local governing body that provides oversight, policy guidance, and direction to any parties involved in the process of providing energy to retail access Customers through regulation and orders.

# Baseline

A Baseline is a method of estimating the electricity that would have been consumed by a Demand Resource in the absence of a Demand Response Event. Figure 2. below illustrates the concept of Baseline relative to a Demand Response Event.

Figure 2.  Illustration of Baseline Concept.

# Baseline Adjustment

An adjustment that modifies the Baseline to reflect actual conditions immediately prior to or during a Demand Response Event to provide a better estimate of the energy the Demand Resource would have consumed but for the Demand Response Event. The adjustments may include but are not limited to weather conditions, near real time event facility Load, current Demand Resource operational information, or other parameters based on the System Operator’s requirements.

# Baseline Type-I (Interval Metered)

A Baseline performance evaluation methodology based on a Demand Resource’s historical interval meter data which may also include other variables such as weather and calendar data.

# Baseline Type-II (Non-Interval Metered)

A Baseline performance evaluation methodology that uses statistical sampling to estimate the electricity consumption of an Aggregated Demand Resource where interval metering is not available on the entire population.

# Baseline Window

The window of time preceding and optionally following, a Demand Response Event over which the electricity consumption data is collected for the purpose of establishing a Baseline.  The applicability of this term is limited to Meter Before/Meter After, and Baseline Type-I and Type-II.

Capacity Service

# A type of Demand Response service in which Demand Resources are obligated over a defined period of time to be available to provide Demand Response upon deployment by the System Operator.

# Critical Peak Pricing:

Customer:

Any entity that takes gas and/or electric service for its own consumption.

Demand Resource Availability Measurement:

The amount of Load available to be dispatched for a given Demand Response Event.

Demand Response Event:

The time periods, deadlines and transitions during which Demand Resources perform.  The System Operator shall specify the duration and applicability of a Demand Response Event.  All deadlines, time periods and transitions may not be not applicable to all Demand Response products or services.

Demand Response Provider

The entity that is responsible for delivering Demand reductions from Demand Resources.

# Demand

The rate at which electric energy is delivered to or by a system or part of a system, generally expressed in kilowatts or megawatts, at a given instant or averaged over any designated interval of time; and the rate at which energy is being used by the customer  (NERC Definition).

# Demand Reduction Value

Quantity of reduced electrical consumption by a Demand Resource, expressed as MW or MWh.

# Demand Resource

A Load or aggregation of Loads capable of measurably and verifiably providing Demand Response.

# Demand Response

Changes in electric use by demand-side resources from their normal consumption patterns in response to changes in the price of electricity, or to incentive payments designed to induce lower electricity use at times of high wholesale market prices or when system reliability is jeopardized

Direct Load Control:

Distribution Company:  A regulated entity which provides distribution services and may provide energy and/or transmission / transportation services in a given area.

Energy Service:  A type of Demand Response service in which Demand Resources are compensated solely based on their performance during a Demand Response Event.

Firm Service Level:

Governing Documents:  Documents that determine the interactions among parties, including but not limited to: regulatory documents (e.g., tariffs, rules, regulations), contractual agreements, and Distribution Company Operational Manuals.

Guaranteed Load Drop

Highly-Variable Load

A Load with a fluctuating or unpredictable electricity consumption pattern.

# Load

An end-use device or customer that receives power from the electric system (NERC Definition).

Market Participant

A party engaged in the process of providing competitive retail energy to end-use Customers including, but not limited to, the Distribution Company, the Supplier, the Registration Agent, the settlement agent, and the meter reading entity.

# Maximum Base Load

A performance evaluation methodology based solely on a Demand Resource’s ability to not exceed a specified level of electricity Demand, regardless of its electricity consumption or Demand at Deployment.

# Measurement and Verification (M&V):

# The Baseline is compared to the actual metered electricity consumption during the Demand Response Event to determine the Demand Reduction Value.  Depending on the type of Demand Response product or service, Baseline calculations may be performed in real-time or after-the-fact.  The Program Administrator may offer multiple Baseline models and may assign a Demand Resource to a model based on the characteristics of the Demand Resource’s Load or allow the Demand Resource to choose a performance evaluation model consistent with its load characteristics from a predefined list.

# Meter Before / Meter After

A performance evaluation methodology where electricity Demand over a prescribed period of time prior to Deployment is compared to similar readings during the Sustained Response Period.

# Meter Data Recording Interval

The time between electricity meter consumption recordings.

# Meter Data Reporting Deadline

The maximum allowed time from the end of a Demand Response Event (Normal Operations) to the time when meter data is required to be submitted for performance evaluation and settlement.  The Meter Data Reporting Deadline may be either relative (a number of hours/days after Normal Operations) or fixed (a fixed calendar time, such as end-of-month).

# Metering Generator Output

A performance evaluation methodology, used when a generation asset is located behind the Demand Resource’s revenue meter, in which the Demand Reduction Value is based on the output of the generation asset.

# Non-Spinning Reserve:

# Performance Window

The period of time in a Demand Response Event analyzed by the System Operator to measure and verify the Demand Reduction Value for a Demand Resource.

Ramp Period

# The time between Deployment and Reduction Deadline, representing the period of time over which a Demand Resource is expected to achieve its change in Demand.

Ramp Rate

The rate, expressed in megawatts per minute, that a generator changes its output. (NERC Definition)   Demand Resource ramp rate is the rate, expressed in megawatts per minute, that a Demand Resource changes its Load.

# Real Time Pricing

Regulation Service

A type of Demand Response service in which a Demand Resource increases and decreases Load in response to real-time signals from the System Operator.  Demand Resources providing Regulation Service are subject to dispatch continuously during a commitment period.  Provision of Regulation Service does not correlate to Demand Response Event timelines, deadlines and durations as depicted in Figure 1.

# Spinning Reserve:

# System Operator

A System Operator is a Balancing Authority, Transmission Operator, or Reliability Coordinator whose responsibility is to monitor and control an electric system in real time (based on NERC definition).  The System Operator is responsible for initiating Advance Notifications, Deployment, and Release/Recall instructions.

# Telemetry

Real-time continuous communication between a Demand Resource or Demand Response Provider and the System Operator.

# Telemetry Interval

The time unit between communications between a Demand Resource or Demand Response Provider and a System Operator.

# Time-of-Use

# Validation, Editing and Estimation

The process of taking raw meter data and performing validation and, as necessary, editing and estimation of corrupt or missing data, to create validated data.  (VEE guidelines are published in the Edison Electric Institute’s Uniform Business Practices for Unbundled Electricity Metering, Volume Two, Published 12/05/00, http://www.naesb.org/REQ/req_form.asp) (Rip – should we refer to AEIC business practices here?)

Business Practice Requirements:

Provision of Demand Response Products

Applicability

The Standard applies to any entity that administers retail Demand Response dispatchable Products.

Purpose

The purpose of this Standard is to ensure that regulatory commissions and participants in retail electric markets in which dispatchable demand response products are administered have access to uniform information that will enable them to report consistent values for measurement and verification of the programs.

  - General Characteristics of a Demand response

  - event Advance Notification

- RXQ.13.3.1 Where is this item, or are the numbers given to the proposed standards incorrect?

RXQ.13.3.1.1	Demand Response may be initiated through either a remote-activated switch or an on-site control such as an advanced thermostat or manual intervention.

RXQ.13.3.1.2	If a remote-activated switch is used, no Advance Notification is required prior to a Demand Response Event.

RXQ.13.3.1.3	If an on-site control is used, then an Advance Notification should be sent.  The timing and number of the Advance Notification(s) should be specified in the Governing Documents.

- Deployment Time

RXQ.13.3.1.4	Deployment of the Demand Resource will depend on the specific circumstance(s) of the Demand Response Event and should be specified by the Program Administrator in the Advance Notification.

- Reduction Deadline

RXQ.13.3.1.5	The Reduction Deadline will depend on the specific circumstance(s) of the Demand Response Event and should be specified by the Program Administrator in the Advance Notification.

RXQ.13.3.1.6	The requirement for a Ramp Period or a specified Ramp Rate will depend on the specific circumstance(s) of the Demand Response Event and should be specified by the Program Administrator in the Advance Notification.

- Release/Recall

RXQ.13.3.1.7	The Release / Recall will depend on the specific circumstance(s) of the Demand Response Event and should be specified by the Program Administrator.

- Normal Operations

RXQ.13.3.1.8	Normal Operations will depend on the specific circumstance(s) of the Demand Response Event and the Recovery Period should be specified by the Program Administrator when the Release / Recall is sent.

- Demand Resource Availability Measurement

RXQ.13.3.1.9	The Demand Resource Availability Measurement is a Megawatt value and is calculated as the number of Direct Load Control units installed times the Operability Factor times the per unit reduction impact.

- Aggregation

RXQ.13.3.1.10	Aggregation may be permitted and should be specified in the Governing Documents.

- Transparency of Requirements

RXQ.13.3.1.11	All specific requirements for a Demand Response Event should be specified in the Governing Documents, thereby insuring transparency.

RXQ.13.3.1.11.1	All Customer specific data must remain confidential.

RXQ.13.3.2 Direct Load Control Programs

RXQ.13.3.2.1	Telemetry is not required for a Direct Load Control program.

RXQ.13.3.2.2	After-the-fact Measurements

RXQ.13.3.2.2.1	After-the-Fact Measurement is required and may be either by metering each individual site or by statistical sampling.

RXQ.13.3.2.2.2	Meter accuracy should meet or exceed industry standards or as specified by the Applicable Regulatory Authority.

RXQ.13.3.2.2.3	Meters and other equipment should meet or exceed industry standards equivalent to ANSI C12 or as specified by the Applicable Regulatory Authority.

RXQ.13.3.2.2.4	The Meter Data Reporting Deadline should be specified in the Governing Documents.

RXQ.13.3.2.2.5	The Meter Data Reporting Interval should be specified in the Governing Documents.

RXQ.13.3.2.2.6	The meter clock / time accuracy should meet or exceed industry standards equivalent to ANSI C12 or as specified by the Applicable Regulatory Authority.

RXQ.13.3.2.2.7	The method of Validating, Editing and Estimation should conform to an accepted methodology (such as the guidelines published in the current edition of the Edison Electric Institute’s Uniform Business Practices for Unbundled Electricity Metering), and should be specified in the Governing Documents.

RXQ.13.3.2.3	Statistical Sampling

RXQ.13.3.2.3.1	Design the sample to meet program objectives.

RXQ.13.3.2.3.2	Define the population

RXQ.13.3.2.3.3	Specify the Frame (the listing of units available to be sampled is the sampling frame).

RXQ.13.3.2.3.4	The sample should achieve an accuracy of 90% confidence with 20% error, but be designed to achieve a minimum accuracy of 90% confidence with 10% error.

RXQ.13.3.2.3.5	Identify design (auxiliary) variables

RXQ.13.3.2.3.6	Choose sampling technique

RXQ.13.3.2.3.6.1	Choose stratification variable(s)

RXQ.13.3.2.3.6.2	Select allocation procedure

RXQ.13.3.2.3.6.3	Estimate means and variances of loads

RXQ.13.3.2.3.6.4	Examine sample size requirements

RXQ.13.3.2.3.6.5	Select sampling techniques and design

RXQ.13.3.2.3.7	Determine sample size

RXQ.13.3.2.8	Select the sample (identify those Customers to be in the sample)

RXQ.12.3.1.3.1	Determine alternate selection criteria (identify the criteria for selecting those Customers to be substituted for sample Customers who decline)

RXQ.12.3.1.3.2	Select sample and alternates

RXQ.12.3.1.3.3	Validate sample

RXQ.13.3.2.9	Implement the sample (contacting and enrolling the Customers to be in the sample, and installing the metering devices)

RXQ.13.3.3	 Guaranteed Load Drop Programs

No review has been performed on this section – it is the exact duplicate of the wholesale electric QUadrant standards

  - TELEMETRY

- Telemetry Requirement

RXQ.13.3.3.1.1	Real time demand measurement may or may not be required.

RXQ.13.3.3.1.2	Real time energy measurement is required

- On-Site Generation Telemetry

RXQ.13.3.3.1.7	Any on-site generation requirements should be specified in the Governing Documents.

- Telemetry Accuracy

.

RXQ.13.3.3.1.3	Meter accuracy should meet or exceed industry standards or as specified by the Applicable Regulatory Authority.

- Telemetry Interval

.

RXQ.13.3.3.1.4	Reporting interval (the time between signals in seconds) should be specified in the Governing Documents.

- Other Telemetry Measurements

.

RXQ.13.3.3.1.5	Other measurements (if other variables are measured, provide the list here; e.g. quality flags, breaker status, response amount, etc.) should be specified in the Governing Documents.

- Communication Protocol

.

RXQ.13.3.3.1.6	All communication protocols should be specified in the Governing Documents.

- Governor Control Equivalent

Not applicable.

  - AFTER-THE-FACT METERING

- After-the-Fact Metering Requirement

After-the-Fact Metering is required unless otherwise specified by the Program Administrator.

RXQ.13.3.3.2.1	After-the-fact measurement is required.

- Meter Accuracy

RXQ.13.3.3.2.2	Meter accuracy should meet or exceed industry standards or as specified by the Applicable Regulatory Authority.

- Details of Meter/Equipment Standards

.

RXQ.13.3.3.2.4	Meters and other equipment should meet or exceed industry standards equivalent to ANSI C12 or as specified by the Applicable Regulatory Authority.

- Meter Data Reporting Deadline

The Program administrator shall specify the Meter Data Reporting Deadline.

- Meter Data Reporting Interval

The Program administrator shall specify the Meter Data Reporting Interval at a value not to exceed 1 hour.

- Clock / Time Accuracy

RXQ.13.3.3.2.3	Clock / time accuracy

RXQ.13.3.3.2.3.1	The meter clock / time accuracy should meet or exceed industry standards equivalent to ANSI C12 or as specified by the Applicable Regulatory Authority.

RXQ.13.3.3.2.3.2	Time should be recorded in local time unless the population spans more than one time zone.  In that event, the predominate time should be used and the other time zone(s) should be converted to the selected time zone.

- Validating, Editing & Estimating (VEE) Method

The Program Administrator shall specify VEE requirements.

RXQ.13.3.3.2.5	Validating, editing and estimating (VEE) (how missing data is managed: e.g. skipped, estimated & flagged, etc.)

- On-Site Generation Meter Requirement

The Program administrator shall specify additional metering requirements if on-site generation is present behind the primary metering point.

  - PERFORMANCE EVALUATION

- Rules for Performance Evaluation

Performance shall be evaluated through the use of one of the following methods unless otherwise specified by the Program Administrator:

- Maximum Base Load

- Meter Before / Meter After

- Baseline Type-I

- Baseline Type-II

- Metering Generator Output

Business Practice Requirements

Maximum Base Load Evaluation

  - BASELINE INFORMATION

There are no Baseline calculations defined for Maximum Base Load evaluations.  The Maximum Base Load Evaluation methodology shall be associated with a maximum demand.

  - EVENT INFORMATION

    - Use of Real-Time Telemetry

The Program administrator shall specify if real-time Telemetry data is to be used to measure performance.

    - Use of After-The-Fact Metering

After-the-fact metering shall be used to measure performance, unless otherwise specified by the Program Administrator.

    - Performance Window

The Performance Window shall be the Sustained Response Period (Reduction Deadline through Release/Recall) unless otherwise specified by the Program administrator.

    - Measurement Type

During the Performance Window, the Demand Resource must maintain its electricity consumption at or below the Maximum Base Load.  The criteria used to evaluate performance shall be one of the following unless otherwise specified by the Program Administrator:

    - Peak Demand

    - Average Demand

  - SPECIAL PROCESSING

The Program Administrator shall specify any special processing rules.

Business Practice Requirements

Meter Before / Meter After

  - BASELINE INFORMATION

    - Baseline Window

The Program Administrator shall specify the Baseline Window.

    - Calculation Type

During the Baseline Window, the energy consumption or Demand of the Demand Resource shall be evaluated using one of the following measurements unless otherwise specified by the Program Administrator:

- Instantaneous

- Maximum

- Average

    - Sampling Precision and Accuracy

Sampling is not permitted for this performance evaluation type, unless otherwise specified by the Program Administrator.

    - Exclusion Rules

The Program Administrator shall specify any exclusion rules.

    - Baseline Adjustments

The Program Administrator shall specify any event-day adjustments.

    - Adjustment Window

No Adjustment Window is used for this model unless otherwise specified by the Program Administrator.

  - EVENT INFORMATION

    - Use of real-time Telemetry

The Program Administrator shall specify if real-time Telemetry data is to be used to measure performance.

    - Use of After-The-Fact Metering

After-the-fact metering shall be used to measure performance, unless otherwise specified by the Program Administrator.

    - Performance Window

The Performance Window shall be the Sustained Response Period (Reduction Deadline through Release/Recall) unless otherwise specified by the Program Administrator.

    - Measurement Type

During the Performance Window, the Demand Resource shall be evaluated using one of the following measurements unless otherwise specified by the Program Administrator:

- Instantaneous

- Maximum

- Average

  - SPECIAL PROCESSING

    - Highly-Variable Load Logic

The Program Administrator shall specify any performance evaluation requirements for Highly-Variable Loads.

    - On-Site Generation Requirements

The Program Administrator shall specify any performance evaluation requirements for on-site generation.

Business Practice Requirements

Baseline Type-I (Interval Meter)

  - BASELINE INFORMATION

    - Baseline Window

The Program Administrator shall specify the Baseline Window.

    - Calculation Type

The Program Administrator shall specify the method of developing the Baseline value using, but not limited to, the following calculation types:

- Maximum

- Average

- Regression

    - Sampling Precision and Accuracy

Sampling is not permitted for this Performance Evaluation type, unless otherwise specified by the Program Administrator.

    - Exclusion Rules

The Program Administrator shall specify any rules for excluding data from the Baseline Window.  Exclusion rules may be based on, but are not limited to the following:

- Historical Demand Response Events

- Testing/Audit Periods

- Calendar data

- Outages

- Weather emergencies or force majeure events

- Usage threshold

- Known, discrete load additions or reductions that have occurred during the Baseline Window

    - Baseline Adjustments

The Program Administrator shall specify any rules for Baseline Adjustments.  Adjustment rules may be based on, but are not limited to the following:

- Temperature

- Humidity

- Calendar data

- Sunrise/Sunset time

- Event  day operating conditions

    - Adjustment Window

The Program Administrator shall specify the Adjustment Window.

  - EVENT INFORMATION

    - Use of Real-Time Telemetry

The Program Administrator shall specify if real-time Telemetry data is to be used to measure performance.

    - Use of After-The-Fact Metering

After-the-fact metering shall be used to measure performance, unless otherwise specified by the Program Administrator.

    - Performance Window

The Program Administrator shall specify the Performance Window.

    - Measurement Type

During the Performance Window, the Demand Resource shall be evaluated using one of the following measurements unless otherwise specified by the Program Administrator:

- Maximum

- Average

- Regression

  - SPECIAL PROCESSING

    - Highly-Variable Load Logic

The Program Administrator may specify performance evaluation requirements for Highly-Variable Loads.

    - On-Site Generation Requirements

The Program Administrator may specify performance evaluation requirements for on-site generation.

Business Practice Requirements

Baseline Type-II (Non-Interval Meter)

  - BASELINE INFORMATION

    - Baseline Window

The Program Administrator shall specify the Baseline Window.

    - Calculation Type

The Program Administrator shall specify the method of developing the Baseline value using, but not limited to, the following calculation types:

- Maximum

- Average

- Regression

    - Sampling Precision and Accuracy

The Program Administrator shall specify sampling precision and accuracy requirements.

    - Exclusion Rules

The Program Administrator shall specify any rules for excluding data from the Baseline Window.  Exclusion rules may be based on, but are not limited to the following:

- Historical Demand Response Events

- Testing/Audit Periods

- Calendar data

- Outages

- Weather emergencies or force majeure events

- Usage threshold

- Known, discrete load additions or reductions that have occurred during the Baseline Window

    - Baseline Adjustments

The Program Administrator shall specify any rules for Baseline Adjustments.  Adjustment rules may be based on, but are not limited to the following:

- Temperature

- Humidity

- Calendar data

- Sunrise/Sunset time

- Event  day operating conditions

    - Adjustment Window

The Program Administrator shall specify the Adjustment Window.

  - EVENT INFORMATION

    - Use of Real-Time Telemetry

The Program Administrator shall specify if real-time Telemetry data is to be used to measure performance.

    - Use of After-The-Fact Metering

After-the-fact metering or other energy measurement technology shall be used to measure performance, as a supplement to real-time Telemetry unless otherwise specified by the Program Administrator.

    - Performance Window

The Program Administrator shall specify the Performance Window.

    - Measurement Type

During the Performance Window, the Demand Resource shall be evaluated using one of the following measurements unless otherwise specified by the Program Administrator:

- Maximum

- Average

- Regression

  - SPECIAL PROCESSING

The Program Administrator shall specify any special processing rules.

Business Practice Requirements

Metering Generator Output

  - BASELINE INFORMATION

The Program Administrator shall specify Baseline calculations for Metering Generator Output.

  - EVENT INFORMATION

    - Use of Real-Time Telemetry

The Program Administrator shall specify if real-time Telemetry data is to be used to measure performance.

    - Use of After-The-Fact Metering

After-the-fact metering on the generator and optionally on the associated Load shall be used to measure performance unless otherwise specified by the Program Administrator.

    - Performance Window

The Program Administrator shall specify the Performance Window.

    - Measurement Type

During the Performance Window, the Demand Resource shall be evaluated using the total measured generation output unless otherwise specified by the Program Administrator.

  - SPECIAL PROCESSING

The Program Administrator shall specify any special processing rules.