1.	RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| X | Accept as requested | X | Change to Existing Practice |
| --- | --- | --- | --- |
|  | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

2.	TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
| X | Initiation | X | Initiation |
|  | Modification |  | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
| X | Principle | X | Principle |
| X | Definition | X | Definition |
| X | Business Practice Standard | X | Business Practice Standard |
|  | Document |  | Document |
|  | Data Element |  | Data Element |
|  | Code Value |  | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
|  | Business Process Documentation |  | Business Process Documentation |

3.	RECOMMENDATION

SUMMARY:

The business process flows and use cases presented in this document illustrate the standard interactions between a System Operator and various Market Participants for the administration and deployment of demand response resources in organized wholesale electric markets.

The following addresses the business objectives and context for standardizing control and pricing signals for DR and DER as part of the Smart Grid implementation, which is called for by NIST PAP 09.  The NAESB SGTF in cooperation with UCAIug, OpenSG Task Force, the ISO/RTO Council, and other organizations, have developed DR use cases that provide requirements for developing consistent DR control and pricing signal standards.

The business process flows and use cases contained in this recommendation address the requirements for standardizing the information exchanged during interactions between the System Operator and various Market Participants for the administration and deployment of demand response resources in organized wholesale electric markets. Additional documents under development through the NAESB SGTF will focus on other aspects of the PAP 09 objectives. To maintain consistency throughout materials developed by the NAESB SGTF, a common set of actors, terms and definitions have been adopted.

The following describes the end-to-end business process flows relating to the wholesale market communications between the SO and the SP common to all ISOs/RTOs that offer opportunities for demand resources to participate in organized wholesale electric markets.

To accelerate the development of standards for Smart Grid Interoperability, NIST is working with all stakeholders of Smart Grid and has developed a set of PAPs.  A number of PAPs (03, 04, and 09) are related to the use of Smart Grid technologies to enable DR and the integration of DER for DR purposes.

Specifically for PAP 09, NAESB, with support from UCAIug OpenSG task forces and the ISO/RTO Council, have responsibility to collect, analyze, and consolidate use cases and develop UML based use case models for DR.  Such responsibility now falls under the NAESB SGTF.

NIST Smart Grid Interoperability Roadmap leverages the GWAC interoperability framework. This framework calls for the establishment of business objectives, procedures and context before technical interoperability standards can be established. To apply this framework to the development of interoperability standards for DR Signals, it is clear that the industry needs an overarching business framework to guide the development of technical standards, given the complexity and range of DR programs.

Recommended Standards:

In response to NIST’s Priority Action Plan 9, this document contains draft requirements specifications, in the form of business process flows and use cases, to support the standardization of the information exchanged during interactions between the System Operator and various Market Participants for the administration and deployment of demand response resources in organized wholesale electric markets. Common terminology from the NAESB Measurement and Verification Standards for Demand Response has been incorporated into the development of the business process flows and use cases. As a result of the development of this document, a standard set of actors and additional terminology will expand the existing NAESB documentation of associated terms and definitions for demand response.

WEQ-000

New Abbreviations/Acronyms and Defined Terms to be added to WEQ-000 are Included in the Recommendation Request Titled Requirements Specifications for Common Electricity Product and Pricing Definitions – for NIST PAP03 to avoid redundancy between recommendations.

WEQ-018

New Business Practice Standards WEQ-018 – Specifications for Wholesale Standard DR Signals

Recommended Standards:

# Introduction

# Specifications for Wholesale Standard Demand Response Signals

Purpose:

The purpose of the following is to provide standards developers with a context for understanding the range of scenarios to which DR programs and DERs may be applied and implemented across the various electricity systems and jurisdictions in the United States of America with some overlaps to Canada. A major objective in producing this document is to emphasize the importance of interoperability at all levels of the GWAC interoperability framework.

Scope:  There are certain principles and boundaries that have been established.  These include:

- Demand Response practices must be consistent with NERC and applicable regional reliability authority requirements.

- All involved entities are registered through the applicable ISO/RTO market participant registration process, which may include credit checks.  However, the specifics related to the business processes associated with registration are not documented in these requirements.

- Settlement input parameters are defined as an output to the measurement and performance business process.  However, specific business processes associated with settlements are not documented in these requirements.

- Intra-system operator information exchanges and specific system operators market rules, calculations, algorithms, and Performance Evaluation models are excluded.

- Planning functions are not documented in these requirements.  This includes, but is not limited to, long-term load forecasting and transmission planning.

- Capacity auctions, awards processes, and resource certification are not documented in these requirements.

- References in these Recommendations to “Dispatch”, “Markets”, and “Reliability”, are made relative to Demand Response and apply to Demand Response resources only, not to Generation resources.

- Compliance standards for Demand Response resources are determined by the market rules of the respective system operator and are specific to the product or service and the reliability need being addressed.

Business Practice Standards

# 018-1	Specifications for Wholesale Standard Demand Response Signals

The following describes the wholesale market business process flow and provides the necessary business context and reference architecture for use cases that connect to the retail level.  The descriptions provide a common linkage to the retail use cases presented in the companion specification for the Retail Electricity Quadrant.

# 018-1.1	Conventions

- Implementation does not require every communication flow shown in the process flows.

018-2	Actors

The actors listed in the table below are specific to this recommendation and are a subset of the actors and related objects defined in WEQ-000. Details on the relationships between the actors and related objects are further defined in Appendix A.

Table 1 - Actor Roles

| Term | Actor ID |
| --- | --- |
| Designated Dispatch Entity | 3.4 |
| Load Serving Entity | 3.2 |
| Meter Authority | 3.6 |
| Scheduling Entity | 3.3 |
| Service Provider | 3.1 |
| System Operator | 2.1 |
| Transmission/Distribution Service Provider | 3.5 |

## 018-1.3	Use Case Overview

The use cases presented in this document represent a combination of three dimensions of demand response participation in wholesale markets: product, deployment, and performance evaluation method.  As shown in the table below, product and deployment dimensions are identified by a single alphabetic character and performance evaluation methods are indicated by a numeric identifier; each character is separated by a dash (-). For example, use case E-R-1 refers to the use case for the energy (economic) product with a resource-specific deployment and performance evaluated using a Baseline.

The use cases included in this Business Practice Standard are not intended to be required or exhaustive and are provided for clarification purposes.

Table 2 - Use Case Dimensions

| Product |  |
| --- | --- |
| Energy (Economic) | E |
| Energy (Reliability) | R |
| Capacity | C |
| Reserve | V |
| Regulation | G |
| Deployment |  |
| Bulk | B |
| Resource | R |
| Self | S |
| Performance Evaluation |  |
| Baseline | 1 |
| MB/MA | 2 |
| MBL | 3 |
| MGO | 4 |

A sample of the use case list is shown in the table below.  Appendix B contains the complete list of use case combinations.

Table 3 - Sample Use Case List

The Figure 1 illustrates the process flows for use cases combinations that are considered to be representative of the types of demand response in wholesale electricity markets today. Each process flow description includes a table of the use cases to which the process flow applies.

The wholesale Demand Response end-to-end business process flow for the communications between the SO and the Service Provider SP is comprised of four major functions:

- Enrollment & Qualification,

- Scheduling & Award Notification,

- Deployment & Real-Time Communications,

- Measurement & Performance.

Two additional processes are shown in the high-level business process flow, but are considered outside the scope of what is required for the communications flow for demand response:

- Market Participant Registration

- Settlements

Figure 1 - Demand Response

## 018-1.4	Specific Use Cases

#### 018-1.4.1	Enrollment and Qualification

Overview: 	The Enrollment and Qualification process documents the steps required to enroll a Resource in a wholesale demand response program.

Use Cases: 	All valid use cases defined in this document (see Figure 2)

- The process begins when the SP submits an enrollment request to the SO.

- The SO processes the enrollment request, which may include interactions with the TDSP, LSE, MA and/or SE.

- The SO evaluates the enrollment request, which may include verification information from the TDSP, LSE, MA and/or SE.

- The result of the evaluation is the approval or rejection of the enrollment request.

  - If the enrollment request has been rejected, the SO sends information to the SP, indicating the rejection details.

  - If the enrollment request has been approved, a determination is made by the SO as to whether qualification of the Resource’s capability is required prior to final approval.

    - If no qualification is required, the SO sends information to the SP, indicating the approval of the enrollment.

    - If qualification is required, the SO coordinates the qualification procedure with the SP.

      - If the Resource has failed to qualify, the SO sends information to the SP, indicating the qualification rejection details.

      - If the Resource achieves qualification, the SO sends information to the SP and enrollment is finalized.

- At the end of the Enrollment and Qualification process, the SO finalizes enrollment, notifies the SP and may also notify the TDSP, LSE, MA and/or SE.

Activity Diagram and Data Flow:

Figure 2 - Enrollment and Qualification

#### 018-1.4.2	Scheduling and Award Notification – Economic

Overview: 	The Scheduling and Award Notification process describes the process from offer submission to award notification. This process also includes the supplemental commitment and reassessment of reliability to determine whether demand resources that are enrolled in reliability-based (emergency) demand response programs should be advised of a possible reliability deployment.

Table 4 - Use Cases for Figure 3

- The process begins when the SE submits a supply offer to the SO.

- The SO evaluates the offer through its market clearing process.

- The resulting resource-specific dispatch information is transformed into dispatch instructions by the SO.

- The SO makes the schedule available.

- The SE collects the forward schedule.

Activity Diagram and Data Flow:

Figure 3 - Scheduling and Award Notification - Economic

#### 018-1.4.3	Scheduling and Award Notification – Reliability

Overview: 	The Scheduling and Award Notification process describes the process from offer submission to award notification. This process also includes the supplemental commitment and reassessment of reliability to determine whether demand resources that are enrolled in reliability-based (emergency) demand response programs should be advised of a possible reliability deployment.

Table 5 - Use Cases for Figure 4

- The process begins when the SE submits availability to the SO.

- The SO performs the Load Forecast and Supplemental Commitment process after the market has been settled. Reliability is reassessed by the SO to determine whether the system is secure (sufficient supply to meet forecasted load conditions).

  - If SO determines that the system is secure, no demand response advance notification is required and reliability is assessed by the SO at the next interval.

  - If SO anticipates that a reliability issue is expected, the SO decides whether demand response is needed for reliability.

    - If demand response will not be needed, the process ends.

    - If the SO determines that demand response will be provided with an advance notification, the reliability event parameters are prepared by the SO to create a reliability event notification.

      - The advance notification message is sent to the SE.

      - The SE identifies the demand resources to notify and relays the message to SPs through the proprietary communication system of the SE.

Activity Diagram and Data Flow:

Figure 4 - Scheduling and Award Notification - Reliability

#### 018-1.4.4	Deployment and Real-Time Communications - Economic

Overview: 	The Deployment and Real-Time Communication process describes the process for real-time communications to demand resources providing market-based services on a real-time basis and dispatch for reliability-based (emergency) demand response programs. The process flows and descriptions below reflect a sequential nature to the real-time process solely for the purposes of describing the process of the real-time data flow; the real-time communication process simultaneously scans and updates in the same step.

Table 6 - Use Cases for Figure 5

- The process begins when the SO evaluates real-time system conditions through its market clearing process.

- The resulting resource-specific real-time dispatch information is transformed into dispatch instructions by the SO.

  - If the dispatch instruction is for Regulation, the SO sends the Regulation signal to the DDE.

  - If the dispatch is not for Regulation, the SO sends the dispatch instruction to the DDE.

- The DDE communicates with the SP through the proprietary communication system of the DDE.

- The SO collects real-time response from the DDE for evaluating the next interval and uses the information to calculate the system state.

- System conditions are made available to the market clearing process for the next interval and to Deployment and Real-Time Communications – Reliability.

Activity Diagram and Data Flow:

Figure 5 - Deployment and Real-Time Communications - Economic

#### 018-1.4.5	Deployment and Real-Time Communications – Reliability

Overview: 	The Deployment and Real-Time Communication process describes the process for real-time communications to demand resources providing market-based services on a real-time basis and dispatch for reliability-based (emergency) demand response programs. The process flows and descriptions below reflect a sequential nature to the real-time process solely for the purposes of describing the process of the real-time data flow; the real-time communication process simultaneously scans and updates in the same step.

Table 7 - Use Cases for Figure 6

- The SO performs the Load Forecast and Supplemental Commitment process after the interval has been dispatched. Reliability is reassessed by the SO to determine whether the system is secure (sufficient supply to meet forecasted load conditions).

  - If SO determines that the system is secure, no demand response deployment is required and reliability is assessed by the SO at the next interval.

  - If SO anticipates that a reliability issue is expected, the SO decides whether demand response is needed for reliability.

    - If demand response will not be needed, the process ends.

    - If the SO determines that demand response will be deployed, the reliability event parameters are prepared by the SO to create a reliability event notification.

      - The deployment instruction is sent to the DDE.

      - The DDE identifies the demand resources to notify and relays the message to SPs through the proprietary communication system of the DDE.

    - Real-time or near real-time response data is collected by DDE using the proprietary communication system of the DDE.

    - The DDE sends the event response information to the SO.

Activity Diagram and Data Flow:

Figure 6 - Deployment and Real-Time Communications - Reliability

#### 018-1.4.6	Measurement and Performance – Baseline

Overview: 	The Measurement and Performance process documents the steps to collect demand resource meter data and prepare the determinants for settlement.

Use Cases: 	All use cases where Baseline applies (see Figure 7).

- The process begins when the SP requests event meter data from the MA.

- The MA sends the event meter data to the SP.

- If the SP is required to calculate the baseline, the SP uses the event meter data to calculate the baseline.

  - The SP uses the baseline and event meter data to calculate the event reduction amount or event performance ratio.

  - The SP sends the calculated event performance and meter data to the SO.

  - The SO evaluates the calculated event performance and meter data.

    - If the calculated event performance data are complete, the SO uses the settlement determinants in the Settlements process.

    - If the calculated event performance data are incomplete, the SO sends an event data rejection to the SP.

- If the SO calculates the baseline, the SP sends the event meter data to the SO.

  - If the event meter data are complete, the SO uses the event meter data to calculate the baseline.

    - The SO uses the baseline and event meter data to calculate the event reduction amount or event performance ratio.

    - The SO uses the settlement determinants in the Settlements process.

  - If the event meter data are incomplete, the SO sends an event data rejection to the SP.

Activity Diagram and Data Flow:

Figure 7 - Measurement and Performance – Baseline

#### 018-1.4.7	Measurement and Performance - All Others

Overview: 	The Measurement and Performance process documents the steps to collect demand resource meter data and prepare the determinants for settlement.

Use Cases: 	All use cases where Meter Before/Meter After, Maximum Base Load, or Meter Generator Output applies (see Figure 8).

- The process begins when the SP collects event meter data from the MA for the specified performance evaluation method.

  - If the performance evaluation method is Meter Before/Meter After or Maximum Base Load, the MA sends the event meter data to the SP.

  - If the performance evaluation method is Meter Generator Output, the SP uses the data from the meter on the generator.

- If the SP is required to calculate performance, the SP uses the event meter data to calculate performance.

  - The SP uses the meter data to calculate the event reduction amount or event performance ratio.

  - The SP sends the calculated event performance and meter data to the SO.

  - The SO evaluates the calculated event performance and meter data.

    - If the calculated event performance data are complete, the SO uses the settlement determinants in the Settlements process.

    - If the calculated event performance data are incomplete, the SO sends an event data rejection to the SP.

- If the SO calculates performance, the SP sends the meter data to the SO.

  - If the event meter data are complete, the SO uses the event meter data to calculate performance.

    - The SO uses the event meter data to calculate the event reduction amount or event performance ratio.

    - The SO uses the settlement determinants in the Settlements process.

  - If the event meter data are incomplete, the SO sends an event data rejection to the SP.

Activity Diagram and Data Flow:

Figure 8 - Measurement and Performance – All Others

018-A	Appendix A – Entity-Relationship Model

The following terms and definitions correspond to a set of actor and objects within the overall information model for demand response.  Figure 1A contains a data model used to illustrate the entity-relationships within the model. Abbreviations/Acronyms correspond to the IDs shown in the figure.

Proper (and Improper) Usage of the Entity-Relationship Model.    The entity-relationship model is used to show cardinality among objects in the model, for example every Resource belongs to one and only one Service Provider and a Premise contains one or more End Devices.  The entity-relationship model does not provide use case information; actors are objects in the model and do not “act” in the model.  The entity-relationship model does not imply process.  For example, a Service Provider is associated with many Resources; however the enrollment of those Resources may be managed by a System Operators or a Utility Distribution Operator.  Both use cases and process maps are separate components of the Business Practice Standards.

Reading Crow’s Foot Notation.    Objects in the model which share a relationship are connected with a cardinality line.  Each end of the cardinality line contains a Crow’s Foot notation, as documents in the legend of the figure.  The four notations utilized are “exactly-one”, “one-or-more”, “zero-or-one”, and “zero-or-more”.  The cardinality line is bi-directional; meaning it can be read in two directions.  For example: a Premise is related to one-or-more End Devices (reading top to bottom) and every End Device is related to exactly-one Premise (reading bottom to top).

Optional Objects.  The entity-relationship model is designed to support multiple business models and not every business model will require all objects to function.  Therefore, all objects in the model are considered optional.  For example, a Utility Distribution Operator may design a Demand Response program which requires the definition of Resources and Premises, but does not require Response Method Aggregations and Response Methods.  In this example, the relationship between Resource and Premise is transitive: a Resource is comprised of one-or-more Premises and each Premise is associated with zero-or-more Resources.

Applicability to Retail and Wholesale.  The entity-relationship model is applicable to both wholesale and retail.  When using the model for wholesale, the applicable business entities are 2.1 (System Operator) and 2.3 (Market Participant), while in retail markets, the parallel business entities are 2.3 (Utility Distribution Operator) and 2.4 (Utility Customers).  Other than the swapping of the two pairs of terms, the models are identical, including the names of and relationships among objects.

Figure 1A. Entity-Relationship Model for Smart Grid Use Cases

018-B	Appendix B – Use Case Combinations

The following table is a complete list of use case combinations included in this Business Practice Standards.

# Supporting Documentation

a.  Description of Request:

On September 30, 2009, the National Institute of Standards and Technology officially assigned NAESB the responsibility to develop Requirements and Use Cases pertinent to Priority Action Plan items 3, 4 and 9. A complete description of each item is available on the NIST Web site located at http://collaborate.nist.gov/twiki-sggrid/bin/view/SmartGrid/WebHome  in the section titled Priority Action Plans (PAPs).

This recommendation specifically pertains to PAP 09, “Standard DR Signals”.

b.  Description of Recommendation:

See Section 3 Summary

c.  Business Purpose:

This recommendation describes the end-to-end business process flows relating to the wholesale market communications between the System Operator (SO) and the Service Provider (SP) common to all ISOs/RTOs that offer opportunities for demand resources to participate in organized wholesale electric markets.

- Commentary/Rationale of Subcommittee(s)/Task Force(s):

Joint Smart Grid Standards Task Force and Fix Protocol Meeting Notes/Documents:

  - September 17, 2009 Meeting Notes – To be posted

  - October 1, 2009 Meeting Notes – To be posted

  - October 15, 2009 Meeting Notes – To be posted

NAESB Smart Grid Standards Task Force Sub-Group Meeting Notes/Documents:

- October 13, 2009 Meeting Notes – To be posted

- October 20, 2009 Meeting Notes – To be posted

- October 27, 2009 Meeting Notes – To be posted

- November 3, 2009 Meeting Notes – To be posted

- November 4-6, 2009 Meeting Notes – To be posted

- November 10, 2009 Meeting Notes – To be posted

NAESB Smart Grid Standards Task Force Meeting Notes/Documents:

- October 22, 2009 Meeting Notes – http://naesb.org/pdf4/smart_grid_ssd102209notes.doc

- October 29, 2009 Meeting Notes – To be posted

- November 5, 2009 Meeting Notes – To be posted

- November 13, 2009 Meeting Notes – To be posted

- December 3, 2009 Meeting Notes – To be posted

- December 10, 2009 Meeting Notes –http://naesb.org/pdf4/smart_grid_ssd121009notes.doc

- December 17, 2009 Meeting Notes – http://naesb.org/pdf4/smart_grid_ssd121709notes.doc

- January 7, 2010 Meeting Notes – To be posted

- January 14, 2010 Meeting Notes – To be posted

- January 21, 2010 Meeting Notes – To be posted

- January 28, 2010 Meeting Notes – To be posted

- February 5, 2010 Meeting Notes – To be posted

## Considerations:

- The scope of DER applications will be limited to the context of grid-connected DR use cases and does not include the other possible applications of DER.

- The standards shall not prescribe a specific implementation technology.

- Information Technology security is explicit in the information model but is implicit in the business process models.

- Outputs from one process flows are considered inputs to subsequent following flows.

- A single entity may have multiple roles (e.g., a participant can function in the roles of DDE and MA).

- The process flows describe the standard path. Subsequent use cases will address exceptions other than standard data validation procedures.

e.  Additional Documentation from Subcommittee(s)/Task Force(s):

- The Task Force is including the following materials to include complete set of abbreviations, acronyms, and definitions.  The changes to the abbreviations, acronyms, and definitions are only listed in the Recommended Standards.

| Abbreviation / Acronym | Meaning |
| --- | --- |
| BAS | Building Automation System |
| CalConnect | Calendaring and Scheduling Consortium |
| CIM | Common Information Model |
| C&I | Commercial and Industrial |
| DDE | Designated Dispatch Entity |
| DER | Distributed Energy Resource |
| DR | Demand Response |
| EA | Environmental Authority |
| ED | End Device |
| EMS | Energy Management System |
| ESI | Energy Services Interface |
| FIX | Financial Information eXchange protocol |
| FR | Federal Regulator |
| GWAC | GridWise Architecture Council |
| iCalendar | Internet Calendaring and Scheduling Core Object Specification |
| IEC TC57 | International Electrotechnical Commission Technical Committee 57 |
| IETF | Internet Engineering Task Force |
| IRC Council | Independent System Operator/Regional Transmission Operator Council |
| ISIN | International Security Identifier Number |
| ISO | International Organization for Standardization |
| ISOs/RTOs | Independent System Operators/Regional Transmission Operators |
| kW | Kilowatt |
| kWh | Kilowatt Hour |
| LA | Local Authority |
| LSE | Load Serving Entity |
| MA | Meter Authority |
| MBL | Maximum Base Load |
| MB/MA | Meter Before/Meter After |
| MGO | Metering Generator Output |
| MP | Market Participant |
| MWh | Megawatt Hour |
| NIST | National Institute of Standards and Technology |
| PAP | Priority Action Plan |
| RM | Response Method |
| RMA | Response Method Aggregation |
| OpenSG | Open Smart Grid |
| PUC | Public Utility Commission |
| SE | Scheduling Entity |
| SGTF | Smart Grid Task Force |
| SO | System Operator |
| SP | Service Provider |
| SR | State Regulator |
| TDSP | Transmission/Distribution Service Provider |
| UC | Utility Customer |
| UCAIug | UCA International Users Group |
| UDO | Utility Distribution Operator |
| UML | Unified Modeling Language |
| XML | Extensible Markup Language |

| Term | Definition |
| --- | --- |
| Baseline | A Baseline is an estimate of the electricity that would have been consumed by a Demand Resource in the absence of a Demand Response Event. The Baseline is compared to the actual metered electricity consumption during the Demand Response Event to determine the Demand Reduction Value. Depending on the type of Demand Response product or service, Baseline calculations may be performed in real-time or after-the-fact. |
| Business Entity | The wholesale or retail entity that interacts with other entities in its market. |
| Communication Method | The method by which an object communicates with another object to instruct, measure, report or control. |
| Control | The role associated with the control of an End Device. |
| Designated Dispatch Entity (DDE) | A role which carries the responsibility of receiving and processing demand resource dispatch instructions or market information and (optionally) providing response information. |
| Distributed Energy Resources (DER) | DERs are small, modular, energy generation and storage technologies that provide electric capacity or energy where it is needed. DERs may be either connected to the local electric power grid (e.g., for voltage support) or isolated from the grid in stand-alone applications, such as part of a MicroGrid. Definition of DER provided by the Department of Energy, http://www1.eere.energy.gov/femp/pdfs/31570.pdf |
| End Device (ED) | A physical end-use device that consumes or supplies electricity. |
| Environmental Authority (EA) | A regulatory authority responsible for the development, reporting and enforcement of environmental activities. |
| Federal Regulator (FR) | A federal regulatory authority. |
| Load Serving Entity (LSE) | The responsible entity that secures energy and Transmission Service (and related Interconnected Operations Services) to serve the electrical demand and energy requirements of its end-use customers. |
| Local Authority (LA) | A regulatory authority responsible for the oversight and administration of utility service-related functions within its jurisdiction. |
| Market Enrollment | The collection of enrollment or tariff data for a Resource Object to provide a specific market product or service. |
| Market Participant (MP) | An organization registered with the System Operator that may take on roles such as SP, LSE, TDSP, DDE, SE, and/or MA in accordance with the SO’s market rules. |
| Maximum Base Load (MBL) | A performance evaluation methodology based solely on a Demand Resource’s ability to reduce to a specified level of electricity Demand, regardless of its electricity consumption or Demand at Deployment. |
| Measurement | The role associated with the device or algorithm that measures the consumption or supply of an End Device. |
| Meter Authority (MA) | A role which carries the responsibility of providing data necessary to determine the performance of a Resource. |
| Meter Before /Meter After (MB/MA) | A role which carries the responsibility of providing data necessary to determine the performance of a Resource. |
| Metering Generator Output (MGO) | A performance evaluation methodology, used when a generation asset is located behind the Demand Resource’s revenue meter, in which the Demand Reduction Value is based on the output of the generation asset. |
| P-Node | The price location of the Premise in the transmission and/or distribution network. |
| Participant | The entity that represents resources to a market or distribution operator. |
| Premise | The location at which connection to the transmission or distribution system is made. |
| Regulator | A rule-making and enforcement entity. |
| Response Method | A measurable action taken in response to an instruction to change consumption. |
| Response Method Aggregation (RMA) | A logical entity that has a reportable interval level consumption, e.g. an RMA may be a physical entity with its own meter, a neighborhood of homes that has a net meter, or an estimate of consumption of an aggregation of retail customers. |
| Resource | A market-dependent group of Response Method Aggregations that represents a dispatchable entity. |
| Resource Object | Physical and logical types of demand response resource objects. |
| Scheduling Entity (SE) | A role which carries the responsibility of submitting bids/offers and receives schedules and awards. |
| Service Provider (SP) | A role which carries the responsibility of coordinating resources to deliver electricity products and services to a market or distribution operator. |
| State Regulator (SR) | A regulatory authority responsible for the oversight and administration of electric utilities. |
| Supporting Objects | Objects that support the interaction of Business Entities and Resource Objects. |
| System Operator (SO) | A System Operator is a Balancing Authority, Transmission Operator, or Reliability Coordinator whose responsibility is to monitor and control an electric system in real time (based on NERC definition). The System Operator is responsible for initiating Advance Notifications, Deployment, and Release/Recall instructions. |
| Transmission/Distribution Service Provider (TDSP) | A role which carries the responsibility of operating a local electricity transmission and/or distribution system. |
| Utility Customer (UC) | An end-use customer of the Utility Distribution Operator that takes on roles such as Facility or Resource. |
| Utility Distribution Operator (UDO) | An entity which carries the responsibility of operating an electricity distribution system. |
| Zone | A physical or electrical region. |

http://collaborate.nist.gov/twiki-sggrid/bin/view/_SmartGridInterimRoadmap/PriorityActionPlans

Interoperability is the capability of two or more networks, systems, devices, applications, or components to share and readily use information securely and effectively with little or no inconvenience to the user. The characteristics of interoperability as set forth in the GridWise® Interoperability Context- Setting Framework are (1) exchange of meaningful, actionable information between two or more systems across organizational boundaries, (2) a shared understanding of the exchanged information, (3) an agreed expectation for the response to the information exchange, and (4) a requisite quality of service: reliability, fidelity, and security.  The result of interoperability is that interfaces, systems, and devices are capable of being integrated, scalable, adaptable, and upgradable and utilize open standards to promote competitive technology and to avoid stranded investments. Interoperability can only be achieved if there is a seamless, end-to-end coordination and exchange of data between many organizations, interfaces, systems, and devices resulting in behavior changing information given to the consumer.

The Actor ID shown in the table refers to the item number of the corresponding actor or related object in the Entity-Relationship Diagram provided in Appendix A. Definitions of the actors and related objects are included in WEQ-000.

http://collaborate.nist.gov/twiki-sggrid/bin/view/SmartGrid/PAP09Schedules