1.	RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| X | Accept as requested | X | Change to Existing Practice |
| --- | --- | --- | --- |
|  | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

2.	TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
| X | Initiation | X | Initiation |
|  | Modification |  | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
| X | Principle | X | Principle |
| X | Definition | X | Definition |
| X | Business Practice Standard | X | Business Practice Standard |
|  | Document |  | Document |
|  | Data Element |  | Data Element |
|  | Code Value |  | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
|  | Business Process Documentation |  | Business Process Documentation |

3.	RECOMMENDATION

SUMMARY:

The NAESB Smart Grid Task Force (SGTF) submits this Recommendation that provides requirements for common pricing attributes that can be used as a basis for messaging protocols throughout the electrical energy system from producer through to a variety of energy consumers and the future intelligent devices employed by these consumers. These attributes are not meant to be a message protocol in and of themselves and further analysis and design work will be completed as part of several initiatives, such as ZigBee smart energy, OpenHAN, and OASIS eMIX. These requirements should also be used as an input to the CIM maintained by UCAIug.

REQ.0

New Abbreviations/Acronyms and Defined Terms to be added to REQ.0 in support of 2010 REQ AP Items 9(a-c)
(NOTE: NAESB Retail numbering convention will be applied in the final action)

NOTE: Many of the new terms are being used in the three PAP recommendations rather than duplicating the abbreviations/acronyms in multiple documents they are only listed in this recommendation.)

ABBREVIATIONS AND ACRONYMS

| Abbreviation / Acronym | Meaning |
| --- | --- |
| BAS | Building Automation System |
| CalConnect | Calendaring and Scheduling Consortium |
| CIM | Common Information Model |
| C&I | Commercial and Industrial |
| DDE | Designated Dispatch Entity |
| DER | Distributed Energy Resource |
| DR | Demand Response |
| EA | Environmental Authority |
| ED | End Device |
| EMS | Energy Management System |
| ESI | Energy Services Interface |
| FIX | Financial Information eXchange protocol |
| FR | Federal Regulator |
| GWAC | GridWise Architecture Council |
| iCalendar | Internet Calendaring and Scheduling Core Object Specification |
| IEC TC57 | International Electrotechnical Commisssion Technical Committee 57 |
| IETF | Internet Engineering Task Force |
| IRC Council | Independent System Operator/Regional Transmission Operator Council |
| ISIN | International Security Identifier Number |
| ISO | International Organization for Standardization |
| ISOs/RTOs | Independent System Operators/Regional Transmission Operators |
| kW | Kilowatt |
| kWh | Kilowatt Hour |
| LA | Local Authority |
| MA | Meter Authority |
| MBL | Maximum Base Load |
| MB/MA | Meter Before/Meter After |
| MGO | Metering Generator Output |
| MP | Market Participant |
| PAP | Priority Action Plan |
| RM | Response Method |
| RMA | Response Method Aggregation |
| OpenSG | Open Smart Grid |
| PUC | Public Utility Commission |
| SE | Scheduling Entity |
| SGTF | Smart Grid Task Force |
| SO | System Operator |
| SP | Service Provider |
| SR | State Regulator |
| TDSP | Transmission/Distribution Service Provider |
| UC | Utility Customer |
| UCAIug | UCA International Users Group |
| UDO | Utility Distribution Operator |
| UML | Unified Modeling Language |

DEFINITION OF TERMS

| Term | Definition |
| --- | --- |
| Business Entity | The wholesale or retail entity that interacts with other entities in its market. |
| Communication Method | The method by which an object communicates with another object to instruct, measure, report or control. |
| Control | The role associated with the control of an End Device. |
| Designated Dispatch Entity (DDE) | A role which carries the responsibility of receiving and processing demand resource dispatch instructions or market information and (optionally) providing response information. |
| Distributed Energy Resources (DER) | DERs are small, modular, energy generation and storage technologies that provide electric capacity or energy where it is needed. DERs may be either connected to the local electric power grid (e.g., for voltage support) or isolated from the grid in stand-alone applications, such as part of a MicroGrid. Definition of DER provided by the Department of Energy, http://www1.eere.energy.gov/femp/pdfs/31570.pdf |
| End Device (ED) | A physical end-use device that consumes or supplies electricity. |
| Environmental Authority (EA) | A regulatory authority responsible for the development, reporting and enforcement of environmental activities. |
| Federal Regulator (FR) | A federal regulatory authority. |
| Load Serving Entity (LSE) | A role which carries the responsibility of serving end-users and selling electric energy to end-users. |
| Local Authority (LA) | A regulatory authority responsible for the oversight and administration of utility service-related functions within its jurisdiction. |
| Market Enrollment | The collection of enrollment or tariff data for a Resource Object to provide a specific market product or service. |
| Market Participant (MP) | An organization registered with the System Operator that may take on roles such as SP, LSE, TDSP, DDE, SE, and/or MA in accordance with the SO’s market rules. |
| Measurement | The role associated with the device or algorithm that measures the consumption or supply of an End Device. |
| Meter Authority (MA) | A role which carries the responsibility of providing data necessary to determine the performance of a Resource. |
| P-Node | The price location of the Premise in the transmission and/or distribution network. |
| Participant | The entity that represents resources to a market or distribution operator. |
| Premise | The location at which connection to the transmission or distribution system is made. |
| Regulator | A rule-making and enforcement entity. |
| Response Method | A measurable action taken in response to an instruction to change consumption. |
| Response Method Aggregation (RMA) | A logical entity that has a reportable interval level consumption, e.g. an RMA may be a physical entity with its own meter, a neighborhood of homes that has a net meter, or an estimate of consumption of an aggregation of retail customers. |
| Resource | A market-dependent group of Response Method Aggregations that represents a dispatchable entity. |
| Resource Object | Physical and logical types of demand response resource objects. |
| Scheduling Entity(SE) | A role which carries the responsibility of submitting bids/offers and receives schedules and awards. |
| Service Provider (SP) | A role which carries the responsibility of coordinating resources to deliver electricity products and services to a market or distribution operator. |
| State Regulator (SR) | A regulatory authority responsible for the oversight and administration of electric utilities. |
| Supporting Objects | Objects that support the interaction of Business Entities and Resource Objects. |
| Transmission/Distribution Service Provider (TDSP) | A role which carries the responsibility of operating a local electricity transmission and/or distribution system. |
| Utility Customer (UC) | An end-use customer of the Utility Distribution Operator that takes on roles such as Premise or Resource. |
| Utility Distribution Operator (UDO) | An entity which carries the responsibility of operating an electricity distribution system. |
| Zone | A physical or electrical region. |

REQ.15

New Model Business Practices REQ.15 – Specificaitons for Common Electricity Product and Pricing Definition
(NOTE: NAESB Retail numbering convention will be applied in the final action)

Recommended MODEL BUSINESS PRACTICES:

# Executive Summary

Specifications for Common Electricity Product and Pricing Definition

The following addresses the business objectives and context for capturing the attributes associated with electricity price and product signals as part of the Smart Grid implementation, which is called for by NIST PAP 03.  The NAESB SGTF in cooperation with UCAIug, OpenSG Task Force, the IRC Council, and other organizations, has developed the use cases that provide requirements for electricity price and product signal Model Business Practices.

Because the NAESB SGTF is involved in activities related to several of NIST’s PAPs, the SGTF has developed a standard list of actors and related objects for all PAP work coming out of the group. These Model Business Practices also contains an overview of the most common retail rate structures and price-related use cases. Finally, a representative list of price and product attributes is provided.

The following deals exclusively with the pricing related requirements at both the wholesale and retail levels.  The requirements are captured in the form of UML models, where business requirements are captured in use case narrative format and data requirements are captured in tabular format.  The main use cases that are affected by pricing are:

- Demand Response

- Non-Price (Environmental) Response

- Load-follower

- Price-takers

Reliability based responses, both wholesale and retail, is addressed by PAP-09 on DR Signaling.

# Introduction

Specifications for Common Electricity Product and Pricing Definition

## Purpose:  The purpose of the following is to capture business and data requirements related to the definition of a common model for capturing the attributes of an electricity product offered at wholesale and/or retail level.  Such attributes will mainly include but are not limited to the pricing information of the product being offered to the end consumers of electricity.

## Scope:  The scope of the following is to provide business and data requirements, not implementation, of a common pricing model for electricity product.

The electricity rate design process is not included in the scope of these use cases.

# Model Business Practices

## REQ.15.1	Specifications for Common Electricity Product and Pricing Definition

REQ.15.1.1	ACTORS LIST

The actors listed in the table below are specific to this recommendation and are a subset of the actors and related objects defined in REQ.0. Details on the relationships between the actors and related objects are further defined in Appendix A.

| Actor Name | Actor ID | Examples |
| --- | --- | --- |
| Communication Method | 5.5 |  |
| Control | 5.2 | Premise, EMS, ESI or Gateway |
| System Operator | 2.1 | Market and system operators |
| End Device | 4.5 | Electric vehicle, or smart appliance, display |
| Load Serving Entity | 3.2 |  |
| Measurement | 5.1 | Meter |
| Premise | 4.3 | Electricity consumer |
| Resource | 4.1 | DER |
| Service Provider | 3.1 | Retail service provider, demand response provider, dynamic price service provider |
| Transmission/ Distribution Service Provider | 3.5 | Utility distribution company |
| Utility Customer | 2.4 | Residential, industrial |
| Utility Distribution Operator | 2.3 | Distribution operator |

REQ.15.1.2	USE CASE DatA Considerations

The following tariff rate types and product and pricing attributes were considered when developing the use cases included in this Model Business Practice.

REQ.15.1.2.1	Tariff Rate Types Considered for Use Cases

A tariff establishes the prices paid for the services received, and is based on a variety of factors including wholesale prices, quality and type of service and costs associated with transmission and distribution grid infrastructure and operation. The use cases in these Model Business Practices illustrate the high-level activities associated with publishing prices and how the information is used to evoke a change in consumption. For the use cases included below the following tariff rate types were considered:

| Tariff Rate Type | Target Customers | Description |
| --- | --- | --- |
| block rate | C&I, residential | An energy supply rate structure in which the per unit energy price increases or decreases for each successive block of energy consumed. |
| critical peak price | C&I, residential | A pricing structure in which the customer receives notification identifying a time interval, (“critical hours”) during which special higher prices apply over and above whatever other rate they may be charged. Critical hours generally represent a small percentage of the hours in the year. |
| demand rate | Mainly for C&I customers, pilot for residential customers | A component of an energy supply rate structure based on the highest demand for electricity measured in a billing period. |
| day ahead market rate | C&I, residential | An energy supply rate structure, typically hourly, based on the day-ahead wholesale market price. |
| market clearing price for energy | C&I | An energy rate structure that allows for price changes every interval (e.g. 5 minutes, 15 minutes, hourly, etc.) based on real-time wholesale market prices. |
| peak time rebate | C&I, residential | An incentive rate in which the utility pays a rebate to customers who reduced demand during peak periods on critical days |
| real time price rate | C&I | An energy supply rate structure in which prices can vary, typically on an hourly basis, based on forecast (day-ahead) or actual (real-time) market conditions. |
| time of use rate | C&I, residential | An energy supply rate structure where the per unit charge (kWh or kW) varies according to the time of day. Time of use rates may have daily and seasonal variations. |
| variable peak pricing | C&I, residential | An energy supply rate structure that combines features of time of use and real time price structures to include the flexibility of market price variation and the fixed time periods of a time of use rate. |

# REQ.15.1.2.2	Product and Pricing Attributes Considered for Use Cases

For the purpose of the development of the use cases in Model Business Practice REQ.15.2, the following price and product attributes were considered.

## REQ.15.1.2.2.1	Product Identification

| Attribute | Description | Considerations |
| --- | --- | --- |
| Product Identifier | Energy, although largely a non-storable and transitory commodity, is nonetheless still a tradable and sellable entity. Any commodity benefits from having a unique identifier (external) that can unambiguously be used by consumers to identify what it is that they are purchasing. | Examples of product identification scheme used in the financial world is the ISIN and stock ticker symbols used by exchanges. This not a transaction identifier – this would be an industry agreed upon identifier for a particular energy product offering. |
| Identifier Source | Identification of the source of the Product Identifier. | In most systems there are competing or alternative product identification schemes. The Identifier Source references the product identification scheme for the Product Identifier. |
| Product Type | A product type. | Attempt to fit the product type within already in place ontologies or product classifications used within industry for trading energy. |
| Product Sub-Type | The sub-type of product | Products may be further divided into subtypes, for example reserve products may be 10-minute or 30-minute response. |
| Unit of Measure for Product | Unit of measure the product. | The unit of measure such as MWh or kWh. |

## REQ.15.1.2.2.2	Pricing Attributes

| Attribute |  | Description | Considerations |
| --- | --- | --- | --- |
| Total Price |  | Total Price of the product at point of delivery | Price is expressed as currency per unit of measure, i.e. quantity is not required, e.g. $/MWh The sum of all Price Components must equal the Total Price. |
| Set of 1..n Price Components |  |  |  |
|  | Price Component | A component of the price. | Price may have one or more components. |
|  | Price Component Type | The type of Price Component | A standard set of price component types shall be enumerated, such as: - Wholesale electricity cost - Transmission costs - Distribution costs |
| Price Type |  | Type of price | A standard set of price types will need to be defined real time, 5 minute, day ahead, as examples for price types |
| Override Unit of Measure for the Product |  | Unit of measure which is used to determine the price | Alternative unit of measure used in calculating the price. |
| Currency |  | Economic unit of exchange in which the total price and price components are stated (i.e. dollars, euros) | Currency is required to determine the units of measure of the Total Price and Price Components. The energy units of measure are defined by the Product Type. |
| Time and Interval |  | Interval of energy delivery | Timezone conventions and use need to be unambiguously defined. Daily blocks, hourly markets, 5 minute markets. A decision in how basic product information will reference and interact with the WS-Calendar initiative being completed as part of PAP-04. |
| Source Location Identifier |  | Location of the source of the electric energy | Electric energy cost and value are greatly impacted by the distance between generation and consumption. Indicating the source of the generation is important in terms of pricing decisions on part of consumers and intermediaries. The use of source location would mainly be used for bilateral transactions. |
| Source Location Identifier Type |  | Type of location. | The mechanism used to identify location can be defined in multiple sources based upon specific usage from control areas to GPS coordinates it is anticipated that there may need to be more than one method to identify a geographical location. Where possible existing industry standards should be adopted to identify geographic locations. |

# REQ.15.2	Product and Pricing Use Cases

REQ.15.2.1	Use Case Overview

The use case modeling technique is used for capturing business and data requirements relative to electric product and price information.  The relationships used in the use case diagrams are as follows:

- Generalization (line with triangle head): this indicates variations (sub-types) of the main use case.

- Precedes: this indicates a time sequence

- Invokes: this indicates a pre-condition or dependency

The use cases included in these Model Business Practices are not intended to be required or exhaustive and are provided for clarification purposes.

# REQ.15.2.2	PRICING USE CASE

The Pricing Use Case below provides an overview of the specific use cases described in these Model Business Practices. The results of bidding produce the settled price which is published and then received.  The recipient reacts to the price, which affects the next bidding cycle. The following use cases where electricity pricing and product information can be used are described in these Model Business Practices:

- Demand response

- Non-Price (Environmental) Response

- Load-follower

- Price-takers

### REQ.15.2.3	Specific Use Cases

Specific use cases for what end use consumers of electricity might do when pricing signals are received are listed as follows:

| Use Case | Actors | Description | SAMPLE Scenarios |
| --- | --- | --- | --- |
| Demand Response | SP ED Control Premise | Utility Customer (or device acting under UC direction) responds to Total Price or Price Component. | The System Operator published day-ahead prices which are statistically higher than average between 2:30 and 5:30 in the afternoon. Illustrate how different End Devices will respond to a forward energy price signal including: - An EMS requests BAS to “pre-cool” until 2:30 PM. - Home-based intelligent device is programmed to operate at a time when electricity is cheaper. An electric plug-in vehicle is programmed to charge when electricity is cheaper. A utility distribution company’s fixed time-of-day tariffs indicates higher price during day, and lower price after 11:00 PM. Demonstrate how different End Devices will response to price differentials: - Time-insensitive building systems schedule their run-time programs after 11:00 PM. Homeowner starts appliance at 5:10. The appliance notifies that by waiting 20 minutes to run, the consumer will get lower cost electricity. Homeowner acknowledges. |
| Non-Price (Environmental) Response | Resource ED Premise | Product characteristics (other than price) are used to influence either the quantity or the time at which the customer uses electricity. | Utility Customer sets a price threshold at which “green” power is desirable. Above the “green price” the customer will take any power, below the “green price” the customer will choose suppliers with renewable portfolios. Test the scenario where, at 5:00 PM wind power picks up such that the price of power from renewable sources falls below the “green price” set by the customer. Utility delivers blend of 20% wind, 80% coal during this time of day at .10/kWh. 25% of customers indicate preference to buy pure wind, an over-subscription. Prices for pure wind are bid up to .15/kWh as the wind-sensitive market decreases. 20% of consumers pay the 50% premium to get only wind-only power. - Local PUC requires that segmentation be revenue-neutral, so bidding for wind power reduces undifferentiated power to .0875 kWh. - No bidding was required for undifferentiated power, so provider is incentivized to find additional sources of wind power. - Consumers unable to buy desired wind power reduce their aggregate demand. Consumers have programmed non-time-sensitive systems and appliances to not run when wind power is not available, time-shifting some demand until wind power is available at the desired price. |
| Load-follower | DER UC |  | A UC is also a DER. The UC would purchase power up to the price at which the DER can economically produce power. The use case must prove that such a resource can receive enough information from the pricing signals to change its output according to the resource owner’s price curve. |
| Price-taker | DER |  | A DER with no storage capability and zero fuel cost, such as a wind farm or solar array, will produce power at whatever the “market rate” for generation is at the time of production. The use case must prove that such a resource can identify its cost curve (zero in this case), supply energy to the grid, and get paid at the appropriate rate. |

REQ.15.A	Appendix A – Entity-Relationship Model

The following terms and definitions correspond to a set of actor and objects within the overall information model for demand response.  Figure 1A contains a data model used to illustrate the entity-relationships within the model.

Proper (and Improper) Usage of the Entity-Relationship Model.    The entity-relationship model is used to show cardinality among objects in the model, for example every Resource belongs to one and only one Service Provider and a Premise contains one or more End Devices.  The entity-relationship model does not provide use case information; actors are objects in the model and do not “act” in the model.  The entity-relationship model does not imply process.  For example, a Service Provider is associated with many Resources; however the enrollment of those Resources may be managed by a System Operators or a Utility Distribution Operator.  Both use cases and process maps are separate components of the Model Business Practices.

Reading Crow’s Foot Notation.    Objects in the model which share a relationship are connected with a cardinality line.  Each end of the cardinality line contains a Crow’s Foot notation, as documents in the legend of the figure.  The four notations utilized are “exactly-one”, “one-or-more”, “zero-or-one”, and “zero-or-more”.  The cardinality line is bi-directional; meaning it can be read in two directions.  For example: a Premise is related to one-or-more End Devices (reading top to bottom) and every End Device is related to exactly-one Premise (reading bottom to top).

Optional Objects.  The entity-relationship model is designed to support multiple business models and not every business model will require all objects to function.  Therefore, all objects in the model are considered optional.  For example, a Utility Distribution Operator may design a Demand Response program which requires the definition of Resources and Premises, but does not require Response Method Aggregations and Response Methods.  In this example, the relationship between Resource and Premise is transitive: a Resource is comprised of one-or-more Premises and each Premise is associated with zero-or-more Resources.

Applicability to Retail and Wholesale.  The entity-relationship model is applicable to both wholesale and retail.  When using the model for wholesale, the applicable business entities are 2.1 (System Operator) and 2.3 (Market Participant), while in retail markets, the parallel business entities are 2.3 (Utility Distribution Operator) and 2.4 (Utility Customers).  Other than the swapping of the two pairs of terms, the models are identical, including the names of and relationships among objects.

Figure 1A. Entity-Relationship Model for Smart Grid Use Cases

# 4.	Supporting Documentation

a.  Description of Request:

On September 30, 2009 the National Institute of Standards and Technology officially assigned NAESB the responsibility to develop Requirements and Use Cases pertinent to Priority Action Plan items 3, 4 and 9. A complete description of each item is available on the NIST Web site located at http://collaborate.nist.gov/twiki-sggrid/bin/view/SmartGrid/WebHome

This recommendation specifically pertains to PAP 03, “Develop Common Specification for Price and Product Definition”.

b.  Description of Recommendation:

See Section 3 Summary

c.  Business Purpose:

This recommendation has been developed in response to a request from NIST to provide use cases and requirements germane to the development of common specification for electric price and production definition.

d.  Commentary/Rationale of Subcommittee(s)/Task Force(s):

Joint Smart Grid Standards Task Force and Fix Protocol Meeting Notes/Documents:

  - September 17, 2009 Meeting Notes – To be posted

  - October 1, 2009 Meeting Notes – To be posted

  - October 15, 2009 Meeting Notes – To be posted

NAESB Smart Grid Standards Task Force Sub-Group Meeting Notes/Documents:

- October 13, 2009 Meeting Notes – To be posted

- October 20, 2009 Meeting Notes – To be posted

- October 27, 2009 Meeting Notes – To be posted

- November 3, 2009 Meeting Notes – To be posted

- November 4-6, 2009 Meeting Notes – To be posted

- November 10, 2009 Meeting Notes – To be posted

NAESB Smart Grid Standards Task Force Meeting Notes/Documents:

- October 22, 2009 Meeting Notes – http://naesb.org/pdf4/smart_grid_ssd102209notes.doc

- October 29, 2009 Meeting Notes – To be posted

- November 5, 2009 Meeting Notes – To be posted

- November 13, 2009 Meeting Notes – To be posted

- December 3, 2009 Meeting Notes – To be posted

- December 10, 2009 Meeting Notes –http://naesb.org/pdf4/smart_grid_ssd121009notes.doc

- December 17, 2009 Meeting Notes – http://naesb.org/pdf4/smart_grid_ssd121709notes.doc

- January 7, 2010 Meeting Notes – To be posted

- January 14, 2010 Meeting Notes – To be posted

- January 21, 2010 Meeting Notes – To be posted

- January 28, 2010 Meeting Notes – To be posted

- February 5, 2010 Meeting Notes – To be posted

- February 18, 2010 Meeting Notes – To be posted

## Considerations:

- Wholesale market price signals may have different characteristics than retail market signals.

- Attempts to standardize pricing signals must not hinder regulatory changes or market innovations.

- Price signals should be flexible enough to accommodate regulatory changes or market innovations.

e.  Additional Documentation from Subcommittee(s)/Task Force(s):

- The Task Force is including the following materials to include complete set of abbreviations, acronyms, and definitions.  The changes to the abbreviations, acronyms, and definitions are only listed in the Recommended Model Business Practices.

| Abbreviation / Acronym | Meaning |
| --- | --- |
| BAS | Building Automation System |
| CalConnect | Calendaring and Scheduling Consortium |
| CIM | Common Information Model |
| C&I | Commercial and Industrial |
| DDE | Designated Dispatch Entity |
| DER | Distributed Energy Resource |
| DR | Demand Response |
| EA | Environmental Authority |
| ED | End Device |
| EMS | Energy Management System |
| ESI | Energy Services Interface |
| FIX | Financial Information eXchange protocol |
| FR | Federal Regulator |
| GWAC | GridWise Architecture Council |
| iCalendar | Internet Calendaring and Scheduling Core Object Specification |
| IEC TC57 | International Electrotechnical Commisssion Technical Committee 57 |
| IETF | Internet Engineering Task Force |
| IRC Council | Independent System Operator/Regional Transmission Operator Council |
| ISIN | International Security Identifier Number |
| ISO | International Organization for Standardization |
| ISOs/RTOs | Independent System Operators/Regional Transmission Operators |
| kW | Kilowatt |
| kWh | Kilowatt Hour |
| LA | Local Authority |
| LSE | Load Serving Entity |
| MA | Meter Authority |
| MBL | Maximum Base Load |
| MB/MA | Meter Before/Meter After |
| MGO | Metering Generator Output |
| MP | Market Participant |
| MWh | Megawatt Hour |
| NIST | National Institute of Standards and Technology |
| PAP | Priority Action Plan |
| RM | Response Method |
| RMA | Response Method Aggregation |
| OpenSG | Open Smart Grid |
| PUC | Public Utility Commission |
| SE | Scheduling Entity |
| SGTF | Smart Grid Task Force |
| SO | System Operator |
| SP | Service Provider |
| SR | State Regulator |
| TDSP | Transmission/Distribution Service Provider |
| UC | Utility Customer |
| UCAIug | UCA International Users Group |
| UDO | Utility Distribution Operator |
| UML | Unified Modeling Language |
| XML | Extensible Markup Language |

| Term | Definition |
| --- | --- |
| Baseline | A Baseline is an estimate of the electricity that would have been consumed by a Demand Resource in the absence of a Demand Response Event. The Baseline is compared to the actual metered electricity consumption during the Demand Response Event to determine the Demand Reduction Value. Depending on the type of Demand Response product or service, Baseline calculations may be performed in real-time or after-the-fact. |
| Business Entity | The wholesale or retail entity that interacts with other entities in its market. |
| Communication Method | The method by which an object communicates with another object to instruct, measure, report or control. |
| Control | The role associated with the control of an End Device. |
| Designated Dispatch Entity (DDE) | A role which carries the responsibility of receiving and processing demand resource dispatch instructions or market information and (optionally) providing response information. |
| Distributed Energy Resources (DER) | DERs are small, modular, energy generation and storage technologies that provide electric capacity or energy where it is needed. DERs may be either connected to the local electric power grid (e.g., for voltage support) or isolated from the grid in stand-alone applications, such as part of a MicroGrid. Definition of DER provided by the Department of Energy, http://www1.eere.energy.gov/femp/pdfs/31570.pdf |
| End Device (ED) | A physical end-use device that consumes or supplies electricity. |
| Environmental Authority (EA) | A regulatory authority responsible for the development, reporting and enforcement of environmental activities. |
| Federal Regulator (FR) | A federal regulatory authority. |
| Load Serving Entity (LSE) | A role which carries the responsibility of serving end-users and selling electric energy to end-users. |
| Local Authority (LA) | A regulatory authority responsible for the oversight and administration of utility service-related functions within its jurisdiction. |
| Market Enrollment | The collection of enrollment or tariff data for a Resource Object to provide a specific market product or service. |
| Market Participant (MP) | An organization registered with the System Operator that may take on roles such as SP, LSE, TDSP, DDE, SE, and/or MA in accordance with the SO’s market rules. |
| Maximum Base Load (MBL) | A performance evaluation methodology based solely on a Demand Resource’s ability to reduce to a specified level of electricity Demand, regardless of its electricity consumption or Demand at Deployment. |
| Measurement | The role associated with the device or algorithm that measures the consumption or supply of an End Device. |
| Meter Authority (MA) | A role which carries the responsibility of providing data necessary to determine the performance of a Resource. |
| Meter Before /Meter After (MB/MA) | A role which carries the responsibility of providing data necessary to determine the performance of a Resource. |
| Metering Generator Output (MGO) | A performance evaluation methodology, used when a generation asset is located behind the Demand Resource’s revenue meter, in which the Demand Reduction Value is based on the output of the generation asset. |
| P-Node | The price location of the Premise in the transmission and/or distribution network. |
| Participant | The entity that represents resources to a market or distribution operator. |
| Premise | The location at which connection to the transmission or distribution system is made. |
| Regulator | A rule-making and enforcement entity. |
| Response Method | A measurable action taken in response to an instruction to change consumption. |
| Response Method Aggregation (RMA) | A logical entity that has a reportable interval level consumption, e.g. an RMA may be a physical entity with its own meter, a neighborhood of homes that has a net meter, or an estimate of consumption of an aggregation of retail customers. |
| Resource | A market-dependent group of Response Method Aggregations that represents a dispatchable entity. |
| Resource Object | Physical and logical types of demand response resource objects. |
| Scheduling Entity (SE) | A role which carries the responsibility of submitting bids/offers and receives schedules and awards. |
| Service Provider (SP) | A role which carries the responsibility of coordinating resources to deliver electricity products and services to a market or distribution operator. |
| State Regulator (SR) | A regulatory authority responsible for the oversight and administration of electric utilities. |
| Supporting Objects | Objects that support the interaction of Business Entities and Resource Objects. |
| System Operator (SO) | A System Operator is a Balancing Authority, Transmission Operator, or Reliability Coordinator whose responsibility is to monitor and control an electric system in real time (based on NERC definition). The System Operator is responsible for initiating Advance Notifications, Deployment, and Release/Recall instructions. |
| Transmission/Distribution Service Provider (TDSP) | A role which carries the responsibility of operating a local electricity transmission and/or distribution system. |
| Utility Customer (UC) | An end-use customer of the Utility Distribution Operator that takes on roles such as Facility or Resource. |
| Utility Distribution Operator (UDO) | An entity which carries the responsibility of operating an electricity distribution system. |
| Zone | A physical or electrical region. |

Reference to the Organization for the Advancement of Structured Information Standards not the Open Access Same-Time Information System

The Actor ID shown in the table refers to the item number of the corresponding actor or related object in the Entity-Relationship Diagram provided in Appendix A. Definitions of the actors and related objects are included in REQ.0.

http://collaborate.nist.gov/twiki-sggrid/bin/view/SmartGrid/PAP03PriceProduct