The following comments are of the Midwest ISO on comments submitted by OATI.  Each Midwest ISO comment has a MISO tag in the comment.

The following are comments of OATI on the Recommendation for Rollover Rights on Redirect.  Of significant note, there appears to be some confusion over the wording versus the intent of Standard 9.7.8 (and related sub standards), and the application of that standard in Examples 11 and 15 particularly.  If the intent is that the capacity granted for the remaining term of the Parent Reservation to the Redirect shall always be limited to at most the Capacity Eligible for Rollover, there should be changes to the Standard made to more accurately reflect that intent.  If, however, the Standard stands as written, several Examples must be revised to reflect that the capacity redirected may be greater than the capacity eligible for rollover that is conveyed from the Parent Reservation.  In either case, the cited examples must be made internally consistent to whichever direction is chosen.

1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

X   Accept as requested			  X Change to Existing Practice

Accept as modified below			      Status Quo

Decline

2.  TYPE OF DEVELOPMENT/MAINTENANCE

Per Request:					Per Recommendation:

Initiation					      Initiation

X  Modification					 X   Modification

Interpretation				      Interpretation

Withdrawal					      Withdrawal

Principle 					      Principle

Definition 					      Definition

X  Business Practice Standard 			  X  Business Practice Standard

Document 					      Document

Data Element 				      Data Element

Code Value 					      Code Value

X12 Implementation Guide			     X12 Implementation Guide

Business Process Documentation		      Business Process Documentation

- 3.  RECOMMENDATION

- SUMMARY:

The Commission has issued various clarifications on its intended policy regarding the granting of rollover (renewal, evergreen) rights to requests for Redirect on a Firm Basis in Orders 676, 890 and 890-A.  The following recommendation is a  modification to WEQ-001-9.7 to reflect these clarifications in policy and implement the necessary standards for treatment of rollover rights for requests to Redirect on a Firm Basis.

Please refer to Section 4(a) for information related to the relevant FERC rulings, and Section 4(d) for the rationale behind the specific recommendations presented.

This recommendation addresses the WEQ 2009 Annual Plan Item2(a)(ii)(3):  Revisions to Standard 9.7

- Recommended Standards:

- Additions and corrections to WEQ-001 Definitions; enumeration to be established by NAESB staff at time of ratification:

- 001-0.nn

Capacity Eligible for Rollover - The net amount of capacity that is held on a qualifying Firm Point-to-Point Transmission Service reservation eligible for on-going rollover of service after subtracting the capacity held on any confirmed service reservations for renewal of those rollover rights for a subsequent term of service and after subtracting any capacity conveyed as an on-going rollover right to a qualifying confirmed Redirect on a Firm basis.

Footnote 1: Example: A Transmission Customer holds a Long-Term Parent Reservation for 100 MW with rollover limited to 90 MW pursuant to conditions identified when the Parent Reservation was negotiated.  The Customer has already renewed 25 MW on the Parent Reservation’s path and redirected an additional 20 MW with conveyance of rollover rights to the redirected path.

Therefore the Parent Reservation’s

Capacity Eligible for Rollover is 55 MW (i.e., 100-25-20).

- 001-0.30 	Unexercised Rollover Rights – the amount of capacity that is held on a Long- Term Firm Point-to-Point Transmission Service reservation and is eligible for ongoing rollover or renewal of service as established by the Transmission Provider at the time service was granted, less the capacity held on any confirmed service reservations for renewal of those rollover rights for a subsequent term of service, and less any capacity conveyed as an on-going rollover right to a qualifying confirmed Redirect on a Firm Basis..

The lower of the Capacity Eligible for Rollover or any restriction or limitation placed on the availability of on-going rollover of service as established by the Transmission Provider in the original service agreement after subtracting the capacity held on any confirmed service reservations for renewal of those rollover rights.

- Footnote 2:  Using the example in the footnote to the definition of Capacity Eligible for Rollover, the Unexercised Rollover Rights on the Parent Reservation shall equal the lower of the Capacity Eligible for Rollover in Footnote 1 (55 MW) or 65 (90 – 25) MW.

- Changes to WEQ-001-9

001-9.7	Unless otherwise mutually agreed to by the primary provider and original customer, a request for Redirect on a Firm basis does not impact the TC’s long term firm renewal rights (e.g., rollover or evergreen rights) on the original path, nor does it confer any renewal rights on the redirected path.A Transmission Customer holding Long-Term Firm Point-to-Point Transmission Service on the Transmission Provider’s system that is eligible for continued rollover rights of service may convey those rights to an alternate path or points of receipt and delivery through a request to Redirect on a Firm basis subject to the following requirements:

001-9.7.1	The Transmission Customer shall indicate their intent to convey on-going rollover rights held on a Long-Term Firm Point-to-Point Transmission Service reservation through a request to Redirect on a Firm basis submitted on the Transmission Provider’s OASIS in accordance with the following:

001-9.7.1.1	The Transmission Service attributes of the Redirect request must correspond to a valid Long-Term Firm Point-to-Point Transmission Service offered by the Transmission Provider; however, the duration (stop/start time) may be as noted in WEQ-001-9.7.4.

001-9.7.1.2	The Redirect request submission time must be prior to the deadline for exercising the Transmission Customer’s rollover rights on the Parent Reservation being redirected.

001-9.7.1.3	The Redirect request service stop time must be identical to the stop time (end of term) of the Parent Reservation being redirected.

001-9.7.1.4	All other aspects of the Redirect request must meet all additional requirements for Redirect on a Firm basis as specified in this Standard WEQ-001-9 or established by the Transmission Provider’s tariff, including any limitations on Redirects with respect to paths available for redirect, handling of differentials in pricing, etc.

001-9.7.2	Any Redirect request submitted that does not meet all of the requirements of WEQ-001-9.7.1.1 through WEQ-001-9.7.1.4 shall not be considered eligible for conveyance of rollover rights and shall be treated as any other Redirect on a Firm basis as provided for in these standards.

001-9.7.2.1	The Transmission Customer may waive the  right to convey rollover rights from the Parent Reservation to the Redirect request by explicitly indicating this intention in the Redirect request submitted on OASIS.  If so indicated, the Redirect request shall not be considered eligible for conveyance of rollover rights and shall be treated as any other Redirect on a Firm basis as provided for in these standards and the Capacity Eligible for Rollover on the Parent Reservation remains unchanged.

001-9.7.3	A Redirect request that complies with all of the requirements of WEQ-001-9.7.1.1 through WEQ-001-9.7.1.4, shall be considered eligible for the conveyance of rollover rights from the Parent Reservation.

001-9.7.4	The Transmission Provider shall treat a request for Redirect with rollover rights as any other new request for Long-Term Firm Point-to-Point Transmission Service (e.g., treated as Tier 1 with respect to other services, etc.) with the exception that the duration of the request may be less than 12 months in duration  and must reflect a service increment or multiple thereof for Firm service under the Transmission Provider’s approved tariff (e.g., days, weeks).

001-9.7.5	Any request to convey rollover rights on a Redirect that is less than 12 months in duration shall not be allowed to subsequently waive the conveyance of rollover rights; however, the Transmission Customer may withdraw such Redirect request.

001-9.7.6	In the event the total of all requests for rollover rights (Redirect and Renewal) exceeds the Capacity Eligible for Rollover associated with a Parent Reservation, the Transmission Provider shall act on  all pending requests to exercise rollover rights (Renewal requests) on the Parent Reservation prior to acting on requests to Redirect with rollover rights which were queued after such requests for renewal.

001-9.7.7	In the event the total of all requests for rollover rights (Redirect and Renewal) exceeds the Capacity Eligible for Rollover associated with a Parent Reservation, the Transmission Provider shall act on  all pending requests to Redirect with rollover rights prior to acting on requests to exercise rollover rights (renewal requests) on the Parent Reservation which were queued after such requests for Redirect.

001-9.7.8	The Transmission Provider shall evaluate the request for Redirect with rollover rights as eligible for on-going rollover rights in the amount of the lower of 1) the MW capacity requested on the Redirect with rollover rights, or 2) the Capacity Eligible for Rollover on the Parent Reservation.

001-9.7.8.1 	In the event the evaluation of the request for Redirect with rollover rights is delayed due to the evaluation of 			Renewal of the Parent Reservation consistent with 001-9.7.6, the Capacity Eligible for Rollover on the Parent 			Reservation  used in the evaluation of such Redirect shall be adjusted to reflect any confirmed Renewal(s).

001-9.7.8.2 	The Transmission Customer shall be allowed to submit and have pending multiple requests for Redirect with rollover rights against the same Capacity Eligible for Rollover. The Transmission Provider shall evaluate each such request with the knowledge that only those requests up to the Capacity Eligible for Rollover may ultimately be confirmed.

001-9.7.8.2.1	The Transmission Customer shall not confirm any request for Redirect with rollover rights that exceeds the Capacity Eligible for Rollover. The Transmission Provider shall have the right to block or nullify any such confirmation.

001-9.7.8.2.2 	The Transmission Customer should withdraw any request for Redirect with rollover rights that would exceed the current Capacity Eligible for Rollover. The Transmission Provider shall have the right to withdraw their acceptance of any such request that cannot be confirmed.

001-9.7.9	The deadline for submitting notice to exercise any on-going rollover rights that may be conveyed to the Redirect with rollover rights shall be identical to the notice deadline applicable to the Parent Reservation.

001-9.7.9.1	The Transmission Customer shall be allowed to submit a notice to exercise on-going rollover rights (renewal request) on the redirected path while the request for Redirect with rollover rights is still pending final action by the Transmission Provider in order to comply with the requirement to supply such notice prior to the applicable deadline.

001-9.7.10	The Transmission Provider shall specify any limitations or restrictions to the on-going rollover rights applicable on the redirected path as part of the information provided to the Transmission Customer on or before the request for Redirect with rollover rights is accepted.

001-9.7.11	Upon acceptance by the Transmission Provider and confirmation by the Transmission Customer, the conveyance of rollover rights to the Redirect on a Firm basis shall be as follows:

001-9.7.11.1	The Capacity Eligible for Rollover on the redirected path shall be set to the lower of 1) the MW capacity granted on the Redirect with rollover rights, or 2) the Capacity Eligible for Rollover on the Parent Reservation.

001-9.7.11.2	The initial value for the Unexercised Rollover Rights held on the redirected path shall be the lower of 1) the Capacity Eligible for Rollover on the redirected path, or 2) any restriction or limitation to the on-going rollover rights on the redirected path as established by the Transmission Provider.

001-9.7.11.3	The Capacity Eligible for Rollover on the Parent Reservation shall be reduced by the amount of the Capacity Eligible for Rollover conveyed to the redirected path.

001-9.7.11.4	The Unexercised Rollover Rights held on the Parent Reservation shall be adjusted to reflect the lower of 1) the remaining Capacity Eligible for Rollover on the Parent Reservation, or 2) any restriction or limitation to the on-going rollover rights on the parent path as established by the Transmission Provider in the original service agreement.

- Changes to WEQ-001-20

001-20.2.4	Upon confirmation of a Long-Term Firm Point-to-Point renewal request by the Transmission Customer exercising their rollover rights, the Transmission Provider shall reduce the rollover capacity (i.e., the Unexercised Rollover Rights) as viewed in the parent reservation’s rollover template by the capacity granted to the renewal reservation. Capacity Eligible for Rollover by the amount granted to the renewal request, and adjust the remaining Unexercised Rollover Rights held on the Parent Reservation accordingly.  The remaining Unexercised Rollover Rights shall be posted in accordance with WEQ-002 and WEQ-013.

001-20.3 	Once the deadline for the Transmission Customer to submit a renewal request has passed for a Long-Term Firm Point-to-Point reservation that is eligible for rollover rights and there are no outstanding pending renewal requests or any outstanding pending renewal requests on Firm Redirects of the Long-Term Firm Point-to-Point reservation, the Transmission Provider shall set the Capacity Eligible for Rollover and the Unexercised Rollover Rights rollover capacity associated with that reservation (i.e., the Unexercised Rollover Rights)to zero.

- Changes to WEQ-001-B

- 001-B 	APPENDIX B – REDIRECT STANDARDS EXAMPLES

- Notes for Examples 1-7: In all cases a Parent Reservation to be Redirected must be Firm Point-to-Point. Unless noted otherwise, example Redirect requests may be on either a firm or non-firm basis. Capacity that is shaded (dotted) shows Parent Capacity Available to Redirect. Capacity in Gray is not available to Redirect.  Requests that are cross-hatched are outside the bounds of the Parent Reservation.

- Additions to WEQ-001-B to be included after Example 7:

- Notes for Examples 8-17: These examples relate specifically to Standards WEQ-001-9.7 for conveyance of rollover rights with a Redirect on a Firm basis.  These Redirect requests must be in compliance with all previous examples 1-7 in addition to these additional restrictions.

- The transmission service attributes in each of these examples is assumed to be a set of valid attributes defined by the Transmission Provider for Long-Term Firm Point-to-Point Transmission Service, e.g. YEARLY, FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED.

- The examples  further assume the Transmission Provider, through their Business Practices, has not specified a 	minimum term for firm redirect requests with conveyance of rollover rights. Consistent with WEQ-001-9.7.4, the 	Transmission Provider may place such a minimum term requirement on a redirect request with conveyance of rollover 	rights.

- Example 8: Rollover Rights Conveyed to Firm Redirect

- Transmission Customer holds Long-Term Point-to-Point service (TSR1) on path “A-B” (Parent) with deadline to exercise rollover rights at 60 days prior to end of service.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status |  | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED |  | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |

- Transmission Customer submits a Redirect on a Firm basis (TSR2) in accordance with WEQ-001-9.7:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 9/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Redirect with conveyance of rollover rights |

- Redirect on a Firm basis (TSR2) must be for a valid Long-Tem Firm service, e.g., “YEARLY FIRM POINT_TO_POINT”, but, per WEQ-001-9.7.4, must not be invalidated due to duration even though its term of service of only 4 months may not meet a Transmission Provider’s requirement that an original request for this service be submitted for a minimum term of 12 months.

- The Transmission Provider evaluates and accepts the Redirect on a Firm basis (TSR2) request with no limitations on on-going rollover rights:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - ACCEPTED | - Y/F/P/F/F | - 9/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Redirect with conveyance of rollover rights |

- The  Transmission Customer confirms the Redirect:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 0 | - A-B | - 0 | - 0 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - CONFIRMED | - Y/F/P/F/F | - 9/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Redirect with conveyance of rollover rights |

- Unexercised Rollover Rights held on the original parent request are conveyed to the confirmed Redirect on a Firm basis per WEQ-001-9.7.11.

- Example 9:  Rollover Rights Renewed on Original Path are not Eligible to be  Conveyed to Firm Redirect

- Transmission Customer holds Long-Term Point-to-Point service (TSR1) on path “A-B” (Parent) with deadline to exercise rollover rights at 60 days prior to end of service.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |

- Transmission Customer submits a Renewal (TSR2) of the Parent:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - A-B | - 100 | - 100 | - Renewal of service for new term |

- The Transmission Provider accepts the Renewal (TSR2) of the Parent:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - ACCEPTED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - A-B | - 100 | - 100 | - Renewal of service for new term |

- Transmission Customer confirms the Renewal (TSR2) of the Parent:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 0 | - 0 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - A-B | - 100 | - 100 | - Renewal of service for new term |

- Capacity Eligible for Rollover and the Unexercised Rollover Rights on the Parent Reservation are reduced to 0 per WEQ-001-20.3.

- Transmission Customer then submits a request to Redirect on a Firm basis with conveyance of rollover rights from Parent Reservation (TSR1) to the Redirect request (TSR3) for the balance of the term of the Parent Reservation (TSR1):

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 0 | - 0 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - A-B | - 100 | - 100 | - Renewal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 9/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Attempt to convey rollover rights on Redirect |

- The Redirect (TSR3) will not be considered eligible for the conveyance of rollover rights per WEQ-001-9.7.7 and shall be deemed INVALID because Capacity Eligible for Rollover and the Unexercised Rollover Rights on the Parent Reservation (TSR1) have been reduced to 0.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 0 | - 0 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - A-B | - 100 | - 100 | - Renewal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - INVALID | - Y/F/P/F/F | - 9/1/2008 | - 1/1/2009 | - N/A | - C-D | - N/A | - N/A | - Attempt to convey rollover rights on Redirect |

- If the Transmission Customer Redirects  the Parent Renewal (TSR2) to path C-D, they must submit a Redirect on a Firm basis in compliance with WEQ-001-9.7 against Renewal (TSR2). If the Transmission Customer requests rollover rights (beyond 1/1/2014) on Path C-D,  they should indicate the request for rollover rights when submitting the Redirect of TSR2.  The Transmission Customer may alternatively request a Redirect on a Firm basis for the balance of the term of the Parent reservation (TSR1) but without a request for the conveyance of rollover rights. This should be submitted as a Short-Term Redirect.

- Example 10: Partial Rollover Rights Conveyed to Partial Firm Redirect Because of Confirmed Partial Capacity Renewal

- Transmission Customer holds Long-Term Point-to-Point service (TSR1) on path “A-B” (Parent) with deadline to exercise rollover rights at 60 days prior to end of service.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |

- Transmission Customer submits a Renewal (TSR2) for a portion of the Parent capacity four months prior to end of service:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 40 | - A-B | - 40 | - 40 | - Renewal of service for new term |

- The Transmission Provider accepts the Renewal (TSR2):

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - ACCEPTED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 40 | - A-B | - 40 | - 40 | - Renewal of service for new term |

- Transmission Customer confirms the Renewal (TSR2) for a portion of the Parent capacity:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 60 | - 60 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 40 | - A-B | - 40 | - 40 | - Renewal of service for new term |

- The Capacity Eligible for Rollover and the Unexercised Rollover Rights on the Parent are reduced to 60 per WEQ-001-20.2.4, and the Parent Renewal is for 40 MWs.

- Transmission Customer submits a request to Redirect the Parent capacity (60 MW) with conveyance of rollover rights from Parent (TSR1) to Redirect (TSR3) for the balance of the term of Parent (TSR1) three months prior to end of the Parent:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 60 | - 60 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 40 | - A-B | - 40 | - 40 | - Renewal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 10/1/2008 | - 1/1/2009 | - 60 | - C-D | - 60 | - 60 | - Rollover rights conveyed to Redirect |

- The Redirect (TSR3) is evaluated and accepted by the Transmission Provider:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 60 | - 60 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 40 | - A-B | - 40 | - 40 | - Renewal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - ACCEPTED | - Y/F/P/F/F | - 10/1/2008 | - 1/1/2009 | - 60 | - C-D | - 60 | - 60 | - Rollover rights conveyed to Redirect |

- Transmission Customer confirms the Redirect:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 40 | - A-B | - 0 | - 0 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 40 | - A-B | - 40 | - 40 | - Renewal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - CONFIRMED | - Y/F/P/F/F | - 10/1/2008 | - 1/1/2009 | - 60 | - C-D | - 60 | - 60 | - Rollover rights conveyed to Redirect |

- The Transmission Customer may alternatively request a Redirect on a Firm basis for the balance of the term of the Parent Reservation (TSR1) but without a request for the conveyance of rollover rights. This should be submitted as a Short-Term Redirect.

- Example 11: Partial Rollover Rights Conveyed to Partial Firm Redirect Because of Confirmed Partial Capacity Renewal on the Parent

- Transmission Customer holds Long-Term Point-to-Point service (TSR1) on path “A-B” (Parent) with deadline to exercise rollover rights at 60 days prior to end of service.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |

- Transmission Customer submits a Renewal (TSR2) for a portion of the Parent capacity four months prior to end  of service:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 40 | - A-B | - 40 | - 40 | - Renewal of service for new term |

- The Transmission Provider accepts the Renewal (TSR2):

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - ACCEPTED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 40 | - A-B | - 40 | - 40 | - Renewal of service for new term |

- Transmission Customer confirms the Renewal (TSR2)for a portion of the Parent capacity:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 60 | - 60 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 40 | - A-B | - 40 | - 40 | - Renewal of service for new term |

- The Capacity Eligible for Rollover and the Unexercised Rollover Rights on the Parent are reduced to 60 per WEQ-001-20.2.4, and the Parent Renewal is for 40 MWs.

- Transmission Customer submits a request to Redirect the Parent capacity (100 MW) with conveyance of rollover rights from Parent (TSR1) to Redirect (TSR3) for the balance of the term of Parent (TSR1) three months prior to end of the Parent:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 60 | - 60 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 40 | - A-B | - 40 | - 40 | - Renewal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 10/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Rollover rights conveyed to Redirect |

- The Redirect (TSR3) is evaluated by the Transmission Provider and deemed to be invalid because the request for Redirect (100 MW) with Rollover (also 100 MW) was greater than the Capacity Eligible for Rollover needed for the request:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 60 | - 60 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 40 | - A-B | - 40 | - 40 | - Renewal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - INVALID | - Y/F/P/F/F | - 10/1/2008 | - 1/1/2009 | - N/A | - C-D | - N/A | - N/A | - Rollover rights conveyed to Redirect |

- Example 12: Firm Redirect Queued too Late to be Conveyed Rollover Rights

- Transmission Customer holds Long-Term Point-to-Point service (TSR1) on path “A-B” (Parent) with deadline to exercise rollover rights at 60 days prior to end of service.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |

- Transmission Customer submits a Redirect (TSR2) in comformance with WEQ-001-9.7, 75 days prior to the end of Parent (TSR1):

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 11/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Rollover rights conveyed to Redirect |

- Regardless of when the Transmission Provider completes assessment of the Redirect request, if the Transmission Customer has not submitted a Renewal request on the Parent and/or the Redirect request prior to the 60 day notice deadline,  no rollover rights are available once the deadline has passed.   The Transmission Customer did not submit a timely Renewal request on the Parent, therefore,  the Capacity Eligible for Rollover and the Unexercised Rollover Rights are reduced to zero by the Transmission Provider.  The Redirect request should not have been a request for long term service, because no rollover is available.  Therefore, the Redirect request is deemed INVALID:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 0 | - 0 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - INVALID | - Y/F/P/F/F | - 11/1/2008 | - 1/1/2009 | - N/A | - C-D | - N/A | - N/A | - Rollover rights conveyed to Redirect |

- Unexercised Rollover Rights on the Parent are forfeited by failure to submit a timely renewal request per WEQ-001-20.3.  All renewal requests, whether for the Parent or related Redirect of Parent’s renewal, or renewal of Redirects, must be submitted prior to the notice deadline for renewal requests. (WEQ-001-9.7.8 and 9.7.8.1).

- Example 13: Renewal Requests Prior to Confirmation of Firm Redirect – Rollover Rights Conveyed to Firm Redirect

- Transmission Customer holds Long-Term Point-to-Point service (TSR1) on path “A-B” (Parent) with deadline to exercise rollover rights at 60 days prior to end of service.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |

- Transmission Customer submits a Redirect (TSR2) in comformance with WEQ-001-9.7 at 75 days prior to the end of Parent:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 11/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Redirect with conveyance of rollover rights |

- At 61 days prior to end of Parent service, the Transmission Customer submits a Renewal request (TSR3) on the Redirect (TSR2).  Since the Redirect has not yet been acted on by the Transmission Provider, the Transmission Customer also submits a Renewal request (TSR4) on the Parent to hedge against the possibility that ongoing rollover rights may be limited or unavailable on the Redirect:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 11/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Redirect with conveyance of rollover rights |
| - TSR3 | - TSR2 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - C-D | - 100 | - 100 | - Renewal of service for new term |
| - TSR4 | - TSR1 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - A-B | - 100 | - 100 | - Renewal of service for new term |

- The Transmission Provider provides information on any rollover limitations to the Transmission Customer off-line.  The information will clarify the available options for the allocation of the 100 MW of Unexercised Rollover Rights between the Parent and Redirect Path.  The example below assumes that the Transmission Customer accepts the Redirect request but limits rollover rights on the Redirect to 75 MWs.  The Transmission Provider will not act on either Renewal request until disposition of the Redirect is determined per WEQ-001-9.7.6.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - ACCEPTED | - Y/F/P/F/F | - 11/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 75 | - Redirect with conveyance of rollover rights |
| - TSR3 | - TSR2 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - C-D | - 100 | - 100 | - Renewal of service for new term |
| - TSR4 | - TSR1 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - A-B | - 100 | - 100 | - Renewal of service for new term |

- The Transmission Customer confirms the Redirect with rollover limitations. Once the Transmission Customer confirms the Redirect, the Capacity Eligible for Rollover held on Parent (TSR1) is conveyed to Redirect (TSR2) and the Unexercised Rollover Rights held on the Parent are reduced to 0. Since the Unexercised Rollover Rights on the Redirect are limited by the Transmission Provider, the renewal request (TSR3) on the Redirect must be limited to that amount. The Transmission Provider will also act on renewal of the Parent based on the Unexercised Rollover Rights remaining on the Parent have been reduced to 0, per WEQ-001-20.2.1

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 0 | - A-B | - 0 | - 0 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - CONFIRMED | - Y/F/P/F/F | - 11/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 75 | - Redirect with conveyance of rollover rights |
| - TSR3 | - TSR2 | - RENEWAL | - N/A | - COUNTEROFFER | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 75 | - C-D | - 75 | - 75 | - Renewal of service for new term |
| - TSR4 | - TSR1 | - RENEWAL | - N/A | - INVALID | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - N/A | - A-B | - N/A | - N/A | - Renewal of service for new term |

- Transmission Customer confirms the Renewal (TSR3) on the Redirect:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Roll Over Flag | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - Y | - 1/1/2000 | - 1/1/2009 | - 0 | - A-B | - 0 | - 0 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - CONFIRMED | - Y/F/P/F/F | - Y | - 11/1/2008 | - 1/1/2009 | - 100 | - C-D | - 0 | - 0 | - Redirect with conveyance of rollover rights |
| - TSR3 | - TSR2 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - Y | - 1/1/2009 | - 1/1/2014 | - 75 | - C-D | - 75 | - 75 | - Renewal of service for new term |
| - TSR4 | - TSR1 | - RENEWAL | - N/A | - INVALID | - Y/F/P/F/F | - Y | - 1/1/2009 | - 1/1/2014 | - N/A | - A-B | - N/A | - N/A | - Renewal of service for new term |

- Example 14: Renewal of Firm Redirect Prior to Confirmation of Firm Redirect – Rollover Rights Retained on Parent Path

- Transmission Customer holds Long-Term Point-to-Point service (TSR1) on path “A-B” (Parent) with deadline to exercise rollover rights at 60 days prior to end of Parent.

| - OASIS Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |

- Transmission Customer submits a Redirect (TSR2) in comformance with WEQ-001-9.7, 75 days prior to end of Parent:

| - OASIS Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 11/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Redirect with conveyance of rollover rights |

- At 61 days prior to end of Parent, the Transmission Customer submits a Renewal request (TSR3) to exercise rollover rights on the Redirect (TSR2).  Since the Redirect has not yet been accepted by the Transmission Provider, the Transmission Customer submits a Renewal request (TSR4) on the Parent (TSR1) to hedge against the possibility that rollover rights may be limited or unavailable on the Redirect:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 11/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Redirect with conveyance of rollover rights |
| - TSR3 | - TSR2 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - C-D | - 100 | - 100 | - Renewal of service for new term |
| - TSR4 | - TSR1 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - A-B | - 100 | - 100 | - Renewal of service for new term |

- The Transmission Provider provides information on any rollover limitations to the Transmission Customer off-line.  The  information will clarify the available options for the allocation of the 100 MW of Unexercised Rollover Rights between the Parent and Redirect Path.  The example below assumes that the Transmission Customer withdraws the Redirect request with a limitation to the ongoing rollover rights of 75 MWs.  The Transmission Provider will not act on either Renewal request until the disposition of the Redirect is determined per WEQ-001-9.7.6.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - COUNTEROFFER | - Y/F/P/F/F | - 11/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 75 | - Redirect with conveyance of rollover rights |
| - TSR3 | - TSR2 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - C-D | - 100 | - 100 | - Renewal of service for new term |
| - TSR4 | - TSR1 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - A-B | - 100 | - 100 | - Renewal of service for new term |

- The Transmission Customer withdraws the Redirect (TSR2). Once the Transmission Customer withdraws the Redirect,Renewal of TSR2 is now invalid, and the Transmission Provider is free to act on Renewal of the Parent:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - WITHDRAWN | - Y/F/P/F/F | - 11/1/2008 | - 1/1/2009 | - N/A | - C-D | - N/A | - N/A | - Redirect with conveyance of rollover rights |
| - TSR3 | - TSR2 | - RENEWAL | - N/A | - INVALID | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - N/A | - C-D | - N/A | - N/A | - Renewal of service for new term |
| - TSR4 | - TSR1 | - RENEWAL | - N/A | - ACCEPTED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - A-B | - 100 | - 100 | - Renewal of service for new term |

- Customer confirms the Renewal of the Parent:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 0 | - 0 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - WITHDRAWN | - Y/F/P/F/F | - 11/1/2008 | - 1/1/2009 | - N/A | - C-D | - N/A | - N/A | - Redirect with conveyance of rollover rights |
| - TSR3 | - TSR2 | - RENEWAL | - N/A | - INVALID | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - N/A | - C-D | - N/A | - N/A | - Renewal of service for new term |
| - TSR4 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - A-B | - 100 | - 100 | - Renewal of service for new term |

- Example 15: Renewal of Firm Redirect Prior to Confirmation of Firm Redirect – Rollover Rights Conveyed to Firm Redirect

- Transmission Customer holds Long-Term Point-to-Point service (TSR1) on path “A-B” (Parent) with deadline to exercise rollover rights at 60 days prior to end of service. Rollover rights on path “A-B” are limited to 70 MWs per the original service agreement.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 70 | - Parent reservation |

- Transmission Customer submits a Renewal request (TSR2) for a portion of  rollover rights on the Parent (50 MWs) and then submits a Redirect (TSR3)  in comformance with WEQ-001-9.7 at 85 days prior to the end of Parent:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 70 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 50 | - A-B | - 50 | - 50 | - Renewal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 10/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Rollover rights conveyed to Redirect |

- The Transmission Provider accepts the Renewal: The Transmission Provider must delay evaluation of the Redirect (TSR3) until the disposition of the pending Renewal (TSR2) is determined per WEQ-001-9.7.6.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 70 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - ACCEPTED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 50 | - A-B | - 50 | - 50 | - Renewal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 10/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Rollover rights conveyed to Redirect |

- Transmission Customer confirms Renewal and the Capacity Eligible for Rollover and Unexercised Rollover Rights on the Parent are updated, per WEQ-001-20.2.4:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 50 | - 20 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 50 | - A-B | - 50 | - 50 | - Renewal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 10/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Rollover rights conveyed to Redirect |

- Note that the Unexercised Rollover Rights held on path A-B is 20 MWs reflecting the original Transmission Provider limitation of 70 MWs less the amount of rollover rights exercised on path A-B of 50 MWs.  The Capacity Eligible for Rollover, however, is 50 MWs reflecting the original contracted capacity (100 MWs) less the amount of rollover rights exercised on path A-B of 50 MWs.  The Transmission Provider must evaluate the Redirect request based on the updated Capacity Eligible for Redirect (50 MWs) on the parent reservation per WEQ-001-9.7.8.

- At 61 days prior to end of Parent, the Transmission Customer submits a Renewal request to ensure they meet the notice requirements for exercising rollover rights on the Redirect path.  Since the Redirect has not yet been acted on by the Transmission Provider, the Transmission Customer also submits a renewal request on the Parent for the remaining rollover rights on the Parent to hedge against the possibility that ongoing rollover rights may be limited or unavailable on the Redirect:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 50 | - 20 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 50 | - A-B | - 50 | - 50 | - Renwal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 10/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Rollover rights conveyed to Redirect |
| - TSR4 | - TSR3 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - C-D | - 100 | - 100 | - Renewal of service for new term |
| - TSR5 | - TSR1 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 20 | - A-B | - 20 | - 20 | - Renewal of service for new term |

- The Transmission Provider  provides information on any rollover limitations to Transmission Customer off-line.  The information will clarify the available options for the allocation of the 100 MW of Unexercised Rollover Rights between the Parent and Redirect Path.  The example below assumes that the Transmission Customer accepts the Redirect with no limitation to the ongoing rollover rights of 50 MWs.  The Transmission Provider will not act on either Renewal request until the disposition of the Redirect is determined per WEQ-001-9.7.6.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 50 | - 20 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 50 | - A-B | - 50 | - 50 | - Renwal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - COUNTEROFFER | - Y/F/P/F/F | - 10/1/2008 | - 1/1/2009 | - 100 | - C-D | - 50 | - 50 | - Rollover rights conveyed to Redirect |
| - TSR4 | - TSR3 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 100 | - C-D | - 100 | - 100 | - Renewal of service for new term |
| - TSR5 | - TSR1 | - RENEWAL | - N/A | - QUEUED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 20 | - A-B | - 20 | - 20 | - Renewal of service for new term |

- The Transmission Customer elects to confirm the Redirect (TSR3). Once the Transmission Customer confirms the Redirect, the Transmission Provider is free to act on the Renewal of the Redirect.  Since the rollover rights conveyed to the Redirect are limited to 50 MWs, the Redirect Renewal must be limited to that amount.  The Renewal of the Parent is now invalid as all remaining Unexercised Rollover Rights on path A-B were conveyed to path C-D:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 50 | - A-B | - 0 | - 0 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 50 | - A-B | - 50 | - 50 | - Renwal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - CONFIRMED | - Y/F/P/F/F | - 10/1/2008 | - 1/1/2009 | - 100 | - C-D | - 50 | - 50 | - Rollover rights conveyed to Redirect |
| - TSR4 | - TSR3 | - RENEWAL | - N/A | - COUNTEROFFER | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 50 | - C-D | - 50 | - 50 | - Renewal of service for new term |
| - TSR5 | - TSR1 | - RENEWAL | - N/A | - INVALID | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - N/A | - A-B | - N/A | - N/A | - Renewal of service for new term |

- Transmission Customer confirms the Renewal:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 0 | - A-B | - 0 | - 0 | - Parent reservation |
| - TSR2 | - TSR1 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 50 | - A-B | - 50 | - 50 | - Renwal of service for new term |
| - TSR3 | - TSR1 | - REDIRECT | - No | - CONFIRMED | - Y/F/P/F/F | - 10/1/2008 | - 1/1/2009 | - 100 | - C-D | - 0 | - 0 | - Rollover rights conveyed to Redirect |
| - TSR4 | - TSR3 | - RENEWAL | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 50 | - C-D | - 50 | - 50 | - Renewal of service for new term |
| - TSR5 | - TSR1 | - RENEWAL | - N/A | - INVALID | - Y/F/P/F/F | - 1/1/2009 | - 1/1/2014 | - 20 | - A-B | - 20 | - 20 | - Renewal of service for new term |

- Example 16: Withdrawal of Request for Redirect with Rollover Due to Limited Rollover

- Transmission Customer holds Long-Term Point-to-Point service (TSR1) on path “A-B” (Parent) with deadline to exercise rollover rights at 60 days prior to end of service.  Transmission Customer submits a Redirect on a Firm basis in accordance with WEQ-001-9.7:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |

- Transmission Customer submits a Redirect (TSR2) in comformance with WEQ-001-9.7, 75 days prior to end of Parent:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 9/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Redirect with conveyance of rollover rights |

- The Transmission Provider provides information on any rollover limitations to the Transmission Customer off-line.  The  information will clarify the available options for the allocation of the 100 MW of Unexercised Rollover Rights between the Parent and Redirect Path.  The example below assumes that the Transmission Provider and Transmission Customer agree to a COUNTEROFFER of 75 MW.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - COUNTEROFFER | - Y/F/P/F/F | - 9/1/2008 | - 1/1/2009 | - 75 | - C-D | - 75 | - 75 | - Redirect with conveyance of rollover rights |

- The Transmission Customer assesses the service offered on the redirect path and elects to withdraw the request and retain the rollover rights on the parent path rather than have rights restricted.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - WITHDRAWN | - Y/F/P/F/F | - 9/1/2008 | - 1/1/2009 | - 75 | - C-D | - 75 | - 75 | - Redirect with conveyance of rollover rights |

- The Transmission Customer may alternatively request a Redirect on a Firm Basis for the balance of the term of the Parent reservation (TSR1) but without a request for the conveyance of rollover rights. This should be submitted as a Short-Term Redirect.

- Example 17: Rebid of Firm Redirect after Restrictions Placed on Rollover Rights

- Transmission Customer holds Long-Term Point-to-Point service (TSR1) on path “A-B” (Parent) with deadline to exercise rollover rights at 60 days prior to end of service.  Transmission Customer submits a Redirect on a Firm Basis in accordance with WEQ-001-9.7:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop TIme | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |

- Transmission Customer submits a Redirect (TSR2) in comformance with WEQ-001-9.7, 75 days prior to end of Parent:

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - NA | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - QUEUED | - Y/F/P/F/F | - 9/1/2008 | - 1/1/2009 | - 100 | - C-D | - 100 | - 100 | - Redirect with conveyance of rollover rights |

- The Transmission Provider provides information on any rollover limitations to the Transmission Customer off-line.  The information will clarify the available options for the allocation of the 100 MW of Unexercised Rollover Rights between the Parent and Redirect Path.  The Transmission Provider and Transmission Customer agree to a COUNTEROFFER of 75 MW.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - COUNTEROFFER | - Y/F/P/F/F | - 9/1/2008 | - 1/1/2009 | - 75 | - C-D | - 75 | - 75 | - Redirect with conveyance of rollover rights |

- The Transmission Customer assesses the service offered on the redirect path and elects to reserve only 50 MW and associated rollover rights on the redirected path.  The following rebid is submitted by the customer:The Transmission Provider

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - REBID | - Y/F/P/F/F | - 9/1/2008 | - 1/1/2009 | - 50 | - C-D | - 75 | - 75 | - Redirect with conveyance of rollover rights |

- The rebid is on the “Capacity” (capacity at end of term) with the understanding that the Transmission Provider will make corresponding reductions to the Capacity Eligible for Rollover and Unexercised Rollover Rights.

- The Transmission Provider  adjusts the Capacity Eligible for Rollover and the Unexercised Rollover Rights to match the Capacity Available at end of term and accepts the Rebid.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 100 | - A-B | - 100 | - 100 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - ACCEPTED | - Y/F/P/F/F | - 9/1/2008 | - 1/1/2009 | - 50 | - C-D | - 50 | - 50 | - Redirect with conveyance of rollover rights |

- The Transmission Customer Confirms TSR2.

| - Assignment Reference | - Related Reference | - Request Type | - Rollover Waived | - Status | - Service Attributes | - Start Time | - Stop Time | - Capacity Availiable at End of Term | - Path Name | - Capacity Eligible for Rollover | - Unexercised Rollover Rights | - Description of Transaction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - TSR1 | - NA | - NA | - N/A | - CONFIRMED | - Y/F/P/F/F | - 1/1/2000 | - 1/1/2009 | - 50 | - A-B | - 50 | - 50 | - Parent reservation |
| - TSR2 | - TSR1 | - REDIRECT | - No | - CONFIRMED | - Y/F/P/F/F | - 9/1/2008 | - 1/1/2009 | - 50 | - C-D | - 50 | - 50 | - Redirect with conveyance of rollover rights |

- Unexercised Rollover Rights remaining on the Parent are not conveyed to the confirmed Redirect on a Firm basis per WEQ-001-9.7.10.   The customer may still exercise a Renewal of the Parent for up to 50 MW if queued before the renewal deadline.

- Changes to WEQ-002

002-4.3.6.1	Customer Capacity Purchase Request (transrequest)

The Customer Capacity Purchase Request (Input) (transrequest) is used by the Customer to request the purchase of transmission services or request changes to previously submitted reservations for transmission services. The response simply acknowledges that the Customer's request was received by the OASIS Node. It does not imply that the Seller has received the request. Inputting values into the reference Data Elements is optional.

CUSTOMER_CODE and CUSTOMER_DUNS shall be determined from the registered connection used to input the request.

Supporting "profiles" of service, which request different capacities and/or price for different time periods within a single request, is at the discretion of the Primary Provider. Continuation records may be used to indicate requests for these service profiles; use of continuation records is only supported when using the CSV Format upload of Template data. Each segment of a profile is represented by the set of Data Elements START_TIME, STOP_TIME, CAPACITY_REQUESTED, and BID_PRICE which define the intervals in time over which the specified capacity and price values apply. The initial segment of a profile is defined by the START_TIME, STOP_TIME, CAPACITY_REQUESTED, and BID_PRICE Data Elements specified in the first or only record submitted; subsequent segments are specified in continuation records each containing the appropriate START_TIME, STOP_TIME, CAPACITY_REQUESTED, and BID_PRICE  values defining the segment.

For requesting transmission services which include multiple paths, the following fields may be specified using continuation records: PATH_NAME, POINT_OF_RECEIPT, and POINT_OF_DELIVERY. Supporting multiple paths or multiple POINT_OF_RECEIPT and POINT_OF DELIVERY is at the discretion of the Provider.

The earliest START_TIME and latest STOP_TIME indicate the overall requested period of service and cannot be modified by subsequent actions.

When a valid request is received at the OASIS Node, the TSIP assigns a unique ASSIGNMENT_REF value and queues the request with a time stamp. The STATUS for the request is QUEUED. The IMPACTED counter is initially set to 0. REQUEST_TYPE must be specified. If the SELLER is not the Primary Provider, REQUEST_TYPE must be one of RESALE, FULL_TRANSFER, or PART_TRANSFER.

If the new request is modifying an existing transmission reservation, the Data Elements REQUEST_TYPE and RELATED_REF must be entered. RELATED_REF contains the ASSIGNMENT_REF for the transmission reservation being modified, and REQUEST_TYPE must be one of MATCHING, REDIRECT, DEFERRAL, RENEWAL, RELINQUISH, or a Primary Provider registered value.

Specification of a value YES in the PRECONFIRMED field authorizes the TSIP to automatically change the STATUS field in the transstatus Template to CONFIRMED when that request is ACCEPTED by the Seller.

Specification of a value Y in the ROLLOVER_WAIVED field indicates to the Transmission Provider that the Transmission Customer is voluntarially waiving the conveyance of any potential on-going rollover (renewal) rights that may otherwise be available in association with the requested service.

Template: transrequest

- Input

CONTINUATION_FLAG

SELLER_CODE (Primary or Reseller)

SELLER_DUNS

PATH_NAME

POINT_OF_RECEIPT

POINT_OF_DELIVERY

SOURCE

SINK

CAPACITY_REQUESTED

SERVICE_INCREMENT

TS_CLASS

TS_TYPE

TS_PERIOD

TS_WINDOW

TS_SUBCLASS

STATUS_NOTIFICATION

START_TIME

STOP_TIME

BID_PRICE

PRECONFIRMED

ANC_SVC_LINK

POSTING_REF (Optionally set by Customer)

SALE_REF (Optionally set by Customer)

REQUEST_REF (Optionally set by Customer)

DEAL_REF (Optionally set by Customer)

CUSTOMER_COMMENTS

REQUEST_TYPE (Required for request changes)

RELATED_REF (Required for request changes)

ROLLOVER_WAIVED (Only applicable to Long-Term Firm PTP service requests)

- Response (acknowledgment)

RECORD_STATUS

CONTINUATION_FLAG

ASSIGNMENT_REF (assigned by TSIP)

SELLER_CODE

SELLER_DUNS

PATH_NAME

POINT_OF_RECEIPT

POINT_OF_DELIVERY

SOURCE

SINK

CAPACITY_REQUESTED

SERVICE_INCREMENT

TS_CLASS

TS_TYPE

TS_PERIOD

TS_WINDOW

TS_SUBCLASS

STATUS_NOTIFICATION

START_TIME

STOP_TIME

BID_PRICE

PRECONFIRMED

ANC_SVC_LINK

POSTING_REF

SALE_REF

REQUEST_REF

DEAL_REF

CUSTOMER_COMMENTS

REQUEST_TYPE

RELATED_REF

ROLLOVER_WAIVED

ERROR_MESSAGE

002-4.3.6.2	Status of Customer Purchase Request/Reservation (transstatus)

The Status of Customer Purchase Request/Reservation (transstatus) is provided upon the request of any Customer or Provider to indicate the current status of one or more reservation records. Users may also view any transaction's status. However, the SOURCE and SINK may be masked for User requests until Transmission Providers must make source and sink information available at the time the request status posting is updated to show that a transmission request is confirmed.

Continuation records may be returned in association with a transmission reservation to convey information regarding: 1) sale or assignment of transmission rights on the secondary market (reassignments), 2) profiled requests, or 3) service over multiple paths.  Each continuation record associated with a transmission reservation shall be identified by the CONTINUATION_FLAG Data Element set to 'Y' and include the ASSIGNMENT_REF Data Element.

When a transmission reservation request acquires its rights to transmission service as the result of a sale or assignment of transmission rights on the secondary market, the identity of the original reservation, capacity, and time interval over which rights are assigned to the new reservation are defined by the Data Elements REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME, and REASSIGNED_STOP_TIME. These Data Elements will be returned in continuation records when more than one set of reassignment information is associated with a reservation.

If the transmission reservation has an associated profile, either as a result of the Transmission Customer’s submission of CAPACITY_REQUESTED and/or BID_PRICE varying over time (support for Customer reservation profiles is at the discretion of the Provider or the Seller’s offering of partial service or negotiation of price, the set of data elements for START_TIME, STOP_TIME, CAPACITY_REQUESTED, BID_PRICE, CAPACITY_GRANTED, OFFER_PRICE and CEILING_PRICE will be returned in continuation records for each segment of the request/reservation profile.

If the Provider supports reservations submitted on multiple paths, continuation records specifying PATH_NAME, POINT_OF_RECEIPT, and POINT_OF_DELIVERY associated with the reservation would be returned in continuation records.

The AFFILIATE_FLAG will be set by the TSIP to indicate whether or not the Customer is an affiliate of the Primary Provider. The NEGOTIATED_PRICE_FLAG will be set by the TSIP to indicate whether the OFFER_PRICE is higher, lower, or the same as the BID_PRICE. Any time that a confirmed transmission reservation's rights to schedule up to the amount of CAPACITY_GRANTED is reduced, either due to secondary market sales, partial displacements, Provider initiated "recalls" of capacity, etc., the IMPACTED Data Element shall be incremented. Specific information regarding the MW level and reason for reduction in reserved capacity is viewable using the reduction Template.

The PRIMARY_PROVIDER_APPROVAL data element is set by the Primary Provider to indicate their approval of PART_TRANSFER and FULL_TRANSFER requests.  In all other types of transmission requests, this element will be null.

The PRIMARY_PROVIDER_PROVISIONS data element is set to ‘Y’ by the Primary Provider when certain provisions are imposed on the reservation that may be viewed by issuing a query for the rollover and cco templates, otherwise null shall be used.

Template: transstatus

- Query

SELLER_CODE*

SELLER_DUNS*

CUSTOMER_CODE*

CUSTOMER_DUNS*

PATH_NAME*

POINT_OF_RECEIPT*

POINT_OF_DELIVERY*

SERVICE_INCREMENT*

TS_CLASS*

TS_TYPE*

TS_PERIOD*

TS_WINDOW*

TS_SUBCLASS*

STATUS*

START_TIME (Beginning time of service)

STOP_TIME

START_TIME_QUEUED (Beginning time queue)

STOP_TIME_QUEUED

NEGOTIATED_PRICE_FLAG

ASSIGNMENT_REF

REASSIGNED_REF

RELATED_REF

SALE_REF

REQUEST_REF

DEAL_REF

COMPETING_REQUEST_FLAG

TIME_OF_LAST_UPDATE

- Response

CONTINUATION_FLAG

ASSIGNMENT_REF

SELLER_CODE

SELLER_DUNS

CUSTOMER_CODE

CUSTOMER_DUNS

AFFILIATE_FLAG (Set by TSIP)

PATH_NAME

POINT_OF_RECEIPT

POINT_OF_DELIVERY

SOURCE

SINK

CAPACITY_REQUESTED

CAPACITY_GRANTED

SERVICE_INCREMENT

TS_CLASS

TS_TYPE

TS_PERIOD

TS_WINDOW

TS_SUBCLASS

NERC_CURTAILMENT_PRIORITY

OTHER_CURTAILMENT_PRIORITY

START_TIME

STOP_TIME

CEILING_PRICE

OFFER_PRICE

BID_PRICE

PRICE_UNITS

PRECONFIRMED

ANC_SVC_LINK

ANC_SVC_REQ

POSTING_REF

SALE_REF

REQUEST_REF

DEAL_REF

IMPACTED (Greater than 0, if another reservation impacts this reservation)

COMPETING_REQUEST_FLAG

REQUEST_TYPE

RELATED_REF

NEGOTIATED_PRICE_FLAG ("L" if Seller accepted Price is lower than OFFER_PRICE in transoffering Template; "H" if higher; otherwise blank)

STATUS

STATUS_NOTIFICATION

STATUS_COMMENTS

TIME_QUEUED

RESPONSE_TIME_LIMIT

TIME_OF_LAST_UPDATE

PRIMARY_PROVIDER_COMMENTS

SELLER_REF

SELLER_COMMENTS

CUSTOMER_COMMENTS

SELLER_NAME

SELLER_PHONE

SELLER_FAX

SELLER_EMAIL

CUSTOMER_NAME

CUSTOMER_PHONE

CUSTOMER_FAX

CUSTOMER_EMAIL

REASSIGNED_REF

REASSIGNED_CAPACITY (Capacity from each previous transaction)

REASSIGNED_START_TIME

REASSIGNED_STOP_TIME

PRIMARY_PROVIDER_APPROVAL

PRIMARY_PROVIDER_PROVISIONS

ROLLOVER_WAIVED

002-4.3.6.2.1 	Renewal Provisions (rollover)

The Renewal Provisions (rollover) template provides users with additional information related to the specific provisions for exercising the right to renewal/rollover the associated transmission service reservation.

The query parameters associated with this template may be specified by the user to limit the set of service reservations whose Renewal Provisions are to be returned in the template response.

The deadline for submitting a RENEWAL request to exercise rollover rights (if any) shall be captured in the RENEWAL_DUE_TIME data element. The maximum amount of reserved capacity that is considered eligible for on-going rollover rights shall be made available in the ROLLOVER_ELIGIBLE data element.

If known and applicable, duration and capacity limitations to renewal/rollover rights associated with a confirmed reservation are documented in one or more sets of ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY data elements returned as continuation records. Each continuation record associated with a given transmission reservation shall be identified by the CONTINUATION_FLAG Data Element set to 'Y' and include the ASSIGNMENT_REF Data Element.

The ROLLOVER_ELIGIBLE and ROLLOVER_CAPACITY over time data elements shall be updated to reflect changes in these related service attributes due to exercising rollover rights (renewals) or conveyance of those rollover rights to other service reservations as applicable.

If there is no additional renewal/rollover information associated with a reservation selected via the query parameters, there will be no record returned in the template response for that reservation.

The query parameters below are to be applied in the same way as to the associated reservation data elements as defined in the transstatus template to select the specific reservations whose renewal/rollover information is to be

returned.

Template: rollover

- Query

SELLER_CODE*

SELLER_DUNS*

CUSTOMER_CODE*

CUSTOMER_DUNS*

PATH_NAME*

POINT_OF_RECEIPT*

POINT_OF_DELIVERY*

SERVICE_INCREMENT*

TS_CLASS*

TS_TYPE*

TS_PERIOD*

TS_WINDOW*

TS_SUBCLASS*

START_TIME

STOP_TIME

START_TIME_QUEUED

STOP_TIME_QUEUED

ROLLOVER_START_TIME

ROLLOVER_STOP_TIME

ASSIGNMENT_REF

TIME_OF_LAST_UPDATE

- Response

CONTINUATION_FLAG

ASSIGNMENT_REF

RENEWAL_DUE_TIME

ROLLOVER_ELIGIBLE

ROLLOVER_START_TIME

ROLLOVER_STOP_TIME

ROLLOVER_CAPACITY

TIME_OF_LAST_UPDATE

- Additions to WEQ-003

| ROLLOVER_ELIGIBLE | ROLLELG | 0{NUMERIC}12 | Non-negative number in units of MW | The maximum amount of reserved capacity that is to be considered eligible for granting of on-going rollover rights. The actual amount available to be renewed over time will be the lower of ROLLOVER_ELIGIBLE and ROLLOVER_CAPACITY. |
| --- | --- | --- | --- | --- |
| ROLLOVER_WAIVED | ROLLWV | 0{ALPHA}1 | Valid Values: Y Null | Used by Transmission Customer to indicate their waiving (ROLLOVER_WAIVED=’Y’) of any consideration for on-going rollover rights that may be applicable to the service requested at the time of submittal. |

- Changes to WEQ-013

013-2.6.1	ORIGINAL Requests

ORIGINAL requests shall be submitted by the Transmission Customer to arrange for new transmission or ancillary service with the Primary Provider.

The following are specific restrictions or requirements for OASIS service requests with REQUEST_TYPE of ORIGINAL.  If REQUEST_TYPE is not specified by the Transmission Customer and SELLER_CODE and SELLER_DUNS are the same as PRIMARY_PROVIDER_CODE and PRIMARY_PROVIDER_DUNS, OASIS shall default REQUEST_TYPE to ORIGINAL.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be ORIGINAL |
| RELATED_REF | Must be null |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| ROLLOVER_WAIVED | If the Transmission Customer does not wish a Long-Term Firm PTP request to be evaluated for conveyance of rollover rights, the Transmission Customer must submit the request with the ROLLOVER_WAIVED data element set to ‘Y’ in the transrequest template. |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

Additional requirements related to the specification of service points, attributes, pricing, and timing are subject to the Primary Provider’s business practices.

The Transmission Customer may submit a time varying profile of capacity as allowed by the Primary Provider’s business practice by repeating the template data elements of BID_PRICE, CAPACITY_REQUESTED, START_TIME and STOP_TIME in template continuation records.  The segments of any submitted profile must not overlap in time.

Once submitted by the Customer the earliest START_TIME and latest STOP_TIME defines the overall term of service.  Any further negotiation or offer of partial service may modify the segments/profiles times, but any such modifications must span the entire period from the original earliest START_TIME to latest STOP_TIME.

- If the Transmission Customer does not wish a Long-Term Firm PTP request to be evaluated for conveyance of rollover rights, the Transmission Customer  must submit the request with the ROLLOVER_WAIVED data element set to ‘Y’ in the transrequest  template.

013-2.6.1.3 	Rollover Rights

For transmission services that have ongoing rollover or renewal rights, the Transmission Provider shall document those rights and any limitations over time through the rollover template data elements RENEWAL_DUE_TIME, ROLLOVER_ELIGIBLE, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY. The Transmission Provider must also flag that the service request/reservation has associated rollover rights by setting the transstatus template PRIMARY_PROVIDER_PROVISIONS data element to “Y”. The ROLLOVER_ELIGIBLE data element is equivalent to the Capacity Eligible for Rollover or conveyance of rollover rights to a qualified Redirect on a Firm Basis; the ROLLOVER_CAPACITY data element is equivalent to the remaining Unexercised Rollover Rights available over time.  Continuation records shall be used if the ROLLOVER_CAPACITY value changes as a function of time by listing each applicable time interval and capacity eligible for rollover/renewal in the ROLLOVER_START_TIME, ROLLOVER_STOP_TIME and ROLLOVER_CAPACITY data elements. The first value for ROLLOVER_START_TIME shall be set to the value of the latest STOP_TIME value associated with the request/reservation as shown in the transstatus template response, i.e., rollover template response records begin at the end of the transmission service requested/reserved. If there is no limitation on rollover rights, ROLLOVER_STOP_TIME shall be null and the ROLLOVER_CAPACITY shall indicate the capacity eligible for rollover for all future time.

On confirmation of any RENEWAL or REDIRECT requests eligible for conveyance of rollover rights, the values for the ROLLOVER_ELIGIBLE, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY data elements shall be updated appropriately and returned in response to the rollover template query.

013-2.6.2	RENEWAL Requests

Transmission Customers shall use the REQUEST_TYPE of RENEWAL only to exercise rollover rights. RENEWAL requests must always specify the Primary Provider as SELLER.

The following are specific restrictions or requirements for OASIS service requests with REQUEST_TYPE of RENEWAL.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be RENEWAL |
| RELATED_REF | Must specify the ASSIGNMENT_REF that is associated with an existing confirmed Firm transmission service reservation or pending Redirect on a Firm basis held by the Transmission Customer that 1) has rollover rights, and 2) that has not had the deadline for the exercise of rollover rights pass. |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| PATH | Must represent the same corresponding service points in the reservation/request specified in RELATED_REF |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must specify a set of valid transmission service attributes recognized by the Primary Provider as a valid service designation eligible for exercising of rollover rights held by the reservation/request specified in RELATED_REF |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must match the STOP_TIME of the reservation/request specified in RELATED_REF |
| STOP_TIME | With START_TIME, must specify a valid interval of service eligible for exercising of renewal/rollover rights held by the reservation/request specified in RELATED_REF |
| CAPACITY_REQUESTED | Must be less than or equal to the amount of capacity eligible for renewal/rollover over the interval of service |
| BID_PRICE | Must specify the price to be paid for the service requested |
| ROLLOVER_WAIVED | If the Transmission Customer does not wish a Long-Term Firm PTP request to be evaluated for conveyance of rollover rights, the Transmission Customer must submit the request with the ROLLOVER_WAIVED data element set to ‘Y’ in the transrequest template. |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

RENEWAL requests must be submitted on OASIS prior to the deadline for exercising rollover rights as established by the Tariff or business practice.

CUSTOMER_CODE and CUSTOMER_DUNS in the RENEWAL request should correspond to the CUSTOMER_CODE and CUSTOMER_DUNS in the RELATED_REF reservation.  If not, the Transmission Provider should verify that the submitting Customer has a valid agency agreement with the original transmission service agreement holder and is authorized to submit such a request on behalf of that entity.

The transmission service attributes, e.g., TS_CLASS, etc., should match the corresponding attributes in the reservation specified in RELATED_REF.  However, changes may be made to these attributes over time such that some differences are necessary to accommodate changes in the Primary Provider’s business practices.  This also applies to changes in service points, e.g., PATH, etc., over time.

RENEWAL requests may be subject to offering of partial service as a result of competition and/or failure to exercise a right of first refusal.

If the Transmission Customer does not wish a Long-Term Firm PTP request to be evaluated for conveyance of rollover rights, the Transmission Customer  must submit the request with the ROLLOVER_WAIVED data element set to ‘Y’ in the transrequest  template.

013-2.6.5.1	REDIRECT on a Firm Basis

A Transmission Customer holding confirmed firm point-to-point transmission rights may request the use of those rights on alternate points of receipt and/or delivery on a firm basis by submission of a REDIRECT request to the Primary Provider as Seller.  The following information must be submitted by the Transmission Customer in the REDIRECT request via the transrequest template.

| Data Element | Restriction/Requirement |
| --- | --- |
| REQUEST_TYPE | Must be REDIRECT |
| RELATED_REF | Must identify by ASSIGNMENT_REF a confirmed transmission service reservation for firm point-to-point service held by the submitting Transmission Customer |
| SELLER_CODE | Must match PRIMARY_PROVIDER_CODE |
| SELLER_DUNS | Must match PRIMARY_PROVIDER_DUNS |
| PATH | Must represent the new transmission service points being requested |
| POINT_OF_RECEIPT |  |
| POINT_OF_DELIVERY |  |
| SOURCE |  |
| SINK |  |
| SERVICE_INCREMENT | Must represent a set of valid transmission service attributes for firm point-to-point service offered by the Primary Provider and being requested on the new service points by the Transmission Customer |
| TS_CLASS |  |
| TS_TYPE |  |
| TS_PERIOD |  |
| TS_WINDOW |  |
| TS_SUBCLASS |  |
| START_TIME | Must specify the requested start of transmission service and must be within the bounds of START_TIME specified in the reservation identified in RELATED_REF |
| STOP_TIME | Must specify the requested stop/end of transmission service and must be within the bounds of STOP_TIME specified in the reservation identified in RELATED_REF, and with START_TIME must represent a valid interval of service for the firm point-to-point service being requested |
| CAPACITY_REQUESTED | Must specify the amount of transmission capacity being requested |
| BID_PRICE | Should specify the price for the service being requested; may be null |
| ROLLOVER_WAIVED | May specify the desire to waive conveyance of rollover to the REDIRECT by specifying ‘Y’ at time of submission of the REDIRECT request (retains rollover rights on the Parent reservation) |

Note: Elements are listed on basis of importance, which may be different from the order required in the template.

The Primary Provider shall evaluate each REDIRECT on a Firm basis as any other new request for firm point-to-point transmission service.  Primary Provider business practices establish the requirements for service duration, submission time, evaluation time, confirmation time, etc.

OASIS or Primary Provider procedures should verify that the transmission reservation identified in RELATED_REF meets all the requirements to support the redirect of transmission rights to the new service points.  This should include the validation that the current rights, the Capacity Available for Redirect, held on that reservation in the amount of the REDIRECT over time have not been encumbered by any other confirmed redirects, resales, schedules, CCO conditions, etc.  This capacity validation may occur at any point in the request process, but shall always be performed prior to setting the REDIRECT STATUS to CONFIRMED.

Once CONFIRMED, the transmission rights held on the RELATED_REF reservation in the amount of the REDIRECT shall be permanently released by the Primary Provider and conveyed to the REDIRECT reservation. The only mechanism for the Transmission Customer to return to the original points of receipt and/or delivery is to submit another REDIRECT request.

The impact on ATC for the reservation identified by RELATED_REF shall be released and the impact of the REDIRECT transaction on ATC shall be accounted for in the amount and over time of the redirect simultaneously.

The impact of the REDIRECT transaction on the reservation(s) identified by RELATED_REF shall be posted and viewable using the reduction  template.

OASIS or Primary Provider procedures shall also apply any outstanding conditions that may exist from the Parent CCO Reservation to the REDIRECT reservation, if applicable, (e.g., Number-of-Hours Criteria or System-Conditions Criteria) in accordance with WEQ 001-n.5.2.2.1 and WEQ 013-2.6.1.4. and any outstanding conditions shall be viewable using the cco template.

If the REDIRECT is eligible for the conveyance of rollover/renewal rights, these rights shall be communicated through the transstatus rollover template data elements RENEWAL_DUE_TIME, ROLLOVER_ELIGIBLE,  ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY.  Conveyance of rollover rights to the REDIRECT request/reservation may have an impact on those rights held on the RELATED_REF reservation.  These impacts on the RELATED_REF reservation shall be documented through an update to the rollover template data elements ROLLOVER_ELIGIBLE, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY data elements associated with the RELATED_REF.

If the Transmission Customer does not wish a Long-Term Firm PTP REDIRECT request to be evaluated for conveyance of rollover rights, the Transmission Customer  must submit the request with the ROLLOVER_WAIVED data element set to ‘Y’ in the transrequest  template.

- 4.  SUPPORTING DOCUMENTATION

- a.  Description of Request:

In the FERC Order 676, under Docket No. RM05-5-000 dated April 25, 2006, the Commission made the following conclusions:

51. Standard 001-9.7 does not specify clearly the parties’ responsibilities with respect to the ability of a customer requesting a firm redirect to obtain rollover rights on the redirect path.[1]  Under section 22.2 of the pro forma OATT, a request for a firm redirect is like a request for new transmission service.  The transmission provider, therefore, is required to offer rollover rights to a customer requesting a firm redirect if rollover rights are available on the redirect path.  However, the transmission provider may not operationally be able to offer rollover rights on the requested redirect path due to reasonably forecasted native load needs for the transmission capacity.

52. Standard 001-9.7 provides that “unless otherwise mutually agreed to by the primary provider and original customer, a request for a Redirect on a Firm basis … [does not] confer any renewal rights on the redirect path.”  (Emphasis added).  This phrase could be interpreted to mean that the parties to an agreement may mutually agree to eliminate rollover rights and that a transmission provider may agree, but is not obligated, to offer rollover rights on the redirect path even when such rights are available.  These provisions are inconsistent with the pro forma OATT and the Commission’s policies.  In addition, the last phrase of the standard also conflicts with the last sentence of section 22.2 of the pro forma OATT, which is limited to the period while the new request for service is pending.  Therefore, we will not adopt Standard 001-9.7 at this time, but will allow the WEQ to reconsider the standard and to adopt a revised standard consistent with the Commission’s policies.

53. The comments on this issue show that there is confusion in the industry regarding the provisions of sections 22.1 and 22.2 of the pro forma OATT.  To assist the WEQ in developing a standard that is consistent with the Commission’s policy, we offer the following guidance.

54. Section 22 of the pro forma OATT addresses changes in service specifications.  Section 22.1 pertains to modifications on a non-firm basis and section 22.2 covers modifications on a firm basis.  Under section 22.1, a firm point-to-point transmission customer may request non-firm transmission service at secondary receipt and delivery points (points other than those specified in the service agreement).  Section 22.1(c) provides that the transmission customer shall retain its right to schedule firm point-to-point transmission service at the receipt and delivery points specified in its relevant service agreement in the amount of its original capacity reservation.

55. Under section 22.2, any request by a transmission customer to modify receipt and delivery points on a firm basis is treated as a new request for service.  This section also provides that, “[w]hile such new request is pending, the Transmission Customer shall retain its priority for service at the existing firm Receipt and Delivery Points specified in its Service Agreement” (emphasis added).  Once the new request is accepted and confirmed, the transmission customer loses all rights to the original receipt and delivery points, including rollover rights associated with the original path.

56. Bonneville asserts that the Commission has stated that the redirect requestor retains section 2.2 reservation priority rights on its original path. [2] Under section 22.1(c), which pertains to redirects on a non-firm basis, the transmission customer retains its right to schedule firm point-to-point service on its original path.  This means that the transmission customer retains its original rights on its original path including its rollover rights on its original path and the requestor does not obtain new rollover rights on the redirected path.  However, there is no similar provision in section 22.2 for redirects on a firm basis.[3]

57. Southern Companies argues that a request by a transmission customer to redirect service on a firm basis cannot change that customer’s rollover rights on the original path and does not confer rollover rights on the redirected path.  We disagree.  Section 22.2 provides that, while a transmission customer’s request for new service on a firm basis is pending, the transmission customer retains its priority for service on its existing path, including rollover rights on its existing path.  However, once a transmission customer’s request for firm transmission service at new receipt and delivery points is accepted and confirmed, the new reservation governs the rights at the new receipt and delivery points and the transmission customer can obtain rollover rights with respect to the redirected capacity.  In addition, at the time the transmission customer’s request for the redirected capacity is accepted and confirmed, the transmission customer loses all rights to the original receipt and delivery points, including rollover rights associated with the original path.

58. As part of its process of review, NAESB identified several questions that were raised regarding rollover rights under the pro forma OATT during members’ deliberations on Standard 001-9.7.  These questions generally raised issues with respect to whether customers retain rollover rights on both the original and the redirected path.

59. A long-term firm transmission customer may request multiple, successive redirects and, as provided in section 22.2 of the pro forma OATT, each such successive request is treated as a new request for service in accordance with section 17 of the pro forma OATT.  As a new request for service, each request is subject to the availability of capacity and subject to the possibility that the transmission provider may not be able to provide rollover rights on the new, redirected path.  For example, assume a transmission customer with a one-year agreement for service between points A and B.  If the transmission customer seeks to redirect on a firm basis in month 4 to points C to D and then redirect back to points A to B thereafter, at the end of the one year agreement the transmission customer would have rollover rights only with respect to points A to B.[4]  With the same assumptions, if the transmission customer begins with points A to B, but redirects in month 4 to points C to D for the remainder of the one-year agreement, the transmission customer would have rollover rights only with respect to points C to D.  If the transmission provider is unable to provide rollover rights on any redirected path, whether to points C to D or, thereafter, to points A to B, it would have to demonstrate at the time of the redirect request that it has native load growth or contracts that commence in the future that prevent it from providing rollover rights.[5]

60. If a transmission provider claims, either at the time of the original transmission request or at the time of a redirect request, that it is unable to provide rollover rights because it has native load growth or a contract that commences in the future, it must still offer transmission service for the time preceding the native load growth or commencement of the future contract.  As explained above, however, it may limit rollover rights based on native load growth or contracts that commence in the future.

61. Further, if a transmission customer with a long-term firm transmission agreement requests to redirect on a firm basis for one month and then redirect on a firm basis back to its original receipt and delivery points for the remainder of the term of the agreement, such requests do not convert its existing long-term firm transmission service agreement into separate short-term transmission service agreements.[6]  Under this scenario, the transmission customer has rollover rights for the original receipt and delivery points, because those are the points to which it has rights at the end of the agreement.

In the FERC Order 676-A, under Docket No. RM05-5-002 issued September 21, 2006, the Commission made the following conclusions:

## Rollover Rights

### Rehearing Requests

- 5. Southern Companies requests that the Commission clarify, or, in the alternative, grant rehearing of, certain findings in Order No. 676.  Specifically, Southern Companies disagrees with the guidance provided in Order No. 676 concerning rollover rights and argues that the guidance provided by the Commission is inconsistent with the Commission’s pro forma OATT adopted in Order No. 888.  Southern Companies objects to the Commission’s guidance that, under section 22.2 of the Commission’s pro forma OATT, a transmission customer may request multiple, successive redirects and that each redirect is treated as a new request for service with rollover rights.  Southern Companies finds this inconsistent with the fact that section 2.2 of the Commission’s pro forma OATT provides rollover rights only for long-term firm transmission service.  It argues that rollover rights do not accompany requests for short-term redirects of firm transmission service.

- 6. Midwest ISO argues that there should be no rollover rights on a redirect path. Midwest ISO contends that the guidance in Order No. 676 that the “transmission provider, therefore, is required to offer rollover rights to a customer requesting a firm redirect if rollover rights are available on the redirect path” is inconsistent with the Commission’s pro forma OATT.

### Commission Conclusion

- 7. In Order No. 676, the Commission adopted a number of NAESB standards regarding redirects, but declined to adopt Standard 001-9.7 regarding rollover rights, finding that the standard did not clearly define parties’ rights and obligations and appeared to be inconsistent with the Commission’s pro forma OATT and the Commission’s policies.  The Commission provided guidance to NAESB concerning our current precedents under Order No. 888 regarding rollover of service as it related to redirects to assist NAESB if it were to try to develop a new standard.  Southern Companies and Midwest ISO seek rehearing of the Commission’s policies with respect to rollover rights and redirects.

- 8. Because the Commission did not adopt the NAESB standard on rollover rights and redirects, the Commission made no changes to its existing Order No. 888 policies concerning rollovers and redirects.  Thus, no action was taken in Order No. 676 that would be the proper subject of a request for rehearing.  As such, Southern Companies and Midwest ISO must continue to comply with their OATTs and Order No. 888 in administering their policies with respect to rollover rights and redirects.  The related issues raised by Southern Companies and Midwest ISO can better be addressed in individual cases in which they arise than in this rulemaking, which has not changed the requirements of Order No. 888 regarding redirects and rollover rights.  Moreover, issues with respect to general modifications to the Commission’s rollover and redirect policies have been posed in Docket No. RM05-25-000, which would be the more appropriate venue for Southern Companies and Midwest ISO to raise their concerns with the Commission’s rollover and redirect policies.

- In the FERC Order 890 under Docket Nos. RM05-17-000 and RM05-25-000 dated February 16, 2007, the Commission made the following conclusions regarding Redirects and Rollover rights:

- 1280. Commission policy allows a redirect of firm, long-term service to retain rollover rights, even if the redirect is requested for a shorter period. In other words, the rollover right follows the redirect, regardless of the duration of the redirect. Contrary to the comments of Bonneville, MISO, and Southern, the Commission did not impose this requirement for the first time in Order No. 676, but merely provided guidance to the industry by restating Commission policy on this matter. The Commission has explained in prior orders that a transmission customer making a firm redirect request does not convert its original long-term firm transmission service to short-term service, nor does it lose its rollover rights under its long-term firm transmission service agreement. The Commission’s concern underlying this policy is that long-term customers should not need to choose between redirecting on a firm basis and maintaining rollover rights, rather their rollover rights should be retained consistent with the long-term nature of their service.

- 1281. In Commonwealth Edison Co., the Commission explained that a “request to change a delivery point on a firm basis for one month and then to revert to its original delivery point does not convert its existing long-term firm transmission service agreement into two separate short-term transmission service agreements.”782 The Commission stated that section 22.2 was intended to provide flexibility to transmission customers to permit them to react in a competitive market and that some amount of this flexibility would be lost if a long-term firm transmission customer seeking to modify its delivery points would lose its rollover rights.783 782 95 FERC ¶ 61,027 at 61,083 (2001). 783 The Commission, however, recognized that this flexibility was not unlimited – any change to a delivery point is treated as a new request for service for purposes of the availability of capacity.

- 1282. The Commission affirmed this policy in American Electric Power Service Corp.784 In that case, a long-term transmission customer (Exelon) had been granted a short-term redirect, but denied rollover rights on the redirected path. The Commission found the denial of rollover rights was improper, since the “redirect request made by Exelon did not convert Exelon’s long-term firm transmission service to short-term service, and therefore, did not affect Exelon’s rollover rights under its long-term firm transmission service agreement.”785 Thus, there is no inconsistency between the Commission’s redirect policy and Order No. 676.

- In the FERC Order 890-A under Docket Nos. RM05-17-001, 002 and RM05-25-001, 002 dated December 28, 2007, the Commission made the following conclusions regarding Redirects and Rollover rights:

### Modification of Receipt or Delivery Points

### [note: Paragraphs 697 and 698 are from the summary section – summarizing prior Commission Decisions]

- 697. Pursuant to Section 22 of the pro forma OATT, a transmission customer taking firm point-to-point service may modify its receipt and delivery points, i.e., redirect its service, on either a non-firm or firm basis.  In Order No. 676, the Commission adopted the “Standards for Business Practices and Communication Protocols for Public Utilities” developed by the NAESB’s Wholesale Electric Quadrant (WEQ).  The WEQ standards include standards addressing requirements for redirects on both a firm and non-firm basis, all of which were incorporated by reference into the Commission’s regulations except for WEQ Standard 001-9.7, which addressed the impact of redirects on the rollover rights of a long-term transmission customer.  Order No. 676 directed the WEQ to reconsider WEQ Standard 001-9.7 and develop a revised standard consistent with Commission policy.

- 698.	In Order No. 890, the Commission affirmed reliance on the NAESB process to develop business practices implementing the Commission’s redirect policy.  The Commission also determined that the reforms adopted in Order No. 676, in combination with the OATT-related reforms adopted in this proceeding, were adequate to ensure that transmission providers do not engage in undue discrimination when a customer seeks to modify its receipt and delivery points on a firm basis.  With respect to the effect of redirects on rollover rights, the Commission affirmed its policy allowing a redirect of firm, long-term service to retain rollover rights, even if the redirect is requested for a shorter period.  The Commission concluded that a transmission customer should not have to choose between maintaining its rollover rights and redirecting on a firm basis.  The Commission noted, however, that any change to a delivery point would be treated as a new request for service for purposes of determining availability of capacity.  As a result, a redirect right does not grant the customer access to system capacity or queue position different from other customers submitting new requests for service.  The Commission also provided guidance regarding the processing of, and pricing for, redirected service.

Commission Determination

- 704.  The Commission denies petitioners’ requests to amend the rights of rollover customers to redirect their service.  Under section 22.2 of the pro forma OATT, a request for a firm redirect must be treated like a request for new transmission service.  As a new request for service, each redirect request is subject to the availability of capacity and subject to the possibility that the transmission provider may not be able to provide rollover rights on the new redirected path.  The transmission provider is required to offer rollover rights to a customer requesting a firm redirect only if rollover rights are available on the redirected path, i.e., to the extent not restricted based on reasonable forecasts of native load growth or preexisting contracts that commence in the future.

- 705.  As the Commission explained in Order No. 890, rollover rights follow the redirect regardless of the duration of the redirect.  A transmission customer making a firm redirect request does not convert its original long-term firm transmission service agreement into two short-term service agreements, nor does it lose its rollover rights under its long-term firm transmission service agreement.  At the same time, a customer can exercise its rollover right only at the end of the contract.  Thus, if a customer with rollover rights chooses to redirect its capacity for less than the full remaining term of the contract, absent some further request to redirect, the original path will automatically be reinstated and rollover rights would remain on only the original path.  By contrast, if the customer chooses to redirect its capacity until the end of its contract, the customer would have rollover rights along only the redirected path, and only to the extent not restricted based on native load growth or future contracts along the redirected path.

- 706.  We therefore reject requests to restrict rollover rights to longer-term redirects.  A long-term transmission customer may request multiple, successive redirects for firm service.  This discretion is limited by the fact that each successive request is treated as a new request for service in accordance with section 17 of the pro forma OATT.  Each request is therefore subject to the availability of capacity and subject to the possibility that the transmission provider may not be able to provide

- rights on the new, redirected path.  If the customer has not been granted rollover rights for a redirect that extends to the end of its contract, the redirected service will terminate on the same date as the parent service.

- 707.  We also reiterate that a customer cannot exercise any rollover rights unless it first has provided the appropriate notice to the transmission provider.  If a customer requests and is granted a rollover right prior to the relevant notice deadline (60 days for pre-Order No. 890 agreements or one year for all others) and subsequently requests and is granted a redirect for firm service for the remainder of the contract term (i.e., within the notice period), the new reservation governs the rights at the new receipt and delivery points and the customer can obtain rollover rights with respect to the redirected capacity to the extent rollover rights are available for the redirected points.  If, however, a customer fails to request a rollover right prior to the relevant notice deadline, the customer forfeits rollover rights along the current or any redirected path.

- 708. We clarify, to the extent necessary, that transfer capability is not freed up for earlier queued service requests until a redirect has been granted.  A redirect request must be evaluated in accordance with section 17 of the pro forma OATT using the same system assumptions and analysis applicable to any other new request for service, including whether sufficient ATC exists to accommodate the request.  If there is insufficient ATC to offer service to customers in the queue, and an existing customer requests redirected service, any increase in ATC along the original path is contingent upon the acceptance and confirmation of the redirect.  It cannot be assumed at the time of a redirect request that the transmission provider will grant the request.

- At the November 4th meeting of the WEQ EC, it was determined that an EC Task Force with the purpose of holding discussion with with FERC Staff regarding remaining questions on the Commission Rollover of Redirect policies. The EC Task Force held a teleconference with FERC Staff on January 8, 2009. In addition to responding verbally to question raised by the EC Task Force, FERC Staff also provided written guidance.

- Written guidance provided by FERC Staff prior to the teleconference:

Redirects and Rollover Rights:

FERC Staff’s Interpretation of the OATT

1.	Eligibility for rollover rights follows the redirect if the redirect is for the remaining term of the contract (i.e., not paired with a redirect back to the original path), although the transmission provider may restrict rollover rights on the redirected path if necessary to serve reasonably forecasted native load growth or preexisting contracts that commence in the future.  See Order No. 890 at P 1280, 1286; Order No. 890-A at P 705; Order No. 676 at P 51, 57-59.

2.	Renewal on a particular path does not limit the customer’s rights associated with subsequent redirects.  See Order No. 890-A at P 707.

3.	Failure to renew a reservation by the renewal deadline date waives all rollover rights for the reservation.  See Order No. 890-A at P 707.

4.	Partial renewal reduces the eligibility for rollover rights to the amount renewed.  Cf. Order No. 890-A at P 707.

5.	Restrictions on rollover rights stated in a service agreement do not follow a redirect.  See Order No. 676 at P 51, 59; Order No. 890-A at P 704, 706.

6.	Granting rollover rights on a redirected path does not affect the ability of the customer to redirect, subject to availability, back to the original or another path.  See Order No. 676 at P 59.

- Written guidance provided by FERC Staff after the teleconference:

Rae –

Following up on our call yesterday, we have the following to offer regarding Ed’s hypothetical of a partial quantity redirect of a 100 MW reservation with rollover rights of 60 MW on the parent reservation.  We are of the opinion that a customer should be eligible to receive up to 100 MW of rollover rights for that reservation if it redirected, subject to availability on the redirect path.

Consistent with the above, if the customer redirected 50 MW to a new path for the remainder of the contract term, and retained 50 MW on the parent path, the customer would have 50 MW of rollover rights on the parent path and, subject to availability, up to 50 MW of rollover rights on the redirected path.  The amount of rollover rights available on the parent path would be within the original limitation stated in the service agreement, and the amount of rollover rights on the redirected path would not be restricted by that limitation (although a separate limitation may apply depending on the results of the transmission provider’s study).  The aggregate rollover rights would be 100 MW, equal to the amount for which the customer is eligible under the contract.  That is, a customer signing, and paying for, a contract for 100 MW of the appropriate length is eligible to receive 100 MW of rollover rights for that contract.  The only reason those rights were limited for the parent path is because capacity was not available on that path.  If capacity is available on an alternative path, the customer should be entitled to receive rollover rights up to the amount associated with its contract.

Similarly, if the customer redirected 30 MW to a new path for the remainder of the contract term, and retained 70 MW on the parent path, the customer would still have only 60 MW of rollover rights on the parent reservation and, subject to availability, up to 30 MW on the redirected path.  The aggregate rollover rights would be up to 90 MW, less than the amount for which the customer is eligible due to the restriction stated in the contract.

Please forward this as appropriate to the participants on yesterday’s call.  Thanks again for the opportunity to consider and discuss the issues the EC is working through, and let us know if there’s anything else we can do to be of assistance.

Mason

- b.  Description of Recommendation:

- c.  Business Purpose:

Implementation of FERC Orders.

- d.  Commentary/Rationale of Subcommittee(s)/Task Force(s):

The following commentary/rationale is provided to aid in understanding this first draft of the recommended standard proposed by OATI:

001-9.7: FERC declined to adopt 9.7 and asked NAESB to reconsider FERC policy with respect to conveyance of rollover rights.  Hence, this standard recommendation.

001-9.7.1: This standard and its subordinate standards 9.7.1.1-9.7.1.5 are proposed to clearly identify when a customer wishes to convey rollover.  The Commission’s Orders actually have less flexibility than proposed in the recommendation in that they do not envision an option to retain rollover regardless of redirect.  One could read the “guidance” that if I redirect on firm basis to end of term of long-term reservation it ALWAYS conveys rollover – assuming they are not limited.  9.7.1 and subordinate standards specify exactly what must be submitted – it’s a long-term (aka YEARLY) Redirect, submitted prior to renewal deadliine (after that you have no rights to rollover), and must match end of term of service.  The only “gotcha” is that it will be a “YEARLY” service that could be for one-day (the last day) of service submitted 60 (or 1 year) in advance.  FERC has been very clear that making a shorter duration (aka MONTHLY) redirect does NOT change the fundamental long-term nature of the rights ot the original.  They have reaffirmed this policy addressing TranServ comments with respect to resale of a conditional firm weekly service – I read this as clearly in physical rights situations, the rights purchased govern, not how they may later be remarketed, redirected, etc.  So lets let them submit a long-term service and ignore timing if it’s a REDIRECT.  This is REALLY great when we look at OASIS Implementation Guide on RENEWAL.  As written, if I submit a renewal against MONTHLY, most would invalidate it out of hand.  So, you have a REDIRECT that’s YEARLY for one day – and RENEWAL is okay as it starts on parent stop, service is same, etc.

Now, we COULD limit the minimum term of the redirect to MONTHLY as that is the only example the Commission (Staff) made – there “guidance” was a less then yearly, monthly request that still conferred rollover.

001-9.7.1.5: This is a catchall for things like trying to redirect more MWs or outside of start/stop of parent, that are already prohibited by the rest of the 001-9 standards.

001-9.7.2:  This clarifies that if you don’t qualtify under 9.7.1, you get no rollover – that means long-term redirects in “mid-stream” are no better than short-term firm redirects that end conincident with the long-term service.

001-9.7.3: These requests get no special treatment with respect to the queue of LONG-TERM requests for service with rollover.  Another justification that even though the “term” of the request doesn’t qualify as “long-term”, the treatment in the queue does set its priority relative to others that are also seeking long-term service with rollover.

001-9.7.4: Specifically suggest that when releasing the rollover rights of parent, consider that only release to the most restrictive limit.  If I hold a limited right to renew 100 MWs on an original request for 200 MWs and I redirect 100 MWs that is accepted with rollover, I did not drop my orignal path rollover to 0, but to the lower of 200 -100 =100 or limited rollover = 100, which means as customer I am not unduly harmed by my decision to move to another path.  I am paying the full boat on both paths, so I don’t see an equity issue here.

Additional Background documentation

- 2006 WEQ Annual Plan Item (3)(a)(x) Recommendation, which was remanded back to the subcommittee by the Executive Committee at the August 14-15, 2006 meeting.

- Standards Request R07004 Business practice and technical standards for assigning, tracking, and limiting rollover rights

- Rollover of Redirects Motions 2009

- Rollover of Redirects Core Concepts 2009

- February 4-5, 2009 (unavailable - DC)

- February 18, 2009 (unavailable – DC)

- March 4, 2009 (unavailable – DC)

- March 17-19, 2009 (unavailable – DC)

- April 7-9, 2009 (unavailable – DC)

[1] Standard 001-9.7 appears consistent with section 22.2 of the existing pro forma OATT insofar as it provides that a customer requesting a firm redirect does not relinquish its rollover rights over its primary path simply by making the request.

[2] As explained in the notice of inquiry in Docket No. RM05-25-000, 70 FR 55796, FERC Stats. & Regs. ¶ 35,553 at P 18 (2005), section 2.2 of the pro forma OATT (Reservation Priority for Existing Firm Service Customers) provides that “existing firm service customers (wholesale requirements and transmission-only, with a contract term of one-year or more) have the right to continue to take transmission service from the public utility transmission provider when the contract expires, rolls over or is renewed.  It specifically provides that this transmission reservation priority is independent of whether the existing customer continues to purchase capacity and energy from the public utility transmission provider or elects to purchase capacity and energy from another supplier.”

[3] Bonneville at 2.

[4] The Commission assumes that a transmission customer would make the two requests to redirect to points C to D and then back to points A to B at the same time.  Otherwise, the transmission customer would put itself at risk of not being able to redirect back to points A to B because of an intervening request for transmission service.

[5] See, e.g., Tenaska Power Services Co. v. Southwest Power Pool, Inc., 99 FERC   61,344 (2002), reh’g denied, 102 FERC ¶ 61,140 at P 33, 38 (2003); Nevada Power Company, 97 FERC ¶ 61,324, at 62,492 (2001).

[6] See, e.g., Commonwealth Edison Co., 95 FERC ¶ 61,027 (2001).

Southern Companies Rehearing Request at 3-5.

Order No. 676 at P 59.

Midwest ISO Rehearing at 5.

Order No. 676 at P 52.

See Commonwealth Edison Co., 95 FERC ¶ 61,027, at 61,083 (2001) (“request to change a delivery point on a firm basis for one month and then to revert to its original delivery point does not convert its existing long-term firm transmission service agreement into two separate short-term transmission service agreements”); American Electric Power Service Corp., 97 FERC ¶ 61,207, at 61,905-06 (2001) (“redirect request made by Exelon did not convert Exelon’s long-term firm transmission service to short-term service, and, therefore, did not affect Exelon’s rollover rights under its long-term firm transmission service agreement”).

Preventing Undue Discrimination and Preference in Transmission Services, Notice of Proposed Rulemaking, FERC Stats. & Regs., Proposed Regulations ¶ 32,603 (2006).

Standards for Business Practices and Communication Protocols for Public Utilities, Order No. 676, 71 FR 26199 (May 4, 2006), FERC Stats. & Regs. ¶ 31,216 (2006), reh’g denied, Order No. 676-A, 116 FERC ¶ 61,255 (2006), order on reh’g, Order No. 676-B, 72 FR 21095 (Apr. 30, 2007), FERC Stats. & Regs. ¶ 31,246 (2007).

See Order No. 890 at P 1268.

See Order No. 676 at P 51.

Order No. 890 at P 1280.

Id.; see also Commonwealth Edison Co., 95 FERC ¶ 61,027 at 61,083 (2001) (explaining that a request to change delivery points on a firm basis for one month, followed by a reversion to the original points does not convert the existing long-term firm agreement into two separate short-term agreements); American Electric Power Service Corp., 97 FERC ¶ 61,207 at 61,905-06 (2001).

For example, assume a transmission customer with a five-year agreement for firm service between points A and B, who qualifies for rollover rights on that path.  If the transmission customer seeks to redirect on a firm basis in year 3 to points C to D and then redirect back to points A and B thereafter, at the end of the five year agreement the transmission customer would have rollover rights only with respect to points A to B.  If, however, the transmission customer seeks to redirect to points C and D for the last six months of the contract term and both qualifies for rollover rights on this path and has requested rollover within the notice period of the contract, the customer would then have rollover rights only with respect to points C and D.  See Order No. 676 at P 59.

Order No. 890 at P 1285.