- 1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| X | Accept as requested | X | Change to Existing Practice |
| --- | --- | --- | --- |
|  | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

- 2.  TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
| X | Initiation | X | Initiation |
|  | Modification |  | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
| X | Principle | X | Principle |
| X | Definition | X | Definition |
| X | Business Practice Standard | X | Business Practice Standard |
| X | Document | X | Document |
| X | Data Element | X | Data Element |
| X | Code Value | X | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
| X | Business Process Documentation | X | Business Process Documentation |

- 3.  RECOMMENDATION

- SUMMARY:

The Joint Retail Electric (REQ) and Wholesale Electric (WEQ) Quadrants’ PAP 10 Smart Grid Subcommittee submit this Recommendation for 2010 Retail Annual Plan Item No. 9d and 2010 WEQ Annual Plan Item No. 6d – Business Practices and Information Models to support Priority Action Plan 10, “Standardized Energy Usage Information,” based on the Tiger Team Report issued on June 7, 2010 by the NIST SGIP PAP10 Committee.

In initiating this standards development, NAESB agreed to by year-end 2010 develop an energy use information model standard defining a common data format that may be used when information is communicated between utilities, third parties and energy end-use customers, via customer devices and/or third party energy services providers.  The energy usage information model standard will enable the exchange of detailed energy information in a consistent format for use by customers, utilities, service providers, consumer devices, and energy applications.  Doing so will let Customers track their power use and help them manage energy consumption and cost.  Without a standardized format for representing energy use data, a variety of approaches could emerge, leading to incompatibilities among energy management products and services.  It is planned that the American Society of Heating, Refrigerating and AC Engineers (ASHRAE) will extend the NAESB standard to create a facilities information model providing additional energy use data elements for facility energy management including buildings.

WEQ-000

New Abbreviations/Acronyms and Defined Terms to be added to WEQ-000 in support of 2010 WEQ AP Items 6(d)

000-1	ABBREVIATIONS AND ACRONYMS

| Abbreviation / Acronym | Meaning |
| --- | --- |
| PEV | Plug-in Electric Vehicle |
| EISA | Energy Independence and Security Act |
| DOE | Department of Energy |
| HAN | Home Area Network |
| XMI | XML Media Exchange |
| XML | Extensible Markup Language |
| HTML | HyperText Markup Language |
| XSD | XML Schema Definition Language |
| CIM | Common Information Model |
| IEC | Internationl Electrotechnical Commission |

000-2	DEFINITION OF TERMS

#### Applicable Regulatory Authority

The state regulatory agency or other governing body that provides oversight, policy guidance, and direction to any parties involved in the process of providing energy to end-use customers through regulations and orders.

#### Distribution Company

A regulated Entity which provides distribution services and may provide energy and/or transmission/transportation services in a given area.

#### Entity

A person or organization with sufficient legal standing to enter into a contract or arrangement with another such person or organization (as such legal standing may be determined by those parties) for the purpose of conducting and/or coordinating energy transactions.

#### Governing Documents

Documents that govern the interactions among parties, including but not limited to: applicable law, regulatory documents (e.g. tariffs, rules, regulations), contractual agreements, and Distribution Company Operational Manuals and other relevant models and procedures.

#### Market Participant

A party engaged in the process of providing competitive retail energy to end-use customers including but not limited to the Distribution Company, the Supplier, the registration agent, the settlement agent, and the meter reading Entity.

#### Supplier

Entities engaged in the competitive sale of energy to end-users.

#### Fine Grained

Characterized by abundant use of detail or thoroughness of treatment.

- RECOMMENDED STANDARDS:

# WEQ-019	CUSTOMER ENERGY USAGE INFORMATION COMMUNICATION

- EXECUTIVE SUMMARY

This document establishes  the Business Practice Standards for end-use energy usage information communication.  Specifically, these Business Practice Standards establish an information model for energy usage information.  The Business Practice Standard does not limit the form or function of the information model and is exemplary, but not exclusive, of the information that may be communicated in a consistent format among a variety of Entities, including but not limited to Distribution Companies, energy service providers, meter-reading entities, and end-use customers.  Such communication may occur via multiple systems and devices.  Establishment of this energy usage information model will standardize a common data format which may be used when information is communicated between Distribution Companies, third parties and energy end-use customers which may aid end-use customers in tracking and managing their energy use.

These Business Practice Standards do not require that wholesale electricity markets administered by System Operators adopt this energy usage information model since System Operators generally do not maintain or have access to the system of record for individual end-use customer energy usage information and load data or individual end-use customer forecasted usage and load data.  These Business Practice Standards are not intended to replace applicable Governing Documents, and, in the event of a conflict, the latter documents shall have precedence over these Business Practice Standards. Without limiting the foregoing, these Business Practice Standards are only applicable to the extent the information covered by this energy usage information model is collected, managed or communicated pursuant to the applicable Governing Documents.  End-use customer energy usage information communication encompasses a variety of interactions between Distribution Companies, end-use customers and energy services providers.  In a business environment where best practices are voluntary, these Business Practice Standards may be applied within the context of regulatory or other market requirements and agreements.

- INTRODUCTION

The North American Energy Standards Board (NAESB) is a voluntary, non-profit organization comprised of members from all aspects of the natural gas and electric industries.  Within NAESB, the Wholesale Electric Quadrant (WEQ) and the Wholesale Gas Quadrant (WGQ) focus on issues impacting the sale of energy to wholesale customers.  WEQ / WGQ Business Practice Standards are intended to provide guidance to energy market participants.  WEQ Business Practice Standards and Retail Electric Quadrant (REQ) Model Business Practices have been developed in concert for market participants not limited to: Distribution Companies, energy Suppliers, and energy service providers involved in providing competitive energy service to end-use customers.  The focus of these Business Practice Standards is the representation of end-use customer energy usage information.  The scope of the energy usage information model is not intended to characterize the data information model for communication of  billing or related information.

These Business Practice Standards are voluntary and do not address policy issues that are the subject of state legislation or regulatory decisions.  These Business Practice Standards have been adopted with the realization that as the industry evolves, additional and amended Business Practice Standards may be necessary.

- BUSINESS PROCESSES AND PRACTICES

Overview

This Business Practice Standard establishes the Business Practice Standards for end-use customer energy usage information communication.  Specifically, these Business Practice Standards establish an information model for energy usage information.  The Business Practice Standard does not limit the form or function of the information model and is exemplary, but not exclusive, of the information that may be communicated in a consistent format among a variety of Entities, including, but not limited to, Distribution Companies, energy service providers, meter-reading entities, and end-use customers.  Such communication may occur via multiple systems and devices.  Establishment of this energy usage information model will standardize a common data format which may be used when information is communicated between Distribution Companies, third parties and energy use customers which may aid end-use customers in tracking and managing their energy use.

The business processes and practices described below are not presently applicable to wholesale markets because wholesale markets do not generally communicate directly with end-use customers and are not the system of record for individual end-use customer energy usage information or individual end-use customer load forecast. The energy usage information model and these model business processes and practices are not required of System Operators.  As the energy usage information model and these business processes and practices evolve, System Operators may determine that use of the energy usage information model in these Business Practice Standards can be applied to other information.  However, such use is not intended to replace or supplant applicable Governing Documents. Without limiting the foregoing, these Business Practice Standards are only applicable to the extent the information covered by this energy usage information model is collected, managed or communicated pursuant to the applicable Governing Documents.

## WEQ-019.1	Principles

### WEQ-019.1.1	Overall Principles

#### WEQ-019.1.1.1	These Business Practice Standards provide an energy usage information model, which defines a collection of structured energy usage information classes and attributes that may be used to enable end-use customer management of their energy usage and costs.

#### WEQ-019.1.1.2	The energy usage information model is specified in UML, as a syntax neutral notation, so that it may be used within exchange protocols using a variety of specific representation syntax and exchange mechanisms, specified outside these Business Practice Standards.

#### WEQ-019.1.1.3	The recommended use of the energy usage information model is in implementation specifications exposing end-use customer energy usage information. Specifications that conform to the energy usage information model shall contain equivalent required and optional classes and attributes, thus resulting in straightforward (preferably lossless) transformations between conformant specifications.

#### WEQ-019.1.1.4	Neither the energy usage information model, nor these Business Practice Standards establish or govern ownership or any other rights in any information or data; such ownership and other rights are subject to and governed by the Governing Documents and/or applicable laws and regulations.

#### WEQ-019.1.1.5	Neither the energy usage information model nor these Business Practice Standards create any requirement to collect, manage or communicate any information.

#### WEQ-019.1.1.6	While these Business Practice Standards define an information model, which may be used when energy usage information is communicated, the Governing Documents and/or Applicable Regulatory Authority determine the ownership of the data, the access to the data, what systems and hardware are necessary to provide this information, and how it is paid for. There are no assumed or implied regulations in this Business Practice Standard.

#### WEQ-019.1.1.8	The energy usage information model represents a series of capabilities for the expressiveness of this standard and is specifically not intended to be a requirement for the use of or on users of this standard.

## WEQ-019.3		Energy Usage Information Business Practice Standards

### WEQ-019.3.1		Introduction

The focus of these Business Practice Standards is the representation of energy usage information,which includes historic, present, and future projected usage and load together with the time period(s) for that information.

These Business Practice Standards draw on actors and use cases defined by the following groups:

- Energy Information Standards Alliance (the EIS Alliance) [EIS]

- NAESB Survey and Consolidation of PAP10 Use Cases [NAESB PAP10]

- UCAIug OpenADE [ADE]

- ZigBee/Home Plug Smart Energy Profile 2.0 Market Requirements [SEP MRD]

The relevant use cases are summarized as follows:

#### WEQ-019.3.1.1	The energy service provider and/or Distribution Company communicates historic and present energy usage information and load information to the Retail Customer or facility.

#### WEQ-019.3.1.2	The Retail Customer or facility communicates future projected usage and load information to the energy service provider, Distribution Company, or grid operations.

#### WEQ-019.3.1.3	The energy service provider and/or Distribution Company communicates their projection of usage and load to the Retail Customer or facility.

#### WEQ-019.3.1.4	Devices within a facility communicate their present and future projected usage and load to controllers or facility EMS for aggregation and to be a component of facility aggregated future projected usage and load.

#### WEQ-019.3.1.5	Devices, business processes, EMS, ESI, and other functional units within the facility communicate usage and load information among themselves.

#### WEQ-019.3.1.6	These Business Practice Standards are limited to the seed specification which shall be usable by others to build standards, or Business Practice Standards, and/or specification for exchange of energy usage information and load information appropriate to their needs without overly constraining those uses or including information that is not required in all implementations of specifications for exchanging load and usage.

### WEQ-019.3.2		Energy Usage Information Model Format and Use

The energy usage information model is developed using a UML modeling tool.  The model classes, attributes, types and descriptions are included in WEQ-019.4.  The model is made available as XMI, which is the standard XML import/export format for UML.  The model is exported as HTML and made available as a downloadable archive viewable with a web browser.

### WEQ-019.3.3	Energy Usage Information Model Technical Considerations

#### WEQ-019.3.3.1	The energy usage information model may be used as the basis for smart grid interfaces between parties exchanging customer energy usage information between energy services providers, consumers, and others.

#### WEQ-019.3.3.2	The energy usage information model permits schemas to be generated from it using XSD and other format specification languages. Profiles may be constructed from the energy usage information model for this purpose.

#### WEQ-019.3.3.3	Implementations may include all or a subset of the elements defined in the model, possibly using a profile of the energy usage information model.

#### WEQ-019.3.3.4	The informative example XSD should conform to Naming and Design Rules best practices as described by IEC 62361-100, Naming and Design Rules for CIM Profiles to XML Schema Mapping.

#### WEQ-019.3.3.5	The model facilitates the use of multiple information exchange standards. The specifics are left to implementating specifications to define.

#### WEQ-019.3.3.6	Though there may be elements useful for the transfer of security-related information elements in the energy usage information model, the specific details related to how to protect sensitive information, and how to authorize specific roles or identities to have access are not defined in this Business Practice Standard.

### WEQ-019.3.4	Conformance

#### WEQ-019.3.4.1	A specification that claims conformance andrefines or extends this Business Practice Standard shall produce information for exchange that can be transformed algorithmically (that is based on the Business Practice Standard alone) into a form that can be validated through the method described in WEQ-19.3.4.2.

This requirement means that various formats for representation and exchange and various subsets and potentially supersets of information content are envisioned based on this Buisness Practice Standard. The use of the informative example schema is not to impose its direct use in message validation. Rather, its use is intended to facilitate verification of conformance to the energy usage information model with respect to message content and semantics without imposing constraints on specific message payload schemas and data element representations. Some representations are anticipated to be entirely binary in nature. Others will trade off strings for integer representations of information contents.  Regardless of the means, the information should be convertible to be testable as described herein.

#### WEQ-019.3.4.2	A specification that claims conformance to this Business Practice Standard shall describe and define an automatable transformation between that specification’s model to and from this UML model, including indicating attributes used and not used, as well as the version of the model stated in NAESB_EUI_Version.

#### WEQ-019.3.4.3	A specification that claims conformance to these Business Practice Standards shall map corresponding energy usage information model components to and from at least the following required core model attributes as exchanged between data provider and data consumer in their defined messages (Refer to WEQ-019.4.1 for definitions of capitalized terms):

- One or more measurement or summary containers: IntervalReading, Reading, PowerQualitySummary, UsageSummary

- At least two of the following attributes, for each IntervalReading: timeStamp, endTimeStamp, duration

- The attribute value (the value of the measurement, from IntervalReading or Reading)

- ReadingType – ID, defaultQuality, direction, kind, multiplier, name, unit

- Association to ReadingType for each measurement (IntervalReading or Reading) (exists in model through MeterReading)

- Measurement source / location – ServiceDeliveryPoint.ID or MeterAsset.ID and association to measurements or summary

## WEQ-019.4		Energy Usage Information Model

The energy usage information model herein is organized consistent with several related models, including the IEC TC57 Common Information Model [IEC 61968 Part 9], ZigBee Smart Energy Profile 2.0 [SEP2.0], that are defined by the Energy Information Standards Alliance [EIS Alliance] and Open Automated Data Exchange [OpenADE]. The energy usage information model, where possible, uses classes, information elements and attribute names drawn from the CIM and the cited references.

The starting point for the energy usage information model is the ServiceDeliveryPoint. ServiceDeliveryPoints identifies key references for the information set optionally including identification of the customer, the location, and the physical asset. ServiceDeliveryPoint is associated in turn with zero or more MeterReadings. A MeterReading composes information about a particular measurement such as kWh or kW. A MeterReading has a ReadingType which describes the nature of the measurement including its units of measure, and zero or more IntervalReadings or Readings and associated quality information. ServiceDeliveryPoint may also be associated with summary information on load and usage, and optionally, power quality.  For applications requiring third party access to this information, additional classes are identified to facilitate associating customer and customer agreement information with the measurements available at a ServiceDeliveryPoint.

To find the usage or load in a particular interval, identify the appropriate ServiceDeliveryPoint, select the MeterReading of interest (measurement) and then select the IntervalReading or Reading associated with the given interval.

The energy usage information model includes many optional components. The complete set of information expressable using the energy usage information model satisfies a wide range of applicability requirements identified by the industry. Users of this Business Practice Standard may optionally take advantage of these extended definitions based on need without requiring them. Applications built on the energy usage information model may elect which optional components to present. However, clients of this information can be expected to recognize all components provided in the application.

WEQ-019.3.4.3 idenitifies the set of core model elements that shall be supported by specifications claiming conformance to this Business Practice Standard. The following class diagram illustrates a view of this core of the energy usage information model:

Figure 1: Energy Usage Information Model – Minimal View

The full energy usage information model, illustrated in Figure 2,  forms the basis of the required Business Practice Standard.  Note, some minor classes detailed in WEQ-019.4.1 are omitted from the original diagram to aid in readability (e.g. DateTimeInterval).

Figure 2: Full Energy Usage Information Model

### WEQ-019.4.1	Energy Usage Information Model Details

The following sections contain the classes and attributes defined in the energy usage information model, along with their descriptions. Elements tagged with <<enumeration>> define the valid values for an enumerated data type and so do not have their own data type, and should be self explanatory, not requiring a definition.

Terms in this section may be based upon IEC 61968 classes and their descriptions.

The following sections contain the classes and attributes defined in the model, along with their descriptions.  Elements tagged with <<enumeration>> define the valid values for an enumerated data type.

      - AbsoluteDateTime  «Datatype»

Date and time as specified in ISO 8601.

      - Boolean  «Primitive»

A type with the value space "true" and "false".

      - Customer

Organisation receiving services from ServiceSupplier.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

      - CustomerAgreement

Agreement between the customer and the service supplier to pay for service at a specific service location. It records certain billing information about the type of service provided at the service location and is used during charge creation to determine the type of service.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

      - CustomerAuthorisation

Holds an authorisation for access to specific user-private data granted to a third party service provider.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |
| validityInterval | DateTimeInterval | Date and time interval this agreement is valid (from going into effect to termination). |

      - DateTimeInterval  «Compound»

Interval of date and time.

| Name | Type | Description |
| --- | --- | --- |
| start | AbsoluteDateTime | Date and time that this interval started. |
| end | AbsoluteDateTime | Date and time that this interval ended. |
| duration | Duration | The duration of this interval, in seconds |

      - Duration  «CIMDatatype»

An interval of time, specified in ISO 8601 compatible format.

      - ElectricPowerQualitySummary

A summary of power quality events. This information represents a summary of power quality information required by customer facility energy management systems. It is not intended to satisfy the detailed requirements of power quality monitoring. All values are as defined by measurementProtocol during the period. The standards typically also give ranges of allowed values; the information attributes are the raw measurements, not the "yes/no" determination by the various standards. See referenced standards for definition, measurement protocol  and period.

| Name | Type | Description |
| --- | --- | --- |
| flickerPlt | Float | A measurement of long term Rapid Voltage Change |
| flickerPst | Integer | A count of Rapid Voltage Change events during the summary interval period |
| harmonicVoltage | Float | A measurement of the Harmonic Voltage during the period. For DC, distortion is with respect to a signal of zero Hz. |
| longInterruptions | Integer | A count of Long Interruption events (as defined by measurementProtocol) during the summary interval period. |
| mainsVoltage | Float | A measurement of the Mains [Signaling] Voltage during the summary interval period. |
| measurementProtocol | String | A reference to the source used as the measurement protocol definition. e.g. "IEEE1519-2009", "EN50160" |
| powerFrequency | Float | A measurement of the power frequency during the summary interval period. |
| rapidVoltageChanges | Integer | A count of Rapid Voltage Change events during the summary interval period |
| shortInterruptions | Integer | A count of Short Interruption events during the summary interval period |
| summaryInterval | DateTimeInterval | Interval of summary period |
| supplyVoltageDips | Integer | A count of Supply Voltage Dip events during the summary interval period |
| supplyVoltageImbalance | Integer | A count of Supply Voltage Imbalance events during the summary interval period |
| supplyVoltageVariations | Integer | A count of Supply Voltage Variations during the summary interval period |
| tempOvervoltage | Integer | A count of Temporary Overvoltage events (as defined by measurementProtocol) during the summary interval period |

      - EndDeviceAsset

AssetContainer that performs one or more end device functions. One type of EndDeviceAsset is a MeterAsset which can perform metering, load management, connect/disconnect, accounting functions, etc. Some EndDeviceAssets, such as ones monitoring and controlling air conditioner, refrigerator, pool pumps may be connected to a MeterAsset. All EndDeviceAssets may have communication capability defined by the associated ComFunction(s). An EndDeviceAsset may be owned by a consumer, a service provider, utility or otherwise.

There may be a related end device function that identifies a sensor or control point within a metering application or communications systems (e.g., water, gas, electricity).

Some devices may use an optical port that conforms to the ANSI C12.18 standard for communications.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

      - EnergyUsageInformation

A collection of customer energy usage information. This class is a container, and has no attributes.

      - Float  «Primitive»

A floating point number. The range is unspecified and not limited.

      - IdentifiedObject

This is a root class to provide common identification for all classes needing identification and naming attributes

      - Integer  «Primitive»

An integer number. The range is unspecified and not limited.

      - IntervalBlock

Time sequence of Readings of the same ReadingType. Contained IntervalReadings may need conversion through the application of an offset and a scalar defined in associated Pending.

      - IntervalReading

Data captured over a specific interval of time. If not specified, the duration is the intervalLength of the associated ReadingType, where the full definition of the units of measure is located.

| Name | Type | Description |
| --- | --- | --- |
| cost | Float | The cost associated with this reading for this interval. |
| interval | DateTimeInterval | The time interval associated with the reading. |
| value | Float | Value of this interval reading. |

      - MeterReading

Set of values obtained from the meter.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |
| valuesInterval | DateTimeInterval | Date and time interval of the data items contained within this meter reading. |

      - Name

The Name class provides the means to define any number of human readable  names for an object. A name is not to be used for defining inter-object relationships. For inter-object relationships instead use the object identification 'mRID'.

| Name | Type | Description |
| --- | --- | --- |
| name | String | Any free text that name the object. |

      - NameType

Type of name. Possible values for attribute 'name' are implementation dependent but standard profiles may specify types. An enterprise may have multiple IT systems each having its own local name for the same object, e.g. a planning system may have different names from an EMS. An object may also have different names within the same IT system, e.g. localName and aliasName as defined in CIM version 14. Their definitions from CIM14 are

The localName is a human readable name of the object. It is only used with objects organized in a naming hierarchy. localName: A free text name local to a node in a naming hierarchy similar to a file directory structure. A power system related naming hierarchy may be: Substation, VoltageLevel, Equipment etc. Children of the same parent in such a hierarchy have names that typically are unique among them.

aliasName: A free text alternate name typically used in tabular reports where the column width is limited.

| Name | Type | Description |
| --- | --- | --- |
| name | String | Name of the name type. |

      - NameTypeAuthority

Authority responsible for creation and management of names of a given type; typically an organization or an enterprise system.

| Name | Type | Description |
| --- | --- | --- |
| name | String | Name of the name type authority. |

      - PositionPoint

Set of spatial coordinates that determine a point, defined in coordinate system specified in 'Location.CoordinateSystem', or "WGS 84" if not specified otherwise. Use a single position point instance to desribe a point-oriented location. Use a sequence of position points to describe a line-oriented object (physical location of non-point oriented objects like cables or lines), or area of an object (like a substation or a geographical zone - in this case, have first and last position point with the same values).

| Name | Type | Description |
| --- | --- | --- |
| xPosition | String | X axis position. |
| yPosition | String | Y axis position. |
| zPosition | String | (if applicable) Z axis position. |

      - QualityOfReading  «enumeration»

List of codes indicating the quality of the reading

| Name | Type | Description |
| --- | --- | --- |
| estimated |  |  |
| forecast |  |  |
| mixed |  |  |
| raw |  |  |
| validated |  |  |
| normalizedForWeather |  |  |
| other |  |  |

      - Reading

Specific value measured by a meter or other asset. Each Reading is associated with a specific ReadingType.

| Name | Type | Description |
| --- | --- | --- |
| cost | Float | Cost in a currency |
| timeStamp | AbsoluteDateTime | The time when the value was last updated |
| value | Float | Value of this reading. |

      - ReadingDirection  «enumeration»

| Name | Type | Description |
| --- | --- | --- |
| forward |  |  |
| reverse |  |  |
| net |  |  |
| total |  |  |

      - ReadingKind  «enumeration»

Kind of reading.

| Name | Type | Description |
| --- | --- | --- |
| energy |  |  |
| power |  |  |
| demand |  |  |
| voltage |  |  |
| current |  |  |
| voltageAngle |  |  |
| currentAngle |  |  |
| phaseAngle |  |  |
| powerFactor |  |  |
| pressure |  |  |
| volume |  |  |
| date |  |  |
| time |  |  |
| frequency |  |  |
| other |  |  |
| carbon |  |  |
| carbonDioxide |  |  |
| currentAverage |  |  |
| currentRMS |  |  |
| currentTHD |  |  |
| distortionPower |  |  |
| HCH |  |  |
| methane |  |  |
| NOx |  |  |
| perfluorocarbons |  |  |
| phasorPower |  |  |
| quantityPowerQ45 |  |  |
| quantityPowerQ60 |  |  |
| SO2 |  |  |
| sulfurHexafluoride |  |  |
| voltageAverage |  |  |
| voltageRMS |  |  |
| voltageTHD |  |  |
| relativeHumidity |  |  |
| volumetricFlow |  |  |

      - ReadingQuality

Quality of a specific reading value or interval reading value. Note that more than one quality may be applicable to a given reading. Typically not used unless problems or unusual conditions occur (i.e., quality for each reading is assumed to be 'good' unless stated otherwise in associated reading quality).

| Name | Type | Description |
| --- | --- | --- |
| quality | QualityOfReading | Quality, to be specified if different than ReadingType.defaultQuality. |

      - ReadingType

Type of data conveyed by a specific Reading.

| Name | Type | Description |
| --- | --- | --- |
| defaultQuality | QualityOfReading | Characteristics of a data value conveyed by a specific Reading, which allow an application to understand how a specific Reading is to be interpreted. |
| direction | ReadingDirection | Specifies the direction of flow of the measurement. |
| intervalLength | Duration | (if incremental reading value) Length of increment interval. |
| kind | ReadingKind | Kind of reading. |
| multiplier | UnitMultiplier | Multiplier for 'unit'. |
| name | String | The name is any free human readable and possibly non unique text naming the object. |
| unit | UnitSymbol | Unit for the reading value. |

      - Seconds  «Datatype»

Time, in seconds

| Name | Type | Description |
| --- | --- | --- |
| value | Float | Time, in seconds |
| unit | UnitSymbol |  |
| multiplier | UnitMultiplier |  |

      - ServiceCategory

Category of service provided to the customer.

| Name | Type | Description |
| --- | --- | --- |
| kind | ServiceKind | Kind of service. |

      - ServiceDeliveryPoint

Logical point on the network where the ownership of the service changes hands. It is one of potentially many service points within a service location, delivering service in accordance with a customer agreement. Used at the place where a meter may be installed.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

      - ServiceKind  «enumeration»

Kind of service.

| Name | Type | Description |
| --- | --- | --- |
| electricity |  |  |
| gas |  |  |
| water |  |  |
| time |  |  |
| heat |  | Includes hot water and steam |
| refuse |  |  |
| sewerage |  |  |
| rates |  |  |
| tvLicence |  |  |
| internet |  |  |
| other |  |  |
| cold |  | Includes chilled water and ice |

      - ServiceSupplier

Organisation that provides services to customers.

| Name | Type | Description |
| --- | --- | --- |
| kind | SupplierKind | Kind of supplier. |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

      - String  «Primitive»

A string consisting of a sequence of 8 bit characters. The character encoding is UTF-8. The string length is unspecified and unlimited.

      - SummaryMeasurement

An aggregated summary measurement reading.

| Name | Type | Description |
| --- | --- | --- |
| multiplier | UnitMultiplier | The multiplier part of the unit of measure, e.g. "kilo" (k) |
| timeStamp | AbsoluteDateTime | The date and time (if needed) of the summary measurement. |
| unit | UnitSymbol | The units of the reading, e.g. "Wh" |
| value | Float | The value of the summary measurement. |

      - SummaryQuality  «enumeration»

List of codes indicating the quality of the summary.

| Name | Type | Description |
| --- | --- | --- |
| estimated |  |  |
| forecast |  |  |
| mixed |  |  |
| validated |  |  |
| raw |  |  |
| normalizedForWeather |  |  |
| other |  |  |

      - SupplierKind  «enumeration»

Kind of supplier.

| Name | Type | Description |
| --- | --- | --- |
| utility |  |  |
| retailer |  |  |
| other |  |  |
| district |  |  |
| intermediary |  |  |
| local |  |  |
| microgrid |  |  |

      - TariffProfile

A schedule of charges; structure associated with Tariff that allows the definition of complex tariff structures such as step and time of use when used in conjunction with TimeTariffInterval and Charge. Inherited 'status.value' is defined in the context of the utility's business rules, for example: active, inactive, etc.

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

      - UnitMultiplier  «enumeration»

The unit multipliers defined for the CIM

| Name | Type | Description |
| --- | --- | --- |
| p |  | Pico 10**-12 |
| n |  | Nano 10**-9 |
| micro |  | Micro 10**-6 |
| m |  | Milli 10**-3 |
| c |  | Centi 10**-2 |
| d |  | Deci 10**-1 |
| k |  | Kilo 10**3 |
| M |  | Mega 10**6 |
| G |  | Giga 10**9 |
| T |  | Tera 10**12 |
| none |  |  |

      - UnitSymbol  «enumeration»

The units defined for usage in the CIM

| Name | Type | Description |
| --- | --- | --- |
| VA |  | Apparent power in volt ampere |
| W |  | Active power in watt |
| VAr |  | Reactive power in volt ampere reactive |
| VAh |  | Apparent energy in volt ampere hours |
| Wh |  | Real energy in what hours |
| VArh |  | Reactive energy in volt ampere reactive hours |
| V |  | Voltage in volt |
| ohm |  | Resistance in ohm |
| A |  | Current in ampere |
| F |  | Capacitance in farad |
| H |  | Inductance in henry |
| C |  | Relative temperature in degrees Celsius |
| degC |  | Relative temperature in degrees Celsius. In the SI unit system the symbol is ºC. Electric charge is measured in coulomb that has the unit symbol C. To destinguish degree Celsius form coulomb the symbol used in the UML is degC. Reason for not using ºC is the special character º is difficult to manage in software. |
| s |  | Time in seconds |
| min |  | Time in minutes |
| h |  | Time in hours |
| deg |  | Plane angle in degrees |
| rad |  | Plane angle in radians |
| J |  | Energy in joule |
| N |  | Force in newton |
| S |  | Conductance in siemens |
| none |  | Dimension less quantity, e.g. count, per unit, etc. |
| Hz |  | Frequency in hertz |
| g |  | Mass in gram |
| Pa |  | Pressure in pascal (n/m2) |
| m |  | Length in meter |
| m2 |  | Area in square meters |
| m3 |  | Volume in cubic meters |
| thm |  | Energy, in therms |
| m3/h |  | Volumetric flow, in cubic meters per hour |
| ft3/h |  | Volumetric flow, in cubic feet per hour |

      - UsagePoint

Logical point on a network at which consumption or production is either physically measured (e.g. metered) or estimated (e.g. unmetered street lights).

| Name | Type | Description |
| --- | --- | --- |
| name | String | The name is any free human readable and possibly non unique text naming the object. |

      - UsageSummary

Summary of usage for a billing period

| Name | Type | Description |
| --- | --- | --- |
| billingPeriod | DateTimeInterval | The billing period to which the included measurements apply |
| billLastPeriod | Float | The amount of the bill for the previous period |
| billToDate | Float | The bill amount related to the billing period as of the date received |
| costAdditionalLastPeriod | Float | Additional charges from the last billing period |
| currency | String | The ISO 4217 code indicating the currency applicable to the bill amounts in the summary. See list at http://www.unece.org/cefact/recommendations/rec09/rec09_ecetrd203.pdf |
| currentBillingPeriodOverAllConsumption | SummaryMeasurement | The total consumption for the billing period |
| currentDayLastYearNetConsumption | SummaryMeasurement | The amount of energy consumed one year ago |
| currentDayNetConsumption | SummaryMeasurement | Net consumption for the current day (delivered - received) |
| currentDayOverallConsumption | SummaryMeasurement | Overall energy consumption for the current day |
| peakDemand | SummaryMeasurement | Peak demand recorded for the current period |
| previousDayLastYearOverallConsumption | SummaryMeasurement | The amount of energy consumed on the previous day one year ago |
| previousDayNetConsumption | SummaryMeasurement | Net consumption for the previous day |
| previousDayOverallConsumption | SummaryMeasurement | The total consumption for the previous day |
| qualityOfReading | QualityOfReading | Indication of the quality of the summary readings |
| ratchetDemand | SummaryMeasurement | The current ratchet demand value for the ratchet demand period |
| ratchetDemandPeriod | DateTimeInterval | The period over which the ratchet demand applies |

#### Organisation receiving services from ServiceSupplier.

#### Agreement between the Customer and the ServiceSupplier to pay for service at a specific ServiceLocation. It specifies the type of service provided at the ServiceLocation and the rate, which determines how the customer bill is calculated.

#### Interval of date and time.

#### Data captured over a specific interval of time. If not specified, the duration is the intervalLength of the associated ReadingType, where the full definition of the units of measure is located.

#### Physical asset that performs the metering role of the ServiceDeliveryPoint. Used for measuring consumption and detection of events.

#### Set of values obtained from the meter.

#### List of codes indicating the quality of the reading

#### Specific value measured by a meter or other asset. Each Reading is associated with a specific ReadingType and was taken at the specified time.

#### Kind of reading.

#### Quality of a specific reading value or interval reading value. If not present, then is assumed to be the defaultQuality indicated by the ReadingType.

#### Type of data conveyed by a specific Reading.

#### Logical point on the network where the ownership of the service changes hands. It is one of potentially many service points within a ServiceLocation, delivering service in accordance with a CustomerAgreement. Used at the place where a meter may be installed.

#### An aggregated summary measurement reading.

#### A schedule of charges; structure associated with Tariff that allows the definition of complex tarif structures such as step and time of use when used in conjunction with TimeTariffInterval and Charge. Inherited 'status.value' is defined in the context of the utility's business rules, for example: active, inactive, etc.

#### The unit multipliers defined for the CIM

#### d for usage in the CIM

#### Summary of usage for a billing period

#### Kind of service.

#### arty service provider. [OpenADE Extension - Specialization of "Agreement"]

#### Category of service provided to the customer.

#### Kind of supplier.

#### Organisation that provides services to Customers.

- 4.  SUPPORTING DOCUMENTATION

- Description of Request:

- Description of Recommendation:

- Business Purpose:

- Commentary/Rationale of Subcommittee(s)/Task Force(s):

NAESB Process for Standards Development - http://www.naesb.org//misc/naesb_process_for_standards_dev.doc

See e.g. EIS Alliance Use Cases v2 including UC-9, UC-11, UC-12, UC-14, UC-15, ZBHP_SE_MRD 3.1 "HAN Device Information Retrieval Request", and OpenADE use case “Publication”.

See e.g. EIS Alliance Use Cases v2 including UC-3, UC-11, UC-14, UC-15, and OpenADE use case “Publication”

Needs refs. This is an addition to EIS and OpenADE.

See e.g. EIS Alliance Use Cases v2 including UC-1, UC-2, UC-3, UC-8, UC-9.

See e.g. EIS Alliance Use Cases v2 including UC-1, UC-2, UC-3

This is but one of several views that might result from choices permitted in the context of the conformance paragraph.