1.	RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| X | Accept as requested | X | Change to Existing Practice |
| --- | --- | --- | --- |
| X | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

2.	TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
|  | Initiation |  | Initiation |
| X | Modification | X | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
| X | Principle | X | Principle |
| X | Definition | X | Definition |
| X | Business Practice Standard | X | Business Practice Standard |
|  | Document |  | Document |
|  | Data Element |  | Data Element |
|  | Code Value |  | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
|  | Business Process Documentation |  | Business Process Documentation |

3.	RECOMMENDATION

SUMMARY:

The business activity diagrams and use cases presented in these Business Practice Standards illustrate the standard interactions between a System Operator and various Market Participants for the administration and deployment of demand response resources in organized wholesale electric markets.

Recommended Standards:

In response to NIST’s Priority Action Plan 9, this recommendation contains draft specifications, in the form of business activity diagrams, use cases and data elements for each interaction, to support the standardization of the information exchanged during interactions between the System Operator and various Market Participants for the administration and deployment of demand response resources in organized wholesale electric markets. Common terminology from the NAESB Measurement and Verification Standards for Demand Response has been incorporated into the development of the business activity diagrams and use cases. As a result of the development of these Business Practice Standards, a standard set of actors and additional terminology will expand the existing NAESB documentation of associated terms and definitions for demand response.

WEQ-018

Recommended Standards:

# Executive Summary

# Specifications for Wholesale Standard Demand Response Signals

The following addresses the business objectives and context for standardizing signals for DR and DER as part of the Smart Grid implementation, which is called for by NIST PAP 09.    This Business Practice Standard and data elements included herein are not intended to (i) require the use of any of these data elements, (ii)require the provision or purchase of any products, programs, rates markets or services, or (iii) supercede applicable Governing Documents, and in the event of a conflict, the latter documents shall have precedence over this standard.

The business activity diagrams and use cases contained in this Business Practice Standard address the standardizing of the information exchanged during interactions between the System Operator and various Market Participants for the administration and deployment of demand response resources in organized wholesale electric markets. To maintain consistency a master list of data elements and a common set of actors, terms and definitions have been adopted.

The following describes the end-to-end business activity diagrams relating to the wholesale market interactions between the System Operator (SO) and various Market Participants common to all ISOs/RTOs that offer opportunities for demand resources to participate in organized wholesale electric markets.

The intent is to clearly specify the business information that may be necessary to be exchanged to make effective and reasonable business decisions, and standards or implementations that do not directly use such schemas and element names may conform to these requirements through clearly defined mapping documentation.

# Introduction

# Specifications for Wholesale Standard Demand Response Signals

Purpose:

The purpose of the following is to provide standards developers with a context for understanding the range of interactions between wholesale electricity market System Operators and Market Participants across various electricity systems and jurisdictions in the United States of America with some overlaps to Canada. A major objective in producing these Business Practice Standards is to emphasize the importance of interoperability at all levels of the GWAC interoperability framework.

The use cases included in these Business Practice Standards are not to be required or exhaustive and are provided for informational purposes.

Scope:  There are certain principles and boundaries that have been established.  These include:

- Demand Response practices must be consistent with NERC and applicable regional reliability authority requirements.

- All involved entities are registered through the applicable ISO/RTO market participant registration process, which may include credit checks.  However, the specifics related to the business processes associated with Market Participant registration are not documented in these requirements.

- Settlement input parameters are defined as an output to the measurement and performance business process.  However, specific business processes associated with settlements are not documented in these requirements.

- Intra-system operator information exchanges and specific market rules, calculations, algorithms, and Performance Evaluation models are excluded.

- Planning functions are not documented in these requirements.  This includes, but is not limited to, long-term load forecasting and transmission planning.

- Capacity auctions, awards processes, and resource certification are not documented in these requirements.

- References in these Recommendations to “Dispatch”, “Markets”, and “Reliability”, are made relative to Demand Response and apply to Demand Response resources only, not to Generation resources.

- Compliance standards for Demand Response resources are determined by the market rules or other Governing Documents and are specific to the product or service and the reliability need being addressed.

Business Practice Standards

# 018-1	Specifications for Wholesale Standard Demand Response Signals

The following describes the wholesale market business activity diagram and provides the necessary business context and reference architecture for use cases between the System Operator and Market Participant.

This Business Practice Standard contains the list of data elements used in Business Practice Standards WEQ-016 and WEQ-018. Specific application of the data elements to a given use case contained in WEQ-016 and WEQ-018 are identified within the appropriate Business Practice Standard.  This Business Practice Standard and data elements included herein are not intended to (i) require the use of any these data elements; (ii) require the provision or purchase of any products, programs, rates, markets or services, or (iii) supersede applicable Governing Documents, and in the event of a conflict, the latter documents shall have precedence over this standard.

# 018-1.1	Conventions

- Glossary terms are capitalized. This convention does not apply to data elements. Capitalization of words within the Short Description or Long Description of a data element refers to the name of another data element within the Master Data Element List, not a NAESB defined term.

- Interactions shown in the business activity diagrams are not required.

018-2	Actors

The actors listed in the table below are specific to this recommendation and are a subset of the actors and related objects defined in WEQ-000. Details on the relationships between the actors and related objects are further defined in Appendix A.

Table 1 - Actor Roles

| Term | Actor ID |
| --- | --- |
| Designated Dispatch Entity | 3.4 |
| Load Serving Entity | 3.2 |
| Meter Authority | 3.6 |
| Scheduling Entity | 3.3 |
| Service Provider | 3.1 |
| System Operator | 2.1 |
| Transmission/Distribution Service Provider | 3.5 |

## 018-1.3	Use Case Overview

The use cases presented in these Business Practice Standards represent a combination of three dimensions of demand response participation in wholesale markets: product, deployment, and performance evaluation method.  As shown in the table below, product and deployment dimensions are identified by a single alphabetic character and performance evaluation methods are indicated by a numeric identifier; each character is separated by a dash (-). For example, use case E-R-1 refers to the use case for the energy (economic) product with a resource-specific deployment and performance evaluated using a Baseline.

The use cases included in this Business Practice Standard are not to be required or exhaustive and are provided for informational purposes.

Table 2 - Use Case Dimensions

| Product |  |
| --- | --- |
| Energy (Economic) | E |
| Energy (Reliability) | R |
| Capacity | C |
| Reserve | V |
| Regulation | G |
| Deployment |  |
| Bulk | B |
| Resource | R |
| Self | S |
| Performance Evaluation |  |
| Baseline | 1 |
| MB/MA | 2 |
| MBL | 3 |
| MGO | 4 |

A sample of the use case list is shown in the table below.  Appendix B contains the complete list of valid use case combinations.

Table 3 - Sample Use Case List

The Figure 1 illustrates the business activity diagrams for use cases combinations that are considered to be representative of the types of demand response in organized wholesale electricity markets today. Each business activity diagram description includes a table of the use cases to which the business activity diagram applies, followed by the specific data elements that are exchanged in each numbered interaction shown in the business activity diagram.

The wholesale Demand Response end-to-end business activity diagram for the interactions between the SO and the Service Provider SP is comprised of four major functions:

- 1.0 Enrollment & Qualification,

- 2.0 Scheduling & Award Notification,

- 3.0 Deployment & Real-Time Communications,

- 4.0 Measurement & Performance.

Two additional activities are shown in the high-level business process flow, but are considered outside the scope of what is required for the interactions flow for demand response:

- Market Participant Registration

- Settlements

Figure 1 - Demand Response

Business activity diagrams are numbered using the sequence shown in Figure 1. If a business process is further specified, the second position in the number sequence indicates this specification, as in 2.1 – Scheduling and Award Notification – Economic and 2.2 – Scheduling and Award Notification – Reliability.

Interactions between the System Operator and the Market Participant are numbered for each business activity diagram. To continue with the example above, 2.1.1 – Offer Parameters, is the first interaction between the System Operator and Market Participant in the business activity diagram 2.1 – Scheduling and Award Notification – Economic.

A table containing the data elements used in the interactions of the business activity diagram is shown. The numbers of the interactions in the table correspond to the numbered interactions in the business activity diagram. Only data elements with at least one active interaction will be displayed in the tables. Any data elements identified in the Master Data Element List but not currently used in one of the interactions in this document were agreed upon to be included in the Master Data Element List to support future market needs.

Cells showing an “M” (mandatory) are data elements that must contain some value when included in the interaction message. Cells showing an “O” (optional) are data elements that may be left blank if not used in a given interaction.

## 018-1.4	Specific Use Cases

#### 018-1.4.1	Enrollment and Qualification

Overview: 	The Enrollment and Qualification process documents the steps required to enroll a Resource in a wholesale demand response program.

Use Cases: 	All valid use cases defined in these Business Practice Standards (see Figure 2)

- The process begins when the SP submits an enrollment request to the SO.

- The SO processes the enrollment request, which may include interactions with the TDSP, LSE, MA and/or SE.

- The SO evaluates the enrollment request, which may include verification information from the TDSP, LSE, MA and/or SE.

- The result of the evaluation is the approval or rejection of the enrollment request.

  - If the enrollment request has been rejected, the SO sends information to the SP, indicating the rejection details.

  - If the enrollment request has been approved, a determination is made by the SO as to whether qualification of the Resource’s capability is required prior to final approval.

    - If no qualification is required, the SO sends information to the SP, indicating the approval of the enrollment.

    - If qualification is required, the SO coordinates the qualification procedure with the SP.

      - If the Resource has failed to qualify, the SO sends information to the SP, indicating the qualification rejection details.

      - If the Resource achieves qualification, the SO sends information to the SP and enrollment is finalized.

- At the end of the Enrollment and Qualification process, the SO finalizes enrollment, notifies the SP and may also notify the TDSP, LSE, MA and/or SE.

Activity Diagram and Data Flow:

Figure 2 - Enrollment and Qualification

Table 4. Data Elements by Interaction Number 1.0.1a through 1.0.9: Enrollment and Qualification

The data elements for Enrollment and Qualification are split into two tables because the number of interactions in this business activity are too numerous to fit into a single table. Some data elements shown in one table may not display an “M” or an “O” for any interactions because they will be shown in interactions that show in the second table for Enrollment and Qualification. Only data elements with at least one active interaction will be displayed in the tables.

| Legend: Cells showing an “M” (mandatory) are data elements that must contain some value when included in the interaction message. Cells showing an “O” (optional) are data elements that may be left blank if not used in a given interaction. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Process | 1.0 Enrollment & Qualification |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | From | SP |  |  | SO |  |  |  | SE | MA | LSE | TDSP |
|  |  |  |  | To | SO |  |  | SE | MA | LSE | TDSP | SO |  |  |  |
|  |  |  |  |  | 1.0.1 a | 1.0.1 b | 1.0.1 c | 1.0.2 | 1.0.3 | 1.0.4 | 1.0.5 | 1.0.6 | 1.0.7 | 1.0.8 | 1.0.9 |
|  |  |  |  | Interaction | Service Location Enrollment | Asset Group Enrollment | Resource Enrollment | SE Version | MA Version | LSE Version | TDSP Version | SE Enrollment Verification | MA Enrollment Verification | LSE Enrollment Verification | TDSP Enrollment Verification |
| Element |  |  |  | Short Description |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  | Common |  |  |  |  |  |  |  |  |  |  |  |  |
| - 0 | - 10 |  | - Submittal Date | - Timestamp for the sender's use | - O | - O | - M | - O | - O | - O | - O |  |  |  |  |
| - 0 | - 11 |  | - Submitted By | - User ID of submitter | - O | - O | - M | - O | - O | - O | - O |  |  |  |  |
| - 0 | - 12 |  | - Submitted Error | - ID of submission error detected | - O | - O | - M | - O | - O | - O | - O |  |  |  |  |
| - 0 | - 20 |  | - NERC CIP Security - Availability | - CIP Security Classification for Availability | - M | - M | - M | - M | - M | - M | - M | - M | - M | - M | - M |
| - 0 | - 21 |  | - NERC CIP Security - Confidentiality | - CIP Security Classification for Confidentiality | - M | - M | - M | - M | - M | - M | - M | - M | - M | - M | - M |
| - 0 | - 22 |  | - NERC CIP Security - Integrity | - CIP Security Classification for Integrity | - M | - M | - M | - M | - M | - M | - M | - M | - M | - M | - M |
| 1 |  |  | General Asset/Resource |  |  |  |  |  |  |  |  |  |  |  |  |
| - 1 | - 10 |  | - Service Location ID | - Identifier assigned to the Service Location | - M | - M | - M | - M | - M | - M | - M | - M | - M | - M | - M |
| - 1 | - 11 |  | - Service Location Name | - Name of the Service Location | - O | - O | - M | - M | - O | - O | - O | - M | - O | - O | - M |
| - 1 | - 12 |  | - Asset ID | - The unique identifier of the asset | - O | - O | - O | - O | - O | - O | - O | - O | - O | - O | - O |
| - 1 | - 13 |  | - Asset Name | - The name of the asset | - O | - O | - O | - O | - O | - O | - O | - O | - O | - O | - O |
| - 1 | - 20 |  | - Resource ID | - Identifier assigned to the Resource | - M | - M | - M | - M | - M | - M | - M | - M | - M | - M | - M |
| - 1 | - 21 |  | - Resource Name | - Name of the Resource | - O | - O | - M | - M | - O | - O | - O | - M | - O | - O | - O |
| - 1 | - 30 |  | - Asset Group ID | - The identifier of a group of assets | - M | - M | - M | - M | - M | - M | - M |  |  |  |  |
| - 1 | - 31 |  | - Asset Group Name | - Name of the aggregated group of assets | - M | - M | - M | - M | - M | - M | - M |  |  |  |  |
| - 1 | - 40 |  | - Gen EMS ID | - Alias or point Identifier assigned to the Resource |  |  | - M |  |  |  |  |  |  |  |  |
| - 1 | - 41 |  | - Gen Billing ID | - Billing Identifier assigned to the Resource |  |  | - M |  |  |  |  |  |  |  |  |
| - 1 | - 50 |  | - Business Segment | - NAICS code of the Service Location | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 1 | - 51 |  | - Batch Load Flag | - Flag set if the Resource is a Batch Load | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 1 | - 991 |  | - General Resource Comments | - General comments associated with the Resource | - O | - O | - O | - O | - O | - O | - O |  |  |  |  |
| 2 |  |  | Location |  |  |  |  |  |  |  |  |  |  |  |  |
| - 2 | - 10 |  | - Address1 | - Address line 1 | - M | - M | - M | - M | - M | - M | - M | - M |  |  | - M |
| - 2 | - 11 |  | - Address2 | - Address line 2 | - O | - O | - O | - O | - O | - O | - O | - O |  |  | - O |
| - 2 | - 12 |  | - City | - City | - M | - M | - M | - M | - M | - M | - M | - M |  |  | - M |
| - 2 | - 13 |  | - State/Province | - State or Province two-letter code | - M | - M | - M | - M | - M | - M | - M | - M |  |  | - M |
| - 2 | - 14 |  | - Zip/Postal Code | - Zip or Postal Code | - M | - M | - M | - M | - M | - M | - M | - M |  |  | - M |
| - 2 | - 15 |  | - Country | - Country | - M | - M | - M | - M | - M | - M | - M | - M |  |  | - M |
| - 2 | - 20 |  | - GPS Coordinates | - Latitude and longitude | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 2 | - 21 |  | - Weather Station | - Weather Station code associated with the Service Location | - M | - M | - M | - O | - O |  |  |  |  |  |  |
| - 2 | - 22 |  | - TimeZoneName | - Name of the Time Zone in which the Service Location is located | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 2 | - 30 |  | - Zone ID | - Identifier assigned to the Zone in which the Service Location is located | - M | - M | - M | - M | - M | - M | - M |  |  |  |  |
| - 2 | - 31 |  | - Zone | - Name of the Zone in which the Service Location is located | - M | - M | - M | - O | - O | - O | - O |  |  |  |  |
| - 2 | - 32 |  | - Zone Type | - Type of Zone | - M | - M | - M | - O | - O | - O | - O |  |  |  |  |
| - 2 | - 40 |  | - Electrical Node ID | - Identifier assigned to the Electrical Node at which the Service Location is connected | - M | - M | - M | - M | - M | - O | - M |  |  |  |  |
| - 2 | - 41 |  | - Electrical Node Name | - Name of the Electrical Node at which the Service Location is connected | - M | - M | - M | - O | - O | - O | - O |  |  |  | - M |
| - 2 | - 42 |  | - Electrical Node Type | - Type of Electrical Node at which the Service Location is attached | - M | - M | - M | - M | - M | - M | - M |  |  |  | - M |
| - 2 | - 43 |  | - PNode | - Name of the Price Node associated with the Service Location | - O | - O | - O | - O | - O | - O | - O |  |  |  |  |
| - 2 | - 44 |  | - PNode ID | - Identifier assigned to the Price Node associated with the Service Location | - M | - M | - M | - M | - M | - M | - M | - M |  |  | - M |
| - 2 | - 50 |  | - Competitive Choice Area | - Flag set if the Service Location is in an area with Competitive Choice | - O | - O | - O | - O | - O | - O | - O | - O |  |  | - O |
| - 2 | - 51 |  | - Asset Multiplier | - Number of identical assets within the Service Location | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 2 | - 60 |  | - NERC Control Area | - The NERC control area of the resource | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 2 | - 71 |  | - Connection Type | - Additional type of connection associated with the Service Location/Resource | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 2 | - 72 |  | - Connection Address | - Address associated with the Connection Type | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 2 | - 991 |  | - Location Comments | - Comments associated with the Service Location | - O | - O | - O | - O | - O | - O | - O |  |  |  |  |
| 3 |  |  | Contact |  |  |  |  |  |  |  |  |  |  |  |  |
| - 3 | - 10 |  | - Contact Type | - Interest code associated with a Contact | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 3 | - 11 |  | - Contact Priority | - Order in which this Contact should be selected | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 3 | - 12 |  | - DUNS Number | - DUNS business number | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 3 | - 13 |  | - Third Party | - Flag set if the Contact is a third-party entity | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 3 | - 20 |  | - Title | - Title of the Contact | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 3 | - 21 |  | - First Name | - First Name of the Contact | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 3 | - 22 |  | - Last Name | - Last Name of the Contact | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 3 | - 23 |  | - Middle Name | - Middle Name of the Contact | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 3 | - 30 |  | - Contact Method | - Method of communication with the Contact | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 3 | - 31 |  | - Contact Address Data | - Method-dependent Address of the Contact | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 3 | - 32 |  | - Contact Method Priority | - Order in which a Communication Method should be selected for a Contact | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 3 | - 991 |  | - Contact Comments | - Comments associated with the Contact | - O | - O | - O |  |  |  |  |  |  |  |  |
| 4 |  |  | Grid Connection |  |  |  |  |  |  |  |  |  |  |  |  |
| - 4 | - 10 |  | - Loss Factor Type | - Type of Loss Factor | - M | - M | - M | - M | - M |  |  |  |  |  | - O |
| - 4 | - 11 |  | - Loss Factor Value | - Loss Factor | - M | - M | - M | - M | - M |  |  |  |  |  | - O |
| - 4 | - 12 |  | - Connect Voltage | - Voltage level | - M | - M | - M | - O | - O | - O | - O |  |  |  |  |
| - 4 | - 991 |  | - Grid Connection Comments | - Comments associated with the Attributes of the Resource | - O | - O | - O |  |  |  |  |  |  |  |  |
| 5 |  |  | Business Entity Relationships |  |  |  |  |  |  |  |  |  |  |  |  |
| - 5 | - 11 |  | - Service Provider ID | - Identifier assigned to the Service Provider | - M | - M | - M | - M | - M |  |  | - M |  |  |  |
| - 5 | - 12 |  | - Service Provider Name | - Name of the Service Provider | - M | - M | - M | - M | - M |  |  | - O |  |  |  |
| - 5 | - 20 |  | - Transmission/Distribution Service Provider ID | - Identifier assigned to the Transmission/Distribution Service Provider | - M | - M | - M | - M | - M |  |  | - M |  |  |  |
| - 5 | - 21 |  | - Transmission/Distribution Service Provider Name | - Name of the Transmission/Distribution Service Provider | - M | - M | - M | - M | - M |  |  | - O |  |  | - M |
| - 5 | - 22 |  | - Transmission/Distribution Service Provider Account Number | - Transmission/Distribution Service Provider's account number for the Resource | - M | - M | - M | - M | - M | - O | - O |  |  |  | - O |
| - 5 | - 30 |  | - Load Serving Entity ID | - Identifier assigned to the Load Serving Entity | - M | - M | - M | - M | - M | - O | - O |  |  |  |  |
| - 5 | - 31 |  | - Load Serving Entity Name | - Name of the Load Serving Entity | - M | - M | - M | - M | - M | - O | - O |  |  |  |  |
| - 5 | - 40 |  | - Retail Rate ID | - Identifier assigned to the Retail Rate | - M | - M | - M |  |  | - O |  |  |  |  |  |
| - 5 | - 41 |  | - Retail Rate Code | - Code representing the Retail Rate for the Service Location | - O | - O | - O |  |  | - O |  |  |  | - O |  |
| - 5 | - 42 |  | - Retail Rate Name | - Name of the Retail Rate for the Service Location | - M | - M | - M |  |  | - O |  |  |  | - M |  |
| - 5 | - 43 |  | - Retail Rate Description | - Description of the Retail Rate | - O | - O | - O |  |  | - O |  |  |  | - O |  |
| - 5 | - 44 |  | - Retail Rate | - Retail Rate | - M | - M | - M |  |  | - O |  |  |  | - M |  |
| - 5 | - 60 |  | - Meter Authority ID | - Identifier assigned to the Metering Authority | - M | - M | - M | - M | - M |  |  | - M |  |  | - M |
| - 5 | - 61 |  | - Meter Authority Name | - Name of the Metering Authority | - M | - M | - M | - M | - M |  |  | - O |  |  | - O |
| - 5 | - 70 |  | - Scheduling Entity ID | - Identifier assigned to the Scheduling Entity | - M | - M | - M | - M |  |  |  | - M |  |  | - M |
| - 5 | - 71 |  | - Scheduling Entity Name | - Name of the Scheduling Entity | - M | - M | - M | - M |  |  |  | - O |  |  | - O |
| - 5 | - 80 |  | - Designated Dispatch Entity ID | - Identifier assigned to the Designated Dispatch Entity | - M | - M | - M | - M |  |  |  | - M |  |  | - M |
| - 5 | - 81 |  | - Designated Dispatch Entity Name | - Name of the Designated Dispatch Entity | - M | - M | - M | - M |  |  |  | - O |  |  | - O |
| - 5 | - 991 |  | - External Entity Comments | - Comments associated with the External Entity | - O | - O | - O |  |  |  |  |  |  |  |  |
| 6 |  |  | Device and Qualification |  |  |  |  |  |  |  |  |  |  |  |  |
| - 6 | - 10 |  | - Meter Configuration | - Configuration of the Meter at the Service Location | - M | - M | - M | - M |  |  |  |  |  |  |  |
| - 6 | - 11 |  | - Parent Meter ID | - ID of a Master or Parent Meter | - O | - O | - O | - O | - O |  |  |  |  |  |  |
| - 6 | - 20 |  | - Meter ID | - Identifier assigned to the Meter | - M | - M | - M | - M | - M | - M | - M | - M |  |  | - M |
| - 6 | - 21 |  | - Meter Type | - Type of Meter installed at the Service Location | - M | - M | - M | - M |  |  |  |  |  |  |  |
| - 6 | - 22 |  | - Meter Manufacturer | - Manufacturer of the Meter | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 221 |  | - Meter Installation Date | - Date of Installation of the Meter | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 222 |  | - PT Ratio | - Ratio of the Potential Transformer | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 223 |  | - Meter Installer License | - Professional License Number of the Installer of the Meter | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 224 |  | - Meter Correction Factors | - Device-specific Factors used to adjust for atmospheric changes at the point of installation of the Meter | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 225 |  | - Meter Test Criteria | - Test Criteria required to qualify the installation of the Meter | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 226 |  | - Meter Test Frequency | - Frequency of Tests of the Meter | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 227 |  | - Meter Device QA Plan | - Quality Assurance plan for maintenance and testing of the Meter | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 228 |  | - Date of Last Meter Test | - Date of Last Test of the Meter | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 229 |  | - Meter Qualification Date | - Date of Qualification of the Meter by the Metering Authority or System Operator | - M | - M | - M | - O | - M |  |  |  |  |  |  |
| - 6 | - 2291 |  | - Meter Test Results | - Results of Tests of the Meter | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 2292 |  | - VEE Compliance | - Compliance standard for Validation, Editing, and Estimation | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 2293 |  | - Measurement Interval | - Interval of time between Measurement readings | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 2294 |  | - ANSI Compliance | - ANSI standard with which the Meter complies | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 2295 |  | - Meter Owner | - Owner of the Meter | - M | - M | - M |  |  |  |  |  |  |  | - O |
| - 6 | - 2296 |  | - Meter Asset Comments | - Comments associated with the Meter Asset | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 23 |  | - Meter Model | - Model of the Meter | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 24 |  | - Meter Rating | - Load Rating of the Meter | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 25 |  | - Meter Multiplier (kH) | - Multiplier used to convert pulses into power units | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 26 |  | - Meter Accuracy Class | - Accuracy Class of the Meter | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 27 |  | - Meter Loss Compensation | - Line Losses included in the Meter | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 28 |  | - Maximum Error | - Error of the Meter, including end-to-end Maximum Error | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 29 |  | - Meter Phase | - Specific Phase information | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 291 |  | - CT Ratio | - Ratio of the Current Transformer | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 30 |  | - Distributed Generator Type | - Type of Distributed Generator | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 31 |  | - Nameplate Rating | - Manufacturer's output rating of the Distributed Generator | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 32 |  | - Distributed Generator Fuel Type | - Type of Fuel consumed by the Distributed Generator | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 33 |  | - Distributed Generator Permit Type | - Type of environmental authorization required to operate the Distributed Generator | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 34 |  | - Distributed Generator Manufacturer | - Manufacturer of the Distributed Generator | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 35 |  | - Manufactured Date of the Distributed Generator | - Date of Manufacture of the Distributed Generator | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 36 |  | - Installation Date of Distributed Generator | - Date of Installation of the Distributed Generator | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 37 |  | - In-Service Date | - Date the Distributed Generator became operational | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 38 |  | - Normal Load Rating | - Average load picked up by the Distributed Generator during the applicable performance hours | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 39 |  | - Distributed Generator Name | - Name of the Distributed Generator | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 41 |  | - Interconnection Agreement Type | - Type of Interconnection Agreement between the Distributed Generator or Service Location and the TDSP | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 42 |  | - Interconnection Limits | - Limits associated with the Interconnection of the Distributed Generator or Service Location | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 43 |  | - Capable of Synchronizing to Grid | - Flag set if the Service Location is capable of Synchronizing to the Grid | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 44 |  | - Normal Breaker Status | - Status of the Breaker for the Service Location under Normal operating conditions | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 45 |  | - Breaker Status During Event | - Status of the Breaker for the Service Location during an Emergency Event | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 46 |  | - Wholesale Delivery Point Status | - Flag set if the Service Location is metered at the Wholesale level | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 47 |  | - Private Use Network | - Flag set if the Resource is an Electric Network connected to the transmission grid | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 51 |  | - UFR Settings | - Setting of the Under-Frequency Relay | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 6 | - 52 |  | - Load Resource Control Device | - Type of Control Device at a Service Location or deployed by a Response Method | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 6 | - 991 |  | - Distributed Generator Comments | - Comments associated with the Distributed Generator | - O | - O | - O |  |  |  |  |  |  |  |  |
| 7 |  |  | Market/Program Enrollment |  |  |  |  |  |  |  |  |  |  |  |  |
| - 7 | - 10 |  | - Program ID | - Program Identifier | - M | - M | - M | - M | - M | - M | - M | - M | - M | - M | - M |
| - 7 | - 11 |  | - Program Name | - Name of the Program | - O | - O | - O | - O | - O | - O | - O | - O | - O | - O | - O |
| - 7 | - 12 |  | - Market | - Type of wholesale market | - M | - M | - M | - M | - M | - M | - M |  |  |  |  |
| - 7 | - 13 |  | - Market Product | - Market Product | - M | - M | - M | - M | - M | - M | - M | - M |  |  | - M |
| - 7 | - 21 |  | - Effective Enrollment Date | - Effective Start Date for the Enrollment | - M | - M | - M | - O | - O | - O | - O | - M |  |  |  |
| - 7 | - 22 |  | - Enrollment Status | - Status of the Enrollment for the Service Location or Resource | - M | - M | - M | - O | - O | - O | - O | - M |  |  |  |
| - 7 | - 23 |  | - Resource Type | - Type of Resource | - M | - M | - M | - M | - M | - M | - M | - M |  |  | - M |
| - 7 | - 24 |  | - Resource Qualification Test Date | - Date the Resource demonstrated its ability to deliver a product or service | - M | - M | - M | - M |  |  |  | - M |  |  | - O |
| - 7 | - 25 |  | - Enrollment End Date | - Date of Termination of Enrollment | - M | - M | - M | - M | - M | - M | - M | - M |  |  | - O |
| - 7 | - 26 |  | - Requalification Test Date | - Date the Resource will retest its ability to deliver a product or service | - M | - M | - M | - M |  |  |  | - M |  |  | - O |
| - 7 | - 30 |  | - Lead Time | - Time between the advanced notification and deployment | - O | - O | - O | - O | - O |  |  |  |  |  |  |
| - 7 | - 31 |  | - Day Ahead Flag | - Flag set if the Resource will be participating in the Day-Ahead market | - M | - M | - M | - M | - M |  |  |  |  |  |  |
| - 7 | - 32 |  | - Real Time Flag | - Flag set if the Resource will be participating in the Real-Time market | - M | - M | - M | - M | - M |  |  |  |  |  |  |
| - 7 | - 33 |  | - Self-Schedule Flag | - Flag set if the Resource is Self-Deploying | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 7 | - 40 |  | - Response Method Type | - Type of Response Method | - M | - M | - M | - O | - O | - O | - O | - O |  |  |  |
| - 7 | - 41 |  | - Response Method ID | - Identifier assigned to the Response Method | - M | - M | - M | - M | - M | - M | - M |  |  |  |  |
| - 7 | - 42 |  | - Response Method Name | - Name of the Response Method | - O | - O | - O | - O | - O | - O | - O |  |  |  |  |
| - 7 | - 43 |  | - Response Method Value | - Value of the Response Method | - M | - M | - M | - M | - O | - O | - O | - M |  |  |  |
| - 7 | - 44 |  | - Verified Capability | - Audited Capability | - O | - O | - O | - M |  |  |  | - M |  |  |  |
| - 7 | - 45 |  | - Verified Capability Factor | - Ratio of the Verified Capability to the qualified capability | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 7 | - 50 |  | - Performance Evaluation Method Type Code | - Code representing the Type of Measurement | - M | - M | - M | - M | - O |  |  | - M |  |  |  |
| - 7 | - 51 |  | - Performance Evaluation Method | - Method used to Evaluate the Performance of a Resource | - O | - O | - O | - O | - O |  |  |  |  |  |  |
| - 7 | - 991 |  | - Market Enrollment Comments | - Comments associated with the Enrollment | - M | - M | - M | - O | - O | - O | - O | - O |  |  |  |
| 8 |  |  | Offer Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| - 8 | - 10 |  | - Offer Limit Value | - Offer Limit Value | - M | - M | - M | - M | - O |  |  | - M |  |  |  |
| - 8 | - 11 |  | - Offer Limit Type | - Type of Offer Limit | - M | - M | - M | - M | - O |  |  | - M |  |  |  |
| - 8 | - 12 |  | - Offer Limit Interval | - Offer Limit Interval | - M | - M | - M | - M | - O |  |  | - M |  |  |  |
| - 8 | - 20 |  | - Physical Min Gen | - Minimum Operating Level of a Resource | - M | - M | - M | - O |  |  |  | - O |  |  | - O |
| - 8 | - 21 |  | - Min Gen MW | - Minimum MW available for dispatch | - M | - M | - M | - O | - O |  |  | - O |  |  | - O |
| - 8 | - 22 |  | - Min Gen Cost | - The cost per hour for each Min Gen MW value. | - M | - M | - M | - O | - O |  |  | - O |  |  | - O |
| - 8 | - 23 |  | - Ramp Rate Type | - Type of Ramp Rate | - M | - M | - M | - M |  |  |  | - M |  |  |  |
| - 8 | - 24 |  | - Ramp Rate Segment | - Energy of Segment of the Ramp Rate | - M | - M | - M | - M |  |  |  | - M |  |  |  |
| - 8 | - 25 |  | - Ramp Rate Direction | - Direction of the selected Ramp Rate Type and Ramp Rate Segment | - M | - M | - M | - M |  |  |  | - M |  |  |  |
| - 8 | - 26 |  | - Ramp Rate Value | - Ramp Rate associated with the selected Ramp Rate Type for the selected Ramp Rate Segment and Ramp Rate Direction | - M | - M | - M | - M |  |  |  | - M |  |  |  |
| - 8 | - 30 |  | - Offer Dispatch Type | - Type of Offer | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 8 | - 31 |  | - Offer Segment MW | - Energy of Segment of the Offer | - O | - O | - M |  |  |  |  |  |  |  |  |
| - 8 | - 32 |  | - Offer Segment Price | - Price of Segment of the Offer | - O | - O | - M |  |  |  |  |  |  |  |  |
| - 8 | - 33 |  | - Offer Segment Hour | - Hour of Segment of the Offer | - O | - O | - M |  |  |  |  |  |  |  |  |
| - 8 | - 34 |  | - Startup Cost | - Cost of starting a response activity | - O | - O | - M |  |  |  |  |  |  |  |  |
| - 8 | - 35 |  | - Strike Price | - Threshold Price | - O | - O | - M |  |  |  |  |  |  |  |  |
| - 8 | - 36 |  | - Offer Expiration Date | - Date of Expiration of the Offer | - O | - O | - M |  |  |  |  |  |  |  |  |
| - 8 | - 37 |  | - Startup Cost Type | - Type of startup cost | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 8 | - 38 |  | - Offer Commit Status | - Commitment status of offer | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 8 | - 39 |  | - Offer Dispatch Status | - Dispatch Status of Offer | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 8 | - 331 |  | - Offer Price Curve Slope | - Flag to smooth offer segments from step function to slope | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 8 | - 332 |  | - Strike Price Type | - Type of strike price | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 8 | - 40 |  | - Market Clearing Day | - Market Clearing Day of the Offer | - O | - O | - O | - O |  |  |  |  |  |  |  |
| - 8 | - 41 |  | - Schedule Name | - Name or tag of the Offer | - O | - O | - O | - O |  |  |  |  |  |  |  |
| - 8 | - 42 |  | - Schedule Description | - Description of the Offer | - O | - O | - O | - O |  |  |  |  |  |  |  |
| - 8 | - 43 |  | - Schedule Status | - Status of the Offer | - M | - M | - M | - O |  |  |  |  |  |  |  |
| - 8 | - 50 |  | - Portfolio Name | - Name of an aggregation of Resources for market participation | - O | - O | - O | - O |  |  |  |  |  |  |  |
| - 8 | - 71 |  | - Constraint Type | - Type of operational, schedule or offer constraint | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 8 | - 72 |  | - Constraint Interval | - The timeframe over which the constraint type applies. | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 8 | - 73 |  | - Constraint Value | - Value of the Constraint Type and Interval | - M | - M | - M |  |  |  |  |  |  |  |  |
| 9 |  |  | Energy Market |  |  |  |  |  |  |  |  |  |  |  |  |
| - 9 | - 0 |  | - Instruction ID | - Unique identifier assigned to each instruction. | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 9 | - 15 |  | - Schedule ID | - Unique identifier assigned to each schedule. | - M | - M | - M |  |  |  |  |  |  |  |  |
| 10 |  |  | Ancillary Service Market |  |  |  |  |  |  |  |  |  |  |  |  |
| - 10 | - 10 |  | - Ancillary Service Product Type | - Type of Ancillary Service product(s) the Resource is enrolling to provide | - M | - M | - M |  |  |  |  |  |  |  |  |
| - 10 | - 11 |  | - Have Engineering Diagram | - Flag set if One-Line Diagrams have been submitted | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 10 | - 20 |  | - Ancillary Service Product Qualification Type | - Type of Qualification | - M | - M | - M | - M |  |  |  | - M |  |  |  |
| - 10 | - 21 |  | - Synchronization Test Date | - Target Date of Test of Synchronization to the grid | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 10 | - 22 |  | - Ancillary Service Product Type Qualification Date | - Date of Qualification of the Resource | - M | - M | - M | - M |  |  |  | - M |  |  |  |
| - 10 | - 23 |  | - Ancillary Service Product Type Qualified | - Type of Ancillary Service Product(s) for which the Resource has Qualified | - M | - M | - M | - M |  |  |  | - M |  |  |  |
| - 10 | - 24 |  | - Requalification Requirement | - Flag set if Requalification is Required | - O | - O | - O |  |  |  |  |  |  |  |  |
| 11 |  |  | Capacity Market |  |  |  |  |  |  |  |  |  |  |  |  |
| - 11 | - 10 |  | - Capacity Type | - Type of Capacity | - M | - M | - M | - O | - O |  |  |  |  |  |  |
| - 11 | - 11 |  | - Capacity Type Description | - Description of the Type of Capacity | - O | - O | - O | - O | - O |  |  |  |  |  |  |
| - 11 | - 20 |  | - Peak Value | - Measurement of Peak | - M | - M | - M | - O | - O |  |  |  |  |  | - M |
| - 11 | - 21 |  | - Peak Type | - Type of measurement of Peak | - M | - M | - M | - O | - O |  |  |  |  |  | - M |
| - 11 | - 22 |  | - Peak Date/Time | - Date and Time of measurement of Peak | - M | - M | - M | - O | - O |  |  |  |  |  | - M |
| - 11 | - 23 |  | - Nominal Capacity | - Nominated load | - M | - M | - M | - O | - O |  |  |  |  |  |  |
| - 11 | - 24 |  | - Qualified Capacity | - Nominal installed capacity (ICAP) derated for performance | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 11 | - 30 |  | - Capacity Obligation Period Type | - Type of Capacity Obligation Time Period within which the Resource is obligated | - M | - M | - M | - O | - O |  |  |  |  |  |  |
| - 11 | - 31 |  | - Capacity Obligation Period | - Period of time for which the Capacity Resource is obligated | - M | - M | - M | - O | - O |  |  |  |  |  |  |
| - 11 | - 40 |  | - Minimum Payment Nomination | - Strike price for the economic dispatch of the energy component of the capacity program | - O | - O | - O |  |  |  |  |  |  |  |  |
| - 11 | - 50 |  | - Capacity Reference ID | - Identifier assigned to the External System award | - O | - O | - O | - O | - O |  |  |  |  |  |  |
| - 11 | - 991 |  | - Capacity Comments | - Comments associated with the Capacity market | - O | - O | - O | - O | - O |  |  |  |  |  |  |

Table 5. Data Requirements by Interaction Number 1.0.10 through 1.0.16: Enrollment and Qualification

The data elements for Enrollment and Qualification are split into two tables because the number of interactions in this business activity are too numerous to fit into a single table. Some data elements shown in one table may not display an “M” or an “O” for any interactions because they will be shown in interactions that show in the second table for Enrollment and Qualification. Only data elements with at least one active interaction will be displayed in the tables for Enrollment and Qualification.

| Legend: Cells showing an “M” (mandatory) are data elements that must contain some value when included in the interaction message. Cells showing an “O” (optional) are data elements that may be left blank if not used in a given interaction. |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Process | 1.0 Enrollment & Qualification |  |  |  |  |  |  |
|  |  |  |  | From | SO |  | SO |  |  |  |  |
|  |  |  |  | To | SP |  | SP | SE | MA | LSE | TDSP |
|  |  |  |  |  | 1.0.10 | 1.0.11 | 1.0.12 | 1.0.13 | 1.0.14 | 1.0.15 | 1.0.16 |
|  |  |  |  | Interaction | Rejection Details | Qualification Rejection | Qualification Status to SP | Qualification Status to SE | Qualification Status to MA | Qualification Status to LSE | Qualification Status to TDSP |
| Element |  |  |  | Short Description |  |  |  |  |  |  |  |
| 0 |  |  | Common |  |  |  |  |  |  |  |  |
| - 0 | - 10 |  | - Submittal Date | - Timestamp for the sender's use | - O | - O | - O |  |  | - O | - O |
| - 0 | - 11 |  | - Submitted By | - User ID of submitter | - O | - O | - O |  |  | - O | - O |
| - 0 | - 12 |  | - Submitted Error | - ID of submission error detected | - O | - O | - O |  |  | - O | - O |
| - 0 | - 13 |  | - Rejection Code Type | - Type of rejection message | - M | - M |  |  |  |  |  |
| - 0 | - 14 |  | - Rejection Code Value | - Code referring to the reason for a rejection message | - M | - M |  |  |  |  |  |
| - 0 | - 20 |  | - NERC CIP Security - Availability | - CIP Security Classification for Availability | - M | - M | - M | - M | - M | - M | - M |
| - 0 | - 21 |  | - NERC CIP Security - Confidentiality | - CIP Security Classification for Confidentiality | - M | - M | - M | - M | - M | - M | - M |
| - 0 | - 22 |  | - NERC CIP Security - Integrity | - CIP Security Classification for Integrity | - M | - M | - M | - M | - M | - M | - M |
| 1 |  |  | General Asset/Resource |  |  |  |  |  |  |  |  |
| - 1 | - 10 |  | - Service Location ID | - Identifier assigned to the Service Location | - M | - M | - M | - M | - M | - M | - M |
| - 1 | - 11 |  | - Service Location Name | - Name of the Service Location | - M | - M | - M | - M | - M | - M | - M |
| - 1 | - 12 |  | - Asset ID | - The unique identifier of the asset | - O | - O | - O | - O | - O | - O | - O |
| - 1 | - 13 |  | - Asset Name | - The name of the asset | - O | - O | - O | - O | - O | - O | - O |
| - 1 | - 20 |  | - Resource ID | - Identifier assigned to the Resource | - M | - M | - M | - M | - M | - M | - M |
| - 1 | - 21 |  | - Resource Name | - Name of the Resource | - O | - M | - M | - M | - O | - O | - O |
| - 1 | - 30 |  | - Asset Group ID | - The identifier of a group of assets |  |  | - O | - O | - O | - O | - O |
| - 1 | - 31 |  | - Asset Group Name | - Name of the aggregated group of assets |  |  | - O | - O | - O | - O | - O |
| - 1 | - 40 |  | - Gen EMS ID | - Alias or point Identifier assigned to the Resource |  |  | - M |  |  |  |  |
| - 1 | - 41 |  | - Gen Billing ID | - Billing Identifier assigned to the Resource |  |  | - M |  |  |  |  |
| - 1 | - 51 |  | - Batch Load Flag | - Flag set if the Resource is a Batch Load |  |  | - O |  |  | - O | - O |
| - 1 | - 991 |  | - General Resource Comments | - General comments associated with the Resource | - O | - O | - O |  |  | - O | - O |
| 2 |  |  | Location |  |  |  |  |  |  |  |  |
| - 2 | - 10 |  | - Address1 | - Address line 1 | - M | - M | - O | - M |  | - O | - O |
| - 2 | - 11 |  | - Address2 | - Address line 2 | - O | - O | - O | - O |  | - O | - O |
| - 2 | - 12 |  | - City | - City | - M | - M | - O | - M |  | - O | - O |
| - 2 | - 13 |  | - State/Province | - State or Province two-letter code | - M | - M | - O | - M |  | - O | - O |
| - 2 | - 14 |  | - Zip/Postal Code | - Zip or Postal Code | - M | - M | - O | - M |  | - O | - O |
| - 2 | - 15 |  | - Country | - Country | - M | - M | - O | - M |  | - O | - O |
| - 2 | - 21 |  | - Weather Station | - Weather Station code associated with the Service Location |  |  | - O |  |  | - O | - O |
| - 2 | - 30 |  | - Zone ID | - Identifier assigned to the Zone in which the Service Location is located | - M | - M | - M |  |  | - M | - M |
| - 2 | - 31 |  | - Zone | - Name of the Zone in which the Service Location is located | - O | - O | - O |  |  | - O | - O |
| - 2 | - 32 |  | - Zone Type | - Type of Zone | - O | - O | - O |  |  | - O | - O |
| - 2 | - 40 |  | - Electrical Node ID | - Identifier assigned to the Electrical Node at which the Service Location is connected | - M |  | - M | - M |  | - M | - M |
| - 2 | - 41 |  | - Electrical Node Name | - Name of the Electrical Node at which the Service Location is connected | - O |  | - O |  |  | - O | - O |
| - 2 | - 42 |  | - Electrical Node Type | - Type of Electrical Node at which the Service Location is attached | - O |  | - O |  |  | - O | - O |
| - 2 | - 43 |  | - PNode | - Name of the Price Node associated with the Service Location | - O | - O | - O |  |  | - O | - O |
| - 2 | - 44 |  | - PNode ID | - Identifier assigned to the Price Node associated with the Service Location | - M | - M | - M | - M |  | - M | - M |
| - 2 | - 50 |  | - Competitive Choice Area | - Flag set if the Service Location is in an area with Competitive Choice | - O | - O | - O | - O |  |  | - O |
| - 2 | - 991 |  | - Location Comments | - Comments associated with the Service Location | - O | - O | - O |  |  |  |  |
| 3 |  |  | Contact |  |  |  |  |  |  |  |  |
| - 3 | - 991 |  | - Contact Comments | - Comments associated with the Contact | - O | - O |  |  |  |  |  |
| 4 |  |  | Grid Connection |  |  |  |  |  |  |  |  |
| - 4 | - 10 |  | - Loss Factor Type | - Type of Loss Factor |  |  | - O |  |  | - O | - O |
| - 4 | - 11 |  | - Loss Factor Value | - Loss Factor |  |  | - O |  |  | - O | - O |
| - 4 | - 991 |  | - Grid Connection Comments | - Comments associated with the Attributes of the Resource | - O | - O | - O |  |  |  |  |
| 5 |  |  | Business Entity Relationships |  |  |  |  |  |  |  |  |
| - 5 | - 11 |  | - Service Provider ID | - Identifier assigned to the Service Provider | - M | - M | - M | - M |  | - M | - M |
| - 5 | - 12 |  | - Service Provider Name | - Name of the Service Provider | - O | - O | - O | - O |  | - O | - O |
| - 5 | - 20 |  | - Transmission/Distribution Service Provider ID | - Identifier assigned to the Transmission/Distribution Service Provider | - M | - M | - M | - M |  | - M | - M |
| - 5 | - 21 |  | - Transmission/Distribution Service Provider Name | - Name of the Transmission/Distribution Service Provider | - O | - O | - O | - O |  | - O | - O |
| - 5 | - 22 |  | - Transmission/Distribution Service Provider Account Number | - Transmission/Distribution Service Provider's account number for the Resource | - O | - O | - O | - O |  | - O | - O |
| - 5 | - 30 |  | - Load Serving Entity ID | - Identifier assigned to the Load Serving Entity | - O | - O | - M |  |  | - M | - M |
| - 5 | - 31 |  | - Load Serving Entity Name | - Name of the Load Serving Entity | - O | - O | - O |  |  | - O | - O |
| - 5 | - 40 |  | - Retail Rate ID | - Identifier assigned to the Retail Rate |  |  | - O |  |  | - O | - O |
| - 5 | - 41 |  | - Retail Rate Code | - Code representing the Retail Rate for the Service Location |  |  | - O |  |  | - O | - O |
| - 5 | - 42 |  | - Retail Rate Name | - Name of the Retail Rate for the Service Location |  |  | - O |  |  | - O | - O |
| - 5 | - 43 |  | - Retail Rate Description | - Description of the Retail Rate |  |  | - O |  |  | - O | - O |
| - 5 | - 44 |  | - Retail Rate | - Retail Rate |  |  | - O |  |  | - O | - O |
| - 5 | - 50 |  | - Meter Installation Provider ID | - Identifier assigned to the Meter Installation Provider |  |  | - O |  |  | - O | - O |
| - 5 | - 51 |  | - Meter Installation Provider Name | - Name of the Meter Installation Provider |  |  | - O |  |  | - O | - O |
| - 5 | - 60 |  | - Meter Authority ID | - Identifier assigned to the Metering Authority | - M | - M | - M |  |  |  |  |
| - 5 | - 61 |  | - Meter Authority Name | - Name of the Metering Authority | - O | - O | - O |  |  |  |  |
| - 5 | - 70 |  | - Scheduling Entity ID | - Identifier assigned to the Scheduling Entity | - M | - M | - M |  |  |  |  |
| - 5 | - 71 |  | - Scheduling Entity Name | - Name of the Scheduling Entity | - O | - O | - O |  |  |  |  |
| - 5 | - 80 |  | - Designated Dispatch Entity ID | - Identifier assigned to the Designated Dispatch Entity | - M | - M | - M |  |  |  |  |
| - 5 | - 81 |  | - Designated Dispatch Entity Name | - Name of the Designated Dispatch Entity | - O | - O | - O |  |  |  |  |
| - 5 | - 991 |  | - External Entity Comments | - Comments associated with the External Entity | - O | - O | - O |  |  |  |  |
| 6 |  |  | Device and Qualification |  |  |  |  |  |  |  |  |
| - 6 | - 20 |  | - Meter ID | - Identifier assigned to the Meter | - M | - M | - M | - M |  |  |  |
| - 6 | - 229 |  | - Meter Qualification Date | - Date of Qualification of the Meter by the Metering Authority or System Operator |  |  | - O |  |  | - O | - O |
| - 6 | - 2295 |  | - Meter Owner | - Owner of the Meter | - O | - O | - O |  | - O |  | - O |
| - 6 | - 2296 |  | - Meter Asset Comments | - Comments associated with the Meter Asset | - O |  |  |  |  |  |  |
| - 6 | - 991 |  | - Distributed Generator Comments | - Comments associated with the Distributed Generator | - O | - O | - O |  |  | - O | - O |
| 7 |  |  | Market/Program Enrollment |  |  |  |  |  |  |  |  |
| - 7 | - 10 |  | - Program ID | - Program Identifier | - M | - M | - M | - M | - M | - M | - M |
| - 7 | - 11 |  | - Program Name | - Name of the Program | - O | - O | - O | - M | - O | - O | - O |
| - 7 | - 12 |  | - Market | - Type of wholesale market | - M | - M | - O |  |  | - O | - O |
| - 7 | - 13 |  | - Market Product | - Market Product | - M | - M | - M | - M |  | - O | - O |
| - 7 | - 21 |  | - Effective Enrollment Date | - Effective Start Date for the Enrollment |  |  | - M | - M |  | - M | - M |
| - 7 | - 22 |  | - Enrollment Status | - Status of the Enrollment for the Service Location or Resource | - M | - M | - M |  |  | - M | - M |
| - 7 | - 23 |  | - Resource Type | - Type of Resource | - O | - O | - O | - O |  |  | - O |
| - 7 | - 24 |  | - Resource Qualification Test Date | - Date the Resource demonstrated its ability to deliver a product or service | - O | - O | - O | - O |  |  | - O |
| - 7 | - 25 |  | - Enrollment End Date | - Date of Termination of Enrollment | - O | - O | - O | - O |  | - O | - O |
| - 7 | - 26 |  | - Requalification Test Date | - Date the Resource will retest its ability to deliver a product or service | - O | - O | - O | - O |  |  | - O |
| - 7 | - 30 |  | - Lead Time | - Time between the advanced notification and deployment |  |  | - O |  |  | - O | - O |
| - 7 | - 31 |  | - Day Ahead Flag | - Flag set if the Resource will be participating in the Day-Ahead market | - O | - O | - O |  |  | - O | - O |
| - 7 | - 32 |  | - Real Time Flag | - Flag set if the Resource will be participating in the Real-Time market | - O | - O | - O |  |  | - O | - O |
| - 7 | - 33 |  | - Self-Schedule Flag | - Flag set if the Resource is Self-Deploying |  |  |  |  |  |  |  |
| - 7 | - 40 |  | - Response Method Type | - Type of Response Method | - O | - O | - O | - O |  | - O | - O |
| - 7 | - 41 |  | - Response Method ID | - Identifier assigned to the Response Method | - O | - O | - M |  |  | - M | - M |
| - 7 | - 42 |  | - Response Method Name | - Name of the Response Method | - O | - O | - O |  |  | - O | - O |
| - 7 | - 43 |  | - Response Method Value | - Value of the Response Method | - O | - O | - M | - M |  | - M | - M |
| - 7 | - 44 |  | - Verified Capability | - Audited Capability | - O | - O | - M | - M |  |  |  |
| - 7 | - 45 |  | - Verified Capability Factor | - Ratio of the Verified Capability to the qualified capability |  |  | - M |  |  |  |  |
| - 7 | - 50 |  | - Performance Evaluation Method Type Code | - Code representing the Type of Measurement | - M | - M | - M | - M |  |  |  |
| - 7 | - 51 |  | - Performance Evaluation Method | - Method used to Evaluate the Performance of a Resource | - O | - O | - M |  |  |  |  |
| - 7 | - 991 |  | - Market Enrollment Comments | - Comments associated with the Enrollment | - O | - O | - O | - O | - O | - O | - O |
| 8 |  |  | Offer Parameters |  |  |  |  |  |  |  |  |
| - 8 | - 10 |  | - Offer Limit Value | - Offer Limit Value | - M | - O | - M | - M |  | - O | - O |
| - 8 | - 11 |  | - Offer Limit Type | - Type of Offer Limit | - M | - O | - M | - M |  | - O | - O |
| - 8 | - 12 |  | - Offer Limit Interval | - Offer Limit Interval | - M | - O | - M | - M |  | - O | - O |
| - 8 | - 20 |  | - Physical Min Gen | - Minimum Operating Level of a Resource | - O | - O | - O | - O |  | - O | - O |
| - 8 | - 21 |  | - Min Gen MW | - Minimum MW available for dispatch | - O | - O | - O | - O |  | - O | - O |
| - 8 | - 22 |  | - Min Gen Cost | - The cost per hour for each Min Gen MW value. | - O | - O | - O | - O |  |  |  |
| - 8 | - 23 |  | - Ramp Rate Type | - Type of Ramp Rate | - M | - O | - O | - O |  |  |  |
| - 8 | - 24 |  | - Ramp Rate Segment | - Energy of Segment of the Ramp Rate | - M | - O | - O | - O |  |  |  |
| - 8 | - 25 |  | - Ramp Rate Direction | - Direction of the selected Ramp Rate Type and Ramp Rate Segment | - M | - O | - O | - O |  |  |  |
| - 8 | - 26 |  | - Ramp Rate Value | - Ramp Rate associated with the selected Ramp Rate Type for the selected Ramp Rate Segment and Ramp Rate Direction | - M | - O | - O | - O |  |  |  |
| - 8 | - 30 |  | - Offer Dispatch Type | - Type of Offer |  | - O |  |  |  |  |  |
| - 8 | - 40 |  | - Market Clearing Day | - Market Clearing Day of the Offer |  |  |  | - O |  |  |  |
| - 8 | - 41 |  | - Schedule Name | - Name or tag of the Offer |  |  |  | - O |  |  |  |
| - 8 | - 42 |  | - Schedule Description | - Description of the Offer |  |  |  | - O |  |  |  |
| - 8 | - 43 |  | - Schedule Status | - Status of the Offer |  |  |  | - O |  |  |  |
| - 8 | - 50 |  | - Portfolio Name | - Name of an aggregation of Resources for market participation |  |  |  | - O |  |  |  |
| 10 |  |  | Ancillary Service Market |  |  |  |  |  |  |  |  |
| - 10 | - 10 |  | - Ancillary Service Product Type | - Type of Ancillary Service product(s) the Resource is enrolling to provide | - O | - O | - O | - M |  | - O | - O |
| - 10 | - 11 |  | - Have Engineering Diagram | - Flag set if One-Line Diagrams have been submitted |  | - O |  |  |  |  |  |
| - 10 | - 20 |  | - Ancillary Service Product Qualification Type | - Type of Qualification | - O | - O | - M | - M |  |  |  |
| - 10 | - 22 |  | - Ancillary Service Product Type Qualification Date | - Date of Qualification of the Resource |  |  | - M | - M |  |  |  |
| - 10 | - 23 |  | - Ancillary Service Product Type Qualified | - Type of Ancillary Service Product(s) for which the Resource has Qualified |  |  | - M | - M |  |  |  |
| - 10 | - 24 |  | - Requalification Requirement | - Flag set if Requalification is Required | - O | - O |  |  |  |  |  |
| 11 |  |  | Capacity Market |  |  |  |  |  |  |  |  |
| - 11 | - 10 |  | - Capacity Type | - Type of Capacity | - O | - O | - M |  |  |  |  |
| - 11 | - 11 |  | - Capacity Type Description | - Description of the Type of Capacity | - O | - O | - O |  |  |  |  |
| - 11 | - 20 |  | - Peak Value | - Measurement of Peak |  |  | - O |  |  | - O | - O |
| - 11 | - 21 |  | - Peak Type | - Type of measurement of Peak |  |  | - O |  |  | - O | - O |
| - 11 | - 22 |  | - Peak Date/Time | - Date and Time of measurement of Peak |  |  | - O |  |  | - O | - O |
| - 11 | - 23 |  | - Nominal Capacity | - Nominated load |  |  | - O |  |  |  |  |
| - 11 | - 24 |  | - Qualified Capacity | - Nominal installed capacity (ICAP) derated for performance |  |  | - M |  |  |  |  |
| - 11 | - 30 |  | - Capacity Obligation Period Type | - Type of Capacity Obligation Time Period within which the Resource is obligated | - O | - O | - M |  |  | - M | - M |
| - 11 | - 31 |  | - Capacity Obligation Period | - Period of time for which the Capacity Resource is obligated | - O | - O | - M |  |  | - M | - M |
| - 11 | - 40 |  | - Minimum Payment Nomination | - Strike price for the economic dispatch of the energy component of the capacity program | - O | - O | - O |  |  |  |  |
| - 11 | - 50 |  | - Capacity Reference ID | - Identifier assigned to the External System award |  |  | - O |  |  |  |  |
| - 11 | - 991 |  | - Capacity Comments | - Comments associated with the Capacity market | - O | - O | - O |  |  |  |  |

018-1.4.2	Scheduling and Award Notification – Economic

Overview: 	The Scheduling and Award Notification process describes the process from offer submission to award notification. This process also includes the supplemental commitment and reassessment of reliability to determine whether demand resources that are enrolled in reliability-based (emergency) demand response programs should be advised of a possible reliability deployment.

Table 6. Use Cases for Figure 3

- The process begins when the SE submits a supply offer to the SO.

- The SO evaluates the offer through its market clearing process.

- The resulting resource-specific dispatch information is transformed into dispatch instructions by the SO.

- The SO makes the schedule available.

- The SE collects the forward schedule.

Activity Diagram and Data Flow:

Figure 3 - Scheduling and Award Notification - Economic

Note: Data elements for Scheduling and Award Notification-Economic are listed with Scheduling and Award Notification-Reliability in Table 8.

#### 018-1.4.3	Scheduling and Award Notification – Reliability

Overview: 	The Scheduling and Award Notification process describes the process from offer submission to award notification. This process also includes the supplemental commitment and reassessment of reliability to determine whether demand resources that are enrolled in reliability-based (emergency) demand response programs should be advised of a possible reliability deployment.

Table 7. Use Cases for Figure 4

- The process begins when the SE submits availability to the SO.

- The SO performs the Load Forecast and Supplemental Commitment process after the market has been settled. Reliability is reassessed by the SO to determine whether the system is secure (sufficient supply to meet forecasted load conditions).

  - If SO determines that the system is secure, no demand response advance notification is required and reliability is assessed by the SO at the next interval.

  - If SO anticipates that a reliability issue is expected, the SO decides whether demand response is needed for reliability.

    - If demand response will not be needed, the process ends.

    - If the SO determines that demand response will be provided with an advance notification, the reliability event parameters are prepared by the SO to create a reliability event notification.

      - The advance notification message is sent to the SE.

      - The SE identifies the demand resources to notify and relays the message to SPs through the proprietary interaction system of the SE.

Activity Diagram and Data Flow:

Figure 4 - Scheduling and Award Notification - Reliability

Table 8. Data Elements by Interaction Number: Scheduling and Award Notification

| Legend: Cells showing an “M” (mandatory) are data elements that must contain some value when included in the interaction message. Cells showing an “O” (optional) are data elements that may be left blank if not used in a given interaction. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Process | 2.0 Scheduling and Award Notification |  |  |  |  |
|  |  |  |  | 2.1 Economic |  |  | 2.2 Reliability |  |
|  |  |  | From | SE | SO |  | SE | SO |
|  |  |  | To | SO | SE | SE | SO | SE |
|  |  |  |  | 2.1.1 | 2.1.2 | 2.1.3 | 2.2.1 | 2.2.2 |
|  |  |  | Interaction | Offer Parameters | Award | Schedule | Availability Parameters | Advanced Notification |
| Element |  |  | Short Description |  |  |  |  |  |
| 0 |  | Common |  |  |  |  |  |  |
| - 0 | - 10 | - Submittal Date | - Timestamp for the sender's use | - O | - O | - O | - O | - O |
| - 0 | - 11 | - Submitted By | - User ID of submitter | - O | - O | - O | - O | - O |
| - 0 | - 12 | - Submitted Error | - ID of submission error detected | - O | - O | - O | - O | - O |
| - 0 | - 20 | - NERC CIP Security - Availability | - CIP Security Classification for Availability | - M | - M | - M | - M | - M |
| - 0 | - 21 | - NERC CIP Security - Confidentiality | - CIP Security Classification for Confidentiality | - M | - M | - M | - M | - M |
| - 0 | - 22 | - NERC CIP Security - Integrity | - CIP Security Classification for Integrity | - M | - M | - M | - M | - M |
| 1 |  | General Asset/Resource |  |  |  |  |  |  |
| - 1 | - 10 | - Service Location ID | - Identifier assigned to the Service Location | - O | - O | - O | - O | - O |
| - 1 | - 11 | - Service Location Name | - Name of the Service Location | - O | - O | - O | - M | - O |
| - 1 | - 12 | - Asset ID | - The unique identifier of the asset | - O | - O | - O | - O | - O |
| - 1 | - 13 | - Asset Name | - The name of the asset | - O | - O | - O | - O | - O |
| - 1 | - 20 | - Resource ID | - Identifier assigned to the Resource | - M | - M | - M | - M | - O |
| - 1 | - 21 | - Resource Name | - Name of the Resource | - O | - O | - O | - M | - O |
| - 1 | - 40 | - Gen EMS ID | - Alias or point Identifier assigned to the Resource | - M | - M | - M |  |  |
| - 1 | - 41 | - Gen Billing ID | - Billing Identifier assigned to the Resource | - M | - M | - M |  |  |
| 2 |  | Location |  |  |  |  |  |  |
| - 2 | - 30 | - Zone ID | - Identifier assigned to the Zone in which the Service Location is located | - M | - M | - M | - M | - M |
| - 2 | - 31 | - Zone | - Name of the Zone in which the Service Location is located | - O | - O | - O | - O | - O |
| - 2 | - 32 | - Zone Type | - Type of Zone | - O | - O | - O | - O | - O |
| - 2 | - 40 | - Electrical Node ID | - Identifier assigned to the Electrical Node at which the Service Location is connected | - M | - M | - M | - O |  |
| - 2 | - 41 | - Electrical Node Name | - Name of the Electrical Node at which the Service Location is connected | - O | - O | - O | - O |  |
| - 2 | - 42 | - Electrical Node Type | - Type of Electrical Node at which the Service Location is attached | - O | - O | - O | - O |  |
| - 2 | - 43 | - PNode | - Name of the Price Node associated with the Service Location | - O | - O | - O | - O | - O |
| - 2 | - 44 | - PNode ID | - Identifier assigned to the Price Node associated with the Service Location | - M | - M | - M | - M | - M |
| 5 |  | Business Entity Relationships |  |  |  |  |  |  |
| - 5 | - 11 | - Service Provider ID | - Identifier assigned to the Service Provider |  |  | - O | - M | - M |
| - 5 | - 12 | - Service Provider Name | - Name of the Service Provider |  |  | - O | - M | - M |
| - 5 | - 20 | - Transmission/Distribution Service Provider ID | - Identifier assigned to the Transmission/Distribution Service Provider |  |  | - O | - O |  |
| - 5 | - 21 | - Transmission/Distribution Service Provider Name | - Name of the Transmission/Distribution Service Provider |  |  | - O | - O |  |
| - 5 | - 22 | - Transmission/Distribution Service Provider Account Number | - Transmission/Distribution Service Provider's account number for the Resource |  |  | - O | - O |  |
| - 5 | - 30 | - Load Serving Entity ID | - Identifier assigned to the Load Serving Entity |  |  | - O | - O |  |
| - 5 | - 31 | - Load Serving Entity Name | - Name of the Load Serving Entity |  |  | - O | - O |  |
| - 5 | - 70 | - Scheduling Entity ID | - Identifier assigned to the Scheduling Entity | - M | - M | - M | - M | - M |
| - 5 | - 71 | - Scheduling Entity Name | - Name of the Scheduling Entity | - O | - O | - O | - O | - O |
| - 5 | - 80 | - Designated Dispatch Entity ID | - Identifier assigned to the Designated Dispatch Entity | - M | - M | - M | - M | - M |
| - 5 | - 81 | - Designated Dispatch Entity Name | - Name of the Designated Dispatch Entity | - O | - O | - O | - O | - O |
| 6 |  | Device and Qualification |  |  |  |  |  |  |
| - 6 | - 20 | - Meter ID | - Identifier assigned to the Meter | - O | - O | - O | - O |  |
| - 6 | - 44 | - Normal Breaker Status | - Status of the Breaker for the Service Location under Normal operating conditions |  |  |  | - M |  |
| - 6 | - 45 | - Breaker Status During Event | - Status of the Breaker for the Service Location during an Emergency Event |  |  |  | - M |  |
| - 6 | - 46 | - Wholesale Delivery Point Status | - Flag set if the Service Location is metered at the Wholesale level |  |  |  | - M |  |
| - 6 | - 47 | - Private Use Network | - Flag set if the Resource is an Electric Network connected to the transmission grid |  |  |  | - M |  |
| - 6 | - 51 | - UFR Settings | - Setting of the Under-Frequency Relay |  |  |  | - M |  |
| - 6 | - 52 | - Load Resource Control Device | - Type of Control Device at a Service Location or deployed by a Response Method |  |  |  | - M |  |
| 7 |  | Market/Program Enrollment |  |  |  |  |  |  |
| - 7 | - 10 | - Program ID | - Program Identifier | - O | - O | - O | - O | - O |
| - 7 | - 11 | - Program Name | - Name of the Program | - O | - O | - O | - O | - O |
| - 7 | - 12 | - Market | - Type of wholesale market | - O | - O | - O | - O | - O |
| - 7 | - 13 | - Market Product | - Market Product | - O | - O | - O | - O | - O |
| - 7 | - 30 | - Lead Time | - Time between the advanced notification and deployment | - O | - O |  | - O | - O |
| - 7 | - 31 | - Day Ahead Flag | - Flag set if the Resource will be participating in the Day-Ahead market | - M | - M | - M | - M | - M |
| - 7 | - 32 | - Real Time Flag | - Flag set if the Resource will be participating in the Real-Time market | - M | - M | - M | - M | - M |
| - 7 | - 33 | - Self-Schedule Flag | - Flag set if the Resource is Self-Deploying | - O | - O | - O | - O |  |
| - 7 | - 40 | - Response Method Type | - Type of Response Method |  | - O | - O |  |  |
| - 7 | - 41 | - Response Method ID | - Identifier assigned to the Response Method |  | - O | - O |  |  |
| - 7 | - 42 | - Response Method Name | - Name of the Response Method |  | - O | - O |  |  |
| - 7 | - 43 | - Response Method Value | - Value of the Response Method |  | - O | - O |  |  |
| - 7 | - 44 | - Verified Capability | - Audited Capability |  |  |  | - O |  |
| - 7 | - 45 | - Verified Capability Factor | - Ratio of the Verified Capability to the qualified capability |  |  |  | - O |  |
| 8 |  | Offer Parameters |  |  |  |  |  |  |
| - 8 | - 10 | - Offer Limit Value | - Offer Limit Value | - M |  |  | - M | - O |
| - 8 | - 11 | - Offer Limit Type | - Type of Offer Limit | - M |  |  | - M | - O |
| - 8 | - 12 | - Offer Limit Interval | - Offer Limit Interval | - O |  |  | - M | - O |
| - 8 | - 20 | - Physical Min Gen | - Minimum Operating Level of a Resource | - O |  |  | - M |  |
| - 8 | - 21 | - Min Gen MW | - Minimum MW available for dispatch | - M |  |  | - M | - O |
| - 8 | - 22 | - Min Gen Cost | - The cost per hour for each Min Gen MW value. | - M |  |  | - M | - O |
| - 8 | - 23 | - Ramp Rate Type | - Type of Ramp Rate | - O |  |  | - M |  |
| - 8 | - 24 | - Ramp Rate Segment | - Energy of Segment of the Ramp Rate | - O |  |  | - M |  |
| - 8 | - 25 | - Ramp Rate Direction | - Direction of the selected Ramp Rate Type and Ramp Rate Segment | - O |  |  |  |  |
| - 8 | - 26 | - Ramp Rate Value | - Ramp Rate associated with the selected Ramp Rate Type for the selected Ramp Rate Segment and Ramp Rate Direction | - O |  |  |  |  |
| - 8 | - 30 | - Offer Dispatch Type | - Type of Offer | - M | - M | - M |  |  |
| - 8 | - 31 | - Offer Segment MW | - Energy of Segment of the Offer | - M | - M | - M | - O | - O |
| - 8 | - 32 | - Offer Segment Price | - Price of Segment of the Offer | - M | - M | - M | - O | - O |
| - 8 | - 33 | - Offer Segment Hour | - Hour of Segment of the Offer | - M | - M | - M | - O | - O |
| - 8 | - 34 | - Startup Cost | - Cost of starting a response activity | - M | - M | - M | - O | - O |
| - 8 | - 35 | - Strike Price | - Threshold Price | - O | - O | - O |  |  |
| - 8 | - 36 | - Offer Expiration Date | - Date of Expiration of the Offer | - M |  |  |  |  |
| - 8 | - 37 | - Startup Cost Type | - Type of startup cost | - M |  |  | - M |  |
| - 8 | - 38 | - Offer Commit Status | - Commitment status of offer | - M |  |  | - M |  |
| - 8 | - 39 | - Offer Dispatch Status | - Dispatch Status of Offer | - M |  |  | - M |  |
| - 8 | - 331 | - Offer Price Curve Slope | - Flag to smooth offer segments from step function to slope | - M |  |  | - M |  |
| - 8 | - 332 | - Strike Price Type | - Type of strike price | - O |  |  | - O |  |
| - 8 | - 40 | - Market Clearing Day | - Market Clearing Day of the Offer | - M | - M | - M | - M | - M |
| - 8 | - 41 | - Schedule Name | - Name or tag of the Offer | - O | - O | - O | - O | - O |
| - 8 | - 42 | - Schedule Description | - Description of the Offer | - O | - O | - O | - O | - O |
| - 8 | - 43 | - Schedule Status | - Status of the Offer | - M | - M | - M | - O | - O |
| - 8 | - 50 | - Portfolio Name | - Name of an aggregation of Resources for market participation | - O | - O | - O | - O | - O |
| - 8 | - 71 | - Constraint Type | - Type of operational, schedule or offer constraint | - M |  |  | - M |  |
| - 8 | - 72 | - Constraint Interval | - The timeframe over which the constraint type applies. | - M |  |  | - M |  |
| - 8 | - 73 | - Constraint Value | - Value of the Constraint Type and Interval | - M |  |  | - M |  |
| 9 |  | Energy Market |  |  |  |  |  |  |
| - 9 | - 0 | - Instruction ID | - Unique identifier assigned to each instruction. | - O | - O | - O |  |  |
| - 9 | - 10 | - Energy Schedule - Start Time | - Start Time of the Energy Schedule | - M | - M | - M |  |  |
| - 9 | - 11 | - Energy Schedule - End Time | - End Time of the Energy Schedule | - M | - M | - M |  |  |
| - 9 | - 12 | - Energy Schedule - Interval Start Time | - Start Time of the Energy Interval within the Energy Schedule | - M | - M | - M |  |  |
| - 9 | - 13 | - Energy Schedule - Cleared MW Interval Value | - The Energy Value of cleared MW for the Time Interval within the Energy Schedule |  | - M | - M |  |  |
| - 9 | - 14 | - Energy Schedule - Cleared Price | - Awarded Price |  | - M | - M |  |  |
| - 9 | - 15 | - Schedule ID | - Unique identifier assigned to each schedule. | - O |  | - O |  |  |
| 10 |  | Ancillary Service Market |  |  |  |  |  |  |
| - 10 | - 0 | - Ancillary Service Instruction ID | - Unique identifier assigned to each ancillary service instruction. |  | - O | - O |  |  |
| - 10 | - 10 | - Ancillary Service Product Type | - Type of Ancillary Service product(s) the Resource is enrolling to provide | - M | - M | - M | - M |  |
| - 10 | - 12 | - Ancillary Service Award - Start Time | - Start Time of the ancillary service Award |  | - M |  |  |  |
| - 10 | - 13 | - Ancillary Service Award - End Time | - End Time of the ancillary service Award |  | - M |  |  |  |
| - 10 | - 14 | - Ancillary Service Award - Interval Start Time | - Start Time of the ancillary service Interval within the ancillary service Award and the ancillary service Value of the Time Interval |  | - M |  |  |  |
| - 10 | - 15 | - Ancillary Service Award - Interval Value | - The awarded Value for the Time Interval within the ancillary service Award |  | - M |  |  |  |
| - 10 | - 16 | - Ancillary Service Award - Cleared Price | - Awarded Price |  | - M |  |  |  |
| - 10 | - 17 | - Ancillary Service Schedule ID | - Unique identifier assigned to each ancillary service schedule. |  |  | - O |  |  |
| - 10 | - 31 | - Reserve Pickup Flag | - Flag set if this is a Reserve Pickup schedule |  | - O | - O |  |  |
| 11 |  | Capacity Market |  |  |  |  |  |  |
| - 11 | - 10 | - Capacity Type | - Type of Capacity | - M | - M | - M | - O |  |
| - 11 | - 20 | - Peak Value | - Measurement of Peak |  |  | - O | - O |  |
| - 11 | - 21 | - Peak Type | - Type of measurement of Peak |  |  | - O | - O |  |
| - 11 | - 22 | - Peak Date/Time | - Date and Time of measurement of Peak |  |  | - O | - O |  |
| - 11 | - 23 | - Nominal Capacity | - Nominated load | - O | - O | - O |  |  |
| - 11 | - 24 | - Qualified Capacity | - Nominal installed capacity (ICAP) derated for performance | - M | - M | - M | - O |  |
| - 11 | - 30 | - Capacity Obligation Period Type | - Type of Capacity Obligation Time Period within which the Resource is obligated | - M | - M | - M | - O |  |
| - 11 | - 31 | - Capacity Obligation Period | - Period of time for which the Capacity Resource is obligated | - M | - M | - M | - O |  |
| - 11 | - 40 | - Minimum Payment Nomination | - Strike price for the economic dispatch of the energy component of the capacity program | - M | - O | - O | - O |  |
| - 11 | - 41 | - Capacity Supply Obligation | - Capacity Obligation | - M | - M | - M | - O |  |
| - 11 | - 50 | - Capacity Reference ID | - Identifier assigned to the External System award | - O | - O | - O | - O |  |
| - 11 | - 991 | - Capacity Comments | - Comments associated with the Capacity market | - O | - O | - O | - O |  |
| 12 |  | Event and Performance |  |  |  |  |  |  |
| - 12 | - 10 | - Event ID | - Identifier assigned to the Event |  |  |  |  | - O |
| - 12 | - 11 | - Deployment Type | - Type of Deployment |  |  |  |  | - M |
| - 12 | - 12 | - Event Day | - Operating Date of Start of Event |  |  |  |  | - M |
| - 12 | - 13 | - Event Description | - Description of the Event |  |  |  |  | - M |
| - 12 | - 14 | - Event Start Time | - Time of Start of deployment period for Event |  |  |  |  | - M |
| - 12 | - 15 | - Operator Comment | - Additional Event information provided by the Operator |  |  |  |  | - O |
| - 12 | - 16 | - Deployment MW | - Absolute or relative Deployment quantity |  |  |  |  | - O |
| - 12 | - 17 | - Event Status | - Status of the Event |  |  |  |  | - O |
| - 12 | - 18 | - Event End Time | - Time of End of deployment period |  |  |  |  | - M |
| - 12 | - 19 | - Event Modification Number | - A modification number for the event. |  |  |  |  | - O |
| - 12 | - 111 | - Notification Acknowledgement | - This is an acknowledgement of the receipt of a DR notification or dispatch. It should include any necessary provisions for non-repudiation. |  |  |  |  | - O |
| - 12 | - 112 | - Event Modification Reason Code | - The reason the event is being cancelled or modified. |  |  |  |  | - O |
| - 12 | - 991 | - Event & Performance Comments | - Comments associated with Event and Performance data |  |  |  |  | - O |

#### 018-1.4.4	Deployment and Real-Time Communications - Economic

Overview: 	The Deployment and Real-Time Communication process describes the process for real-time communications to demand resources providing market-based services on a real-time basis and dispatch for reliability-based (emergency) demand response programs. The business activity diagrams and descriptions below reflect a sequential nature to the real-time process solely for the purposes of describing the process of the real-time data flow; the real-time communication process simultaneously scans and updates in the same step.

Table 9. Use Cases for Figure 5

- The process begins when the SO evaluates real-time system conditions through its market clearing process.

- The resulting resource-specific real-time dispatch information is transformed into dispatch instructions by the SO.

  - If the dispatch instruction is for Regulation, the SO sends the Regulation signal to the DDE.

  - If the dispatch is not for Regulation, the SO sends the dispatch instruction to the DDE.

- The DDE communicates with the SP through the proprietary communication system of the DDE.

- The SO collects real-time response from the DDE for evaluating the next interval and uses the information to calculate the system state.

- System conditions are made available to the market clearing process for the next interval and to Deployment and Real-Time Communications – Reliability.

Activity Diagram and Data Flow:

Figure 5 - Deployment and Real-Time Communications - Economic

Note: Data elements for Deployment & Real-Time Communications-Economic are listed with Deployment & Real-Time Communications-Reliability in Table 11.

#### 018-1.4.5	Deployment and Real-Time Communications – Reliability

Overview: 	The Deployment and Real-Time Communication process describes the process for real-time communications to demand resources providing market-based services on a real-time basis and dispatch for reliability-based (emergency) demand response programs. The business activity diagrams and descriptions below reflect a sequential nature to the real-time process solely for the purposes of describing the process of the real-time data flow; the real-time communication process simultaneously scans and updates in the same step.

Table 10. Use Cases for Figure 6

- The SO performs the Load Forecast and Supplemental Commitment process after the interval has been dispatched. Reliability is reassessed by the SO to determine whether the system is secure (sufficient supply to meet forecasted load conditions).

  - If SO determines that the system is secure, no demand response deployment is required and reliability is assessed by the SO at the next interval.

  - If SO anticipates that a reliability issue is expected, the SO decides whether demand response is needed for reliability.

    - If demand response will not be needed, the process ends.

    - If the SO determines that demand response will be deployed, the reliability event parameters are prepared by the SO to create a reliability event notification.

      - The deployment instruction is sent to the DDE.

      - The DDE identifies the demand resources to notify and relays the message to SPs through the proprietary communication system of the DDE.

    - Real-time or near real-time response data is collected by DDE using the proprietary communication system of the DDE.

    - The DDE sends the event response information to the SO.

Activity Diagram and Data Flow:

Figure 6 - Deployment and Real-Time Communications - Reliability

Table 11. Data Elements by Interaction Number: Deployment & Real-Time Communications

| Legend: Cells showing an “M” (mandatory) are data elements that must contain some value when included in the interaction message. Cells showing an “O” (optional) are data elements that may be left blank if not used in a given interaction. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Process | 3.0 Deployment & Real-Time Communications |  |  |  |  |
|  |  |  |  |  | 3.1 Economic |  |  | 3.2 Reliability |  |
|  |  |  |  | From | SO |  | DDE | SO | DDE |
|  |  |  |  | To | DDE | DDE | SO | DDE | SO |
|  |  |  |  |  | 3.1.1 | 3.1.2 | 3.1.3 | 3.2.1 | 3.2.2 |
|  |  |  |  | Interaction | Regulation Signal | Dispatch Instruction | Real-Time Response | Deployment Instructions | Event Response |
| Element |  |  |  | Short Description |  |  |  |  |  |
| 0 |  |  | Common |  |  |  |  |  |  |
| - 0 | - 10 |  | - Submittal Date | - Timestamp for the sender's use | - O |  |  | - O | - O |
| - 0 | - 11 |  | - Submitted By | - User ID of submitter | - O |  |  | - O | - O |
| - 0 | - 12 |  | - Submitted Error | - ID of submission error detected | - O |  |  | - O | - O |
| - 0 | - 20 |  | - NERC CIP Security - Availability | - CIP Security Classification for Availability | - M | - M | - M | - M | - M |
| - 0 | - 21 |  | - NERC CIP Security - Confidentiality | - CIP Security Classification for Confidentiality | - M | - M | - M | - M | - M |
| - 0 | - 22 |  | - NERC CIP Security - Integrity | - CIP Security Classification for Integrity | - M | - M | - M | - M | - M |
| 1 |  |  | General Asset/Resource |  |  |  |  |  |  |
| - 1 | - 10 |  | - Service Location ID | - Identifier assigned to the Service Location | - M | - O | - O | - M | - M |
| - 1 | - 11 |  | - Service Location Name | - Name of the Service Location | - O | - O | - O | - M | - M |
| - 1 | - 12 |  | - Asset ID | - The unique identifier of the asset | - O | - O | - O | - O | - O |
| - 1 | - 13 |  | - Asset Name | - The name of the asset | - O | - O | - O | - O | - O |
| - 1 | - 20 |  | - Resource ID | - Identifier assigned to the Resource | - M | - M | - M | - M | - M |
| - 1 | - 21 |  | - Resource Name | - Name of the Resource | - O | - O | - O | - M | - M |
| - 1 | - 30 |  | - Asset Group ID | - The identifier of a group of assets |  |  | - O |  | - O |
| - 1 | - 31 |  | - Asset Group Name | - Name of the aggregated group of assets |  |  | - O |  | - O |
| - 1 | - 40 |  | - Gen EMS ID | - Alias or point Identifier assigned to the Resource | - M | - M | - M |  |  |
| - 1 | - 41 |  | - Gen Billing ID | - Billing Identifier assigned to the Resource | - M | - M | - M |  |  |
| 2 |  |  | Location |  |  |  |  |  |  |
| - 2 | - 30 |  | - Zone ID | - Identifier assigned to the Zone in which the Service Location is located |  |  |  | - M | - M |
| - 2 | - 31 |  | - Zone | - Name of the Zone in which the Service Location is located |  |  |  | - O | - O |
| - 2 | - 32 |  | - Zone Type | - Type of Zone |  |  |  | - O | - O |
| - 2 | - 40 |  | - Electrical Node ID | - Identifier assigned to the Electrical Node at which the Service Location is connected | - O | - O | - O | - O | - O |
| - 2 | - 41 |  | - Electrical Node Name | - Name of the Electrical Node at which the Service Location is connected | - O | - O | - O |  |  |
| - 2 | - 43 |  | - PNode | - Name of the Price Node associated with the Service Location |  | - O |  |  |  |
| - 2 | - 44 |  | - PNode ID | - Identifier assigned to the Price Node associated with the Service Location |  | - M |  |  |  |
| - 2 | - 71 |  | - Connection Type | - Additional type of connection associated with the Service Location/Resource | - O | - O |  | - O |  |
| - 2 | - 72 |  | - Connection Address | - Address associated with the Connection Type | - O | - O |  | - O |  |
| 5 |  |  | Business Entity Relationships |  |  |  |  |  |  |
| - 5 | - 11 |  | - Service Provider ID | - Identifier assigned to the Service Provider |  |  |  | - M | - M |
| - 5 | - 12 |  | - Service Provider Name | - Name of the Service Provider |  |  |  | - O | - O |
| - 5 | - 20 |  | - Transmission/Distribution Service Provider ID | - Identifier assigned to the Transmission/Distribution Service Provider | - O | - O | - O |  |  |
| - 5 | - 21 |  | - Transmission/Distribution Service Provider Name | - Name of the Transmission/Distribution Service Provider | - O | - O | - O |  |  |
| - 5 | - 70 |  | - Scheduling Entity ID | - Identifier assigned to the Scheduling Entity | - M | - M | - M | - M | - M |
| - 5 | - 71 |  | - Scheduling Entity Name | - Name of the Scheduling Entity | - O | - O | - O | - O | - O |
| - 5 | - 80 |  | - Designated Dispatch Entity ID | - Identifier assigned to the Designated Dispatch Entity | - M | - M | - M | - M | - M |
| - 5 | - 81 |  | - Designated Dispatch Entity Name | - Name of the Designated Dispatch Entity | - O | - O | - O | - O | - O |
| 6 |  |  | Device and Qualification |  |  |  |  |  |  |
| - 6 | - 20 |  | - Meter ID | - Identifier assigned to the Meter | - O | - O | - O |  |  |
| - 6 | - 44 |  | - Normal Breaker Status | - Status of the Breaker for the Service Location under Normal operating conditions |  |  |  | - M |  |
| - 6 | - 45 |  | - Breaker Status During Event | - Status of the Breaker for the Service Location during an Emergency Event |  |  | - M | - M | - M |
| - 6 | - 46 |  | - Wholesale Delivery Point Status | - Flag set if the Service Location is metered at the Wholesale level |  |  | - M | - M | - M |
| - 6 | - 47 |  | - Private Use Network | - Flag set if the Resource is an Electric Network connected to the transmission grid |  |  | - M | - M | - M |
| - 6 | - 51 |  | - UFR Settings | - Setting of the Under-Frequency Relay |  |  | - M | - M | - M |
| - 6 | - 52 |  | - Load Resource Control Device | - Type of Control Device at a Service Location or deployed by a Response Method |  |  | - M | - M | - M |
| 7 |  |  | Market/Program Enrollment |  |  |  |  |  |  |
| - 7 | - 10 |  | - Program ID | - Program Identifier | - O | - O | - O | - M | - M |
| - 7 | - 11 |  | - Program Name | - Name of the Program | - O | - O | - O | - M | - M |
| - 7 | - 12 |  | - Market | - Type of wholesale market | - O | - O | - O | - O | - O |
| - 7 | - 13 |  | - Market Product | - Market Product | - O | - O | - O | - O | - O |
| - 7 | - 30 |  | - Lead Time | - Time between the advanced notification and deployment | - O | - O | - O |  |  |
| - 7 | - 31 |  | - Day Ahead Flag | - Flag set if the Resource will be participating in the Day-Ahead market | - O | - O | - O |  |  |
| - 7 | - 32 |  | - Real Time Flag | - Flag set if the Resource will be participating in the Real-Time market | - O | - O | - O |  |  |
| - 7 | - 33 |  | - Self-Schedule Flag | - Flag set if the Resource is Self-Deploying |  | - O | - O |  |  |
| 8 |  |  | Offer Parameters |  |  |  |  |  |  |
| - 8 | - 10 |  | - Offer Limit Value | - Offer Limit Value | - O | - O |  |  |  |
| - 8 | - 11 |  | - Offer Limit Type | - Type of Offer Limit | - O | - O |  |  |  |
| - 8 | - 12 |  | - Offer Limit Interval | - Offer Limit Interval | - O | - O |  |  |  |
| - 8 | - 21 |  | - Min Gen MW | - Minimum MW available for dispatch | - O | - O |  |  |  |
| - 8 | - 22 |  | - Min Gen Cost | - The cost per hour for each Min Gen MW value. | - O | - O |  |  |  |
| - 8 | - 40 |  | - Market Clearing Day | - Market Clearing Day of the Offer | - M | - M | - M | - M | - M |
| - 8 | - 41 |  | - Schedule Name | - Name or tag of the Offer | - O | - O | - O | - O | - O |
| - 8 | - 42 |  | - Schedule Description | - Description of the Offer | - O | - O | - O | - O | - O |
| - 8 | - 43 |  | - Schedule Status | - Status of the Offer | - O | - O | - O | - O | - O |
| - 8 | - 50 |  | - Portfolio Name | - Name of an aggregation of Resources for market participation | - O | - O | - O | - O | - O |
| 9 |  |  | Energy Market |  |  |  |  |  |  |
| - 9 | - 0 |  | - Instruction ID | - Unique identifier assigned to each instruction. | - O | - O |  |  |  |
| - 9 | - 10 |  | - Energy Schedule - Start Time | - Start Time of the Energy Schedule | - O | - O |  |  |  |
| - 9 | - 11 |  | - Energy Schedule - End Time | - End Time of the Energy Schedule | - O | - O |  |  |  |
| - 9 | - 12 |  | - Energy Schedule - Interval Start Time | - Start Time of the Energy Interval within the Energy Schedule | - O | - O |  |  |  |
| - 9 | - 13 |  | - Energy Schedule - Cleared MW Interval Value | - The Energy Value of cleared MW for the Time Interval within the Energy Schedule | - O | - O |  |  |  |
| - 9 | - 14 |  | - Energy Schedule - Cleared Price | - Awarded Price | - O |  |  |  |  |
| - 9 | - 15 |  | - Schedule ID | - Unique identifier assigned to each schedule. | - O | - O |  |  |  |
| - 9 | - 20 |  | - Base Point | - Per-interval dispatch instruction | - M | - M | - O | - M |  |
| - 9 | - 21 |  | - Breaker Status | - Status of the Breaker for the Resource | - M | - M | - O | - M | - O |
| - 9 | - 22 |  | - Output MW | - Real Power Output of the Generation Device | - M | - M | - M | - M | - O |
| - 9 | - 23 |  | - Output MVAR | - Reactive Power Output of the Generation Device | - M | - M | - O | - M | - O |
| - 9 | - 24 |  | - DR Bus Load MW | - Instantaneous Bus Load of a Demand Resource providing Ancillary Services | - O | - O | - O | - O | - O |
| - 9 | - 27 |  | - Set Point | - Final dispatch target level | - M | - M | - O | - M |  |
| 10 |  |  | Ancillary Service Market |  |  |  |  |  |  |
| - 10 | - 0 |  | - Ancillary Service Instruction ID | - Unique identifier assigned to each ancillary service instruction. | - O | - O |  |  |  |
| - 10 | - 10 |  | - Ancillary Service Product Type | - Type of Ancillary Service product(s) the Resource is enrolling to provide | - M | - M | - O |  |  |
| - 10 | - 12 |  | - Ancillary Service Award - Start Time | - Start Time of the ancillary service Award |  | - M |  |  |  |
| - 10 | - 13 |  | - Ancillary Service Award - End Time | - End Time of the ancillary service Award |  | - M |  |  |  |
| - 10 | - 14 |  | - Ancillary Service Award - Interval Start Time | - Start Time of the ancillary service Interval within the ancillary service Award and the ancillary service Value of the Time Interval |  | - M |  |  |  |
| - 10 | - 15 |  | - Ancillary Service Award - Interval Value | - The awarded Value for the Time Interval within the ancillary service Award |  | - M |  |  |  |
| - 10 | - 17 |  | - Ancillary Service Schedule ID | - Unique identifier assigned to each ancillary service schedule. | - O | - O |  |  |  |
| - 10 | - 31 |  | - Reserve Pickup Flag | - Flag set if this is a Reserve Pickup schedule | - O | - O | - O |  |  |
| - 10 | - 32 |  | - Regulation Base Point | - Per-interval Regulation dispatch instruction | - M | - M | - O |  |  |
| - 10 | - 33 |  | - Base Load MW | - Level of Load at time of dispatch | - O | - O | - O |  | - M |
| 12 |  |  | Event and Performance |  |  |  |  |  |  |
| - 12 | - 10 |  | - Event ID | - Identifier assigned to the Event |  |  |  | - O | - M |
| - 12 | - 11 |  | - Deployment Type | - Type of Deployment |  |  |  | - M |  |
| - 12 | - 12 |  | - Event Day | - Operating Date of Start of Event |  |  |  | - M | - M |
| - 12 | - 13 |  | - Event Description | - Description of the Event |  |  |  | - M | - O |
| - 12 | - 14 |  | - Event Start Time | - Time of Start of deployment period for Event |  |  |  | - M | - M |
| - 12 | - 15 |  | - Operator Comment | - Additional Event information provided by the Operator |  |  |  | - O | - O |
| - 12 | - 16 |  | - Deployment MW | - Absolute or relative Deployment quantity |  |  |  | - M |  |
| - 12 | - 17 |  | - Event Status | - Status of the Event |  |  |  | - O | - O |
| - 12 | - 18 |  | - Event End Time | - Time of End of deployment period |  |  |  | - M | - M |
| - 12 | - 19 |  | - Event Modification Number | - A modification number for the event. |  |  |  | - O | - O |
| - 12 | - 111 |  | - Notification Acknowledgement | - This is an acknowledgement of the receipt of a DR notification or dispatch. It should include any necessary provisions for non-repudiation. |  |  |  | - O | - O |
| - 12 | - 112 |  | - Event Modification Reason Code | - The reason the event is being cancelled or modified. |  |  |  | - O | - O |
| - 12 | - 21 |  | - Baseline Dates | - Dates of days used to calculate the Energy Baseline |  |  |  |  | - O |
| - 12 | - 22 |  | - Baseline Exclusion Dates | - Dates of days Excluded from the calculation of the Energy Baseline |  |  |  |  | - O |
| - 12 | - 23 |  | - Energy Baseline Value | - Calculated Energy Baseline |  |  |  |  | - O |
| - 12 | - 24 |  | - Energy Baseline Timestamp | - Timestamp of Energy Baseline |  |  |  |  | - O |
| - 12 | - 30 |  | - Reporting Interval | - Interval size required for Reporting |  |  |  |  | - O |
| - 12 | - 31 |  | - Measurement Value Timestamp (CIM) | - The time when the value was last updated | - O | - O | - M |  | - M |
| - 12 | - 32 |  | - Measurement Value (CIM) | - Value of the interval reading | - O | - O | - M |  | - M |
| - 12 | - 33 |  | - Response Value | - Response | - O | - O | - O |  | - O |
| - 12 | - 991 |  | - Event & Performance Comments | - Comments associated with Event and Performance data |  |  |  | - O | - O |

#### 018-1.4.6	Measurement and Performance – Baseline

Overview: 	The Measurement and Performance process documents the steps to collect demand resource meter data and prepare the determinants for settlement.

Use Cases: 	All use cases where Baseline applies (see Figure 7).

- The process begins when the SP requests event meter data from the MA.

- The MA sends the event meter data to the SP.

- If the SP is required to calculate the baseline, the SP uses the event meter data to calculate the baseline.

  - The SP uses the baseline and event meter data to calculate the event reduction or response amount or event performance ratio.

  - The SP sends the calculated event performance and meter data to the SO.

  - The SO evaluates the calculated event performance and meter data.

    - If the calculated event performance data are complete, the SO uses the settlement determinants in the Settlements process.

    - If the calculated event performance data are incomplete, the SO sends an event data rejection to the SP.

- If the SO calculates the baseline, the SP sends the event meter data to the SO.

  - If the event meter data are complete, the SO uses the event meter data to calculate the baseline.

    - The SO uses the baseline and event meter data to calculate the event reduction or response amount or event performance ratio.

    - The SO uses the settlement determinants in the Settlements process.

  - If the event meter data are incomplete, the SO sends an event data rejection to the SP.

Activity Diagram and Data Flow:

Figure 7 - Measurement and Performance – Baseline

Note: Data elements for Measurement and Performance-Baseline are listed with Measurement and Performance-All Others in Table 12.

#### 018-1.4.7	Measurement and Performance - All Others

Overview: 	The Measurement and Performance process documents the steps to collect demand resource meter data and prepare the determinants for settlement.

Use Cases: 	All use cases where Meter Before/Meter After, Maximum Base Load, or Meter Generator Output applies (see Figure 8).

- The process begins when the SP collects event meter data from the MA for the specified performance evaluation method.

  - If the performance evaluation method is Meter Before/Meter After or Maximum Base Load, the MA sends the event meter data to the SP.

  - If the performance evaluation method is Meter Generator Output, the SP uses the data from the meter on the generator.

- If the SP is required to calculate performance, the SP uses the event meter data to calculate performance.

  - The SP uses the meter data to calculate the event reduction or response amount or event performance ratio.

  - The SP sends the calculated event performance and meter data to the SO.

  - The SO evaluates the calculated event performance and meter data.

    - If the calculated event performance data are complete, the SO uses the settlement determinants in the Settlements process.

    - If the calculated event performance data are incomplete, the SO sends an event data rejection to the SP.

- If the SO calculates performance, the SP sends the meter data to the SO.

  - If the event meter data are complete, the SO uses the event meter data to calculate performance.

    - The SO uses the event meter data to calculate the event reduction or response amount or event performance ratio.

    - The SO uses the settlement determinants in the Settlements process.

  - If the event meter data are incomplete, the SO sends an event data rejection to the SP.

Activity Diagram and Data Flow:

Figure 8 - Measurement and Performance – All Others

Table 12. Data Elements by Interaction Number: Measurement and Performance

| Legend: Cells showing an “M” (mandatory) are data elements that must contain some value when included in the interaction message. Cells showing an “O” (optional) are data elements that may be left blank if not used in a given interaction. |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Process | 4.0 Measurement & Performance |  |  |  |  |  |  |  |
|  |  |  |  | 4.1 Baseline |  |  |  | 4.2 Other |  |  |  |
|  |  |  | From | SP | SO | SP | SO | SP | SO | SP | SO |
|  |  |  | To | SO | SP | SO | SP | SO | SP | SO | SP |
|  |  |  |  | 4.1.1 | 4.1.2 | 4.1.3 | 4.1.4 | 4.2.1 | 4.2.2 | 4.2.3 | 4.2.4 |
|  |  |  | Interaction | Meter Data | Meter Data Rejection | Calc. Perf. and Meter Data | Meter Data Rejection | Meter Data | Meter Data Rejection | Calc. Perf. and Meter Data | Meter Data Rejection |
| Element |  |  | Short Description |  |  |  |  |  |  |  |  |
| 0 |  | Common |  |  |  |  |  |  |  |  |  |
| - 0 | - 10 | - Submittal Date | - Timestamp for the sender's use | - O | - O | - O | - O | - O | - O | - O | - O |
| - 0 | - 11 | - Submitted By | - User ID of submitter | - O | - O | - O | - O | - O | - O | - O | - O |
| - 0 | - 12 | - Submitted Error | - ID of submission error detected | - O | - O | - O | - O | - O | - O | - O | - O |
| - 0 | - 13 | - Rejection Code Type | - Type of rejection message |  | - M | - O | - M |  | - M | - O | - M |
| - 0 | - 14 | - Rejection Code Value | - Code referring to the reason for a rejection message |  | - M | - O | - M |  | - M | - O | - M |
| - 0 | - 20 | - NERC CIP Security - Availability | - CIP Security Classification for Availability | - M | - M | - M | - M | - M | - M | - M | - M |
| - 0 | - 21 | - NERC CIP Security - Confidentiality | - CIP Security Classification for Confidentiality | - M | - M | - M | - M | - M | - M | - M | - M |
| - 0 | - 22 | - NERC CIP Security - Integrity | - CIP Security Classification for Integrity | - M | - M | - M | - M | - M | - M | - M | - M |
| 1 |  | General Asset/Resource |  |  |  |  |  |  |  |  |  |
| - 1 | - 10 | - Service Location ID | - Identifier assigned to the Service Location | - M | - M | - M | - M | - M | - M | - M | - M |
| - 1 | - 11 | - Service Location Name | - Name of the Service Location | - M | - M | - M | - M | - M | - M | - M | - M |
| - 1 | - 12 | - Asset ID | - The unique identifier of the asset | - O | - O | - O | - O | - O | - O | - O | - O |
| - 1 | - 13 | - Asset Name | - The name of the asset | - O | - O | - O | - O | - O | - O | - O | - O |
| - 1 | - 20 | - Resource ID | - Identifier assigned to the Resource | - M | - M | - M | - M | - M | - M | - M | - M |
| - 1 | - 21 | - Resource Name | - Name of the Resource | - M | - O | - O | - O | - O | - O | - O | - O |
| - 1 | - 30 | - Asset Group ID | - The identifier of a group of assets | - M | - M | - M | - M | - M | - M | - M | - M |
| - 1 | - 31 | - Asset Group Name | - Name of the aggregated group of assets | - O | - O | - O | - O | - O | - O | - O | - O |
| 2 |  | Location |  |  |  |  |  |  |  |  |  |
| - 2 | - 30 | - Zone ID | - Identifier assigned to the Zone in which the Service Location is located | - M | - M | - M | - M | - M | - M | - M | - M |
| - 2 | - 31 | - Zone | - Name of the Zone in which the Service Location is located | - O | - O | - O | - O | - O | - O | - O | - O |
| - 2 | - 32 | - Zone Type | - Type of Zone | - O | - O | - O | - O | - O | - O | - O | - O |
| - 2 | - 40 | - Electrical Node ID | - Identifier assigned to the Electrical Node at which the Service Location is connected | - O | - M | - M | - M | - O | - M | - M | - M |
| - 2 | - 41 | - Electrical Node Name | - Name of the Electrical Node at which the Service Location is connected | - O | - O | - O | - O | - O | - O | - O | - O |
| - 2 | - 42 | - Electrical Node Type | - Type of Electrical Node at which the Service Location is attached | - O |  |  |  | - O |  |  |  |
| - 2 | - 43 | - PNode | - Name of the Price Node associated with the Service Location | - O |  |  |  | - O |  |  |  |
| - 2 | - 44 | - PNode ID | - Identifier assigned to the Price Node associated with the Service Location | - O |  |  |  | - O |  |  |  |
| 4 |  | Grid Connection |  |  |  |  |  |  |  |  |  |
| - 4 | - 10 | - Loss Factor Type | - Type of Loss Factor |  |  | - M |  |  |  | - M |  |
| - 4 | - 11 | - Loss Factor Value | - Loss Factor |  |  | - M |  |  |  | - M |  |
| 5 |  | Business Entity Relationships |  |  |  |  |  |  |  |  |  |
| - 5 | - 11 | - Service Provider ID | - Identifier assigned to the Service Provider | - O | - O | - O | - O | - O | - O | - O | - O |
| - 5 | - 12 | - Service Provider Name | - Name of the Service Provider | - O | - O | - O | - O | - O | - O | - O | - O |
| - 5 | - 20 | - Transmission/Distribution Service Provider ID | - Identifier assigned to the Transmission/Distribution Service Provider | - O | - O |  |  | - O | - O |  |  |
| - 5 | - 21 | - Transmission/Distribution Service Provider Name | - Name of the Transmission/Distribution Service Provider | - O | - O |  |  | - O | - O |  |  |
| - 5 | - 22 | - Transmission/Distribution Service Provider Account Number | - Transmission/Distribution Service Provider's account number for the Resource | - O | - O |  |  | - O | - O |  |  |
| - 5 | - 44 | - Retail Rate | - Retail Rate |  |  | - O |  |  |  | - O |  |
| - 5 | - 60 | - Meter Authority ID | - Identifier assigned to the Metering Authority | - O | - O | - O | - O | - O | - O | - O | - O |
| - 5 | - 61 | - Meter Authority Name | - Name of the Metering Authority | - O | - O | - O | - O | - O | - O | - O | - O |
| - 5 | - 70 | - Scheduling Entity ID | - Identifier assigned to the Scheduling Entity | - M | - O | - O | - O | - O | - O | - O | - O |
| - 5 | - 71 | - Scheduling Entity Name | - Name of the Scheduling Entity | - O | - O | - O | - O | - O | - O | - O | - O |
| - 5 | - 80 | - Designated Dispatch Entity ID | - Identifier assigned to the Designated Dispatch Entity | - M | - O | - O | - O | - O | - O | - O | - O |
| - 5 | - 81 | - Designated Dispatch Entity Name | - Name of the Designated Dispatch Entity | - O | - O | - O | - O | - O | - O | - O | - O |
| 6 |  | Device and Qualification |  |  |  |  |  |  |  |  |  |
| - 6 | - 20 | - Meter ID | - Identifier assigned to the Meter | - M | - M | - M | - M | - M | - M | - M | - M |
| - 6 | - 2293 | - Measurement Interval | - Interval of time between Measurement readings | - O |  |  |  | - O |  |  |  |
| - 6 | - 45 | - Breaker Status During Event | - Status of the Breaker for the Service Location during an Emergency Event | - O | - O | - O | - O | - O | - O | - O | - O |
| - 6 | - 46 | - Wholesale Delivery Point Status | - Flag set if the Service Location is metered at the Wholesale level | - O | - O | - O | - O | - O | - O | - O | - O |
| - 6 | - 47 | - Private Use Network | - Flag set if the Resource is an Electric Network connected to the transmission grid | - O | - O | - O | - O | - O | - O | - O | - O |
| - 6 | - 51 | - UFR Settings | - Setting of the Under-Frequency Relay | - O | - O | - O | - O | - O | - O | - O | - O |
| - 6 | - 52 | - Load Resource Control Device | - Type of Control Device at a Service Location or deployed by a Response Method | - O | - O | - O | - O | - O | - O | - O | - O |
| 7 |  | Market/Program Enrollment |  |  |  |  |  |  |  |  |  |
| - 7 | - 10 | - Program ID | - Program Identifier | - O | - O | - O | - O | - O | - O | - O | - O |
| - 7 | - 11 | - Program Name | - Name of the Program | - O | - O | - O | - O | - O | - O | - O | - O |
| - 7 | - 12 | - Market | - Type of wholesale market | - O | - O | - O | - O | - O | - O | - O | - O |
| - 7 | - 13 | - Market Product | - Market Product | - O | - O | - O | - O | - O | - O | - O | - O |
| - 7 | - 40 | - Response Method Type | - Type of Response Method |  |  | - O |  |  |  | - O |  |
| - 7 | - 41 | - Response Method ID | - Identifier assigned to the Response Method |  |  | - O |  |  |  | - O |  |
| - 7 | - 42 | - Response Method Name | - Name of the Response Method |  |  | - O |  |  |  | - O |  |
| - 7 | - 51 | - Performance Evaluation Method | - Method used to Evaluate the Performance of a Resource |  |  | - O |  |  |  | - O |  |
| - 7 | - 991 | - Market Enrollment Comments | - Comments associated with the Enrollment |  | - O |  | - O |  | - O |  | - O |
| 11 |  | Capacity Market |  |  |  |  |  |  |  |  |  |
| - 11 | - 30 | - Capacity Obligation Period Type | - Type of Capacity Obligation Time Period within which the Resource is obligated | - O | - O | - O | - O | - O | - O | - O | - O |
| - 11 | - 31 | - Capacity Obligation Period | - Period of time for which the Capacity Resource is obligated | - O | - O | - O | - O | - O | - O | - O | - O |
| - 11 | - 40 | - Minimum Payment Nomination | - Strike price for the economic dispatch of the energy component of the capacity program |  |  | - O |  |  |  | - O |  |
| - 11 | - 41 | - Capacity Supply Obligation | - Capacity Obligation |  |  | - O |  |  |  | - O |  |
| - 11 | - 50 | - Capacity Reference ID | - Identifier assigned to the External System award |  |  | - O |  |  |  | - O |  |
| - 11 | - 991 | - Capacity Comments | - Comments associated with the Capacity market |  |  | - O |  |  |  | - O |  |
| 12 |  | Event and Performance |  |  |  |  |  |  |  |  |  |
| - 12 | - 10 | - Event ID | - Identifier assigned to the Event | - M | - M | - O | - M | - M | - M | - O | - M |
| - 12 | - 11 | - Deployment Type | - Type of Deployment |  |  | - O |  |  |  | - O |  |
| - 12 | - 12 | - Event Day | - Operating Date of Start of Event |  |  | - M |  |  |  | - M |  |
| - 12 | - 14 | - Event Start Time | - Time of Start of deployment period for Event | - O | - O | - M | - O | - O | - O | - M | - O |
| - 12 | - 16 | - Deployment MW | - Absolute or relative Deployment quantity |  |  | - O |  |  |  | - O |  |
| - 12 | - 17 | - Event Status | - Status of the Event |  |  | - O |  |  |  | - O |  |
| - 12 | - 18 | - Event End Time | - Time of End of deployment period | - O | - O | - M | - O | - O | - O | - M | - O |
| - 12 | - 21 | - Baseline Dates | - Dates of days used to calculate the Energy Baseline | - O | - O | - O | - O | - O | - O | - O | - O |
| - 12 | - 22 | - Baseline Exclusion Dates | - Dates of days Excluded from the calculation of the Energy Baseline | - O | - O | - O | - O | - O | - O | - O | - O |
| - 12 | - 23 | - Energy Baseline Value | - Calculated Energy Baseline | - O | - O | - O | - O | - O | - O | - O | - O |
| - 12 | - 24 | - Energy Baseline Timestamp | - Timestamp of Energy Baseline | - O | - O | - O | - O | - O | - O | - O | - O |
| - 12 | - 30 | - Reporting Interval | - Interval size required for Reporting | - O | - O | - O | - O | - O | - O | - O | - O |
| - 12 | - 31 | - Measurement Value Timestamp (CIM) | - The time when the value was last updated | - M | - M | - M | - O | - M | - O | - M | - O |
| - 12 | - 32 | - Measurement Value (CIM) | - Value of the interval reading | - M | - M | - M | - O | - M | - O | - M | - O |
| - 12 | - 33 | - Response Value | - Response | - O | - O | - O | - O | - O | - O | - O | - O |
| - 12 | - 34 | - Measurement Value Type | - The type of measurement value. | - O |  | - O |  | - O |  | - O |  |
| - 12 | - 991 | - Event & Performance Comments | - Comments associated with Event and Performance data |  | - O | - O | - O |  | - O | - O | - O |

018-A	Appendix A – Entity-Relationship Model

The following terms and definitions correspond to a set of actor and objects within the overall information model for demand response.  Figure 1A contains a data model used to illustrate the entity-relationships within the model. Abbreviations/Acronyms correspond to the IDs shown in the figure.

Proper (and Improper) Usage of the Entity-Relationship Model.    The entity-relationship model is used to show cardinality among objects in the model, for example every Resource belongs to one and only one Service Provider and a Service Location contains one or more Service Delivery Points.  The entity-relationship model does not provide use case information; actors are objects in the model and do not “act” in the model.  The entity-relationship model does not imply process.  For example, a Service Provider is associated with many Resources; however the enrollment of those Resources may be managed by a System Operators or a Utility Distribution Operator.  Both use cases and process maps are separate components of the Business Practice Standards.

Reading Crow’s Foot Notation.    Objects in the model which share a relationship are connected with a cardinality line.  Each end of the cardinality line contains a Crow’s Foot notation, as documented in the legend of the figure.  The four notations utilized are “exactly-one”, “one-or-more”, “zero-or-one”, and “zero-or-more”.  The cardinality line is bi-directional; meaning it can be read in two directions.  For example: a Service Location is related to one-or-more Service Delivery Points (reading top to bottom) and every Service Delivery Point is related to exactly-one Service Location (reading bottom to top).

Optional Entities, Roles and Objects.  The entity-relationship model is designed to support multiple business models and not every business model will require all entities, roles and/or objects to function.  Therefore, entities, roles and objects in the model are considered optional.  For example, a Utility Distribution Operator may design a Demand Response program which requires the definition of Resources and Service Locations, but does not require Asset Groups and Assets.

Applicability to Retail and Wholesale.  The entity-relationship model is applicable to both wholesale and retail.  When using the model for wholesale, the typical applicable business entities are 2.1 (System Operator) and 2.3 (Market Participant), while in retail markets, the parallel business entities are 2.3 (Utility Distribution Operator) and 2.4 (Utility Customers).  Other than the swapping of the two pairs of terms, the models are identical, including the names of and relationships among objects.

Figure 1A. Entity-Relationship Model for Smart Grid Use Cases

018-B	Appendix B – Use Case Combinations

The following table is a complete list of use case combinations referenced in these Business Practice Standards.

# Supporting Documentation

a.  Description of Request:

- WEQ Annual Plan Item 6(c) (ii) – “Develop Requirements and Use Cases for PAP 09 – Demand Response/Distributed Energy Resources” – Phase Two.

b.  Description of Recommendation:

This recommendation associates the data elements to the use cases developed during the Phase One effort.

c.  Business Purpose:

This recommendation has been developed in response to a request to include a Phase two effort for PAP-09 to associate data elements with the use cases developed in Phase 1 for standard DR and DER signals.  Additionally, the recommendation addresses the directive from the WEQ Executive Committee for “the Smart Grid Standards Subcommittee to review the Executive Summary and Introduction to reconsider language that may not be appropriate for business practices long-term in their Phase 2 work.”

d.  Commentary/Rationale of Subcommittee(s)/Task Force(s):

NAESB Smart Grid Standards Subcommittee Meeting Notes/Documents:

- June 10, 2010 Meeting Notes – To be posted

- June 17, 2010 Meeting Notes –  http://www.naesb.org/pdf4/smart_grid_ssd061710notes.doc

- July 1, 2010 Meeting Notes – To be posted

- July 13-14, 2010 Meeting Notes – To be posted

- July 22, 2010 Meeting Notes – To be posted

- August 5, 2010 Meeting Notes –http://www.naesb.org/pdf4/smart_grid_ssd080510notes.doc

- August 12, 2010 Meeting Notes – http://www.naesb.org/pdf4/smart_grid_ssd081210notes.doc

- August 26, 2010 Meeting Notes – http://www.naesb.org/pdf4/smart_grid_ssd082610notes.doc

- September 9, 2010 Meeting Notes – http://www.naesb.org/pdf4/smart_grid_ssd090910notes.doc

- September 16, 2010 Meeting Notes – http://www.naesb.org/pdf4/smart_grid_ssd091610notes.doc

- September 23, 2010 Meeting Notes – To be posted

- October 7, 2010 Meeting Notes – To be posted

http://collaborate.nist.gov/twiki-sggrid/bin/view/_SmartGridInterimRoadmap/PriorityActionPlans

The Actor ID shown in the table refers to the item number of the corresponding actor or related object in the Entity-Relationship Diagram provided in Appendix A. Definitions of the actors and related objects are included in WEQ-000.