Introduction

The Regional Evaluation, Measurement and Verification Forum (‘the Forum’) submits these formal comments and recommendations on the Draft NAESB Business Practice Standards for Measurement and Verification of Energy Efficiency Products.  These comments were assembled with input from energy efficiency stakeholders (“Stakeholders”), all of whom reserve the right to separately submit their own comments directly to NAESB.

Section WEQ.020.3.11.1.9:  Measurement Equipment Requirements

“Any measurement or monitoring equipment for proxy variables that does not directly measure electrical demand, including but not limited to voltage, current, temperature, flow rates and operating hours, shall have an accuracy within ± 2% unless otherwise provided in the Governing Documents.”

Requirement #9, shown above, presents the single most challenging M&V requirement to the evaluation community. Stakeholder concerns with literal interpretation of requirement #9 revolve around the following central issues:

- Requirement #9 is superfluous because the overall accuracy of the Demand Reduction Value is already addressed in Section WEQ.020.3.8. (see discussion below).

- Accuracy criteria for “any” measured data used in the development of demand impacts would in some cases substantially increase the cost of compliance because additional resources are required to document the accuracy of measurements (e.g.TMY2 temperature, manufacturing production, and energy management system (EMS) temperature, pressure and flow data).  If after considerable on-site data collection effort it is determined that the sampled site does not comply with the stated criteria, the site would have to be dropped from the sample resulting in significant extra costs.

- The requirement could have the effect of discouraging  the use of these economical data resources which would in many instances necessitate recourse to a different methodological approach such as IPMVP Option A (WEQ.020.3.6.1.1) that allows for the use of stipulated factors  that could produce a calculated demand estimate that is less accurate than one based on the excluded data.

- Evaluation Contractors typically find that measurement error is a relatively insignificant component of the total variability of the Demand Reduction Value because its contribution to measured precision is small in comparison to sampling variability.

- Prescribed accuracy for measurement of “proxy variables” extends beyond the hardware-specific scope of Section WEQ.020.3.11. The provisions of this section are generally focused upon the accuracy of electrical metering equipment designed to directly measure electrical demand or its components (e.g. voltage and current). Other types of measurements (e.g. temperature, flow and operating hours) are derived from equipment which is not designed to calculate electrical demand.

The Stakeholders submit that the accuracy and precision of proxy variable measurements are effectively addressed in Section WEQ.020.3.8: Statistical Significance.  Section WEQ.020.3.8.6: Sample Bias Control requires that the Project Sponsor describe methods to control sources of potential bias due to “error in measuring variables”. Proper calibration of measurement equipment, as required in Section WEQ.020.3.11.1.12, serves to effectively mitigate systematic bias in measured data. Section WEQ.020.3.8.2 requires that sample estimates achieve a standard of precision of no less than 80% confidence level and 10% relative precision.  Proxy variable measurements are typically derived from a statistical sample of buildings drawn from the population that in aggregate defines the demand resource being measured.

The achieved statistical precision of an estimate derived from sample data is a measure of the magnitude of the combined effect of random sampling error and random measurement error on the estimated value. The statistical precision of the estimate, and therefore the magnitude of the error, is controlled by designing a sample that is expected to achieve the required precision. Evaluation contractors base their judgment of the coefficient of variation (CV) employed in the sample design on their experience with similar measurements in comparable populations. CVs derived from sample data collected in previous studies implicitly incorporate estimates of the random measurement error in those studies.

For these reasons, compliance with the accuracy and precision requirements in Section WEQ.020.3.8 will account for both the sampling and the measurement errors that normally occur in the measurement and verification of demand reduction. Therefore compliance with the Section WEQ.020.3.8 requirements is sufficient to establish the overall accuracy of the Demand Reduction Value, rendering the additional requirements of Section WEQ.020.3.11.1.9 superfluous.

Recommendation:  Stakeholders respectfully request deletion of WEQ.020.3.11.1.9 on the grounds that 1)  it is unnecessary because proxy measurement error is effectively addressed in Section WEQ.020.3.8; 2) literal interpretation of  WEQ.020.3.11.1.9  would substantially increase the cost of compliance and 3) potentially exclude use of many invaluable data resources currently used as best practice to inform calculated savings data; 4) measurement error is typically a relatively insignificant component of the total variability of the Demand Reduction Value because its contribution to measured precision is small in comparison to sampling variability; and 5) WEQ.020.3.11.9 is beyond the hardware-specific scope of Section WEQ.020.3.11. Moreover, the Stakeholders note that after careful consideration of reasons similar to those presented in these comments, PJM excluded this provision from its M&V Standards.