1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| X | Accept as requested | X | Change to Existing Practice |
| --- | --- | --- | --- |
|  | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

2.  TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
|  | Initiation |  | Initiation |
| X | Modification | X | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
|  | Principle |  | Principle |
|  | Definition |  | Definition |
|  | Business Practice Standard |  | Business Practice Standard |
|  | Document |  | Document |
|  | Data Element |  | Data Element |
|  | Code Value |  | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
|  | Business Process Documentation |  | Business Process Documentation |

3.  RECOMMENDATION

SUMMARY:

Provide consistent definitions for terms included in the Business Practice Standards.

Recommended Standards:

In addition to the changes proposed below, approval by the EC of this recommendation will also include incorporation of any changes and deletions to abbreviations, acronyms and definitions approved by the EC during the period October 27, 2009 through the approval date of this recommendation. Any new abbreviations, acronyms and definitions approved by the EC after October 27, 2009 will be included in the Business Practice Standards WEQ-000 and will not be defined in the individual Business Practice Standards.

New Business Practice Standards WEQ-000

Abbreviations, Acronyms, and Definition of Terms

Introduction

“Abbreviations, Acronyms, and Definition of Terms” provides a common location for all abbreviations, acronyms, and definitions of terms that are referenced in the Business Practice Standards.  Since these abbreviations, acronyms, and defintions are not requirements, they are not enumerated in this Business Practice Standards WEQ-000, similar to Business Practice Standards WEQ-003.

Applicability

Refer to specific Business Practice Standards for applicability.

000-1	ABBREVIATIONS AND ACRONYMS

The abbreviations and acronymns listed in the following table are used in the Business Practice Standards and shall have the meaning specified in the table.

| Abbreviation / Acronym | Meaning |
| --- | --- |
| ACE | Area Control Error |
| AFC | Available Flowgate Capability |
| AGC | Automatic Generation Control |
| ASCII | American Standard Code for Information Interchange |
| ASN.1 | Abstract Syntax Notation One |
| ATC | Available Transfer Capability |
| ATCID | Available Transfer Capability Implementation Document |
| BA | Balancing Authority |
| BAA | Balancing Authority Area |
| CBM | Capacity Benefit Margin |
| CBMID | Capacity Benefit Margin Implementation Document |
| CCO | Conditional Curtailment Option |
| CEII | Critical Energy Infrastructure Information |
| CF | Coordinated Flowgate |
| CFR | Code of Federal Regulations |
| CGI | Common Gateway Interface |
| CP | Certificate Policy |
| CRL | Certificate Revocation List |
| CRLF | Carriage Return/Line Feed |
| CSV | Comma Separated Value |
| DC | Direct Current |
| DNR | Designated Network Resource |
| DN | Distinguished Name |
| DNS | Domain Name Service |
| DoS | Denial of Service |
| Dth | Dekatherm |
| DUNS | Data Universal Numbering System |
| E-commerce | Electronic commerce |
| eFiling | Electronic Filing |
| eLibrary | Electronic Library |
| E-mail | Electronic mail |
| eRegistered | Electronic Registered |
| e-Tag | Electronic Tag |
| eTariff | Electronic Tariff |
| EIDE | Electric Industry Data Exchange |
| EIR | Electric Industry Registry |
| ERCOT | Electric Reliability Council of Texas |
| ETC | Existing Transmission Commitments |
| FDDI | Fiber Distributed Data Interface |
| FERC | Federal Energy Regulatory Commission |
| FIPS | Federal Information Processing Standards |
| FOTC | Financially Obligated Transmission Customer |
| FTAA | Funds Transfer Agent Agreement |
| FTP | File Transfer Protocol |
| GCA | Generating Control Area |
| GLDF | Generator-to-Load Distribution Factor |
| GOP | Generator Operator |
| GPE | Generator-Providing Entity |
| GPS | Global Positioning System |
| GSF | Generator Shift Factor |
| HTML | Hypertext Markup Language |
| HTTP | Hypertext Transport Protocol |
| Hz | Hertz |
| IA | Interchange Authority |
| IDC | Interchange Distribution Calculator |
| InterNIC | Internet Network Information Center |
| IP | Internet Protocol |
| IPCP | Internet Protocol Control Protocol |
| IROL | Interconnection Reliability Operating Limit |
| ISO | Independent System Operator |
| JOU | Jointly Owned Units |
| kV | Kilovolt |
| LDAP | Lightweight Directory Access Protocol |
| LMP | Locational Marginal Pricing |
| LSE | Load-Serving Entity |
| LSF | Load Shift Factor |
| MIME | Multipurpose Internet Mail Extensions |
| MRO | Midwest Reliability Organization |
| MVar | Megavolt Ampere Reactive |
| MW | Megawatt |
| MWh | Megawatt Hour |
| NAESB | North American Energy Standards Board |
| NERC | North American Electric Reliability Corporation |
| NHM | Next Hour Market |
| NIST | National Institute of Standards and Technology |
| NITS | Network Integration Transmission Service |
| NL | Native Load |
| NNL | Network and Native Load Calculations |
| NTP | Network Time Protocol |
| OASIS | Open Access Same-Time Information System |
| OATT | Open Access Transmission Tariff |
| PDF | Portable Document Format |
| PKI | Public Key Infrastructure |
| POD | Point of Delivery |
| POP | Post Office Protocol |
| POR | Point of Receipt |
| PPO | Power Plant Operator |
| PPP | Point-to-Point Protocol |
| PSE | Purchasing-Selling Entity |
| PTP | Point-to-Point Transmission Service |
| RA | Registration Authority |
| RC | Reliability Coordinator |
| RCIS | Reliability Coordinator Information System |
| RFC | Request for Comments |
| RFI | Request for Interchange |
| RTO | Regional Transmission Organization |
| S&CP | Standards and Communication Protocols |
| SLIP | Serial Line Internet Protocol |
| SMTP | Simple Messaging Transfer Protocol |
| SNMP | Simple Network Management Protocol |
| SOC | Secretary of the Commission (FERC) |
| SOL | System Operating Limit |
| SPP | Southwest Power Pool |
| SSL | Secure Sockets Layer |
| TCP/IP | Transmission Control Protocol/Internet Protocol |
| TDF | Transfer Distribution Factor |
| TFC | Total Flowgate Capacity |
| TLR | Transmission Loading Relief |
| TO | Transmission Owner |
| TPSE | Transmission Purchase Selling Entity |
| TRM | Transmission Reliability Margin |
| TRMID | Transmission Reliability Margin Implementation Document |
| TSIP | Transmission Services Information Provider |
| TSIN | Transmission System Information Network |
| TTC | Total Transfer Capability |
| URL | Uniform Resource Locator |
| USF | Unschedule Flow Reduction Procedure |
| WECC | Western Electric Coordinating Council |
| WECCNet | Western Electric Coordinating Council Network |
| WEQ | Wholesale Electric Quadrant |
| WGQ | Wholesale Gas Quadrant |
| WWW | World Wide Web |
| WWVB | See definition for WWVB |
| XML | Extensible Markup Language |
| XSD | XML Schema Definition Language |

000-1.1	Abbreviations and Acronyms Do Not Use

The following abbreviations and acronymns should not be used in the Business Practice Standards.  Though the abbreviations and acronyms are commonly accepted, they may have multiple meanings when used in these standards.

| Abbreviation / Acronym | Meanings |
| --- | --- |
| CA | Control Area Certificate Authority |
| TSP | Transmission Service Provider Transportation Service Provider |
| CPS | Certificate Practice Statement Control Performance Standard |

000-2	DEFINTION OF TERMS

The terms listed in the following table are used in the Business Practice Standards and shall have the definitions specified in the table.

| TERM | DEFINITION |
| --- | --- |
| Actual Daily Native Peak Load | Shall have the meanings as defined in Business Practice Standards WEQ-001-17. |
| Actual Daily System-Wide Peak Load | Shall have the meanings as defined in Business Practice Standards WEQ-001-17. |
| Actual Daily Zonal Load | Shall have the meanings as defined in Business Practice Standards WEQ-001-17. |
| Affiliate | (1) For any exempt wholesale generator, as defined under section 32(a) of the Public Utility Holding Company Act of 1935, as amended, the same as provided in section 214 of the Federal Power Act; and (2) For any other entity, the term Affiliate has the same meaning as given in Title 18 CFR 161.2(a). |
| Applicant | An authorized individual of a registered End Entity that may submit applications for issuance of Certificates to an Authorized Certification Authority for the End Entity. |
| Approval Entity | An entity that has approval rights for Arranged Interchange. |
| Area Control Error (ACE) | The instantaneous difference between a BA’s Net Actual Interchange and Net Interchange Schedule, taking into account the effects of Frequency Bias and correction for meter error. |
| Arranged Interchange | The state where the IA has received the Interchange information (initial or revised). |
| Assignee | An Eligible Transmission Customer that receives PTP rights from a Reseller either through a Resale or a Transfer. |
| Assignment Reference Number | The unique accounting number that marks each request for purchase of transmission or ancillary services. |
| Authorized Certification Authority | A certification authority that complies with the Business Practice Standards WEQ-012, has met all terms and conditions and executed all requirements as set forth by the NAESB certification program for the Business Practice Standards WEQ -012, and is registered in the EIR. |
| Automatic Generation Control (AGC) | Equipment that automatically adjusts generation in a BAA from a central location to maintain the BA’s Interchange Schedule plus Frequency Bias. AGC may also accommodate automatic Inadvertent Interchange payback and Time Error Correction. |
| Balancing Authority (BA) | The responsible entity that integrates resource plans ahead of time, maintains load-interchange-generation balance within a BAA, and supports Interconnection frequency in real time. (Note: This definition also applies to NAESB WGQ Standard Number 0.3.15) |
| Balancing Authority Area (BAA) | The collection of generation, transmission, and loads within the metered boundaries of the BA. The BA maintains load-resource balance within this area. |
| Biennial Reassessment | A study performed by the Transmission Provider to reassess the provision of service under a CCO Reservation and service agreement once every two years to determine if (i) the Curtailment condition(s) in the service agreement needs to be modified for purposes of maintaining reliability, or (ii) the CCO Reservation can no longer be reliably provided. |
| Book of Flowgates | The master list of Flowgates used by the IDC. |
| Bulk Electric System | The electrical generation resources, transmission lines, interconnections with neighboring systems, and associated equipment, generally operated at voltages of 100 kV or higher. Radial transmission facilities serving only load with one transmission source are generally not included in this definition. |
| Business Practice | Refers collectively to any practices adopted by the Transmission Provider as defined in their OATT, or Transmission Provider specific practices or requirements. |
| Business Practice Standards | Refers to the collection of ratified NAESB WEQ practices or requirements. |
| Capacity Available to Redirect | The granted capacity of the Parent Reservation at the time of Transmission Customer confirmation (CAPACITY_GRANTED) less all confirmed reassignments (e.g., Resales), confirmed Redirects on a firm basis, confirmed Redirects on a non-firm basis, displacements, and approved schedules. |
| Certificate | A digital document that typically includes the public key, information about the identity of the party holding the corresponding private key, the operational periods of the Certificate, and the Issuing Certification Authority’s own digital signature. |
| Certificate Policy (CP) | A named set of rules that indicates the applicability of a Certificate to a particular community and/or class of application with common security requirements. For example, a particular CP might indicate applicability of a type of Certificate to the authentication of electronic data Interchange Transactions for the trading of goods within a given price range. |
| Certificate Revocation List (CRL) | A list of Certificate serial numbers which have been revoked, are no longer valid, and should not be relied upon by any system user. |
| Certification Path | An ordered sequence of Certificates which, together with the public key of the initial object in the path, can be processed to obtain that of the final object in the path. |
| Certification Practice Statement | A statement of the practices which a certification authority employs in issuing Certificates. |
| Checkout Partners | Entities that perform a checkout process. Most commonly two adjacent BAs checking net Interchange. It might also be two marketers checking sales and purchases, or a Transmission Customer checking schedules with a Transmission Service Provider. |
| Commission | The FERC, or other entity which has regulating authority over a given Transmission Provider. |
| Conditional Curtailment Option (CCO) | An option subject to either a Number-of-Hours Criteria or a System-Conditions Criteria that may be specified in a service agreement for long-term firm PTP when a Transmission Provider determines that it cannot accommodate a request because of insufficient transfer capability. |
| Conditional Curtailment Option (CCO) Reservation | A Transmission Service reservation defined by a long-term firm PTP agreement wherein the Transmission Customer has chosen the CCO. |
| Conditional Curtailment Priority Level | A Curtailment priority level associated with a CCO Reservation equal to the priority level of Secondary Network Transmission Service. |
| Confirmed Interchange | The state where the IA has verified the Arranged Interchange. |
| Constrained Facility | A transmission facility (line, transformer, breaker, etc.) that is approaching, is at, or is beyond its SOL or IROL. |
| Constrained Flowgate | A Flowgate that is approaching, is at, or is beyond SOL or IROL. |
| Constraint | A limitation placed on Interchange Transactions that flow over a Constrained Facility or Flowgate. |
| Contract Path | A predetermined Transmission Service electrical path between contiguous Transmission Providers established for scheduling and commercial settlement purposes that represents the continuous flow of electrical energy between the parties to a transaction. |
| Control Area | An electric power system or combination of electric power systems to which a common AGC scheme is applied in order to: - match, at all times, the power output of the generators within the electric power system(s) and capacity and energy purchased from entities outside the electric power system(s), with the load within the electric power system(s); - maintain scheduled interchange with other Control Areas, within the limits of good utility practice; - maintain the frequency of the electric power system(s) within reasonable limits in accordance with good utility practice; and - provide sufficient generating capacity to maintain operating reserves in accordance with good utility practice. |
| Control Performance Standard | The reliability standard that sets the limits of a BA’s ACE over a specified time period. |
| Curtailment | A reduction in the scheduled capacity or energy delivery of an Interchange Transaction. |
| Curtailment Threshold | The minimum TDF which, if exceeded, will subject an Interchange Transaction to Curtailment to relieve a transmission facility Constraint. |
| Data Element | Refers generically, by either full name or alias, to a Data Element defined in Business Practice Standards WEQ-003 or WEQ-014. No distinction between the use of full name or alias is intended. |
| Denial of Service (DoS) | The intentional or unintentional degradation of OASIS performance that impacts all customer interactions with OASIS by consuming cyber resources. |
| Distribution Factors | The portion of an Interchange Transaction, typically expressed in per unit that flows across a transmission facility (Flowgate). |
| Dynamic Schedule | A telemetered reading or value that is updated in real time and used as a schedule in the AGC/ACE equation and the integrated value of which is treated as a schedule for Interchange accounting purposes. Commonly used for scheduling jointly owned generation to or from another BAA. |
| Eastern Interconnection | One of the major electric system networks in North America. |
| Electric Reliability Council of Texas (ERCOT) Interconnection | One of the major electric system networks in North America. |
| Eligible Transmission Customer | Shall have the same meaning as the term Eligible Customer defined in the FERC Pro Forma OATT. |
| End Entity | A registered business entity or other recognized organization to which Certificates are issued by one or more Authorized Certification Authorities. |
| Electronic Tag (e-Tag) | Information describing a physical Interchange Transaction and its participants. |
| Electronic Tariff (eTariff) Submission Data Dictionary | eTariff Submission Data Dictionary as defined in Business Practice Standards WEQ-014 Appendix A. |
| Electronic Tariff (eTariff) Submission Implementation Guide | eTariff Submission Implementation Guide as defined in Business Practice Standards WEQ-014 Appendix A. |
| Facility Rating | The maximum or minimum voltage, current, frequency, or real or reactive power flow through a facility that does not violate the applicable equipment rating of any equipment comprising the facility. |
| Financially Obligated Transmission Customer (FOTC) | The Transmission Customer financially obligated to the Transmission Provider for Transmission Service (i.e., service procured either through direct purchase from the Transmission Provider, Reseller, or through a Transfer of transmission rights). |
| Firm Transmission Service | The highest quality service offered to Transmission Customers under a filed rate schedule that anticipates no planned interruption. Firm Transmission Service includes firm PTP and firm NITS. |
| Flowgate | - A portion of the transmission system through which the IDC calculates the power flow from Interchange Transactions. - A mathematical construct, comprised of one or more monitored transmission facilities and optionally one or more contingency facilities, used to analyze the impact of power flows upon the Bulk Electric System. |
| Forecasted Daily Native Peak Load | Shall have the meanings as defined in Business Practice Standards WEQ-001-17. |
| Forecasted Daily System-Wide Peak Load | Shall have the meanings as defined in Business Practice Standards WEQ-001-17. |
| Forecasted Daily Zonal Load | Shall have the meanings as defined in Business Practice Standards WEQ-001-17. |
| Frequency Bias | A value, usually expressed in MWs per 0.1 Hz (MW/0.1 Hz), associated with a BAA that approximates the BAA’s response to Interconnection frequency error. |
| Frequency Bias Setting | A value, usually expressed in MW/0.1 Hz, set into a BA ACE algorithm that allows the BA to contribute its frequency response to the Interconnection. |
| Generator Shift Factor (GSF) | A factor to be applied to a generator’s expected change in output to determine the amount of flow contribution that change in output will impose on an identified transmission facility or Flowgate. |
| Generator-Providing Entity (GPE) | The PSE who is responsible for providing the source generation from owned, affiliated, or contractually bound generation. |
| Generator-to-Load Distribution Factor (GLDF) | The algebraic sum of a GSF and a LSF to determine the total impact of an Interchange Transaction on an identified transmission facility or Flowgate. |
| GET | An HTTP method to request a representation of the specified information. |
| Hypertext Markup Language (HTML) Form | A section of a document containing normal content, markup, special elements called controls (checkboxes, radio buttons, menus, etc.), and labels on those controls. Users generally "complete" a form by modifying its controls (entering text, selecting menu items, etc.), before submitting the form. |
| Identical Service Requests | Those OASIS Transmission Service requests that have exactly the same values for the following OASIS Template Data Elements: - CUSTOMER_CODE - CUSTOMER_DUNS - SERVICE_INCREMENT - TS_CLASS - START_TIME - STOP_TIME - POR* - POD* - PATH* *Service requests where any combination of PATH, POR and/or POD represent exactly the same commercial transmission elements shall be considered as “having the exact same value.” |
| Implemented Interchange | The state where the BA enters the Confirmed Interchange into its ACE equation. |
| Inadvertent Interchange | The difference between the BA’s Net Actual Interchange and Net Scheduled Interchange. |
| Interchange | Energy transfers that cross BA boundaries. |
| Interchange Authority (IA) | The responsible entity that authorizes implementation of valid and balanced Interchange Schedules between BAAs, and ensures communication of Interchange information for reliability assessment purposes. |
| Interchange Authority (IA) Service Entity | The entity that provides the IA service for an e-Tag. The IA service itself is typically a computer system that maintains the master database for the e-Tag and communicates status with other computer systems. The IA Service Entity is the utility industry entity that is responsible for providing the service. In e-Tag 1.8.0, this entity is the Sink BA. |
| Interchange Block Accounting | Energy accounting that assumes a beginning and ending ramp time of zero minutes. For accounting purposes, this moves the energy associated with the starting and ending ramps into the adjacent starting and ending clock time of the Interchange. |
| Interchange Distribution Calculator (IDC) | The mechanism used by RCs in the Eastern Interconnection to calculate the distribution of Interchange Transactions over specific Flowgates. It includes a database of all Interchange Transactions and a matrix of the Distribution Factors for the Eastern Interconnection. |
| Interchange Schedule | An agreed-upon Interchange Transaction size (MWs), start and end time, beginning and ending ramp times and rate, and type required for delivery and receipt of power and energy between the Source and Sink BAs involved in the transaction. |
| Interchange Transaction | An agreement to transfer energy from a Seller to a buyer that crosses one or more BAAs boundaries. |
| Interconnected Operations Service | A service (exclusive of basic energy and Transmission Services) that is required to support the reliable operation of the interconnected Bulk Electric Systems. |
| Interconnection | Any one of the four major electric system networks in North America: Eastern Interconnection, Western Interconnection, ERCOT Interconnection, and Quebec Interconnection. |
| Interconnection Reliability Operating Limit (IROL) | A SOL that, if violated, could lead to instability, uncontrolled separation, or cascading outages that adversely impact the reliability of the Bulk Electric System. |
| Interconnection Time Monitor | An entity that monitors Time Error and initiates and terminates Time Error Corrections. |
| Internet | A global network of interconnected computers, enabling users to share information along multiple channels. |
| Issuing Certification Authority | In the context of a particular Certificate, the Issuing Certification Authority is the certificate authority that has digitally signed and issued the Certificate to a Subscriber. |
| Jointly Owned Unit (JOU) | A unit in which two or more entities share ownership. |
| L10 | A control error limitation specified in NERC standards. |
| Leap Second | A second of time added to coordinated Universal Time to make it agree with astronomical time to within 0.9 seconds. Historically, Leap Seconds are implemented as needed on June 30th or December 31st. (NIST) |
| Load Shift Factor (LSF) | A factor to be applied to a load’s expected change in demand to determine the amount of flow contribution that change in demand will impose on an identified transmission facility or Flowgate. |
| Load-Serving Entity (LSE) | The responsible entity that secures energy and Transmission Service (and related Interconnected Operations Services) to serve the electrical demand and energy requirements of its end-use customers. |
| Market Period | The period of time beginning with the Requesting PSE, or its designee, making required purchase, sale, and Transmission Service arrangements to support the Arranged Interchange through the period of time when the IA receives the Arranged Interchange. |
| May, should, or optional | Define optional requirements that are recommended for implementation but are not specifically required under these Business Practice Standards. |
| Must, shall, or required | Define specific requirements that are not optional and must be implemented as described under these Business Practice Standards. |
| Native Load (NL) | The demand imposed on an electric utility or an entity by the requirements of all customers located within a franchised service territory that the electric utility or entity has statutory or contractual obligation to serve. Note: This definition is only applicable to Business Practice Standards WEQ-008. |
| Native Load Customer | The wholesale and retail power customers of the Transmission Provider on whose behalf the Transmission Provider, by statute, franchise, regulatory requirement, or contract, has undertaken an obligation to construct and operate the Transmission Provider's system to meet the reliable electric needs of such customers. Note: This definition is only applicable to Business Practice Standards WEQ-001, WEQ-002, WEQ-003, and WEQ-013. |
| Net Actual Interchange | The algebraic sum of all metered Interchange over all interconnections between two physically adjacent BAAs. |
| Net Interchange Schedule | The algebraic sum of all Interchange Schedules with each adjacent BAA. |
| Network Integration Transmission Service (NITS) | As specified in the Transmission Provider’s tariff, service that allows an electric Transmission Customer to integrate, plan, economically dispatch and regulate its network resources in a manner comparable to that in which the Transmission Provider serves Native Load Customers. |
| Non-Firm Transmission Service | As specified in the Transmission Provider’s tariff, Transmission Service that is reserved and scheduled on an as-available basis, is subject to Curtailment or interruption, and has less priority than Firm Transmission Service. |
| Number-of-Hours Criteria | The maximum number of hours in a defined period (e.g., annually) that the Transmission Provider is allowed to curtail Transmission Service for reliability reasons at the Conditional Curtailment Priority Level, as specified in the long-term firm PTP agreement which specifies a CCO. |
| Open Access Same-Time Information System (OASIS) | Comprises the computer systems and associated communications facilities that public utilities are required to provide for the purpose of making available to all transmission users comparable interactions with Transmission Service Information and any back-end supporting systems or user procedures that collectively perform the transaction processing functions associated with handling of requests on OASIS. |
| Open Access Same-Time Information System (OASIS) Data Dictionary | Business Practice Standards WEQ-003, NAESB Open Access Same-Time Information System (OASIS) Data Dictionary Business Practice Standards, Version 1.5 |
| Open Access Same-Time Information System (OASIS) Implementation Guide | Business Practice Standards WEQ-013, NAESB Open Access Same-Time Information System (OASIS) Implementation Guide Business Practice Standards, Version 1.5 |
| Open Access Same-Time Information System (OASIS) Node | Is a subsystem of the OASIS. It is one computer system in the OASIS that provides access to Transmission Service Information to a Transmission Customer. |
| Open Access Same-Time Information System (OASIS) Standards and Communication Protocols (S&CP) | Business Practice Standards WEQ-002, NAESB Open Access Same-Time Information System (OASIS) Business Practice Standards & Communication Protocol (S&CP), Version 1.5 |
| Open Access Same-Time Information System (OASIS) Template | Refers generically, or by reference to a specifically named OASIS Template, to the OASIS Templates defined for the Transmission Customer interface to OASIS in Business Practice Standards WEQ-002, including the displays and forms associated with the web browser based user interface implementing the functions of an OASIS defined OASIS Template. |
| Option Set | Alternate sets of Tariff Records. |
| Parent Reservation | An existing, confirmed reservation (original, Redirect, Transfer, Resale, etc,) being modified by a Redirect, Transfer, Resale, etc. |
| Partial Service | Transmission Service offered or granted by a Transmission Provider that is for a shorter duration than requested (start-time through stop-time), less capacity than requested, or both. |
| Path Participant | Any of the entities that are part of an Interchange Transaction. |
| Per Generator Method | A methodology used by the IDC to calculate the portion of parallel flows on any Constrained Facility or Flowgate due to NITS customers and service to NL customers for each BA. |
| Point-to-Point Transmission Service (PTP) | The reservation and transmission of capacity and energy on either a firm or non-firm basis from the PORs to the PODs as specified in the Transmission Provider's tariff. |
| Policy Qualifier | Policy-dependent information that accompanies a CP Identifier in an X.509 Certificate. |
| POST | An HTTP method that submits data to be processed to an identified resource. |
| Postback | A variable component of the Transmission Provider’s selected ATC or AFC calculation methodology that positively impacts ATC or AFC based on a change in status of a Transmission Service reservation or use of reserved capacity, or other conditions as specified by the Transmission Provider. |
| Posted Path | Shall mean any Control Area to Control Area interconnection; any path for which service is denied, curtailed or interrupted for more than 24 hours in the past 12 months; and any path for which a Transmission Customer requests to have ATC or TTC posted. For this last category, the posting must continue for 180 days and thereafter until 180 days have elapsed from the most recent request for service over the requested path. For purposes of this definition, an hour includes any part of an hour during which service was denied, curtailed or interrupted. |
| Power Plant Operator (PPO) | The entity(ies) that has responsibility for gas requirements for a natural gas-fired electric generating facility(ies) and is responsible for coordinating natural gas deliveries with the appropriate Transportation Service Provider(s) to meet those requirements. The PPO performs a number of coordinated activities, including, but not limited to, power plant operations, unit dispatch, natural gas procurement and/or gas transportation arrangements. Because each PPO is structured differently, specific responsibilities within each PPO should be determined by the PPO and the point of contact for the PPO should be communicated to the Transmission Service Provider(s). (Note: This definition applies NAESB WGQ Standard Numbers 0.2.2, 0.3.11, 0.3.12, 0.3.13, 0.3.14, and 0.3.15.) |
| Power Plant Operator (PPO) Facility | The natural gas-fired electric generating unit(s) under the direct control of the PPO. (Note: This definition applies to NAESB WGQ Standard Numbers 0.3.12 and 0.3.13.) |
| Pseudo-Tie | A telemetered reading or value that is updated in real time and used as a “virtual” tie line flow in the AGC/ACE equation but for which no physical tie or energy metering actually exists. The integrated value is used as a metered MWh value for Interchange accounting purposes. |
| Purchasing-Selling Entity (PSE) | The entity that purchases or sells, and takes title to, energy, capacity, and Interconnected Operations Services. PSEs may be affiliated or unaffiliated merchants and may or may not own generating facilities. |
| Quebec Interconnection | One of the major electric system networks in North America. |
| Query Variables | A list of query variables with their settings formatted as defined by the HTTP protocol (i.e., OASIS URL encoded separated by ampersands). |
| Queue Flooding | Excessive submission of Identical Service Requests. |
| Queue Hoarding | This is the act, intentionally or unintentionally, of not confirming or withdrawing an accepted service request such that it impacts the ability of other willing buyers to secure service in a timely fashion. |
| Reallocation | The process used to totally or partially curtail Transactions during TLR levels 3a, 3b or 5a events to allow Transactions using equal or higher priority to be implemented. |
| Redirect | A modification to all or a portion of a transmission reservation to an alternate POR/POD and/or make other changes to the terms of service as permitted. |
| Region | One of the regional entities that work with NERC to improve the reliability of the bulk power system. |
| Registration Authority (RA) | An entity that is responsible for identification and authentication of Certificate subjects, but that does not sign or issue Certificates (i.e., an RA is delegated certain tasks on behalf of a certificate authority). |
| Reliability Coordinator (RC) | The entity that is the highest level of authority who is responsible for the reliable operation of the Bulk Electric System, has the wide area view of the Bulk Electric System, and has the operating tools, processes and procedures, including the authority to prevent or mitigate emergency operating situations in both next-day analysis and real-time operations. The RC has the purview that is broad enough to enable the calculation of IROLs, which may be based on the operating parameters of transmission systems beyond any Transmission Operator’s vision. (Note: This definition also applies to the NAESB WGQ Standard Numbers 0.2.4 and 0.3.14 through 0.3.15) |
| Reliability Coordinator Information System (RCIS) | The system that RCs use to post messages and share operating information in real time. |
| Relinquish | Release of all or a portion of the capacity of a Redirect on a non-firm basis to the firm Parent Reservation. |
| Relying Party | A recipient of a Certificate who acts in reliance on that Certificate and/or digital signatures verified using that Certificate. In this document, the terms "certificate user" and "Relying Party" are used interchangeably. |
| Repository | A database of active digital signatures for a certificate authority system. Certificate authorities post Certificates and CRLs to Repositories that allows Relying Parties to confirm the status of the digital signatures. |
| Request for Interchange (RFI) | A collection of required data, as defined in Business Practice Standards WEQ-004-C, necessary for the purpose of submitting to the IA as an Arranged Interchange. |
| Requesting Purchasing-Selling Entity (PSE) | The entity that prepares and submits the RFI to the IA also referred to as the e-Tag author. |
| Resale | The request to convey scheduling rights associated with a reservation for PTP from a Reseller to an Assignee. |
| Reseller | The Transmission Customer that holds PTP rights and offers those rights for sale on the (secondary) transmission market. |
| Reserve Sharing Group | A group whose members consist of two or more BAs that collectively maintain, allocate, and supply operating reserves required for each BA’s use in recovering from contingencies within the group. Scheduling energy from an adjacent BA to aid recovery need not constitute reserve sharing provided the transaction is ramped in over a period the supplying part could reasonably be expected to load generation in (e.g., ten minutes). If the transaction is ramped in quicker (e.., between zero and ten minutes) then, for the purposes of disturbance control performance, the BAs become a Reserve Sharing Group. |
| Secondary Network Transmission Service | Transmission Service provided to a network customer to deliver energy to network loads from non-DNRs. |
| Section-Based Format | Describes the subdivision into sections of a tariff, rate schedule, or service agreement for purposes of a Tariff Filing. |
| Seller | Reseller or Transmission Provider |
| Service Request Tiers | Relative queue priorities when negotiating Transmission Service. |
| Set of Provisions | A collection of practice and/or policy statements, spanning a range of standard topics, for use in expressing a CP definition or Certificate Practice Statement employing the approach described in this framework. |
| Sheet-Based Format | Describes the subdivision into sheets of a tariff, rate schedule, or service agreement for purposes of a Tariff Filing. |
| Sink Balancing Authority (BA) | The BA in which the load (sink) is located for an Interchange Transaction. (This will also be a receiving BA for the resulting Interchange Schedule.) |
| Source Balancing Authority (BA) | The BA in which the generation (source) is located for an Interchange Transaction. (This will also be a sending BA for the resulting Interchange Schedule.) |
| Standards of Conduct | The regulations contained in Title 18 CFR Part 358, Standards of Conduct for Transmission Providers. |
| Straddle Ramp | Ramp that divides the start ramp duration equally across the profile block start or end time. |
| Subscriber | An authorized individual, application, server, or ‘role’ associated with a registered End Entity that has been issued a Certificate by an Authorized Certification Authority. |
| Supplemental Regulation Service | A method of providing regulation service in which the BA providing the regulation service receives a signal representing all or a portion of the other BA’s ACE. |
| System-Conditions Criteria | One or more specific system conditions during which the Transmission Provider is allowed to curtail Transmission Service for reliability reasons at the Conditional Curtailment Priority Level, as specified in the long-term firm PTP agreement which specifies a CCO. |
| System Impact Study | An assessment by the Transmission Provider of (i) the adequacy of the transmission system to accommodate a request for either firm PTP or NITS and (ii) whether any additional costs may be incurred in order to provide Transmission Service. |
| System Impact Study Agreement | An agreement to perform a System Impact Study. |
| System Operating Limit (SOL) | The value (such as MW, MVar, Amperes, frequency or volts) that satisfies the most limiting of the prescribed operating criteria for a specified system configuration to ensure operation within acceptable reliability criteria. SOL are based upon certain operating criteria. These include, but are not limited to: - Facility Rating (applicable pre- and post- contingency equipment or Facility Ratings) - Transient stability ratings (applicable pre- and post-contingency stability limits) - Voltage stability ratings (applicable pre- and post-contingency voltage limits) - System Voltage Limits (applicable pre- and post-contingency voltage limits) |
| Tariff Filing | A compilation of all document(s), associated data and supporting documents that is required to be filed electronically by a Tariff Submitter pursuant to Title 18 CFR Parts 35, 154, 284, 300, 341 or other parts as required by the FERC. |
| Tariff Record | The part of a Tariff Filing that consists of the Data Elements that relate to a sheet, section, or entire tariff, rate schedule, or service agreement that is required to be filed with the FERC. |
| Tariff Submitter | Any entity that is required to submit a Tariff Filing. |
| Tie Facility(ies) | The transmission facility(ies) interconnecting BAAs. |
| Time Error | The difference between the Interconnection time measured at the BA(s) and the time specified by the NIST. Time Error is caused by the accumulation of frequency error over a given period. |
| Time Error Correction | An offset to the Interconnection’s scheduled frequency to return the Interconnection’s Time Error to a predetermined value. |
| Transfer | Request to convey all rights and obligations associated with a reservation for PTP from a Reseller to an Assignee. |
| Transfer Distribution Factor (TDF) | The portion of an Interchange Transaction, expressed in percent that flows across a transmission facility (Flowgate). |
| Transmission Customer | Any Eligible Transmission Customer (or its designated agent) that can or does execute a service agreement or can or does receive Transmission Service. |
| Transmission Loading Relief (TLR) | A procedure used in the Eastern Interconnection to relieve potential or actual loading on a Constrained Facility or Flowgate. |
| Transmission Operator | The entity responsible for the reliability of its “local” transmission system, and that operates or directs the operations of the transmission facilities. |
| Transmission Provider | A utility (or its designated agent) that owns, controls, or operates facilities used for the transmission of electric energy in interstate commerce and provides Transmission Service. |
| Transmission Purchasing Selling Entity (TPSE) | PSE that holds the transmission rights on a physical path segment referenced in an e-Tag. |
| Transmission Service | Services needed to move energy from a POR to a POD provided to Transmission Customers by Transmission Providers. |
| Transmission Service Provider | The entity that administers the transmission tariff and provides Transmission Service to Transmission Customers under applicable transmission service agreements. |
| Transmission Services Information | Transmission and ancillary services information that must be made available by public utilities on a non-discriminatory basis to meet the regulatory requirements of transmission open access. |
| Transmission Services Information Provider (TSIP) | Transmission Provider or agent to whom the Transmission Provider has delegated the responsibility of meeting any of the requirements associated with OASIS. |
| Transportation Service Provider | The entity that is providing natural gas delivery services (e.g., transportation). (Note: This definition also applies to the NAESB Business Practices WGQ Standard Numbers 0.2.1and 0.3.11 through 0.3.15) |
| Unexercised Rollover Rights | The amount of capacity that is held on a long-term firm PTP Transmission Service reservation and is eligible for ongoing rollover or renewal of service as established by the Transmission Provider at the time service was granted, less the capacity held on any confirmed service reservations for renewal of those rollover rights for a subsequent term of service, and less any capacity conveyed as an on-going rollover right to a qualifying confirmed Redirect on a firm basis. |
| Western Electric Coordinating Council Network (WECCNet) | A messaging system used by the WECC for use by participating utility’s dispatchers and network administrators. |
| Western Interconnection | One of the major electric system networks in North America. |
| Whole Document Format | Describes a tariff, rate schedule, or service agreement submitted as a single document within a Tariff Record for purposes of a Tariff Filing. |
| WWVB | The NIST time-signal radio station near Fort Collins, Colorado, co-located with WWV. WWVB is the station that radio-controlled clocks throughout North America use to synchronize themselves. |

Updates to Existing Business Practice Standards WEQ-001 through WEQ-014

Conforming changes to the existing Business Practice Standards version 002.1 will be made by the NAESB office after this recommendation has been ratified.  The subcommittee co-chairs will work with the NAESB office to ensure the appropriate application of the ratified recommendation to Business Practice Standards Version 2.1.

It should be noted that, as part of the recommendation, conforming changes will need to be made to all recommendations within the approval queue and all final actions that were not included in Business Practice Standards version 002.1

Additionally, any new definitions or revised definitions approved since the Business Practice Standards version 002.1 were published will be need to be moved to Business Practice Standards WEQ-000-2.

4.  SUPPORTING DOCUMENTATION

a.  Description of Request:

In FERC Order 676-C, the Commission noted:

8.	In the NOPR, we asked for comment on differences in definitions in a few of the NERC’s and NAESB’s TLR standards.  The comments indicate that NERC and NAESB have formed a subcommittee to ensure that their definitions are complementary in the future.  We are very pleased that NERC and NAESB have taken active steps to ensure that their respective definitions are harmonized so as to ensure that these standards will operate efficiently in the future.

The following line item was added to the 2009 WEQ Annual Plan, which was approved by the NAESB Board of Directors on March 26, 2009.

5.b	Modify NAESB definitions to address internal inconsistencies and inconsistencies with the NERC glossary.  Revise existing NAESB glossary/definition of terms to be applicable to entire set of WEQ Business Practices. (http://www.naesb.org/pdf3/weq_ec051308w3.doc)

b.  Description of Recommendation:

The recommendation is intended to address the differences in definitions within the Business Practice Standards and between the NERC Glossary and the NAESB Definition of Terms.

c.  Business Purpose:

Address consistency of definitions as discussed in FERC Order 676-C.

- Commentary/Rationale of Subcommittee(s)/Task Force(s):

To the extent possible, the differences between the Business Practice Standards and NERC Standards have been reconciled.  The developers of this recommendation accepted the NERC definitions in most instances.  However, there are a few exceptions where the drafters of the recommendation consider the NAESB definitions superior to the NERC definitions.  In those instances, we would request that NERC modify their definitions to be consistent with the NAESB definitions.

Supporting Documentation:

- Comparison on NERC/NAESB Definitions.

- Docket No. RM05-5-005 Order No.676-C Standards for Business Practices and Communication Protocols for Public Utilities

- Docket No. RM05-5-005 Standards for Business Practices Communication Protocols for Public Utilities (NOPR)

The subcommittee co-chairs utilized the following logic to develop the recommendation:

- Acronyms

  - Acronyms would be listed in only Business Practice Standards sections WEQ-000-1 and in WEQ-000-2 of Business Practice Standards WEQ-000 along with the definition if the acronym was also representing a defined term.

  - Values for Data Elements were not included in the acronym list.  Values for Data Elements are documented in Business Practice Standards WEQ-003 and WEQ-013.

  - If an acronym had more than one meaning in the Business Practices Standards, a decision was made to not use the acronym.  These acronyms were documented in Business Practice Standards WEQ-000-1.1

  - If an acronym was only occassionally used rather than the term, where the acronym was listed it was replaced by the term.

  - Acronyms only used in algorithms were not included in the acronym list since they are defined for the algorithm.

- Definitions

  - In addition to the terms listed in each standard, the co-chairs identified all terms that were listed as proper case in the entire set of standards.  Upper case terms were excluded as they referred to Data Element values.  This became the set of terms that needed to be addressed.

  - Existing terms with definitions

    - If a term was defined and was not used in any of the standards, the term was not added to Business Practice Standards WEQ-000-2

    - If the term had multiple definitions, one definition was selected.  If the definition was also defined by NERC, to the extent appropriate, the NERC definition was used.

    - If  the term had only one definition, but also had a NERC definition, to the extent appropriate, the NERC definition was used.

    - If there were multiple terms which had the same meaning, one term was selected and the other terms were replaced with the selected term throughout the document.

    - OASIS standards used terms commonly used in FERC Orders such as Control Area and Transmission Provider to be consistent with FERC.

    - Standards tightly coupled with NERC standards such as Transmission Loading Relief used NERC terminology such as Balancing Authority Area and Transmission Service Provider.

  - Data Element names were not defined since they are addressed in Business Practice Standards WEQ-003 and WEQ-014.

  - If a term was not defined, an attempt was made to define the term by:

    - Reviewing the NERC glossary;

    - Reviewing FERC Orders;

    - Reviewing functional specifications;

    - Reviewing public internet sites;

    - Soliciting input from JESS and the WGQ BPS; and

    - Contacting Subject Matter Experts.

  - If a term was a NERC term that is no longer used by NERC, the term was replaced with the current NERC term.  For example, Security Coordinator was replaced with Reliability Coordinator.

  - If a term could not be defined, it was changed from proper to lower case.

  - The following terms were removed from the NAESB glossary. A brief explanation for each removal is provided by term.

    - Appropriate Regulating Authority – Revised the definition for Commission to remove Appropriate Regulating Authority.

    - Eligible Customer – Replaced with Eligible Transmission Customer.

    - FERC – Moved to list of acronyms.

    - Interchange Transaction Tag (Tag) – Decision was made to use the term e-Tag.

    - Market Assembly – In latest changes for Coordinate Interchange to support the Electronic Tagging Functional Specification, the references to Market Assembly are being removed from the Business Practice Standards.

    - NERC – Moved to list of acronyms.

    - Primary Provider – Decision was made to use only the term Transmission Provider for OASIS related standards and Transmission Service Provider for standards that are tightly coupled with NERC Standards.

    - Reliability Area – Term was defined but not used in the Business Practice Standards.

    - Reliability Period – Term was defined but not used in the Business Practice Standards.

    - Responsible Party – Deleted since it is a Data Element name.

    - Secondary Provider – decision was made to use only the term “Reseller”.

    - Secondary Transmission Provider – decision was made to use only the term “Reseller”

    - Tag – Replaced with the term “e-Tag”

    - Template – Replaced with “Open Access Same-Time Information System (OASIS) Template”.

    - Value-Added Transmission Services Information Provider (VTSIP) – the term was defined but not used.

    - Wholesale Merchant Function – Term was defined but not used in the Business Practice Standards.

  - Definitions were created for the following proper case terms in the Business Practice Standards.  In several instances, the terms were defined in the Business Practice Standards.

    - Actual Daily Native Peak Load

    - Actual Daily System-Wide Peak Load

    - Actual Daily Zonal Load

    - Assignment Reference Number

    - Business Practice Standards

    - Checkout Partner

    - Distribution Factor

    - Eastern Interconnection

    - Electric Reliability Council of Texas (ERCOT) Interconnection

    - eTariff Submission Data Dictionary

    - eTariff Submission Implementation Guide

    - Facility Rating

    - Forecasted Daily Native Peak Load

    - Forecasted Daily System-Wide Peak Load

    - Forecasted Daily Zonal Load

    - GET

    - Hyper Text Markup Language (HTML) Form

    - Interchange Authority (IA) Service

    - Internet

    - Net Interchange Schedule

    - Open Access Same-Time Information System (OASIS) Data Dictionary

    - Open Access Same-Time Information System (OASIS) Implementation Guide

    - Open Access Same-Time Information System (OASIS) Standards and Communication Protocols (S&CP)

    - Open Access Same-Time Information System (OASIS) Template

    - Path Participant

    - POST

    - Posted Path

    - Pseudo-Tie

    - Quebec Interconnection

    - Query Variables

    - Reserve Sharing Group

    - Seller

    - Service Request Tiers

    - Standards of Conduct

    - System Impact Study

    - System Impact Study Agreement

    - Western Interconnection

    - WWWB

  - The following proper case terms in the Business Practice Standards and either are not proper cased (because they were not defined) or have been replaced with another defined term.  Where a term has been replaced with another term, the replacement term is also identified.

    - Access Privledges

    - Administrator

    - Ancillary Service Type  (note: this is a Data Element)

    - Approval Service

    - Associated Filing

    - Authority Service

    - Authorized Certificate Authority

    - Certificate Application Procedure

    - Certificate Authority

    - Certificate Management Practice

    - Certificate Management Program

    - Certificate Policy Extension (note: only “Extension” was lower cased)

    - Certificate Policy Identifier (note: only “Identifier” was lower cased)

    - Certificate User

    - Common Name

    - Conditional Curtailment Options Provisions (note: only “provisions” was lower cased)

    - Confirmation of Receipt

    - Constraint Path

    - Daily Firm Service (replaced with daily Firm Transmission Service)

    - Data Records

    - DC Tie

    - Dynamic Transfer

    - Electronic Control Area (replaced with Control Area)

    - Energy Profile

    - Energy Title Holder

    - FERC Policy (note: only “Policy” was lower cased)

    - FERC Regulation (note: only “Regulation” was lower cased)

    - FERC Statutes and Regulation (note: only “Statues and Regulations” was lower cased)

    - Firm  (defined as Firm Transmission Service)

    - Firm Point-to-Point (defined as Firm Transmission Service and Point-to-Point Transmission Service)

    - FORM (replaced with Hyper Text Markup (HTML) Form)

    - Host Balancing Authority  (note: “host” was lower cased)

    - Host Control Area (note: “host” was lower cased)

    - Input

    - Input/Response

    - Inadvertent Energy

    - Internet Service Provider (note: only “Service Provider” was lower cased)

    - Investigation Requestor

    - Issuing CA (note: only “Issuing” was lower cased and CA was spelled out to Control Area)

    - Load CA (note: only “Load” was lower case and CA was spelled out to Control Area)

    - Long-Term Firm Point-to-Point Transmission Service

    - Market Operator

    - Methodology Contact

    - Multi-Balancing Authority Energy Market (note: only lower cased “Multi-“ and “Energy Market”)

    - Next Hour

    - Non-Firm Next Hour Market Services (note: defined as Non-Firm Transmission Service)

    - Non-Firm Point-to-Point Transmission Service [note: defined as Non-Firm Transmission Service and Point-to-Point Transmission Service  (Non-Firm is an attribute of Point-to-Point Transmission Service)]

    - Numbers

    - Observers

    - Organizational

    - Open Access Same-Time Information System (OASIS) Users (note: “users” was lower cased)

    - Part II (note: removed references to Part II due to pre-FERC Order 890 concepts)

    - Part III (note: removed references to Part II due to pre-FERC Order 890 concepts)

    - Phase 1A (note” removed references to Part II due to pre-FERC Order 890 concepts)

    - Policy Object ID

    - Receiving Balancing Authority (replaced with Sink BA)

    - Receiving Control Area (replaced with Sink BA)

    - Redirect on a Firm Basis

    - Redirect on a Non-Firm basis

    - Regional Reliability Organization (replaced with Reliability Entity)

    - Request for Investigation

    - Scheduling Entity

    - Sending Balancing Authority (replaced with Source BA)

    - Sending Control Area (replaced with Sending BA)

    - Security Coordinator  (replaced with Reliability Coordinator with the exception of code values and Data Element names)

    - Security Officer

    - Service Agreement

    - Service Level Agreement

    - Service Requestor

    - Short-Term

    - Short-Term Firm Point-to-Point Transmission Service

    - State Regulatory Authorities (revised definition of Commission)

    - Tag Authority Service Operator

    - Temporary Logs

    - Transient Stability Ratings

    - Tranmission Customer Confirmation Time Limit (note: “Confirmation Time Limit” was lower cased)

    - Transmission Service Agreement

    - Transmission System Information Networks (replaced with Electric Industry Registry)

    - FORM (replaced with Hyper Text Markup (HTML) Form)

    - Transmission Service Provider Counter Time Limit (note: “Counter Time Limit” was lower cased)

    - Transmission Service Provider Evaluation Time Limit (note: “Evaluation Time Limit” was lower cased)

    - Transmission Service Provider Loss Table (note: references a specific table in the Business Practice Standards so no definition was developed)

    - Transmission Service Provider Product (note: “Product” was lower cased)

    - Same-Day

    - Unscheduled Flow Reduction Procedure

    - User

    - User Interface

    - Voltage Stability Ratings

    - Want-ad (note: lower cased when not referencing a value for a Data Element

  - The following are a list of terms the NAESB Subcommittee co-chairs identified that NAESB should keep the NAESB definition rather than using the NERC definition

    - Network Integration Transmission Service

    - Open Access Same-Time Information System

    - Point to Point Transmission Service  - The definition included in the recommendation is taken directly from the Pro Forma OATT.

    - Postback

    - Reallocation