1.	RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| X | Accept as requested | X | Change to Existing Practice |
| --- | --- | --- | --- |
|  | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

2.	TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
| X | Initiation | X | Initiation |
|  | Modification |  | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
| X | Principle | X | Principle |
| X | Definition | X | Definition |
| X | Business Practice Standard | X | Business Practice Standard |
|  | Document |  | Document |
|  | Data Element |  | Data Element |
|  | Code Value |  | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
|  | Business Process Documentation |  | Business Process Documentation |

3.	RECOMMENDATION

SUMMARY:

The NAESB Smart Grid Task Force (SGTF) submits this Recommendation to define the range of information required to establish the breadth of the communication requirement. There is no need for the cases to be exhaustive, merely illustrative of the full scope.

WEQ-000

New Abbreviations/Acronyms and Defined Terms to be added to WEQ-000 are Included in the Recommendation Request Titled Requirements Specifications for Common Electricity Product and Pricing Definitions – for NIST PAP03 to avoid redundancy between recommendations.

WEQ-017

New Business Practice Standards WEQ-017 – Specifications for Common Scheduling Mechanism for Energy Transactions

(NOTE: NAESB Retail numbering convention will replace WEQ numbering convention in NAESB Retail publication)

Recommended Standards:

# Executive Summary

# Specifications for Common Scheduling Mechanism for Energy Transactions

The following contains a set of requirements relating to the use of date and time based data elements that are used in transactions for Demand Response programs. This information is being provided to NIST in order to aid in the development of a standard representation for date/time based data elements derived from an XML representation of iCalendar.

Although there are many other areas where date/time based data elements are used in energy industry transactions this recommendation is limited in scope to only those date/time based data elements that are utilized in Demand Response programs.

Introduction

Specifications for Common Scheduling Mechanism for Energy Transactions

Smart grid interfaces must communicate between domains,that is between different business and consumer entities with different values and approaches (ontologies). Such interfaces must be simplified to bare essentials, and be understandable by parties with different backgrounds and expectations.

To achieve cross-domain understandability, and thereby enhance interoperation, smart grid transactions must use semantic models from different areas developed by different standards groups. Each of these vocabularies is most fully defined in one area. We have agreed, within the NIST process, to use the vocabularies from the appropriate domain experts to compose the message that flow between domains on the smart grid.

In the following, we defined transactions as grid events that occur between domains, or whose effects and outcomes must be communicated between domains. This activity makes no assertions about any other events.

Smart grid transactions communicate three classes of information. They communicate information about events that that occur during power management and distribution. They invoke financial or business transactions that guide those events. They communicate schedules and interval information for the flows of energy. Per agreement, all semantics relating to Power Management will use the CIM from IEC TC57. All financial and market semantics will be based upon the definitions found in ISO 20022 and in their most common expression, FIX. All elements relating to schedule and interval will be expressed in the notation of the IETF iCalendar (RFC 5545) standard, as defined in the XML representation iCalendar developed by CalConnect.  The purpose of this approach is to increase interoperation and understanding between domains.

The group primarily associated with the development of interoperable enterprise date, time, and interval standards for the IETF is the CalConnect. CalConnect and the Organization for the Advancement of Structured Information Standards will work together to define the common communication of date time and interval to meet the needs of smart grid transaction communications as well as for building systems integration, enterprise interaction, and financial transactions. This commonality is anticipated to reduce barriers to interoperation and thereby to expand participation in DR and DER.

The purpose of this action is to define the requirements for standard communication of date, time, schedule, and interval by smart grid actors, with particular attention to DR. These requirements will be submitted to the Organization for the Advancement of Structured Information Standards and CalConnect technical committee to define the applicable communication standards to support them.

Scope: The scope of the following is to define the range of information required to establish the breadth of the communication requirement. There is no need for the cases to be exhaustive, merely illustrative of the full scope.

Business Practice Standards

017-1	Specification for Common Scheduling Mechanism for Energy Transactions

017-1.1	ACTORS LIST

There were no actors provided for the associated use cases.

017-2	Common Scheduling Mechanism for Energy Transactions Use Cases

017-2.1	Use Case Overview

The use case modeling technique is used for capturing business and date requirements relative to electric product and price information.

The use cases included in this document are not intended to be required or exhaustive and are provided for clarification purposes.

## 017-2.1.1	Requirements for Date and Time Usage

Demand Response programs employ a combination of absolute dates/times, absolute periods (range) of dates/times, relative dates/times, recurring dates/times, absolute intervals and absolute durations in order to communicate temporal information pertaining to demand response transactions. Demand Response programs must also accommodate time zone variations, leap years, leap seconds, and the use of daylight savings time. Any calendaring/scheduling standard that is applicable to Demand Response programs must be capable of supporting this range of requirements.

An incomplete, but representative, set of instances is provided below. The sample data included are non-normative representations; they are included to clarify the expectation of information that will be in the standard.

| Type of Representation | Representative Instance | Sample Data |
| --- | --- | --- |
| Absolute date/time | Effective start date Effective start date and time | 2009-11-01 2009-11-01T01:00:00-Z |
| Absolute Periods (range) | Deployment Period | 2009-11-01T01:00:00-Z/ 2009-11-02T24:00:00-Z |
| Relative date/time (time-point) | Reduction Deadline | 30 minutes from a specified point in time |
| Recurring date/time | Market Closing | Every day at 12:00 noon |
| Absolute Interval | Meter Data Recording Interval System Frequency | Every 5 minutes 60 cycles per second |
| Absolute Duration | Outage Duration Minimum Run Time | 2 days 4 hours |

It is an additional requirement that all schedule and interval communications be able to express the precision required in the response. For example, an actor may be required to perform within five minutes, or within five milliseconds of the agreed upon time.

017-A	Appendix A – Entity-Relationship Model

The following terms and definitions correspond to a set of actor and objects within the overall information model for demand response.  Figure 1A contains a data model used to illustrate the entity-relationships within the model.

Proper (and Improper) Usage of the Entity-Relationship Model.    The entity-relationship model is used to show cardinality among objects in the model, for example every Resource belongs to one and only one Service Provider and a Premise contains one or more End Devices.  The Entity-Relationship model does not provide use case information; actors are objects in the model and do not “act” in the model.  The Entity-Relationship model does not imply process.  For example, a Service Provider is associated with many Resources; however the enrollment of those Resources may be managed by a System Operators or a Utility Distribution Operator.  Both Use Cases and Process Maps are separate components of the Business Practice Standards.

Reading Crow’s Foot Notation.    Objects in the model which share a relationship are connected with a cardinality line.  Each end of the cardinality line contains a Crow’s Foot notation, as documents in the legend of the figure.  The four notations utilized are “exactly-one”, “one-or-more”, “zero-or-one”, and “zero-or-more”.  The cardinality line is bi-directional; meaning it can be read in two directions.  For example: a Premise is related to one-or-more End Devices (reading top to bottom) and every End Device is related to exactly-one Premise (reading bottom to top).

Optional Objects.  The entity-relationship model is designed to support multiple business models and not every business model will require all objects to function.  Therefore, all objects in the model are considered optional.  For example, a Utility Distribution Operator may design a Demand Response program which requires the definition of Resources and Premises, but does not require Response Method Aggregations and Response Methods.  In this example, the relationship between Resource and Premise is transitive: a Resource is comprised of one-or-more Premises and each Premise is associated with zero-or-more Resources.

Applicability to Retail and Wholesale.  The entity-relationship model is applicable to both wholesale and retail.  When using the model for wholesale, the applicable business entities are 2.1 (System Operator) and 2.3 (Market Participant), while in retail markets, the parallel business entities are 2.3 (Utility Distribution Operator) and 2.4 (Utility Customers).  Other than the swapping of the two pairs of terms, the models are identical, including the names of and relationships among objects.

Figure 1A. Entity-Relationship Model for Smart Grid Use Cases

# 4.  Supporting Documentation

a.  Description of Request:

On September 30, 2009, the National Institute of Standards and Technology officially assigned NAESB the responsibility to develop Requirements and Use Cases pertinent to Priority Action Plan items 3, 4 and 9. A complete description of each item is available on the NIST Web site located at http://collaborate.nist.gov/twiki-sggrid/bin/view/SmartGrid/WebHome  in the section titled Priority Action Plans (PAPs).

This recommendation specifically pertains to PAP 04, “A Common Scheduling Mechanism for Energy Transactions”.

b.  Description of Recommendation:

The scope of this recommendation is to define the range of information required to establish the breadth of the communication requirement. There is no need for the cases to be exhaustive, merely illustrative of the full scope.

c.  Business Purpose:

This recommendation has been developed in response to a request from NIST to provide use cases and requirements germane to the use of dates/times within a broad spectrum of energy transactions. The scope of this effort has been limited to Demand Response Programs in order to meet NIST’s deadline for this effort.

d.  Commentary/Rationale of Subcommittee(s)/Task Force(s):

Assumptions – The Smart Grid Task Force assumes as given that common semantics for communicating schedule, interval, and sequence will enhance interoperation between domains and between business entities.

Joint Smart Grid Standards Task Force and Fix Protocol Meeting Notes/Documents:

  - September 17, 2009 Meeting Notes – To be posted

  - October 1, 2009 Meeting Notes – To be posted

  - October 15, 2009 Meeting Notes – To be posted

NAESB Smart Grid Standards Task Force Sub-Group Meeting Notes/Documents:

- October 13, 2009 Meeting Notes – To be posted

- October 20, 2009 Meeting Notes – To be posted

- October 27, 2009 Meeting Notes – To be posted

- November 3, 2009 Meeting Notes – To be posted

- November 4-6, 2009 Meeting Notes – To be posted

- November 10, 2009 Meeting Notes – To be posted

NAESB Smart Grid Standards Task Force Meeting Notes/Documents:

- October 22, 2009 Meeting Notes – http://naesb.org/pdf4/smart_grid_ssd102209notes.doc

- October 29, 2009 Meeting Notes – To be posted

- November 5, 2009 Meeting Notes – To be posted

- November 13, 2009 Meeting Notes – To be posted

- December 3, 2009 Meeting Notes – To be posted

- December 10, 2009 Meeting Notes –http://naesb.org/pdf4/smart_grid_ssd121009notes.doc

- December 17, 2009 Meeting Notes – http://naesb.org/pdf4/smart_grid_ssd121709notes.doc

- January 7, 2010 Meeting Notes – To be posted

- January 14, 2010 Meeting Notes – To be posted

- January 21, 2010 Meeting Notes – To be posted

- January 28, 2010 Meeting Notes – To be posted

e.  Additional Documentation from Subcommittee(s)/Task Force(s):

- The Task Force is including the following materials to include complete set of abbreviations, acronyms, and definitions.  The changes to the abbreviations, acronyms, and definitions are only listed in the Recommended Standards.

| Abbreviation / Acronym | Meaning |
| --- | --- |
| BAS | Building Automation System |
| CalConnect | Calendaring and Scheduling Consortium |
| CIM | Common Information Model |
| C&I | Commercial and Industrial |
| DDE | Designated Dispatch Entity |
| DER | Distributed Energy Resource |
| DR | Demand Response |
| EA | Environmental Authority |
| ED | End Device |
| EMS | Energy Management System |
| ESI | Energy Services Interface |
| FIX | Financial Information eXchange protocol |
| FR | Federal Regulator |
| GWAC | GridWise Architecture Council |
| iCalendar | Internet Calendaring and Scheduling Core Object Specification |
| IEC TC57 | International Electrotechnical Commisssion Technical Committee 57 |
| IETF | Internet Engineering Task Force |
| IRC Council | Independent System Operator/Regional Transmission Operator Council |
| ISIN | International Security Identifier Number |
| ISO | International Organization for Standardization |
| ISOs/RTOs | Independent System Operators/Regional Transmission Operators |
| kW | Kilowatt |
| kWh | Kilowatt Hour |
| LA | Local Authority |
| LSE | Load Serving Entity |
| MA | Meter Authority |
| MBL | Maximum Base Load |
| MB/MA | Meter Before/Meter After |
| MGO | Metering Generator Output |
| MP | Market Participant |
| MWh | Megawatt Hour |
| NIST | National Institute of Standards and Technology |
| PAP | Priority Action Plan |
| RM | Response Method |
| RMA | Response Method Aggregation |
| OpenSG | Open Smart Grid |
| PUC | Public Utility Commission |
| SE | Scheduling Entity |
| SGTF | Smart Grid Task Force |
| SO | System Operator |
| SP | Service Provider |
| SR | State Regulator |
| TDSP | Transmission/Distribution Service Provider |
| UC | Utility Customer |
| UCAIug | UCA International Users Group |
| UDO | Utility Distribution Operator |
| UML | Unified Modeling Language |
| XML | Extensible Markup Language |

| Term | Definition |
| --- | --- |
| Baseline | A Baseline is an estimate of the electricity that would have been consumed by a Demand Resource in the absence of a Demand Response Event. The Baseline is compared to the actual metered electricity consumption during the Demand Response Event to determine the Demand Reduction Value. Depending on the type of Demand Response product or service, Baseline calculations may be performed in real-time or after-the-fact. |
| Business Entity | The wholesale or retail entity that interacts with other entities in its market. |
| Communication Method | The method by which an object communicates with another object to instruct, measure, report or control. |
| Control | The role associated with the control of an End Device. |
| Designated Dispatch Entity (DDE) | A role which carries the responsibility of receiving and processing demand resource dispatch instructions or market information and (optionally) providing response information. |
| Distributed Energy Resources (DER) | DERs are small, modular, energy generation and storage technologies that provide electric capacity or energy where it is needed. DERs may be either connected to the local electric power grid (e.g., for voltage support) or isolated from the grid in stand-alone applications, such as part of a MicroGrid. Definition of DER provided by the Department of Energy, http://www1.eere.energy.gov/femp/pdfs/31570.pdf |
| End Device (ED) | A physical end-use device that consumes or supplies electricity. |
| Environmental Authority (EA) | A regulatory authority responsible for the development, reporting and enforcement of environmental activities. |
| Federal Regulator (FR) | A federal regulatory authority. |
| Load Serving Entity (LSE) | A role which carries the responsibility of serving end-users and selling electric energy to end-users. |
| Local Authority (LA) | A regulatory authority responsible for the oversight and administration of utility service-related functions within its jurisdiction. |
| Market Enrollment | The collection of enrollment or tariff data for a Resource Object to provide a specific market product or service. |
| Market Participant (MP) | An organization registered with the System Operator that may take on roles such as SP, LSE, TDSP, DDE, SE, and/or MA in accordance with the SO’s market rules. |
| Maximum Base Load (MBL) | A performance evaluation methodology based solely on a Demand Resource’s ability to reduce to a specified level of electricity Demand, regardless of its electricity consumption or Demand at Deployment. |
| Measurement | The role associated with the device or algorithm that measures the consumption or supply of an End Device. |
| Meter Authority (MA) | A role which carries the responsibility of providing data necessary to determine the performance of a Resource. |
| Meter Before /Meter After (MB/MA) | A role which carries the responsibility of providing data necessary to determine the performance of a Resource. |
| Metering Generator Output (MGO) | A performance evaluation methodology, used when a generation asset is located behind the Demand Resource’s revenue meter, in which the Demand Reduction Value is based on the output of the generation asset. |
| P-Node | The price location of the Premise in the transmission and/or distribution network. |
| Participant | The entity that represents resources to a market or distribution operator. |
| Premise | The location at which connection to the transmission or distribution system is made. |
| Regulator | A rule-making and enforcement entity. |
| Response Method | A measurable action taken in response to an instruction to change consumption. |
| Response Method Aggregation (RMA) | A logical entity that has a reportable interval level consumption, e.g. an RMA may be a physical entity with its own meter, a neighborhood of homes that has a net meter, or an estimate of consumption of an aggregation of retail customers. |
| Resource | A market-dependent group of Response Method Aggregations that represents a dispatchable entity. |
| Resource Object | Physical and logical types of demand response resource objects. |
| Scheduling Entity (SE) | A role which carries the responsibility of submitting bids/offers and receives schedules and awards. |
| Service Provider (SP) | A role which carries the responsibility of coordinating resources to deliver electricity products and services to a market or distribution operator. |
| State Regulator (SR) | A regulatory authority responsible for the oversight and administration of electric utilities. |
| Supporting Objects | Objects that support the interaction of Business Entities and Resource Objects. |
| System Operator (SO) | A System Operator is a Balancing Authority, Transmission Operator, or Reliability Coordinator whose responsibility is to monitor and control an electric system in real time (based on NERC definition). The System Operator is responsible for initiating Advance Notifications, Deployment, and Release/Recall instructions. |
| Transmission/Distribution Service Provider (TDSP) | A role which carries the responsibility of operating a local electricity transmission and/or distribution system. |
| Utility Customer (UC) | An end-use customer of the Utility Distribution Operator that takes on roles such as Facility or Resource. |
| Utility Distribution Operator (UDO) | An entity which carries the responsibility of operating an electricity distribution system. |
| Zone | A physical or electrical region. |

http://tools.ietf.org/html/rfc5545

International Organization for Standardization ISO 20022 Universal financial industry message scheme

Internet Calendaring and Scheduling Core Object Specification (iCalendar)

http://collaborate.nist.gov/twiki-sggrid/bin/view/SmartGrid/PAP04Schedules