Date of Request:   October 15, 2010

1.  Submitting Entity & Address:

El Paso Natural Gas_____________________________________

Two North Nevada Avenue_______________________________

Colorado Springs, CO 80903_____________________________

______________________________________________________

2.  Contact Person, Phone #, Fax #, Electronic Mailing Address:

Name 	:     	William M. Griffith__________________

Title 	:     	Consultant_________________________

Phone :  	719-520-4334_____________________

Fax 	:	__________________________________

E-mail	:	William.griffith@elpaso.com__________

3.  Version and Standard Number(s) suggested for correction or clarification:

Version 2.0______________________________________________________________

Nomination (1.4.1), Confirmation Response (1.4.4), Scheduled Quantity (1.4.5), Scheduled Quantity for Operators (1.4.6), and Confirmation Response data sets as needed to support these requirements.

4.  Description of Minor Correction/Clarification including redlined standards corrections:

Add the following transaction types to NAESB WGQ Standard 1.4.1 Nomination and 1.4.5 Scheduled Quantity data sets.:

‘Flow Day Diversion - Receipt’  or ‘FDR’

‘Flow Day Diversion – From Pipeline’  or ‘FDP’

‘Flow Day Diversion – To Pipeline’  or ‘FDS’

Add the reduction reason code value ‘Contract Balancing – Receipt’ (CBR) as needed to Standards Nomination (1.4.1), Confirmation Response (1.4.4), Scheduled Quantity (1.4.5), Scheduled Quantity for Operators (1.4.6), Confirmation Quick response (1.4.7).

Descriptions:

Transaction Type:  ‘Flow Day Diversion - Receipt’  or ‘FDR’ to be used to identify an intra-day nomination that replaces a previously scheduled nomination under the same contract that does not increase the previously scheduled capacity through any point of constraint to a previously scheduled point of delivery.

Transaction Type:  ‘Flow Day Diversion – From Pipeline’  or ‘FDP’ to be used to identify an intra-day nomination using an imbalance owed to the service requestor by the pipeline that replaces a previously scheduled nomination under the same contract that does not increase the previously scheduled capacity through any point of constraint to a previously scheduled point of delivery.

Transaction Type:  ‘Flow Day Diversion – To Pipeline System’  or ‘FDS’ to be used to identify an intra-day nomination using an imbalance owed to the pipeline  by the service request that replaces a previously scheduled nomination under the same contract that does not increase the previously scheduled capacity through any point of constraint from a previously scheduled point of delivery.

Reduction Reason Code: ‘Contract Balancing – Receipt’ or ‘CBR’ to be used to report the reduction of an FDR, FDP or FDS nomination in the event that capacity usage through a point of constraint exceeds the previously scheduled quantity through that constraint point under the same contract.

5.  Reason for of Minor Correction/Clarification:

Add three new Transaction Types and code values to support the implementation of the flexible receipt and delivery point requirements of NAESB WGQ Standard 1.3.80.  Additionally, add a new Reduction Reason Code value to identify reductions related to intra-day reductions related to flexible receipt and/or delivery nominations.  _______________________________________________________________________

_______________________________________________________________________

_______________________________________________________________________