TECHNICAL IMPLEMENTATION OF BUSINESS PROCESS

(redlined)

Natural gas is allocated among producers, operators, transportation service providers, service requesters and others after gas flows, using various methodologies to allocate actual quantities.  The Allocation identifies the distribution of quantities of gas at a specific location (allocation quantity).  The preparer name indicates the party that provides the Allocation is provided to the party(ies) that submitted the Pre-determined Allocation (PDA), if any, and any other appropriate party(ies) (statement recipient name).

The statement basis for the Allocation indicates whether the information being reported is actual, estimate or revision.  The statement date/time will indicate the date and time at which the statement was produced.  The contact person (name) and contact person (phone) includes information regarding the person who is responsible for responding to inquiries about the measurement information being provided.  The statement recipient ID is an industry common code indicating the party who is to receive the Allocation being provided.  The preparer ID is an industry common code indicating the party who prepared the Allocation.  Parties should mutually agree to use the Transportation Service Provider’s proprietary entity code when the D-U-N-S® Number is not available.

The report will include the beginning flow date, beginning flow time, ending flow date and ending flow time.  The accounting period represents the month and year the information was recorded.

The allocation transaction type code is a mandatory data element that identifies the type of allocation transaction.  It is used to identify the set of data elements (template) needed to communicate the results of the allocation process.  The preparer of the Allocation determines the allocation transaction type code that is appropriate, given the role of the party receiving the allocation information.  Reference the Allocation Transaction Type Matrix for essential implementation information.

Since allocations are performed at a location and it is possible for a location to have both receipt and delivery transactions, certain information is provided depending on the direction of flow.  If the gas is being received into the allocating Transportation Service Provider’s system it is considered a direction of flow of “receipt” and conversely, if the gas is being delivered out of the allocating Transportation Service Provider’s system it is considered a direction of flow of “delivery”.  The usage of the upstream and downstream identification information is dependent upon the allocation transaction type code.  It may be appropriate to communicate the upstream identifier code and the upstream contract identifier for receipt transactions at a location.  It may be appropriate to communicate the downstream identifier code and the downstream contract identifier for delivery transactions at a location.

Additional information providing details for the specific allocation data being reported includes, but is not limited to, scheduled quantity Parties should mutually agree to use the Transportation Service Provider’s proprietary entity code when the D-U-N-S® Number is not available.

SAMPLE PAPER TRANSACTION

(new)

The following example illustrates a possible representation of a paper transaction for the Allocation Transaction Type 6R:

Preparer ID:					999999999

Preparer Name:				XYZ Allocation Service

Contact Person (Name):			Joe Dallas

Contact Person (Phone):			214-555-1414

Statement Recipient ID:			111111111

Statement Recipient Name:			ABC Oil & Gas

Statement Date/Time:				04/01/96	4:00 PM

Allocation Transaction Type Code:		Allocation Transaction Type 6R

Accounting Period:				03/96

Beginning Flow Date:				03/15/96

Beginning Flow Time:				09:00 AM

Ending Flow Date:				04/01/96

Ending Flow Time:				09:00 AM

Location Code:				421331122

Location Name:				Mustang Island A-101

Direction of Flow:				Receipt

Upstream Identifier Code:			144326791

Upstream Identifier Entity Name:		Alpha Producing

Upstream Contract Identifier:			T-1882

Service Requester ID:				671234567

Service Requester Name:			Delta Shipper

Statement Basis Code Name			Actual

Scheduled Quantity:				950

Allocated Quantity:				1,000

#### DATA DICTIONARY

(redlined)

Standard No. 2.4.3

| ### Business Name (Abbreviation) | Definition | Data Group | EBB Usage | EDI / FF Usage | Condition |
| --- | --- | --- | --- | --- | --- |
| Direction of Flow (Dir Flo) | Allocated direction of flow (receipt from into or delivery toout of) from the perspective of the allocating tTransportation sService PProvider. | TSDG | M | M |  |

CODE VALUES DICTIONARY

(redlined)

## Direction of Flow

| Code Value Description | Code Value Definition | Code Value |
| --- | --- | --- |
| Delivery | [no definition necessary]Delivery out of allocating Transportation Service Provider’s system. | D |
| Receipt | [no definition necessary]Receipt into allocating Transportation Service Provider’s system. | R |

DATA ELEMENT CROSS REFERENCE TO ASC X12

(new)

Usage Codes: 	M - Mandatory, C - Conditional, SO - Sender’s Option, BC - Business Conditional, MA - Mutually Agreeable, nu - not used

Heading:

| Segment | Usage | Segment Name/GISB Data Element Name |
| --- | --- | --- |
| ST | M | Transaction Set Header segment |
| BGN | M | Beginning Segment |
| DTM | M | Accounting Period |
| DTM | M | Statement Date/Time |
| N1 | M | Statement Recipient ID/Statement Recipient ID Proprietary Code |
| N1 PER | M M M | Preparer ID/Preparer ID Proprietary Code Contact Person (Name) Contact Person (Phone) |

Detail:

| Segment | Usage | Segment Name/GISB Data Element Name |
| --- | --- | --- |
| DTM | M M M M | Beginning Flow Date Beginning Flow Time Ending Flow Date Ending Flow Time |
| LQ | M | Allocation Transaction Type Code |
| LQ | M | Direction of Flow |
| LQ | M | Statement Basis |
| LQ | C | Adjustment Type |
| LCD | M | Location Code/Location Proprietary Code |
| QTY | MA | Ending Imbalance Quantity |
| AMT | MA | Ending Imbalance Value |
| Sub-detail: |  |  |
| SLN | C | Bid Transportation Rate |
| LQ | C | Transaction Type |
| N9 | C | Service Provider’s Activity Code |
| N9 | C | Associated Contract |
| N9 | C | Package ID |
| QTY | M | Allocated Quantity |
| QTY | M | Scheduled Quantity |
| QTY | SO | Operational Quantity |
| QTY | SO | Penalty Variance Quantity |
| N1 N9 | C C | Service Requester ID/Service Requester Proprietary Code Service Requester Contract |
| N1 LCD N9 N9 | C C C C | Downstream Identifier Code/Downstream Identifier Proprietary Code Delivery Location/Delivery Location Proprietary Code Downstream Contract Identifier Downstream Package ID |
| N1 LCD N9 N9 | C C C C | Upstream Identifier Code/Upstream Identifier Proprietary Code Receipt Location/Receipt Location Proprietary Code Upstream Contract Identifier Upstream Package ID |

Summary:

| Segment | Usage | Segment Name/GISB Data Element Name |
| --- | --- | --- |
| SE | M | Transaction Set Trailer segment |

SAMPLE ASC X12 TRANSACTION

(new)

ST*873*0001

BGN*00*5678*19960401****F5

DTM*582****CM*199603

DTM*102****DT*199604011600

N1*40**1*111111111

N1*41**1*999999999

PER*IC*Joe Dallas*TE*2145551414

DTM*405****RDT*199603150900-199604010900

LQ*ATT*06R

LQ*DOF*R

LQ*SMB*A

LCD**LCN***421331122

SLN*1**I

QTY*H8*1000*BZ

QTY*H9*950*BZ

N1*78**1*671234567

N1*US**1*144326791

N9*UP*T-1882

SE*19*0001

873 Commodity Movement Services

(New)

Functional Group ID=CU

Heading:

Pos.	Seg.		Req.		Loop

No.	ID	Name	Des.	Max.Use	Repeat

| Must Use | 0100 | ST | Transaction Set Header | M | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Must Use | 0200 | BGN | Beginning Segment | M | 1 |  |  |  |  |  |  |  |  |
| Must Use | 0400 | DTM | Date/Time Reference | O | >1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - N1 |  |  | >1 |  |  |  |  |  |  |  |
| Must Use | 0500 | N1 | Party Identification | M | 1 |  |  |  |  |  |  |  |  |
| Must Use | 0900 | PER | Administrative Communications Contact | O | 3 |  |  |  |  |  |  |  |  |

Detail:

Pos.	Seg.		Req.		Loop

No.	ID	Name	Des.	Max.Use	Repeat

|  |  |  | LOOP ID - DTM |  |  | >1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Must Use | 0100 | DTM | Date/Time Reference | M | 1 |  |  |  |  |  |  |  |  |
| Must Use | 0300 | LQ | Industry Code Identification | O | >1 |  |  |  |  |  |  |  |  |
| Must Use | 0400 | LCD | Place/Location Description | O | 1 |  |  |  |  |  |  |  |  |
|  | 0470 | QTY | Quantity Information | O | >1 |  |  |  |  |  |  |  |  |
|  | 0490 | AMT | Monetary Amount Information | O | 1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - SLN |  |  | >1 |  |  |  |  |  |  |  |
| Must Use | 0700 | SLN | Subline Item Detail | O | 1 |  |  |  |  |  |  |  |  |
|  | 0800 | LQ | Industry Code Identification | O | >1 |  |  |  |  |  |  |  |  |
|  | 0900 | N9 | Extended Reference Information | O | >1 |  |  |  |  |  |  |  |  |
| Must Use | 1000 | QTY | Quantity Information | O | >1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - N1 |  |  | >1 |  |  |  |  |  |  |  |
|  | 1100 | N1 | Party Identification | O | 1 |  |  |  |  |  |  |  |  |
|  | 1200 | LCD | Place/Location Description | O | 1 |  |  |  |  |  |  |  |  |
|  | 1300 | N9 | Extended Reference Information | O | >1 |  |  |  |  |  |  |  |  |
| Must Use | 1600 | SE | Transaction Set Trailer | M | 1 |  |  |  |  |  |  |  |  |

Segment:	ST Transaction Set Header

Position:	0100

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | ST01 | 143 | Transaction Set Identifier Code |  |  |  | M | 1 | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 873 |  | Commodity Movement Services |  |  |  |  |
| Must Use | ST02 | 329 | Transaction Set Control Number |  |  |  | M | 1 | AN 4/9 |  |

Segment:	BGN Beginning Segment

Position:	0200

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | BGN01 | 353 | Transaction Set Purpose Code |  |  |  | M | 1 | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 00 |  | Original |  |  |  |  |
| Must Use | BGN02 | 127 | Reference Identification |  |  |  | M | 1 | AN 1/22 |  |
|  |  |  | The allocation number assigned by the allocation originator. The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |
| Must Use | BGN03 | 373 | Date |  |  |  | M | 1 | DT 8/8 |  |
|  |  |  | The allocation issue date. |  |  |  |  |  |  |  |
| Must Use | BGN07 | 640 | Transaction Type Code |  |  |  | O | 1 | ID 2/2 |  |
|  |  |  |  | F5 |  | Allocation |  |  |  |  |

Segment:	DTM Date/Time Reference

Position:	0400

Loop:

Level:	Heading

Usage:	Optional (Must Use)

Max Use:	>1

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier | M | 1 | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Refer to "DTM Segments (Heading)" table for usage and values. |  |  |  |  |
| Must Use | DTM05 | 1250 | Date Time Period Format Qualifier | X | 1 | ID 2/3 |  |
|  |  |  | Refer to "DTM Segments (Heading)" table for usage and values. |  |  |  |  |
| Must Use | DTM06 | 1251 | Date Time Period | X | 1 | AN 1/35 |  |
|  |  |  | Refer to "DTM Segments (Heading)" table for usage and values. Accounting Period, Statement Date/Time |  |  |  |  |

Segment:	N1 Party Identification

Position:	0500

Loop:	N1        Mandatory

Level:	Heading

Usage:	Mandatory

Max Use:	1

| Notes: |  | For NAESB WGQ, this segment is mandatory and should occur once for each value in the N101 element. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M | 1 | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 40 |  | Receiver |  |  |  |  |
|  |  |  |  |  |  | Statement Recipient ID |  |  |  |  |
|  |  |  |  | 41 |  | Submitter |  |  |  |  |
|  |  |  |  |  |  | Preparer ID |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X | 1 | ID 1/2 |  |
|  |  |  | Parties should mutually agree to use the Transportation Service Provider's proprietary entity code when the D-U-N-S® Number is not available. |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |
|  |  |  |  |  |  | For NAESB WGQ, this code value may only be used when sending the Statement Recipient ID or the Preparer ID. |  |  |  |  |
|  |  |  |  | SV |  | Service Provider Number |  |  |  |  |
|  |  |  |  |  |  | For NAESB WGQ, this code value may only be used when sending the Statement Recipient ID Proprietary Code or Preparer ID Proprietary Code. |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X | 1 | AN 2/17 |  |
|  |  |  | Statement Recipient ID/Statement Recipient ID Proprietary Code, Preparer ID/Preparer ID Proprietary Code The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |

Segment:	PER Administrative Communications Contact

Position:	0900

Loop:	N1        Mandatory

Level:	Heading

Usage:	Optional (Must Use)

Max Use:	3

| Notes: |  | For NAESB WGQ, this segment is mandatory and may only be used in the N1 loop for the Preparer ID (N101 = '41'). |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | PER01 | 366 | Contact Function Code |  |  |  | M | 1 | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | IC |  | Information Contact |  |  |  |  |
| Must Use | PER02 | 93 | Name |  |  |  | O | 1 | AN 1/35 |  |
|  |  |  | Contact Person (Name) The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |
| Must Use | PER03 | 365 | Communication Number Qualifier |  |  |  | X | 1 | ID 2/2 |  |
|  |  |  |  | TE |  | Telephone |  |  |  |  |
| Must Use | PER04 | 364 | Communication Number |  |  |  | X | 1 | AN 1/80 |  |
|  |  |  | Contact Person (Phone) The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference

Position:	0100

Loop:	DTM        Mandatory

Level:	Detail

Usage:	Mandatory

Max Use:	1

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M | 1 | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 405 |  | Production |  |  |  |  |
| Must Use | DTM05 | 1250 | Date Time Period Format Qualifier |  |  |  | X | 1 | ID 2/3 |  |
|  |  |  |  | RD8 |  | Range of Dates Expressed in Format CCYYMMDD-CCYYMMDD |  |  |  |  |
|  |  |  |  |  |  | This code designates the first "gas day" through the last "gas day" in which gas will flow. Example: For a gas day beginning and ending at 9:00am, the entire month of April 2011 would be stated 20110401-20110430. |  |  |  |  |
|  |  |  |  | RDT |  | Range of Date and Time, Expressed in Format CCYYMMDDHHMM-CCYYMMDDHHMM |  |  |  |  |
|  |  |  |  |  |  | This code designates the instance in time when the gas will begin to flow through the instance in time when the gas will cease to flow. Example: For a gas day beginning and ending at 9:00am, the entire month of April 2011 would be stated 201104010900-201105010900. |  |  |  |  |
| Must Use | DTM06 | 1251 | Date Time Period |  |  |  | X | 1 | AN 1/35 |  |
|  |  |  | Beginning Flow Date, Beginning Flow Time, Ending Flow Date, Ending Flow Time |  |  |  |  |  |  |  |

Segment:	LQ Industry Code Identification

Position:	0300

Loop:	DTM        Mandatory

Level:	Detail

Usage:	Optional (Must Use)

Max Use:	>1

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | LQ01 | 1270 | Code List Qualifier Code | O | 1 | ID 1/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Refer to "LQ Segments (Detail)" table for usage and values. |  |  |  |  |
| Must Use | LQ02 | 1271 | Industry Code | X | 1 | AN 1/30 |  |
|  |  |  | Allocation Transaction Type Code, Direction of Flow, Statement Basis, Adjustment Type Refer to "LQ Segments (Detail)" table for usage and values. |  |  |  |  |

Segment:	LCD Place/Location Description

Position:	0400

Loop:	DTM        Mandatory

Level:	Detail

Usage:	Optional (Must Use)

Max Use:	1

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | LCD02 | 98 | Entity Identifier Code |  |  |  | O | 1 | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | LCN |  | Gas Nomination Location |  |  |  |  |
| Must Use | LCD05 | 66 | Identification Code Qualifier |  |  |  | X | 1 | ID 1/2 |  |
|  |  |  | When a Transportation Service Provider's proprietary code is employed pursuant to this standard, the parties agree that nominations, confirmations, scheduled quantities, and capacity release documents employing such code should be for one gas day at a time, and used only until there is a verified common code for the point associated with the proprietary location code. This would include daily nominations over a weekend. Within two months following the availability of the location the parties should employ the common code and no longer employ the proprietary code for identifying such location in the datasets related to the identified standards. |  |  |  |  |  |  |  |
|  |  |  |  | DR |  | Gas Industry Standards Board (GISB) Data Reference Number (DRN) |  |  |  |  |
|  |  |  |  |  |  | For NAESB, this code value may only be used when sending the Location Code. |  |  |  |  |
|  |  |  |  | SV |  | Service Provider Number |  |  |  |  |
|  |  |  |  |  |  | For NAESB, this code value may only be used when sending the Location Proprietary Code. |  |  |  |  |
| Must Use | LCD06 | 67 | Identification Code |  |  |  | X | 1 | AN 2/17 |  |
|  |  |  | Location Code/Location Proprietary Code The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |

Segment:	QTY Quantity Information

Position:	0470

Loop:	DTM        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	>1

| Notes: |  | For NAESB WGQ, this segment is mutually agreed. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | QTY01 | 673 | Quantity Qualifier |  |  |  | M | 1 | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | CB |  | Closing Statement Balance |  |  |  |  |
| Must Use | QTY02 | 380 | Quantity |  |  |  | X | 1 | R 1/15 |  |
|  |  |  | Ending Imbalance Quantity |  |  |  |  |  |  |  |
|  | QTY03 | C001 | Composite Unit of Measure |  |  |  | O | 1 |  |  |
| Must Use | C00101 | 355 | Unit or Basis for Measurement Code |  |  |  | M |  | ID 2/2 |  |
|  |  |  |  | BZ |  | Million BTU's |  |  |  |  |
|  |  |  |  | G8 |  | Gigacalories |  |  |  |  |
|  |  |  |  | GV |  | Gigajoules |  |  |  |  |

Segment:	AMT Monetary Amount Information

Position:	0490

Loop:	DTM        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	1

| Notes: |  | For NAESB WGQ, this segment is mutually agreed. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | AMT01 | 522 | Amount Qualifier Code |  |  |  | M | 1 | ID 1/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | J8 |  | Ending Balance |  |  |  |  |
| Must Use | AMT02 | 782 | Monetary Amount |  |  |  | M | 1 | R 1/15 |  |
|  |  |  | Ending Imbalance Value The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |

Segment:	SLN Subline Item Detail

Position:	0700

Loop:	SLN        Optional (Must Use)

Level:	Detail

Usage:	Optional (Must Use)

Max Use:	1

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | SLN01 | 350 | Assigned Identification |  |  |  | M | 1 | AN 1/11 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | An identifier assigned by the allocation originator. The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |
| Must Use | SLN03 | 662 | Relationship Code |  |  |  | M | 1 | ID 1/1 |  |
|  |  |  |  | I |  | Included |  |  |  |  |
|  | SLN06 | 212 | Unit Price |  |  |  | X | 1 | R 1/14 |  |
|  |  |  | Bid Transportation Rate For NAESB WGQ, this element is conditional. The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |

Segment:	LQ Industry Code Identification

Position:	0800

Loop:	SLN        Optional (Must Use)

Level:	Detail

Usage:	Optional

Max Use:	>1

| Notes: |  | For NAESB WGQ, this segment is conditional based upon the usage as defined by the Allocation Transaction Type Matrix. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | LQ01 | 1270 | Code List Qualifier Code |  |  |  | O | 1 | ID 1/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TT |  | Natural Gas Transaction Type |  |  |  |  |
| Must Use | LQ02 | 1271 | Industry Code |  |  |  | X | 1 | AN 1/30 |  |
|  |  |  | 01 Current Business (default) 02 Authorized Contract Overrun 03 Imbalance Payback from Transportation Service Provider 04 Imbalance Payback to Transportation Service Provider 05 Plant Thermal Reduction 06 Storage Injection 07 Storage Withdrawal 08 Pooling 09 Imbalance Transfer 11 Storage Inventory Transfer 12 Authorized Injection Overrun 13 Authorized Withdrawal Overrun 14 Extended Receipt / Delivery Service 16 No-Notice Balancing 17 No-Notice Pre-Injection 18 Suspense Gas Claim 19 Delivery of Claimed Suspense Gas 22 No-Notice Service 24 No-Notice Due Transportation Service Provider Balancing 25 No-Notice Due Service Requester Balancing 26 Park 27 Park Withdrawal 28 Loan 29 Loan Payback 31 Meter Bounce 41 Storage Inventory Cycling 48 Authorized Point Overrun 50 Unauthorized Overrun 52 TSP Deficiency Credit 53 SR Deficiency Credit 54 Pool-to-Pool 55 Backhaul 56 Flow Day Diversion 57 Authorized Point Overrun - Backhaul 58 Authorized Point Overrun - Shorthaul 59 Bank 60 Bank Payback 61 Take 62 Take Payback 63 Imbalance Payback from Transportation Service Provider – Prior Business Period 64 Imbalance Payback to Transportation Service Provider – Prior Business Period 84 Segmented 85 Segmented – Volumetric 86 Segmented / Firm Storage 87 Segmented / Interruptible Storage 88 Segmented – Volumetric / Firm Storage 89 Segmented – Volumetric / Interruptible Storage 90 Reservation / Firm Storage 91 Reservation / Interruptible Storage 92 Nominated No-Notice / Firm Storage 93 Nominated No-Notice / Interruptible Storage 94 Nominated No-Notice – Small Shipper / Firm Storage 95 Nominated No-Notice – Small Shipper / Interruptible Storage 96 Volumetric / Firm Storage 97 Volumetric / Interruptible Storage 98 Nominated No-Notice 99 Volumetric 100 Nominated No-Notice – Small Shipper 101 No-Notice – Small Shipper 102 Meter Bounce - Overrun 108 Lost/Unaccounted For Gas 109 Retrograde 110 Retrograde - Flash 117 Off-system Market 118 Off-system Supply 119 Purchase 120 Sale 132 Wheeling 134 Incremental Capacity – TSP Defined 1 135 Authorized Overrun Incremental Capacity – TSP Defined 1 138 Flash Gas 139 Plant Liquids 140 Out of Cycle |  |  |  |  |  |  |  |

Segment:	N9 Extended Reference Information

Position:	0900

Loop:	SLN        Optional (Must Use)

Level:	Detail

Usage:	Optional

Max Use:	>1

| Notes: |  | For NAESB WGQ, this segment is conditional based upon the usage as defined by the Allocation Transaction Type Matrix. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | N901 | 128 | Reference Identification Qualifier |  |  |  | M | 1 | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | BE |  | Business Activity |  |  |  |  |
|  |  |  |  |  |  | Service Provider's Activity Code |  |  |  |  |
|  |  |  |  | KAS |  | Associated Contract Identifier |  |  |  |  |
|  |  |  |  | PKG |  | Package Identifier |  |  |  |  |
| Must Use | N902 | 127 | Reference Identification |  |  |  | X | 1 | AN 1/30 |  |
|  |  |  | Service Provider's Activity Code, Associated Contract, Package ID The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |

Segment:	QTY Quantity Information

Position:	1000

Loop:	SLN        Optional (Must Use)

Level:	Detail

Usage:	Optional (Must Use)

Max Use:	>1

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | QTY01 | 673 | Quantity Qualifier |  |  |  | M | 1 | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Refer to "QTY Segments (Sub-detail)" table for usage and values. |  |  |  |  |  |  |  |
| Must Use | QTY02 | 380 | Quantity |  |  |  | X | 1 | R 1/15 |  |
|  |  |  | Allocated Quantity, Scheduled Quantity, Operational Quantity, Penalty Variance Quantity Refer to "QTY Segments (Sub-detail)" table for usage and values. |  |  |  |  |  |  |  |
|  | QTY03 | C001 | Composite Unit of Measure |  |  |  | O | 1 |  |  |
| Must Use | C00101 | 355 | Unit or Basis for Measurement Code |  |  |  | M |  | ID 2/2 |  |
|  |  |  |  | BZ |  | Million BTU's |  |  |  |  |
|  |  |  |  | G8 |  | Gigacalories |  |  |  |  |
|  |  |  |  | GV |  | Gigajoules |  |  |  |  |

Segment:	N1 Party Identification

Position:	1100

Loop:	N1        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

| Notes: |  | For NAESB WGQ, this segment is conditional based upon the usage as defined by the Allocation Transaction Type Matrix. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M | 1 | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 78 |  | Service Requester |  |  |  |  |
|  |  |  |  | DW |  | Downstream Party |  |  |  |  |
|  |  |  |  | US |  | Upstream Party |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X | 1 | ID 1/2 |  |
|  |  |  | Parties should mutually agree to use the Transportation Service Provider's proprietary entity code when the D-U-N-S® Number is not available. |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |
|  |  |  |  |  |  | For NAESB WGQ, this code value may only be used when sending the Service Requester ID, Downstream Identifier Code, or Upstream Identifier Code. |  |  |  |  |
|  |  |  |  | SV |  | Service Provider Number |  |  |  |  |
|  |  |  |  |  |  | For NAESB WGQ, this code value may only be used when sending the Service Requester Proprietary Code, Downstream Identifier Proprietary Code, or Upstream Identifier Proprietary Code. |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X | 1 | AN 2/17 |  |
|  |  |  | Service Requester ID/Service Requester Proprietary Code, Downstream Identifier Code/Downstream Identifier Proprietary Code, Upstream Identifier Code/Upstream Identifier Proprietary Code The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |

Segment:	LCD Place/Location Description

Position:	1200

Loop:	N1        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

| Notes: |  | For NAESB WGQ, this segment is conditional based upon the usage as defined by the Allocation Transaction Type Matrix. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | LCD02 | 98 | Entity Identifier Code |  |  |  | O | 1 | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | M2 |  | Receipt Location |  |  |  |  |
|  |  |  |  |  |  | For NAESB WGQ, this code value may only be used in the upstream N1 loop (N101 = 'US'). |  |  |  |  |
|  |  |  |  | MQ |  | Delivery Location |  |  |  |  |
|  |  |  |  |  |  | For NAESB WGQ, this code value may only be used in the downstream N1 loop (N101 = 'DW'). |  |  |  |  |
| Must Use | LCD05 | 66 | Identification Code Qualifier |  |  |  | X | 1 | ID 1/2 |  |
|  |  |  | When a Transportation Service Provider's proprietary code is employed pursuant to this standard, the parties agree that nominations, confirmations, scheduled quantities, and capacity release documents employing such code should be for one gas day at a time, and used only until there is a verified common code for the point associated with the proprietary location code. This would include daily nominations over a weekend. Within two months following the availability of the location the parties should employ the common code and no longer employ the proprietary code for identifying such location in the datasets related to the identified standards. |  |  |  |  |  |  |  |
|  |  |  |  | DR |  | Gas Industry Standards Board (GISB) Data Reference Number (DRN) |  |  |  |  |
|  |  |  |  |  |  | For NAESB WGQ, this code value may only be used when sending the Receipt Location or Delivery Location. |  |  |  |  |
|  |  |  |  | SV |  | Service Provider Number |  |  |  |  |
|  |  |  |  |  |  | For NAESB WGQ, this code value may only be used when sending the Receipt Location Proprietary Code or Delivery Location Proprietary Code. |  |  |  |  |
| Must Use | LCD06 | 67 | Identification Code |  |  |  | X | 1 | AN 2/17 |  |
|  |  |  | Receipt Location/Receipt Location Proprietary Code, Delivery Location/Delivery Location Proprietary Code The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |

Segment:	N9 Extended Reference Information

Position:	1300

Loop:	N1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

| Notes: |  | For NAESB WGQ, this segment is conditional based upon the usage as defined by the Allocation Transaction Type Matrix. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | N901 | 128 | Reference Identification Qualifier |  |  |  | M | 1 | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DT |  | Downstream Shipper Contract Number |  |  |  |  |
|  |  |  |  |  |  | For NAESB WGQ, this code value may only be used in the upstream N1 loop (N101 = 'DW'). |  |  |  |  |
|  |  |  |  | KSR |  | Service Requester Contract Identifier |  |  |  |  |
|  |  |  |  |  |  | For NAESB WGQ, this code value may only be used in the service requester N1 loop (N101 = '78'). |  |  |  |  |
|  |  |  |  | PGD |  | Downstream Package Identifier |  |  |  |  |
|  |  |  |  |  |  | For NAESB WGQ, this code value may only be used in the upstream N1 loop (N101 = 'DW'). |  |  |  |  |
|  |  |  |  | PKU |  | Upstream Package Identifier |  |  |  |  |
|  |  |  |  |  |  | For NAESB WGQ, this code value may only be used in the upstream N1 loop (N101 = 'US'). |  |  |  |  |
|  |  |  |  | UP |  | Upstream Shipper Contract Number |  |  |  |  |
|  |  |  |  |  |  | For NAESB WGQ, this code value may only be used in the upstream N1 loop (N101 = 'US'). |  |  |  |  |
| Must Use | N902 | 127 | Reference Identification |  |  |  | X | 1 | AN 1/30 |  |
|  |  |  | Downstream Contract Identifier, Service Requester Contract, Downstream Package ID, Upstream Package ID, Upstream Contract Identifier The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |

Segment:	SE Transaction Set Trailer

Position:	1600

Loop:

Level:	Detail

Usage:	Mandatory

Max Use:	1

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | SE01 | 96 | Number of Included Segments | M | 1 | N0 1/10 |
| --- | --- | --- | --- | --- | --- | --- |
| Must Use | SE02 | 329 | Transaction Set Control Number | M | 1 | AN 4/9 |

TRANSACTION SET TABLES

(new)

DTM Segments (Heading)

| Element Name (DTM06) | Usage | DTM01 | DTM05 |
| --- | --- | --- | --- |
| Accounting Period | M | 582 | CM |
| Statement Date/Time | M | 102 | DT |

LQ Segments (Detail)

| Element Name (LQ02) | Usage | LQ01 | LQ02 | LQ02 Description |
| --- | --- | --- | --- | --- |
| Allocation Transaction Type Code | M | ATT | 01X | Allocation Transaction Type 1 |
|  |  |  | 02X | Allocation Transaction Type 2 |
|  |  |  | 03R | Allocation Transaction Type 3R |
|  |  |  | 03D | Allocation Transaction Type 3D |
|  |  |  | 04R | Allocation Transaction Type 4R |
|  |  |  | 04D | Allocation Transaction Type 4D |
|  |  |  | 05R | Allocation Transaction Type 5R |
|  |  |  | 05D | Allocation Transaction Type 5D |
|  |  |  | 06R | Allocation Transaction Type 6R |
|  |  |  | 06D | Allocation Transaction Type 6D |
|  |  |  | 07R | Allocation Transaction Type 7R |
|  |  |  | 07D | Allocation Transaction Type 7D |
|  |  |  | 08R | Allocation Transaction Type 8R |
|  |  |  | 08D | Allocation Transaction Type 8D |
|  |  |  | 09R | Allocation Transaction Type 9R |
|  |  |  | 09D | Allocation Transaction Type 9D |
|  |  |  | 10R | Allocation Transaction Type 10R |
|  |  |  | 10D | Allocation Transaction Type 10D |
|  |  |  | 11R | Allocation Transaction Type 11R |
|  |  |  | 11D | Allocation Transaction Type 11D |
|  |  |  | 12R | Allocation Transaction Type 12R |
|  |  |  | 12D | Allocation Transaction Type 12D |
|  |  |  | 13X | Allocation Transaction Type 13 |
|  |  |  | 14R | Allocation Transaction Type 14R |
|  |  |  | 14D | Allocation Transaction Type 14D |
|  |  |  | 15R | Allocation Transaction Type 15R |
|  |  |  | 15D | Allocation Transaction Type 15D |
| Direction of Flow | M | DOF | D | Delivery |
|  |  |  | R | Receipt |
| Statement Basis | M | SMB | A | Actual |
|  |  |  | E | Estimate |
|  |  |  | R | Revision |
| Adjustment Type | C | AJT | ADC | Allocation detail correction |
|  |  |  | AMC | Allocation Method correction |
|  |  |  | AQC | Actual quantity correction |
|  |  |  | FQC | Fuel Quantity correction |
|  |  |  | IKP | Imbalance Payback Prior Business Period |
|  |  |  | IRP | Imbalance Payback Prior Business Residual |
|  |  |  | ITS | Imbalance to Storage |
|  |  |  | LQC | Allocated Quantity correction |
|  |  |  | SQC | Scheduled Quantity correction |

QTY Segments (Sub-detail)

| Element Name (QTY02) | Usage | QTY01 |
| --- | --- | --- |
| Allocated Quantity | M | H8 |
| Scheduled Quantity | M | H9 |
| Operational Quantity | SO | H6 |
| Penalty Variance Quantity | SO | H7 |