TECHNICAL IMPLEMENTATION OF BUSINESS PROCESS

(new)

The Measured Volume Audit Statement (MAUS) is a mutually agreed data set used to report gas measurement details to the operator and other parties at a metering location.  The statement reflects the components of the volume calculation.

The statement type for the MAUS indicates whether the information being reported is original, replacement or advance notification.  The statement date/time will indicate the date and time at which the statement was produced.  The contact person (name) and contact person (phone) provides information regarding the person who is responsible for responding to inquiries about the measurement information being provided.  The preparer ID, when sent, is an industry common code indicating the party who prepared the measurement information.  Parties should mutually agree to use the Transportation Service Provider’s proprietary entity code when the D-U-N-S® Number is not available.

When the meter operator is known, this information should be included in the statement.  When the meter operator is not known, this information does not need to be sent.

The device station number represents the identification assigned by the meter operator of the station.  A station can have one or more meters and each one is identified by the meter ID.  Both the device station number and the meter ID are proprietary numbers.

The report will include the measured quantity for a beginning flow date, beginning flow time, ending flow date, and ending flow time.  The business period indicates whether the report is for the current period (default) or a prior period.  If the detail line is for an adjustment, the adjustment type will indicate the adjustment being made.

Additional information providing details for the specific measurement data being reported include, but are not limited to, flow period, maximum static pressure range, minimum static pressure range, meter status, meter type, physical meter effective date, reporting pressure base, sample device, specific gravity, and volume.

SAMPLE PAPER TRANSACTION

(new)

This sample paper is only a visual representation of the data provided in the ANSI X12 EDI file for an orifice EFM meter.  Additional optional data can be sent for this type of meter, but such information is not reflected in this sample.  There is no NAESB requirement to provide this data on the EBB.

Statement Date/Time:					July 3, 1997 10:30 AM

Statement Type:						Original

Contact Person (Name):				Mary Metermaid

Contact Person (Phone):				867-123-5309

Device Station Number:				0012345-00

Meter ID:							0012345-12

Physical Meter Effective Date:			June 1, 1997

Meter Type:							Orifice - EFM

Meter Status:						Active

Reporting Pressure Base:				14.65 		Unit of Measure:	PSIA

Tube Inside Diameter:				10.020 		Unit of Measure:	Inches, Decimal - Actual

Tap Type:							Flange Tap

Minimum Static Pressure Range:		0 			Unit of Measure:	PSIA

Maximum Static Pressure Range:		500 			Unit of Measure:	PSIA

Tap Location:						Downstream Tap

Maximum Differential Pressure:		150 			Unit of Measure:	Inches of Water

Orifice Diameter:						6.000 		Unit of Measure:	Inches, Decimal - Actual

Beginning Flow Date:					June 1, 1997

Beginning Flow Time:					09:00 AM

Ending Flow Date:					June 2, 1997

#### Ending Flow Time:					09:00 AM

#### Measured Quantity:					7221 		Unit of Measure:	Million BTUs

Volume:								6903 		Unit of Measure:	Thousand Cubic Feet

Flow Period:							19.4			Unit of Measure:	Hours

Flow Rate:							667

Business Period:						Current

Specific Gravity:						0.6000

Static Pressure:						214 			Unit of Measure:	PSIA

Differential Pressure:					26.2 		Unit of Measure: Inches of Water

Sample Device:						Chromatograph

*******

Note: More detail pages, 1 for each date range of Flowing Gas Information, would be provided for this reporting period.  Only one date range is depicted here.

DATA DICTIONARY

(new)

# Standard No. 2.4.6

| ### Business Name (Abbreviation) | Definition | EDI / FF Usage | Condition |
| --- | --- | --- | --- |
| Adjustment Type (Adj Type) | Identifies the type of adjustment. | C | Based upon the Statement Type not being ‘original’. |
| Atmospheric Pressure (Atmos Press) | The site atmospheric pressure measurement. | SO |  |
| Beginning Flow Date (Beg Date) | The date on which the transportation/transaction first started. | M |  |
| Beginning Flow Time (Beg Time) | The time on which the transportation/transaction first started. If the Beginning Flow Time is not sent, the time defaults to the beginning of the gas day. | M |  |
| Business Period (Bus Per) | Current (default) or prior period indicator. | M |  |
| Chart Revolution Time (Chart Rev Time) | Specifies the chart revolution time for this metering device. | C | Mandatory when Meter Type is one of the following: - ‘Orifice – chart’; or - ‘Time Driven Positive Displacement – chart’, otherwise not used. |
| Chromatograph (Chrmtg) | Specifies the source of gas quality information. | C | Used only when the following conditions are met: - Sample Device is ‘Chromatograph’, and, - Component (quality) information is provided. |
| Coefficient (Coeff) | Provides coefficient factor. | C | Mandatory when Meter Type is ‘Orifice – chart’; otherwise not used. |
| Component (Cmpnt) | Identifies the gas component being reported. | SO |  |
| Component Value (Cmpnt Val) | The value of a component of gas. | C | Mandatory when Component is sent. |
| Contact Person Data | The name and telephone number of the contact for questions regarding the reported measurement information. |  |  |
| Contact Person (Name) (Contact Name) |  | M |  |
| Contact Person (Phone) (Contact Phone) |  | M |  |
| Date/Time Off (D/T Off) | Specifies the date and time for which measurement ended. | C | Mandatory when Meter Type is one of the following: - ‘Orifice – chart’; or - ‘Time Driven Positive Displacement – chart’, otherwise not used. |
| Date/Time On (D/T On) | Specifies the date and time for which measurement began. | C | Mandatory when Meter Type is one of the following: - ‘Orifice – chart’; or - ‘Time Driven Positive Displacement – chart’, otherwise not used. |
| Device Station Number (Sta No) | The station number assigned to this device by the operator. | M |  |
| Differential Pressure (Diff Press) | The differential pressure for the meter during the flow period. | C | Mandatory when Meter Type is ‘Orifice – EFM’; otherwise not used. |
| Downstream Party Data | Identifies the party to whom gas is flowing. |  |  |
| Downstream Party * 4 (Dn Pty) |  | SO |  |
| Downstream Party Proprietary Code (Dn Pty Prop) |  | SO |  |
| Ending Flow Date (End Date) | The date on which the transportation/transaction ended. | M |  |
| Ending Flow Time (End Time) | The time on which the transportation/transaction ended. If the Ending Flow Time is not sent, the time defaults to the end of the gas day. | M |  |
| Extension (Ext) | A factor used to calculate the gas volume for orifice type meters. | MA | Used only when Meter Type is one of the following: - ‘Orifice – chart’; or - ‘Orifice – EFM’. |
| Flow Period (Flow Per) | The length of time flow. | M |  |
| Flow Rate (Flow Rate) | The flow rate of the EFM orifice meter. | C | Mandatory when Meter Type is ‘Orifice – EFM; otherwise not used. |
| Gas Analysis Effective Date (G A Eff Date) | Specifies the date the gas quality information was determined | C | Mandatory when Component (quality) information is provided. |
| Heating Factor (Heat Fctr) | Energy factor associated with the measured volume. | C | Mandatory when Component (quality) information is sent. |
| Index Differential (Index Diff) | Provides the indexed differential. | C | Mandatory when Meter Type is one of the following: - ‘Time Driven Positive Displacement – chart’; - ‘Time Driven Positive Displacement – EFM’; or - Flow Driven Positive Displacement, otherwise not used. |
| Integrated Differential (Intgr Diff) | Provides the integrated differential. | C | Mandatory when Meter Type is ‘Orifice – chart’; otherwise not used. |
| Location Data | Unique identification of a point. |  |  |
| Location Code* ** (Loc) |  | SO |  |
| Location Proprietary Code (Loc Prop) |  | SO |  |
| Machine Constant (Mach Cnst) | Conversion factor for scanners or analyzers. | C | Mandatory when Meter Type is one of the following: - ‘Orifice – chart’; or - ‘Time Driven Positive Displacement – chart’, otherwise not used. |
| Maximum Differential Pressure (Max Diff Press) | Specifies the maximum value of the differential pressure. | C | Mandatory when Meter Type is one of the following: - ‘Orifice – chart’; or - ‘Orifice – EFM’, otherwise not used. |
| Maximum Static Pressure Range (Max Static Press) | Specifies the maximum value of the static pressure range. | M |  |
| Measured Quantity (Meas Qty) | The quantity as measured in MMBTUs, gigacalories, or gigajoules. | M |  |
| Meter ID (Meter ID) | The Operator's ID number for the measurement device being reported. Proprietary meter number. | M |  |
| Meter Operator Data | The party contractually responsible for the measurement of gas at a meter. |  |  |
| Meter Operator * 4 (Operator) |  | SO | When the meter operator is known, at least one of Meter Operator or Meter Operator Proprietary Code should be included in the statement. |
| Meter Operator Proprietary Code (Operator Prop) |  | SO | When the meter operator is known, at least one of Meter Operator or Meter Operator Proprietary Code should be included in the statement. |
| Meter Status (Meter Stat) | Identifies the meter as active (default), removed, or on standby. | M |  |
| Meter Type (Meter Type) | Identifies the type of meter being used. | M |  |
| Minimum Static Pressure Range (Min Static Press) | Specifies the minimum value of the static pressure range. | M |  |
| Number Dials (Dials) | Specifies the number of dials for a positive meter. Used for positive meters only. | C | Mandatory when Meter Type is one of the following: - ‘Time Driven Positive Displacement – chart’; - ‘Time Driven Positive Displacement – EFM’; or - Flow Driven Positive Displacement, otherwise not used. |
| Orifice Diameter (Orif Diam) | Measurement of the diameter of the orifice plate. | C | Mandatory when Meter Type is one of the following: - ‘Orifice – chart’; or - ‘Orifice – EFM’, otherwise not used. |
| Physical Meter Effective Date (Meter Eff Date) | The effective date of physical meter information. | M |  |
| Physical Meter Effective Time (Meter Eff Time) | The effective time of physical meter information. | SO |  |
| Preparer Data | The name of the business party preparing the report. |  |  |
| Preparer ID * 4 (Prep ID) |  | SO |  |
| Preparer ID Proprietary Code (Prep ID Prop) |  | SO |  |
| Pressure Factor (Press Fctr) | Specifies the pressure factor for positive meter measurement. | C | Mandatory when Meter Type is one of the following: - ‘‘Time Driven Positive Displacement – chart’; - ‘Time Driven Positive Displacement – EFM’; or - Flow Driven Positive Displacement, otherwise not used. |
| Reporting Pressure Base (Rpt Press Base) | Pressure base used in reporting volume in MCFs. The default is 14.73 | M |  |
| Reporting Temperature (Rpt Temp) | Temperature used to report volume if different from actual temperature. | C | Mandatory when Reporting Temperature is different than Temperature, otherwise not used. |
| Sample Device (Smpl Dev) | Type of equipment used for sampling. | M |  |
| Sample Type (Smpl Type) | Specifies the sample as spot or accumulated. | C | Used only when Component (quality) information is provided. |
| Specific Gravity (Spec Grav) | The ratio of the weight of a given volume of a substance at a given temperature to the weight of an equal volume of a standard substance at the same temperature. | M |  |
| Statement Date/Time (Stmt D/T) | Date and time statement was produced. | M |  |
| Statement Type (Stmt Type) | Specifies the statement as original, replacement or advance notification. | M |  |
| Static Pressure (Static Press) | The static pressure (PSIA) for the meter during the flow period. | C | Mandatory when Meter Type is one of the following: - ‘Orifice – chart’; - ‘Orifice – EFM’, - ‘Time Driven Positive Displacement – chart’; - ‘Time Driven Positive Displacement – EFM’; or - Flow Driven Positive Displacement, otherwise not used. |
| Tap Location (Tap Loc) | The location of the meter tap. Locations are upstream or downstream. | C | Mandatory when Meter Type is one of the following: - ‘Orifice – chart’; or - ‘Orifice – EFM’, otherwise not used. |
| Tap Type (Tap Type) | Identifies the tap as flange or pipe. | C | Mandatory when Meter Type is one of the following: - ‘Orifice – chart’; or - ‘Orifice – EFM’, otherwise not used. |
| Temperature (Temp) | The temperature of the gas flow. | SO |  |
| Temperature Range Maximum (Temp Max) | The maximum temperature range for the recorder. | SO |  |
| Temperature Range Minimum (Temp Min) | The minimum temperature range for the recorder. | SO |  |
| Tube Inside Diameter (Tube Diam) | The inside diameter measurement of the tube. | C | Mandatory when Meter Type is one of the following: - ‘Orifice – chart’; or - ‘Orifice – EFM’, otherwise not used. |
| Unit of Measure (U/Meas) | Specifies the unit or basis for measurement for the corresponding measurement value. | C | Must be sent in conjunction with each of the following data elements: - Atmospheric Pressure - Chart Revolution Time - Differential Pressure - Flow Period - Heating Factor - Maximum Differential Pressure - Maximum Static Pressure Range - Measured Quantity - Minimum Static Pressure Range - Orifice Diameter - Reporting Pressure Base - Reporting Temperature - Static Pressure - Temperature - Temperature Range Maximum - Temperature Range Minimum - Tube Inside Diameter - Volume |
| Upstream Party Data | Identifies the party from whom gas is flowing. |  |  |
| Upstream Party * 4 (Up Pty) |  | SO |  |
| Upstream Party Proprietary Code (Up Pty Prop) |  | SO |  |
| Volume (Vol) | The quantity of gas expressed in MCF. | M |  |
| Volume Cycle (Vol Cycle) | Identifies the volume cycle for a positive meter. Used for positive meters only. | C | Mandatory when Meter Type is one of the following: - ‘Time Driven Positive Displacement – chart’; - ‘Time Driven Positive Displacement – EFM’; or - Flow Driven Positive Displacement, otherwise not used. |

RELEVANT FOOTNOTES:

* Indicates Common Code

4  Refer to NAESB WGQ Standard No. No. 0.3.2.

Code Values Dictionary

(new)

Adjustment Type

| Code Value Description | Code Value Definition | Code Value |
| --- | --- | --- |
| Audit Review - No Adjustment Necessary | An audit review was requested and performed. No adjustment was necessary based on the audit. | ARN |
| Data Entry Error | Previous data replaced with best available data. A data entry error occurred when the data was manually entered. | DEE |
| EFM Communication Failure | An EFM communication failure occurred. Previous data replaced with best available data. | ECF |
| Estimate Replaced with Actual: Late | Estimate replaced with actual. The actual data was not received in time to calculate volumes. | ERA |
| Incorrect Chromatograph Assignment/Data | Previous data replaced with best available data. Gas chromatograph was assigned incorrectly to a station. | ICA |
| Incorrect EFM Station Configuration | Previous data replaced with best available data. Incorrect gas analysis data was used for the station. | IEC |
| Incorrect Gas Quality | Previous data replaced with best available data. A reevaluation of the scan or integration process was performed resulting in a correction of data. | IGQ |
| Integration/Scan Process Incorrect | The original gas analysis taken was invalid and has been replaced. Examples of invalid gas samples include: liquids in the sample bottle, non-heated sample, calibration of equipment, etc. | IPI |
| Invalid Calculation Type | The EFM station configuration was incorrect causing invalid data. Previous data replaced with best available data. | ICT |
| Invalid Gas Sample | Previous data replaced with best available data. An invalid calculation methodology (e.g. AGA3, AGA7, AGA8, NX9) was used to calculate volume(s). | IGS |
| Late Notification of Station Turn On | Missing data replaced with best available data. A station started the initial physical flow prior to the receipt of station setup information. | LNS |
| Measured Change Not Received | A measurement parameter change occurred at the physical meter (e.g. plate change, static range change, etc.) and was not received in time to calculate volumes. | MCN |
| Mechanical Field Equipment Failure | A mechanical equipment failure occurred. Previous data replaced with best available data. | MEF |
| Missing Data, No Volume Estimate | Missing data replaced with best available data. No actual was available and no estimate was previously provided. | MNV |
| Quality Data Received Late | Estimated quality data is replaced with actual. Original gas analysis data was not received in time to calculate volumes. | QRL |
| Volume Estimate - No Data Received | Estimate replaced with best available data. No actual data received. | VEN |

Business Period

| ##### Code Value Description | Code Value Definition | Code Value |
| --- | --- | --- |
| Current | [no definition necessary] | 1 |
| Prior Period | [no definition necessary] | 2 |

# Component

| Code Value Description | Code Value Definition | Code Value |
| --- | --- | --- |
| Carbon Dioxide | [no definition necessary] | ZCD |
| Carbon Monoxide | [no definition necessary] | ZZZ |
| Ethane | [no definition necessary] | ZET |
| Helium | [no definition necessary] | HC |
| Heptane | [no definition necessary] | ZHP |
| Hexane | [no definition necessary] | ZHX |
| Hydrogen | [no definition necessary] | ZH |
| Hydrogen Sulfide | [no definition necessary] | ZHS |
| I-Butane | [no definition necessary] | ZIB |
| I-Pentane | [no definition necessary] | ZIP |
| Methane | [no definition necessary] | ZMT |
| N-Butane | [no definition necessary] | ZBT |
| Neo-Pentane | [no definition necessary] | ZNP |
| Nitrogen | [no definition necessary] | ZN |
| N-Pentane | [no definition necessary] | ZPT |
| Octane | [no definition necessary] | ZOC |
| Oxygen | [no definition necessary] | ZO |
| Propane | [no definition necessary] | ZPP |
| Sulphur | [no definition necessary] | ZS |
| Water | [no definition necessary] | H20 |

Meter Status

| Code Value Description | Code Value Definition | Code Value |
| --- | --- | --- |
| Active | [no definition necessary] | A |
| Removed | [no definition necessary] | R |
| Standby | The meter is available for flow but not currently active. | S |

# Meter Type

| Code Value Description | Code Value Definition | Code Value |
| --- | --- | --- |
| Flow Driven Positive Displacement | [no definition necessary] | FD |
| Orifice - chart | [no definition necessary] | OC |
| Orifice - EFM | [no definition necessary] | OM |
| Rotary | [no definition necessary] | RM |
| Time Driven Positive Displacement - chart | [no definition necessary] | DC |
| Time Driven Positive Displacement - EFM | [no definition necessary] | DM |
| Turbine | [no definition necessary] | TM |
| Ultrasonic | [no definition necessary] | UM |
| Venturi | [no definition necessary] | VM |

# Sample Device

| Code Value Description | Code Value Definition | Code Value |
| --- | --- | --- |
| Bottle | [no definition necessary] | B |
| Chromatograph | [no definition necessary] | C |
| Inline | [no definition necessary] | I |

# Sample Type

| Code Value Description | Code Value Definition | Code Value |
| --- | --- | --- |
| Accumulated | [no definition necessary] | A |
| Spot | [no definition necessary] | S |

# Statement Type

| Code Value Description | Code Value Definition | Code Value |
| --- | --- | --- |
| Advance Notification | [no definition necessary] | 14 |
| Original | [no definition necessary] | 00 |
| Replacement | [no definition necessary] | 05 |

# Tap Location

| Code Value Description | ##### Code Value Definition | Code Value |
| --- | --- | --- |
| Downstream Tap | [no definition necessary] | DT |
| Upstream Tap | [no definition necessary] | UT |

Tap Type

| Code Value Description | Code Value Definition | Code Value |
| --- | --- | --- |
| Flange Tap | [no definition necessary] | FT |
| Pipe Tap | [no definition necessary] | PT |

Unit of Measure

| Code Value Description | Code Value Definition | Code Value |
| --- | --- | --- |
| Centigrade, Celsius | [no definition necessary] | CE |
| Days | [no definition necessary] | DA |
| Fahrenheit | [no definition necessary] | FA |
| Gigacalories | [no definition necessary] | G8 |
| Gigajoules | [no definition necessary] | GV |
| Grains | [no definition necessary] | GX |
| Hours | [no definition necessary] | HR |
| Inches of Water | [no definition necessary] | IF |
| Inches, Decimal - Actual | [no definition necessary] | E8 |
| Kilopascal | [no definition necessary] | KQ |
| Megajoule/Cubic Meter | [no definition necessary] | JM |
| Millimeter - Actual | [no definition necessary] | M2 |
| Million BTUs | [no definition necessary] | BZ |
| Million BTUs per 1000 Cubic Feet | [no definition necessary] | M9 |
| Percent | [no definition necessary] | P1 |
| Pounds per Million Cubic Feet | [no definition necessary] | ZZ |
| PSI | Pounds per square inch | PS |
| PSIA | Pounds per square inch absolute | 80 |
| PSIG | Pounds per square inch gauge | 64 |
| Thousand Cubic Feet | [no definition necessary] | TZ |

DATA ELEMENT CROSS REFERENCE TO ASC X12

(new)

Usage Codes: 	M - Mandatory, C - Conditional, SO - Sender’s Option, BC - Business Conditional, MA - Mutually Agreeable, nu - not used

Heading:

| Segment | Usage | Segment Name/NAESB Data Element Name |
| --- | --- | --- |
| ST | M | Transaction Set Header segment |
| BPT | M | Statement Type |
| DTM | M | Statement Date/Time |
| PER | M M | Contact Person (Name) Contact Person (Phone) |
| N1 | M | Device Station Number |
| N1 | SO | Downstream Party/Downstream Party Proprietary Code |
| N1 | SO | Location Code/Location Proprietary Code |
| N1 | SO | Meter Operator/Meter Operator Proprietary Code |
| N1 | SO | Preparer ID/Preparer ID Proprietary Code |
| N1 | SO | Upstream Party/Upstream Party Proprietary Code |

Detail:

| Segment | Usage | Segment Name/NAESB Data Element Name |
| --- | --- | --- |
| PTD | - M | Detail segment (Flowing Gas Information) |
| DTM | - M SO | Physical Meter Effective Date Physical Meter Effective Time |
| REF | - C | Chromatograph |
| LCD | - M | Meter ID |
| LQ | - M | Meter Status |
| LQ | - M | Meter Type |
| LQ | - C | Tap Location |
| LQ | - C | Tap Type |
| MEA | - M M M | Maximum Static Pressure Range Minimum Static Pressure Range Unit of Measure |
| MEA | - M M | Reporting Pressure Base Unit of Measure |
| MEA | - C C | Chart Revolution Time Unit of Measure |
| MEA | - C | Machine Constant |
| MEA | - C C | Maximum Differential Pressure Unit of Measure |
| MEA | - C | Number Dials |
| MEA | - C C | Orifice Diameter Unit of Measure |
| MEA | - C C | Tube Inside Diameter Unit of Measure |
| MEA | - C | Volume Cycle |
| MEA | - SO C | Atmospheric Pressure Unit of Measure |
| MEA | - SO SO C | Temperature Range - Maximum Temperature Range – Minimum Unit of Measure |
| Sub-detail: |  |  |
| QTY | M M | Measured Quantity Unit of Measure |
| MEA | M M | Flow Period Unit of Measure |
| MEA | M | Specific Gravity |
| MEA | M M | Volume Unit of Measure |
| MEA | C | Coefficient |
| MEA | C C | Differential Pressure Unit of Measure |
| MEA | C | Flow Rate |
| MEA | C C | Heating Factor Unit of Measure |
| MEA | C | Index Differential |
| MEA | C | Integrated Differential |
| MEA | C | Pressure Factor |
| MEA | C C | Reporting Temperature Unit of Measure |
| MEA | C C | Static Pressure Unit of Measure |
| MEA | SO C C | Component Component Value Unit of Measure |
| MEA | SO C | Temperature Unit of Measure |
| MEA | MA | Extension |
| DTM | M M M M | Beginning Flow Date Beginning Flow Time Ending Flow Date Ending Flow Time |
| DTM | C C | Date/Time Off Date/Time On |
| DTM | C | Gas Analysis Effective Date |
| LM | M | Code Source Information segment |
| LQ | M | Business Period |
| LQ | M | Sample Device |
| LQ | C | Adjustment Type |
| LQ | C | Sample Type |

Summary:

| Segment | Usage | Segment Name/NAESB Data Element Name |
| --- | --- | --- |
| SE | M | Transaction Set Trailer segment |

SAMPLE ASC X12 TRANSACTION

(new)

ST*867*000000001

BPT*00**19970703*R5

DTM*102****DT*199707031030

PER*IC*MARY METERMAID*TE*8671235309

N1*RL**32*0012345-00

PTD*FG

DTM*007****D8*19970601

LCD**MTR***SV*0012345-12

LQ*MT*OM

LQ*MS*A

LQ*TL*DT

LQ*TP*FT

MEA**PU*14.65*80

MEA**SPS**80*0*500

MEA**T2*10.020*E8

MEA**M2*150*IF

MEA**01*6.000*E8

QTY*MS*7221*BZ

MEA**SPG*0.6000

MEA**TT*19.4*HR

MEA**FR*667

MEA**VOL*6903*TZ

MEA**A6*214*80

MEA**A5*26.2*IF

DTM*405****RDT*199706010900-199706020900

LM*GI

LQ*BSP*1

LQ*SMD*C

SE*29*000000001

867 Product Transfer and Resale Report

(new – not redlined)

Functional Group ID=PT

Heading:

Pos.	Seg.		Req.		Loop

No.	ID	Name	Des.	Max.Use	Repeat

| Must Use | 0100 | ST | Transaction Set Header | M | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Must Use | 0200 | BPT | Beginning Segment for Product Transfer and Resale | M | 1 |  |  |  |  |  |  |  |  |
| Must Use | 0500 | DTM | Date/Time Reference | O | 10 |  |  |  |  |  |  |  |  |
| Must Use | 0700 | PER | Administrative Communications Contact | O | 3 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - N1 |  |  | >1 |  |  |  |  |  |  |  |
| Must Use | 0800 | N1 | Party Identification | O | 1 |  |  |  |  |  |  |  |  |

Detail:

Pos.	Seg.		Req.		Loop

No.	ID	Name	Des.	Max.Use	Repeat

|  |  |  | LOOP ID - PTD |  |  | >1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Must Use | 0100 | PTD | Product Transfer and Resale Detail | M | 1 |  |  |  |  |  |  |  |  |
| Must Use | 0200 | DTM | Date/Time Reference | O | 10 |  |  |  |  |  |  |  |  |
|  | 0300 | REF | Reference Information | O | 20 |  |  |  |  |  |  |  |  |
| Must Use | 0475 | LCD | Place/Location Description | O | 2 |  |  |  |  |  |  |  |  |
| Must Use | 0490 | LQ | Industry Code Identification | O | >1 |  |  |  |  |  |  |  |  |
| Must Use | 0495 | MEA | Measurements | O | >1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - QTY |  |  | >1 |  |  |  |  |  |  |  |
| Must Use | 1100 | QTY | Quantity Information | O | 1 |  |  |  |  |  |  |  |  |
| Must Use | 1600 | MEA | Measurements | O | 40 |  |  |  |  |  |  |  |  |
| Must Use | 2100 | DTM | Date/Time Reference | O | 10 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - LM |  |  | >1 |  |  |  |  |  |  |  |
| Must Use | 2600 | LM | Code Source Information | O | 1 |  |  |  |  |  |  |  |  |
| Must Use | 2700 | LQ | Industry Code Identification | O | 100 |  |  |  |  |  |  |  |  |

Summary:

Pos.	Seg.		Req.		Loop

No.	ID	Name	Des.	Max.Use	Repeat

| Must Use | 0300 | SE | Transaction Set Trailer | M | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Segment:	ST Transaction Set Header

Position:	0100

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | ST01 | 143 | Transaction Set Identifier Code |  |  |  | M | 1 | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 867 |  | Product Transfer and Resale Report |  |  |  |  |
| Must Use | ST02 | 329 | Transaction Set Control Number |  |  |  | M | 1 | AN 4/9 |  |

Segment:	BPT Beginning Segment for Product Transfer and Resale

Position:	0200

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | BPT01 | 353 | Transaction Set Purpose Code |  |  |  | M | 1 | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Statement Type |  |  |  |  |  |  |  |
|  |  |  |  | 00 |  | Original |  |  |  |  |
|  |  |  |  | 05 |  | Replace |  |  |  |  |
|  |  |  |  |  |  | Replacement |  |  |  |  |
|  |  |  |  | 14 |  | Advance Notification |  |  |  |  |
| Must Use | BPT03 | 373 | Date |  |  |  | M | 1 | DT 8/8 |  |
|  |  |  | The measured volume audit statement issue date. |  |  |  |  |  |  |  |
| Must Use | BPT04 | 755 | Report Type Code |  |  |  | O | 1 | ID 2/2 |  |
|  |  |  |  | R5 |  | Technical Information |  |  |  |  |

Segment:	DTM Date/Time Reference

Position:	0500

Loop:

Level:	Heading

Usage:	Optional (Must Use)

Max Use:	10

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M | 1 | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 102 |  | Issue |  |  |  |  |
| Must Use | DTM05 | 1250 | Date Time Period Format Qualifier |  |  |  | X | 1 | ID 2/3 |  |
|  |  |  |  | DT |  | Date and Time Expressed in Format CCYYMMDDHHMM |  |  |  |  |
| Must Use | DTM06 | 1251 | Date Time Period |  |  |  | X | 1 | AN 1/35 |  |
|  |  |  | Statement Date/Time |  |  |  |  |  |  |  |

Segment:	PER Administrative Communications Contact

Position:	0700

Loop:

Level:	Heading

Usage:	Optional (Must Use)

Max Use:	3

| Notes: |  | For NAESB WGQ, the segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | PER01 | 366 | Contact Function Code |  |  |  | M | 1 | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | IC |  | Information Contact |  |  |  |  |
| Must Use | PER02 | 93 | Name |  |  |  | O | 1 | AN 1/35 |  |
|  |  |  | Contact Person (Name) The data element maximum length indicated is reduced from that which is specificed in the ASC X12 standards. |  |  |  |  |  |  |  |
| Must Use | PER03 | 365 | Communication Number Qualifier |  |  |  | X | 1 | ID 2/2 |  |
|  |  |  |  | TE |  | Telephone |  |  |  |  |
| Must Use | PER04 | 364 | Communication Number |  |  |  | X | 1 | AN 1/80 |  |
|  |  |  | Contact Person (Phone) The data element maximum length indicated is reduced from that which is specificed in the ASC X12 standards. |  |  |  |  |  |  |  |

Segment:	N1 Party Identification

Position:	0800

Loop:	N1        Optional (Must Use)

Level:	Heading

Usage:	Optional (Must Use)

Max Use:	1

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | N101 | 98 | Entity Identifier Code | M | 1 | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Refer to "N1 Segments (Heading)" table for usage and values. |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier | X | 1 | ID 1/2 |  |
|  |  |  | Refer to "N1 Segments (Heading)" table for usage and values. |  |  |  |  |
| Must Use | N104 | 67 | Identification Code | X | 1 | AN 2/17 |  |
|  |  |  | Refer to "N1 Segments (Heading)" table for usage and values. Device Station Number, Downstream Party/Downstream Party Proprietary Code, Location Code/Location Proprietary Code, Meter Operator/Meter Operator Proprietary Code, Preparer ID/Preparer ID Proprietary Code, Upstream Party/Upstream Party Proprietary Code The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |

Segment:	PTD Product Transfer and Resale Detail

Position:	0100

Loop:	PTD        Mandatory

Level:	Detail

Usage:	Mandatory

Max Use:	1

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | PTD01 | 521 | Product Transfer Type Code |  |  |  | M | 1 | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FG |  | Flowing Gas Information |  |  |  |  |

Segment:	DTM Date/Time Reference

Position:	0200

Loop:	PTD        Mandatory

Level:	Detail

Usage:	Optional (Must Use)

Max Use:	10

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier | M | 1 | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Refer to "DTM Segments (Detail)" table for usage and values. |  |  |  |  |
| Must Use | DTM05 | 1250 | Date Time Period Format Qualifier | X | 1 | ID 2/3 |  |
|  |  |  | Refer to "DTM Segments (Detail)" table for usage and values. |  |  |  |  |
| Must Use | DTM06 | 1251 | Date Time Period | X | 1 | AN 1/35 |  |
|  |  |  | Refer to "DTM Segments (Detail)" table for usage and values. Physical Meter Effective Date, Physical Meter Effective Time |  |  |  |  |

Segment:	REF Reference Information

Position:	0300

Loop:	PTD        Mandatory

Level:	Detail

Usage:	Optional

Max Use:	20

| Notes: |  | For NAESB WGQ, this segment is conditional. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M | 1 | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | CHR |  | Chromatograph Identifier |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X | 1 | AN 1/30 |  |
|  |  |  | Chromatograph The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |

Segment:	LCD Place/Location Description

Position:	0475

Loop:	PTD        Mandatory

Level:	Detail

Usage:	Optional (Must Use)

Max Use:	2

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | LCD02 | 98 | Entity Identifier Code |  |  |  | O | 1 | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MTR |  | Meter Location |  |  |  |  |
| Must Use | LCD05 | 66 | Identification Code Qualifier |  |  |  | X | 1 | ID 1/2 |  |
|  |  |  |  | SV |  | Service Provider Number |  |  |  |  |
| Must Use | LCD06 | 67 | Identification Code |  |  |  | X | 1 | AN 2/17 |  |
|  |  |  | Meter ID The data element maximum length indicated is reduced from that which is specified in the ASC X12 standards. |  |  |  |  |  |  |  |

Segment:	LQ Industry Code Identification

Position:	0490

Loop:	PTD        Mandatory

Level:	Detail

Usage:	Optional (Must Use)

Max Use:	>1

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | LQ01 | 1270 | Code List Qualifier Code | O | 1 | ID 1/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Refer to "LQ Segments (Detail')" table for usage and values. |  |  |  |  |
| Must Use | LQ02 | 1271 | Industry Code | X | 1 | AN 1/30 |  |
|  |  |  | Refer to "LQ Segments (Detail')" table for usage and values. Meter Status, Meter Type, Tap Location, Tap Type |  |  |  |  |

Segment:	MEA Measurements

Position:	0495

Loop:	PTD        Mandatory

Level:	Detail

Usage:	Optional (Must Use)

Max Use:	>1

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | MEA02 | 738 | Measurement Qualifier | O | 1 | ID 1/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Refer to "MEA Segments (Detail)" table for usage and values. |  |  |  |  |
|  | MEA03 | 739 | Measurement Value | X | 1 | R 1/20 |  |
|  |  |  | Refer to "MEA Segments (Detail)" table for usage and values. Reporting Pressure Base, Chart Revolution Time, Machine Constant, Maximum Differential Pressure, Number Dials, Orifice Diameter, Tube Inside Diameter, Volume Cycle, Atmospheric Pressure |  |  |  |  |
|  | MEA04 | C001 | Composite Unit of Measure | X | 1 |  |  |
| Must Use | C00101 | 355 | Unit or Basis for Measurement Code | M |  | ID 2/2 |  |
|  |  |  | Refer to "MEA Segments (Detail)" table for usage and values. Unit of Measure |  |  |  |  |
|  | MEA05 | 740 | Range Minimum | X | 1 | R 1/20 |  |
|  |  |  | Refer to "MEA Segments (Detail)" table for usage and values. Minimum Static Pressure Range, Temperature Range Minimum |  |  |  |  |
|  | MEA06 | 741 | Range Maximum | X | 1 | R 1/20 |  |
|  |  |  | Refer to "MEA Segments (Detail)" table for usage and values. Maximum Static Pressure Range, Temperature Range Maximum |  |  |  |  |

Segment:	QTY Quantity Information

Position:	1100

Loop:	QTY        Optional (Must Use)

Level:	Detail

Usage:	Optional (Must Use)

Max Use:	1

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | QTY01 | 673 | Quantity Qualifier |  |  |  | M | 1 | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MS |  | Measured Quantity |  |  |  |  |
| Must Use | QTY02 | 380 | Quantity |  |  |  | X | 1 | R 1/15 |  |
|  |  |  | Measured Quantity |  |  |  |  |  |  |  |
|  | QTY03 | C001 | Composite Unit of Measure |  |  |  | O | 1 |  |  |
| Must Use | C00101 | 355 | Unit or Basis for Measurement Code |  |  |  | M |  | ID 2/2 |  |
|  |  |  | Unit of Measure |  |  |  |  |  |  |  |
|  |  |  |  | BZ |  | Million BTU's |  |  |  |  |
|  |  |  |  | G8 |  | Gigacalories |  |  |  |  |
|  |  |  |  | GV |  | Gigajoules |  |  |  |  |

Segment:	MEA Measurements

Position:	1600

Loop:	QTY        Optional (Must Use)

Level:	Detail

Usage:	Optional (Must Use)

Max Use:	40

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

|  | MEA01 | 737 | Measurement Reference ID Code | O | 1 | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Refer to "MEA Segments (Sub-detail)" table for usage and values. |  |  |  |  |
| Must Use | MEA02 | 738 | Measurement Qualifier | O | 1 | ID 1/3 |  |
|  |  |  | Refer to "MEA Segments (Sub-detail)" table for usage and values. Component |  |  |  |  |
| Must Use | MEA03 | 739 | Measurement Value | X | 1 | R 1/20 |  |
|  |  |  | Refer to "MEA Segments (Sub-detail)" table for usage and values. Flow Period, Specific Gravity, Volume, Coefficient, Differential Pressure, Flow Rate, Heating Factor, Index Differential, Integrated Differential, Pressure Factor, Reporting Temperature, Static Pressure, Component Value,Temperature, Extension |  |  |  |  |
|  | MEA04 | C001 | Composite Unit of Measure | X | 1 |  |  |
| Must Use | C00101 | 355 | Unit or Basis for Measurement Code | M |  | ID 2/2 |  |
|  |  |  | Refer to "MEA Segments (Sub-detail)" table for usage and values. Unit of Measure |  |  |  |  |

Segment:	DTM Date/Time Reference

Position:	2100

Loop:	QTY        Optional (Must Use)

Level:	Detail

Usage:	Optional (Must Use)

Max Use:	10

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier | M | 1 | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Refer to "DTM Segments (Sub-detail)" table for usage and values. |  |  |  |  |
| Must Use | DTM05 | 1250 | Date Time Period Format Qualifier | X | 1 | ID 2/3 |  |
|  |  |  | Refer to "DTM Segments (Sub-detail)" table for usage and values. |  |  |  |  |
| Must Use | DTM06 | 1251 | Date Time Period | X | 1 | AN 1/35 |  |
|  |  |  | Refer to "DTM Segments (Sub-detail)" table for usage and values. Beginning Flow Date, Beginning Flow Time, Ending Flow Date, Ending Flow Time, Date/Time Off, Date/Time On, Gas Analysis Effective Date |  |  |  |  |

Segment:	LM Code Source Information

Position:	2600

Loop:	LM        Optional (Must Use)

Level:	Detail

Usage:	Optional (Must Use)

Max Use:	1

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | LM01 | 559 | Agency Qualifier Code |  |  |  | M | 1 | ID 2/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | GI |  | Gas Industry Standards Board |  |  |  |  |

Segment:	LQ Industry Code Identification

Position:	2700

Loop:	LM        Optional (Must Use)

Level:	Detail

Usage:	Optional (Must Use)

Max Use:	100

| Notes: |  | For NAESB WGQ, this segment is mandatory. |
| --- | --- | --- |

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | LQ01 | 1270 | Code List Qualifier Code | O | 1 | ID 1/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Refer to "LQ Segments (Sub-detail)" table for usage and values. |  |  |  |  |
| Must Use | LQ02 | 1271 | Industry Code | X | 1 | AN 1/30 |  |
|  |  |  | Refer to "LQ Segments (Sub-detail)" table for usage and values. Business Period, Sample Device, Adjustment Type, Sample Type |  |  |  |  |

Segment:	SE Transaction Set Trailer

Position:	0300

Loop:

Level:	Summary

Usage:	Mandatory

Max Use:	1

Data Element Summary

Ref.	Data

Des.	Element	Name	    Attributes

| Must Use | SE01 | 96 | Number of Included Segments | M | 1 | N0 1/10 |
| --- | --- | --- | --- | --- | --- | --- |
| Must Use | SE02 | 329 | Transaction Set Control Number | M | 1 | AN 4/9 |

TRANSACTION SET TABLES

(new)

N1 Segments (Heading)

| Element Name (N104) | Usage | N101 | N103 | N103 Description |
| --- | --- | --- | --- | --- |
| Device Station Number | M | RL | 32 | The station number assigned to this device by the operator. |
| Downstream Party/Downstream Party Proprietary Code | SO | DW | 1 | D-U-N-S Number, Dun & Bradstreet (see n2) |
|  |  |  | SV | Transportation Service Provider’s proprietary code (see n2) |
| Location Code/Location Proprietary Code | SO | LCN | DR | NAESB/PI Data Reference Number (DRN) (see n1) |
|  |  |  | SV | Transportation Service Provider’s proprietary code (see n1) |
| Meter Operator/Meter Operator Proprietary Code | SO | OP | 1 | D-U-N-S Number, Dun & Bradstreet (see n2) |
|  |  |  | SV | Transportation Service Provider’s proprietary code (see n2) |
| Preparer ID/Preparer ID Proprietary Code | SO | 41 | 1 | D-U-N-S Number, Dun &Bradstreet (see n2) |
|  |  |  | SV | Transportation Service Provider’s proprietary code (see n2) |
| Upstream Party/Upstream Party Proprietary Code | SO | US | 1 | D-U-N-S Number, Dun & Bradstreet (see n2) |
|  |  |  | SV | Transportation Service Provider’s proprietary code (see n2) |

Notes:

n1	When a Transportation Service Provider’s proprietary location code is employed pursuant to this standard, the parties agree that nominations, confirmations, scheduled quantities, and capacity release documents employing such code should be for one gas day at a time, and used only until there is a verified common code for the point associated with the proprietary location code. This would include daily nominations over a weekend. Within two months following the availability of the location the parties should employ the common code and no longer employ the proprietary code for identifying such location in the datasets related to the identified standards.

n2	Parties should mutually agree to use the Transportation Service Provider’s proprietary entity code when the D-U-N-S® Number is not available.

DTM Segments (Detail)

| Element Name (DTM06) | Usage | DTM01 | DTM05 | DTM05 Description |
| --- | --- | --- | --- | --- |
| Physical Meter Effective Date | M | 007 | D8 | CCYYMMDD |
| Physical Meter Effective Date Physical Meter Effective Time (see n1) | M SO | 007 | DT | CCYYMMDDHHMM |

Notes:

n1:	Physical Meter Effective Date is mandatory.  Physical Meter Effective Time is sender’s option.  If sending the Physical Meter Effective Time, it must be sent with the Physical Meter Effective Date.

LQ Segments (Detail)

| Element Name (LQ02) | Usage | LQ01 | LQ02 | LQ02 Description |
| --- | --- | --- | --- | --- |
| Meter Status | M | MS | A | Active |
|  |  |  | R | Removed |
|  |  |  | S | Standby |
| Meter Type | M | MT | OC | Orifice - chart |
|  |  |  | OM | Orifice - EFM |
|  |  |  | DC | Time Driven Positive Displacement - chart |
|  |  |  | DM | Time Driven Positive Displacement - EFM |
|  |  |  | FD | Flow Driven Positive Displacement |
|  |  |  | RM | Rotary |
|  |  |  | TM | Turbine |
|  |  |  | UM | Ultrasonic |
|  |  |  | VM | Venturi |
| Tap Location | C | TL | DT | Downstream Tap |
|  |  |  | UT | Upstream Tap |
| Tap Type | C | TP | FT | Flange Tap |
|  |  |  | PT | Pipe Tap |

MEA Segments (Detail)

| Element Name | Usage | MEA02 | MEA04 | MEA04 Description |
| --- | --- | --- | --- | --- |
| Maximum Static Pressure Range (MEA06) Minimum Static Pressure Range (MEA05) Unit of Measure (MEA04) | M M M2 | SPS | 64 80 KQ | PSIG PSIA Kilopascal |
| Reporting Pressure Base (MEA03) Unit of Measure (MEA04) | M1 M2 | PU | 64 80 KQ | PSIG PSIA Kilopascal |
| Chart Revolution Time (MEA03) Unit of Measure (MEA04) | C C1 | TT | DA HR | Days Hours |
| Machine Constant (MEA03) | C | MEF | not used | not used |
| Maximum Differential Pressure (MEA03) Unit of Measure (MEA04) | C C1 | M2 | IF KQ PS | Inches of Water Kilopascal PSI |
| Number Dials (MEA03) | C | DLC | not used | not used |
| Orifice Diameter (MEA03) Unit of Measure (MEA04) | C C1 | O1 | E8 M2 | Inches, Decimal - Actual Millimeter - Actual |
| Tube Inside Diameter (MEA03) Unit of Measure (MEA04) | C C1 | T2 | E8 M2 | Inches, Decimal - Actual Millimeter - Actual |
| Volume Cycle (MEA03) | C | VOL | not used | not used |
| Atmospheric Pressure (MEA03) Unit of Measure (MEA04) | SO C1 | S9 | 80 KQ | PSIA Kilopascal |
| Temperature Range Maximum (MEA06) Temperature Range Minimum (MEA05) Unit of Measure (MEA04) | SO SO C1 | TC | CE FA | Centigrade, Celsius Fahrenheit |

Usage:

C1	Data element usage in the data dictionary is conditional.  However, when sending the corresponding conditional or sender’s option data element, as noted above, the unit of measure is required.

M1	Mandatory when the reporting pressure base is not 14.73 (the default).

M2	Data element usage in the data dictionary is conditional.  However, when sending the corresponding mandatory data element, as noted above, the unit of measure is required.

MEA Segments (Sub-detail)

| Element Name | Usage | MEA01 | MEA02 | ME02 Description | MEA04 | MEA04 Description |
| --- | --- | --- | --- | --- | --- | --- |
| Flow Period (MEA03) Unit of Measure (MEA04) | M M1 | not used | TT | Time | DA HR | Days Hours |
| Specific Gravity (MEA03) | M | not used | SPG | Specific Gravity | not used | not used |
| Volume (MEA03) Unit of Measure (MEA04) | M M1 | not used | VOL | Volume | TZ | Thousand Cubic Feet |
| Coefficient (MEA03) | C | not used | COF | Coefficient Factor | not used | not used |
| Differential Pressure (MEA03) Unit of Measure (MEA04) | C C1 | not used | A5 | Average Differential Pressue | IF KQ PS | Inches of Water Kilopascal PSI |
| Flow Rate (MEA03) | C | not used | FR | Flow Rate | not used | not used |
| Heating Factor (MEA03) Unit of Measure (MEA04) | C C1 | not used | QA | Quality Index | JM M9 | Megajoule/Cubic Meter Million BTUs per 1000 Cubic Feet |
| Index Differential | C | not used | IXD | Index Differential | not used | not used |
| Integrated Differential (MEA03) | C | not used | ITD | Integrated Differential | not used | not used |
| Pressure Factor (MEA03) | C | not used | PRF | Pressure Factor | not used | not used |
| Reporting Temperature (MEA03) Unit of Measure (MEA04) | C C1 | not used | RTB | Reporting Temperature Base | CE FA | Centigrade, Celsius Fahrenheit |
| Static Pressure (MEA03) Unit of Measure (MEA04) | C C1 | not used | A6 | Average Static Pressure | 64 80 KQ | PSIG PSIA Kilopascal |
| Component (MEA02) Component Value (MEA03) Unit of Measure (MEA04) | SO C C1 | TR | H20 | Water | ZZ | Pounds per Million Cubic Feet |
|  |  |  | HC | Helium | P1 | Percent |
|  |  |  | ZBT | N-Butane | P1 | Percent |
|  |  |  | ZCD | Carbon Dioxide | P1 | Percent |
|  |  |  | ZET | Ethane | P1 | Percent |
|  |  |  | ZH | Hydrogen | 59 | Parts per million |
|  |  |  | ZHP | Heptane | P1 | Percent |
|  |  |  | ZHS | Hydrogen Sulfide | GX | Grains |
|  |  |  | ZHX | Hexane | P1 | Percent |
|  |  |  | ZIB | I-Butane | P1 | Percent |
|  |  |  | ZIP | I-Pentane | P1 | Percent |
|  |  |  | ZMT | Methane | P1 | Percent |
|  |  |  | ZN | Nitrogen | P1 | Percent |
|  |  |  | ZNP | Neo-Pentane | P1 | Percent |
|  |  |  | ZO | Oxygen | 59 | Parts per million |
|  |  |  | ZOC | Octane | P1 | Percent |
|  |  |  | ZPP | Propane | P1 | Percent |
|  |  |  | ZPT | N-Pentane | P1 | Percent |
|  |  |  | ZS | Sulphur | GX | Grains |
|  |  |  | ZZZ | Carbon Monoxide | P1 | Percent |
| Temperature (MEA03) Unit of Measure (MEA04) | SO C1 | not used | OBT | Observed Temperature | CE FA | Centigrade, Celsius Fahrenheit |
| Extension (MEA03) | MA | not used | MEF | Meter Factor | not used | not used |

Usage:

C1	Data element usage in the data dictionary is conditional.  However, when sending the corresponding conditional or sender’s option data element, as noted above, the unit of measure is required.

M1	Data element usage in the data dictionary is conditional however, when sending the corresponding mandatory data element as noted above, the unit of measure is mandatory.

DTM Segments (Sub-detail)

| Element Name (DTM06) | Usage | DTM01 | DTM05 | DTM05 Description |
| --- | --- | --- | --- | --- |
| Beginning Flow Date Beginning Flow Time Ending Flow Date Ending Flow Time | M | 405 | RDT RD8 | CCYYMMDDHHMM-CCYYMMDDHHMM CCYYMMDD-CCYYMMDD |
| Date/Time Off Date/Time On | C | 119 | RDT | CCYYMMDDHHMM-CCYYMMDDHHMM |
| Gas Analysis Effective Date | C | 799 | D8 | CCYYMMDD |

LQ Segments (Sub-detail)

| Element Name (LQ02) | Usage | LQ01 | LQ02 | LQ02 Description |
| --- | --- | --- | --- | --- |
| Business Period | M | BSP | 1 | Current |
|  |  |  | 2 | Prior Period |
| Sample Device | M | SMD | B | Bottle |
|  |  |  | C | Chromatograph |
|  |  |  | I | Inline |
| Adjustment Type | C | AJT | ARN | Audit Review - No Adjustment Necessary |
|  |  |  | DEE | Data Entry Error |
|  |  |  | ECF | EFM Communication Failure |
|  |  |  | ERA | Estimate Replaced with Actual: Late |
|  |  |  | ICA | Incorrect Chromatograph Assignment/Data |
|  |  |  | ICT | Invalid Calculation Type |
|  |  |  | IEC | Incorrect EFM Station Configuration |
|  |  |  | IGQ | Incorrect Gas Quality |
|  |  |  | IGS | Invalid Gas Sample |
|  |  |  | IPI | Integration/Scan Process Incorrect |
|  |  |  | LNS | Late Notification of Station Turn On |
|  |  |  | MCN | Measured Change Not Received |
|  |  |  | MEF | Mechanical Field Equipment Failure |
|  |  |  | MNV | Missing Data, No Volume Estimate |
|  |  |  | QRL | Quality Data Received Late |
|  |  |  | VEN | Volume Estimate - No Data Received |
| Sample Type | C | SMT | A | Accumulated |
|  |  |  | S | Spot |